3GPP TSG RAN Meeting #101	RP-231960

Bangalore, India, September 11-15, 2023

Agenda item:	8A.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Rel-19 Mobility Enhancements

Document for:	Discussion and decision

# 1	Introduction

Mobility enhancement in Rel-18 [1] has introduced a new mobility procedure, low layer triggered mobility (LTM) among advances such as selective activation of secondary cell group, conditional handover with multiple candidate PSCells and early measurement reporting enhancements. The on-going Rel-18 work is followed by many companies. The similar interest is raised at the Rel-19 RAN Workshop in June 2023 where mobility enhancements were discussed and numerous companies expressed the need to consider multiple mobility enhancements in Rel-19, such as inter-CU LTM and RACHless handover.

In this paper, we provide the justification and details of a potential mobility enhancement work item objectives based on the Rel-19 workshop content [2].

# 2	Discussion

## 2.1	Motivation

LTM is a procedure that brings the benefit of the disaggregated RAN architecture to mobility procedure via allowing the single entity to coordinate the procedure and delegate this to the distributed units. Currently, the LTM procedure is limited in intra-CU deployments only. This limits the usefulness of this procedure since (a) forces the network to use two different mobility procedures, LTM in intra-CU and baseline handover or conditional handover for inter-CU scenarios and (b) doesn’t introduce the merits of LTM in inter-CU deployments. Considering the above, we see that network can benefit from the introduction of inter-CU LTM. Additionally, LTM has been predominately considered for PCell change without considering either how it would interwork with DC. Specifically, LTM PCell change with/without PSCell should be analyzed together with LTM PSCell change with/without SRB3. Finally, methods to enhance TA acquisition in LTM should be considered. Specifically, Rel-18 is restricted to TA acquisition without RAR (TA is shared from the serving DU with the cell switch command); this can be suboptimum since it may introduce unacceptable delay thus requiring new ways to estimate/acquire/maintain TA.

The interruption time of the mobility procedures needs to be reduced to make sure the connection up time is enhanced. This is critical for use-cases that has strict end to end delay requirements. This is previously specified through dual active protocol stack, however, this is a complex feature and other enhancements can be achieved to lower the interruption time, such as RACH-less handover. However, these improvements currently cannot be used by layer 3 mobility features, as such the RACHless handover procedure should be defined for NR, also including conditional handover.

In case UEs mobility pattern is not changing frequently, UE is executing handover within only a set of PCells. After each cell change the target cell configuration has to be re-signalled to the UE. This means UE has to receive almost the same target cell information over and over. The subsequent cell changes with rapidly changing UE mobility pattern is an issue for preparation and execution of the handover. In such scenarios the signalling from the network side may prove to be quite slow. Using selective activation of MCG can be a solution to resolve this issue.

Beam specific mobility key performance metrics have been introduced in SA5 to enable beam specific Mobility robustness optimisation, e.g., the network can collect successful handover statistics with respect to serving beam and the target beam. Optimising handover decisions based on the serving beam, as it provides more granular information, decreases mobility failures. Currently, update of handover parameters such as time to trigger and cell individual offset requires updating these parameters through RRC signalling each time UE moves to a new beam. This can be avoided if the handover parameters such as TTT and CIO are defined for beam or beam group.

Reducing secondary link establishment delay for Pscell/Pcell setup significantly enhances the user throughput and data transmission latency. It also enables the network to timely perform load balancing and contributes to lower user energy consumption by reducing the required time to stay in RRC connected mode for data transmission/reception and enables the user equipment with faster transition from RRC connected to idle/inactive modes. This is especially important when the UE switches from the idle/inactive to connected mode as the UE perform less inter-frequency measurement and the network information about the UE measurements is limited. Considering the above, we believe Rel-19 Further enhanced Early Measurement  and Reporting (FeEMR) can well address the remining issues of Rel-16 EMR and enables fast DC/CA setup.

## 2.2	Potential objectives

Our proposal for the detailed objectives of this work item are:

- Specify inter-CU L1/L2 triggered mobility procedure. [RAN1, RAN2, RAN3, RAN4]

  - Rel. 18 LTM procedure is considered as baseline.

  - Including interruption time enhancements.

  - RRC configuration and interworking with L3 mobility

  - Leftovers from Rel. 18 LTM are not precluded, L1 event-triggered reporting, L3 measurement reporting in L1, Secondary Cell Group LTM, etc.

- Specify reduced interruption time for mobility procedures including RACHless HO procedure including CHO and HO. [RAN1, RAN2, RAN3, RAN4]

  - Rel. 18 timing advance acquisition is considered as baseline.

- Specify selective activation for MCG (L3 mobility)  [RAN2, RAN3, RAN4]

  - Rel. 18 selective activation for SCG is considered as baseline.

  - Excluding – LTM work.

  - Selective activation is limited to CHO with single connectivity.

- Specify signaling for beam specific handover [RAN2, RAN3, RAN4]

  - Handover parameters, e.g., TTT, CIO are extended to trigger handover based on beam\beam group specific manner.

  - Specify if needed signaling for disaggregated architecture.

- Specify solutions which improves UE measurement and reporting behavior which enhances faster DC/CA setup when the UE switches idle/inactive mode to RRC connected-mode [RAN4, RAN2]

  - Specify UE measurement and reporting configuration in idle/inactive mode that extends to connected mode.

### 2.3 Detailed description on selected objectives

Detailed discussions on Objective 2

- In LTE RACHless handover is possible if the timing advance is 0. In NR RACHless handover procedure is not enabled. Moreover, in Rel-18, also there are scenarios where UE can acquire the timing advance for the target cell for the LTM procedure. Two procedures specified are early timing advance acquisition procedure and UE based timing advance estimation.

  - The first part of the objective is to define a handover procedure where the handover is executed in a RACHless manner.

    - This would be agnostic to the timing advance acquisition procedure used.

  - Second part of the objective is to define how the UE can re-use the timing advance acquisition techniques defined for Rel-18 LTM in L3 handover procedures. In case of high load in the work item, the second part can be subject to downscoping.

Detailed discussions on Objective 4

- The current handover parameters such as TTT and CIO are cell specific. Every time UE moves to a new beam,  the network has to reconfigure the UE with new TTT and CIO values to use beam specific optimisation of handover parameters. In order to avoid that,

  - Enhance TTT and CIO to be beam/beam-group-specific in order to enhance conditional configuration execution and reporting configuration.

  - In case of disaggregated architecture, the beam switch information might not be visible to the centralized unit to make a beam specific handover decision at RRC level. So the beam change indication can be forwarded to the centralized unit to make handover decisions based on the current beam of the UE.

# 3	Conclusions

In this contribution, we have discussed the motivation and scope proposal for the work item on Mobility Enhancements in Rel-19 and provided details of the work that each objective requires.  The following is proposed:

Proposal:	Agree to include a RAN2-led work item on Mobility enhancements in Rel-19 including the objectives as proposed in the section 2.2 of this contribution.

# References

- RP-223520, Rel. 18 Mobility Enhancements Work Item Description

- RWS-230040, Mobility Enhancements in Rel-19, 3GPP TSG-RAN Rel. 19 Workshop, Taipei, June 15-16, 2023