3GPP TSG RAN Meeting #101							RP-231977

Bangalore, India, September 11-15, 2023

Agenda Item:	9.3.1.9

Source:	Apple Inc.

Title:	On Rel-18 positioning scope, progress, and timely completion

Document for:	Discussion/Decision

# Introduction

The WI on Expanded and Improved NR Positioning [1] has been progressing slower than expected. This, combined with the fact that the WI scope as it currently stands is rather ambitious, presents a challenge to complete the work on time in Rel-18.

Furthermore, in the last RAN2 meeting RAN2#123, the group acknowledged the situation as captured in the positioning session meeting minutes [2]:

RAN2 see risk to completion of sidelink positioning with the current scope.

In this contribution we elaborate on the situation and provide our views on how to resolve the issue in order to ensure timely completion of the WI and high quality Rel-18 specifications.

# Discussion

The WID [2] has the following positioning technologies in the scope for Rel-18:

- SL positioning

- RAT-dependent integrity

- LPHAP

- RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning

Among these, the SL positioning objective is by far the largest as it requires a brand new protocol and has in its scope many scenarios (e.g. in-coverage, out-of-coverage and partial coverage) all of which require standardization of different architectures which are radically different from the “legacy” Uu-based positioning.

Observation 1: SL positioning is by far the largest objective of the Rel-18 positioning WI.

Incidentally, SL positioning is the one objective which is lagging behind the most. For example, RAN2 have not yet agreed on the basic stage-2 call flows, let alone protocol details.

Observation 2: SL positioning is the one objective which is lagging behind the most.

## WI status in RAN2

In this and subsequent sections, we suggest a number of potential scope reduction options for discussion. These are proposed from the point of view of RAN2 progress, under the assumption that RAN1 and SA2 (which are the WGs heavily involved in the WI) do complete their parts in time.

Before we discuss the details of the scope reduction, we would like to stress the observation that with 2 meetings remaining, it is not possible to complete the WI in Rel-18. Not only that, if we are to downscope, the scope reduction must be significant.

Which brings us to an alternative proposal for discussion – one possibility is to continue the work in its current scope, with the understanding it will not be part of the Rel-18 feature and ASN.1 freeze and will be completed in Rel-19.

Proposal 1: to discuss whether to keep the current scope of the positioning WI and complete the work in Rel-19 or to reduce the scope significantly.

## WI scope reduction options

The following SL positioning methods are currently in the scope of the WI: RTT, SL-AoA, and SL-TDOA. One possibility for scope reduction is to eliminate some of the positioning methods supported in Rel-18. While acknowledging that the work remaining in RAN2 to design the protocol for coordination needed for SL-TDOA is not insignificant, we see some difficulties with this particular scope reduction for the following reasons:

- RAN1 have already finalized their part to support RTT, SL-AoA and SL-TDOA

- SL-TDOA is the only positioning methods that would support the true equivalent of the “legacy” UE-based positioning, where the final position calculation is performed in the target UE

From our perspective, the latter is particularly important for the reasons of protecting user privacy. On the other hand, eliminating RTT seems unnecessary as it is a simpler option and if SL-TDOA is supported, RTT can be supported relatively easily (from RAN2 point of view).

Observation 3: SL-TDOA is the only positioning methods that would support the true equivalent of the “legacy” UE-based positioning, which is important from user privacy considerations.

As far as coverage scenarios go, the WID mandates support of all coverage scenarios, specifically: out-of-coverage, in-coverage and partial-coverage. It must be noted that from RAN2 perspective, the main difference is not so much whether a UE is in coverage of a cell, but whether it has direct connectivity to an LMF. In case of partial coverage, some UEs do not have such direct connection and therefore some form of relaying/forwarding functionality is needed, which increases the scope. Out-of-coverage and in-coverage would be radically different, as there is no connectivity to an LMF at all in the former case.

Scope reduction option 2: remove partial coverage from Rel-18 scope.

Unfortunately, as far as coverage scenarios go, removing only the partial coverage is not sufficient to complete the WI in time. In other words, either in-coverage or out-of-coverage must be removed too. Among these options, we think that out-of-coverage (referred to as “UE-based operation” in SA2) is the most important as this is the case where “legacy” Uu-based positioning cannot be used. This is the single most important SL positioning feature.

Scope reduction option 3: limit the Rel-18 to out-of-coverage only (i.e. only support “UE-only Operation”), which would also imply removing joint PC5-Uu positioning from the Rel-18 scope.

A number of additional features, not explicitly spelled out in the WID but rather coming from SA2 [3], may also be candidates for scope reduction.

SLPP messages can be delivered using unicast, but in some cases multicast/broadcast can be beneficial to reduce latency. This is an optimization which would be nice to have, but at this stage we cannot consider optimizations, hence the proposal.

Scope reduction option 4: no support in Rel-18 for multicast/broadcast SLPP signalling.

SA2 have also introduced the option to support multiple target UEs in the same positioning session. This is clearly an optimization and for the same reason as the one mentioned above we cannot support it in Rel-18.

Scope reduction option 5: no support in Rel-18 for multiple target UEs in SLPP signalling.

Additionally, in SA2 specification there is the option to support SL positioning server UE collocated with the target UE and a standalone SL positioning server UE, which would be a UE not involved in a positioning session but responsible for SL position calculation. We think that considering the situation, this is yet another feature which can be pushed to Rel-19.

Scope reduction option 6: no support in Rel-18 for standalone SL positioning server UE.

Proposal 2: if we are to strive to complete the positioning WI in Rel-18, consider the following scope reduction options:

- remove partial coverage from Rel-18 scope.

- limit the Rel-18 to out-of-coverage only (i.e. only support “UE-only Operation”), which would also imply removing joint PC5-Uu positioning from the Rel-18 scope.

- no support in Rel-18 for multicast/broadcast SLPP signalling.

- no support in Rel-18 for multiple target UEs in SLPP signalling.

- no support in Rel-18 for standalone SL positioning server UE.

As we mentioned above, RAN1 and SA2 have progressed under the assumption the WI will be completely in Rel-18 in its entirety. Whether we agree to push the WI to Rel-19 or reduce the scope, at least SA2 must be notified via a LS.

Proposal 3: send a LS to SA2, CC RAN1, to notify about our decision.

A revised WID draft is provided in the appendix below.

# Conclusion

Observation 1: SL positioning is by far the largest objective of the Rel-18 positioning WI.

Observation 2: SL positioning is the one objective which is lagging behind the most.

Observation 3: SL-TDOA is the only positioning methods that would support the true equivalent of the “legacy” UE-based positioning, which is important from user privacy considerations.

Proposal 1: to discuss whether to keep the current scope of the positioning WI and complete the work in Rel-19 or to reduce the scope significantly.

Proposal 2: if we are to strive to complete the positioning WI in Rel-18, consider the following scope reduction options:

- remove partial coverage from Rel-18 scope.

- limit the Rel-18 to out-of-coverage only (i.e. only support “UE-only Operation”), which would also imply removing joint PC5-Uu positioning from the Rel-18 scope.

- no support in Rel-18 for multicast/broadcast SLPP signalling.

- no support in Rel-18 for multiple target UEs in SLPP signalling.

- no support in Rel-18 for standalone SL positioning server UE.

Proposal 3: send a LS to SA2, CC RAN1, to notify about our decision.

# Reference

- RP-231460 Revised WID on Expanded and Improved NR Positioning

- R2-2308964 Report from session on positioning and sidelink relay

- TS 23.586 Architectural Enhancements to support Ranging based services and Sidelink Positioning

# Appendix – WID modifications

# Appendix – WID modifications

# 4	Objective

### 4.1	Objective of Core part WI

The objective of this work item is to specify solutions to introduce sidelink ranging/positioning, to introduce integrity for RAT-dependent positioning methods, to enable LPHAP use-case 6 defined in TS 22.104, to improve positioning accuracy, and to introduce support of positioning for RedCap UEs.

The specific objectives of this work item are:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in the out-of-coverage scenario [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

  - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including:

  - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4]

    - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning.

      - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap.

    - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs

  - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3].

    - SRS for positioning configurations in multiple cells [RAN2, RAN1].

      - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

    - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3].

    - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

  - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

  - Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2].

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs)

  - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

    - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs.

  - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4].

  - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2].

    - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

  - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements with measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

- Define extensions of signalling, protocol, and procedure for NR positioning enhancement, as needed for the above objectives [RAN3].