3GPP TSG RAN Meeting #101	RP-231985

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.4

Source:	InterDigital, Inc.

Title:	On Rel-19 Ambient IoT

Document for:	Discussion

# 1. Introduction

3GPP SA has been studying ambient IoT networks in an ongoing study item [1]. One of the main motivations of this study is that despite various LPWA technologies specified by 3GPP, there is still unmet demand for even simpler and more power efficient solutions. Specifically, battery-free and maintenance free devices with ultra low complexity and longer life cycle are needed to address new use cases. The SA TR 22.840 lists thirty such use cases [1] which are listed in Table 1 below. The use cases can be classified into four main categories as inventory, sensor, positioning, and command. Both indoor and outdoor use cases are covered in the TR.

Most of the existing wireless communication devices are powered by batteries that need to be replaced or recharged. New IoT technologies supporting batteryless devices without energy storage capability or devices with energy storage that do not need to be recharged manually are needed to address new markets.  Based on this objective, 3GPP RAN also has an ongoing study item to investigate the feasibility of a new IoT technology which can provide complexity and power consumption orders-of-magnitude lower than existing 3GPP LPWA technologies [2]. The scope of the RAN SI is to address use cases and scenarios that cannot be fulfilled using existing 3GPP IoT technology.

Ambient IoT is a promising technology to fulfil the needs proposed by 3GPP SA and RAN. Ambient power-enabled Internet of Things device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g., using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, etc.

It is noteworthy to add that 802.11 has also formed a TIG/SG within the IEEE 802.11 working group that will address the problem of support of Ambient Power communication in 802.11 network [3].

Table 1 Ambient IoT use cases

| Inventory | Sensor | Positioning | Command |
| --- | --- | --- | --- |
| 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| Inventory | Sensor | Positioning | Command |
| 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |

# 2. Discussion

## Device Types

3GPP RAN and SA study items have considered three types of ambient IoT devices:

Device A (passive device): This device cannot independently generate an RF signal and does not have energy storage (beyond that is needed to run the device circuitry). Device A may modulate and reflect an incoming signal (i.e., backscatter communication). This type of device is considered to be similar to an RFID tag in terms of capability, power consumption, etc.

Device B (semi-passive device): This device is similar to Device A with the main difference being that Device B is assumed to have energy storage. The stored energy can be used to amplify the backscattered signal.

Device C (active device): This device can generate an RF signal and has energy storage. The differentiating features of this device are energy harvesting capability and much less complexity and power consumption than existing IoT devices.

Device A and B can be used in indoor and target use cases where cheap and large number of devices are needed. Device C, on the other hand, can be used both in indoor and outdoor. It is more of a conventional device with less complexity. So, it can be viewed as an evolution of 3GPP LPWA devices.

Each of the devices has a unique set of advantages and can be used for differing use cases. So, we do not think any one of them should be excluded from a potential 3GPP study phase.

Proposal 1: Study passive/semi-passive and active devices in Rel-19.

## Topologies

The four topologies included in the 3GPP RAN TR are illustrated in Figure 1:

Topology 1: This topology has a direct link from the base station to the IoT device.

Topology 2: In this topology, the base station to IoT device link is through an intermediate node.

Topology 3: In this topology, the node transmitting to the IoT device and the node receiving from the IoT device are different. For example, a base station may transmit to the IoT device in the DL and a UE may receive from the IoT device in the UL.

Topology 4: This is similar to topology 1 except that the direct link is between a UE and the IoT device.

Figure 1 Ambient IoT topologies

In topologies 1, 2, and 4, the node communicating with the IoT device should support some form of full duplex capability for device type A and B since the node transmits an energising signal (e.g., a carrier wave) and at the same time receives the backscattered signal from the IoT device. So, the receiver should be able to cancel the received carrier wave.

Out of these topologies, Topology 1 is the most essential one and should be considered as the baseline. The signals and channels applicable in Topology 1 (and higher layer processing) can be extended to other topologies.

Topology 2 can be used for coverage extension, but it is not necessary to study it in Rel-19.

Topology 3 does not need full duplex capability but may increase protocol complexity since some coordination is needed between the base station and the assisting node.

Topology 4 may be useful to open new markets for ambient IoT. For example, a UE can be used as a reader in a warehouse to track the movement of goods.

Proposal 2: Study Topologies 1, 3, and 4 in Rel-19.

## Spectrum

Ambient IoT can use licensed or the unlicensed bands. Certain restrictions such as limitations in transmit power or interference management in unlicensed bands make it not very appropriate for ambient IoT. Licensed bands offer higher reliability and service availability. FDD bands, in particular, can provide better coverage due to the lower carrier frequencies.

Proposal 3:  Consider licensed band for ambient IoT.

## Scope

The PHY layer and higher layer design is expected to be different for passive and active devices. Although a new redesign may be needed for passive devices to support backscattering, design for active devices may use the existing NR philosophy but should be significantly revised to support energy harvesting and reduced complexity/power consumption.

The design of the signals and channels for ambient IoT should support devices without energy storage and with lower complexity and higher power efficiency. For device A and B, PHY layer based on backscattering could be considered. In backscattering, used for RFID [4], the IoT device backscatters (reflects) and incoming signal, e.g., a carrier wave. Data can be embedded on the backscattered carrier wave using simple modulation schemes such as ASK or FSK. The device circuitry is energized by the carrier wave which enables the device receiver to perform simple receive operations. For device C, backscattering is not needed but a lean PHY layer is needed to reduce complexity and power consumption. Energy harvesting should also be considered.

For ambient IoT, the MAC layer should also be redesigned to support backscattering and energy harvesting. New collision avoidance and channel access mechanisms are needed to support IoT devices with high density.

A lean protocol stack should be designed for ambient IoT devices. For example, all or some of the PDCP/RLC functionalities may not needed. RRC should be simplified significantly.

Proposal 4: Study new PHY and higher layer protocols to support backscattering, energy harvesting, and reduced complexity/power consumption.

# Summary

In this contribution, we have discussed the scope of ambient IoT for Rel-19. The following are proposed:

Proposal 1: Study passive/semi-passive and active devices in Rel-19.

Proposal 2: Study Topologies 1, 3, and 4 in Rel-19.

Proposal 3:  Consider licensed band for ambient IoT.

Proposal 4: Study new PHY and higher layer protocols to support backscattering, energy harvesting, and reduced complexity/power consumption.

# References

[1] 3GPP TR 22.840

[2] 3GPP TR 38.848

[3] IEEE 802.11 AMP TIG/SG

[4] EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID