3GPP TSG RAN Meeting #101	RP-231987

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.6

Source:	InterDigital, Inc.

Title:	Mobility Enhancements for Rel-19

Document for:	Discussion, Decision

# Introduction

In Rel-15, the mobility support was limited to basic L3 handover and basic beam level mobility (L1/2 intra-cell).

The goal of conditional handover (CHO) in Rel-16 was to improve mobility robustness by reducing the radio link failure (RLF) and handover failure (HOF) rates through RRC pre-configuration of target cells, and configuration of a UE trigger (conditional trigger) by means of RRC measurement event. In Rel-17, additional scenarios were addressed (inter-SN, CPC, CPA) and in Rel-18 further enhancements were introduced (selective activation of SCG, CHO including MCG and target SCG for CPAC) A main limitation of CHO/CPAC is that handover interruption time is not reduced compared to basic L3 reconfiguration.

The goal of dual active protocol stack (DAPs) in Rel-16 was to reduce the handover interruption time by maintaining the source gNB connection for the DL for some selected bearers after the reception of the handover command (RRC Reconfiguration) until an RRC reconfiguration indicating the release of the source connection is received from the target (i.e., after the handover is completed). Limitations of DAPs include only being applicable to the DL, PCell handover, and no support for FR2-FR2.

The goal of inter-cell beam management (ICBM) in Rel-17 was to enable the UE to receive or transmit dedicated channels via a TRP associated with a PCI different from the PCI of a serving cell. A main limitation of ICBM is not supporting cell switch.

The goal of L1/L2 mobility enhancements in Rel-18 [1] is to enable a serving cell change via L1/L2 signalling in order to reduce the latency, overhead and interruption time by pre-configuration of multiple candidate cells by RRC, early L1 measurement reporting of target cells/beams, early DL synchronization (TCI state activation), early UL synchronization (TA acquisition), and handover (reconfiguration) trigger using MAC CE. Limitations of LTM in rel-18 include only being applicable to intra-gNB handover, mobility robustness (due to not supporting CHO), throughput degradation (due to not supporting early beam refinement, TRS tracking and CSI acquisition), and data loss during/after handover (due to not supporting HARQ continuation).

While each of the mobility enhancements introduced in previous releases provides improvements in one or multiple areas, each also has its own limitations. In Rel-19, the handover interruption time, latency, overhead can be addressed along with mobility robustness (RLF, HOF KPIs) and LTM can be supported in inter-gNB scenarios.

# Discussion

  - Summary of proposals to the Rel-19 workshop

A number of tdocs were submitted to the Rel-19 workshop, motivating further work for mobility enhancement, which can be broadly categorized as follows:

- Inter-gNB LTM

  - Security update and L2 reset.

  - Xn/NG enhancement

- LTM Measurement enhancements:

  - L1 measurement Dynamic update / overhead reduction / efficiency

  - Event triggered L1 report

  - CSI-RS support

  - Inter-Freq support

  - SINR support

  - Introduction of L1 Filtering

- CHO:

  - Conditional LTM (UE based LTM trigger)

  - RACH-less CHO

  - LTM/CHO co-existence

  - Inter-RAT CHO

- Other

  - Selective activation of MCG

  - Daps enhancement

  - Group handover

  - L3 measurement prioritisation

  - L3 measurement reporting enhancement

  - Inter-gNB LTM

[Motivation]

Rel-18 LTM is limited to the intra-gNB scenario. While DAPs could be used to reduce interruption in the inter-gNB scenario, this has other limitations which could require significant effort to overcome. Rather, extending LTM to cover the inter-gNB scenario would be more attractive to provide the reduced latency and interruption time.

[Proposed work]

To support LTM for inter-gNB, at least the security update needs to be supported in LTM cell switch and hence the security key information needs to be provided in some way in the LTM candidate configuration. Other L2 enhancements are likely to be needed, for example PDCP re-establishment is not performed for R18 intra-gNB cell switch.

One of the main benefits of LTM is reduced interruption time due to performing DL and UL synchronization to a target cell ahead of the cell switch to allow a RACH-less handover to occur when the L1 measurement reports indicate a suitable candidate cell. Therefore, the early synchronization procedures introduced in Rel-18, as well as the L1 reporting framework need to be extended to cover the inter-gNB scenario.

In addition, some enhancement to Xn and/or NG signalling is necessary to exchange the candidate configurations between gNBs.

For inter-gNB we propose the following objective:

- Proposal 1: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify mechanisms and procedures for L1/L2-triggered mobility for inter-gNB [RAN2, RAN1, RAN3]

- Signalling support for configuration of inter-gNB candidate cells, including Xn/NG interface signalling [RAN2, RAN3]

- Layer 2 enhancement including security update and PDCP anchor relocation. [RAN2]

- Support for inter-gNB TA management and early DL sync. [RAN2, RAN1, RAN3]

- Enhanced L1 measurement and reporting framework [RAN1, RAN2, RAN3]

  - CHO with LTM

[Motivation]

While LTM provides improved latency and handover interruption time, support for CHO improves robustness. Like in L3 mobility, LTM can suffer from RLF due to late L1 measurement reports or HOF due to late LTM command. Thus, by combining LTM with CHO, we allow to get the best of both worlds, with a handover mechanism that is both robust (in terms of RLF, HOF reduction) and with a reduced latency and interruption.

[Proposed work]

By utilizing the LTM TA acquisition and early DL sync ahead of a conditional (UE triggered) cell switch, we can improve the handover robustness, while allowing for low handover interruption by executing a RACH-less handover. The detailed mechanism needs to be discussed in the WGs, for example whether to (re)use a L3 based conditional reconfiguration evaluation or whether to introduce a L1 based conditional trigger, but the overall objective should be to enable a RACH-less CHO by re-using the early DL/UL sync procedures for LTM and executing LTM based on a UE evaluated trigger.

For CHO with LTM we propose the following objective:

- Proposal 2: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify mechanisms and procedures for RACH-less CHO [RAN2, RAN1, RAN3]

- Support for conditional (UE evaluated) trigger for RACH-less LTM execution. [RAN2, RAN1, RAN3]

Note: Early UL/DL sync and LTM execution introduced in Rel-18 should be the baseline.

  - LTM measurement enhancements

[Motivation]

LTM in Rel-18 relies on a relatively static configuration for L1 measurements, which results in significant overhead and/or limits the amount of pre-configuration that can be done. During Rel-18, enhancements were discussed to allow for a more efficient L1 measurement and reporting mechanism but due to lack of time/too large scope these have not been specified in Rel-18, for example L1 event triggered reporting and dynamic activation of L1 measurements was discussed in order to reduce overhead from frequent RRC reconfiguration to update handover/measurement parameters in different areas of the cell or group of LTM cells.

In addition, the Rel-18 LTM does not support CSI-RS measurements and therefore beam refinement ahead of LTM trigger is not possible, resulting in a reduced throughput after cell switch (i.e., beam refinement has to be done after the cell switch).

[Proposed work]

We propose to work on improving the L1 measurements and reporting procedure by introducing L1 event triggered measurement reporting and dynamic measurement activation for overhead reduction. In addition, beam refinement before LTM (including CHO) should be made possible by introducing CSI-RS measurements, TRS tracking, and CSI acquisition for LTM.

For improving LTM measurement procedures we propose the following objective:

- Proposal 3: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify enhancements to L1 measurement and reporting procedures [RAN1, RAN2]

- Support for L1 event triggered reporting. [RAN1, RAN2]

- Support for dynamic measurement activation/deactivation for overhead reduction [RAN1, RAN2]

- Support for CSI-RS, TRS tracking, and CSI acquisition for beam refinement before LTM (including CHO) [RAN1, RAN2]

  - Other enhancements

Based on the RAN workshop contributions, it seems that there is not enough company support for other enhancements, and they are not clearly motivated. However, depending on Rel-18 progress, some important leftover scenarios may exist. For example, in RAN2#123 the following was agreed regarding LTM for SCG/NR-DC.

- 1b) The case of PCell change (MCG) by LTM, without SCG, is supported (If there is an SCG configuration it is released at LTM execution). - 2b) The case of SCG LTM, without MN involvement is supported - as a working assumption (can be revisited e.g. at the last meeting), it is assumed that other MCG/SCG cases are not supported.

- Proposal 4: A Rel-19 work item on NR mobility enhancements needs to be revisited upon Rel-18 completion to ensure that any crucial leftover scenarios are addressed.

# Conclusion

In this contribution, R19 scope for mobility enhancements is discussed. The following proposals are provided:

- Proposal 1: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify mechanisms and procedures for L1/L2-triggered mobility for inter-gNB [RAN2, RAN1, RAN3]

- Signalling support for configuration of inter-gNB candidate cells, including Xn/NG interface signalling [RAN2, RAN3]

- Layer 2 enhancement including security update and PDCP anchor relocation. [RAN2]

- Support for inter-gNB TA management and early DL sync. [RAN2, RAN1, RAN3]

- Enhanced L1 measurement and reporting framework [RAN1, RAN2, RAN3]

- Proposal 2: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify mechanisms and procedures for RACH-less CHO [RAN2, RAN1, RAN3]

- Support for conditional (UE evaluated) trigger for RACH-less LTM execution. [RAN2, RAN1, RAN3]

Note: Early UL/DL sync and LTM execution introduced in Rel-18 should be the baseline.

- Proposal 3: A Rel-19 work item on NR mobility enhancements includes the following objective:

Specify enhancements to L1 measurement and reporting procedures [RAN1, RAN2]

- Support for L1 event triggered reporting. [RAN1, RAN2]

- Support for dynamic measurement activation/deactivation for overhead reduction [RAN1, RAN2]

- Support for CSI-RS, TRS tracking, and CSI acquisition for beam refinement before LTM (including CHO) [RAN1, RAN2]

- Proposal 4: A Rel-19 work item on NR mobility enhancements needs to be revisited upon Rel-18 completion to ensure that any crucial leftover scenarios are addressed.

# References

RP-223520 “Revised WID on Further NR mobility enhancements” MediaTek Inc., Apple.

RWS-230480 “Views on Mobility enhancements for Rel-19” InterDigital, Inc.