3GPP TSG RAN Meeting #101	RP-231994

Bangalore, India, September 11-15, 2023

Source:	InterDigital, Inc.

Title:	Views on Sidelink enhancements for Rel-19

Agenda item:	8.A.2.12.5

Document for:	Discussion

# Introduction

Rel-16 NR Sidelink (SL) design is mainly targeted at advanced V2V applications over the NR PC5 interface. The Rel-16 SL framework is based on NR waveform, new physical layer structure of PSCCH/PSSCH/PSFCH, SL Mode 1 and Mode 2 resource selection, support of HARQ for unicast transmissions, CSI reporting and congestion control.

In Rel-17 Sidelink enhancements, new mechanisms are designed to improve reliability, reduce latency, and enhance power saving. The main purpose is to support additional sidelink use cases for V2P and D2D communication. The Rel-17 SL enhancements include Inter-UE-Coordination (IUC) Scheme 1 and Scheme 2 for Mode 2 enhancements, partial sensing, and SL DRX for SL power saving.

Subsequently, Rel-18 SL enhancements are striving to meet higher demands for SL data throughput, reliability and latency performance required in commercial uses cases. The primary approach is to access more spectrum for SL operation and leverage FR2 beamforming to deliver the required performance. The Rel-18 scope (RP-221798) initially includes the following objectives:

- NR SL CA mechanism to support NR sidelink CA operation based on LTE sidelink CA operation.

- SL U channel access and interlaced physical channel design framework.

- SL FR2 beam management by reusing existing SL CSI framework and Uu beam management.

- Co-channel Coexistence to enable LTE and NR SL to coexist in the same frequency channel.

Due to TU limit and workload management, it was decided in RANP #100 to adjust the scope of R18 SL CA to support only LTE SL CA feature and as a result, NR SL-specific enhancements are explicitly excluded in the updated Rel-18 SL WI ([5]). Also, Rel-18 SL FR2 work includes only a study phase with focus on beam management for unicast communication and the corresponding normative work and potential further enhancements will be considered in Rel-19.

In this contribution, we present the justification to support a WI of further SL enhancement as a part of Rel-19 and discuss detailed objectives of proposed Rel-19 SL WI.

# Discussion

## Overview of SL enhancement discussion at Rel-19 workshop

A large number of companies in R19 workshop proposed a new work item for further SL enhancements in Rel-19. Two main objectives of SL technology enhancements preferred by all companies are the CA enhancements for NR SL-specific features and normative work of R18 SL FR2 study phase outcome. In addition, further enhancement on SL power saving, operation with reduced UE capability, further enhancements for SL FR1 in unlicensed spectrum and SL FR2 support in unlicensed spectrum are proposed by certain companies.

- CA enhancements for NR SL-specific features

All companies proposing a Rel-19 SL WI support CA enabled with Rel-17 enhancements. The widely discussed technical features include Mode 1 CA operation, support of carrier of different SCS, primary and secondary carrier differentiation, cross carrier HARQ operation, handling of limited TX/RX capability, enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation), cross-carrier IUC, synchronization procedure, etc.

In addition, several companies proposed an extended multi-carrier operation using combinations of different types of carriers. The combinations include NR FR1 licensed + unlicensed, NR FR1 unlicensed + unlicensed, NR FR1 licensed + FR2 licensed, FR2 + FR2 licensed.

- Normative work of R18 SL FR2 study phase outcome and further enhancement

The normative work of R18 SL FR2 study phase outcome also garners significant support from many companies. According R18 SL WI, FR2 study phase is focused on beam management for unicast transmissions. Thus, further enhancements have been considered for resource selection for beamformed SL transmissions and beam management for groupcast and broadcast transmissions.

The objectives with less support from companies include enhancements on SL power saving, SL support for reduced UE capability (RedCap UE), further enhancements for SL FR1 in unlicensed spectrum and SL FR2 support in unlicensed spectrum SL have much less support from the companies.

The following potential enhancements were also proposed by smaller sets of companies.

- SL power saving including SL LP WUS, enhanced SL DRX and flexible SL BWP operation.

- SL support for RedCap UE including support for reduce bandwidth and transmission power.

- Further enhancements for SL FR1 in unlicensed spectrum mainly focused on enabling Mode 1 operation with NR U and FBE channel access, which are down-prioritized features in R18 SL U work.

- SL FR2 support in unlicensed spectrum SL with directional LBT and enhanced PHY structure and procedure.

Observation 1: A great number of companies proposed a Rel-19 SLE WI including at least two main objectives, which are CA enhancements to support NR SL specific features and normative work of R18 SL FR2 study phase outcome.

## Justification for inclusion of SL enhancement WI in Rel-19

Advanced use cases and development of new devices has progressed increasingly in vertical applications, e.g., indoor interactive services ([8]), XR and gaming ([7]), personal IoT communication, robot operation in IIoT factory setting, etc. These applications have imposed requirements on SL operation for higher data throughput, more improved reliability, and further reduced latency. For example, audio-visual interactions for AR/VR applications including online XR gaming and interactive data sharing between variable devices can require up a few Gbps user-experienced data rate with less-than-10ms latency and 99.99% reliability (TS 22.261). In many vertical use cases, new types of devices, e.g., wearables are deployed, and the battery consumption becomes a critical consideration for SL operation.

Also, for high degree of automation V2X operation, advanced sensor and video information can be shared between V2X UEs in an extended sensors sharing group. In this use case, up to 1 Gbps data rates can be required at an inter-UE distance of up to 200 meters with a reliability of 99.99%-99.999% reliability (TS 22.186).

Observation 2: Emerging vertical and advanced V2X use cases continue imposing higher requirement for SL operation.

As discussed above in the use cases, it is very clear there is a growing strong commercial market that demands a high-performance SL operation. SL NR CA, SL in unlicensed spectrum, and SL FR2 operation are critical objectives precisely intended to address the acute need for supporting these commercial use cases. The additional spectrum enabled by these SL technologies are crucial to meet the market and customer demand on SL performance.

Observation 3: Normative work completion with further enhancement for SL CA, SL U and SL FR2 is essential for SL deployment targeting the new commercial use cases.

It is thus important for 3GPP RAN that SL technologies are ready for the future development. Therefore, we think it is imperative to include a SL enhancement WI as a part of Rel-19 to continue evolving SL technologies. We think the basic objectives of a Rel-19 SL enhancement WI should focus on SL CA and FR2 operation, and if TU and workload allows, additional SL enhancements, e.g., SL power saving, can also be considered.

Proposal 1: Support further SL enhancement WI as a part of Rel-19.

## Proposed Rel-19 SL enhancement WI objectives

### NR SL-specific enhancements for SL CA

In the updated Rel-18 SL WID, the following restriction and exclusions are specified for the SL CA scope. The purpose of this down-scoping is to complete the work within the assigned Rel-18 TU for SL discussions.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) - The work is limited to intra-band CA for the ITS band in FR1 (Band n47). - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Only Mode 2 operation - Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues - Time resources for PSFCH are aligned among the carriers for CA - No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation) - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI) - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH - No consideration for limited transmission and reception capability - No primary/secondary carrier differentiation - Reuse the LTE sidelink CA design for the following aspects: - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication - The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18. - Note: The SL CA work in Rel-18 mainly targets some V2X use cases

In our view, without enhancements to leverage NR SL-specific features including e.g., HARQ and CSI feedback, IUC etc, this down-scoped SL CA design will likely not provide the intended performance and benefit. We think it is critical to continue the SL CA design with NR SL-specific enhancements to leverage the inherent performance and reliability of NR SL design and enable flexibility and configurability in SL CA operation. Thus, we propose to prioritize in Rel-19 WI the further design and specification of SL CA operation based on Rel-18 SL CA and with NR SL-specific enhancements.

Proposal 2: Rel-19 SL WI specifies at least the following enhancements for Rel-18 NR SL CA

  - Mode 1 CA operation, e.g., multi-carrier HARQ reporting to gNB

  - Flexible SL carrier configuration e.g., different SCS and primary/secondary carrier differentiation

  - Resource selection and indication, e.g., cross-carrier scheduling in SCI

  - SL carrier (re)selection for PSFCH transmission

  - Synchronization procedure

  - SL RSRP and CSI measurement and reporting procedure.

### SL FR2 normative work and further enhancements

Rel-18 SL design is intended to utilize large bandwidth in FR2 and achieve high throughput performance for indoor commercial use cases. The study phase in Rel-18 WI is specified as shown below.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18) - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

We think it is critical in Rel-19 to prioritize normative work for the outcome of the study phase completed in Rel-18. In addition, to fully achieve the performance potential in FR2, it would be beneficial to identify further enhancements in addition to beam management procedures. For example, beam-based resource selection and sensing is a critical feature for the SL operation. Also, PHY layer CSI feedback enhancement e.g., based on a new PHY feedback channel can be considered to improve beam management performance. Therefore, we propose to study and specify mechanisms to complete the SL FR2 design framework.

Proposal 3: Rel-19 SL WI supports FR2 objective including:

  - Completing normative work of the Rel-18 study phase outcome for beam management including initial beam pairing, beam maintenance and beam failure recovery.

  - Studying, and specifying if needed, mechanisms for beam-based resource selection and sensing and PHY CSI feedback enhancements.

### Enhancement on SL power saving

SL power saving mechanisms including SL DRX and power-saving sensing (partial sensing) developed in Rel-17 targeting at PUE (Pedestrian UE) and VRU (Vulnerable Road User). For commercial use cases, a group of new device types with different form factors, e.g., XR devices, wearables, sensors, etc., can benefit from further reduction in power consumption and longer battery operation. In addition, we think further enhancements on SL power saving should ensure the SL performance is not compromised. As demonstrated in Uu design, a low power wake up signal and related procedures can be beneficial.

Proposal 4: Rel-19 SL WI support an objective of enhancements on SL power saving including:

  - SL wake-up signaling and procedures.

### Further enhancements for SL U

Rel-18 SL U operation is intended to utilize the wide unlicensed spectrum. The channel access and physical channel design framework is expected to conclude in Rel-18. However, we think there are a few additional SL U features for consideration in Rel-19 to improve the performance of SL U operation and leverage the unlicensed spectrum more efficiently. One of the features is Mode 1 SL U operation with NR U. In Rel-18 Mode 1, the NR Uu interface is limited to licensed spectrum for workload management, but the scenario with both NR Uu and SL operation in unlicensed spectrum can be addressed in Rel-19 SL WI. Another feature is the FBE-based channel access. Due to limited TU, Rel-18 SL U may complete the design for LBE-based channel access only. However, one of important SL U deployment (e.g., in an IIoT factory setting) is a controlled environment where no other non-3GPP device operates in the unlicensed spectrum. In such a SL deployment scenario, FBE-based channel access can provide better performance than LBE-based channel access. A further additional feature we think it is important to be included in Rel-19 is SL DRX for SL Unlicensed operation to achieve power saving benefits.

Proposal 5: Rel-19 SL WI supports an objective of further enhancements for SL U including:

  - Mode 1 SL operation with Uu based on NR U signaling

  - FBE channel access

  - SL U DRX

### Prioritization of proposed SL enhancement Rel-19 WI objectives

Depending on the agreed work items and study items in Release 19, it may be challenging to support all the discussed features for Rel-19 sidelink enhancements. Based on the TU given for Rel-19 SL enhancements, prioritization can happen to ensure that workload is manageable, and the objectives are realistic. In case of prioritization, we prefer to prioritize SL CA and SL FR2 enhancements. The SL power saving and SL U enhancements can be further considered when the TU and workload are reasonable to include them.

Proposal 6: Prioritize SL CA and FR2 enhancement objectives in SL enhancement Rel-19 WI if agreed.

# Conclusion

In this contribution, the following observations have been discussed:

Observation 1: A great number of companies proposed a Rel-19 SLE WI including at least two main objectives, which are CA enhancements to support NR SL specific features and normative work of R18 SL FR2 study phase outcome.

Observation 2: Emerging vertical and advanced V2X use cases continue imposing higher requirement for SL operation.

Observation 3: Normative work completion with further enhancement for SL CA, SL U and SL FR2 is essential for SL deployment targeting the new commercial use cases.

Based on the observations, the following proposals are made for the potential scope of Rel-19 SL enhancements:

Proposal 1: Support further SL enhancement WI as a part of Rel-19.

Proposal 2: Rel-19 SL WI specifies at least the following enhancements for Rel-18 NR SL CA

  - Mode 1 CA operation, e.g., multi-carrier HARQ reporting to gNB

  - Flexible SL carrier configuration e.g., different SCS and primary/secondary carrier differentiation

  - Resource selection and indication, e.g., cross-carrier scheduling in SCI

  - SL carrier (re)selection for PSFCH transmission

  - Synchronization procedure

  - SL RSRP and CSI measurement and reporting procedure.

Proposal 3: Rel-19 SL WI supports FR2 objective including:

  - Completing normative work of the Rel-18 study phase outcome for beam management including initial beam pairing, beam maintenance and beam failure recovery.

  - Studying, and specifying if needed, mechanisms for beam-based resource selection and sensing and PHY CSI feedback enhancements.

Proposal 4: Rel-19 SL WI support an objective of enhancements on SL power saving including:

  - SL wake-up signaling and procedures.

Proposal 5: Rel-19 SL WI support an objective of further enhancements for SL U including:

  - Mode 1 SL operation with Uu based on NR U signaling.

  - FBE channel access

  - SL U DRX

Proposal 6: Prioritize SL CA and FR2 enhancement objectives in SL enhancement Rel-19 WI if agreed.

# Reference

- RP-190984, “Revised WID on 5G V2X with NR sidelink”, LG Electronics, RAN #84, Newport Beach, USA, June 2019.

- RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, RAN#90-e, December 2020.

- RP-213678, “New WID on NR sidelink evolution”, OPPO, RAN#94e, December 2021

- RP-222806, “WID revision: NR sidelink evolution”, OPPO, RAN#98e, December 2022

- RP-230077, “WID revision: NR sidelink evolution”, OPPO, RAN#99, Rotterdam, Netherlands, March 2023

- TS 22.186, “Service requirements for enhanced V2X scenarios”, v17.0.0, 2022

- TS 22.161, “Service requirements for the 5G system”, v19.3.0, 2023

- TR 22.842, “Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS)”, v17.2.0, 2019.