3GPP TSG RAN meeting #101			   							RP-232001

Bangalore, India, September 11-15, 2023

## Status Report to TSG

Agenda item:			9.2.7

| WI / SI Name | Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No | Testing part: No |
| Acronym | FS_NR_FR2_OTA_enh |  |  |  |
| Unique ID | 950068 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-230814 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 | Core part: - | Performance part: - | Testing part: - |
| Overall Completion level | Study Item: 80% | Core part: - | Performance Part: - | Testing part: - |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Bin Han |
|  | Company | Qualcomm Incorporated |
|  | Email | binhan@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### Void

## 2.2	RAN2

#### Void

## 2.3	RAN3

#### Void

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#108 (WF in R4-2313888):

Test Method for UE RF

Weighting of the measurement grid

- Agreements:

  - Both sin θ and Clenshaw-Curtis weighting can be used in measurement grid analysis. Clenshaw-Curtis weighting is recommended.

MU analysis of the measurement grid

- Agreements:

  - RAN4 to focus on analyzing the starting point of step size per the candidates of measurement step size as following.

    - The candidates of measurement step size include: 30deg, 15deg, 10deg.

    - The reference step size for the simulation is 1deg as the basis to calculate the gap with candidates of measurement step size.

  - The uncertainty mechanism is limited to the coarseness of the grid and doesn’t depend on the UE random orientations as the legacy approach.

  - Encourage companies to provide suggestions for the MU analysis framework and to agree on a framework at next meeting (RAN4#108bis).

Absolute probe locations

- Agreements:

  - The probe location should be on the measurement grid, i.e., multiples constant-step grid step size

Test procedure

- Agreements:

  - Consider Option 2 as the starting point. Further updates are not precluded to capture the progress in RF requirements.

  - The test procedure implemented in the actual test system may be improved (e.g. to reduce test time) assuming the same test points are executed.

Figure 2.4.1-1: Test procedure in Option 2

Test Method for UE RRM

Testability analysis for the RRM testing scenarios

- Agreements:

  - RAN4 to further discuss the testability of option 1: Time and Frequency multiplexed downlink transmission should be supported by 2AoA measurement setup for multi-Rx RRM testing.

Test directions for 2AoA measurement setup

- Agreement:

  - [2 AoA Spherical coverage requirements] defined in the UE RF session should be taken as the baseline of test directions selection in multi-Rx RRM testing.

Dual TCI switching

- Agreements:

  - Capture all three candidate options into TR38.871 listing pros and cons, and further discuss based on the conclusion from RRM session for introduced RRM requirements.

    - Option 1: Dual TCI switches simultaneously, probe number for multiple AoA test system is at least 4

    - Option 2: Dual TCI switches sequentially, probe number for multiple AoA test system is at least

    - Option 3: Dual TCI switches simultaneously, but the beam directions are not changed, probe number for multiple AoA test system is at least 2

SINR control for rough beam

- Agreements:

  - For rough beam, the lower bound of G1/G2 is the gain difference from legacy REFSENS and legacy EIS spherical coverage

Test Method for UE Demodulation

X value in Noc level configuration

- Agreements:

  - Consider maximum Tx power at TE amplifier output as 23dBm per TRP as the baseline to study the test parameters and test directions for UE demodulation testing.

Assumptions for minimum isolation simulation and corresponding MU

- Agreements:

  - The following assumptions are adopted for simulation of minimum isolation requirements.

    - For the reference SNR, the following assumptions could be considered:

      - Assume α = 0, β = 0, and ρ=γ = good enough isolation, e.g., 100dB

      - Channel model parameters

      - TDLA30-75 is assumed for 100 MHz/120 kHz

      - Time offset values: 0; Frequency offset: 0

      - MCS: MCS17 with 1+1

      - Receiver assumptions: Separate processing per Rx chain.

    - For comparison, to run the simulation with the isolation range of [8dB, 20dB] including both cross-polarizations and cross-talk and then compare the offset between reference SNR and required SNR with different isolation values.

#### Remaining Open issues

Test methodology for RF requirements

- Measurement grid and MU framework for multi-Rx UE RF testing

- Test procedure for non-parametric test approach

Test methodology for RRM requirements

- The testability for the RRM testing scenarios including Dual TCI switching which depends on the RRM session progress.

Test methodology for Demodulation requirements

- Study and decide Noc level configuration such as X which is the allowable degradation in sensitivity from legacy REFSENS requirements in dB, X is within [2-5dB]

- Verification on the minimum isolation of [12dB] for the demodulation test directions selection

Uncertainty assessments

- Develop the related preliminary uncertainty assessments for the RF, RRM and Demodulation test methodologies

## 2.5	RAN5

#### Void

## 2.6	RAN6

#### Void

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

- R4-2313888,WF for FR2 test SI, Qualcomm Incorporated

- R4-2314265, Topic summary for [108][329] FS_NR_FR2_OTA_enh, Qualcomm

- R4-2313884, Ad-hoc minutes for FS_NR_FR2_OTA_enh, Qualcomm

- R4-2313887, TP for assistant coordination system, OPPO, ROHDE & SCHWARZ, Qualcomm Incorporated

- R4-2312889, TP to TR38.871 on UE RF testing methodology, Qualcomm

- R4-2312507, On measurement grid and other testing issues for 2AoA spherical coverage, Samsung

- R4-2312579, Discussion on the impact of measurement grid in multi-Rx RF test, vivo

- R4-2312886, Views on RF test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2312915, Discussion of test procedure, OPPO

- R4-2313219, Discussion on Test methods for RF requirements, Huawei, HiSilicon

- R4-2313781, On Multi-RX UE RF topics, Keysight Technologies UK Ltd

- R4-2312888, Views on RRM test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2312887, Views on demodulation test method for FR2 multi-Rx UE, Qualcomm Incorporated

- R4-2313223, Discussion on Test methods for demod requirements, Huawei,HiSilicon

- R4-2313782, On Multi-RX UE demod topics, Keysight Technologies UK Ltd

- R4-2312916, For measurement grid analysis, OPPO