3GPP TSG RAN #101	                                         	      RP-232040

Bangalore, India, September 11-15, 2023

Agenda Item:	9.3.2.9

Source:	Samsung

Title:	On PDCCH monitoring resumption after UL NACK

Document for:	Discussion/Decision

Introduction

This contribution provides a brief overview of discussions on PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping and proposes conclusions.

Discussion on PDCCH monitoring resumption after UL NACK

PDCCH monitoring resumption after UL NACK was discussed in RAN#100 with the following conclusion [1]:

The offline session noted that there will be no further discussion in RAN1 on this topic, and that the status of this functionality will be independent of the completion of the XR work item. A proposed baseline text from RAN#100 for a CR for consideration for approval at RAN#101 is given below: When the UE is configured with [PdcchMonitoringResumptionAfterNack], after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, if the UE transmits a PUCCH or a PUSCH providing a NACK value due to incorrectly decoding a PDSCH scheduled[/activated] by a DCI format received from the serving cell, the UE terminates PDCCH skipping, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell. Further checking is needed for the text highlighted in yellow. It is understood that the above behaviour does not impact RAN2 DRX procedures.

Without considering the text highlighted in yellow, /activated, the TP introduces an artificial restriction for when a UE resumes PDCCH monitoring after an UL NACK since that is conditioned on a PDSCH reception being scheduled by a DCI format (i.e. “due to incorrectly decoding a PDSCH scheduled by a DCI format received from the serving cell”) and SPS PDSCHs are therefore excluded. It is noted that “scheduled” (for DG-PDSCH) is not same as “activated” (for SPS PDSCH). If the feature is to be supported, any potential benefits should not be limited, for no apparent reason, by how the UE was informed to receive a PDSCH.

Considering the above, we propose the following:

Proposal: For PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping, the proposed baseline text from RAN#100 with the text highlighted in yellow is approved.

Conclusion

This contribution considered the feature of PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping and proposes the following.

Proposal: For PDCCH monitoring resumption after UL NACK in conjunction with PDCCH skipping, the proposed baseline text from RAN#100 with the text highlighted in yellow is approved.

- References

[1] RP-231483	Endorsed summary of offline discussion on XR, Offline session moderator (Nokia)