3GPP TSG RAN Meeting #101															               RP-232060

Bangalore, India, September 11-15, 2023

Agenda item:	8A.2.3

Source:	CATT

Title:	Duplex evolution in Rel-19

Document for:	Discussion and Decision

- Introduction

Study on evolution of NR duplex operation was approved in Rel-18 with following objectives [1]. RAN1 study is completed and RAN4 study is expected to be completed in November.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide the current status of Rel-18 study and our views on duplex evolution in Rel-19.

- Rel-18 study status

  - Status of SBFD study

LLS and SLS have been conducted in RAN1 to evaluate the coverage and UPT performance of SBFD. Evaluation assumptions and results are captured in TR 38.858.

Based on LLS, it is observed that semi-static SBFD with PUSCH repetition type A/TBoMS provides UL coverage gain for FR1 UMa and FR2-1 Dense UMa.

Observation 1: Semi-static SBFD with PUSCH repetition type A/TBoMS provides UL coverage gain for FR1 UMa and FR2-1 Dense UMa.

In terms of UPT performance, with assumption of 1dB desense for self-interference suppression and “twice the total number of antenna elements and same total number of TxRUs”, the UPT gain/loss for low, medium and high loads for indoor scenario are summarized below.

Table 1: semi-static SBFD UPT gain for indoor (deployment case 1)

| Slot format | Scenario | Packet size | Mean/5% UPT | DL | UL |
| --- | --- | --- | --- | --- | --- |
| XXXXX vs. DDDSU | FR1 indoor | Small packet size | Mean UPT | {9.56%, 9.35%, 8.58%} | {101.83%, 97.42%, 93.85%} |
|  |  |  | 5% UPT | {10.50%, 12.71%, 8.79%} | {107.58%, 105.44%, 106.52%} |
|  |  | Large packet size | Mean UPT | {1.86%, 2.21%, 2.73%} | {10.78%, 13.38%, 13.75%} |
|  |  |  | 5% UPT | {1.73%, -1.19%, 0.54%} | {14.13%, 19.91%, 17.70%} |
|  | FR2-1 indoor | Small packet size | Mean UPT | {4.84%, 7.57%, 5.95%} | {55.30%, 54.71%, 72.66%} |
|  |  |  | 5% UPT | {5.18%, 4.45%, -10.25%} | {50.71%, 46.45%, 59.26%} |
|  |  | Large packet size | Mean UPT | {3.63%, 3.36%, 3.60%} | {16.6%, 22.22%, 20.61%} |
|  |  |  | 5% UPT | {6.03%, 6.67%, 3.35%} | {38.79%, 71.05%, 86.18%} |
| XXXXU vs. DDDSU | FR1 indoor | Small packet size | Mean UPT | {-0.52%, -1.19%, -6.26%} | {99.5%, 98.83%, 104.00%} |
|  |  |  | 5% UPT | {-0.35%, -1.66%, -17.59%} | {116.24%, 110.00%, 127.81%} |
|  |  | Large packet size | Mean UPT | {-20.38%, -26.30%, -33.95%} | {78.53%, 93.92%, 113.75%} |
|  |  |  | 5% UPT | {-22.88%, -29.57%, -53.83%} | {81.03%, 106.39%, 150.17%} |

For SBFD with XXXXX slot format, semi-static SBFD provides performance improvement for both DL and UL for all load levels and small/large packet size with two exceptions. For SBFD with XXXXU slot format, semi-static SBFD provides performance improvement for UL but may suffer from degradation for DL at least due to more UL resources and less DL resources compared to legacy TDD. However, it is noted that the UL UPT gain is higher than DL UPT loss.

For Urban Macro and Dense Urban Macro scenarios, the UPT performance is impacted by co-site inter-section CLI suppression capability.

Observation 2: Semi-static SBFD provides UPT gain especially for indoor scenario.

Dynamic SBFD which dynamically converts an SBFD symbol to a full-DL/full-UL symbol has been evaluated by several companies. However, due to limited results and/or conflict results, no conclusion was made in terms of dynamic SBFD performance.

Observation 3: Dynamic SBFD performance gain has not been justified.

SBFD operation scheme has been studied in RAN1 with the following conclusion. SBFD operation Option 4 was concluded as feasible for RRC_CONNECTED state from the RAN1 specification perspective and potential specification impact has been identified.

SBFD operation at gNB for UEs was studied under the following assumptions, - SBFD operation within a TDD carrier, - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies, and - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier. RAN1 concluded SBFD operation Option 4 is feasible for RRC_CONNECTED state from the RAN1 specification perspective, where SBFD operation Option 4 assumes - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side from RAN1 specification point of view. To support SBFD operation Option 4 for RRC_CONNECTED state, RAN1 identified the following potential specification impact for SBFD-aware UE: - Indication of time and frequency domain locations of SBFD subbands to UEs - UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols

Observation 4: SBFD operation is feasible and backward compatible from RAN1 specification perspective.

Random access in SBFD symbols is studied in RAN1 with the following outcome in the TR.

If random access is allowed in SBFD symbols for SBFD-aware UEs, it may potentially reduce the random access latency, reduce the PRACH collision probability and/or improve the coverage of PRACH and Msg3. These aspects were not fully evaluated in RAN1. PRACH and Msg3 transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI. The system performance impact is not evaluated in RAN1. Specification impact is expected to allow random access in SBFD symbols at least for PRACH and Msg3 transmissions in symbols configured as DL in TDD-UL-DL-ConfigCommon.

The following inter-UE CLI handling schemes specific for SBFD were also studied, which may or may not have specification impact.

-	Method#1: victim UE measures RSSI within DL subband

-	Method#2: victim UE measures RSRP of aggressor UE within UL subband

-	Method#3: victim UE measures RSSI within UL subband

The above inter-UE CLI handling methods are not evaluated in RAN1. Semi-static SBFD with some other inter-UE/inter-gNB CLI handling schemes which are common for SBFD and dynamic/flexible TDD was evaluated by several companies but the results are quite limited and no conclusion is made.

The study for the implementation feasibility of SBFD is still on-going in RAN4. For BS feasibility study, it is RAN4’s common understanding that legacy BS cannot support SBFD without implementation improvement. In order to solve the problem of self-interference, co-site inter-sector interference and inter-site inter-sector interference, some technologies may need to be used. The technologies mentioned in the study are digital self-interference cancellation, sub-band filter, beam nulling, improved antenna isolation, improved Rx linearity, etc.

For the BS self-interference study, RAN4 has reached the following agreement.

- For implementation feasibility study, RAN4 confirm FR1 MR and LA SBFD gNB with 1dB sensitivity degradation due to self-interference is achievable.

Until RAN4#108, there is no conclusion on the self-interference study for FR1 WA BS and FR2 BS. There is no conclusion on the co-site inter-sector interference either. How to conclude the open issues will be discussed further and addressed in the TR.

For UE RF feasibly study, half duplex operation at the UE side is assumed as stated in the SID. Legacy UE capabilities, such as ACLR, ACS, IBE, etc., are used to do the study. There was some discussion on the sub-band filter for SBFD aware UE, but no conclusion was reached.

RAN4 is conducting adjacent channel co-existence evaluation for SBFD with the assumption that current BS/UE ACLR/ACS performance is assumed. The scenario of SBFD channel adjacent with legacy TDD channel is simulated. RAN4 simulated 9 scenarios including UMa, Umi and indoor. According to R4-2313872, for most of the scenarios/cases, interference is acceptable for 100% grid shift. Higher than 5% throughput loss is observed for some scenarios. For some FR2 scenarios, such as Umi and Indoor, more simulation results are needed to draw the conclusions.

The impacts on SBFD capable BS RF requirements are studied in RAN4. The study has not been concluded completely. All of the existing requirements were reviewed for SBFD capable BS, some existing requirements were identified for further study, such as Tx intermodulation, co-location and co-existence requirements

For the new requirements discussion, one requirement is agreed, i.e. OTA sensitivity in SBFD time slot with self-interference only. The exact requirement will be decided in WI phase considering [0.5-1] dB. Some other requirements are also discussed without conclusion yet, for example, in-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity, etc. RAN4 will continue the discussion in future meetings. More detail study and conclusion will be conducted in WI phase.

The collection of regulatory information is conducted in RAN4. The information for TDD deployment was provided for different regions. RAN4 is discussing the wording on how to summarize the regulation status in TR.

  - Status of dynamic/flexible TDD study

For dynamic/flexible TDD evaluation, the following scenarios are considered. SLS is used for evaluation.

- FR1 - 1-layer scenario - Indoor office - (Optional) Urban Macro - 2-layer Scenario B - Layer 1: Urban Macro - Layer 2: - Baseline: Indoor office - Optional: Indoor factory - FR2-1 - 1-layer scenario - Indoor office - (Optional) Dense Urban Macro layer

For gNB-to-gNB CLI handling, the following schemes are studied.

- gNB-to-gNB CLI measurement and reporting

- Coordinated scheduling

- Spatial domain enhancements

- Advanced receiver

- UE and gNB transmission and reception timing

- Power control based solution

- Potential enhancements to Rel-16 RIM

- Sensing based mechanism

- Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified

- Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded.

- Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

The discussion on both potential enhancement to Rel-16 RIM and sensing based mechanism for gNB-to-gNB co-channel CLI handling is agreed to be deprioritized.

For UE-to-UE CLI handling, the following solutions are studied.

- Potential enhancements to UE-to-UE CLI measurement/reporting

- Coordinated scheduling

- Spatial domain enhancements,

- Advanced Receiver

- UE and gNB transmission and reception timing

- Power control based solution

- Sensing based mechanism

- Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified

- Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded.

- Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

It is agreed that discussion on sensing-based mechanism (i.e. LBT) and UE side advanced receiver for UE-to-UE co-channel CLI handling is deprioritized.

For these schemes, corresponding performance evaluations (if conducted), observations and specification impacts are captured in the TR.

- Rel-19 duplex evolution

The implementation feasibility for 1dB sensitivity degradation due to self-interference has been confirmed for FR1 MR and LA SBFD gNB and is pending for FR1 WA BS and FR2 BS based on the study in RAN4. UL coverage gain for semi-static SBFD with PUSCH repetition type A/TBoMS has been observed for FR1 UMa and FR2-1 Dense UMa. UPT gain has been observed for semi-static SBFD. We therefore support normative work of SBFD in Rel-19.

The baseline SBFD operation is semi-static SBFD where UL transmissions within UL subband and DL receptions within DL subband(s) are allowed but UL transmissions outside UL subband and DL receptions outside DL subband(s) are not allowed. For dynamic SBFD, the main motivation is for better resource adaptation between DL and UL based on UL/DL traffic ratio, which allows DL receptions outside semi-statically configured DL subband(s) and may additional allow UL transmissions outside the semi-statically configured UL subbands in a symbol configured as flexible in TDD-UL-DL-ConfigCommon. However, the performance gain of dynamic SBFD compared with semi-static SBFD/dynamic TDD has not been justified. Therefore, it is proposed that dynamic SBFD is not supported in Rel-19. Furthermore, the following assumptions for SBFD study should still be applicable.

  - SBFD operation within a TDD carrier,

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies, and

  - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier.

In addition, potential limitation on the maximum number of transition points between SBFD and non-SBFD symbols was considered in order to avoid frequent transition between SBFD and non-SBFD symbols during the study.  Maximum of two transition points including one transition point from non-SBFD symbols to SBFD symbols and one transition point from SBFD symbols to non-SBFD symbols within a TDD UL/DL pattern period was considered as a starting point where the transition point can be aligned with slot boundary or within a slot. We think it is a reasonable assumption which simplifies the design and implementation and hence propose to agree on the assumption.

RAN1 also studied whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot but no conclusion was reached. It is not desirable to allow UE to transmit or receive a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, which may significantly complicate the implementation and the design. Therefore, we propose that in Rel-19 semi-static SBFD normative work, it is assumed that the transmission/reception occasion of a physical channel/signal cannot be mapped to SBFD and non-SBFD symbols within a slot.

Proposal 1: Support semi-static SBFD only in Rel-19 under the following assumptions:

  - SBFD operation within a TDD carrier,

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies,

  - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier,

  - Maximum of two transition points including one transition point from non-SBFD symbols to SBFD symbols and one transition point from SBFD symbols to non-SBFD symbols within a TDD UL/DL pattern period where the transition point can be aligned with slot boundary or within a slot, and

  - UE does not transmit or receive a physical channel/signal if the transmission or reception occasion of a physical channel/signal is mapped to SBFD and non-SBFD symbols within a slot.

From SBFD operation scheme design perspective, SBFD operation Option 4 is concluded as feasible for RRC_CONNECTED state from the RAN1 specification perspective. It is proposed to support SBFD operation Option 4, where SBFD operation Option 4 assumes

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

The potential specification impact identified in RAN1 includes:

  - Indication of time and frequency domain locations of SBFD subbands to UEs

  - UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols

For indication of time and frequency domain locations of SBFD subbands to UEs, only semi-static configuration is considered based on the proposal to consider semi-static SBFD only. The conclusions/agreements made during SI which are captured in TR 38.858 section 6.1.1.1 should be the starting point for normative work.

Proposal 2: Specify semi-static configuration of SBFD subbands time and frequency locations in symbols configured as DL or flexible in TDD-UL-DL-ConfigCommon following the conclusions/agreements captured in TR 38.858 section 6.1.1.1.

For enhancements for transmissions and receptions for SBFD aware UEs, various enhancements to PDCCH, PDSCH, PUSCH, PUCCH, CSI-RS and SRS were studied during SI phase e.g. for transmissions and receptions across SBFD symbols and non-SBFD symbols, non-contiguous resource allocation across two DL subbands, unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands etc. Among the physical channels and signals, we prefer to de-prioritize PDCCH considering the significant impact on UE implementation, specification efforts etc.

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following options are studied for SBFD-aware UEs:

-	Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

-	Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Between the two options, we prefer Option 2 which can reduce the latency and improve the coverage which are the motivations to support SBFD.

Besides, collision handling also needs to be considered for SBFD aware UEs since UE is only capable of half-duplex operation.

Proposal 3: Specify the following behaviours and procedures for the SBFD aware UEs:

  - SBFD aware UE transmission, reception and measurement behaviour and procedure in SBFD symbols, including:

    - UL transmissions within UL subband only and DL receptions within DL subband(s) only

    - Non-contiguous frequency resources across two DL subband(s) for PDSCH and CSI-RS

    - Handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

  - SBFD aware UE transmission/reception behaviour and procedure across SBFD symbols and non-SBFD symbols in different slots

    - Note: PDCCH reception follows the existing behaviour and procedure

  - Collision handling behaviours

For SBFD specific CLI handling, the study focused on inter-UE inter-subband CLI handling. For both measurements in DL subband(s) and in UL subband, existing framework can be reused to a large extent. Specification efforts, if any, would be quite limited according to our understanding.

In addition, it needs to be discussed whether random access is allowed in SBFD symbols for SBFD-aware UEs. Given that more resources and transmission occasions can be provided for UL transmissions in SBFD symbols, potential benefits are expected in terms of latency, capacity and coverage. We think it beneficial to support random access in SBFD symbols from the first release in which SBFD is introduced so that the PRACH resources configured in SBFD symbols can be fully utilized. It can be left to gNB implementation to decide whether to configure/allocate resources in SBFD symbols for random access.

Proposal 4: Random access in SBFD symbols is supported in Rel-19.

For the enhancement of CLI handling for both gNB-to-gNB inter-cell co-channel interference and UE-to-UE inter-cell co-channel interference scenario, based on the study from SI, co-channel CLI measurement for CLI handling and coordinated scheduling for time/frequency resource should be prioritized, including L1/L2 based UE-to-UE CLI measurement and exchange of semi-static SBFD time and frequency configuration for coordinated scheduling etc.

Proposal 5: For inter-cell co-channel interference handling, based on the outcome of the SI, specify enhancement for co-channel CLI measurement/reporting and coordinated scheduling between gNBs.

4	Conclusion

In this contribution, we share our views on duplex evolution in RAN Rel-19. The observations and proposals are summarized in the following.

Observation 1: Semi-static SBFD with PUSCH repetition type A/TBoMS provides UL coverage gain for FR1 UMa and FR2-1 Dense UMa.

Observation 2: Semi-static SBFD provides UPT gain especially for indoor scenario.

Observation 3: Dynamic SBFD performance gain has not been justified.

Observation 4: SBFD operation is feasible and backward compatible from RAN1 specification perspective.

Proposal 1: Support semi-static SBFD only in Rel-19 under the following assumptions:

  - SBFD operation within a TDD carrier,

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies,

  - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier,

  - Maximum of two transition points including one transition point from non-SBFD symbols to SBFD symbols and one transition point from SBFD symbols to non-SBFD symbols within a TDD UL/DL pattern period where the transition point can be aligned with slot boundary or within a slot, and

  - UE does not transmit or receive a physical channel/signal if the transmission or reception occasion of a physical channel/signal is mapped to SBFD and non-SBFD symbols within a slot.

Proposal 2: Specify semi-static configuration of SBFD subbands time and frequency locations in symbols configured as DL or flexible in TDD-UL-DL-ConfigCommon following the conclusions/agreements captured in TR 38.858 section 6.1.1.1.

Proposal 3: Specify the following behaviours and procedures for the SBFD aware UEs:

  - SBFD aware UE transmission, reception and measurement behaviour and procedure in SBFD symbols, including:

    - UL transmissions within UL subband only and DL receptions within DL subband(s) only

    - Non-contiguous frequency resources across two DL subband(s) for PDSCH and CSI-RS

    - Handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

  - SBFD aware UE transmission/reception behaviour and procedure across SBFD symbols and non-SBFD symbols in different slots

    - Note: PDCCH reception follows the existing behaviour and procedure

  - Collision handling behaviours

Proposal 4: Random access in SBFD symbols is supported in Rel-19.

Proposal 5: For inter-cell co-channel interference handling, based on the outcome of the SI, specify enhancement for co-channel CLI measurement/reporting and coordinated scheduling between gNBs.

References

- RP-223041, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN#98-e

- R4-2313872, WF for SBFD co-existence study