3GPP TSG RAN Meeting #101									  RP-232104

Bangalore, India, September 11-15, 2023

## Status Report to TSG

Agenda item:			9.3.4.6

| WI / SI Name | Even Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | 950078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221696 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:90% RAN4: 90% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#123 meeting

- R2 assumes that a new reportType is used.

- R2-2309254 Introduction of FR2 SCell enhancements

Use this CR as baseline for further update

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#108 meeting

The following tdocs have been approved in RAN4#108meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2314446 | WF on NR FeRRM (part1) | Apple |
| R4-2314338 | LS on FR2 unknown SCell activation enhancements | Apple |
| R4-2314337 | WF on NR FeRRM (part2) | OPPO |

RRM core requirements for FR2 SCell activation delay reduction

Issue 1-1: 	Enhancement for multiple FR2 unknown SCells’ activation

Agreements:

- Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band.

  - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized

Issue 1-2: 	TS38.133 sections to specify the unknown FR2 SCell activation enhancement

Agreement:

- Capture single/multiple SCell Activation enhancement based on L3 reporting in a new section, and use the existing sections to capture the enhancements in other scenarios.

Issue 1-3: 	CR splitting

Agreement:

| Section in TS38.133 | Company |
| --- | --- |
| New section 8.3.x SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation | QC |
| 8.3.4 Direct SCell Activation at SCell addition (only except the L3 reporting based enhancement) 8.3.5 Direct SCell Activation at Handover (only except the L3 reporting based enhancement) 8.3.9 Direct SCell Activation of Multiple Downlink SCells at SCell addition (only except the L3 reporting based enhancement) 8.3.10 Direct SCell Activation of Multiple Downlink SCells at Handover (only except the L3 reporting based enhancement) | Ericsson |
| New section 8.3.y SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with the L3 reporting during activation | ZTE |
| 8.3.12 SCell Activation Delay Requirement for Deactivated PUCCH SCell | Nokia |
| 8.3.13 SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells | HW |
| 9.2.4 Measurement Reporting Requirement | vivo |
| 8.3.2 SCell Activation Delay Requirement for Deactivated SCell (for enhancement excepts the L3 reporting) | MediaTek |

Issue 2-1-1: delay requirement or time margin for “L3 measurement reporting after SCell activation command”?

Agreements:

- Define delay requirement or time margin for L3 measurement reporting after SCell activation command:

  - UE shall report L3-RSRP report with SSB index within [Y]ms after receiving SCell activation command MAC CE if the UL grant is available.

  - UE is not required to report L3-RSRP report after exceeding [Y]ms

  - Where Y= THARQ + 3ms + [M]ms, M is FFS.

Issue 2-1-2: Delay requirement for the case when the valid L3 measurement result with SSB index is reported after SCell activation command, and UE reported L1 measurement result before NW configures TCI to this UE

Agreements:

- Update the definition of Tuncertaitny_MAC to include L3 measurement report:

  - Tuncertaitny_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

    - SCell activation command for known case;

    - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP and L3-RSRP report is earlier than TCI command

    - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results

Issue 2-1-3: TCI activation command transmitted is based on L3 measurement results or L1 measurement results?

Agreements:

  - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on:

    - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command.

    - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

Issue 2-2-1: whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown SCell activation

Agreements:

- Use SSB periodicity instead of SMTC periodicity when the SMTC is only configured in MO for enhanced unknown FR2 Scell activation requirement.

RRM core requirements for FR1-FR1 NR-DC

Agreement: RAN4 can leverage the conclusion from R17 maintenance of SCG activation/deactivation if any.

#### 2.4.2	Remaining Open issues

As summarized in approved way forward R4-2314446 and R4-2314337 (open options in the way forward).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #108 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2311166 | Discussion on remaining issues on L3 part enhancements for FR2 Unknown SCell activation | Qualcomm Incorporated |
| R4-2311167 | Discussion on remaining issues on L1 part enhancements for FR2 Unknown SCell activation | Qualcomm Incorporated |
| R4-2311168 | Discussion on remaining issues on other enhancements for FR2 Unknown SCell activation | Qualcomm Incorporated |
| R4-2311318 | On CR splitting for FR2 SCell activation enhancement | Apple |
| R4-2311319 | On L3 part enhancement for FR2 SCell activation | Apple |
| R4-2311320 | On RRM requirements for FR1-FR1 NR-DC | Apple |
| R4-2311572 | L3 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2311573 | L1 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2311574 | R18 enhancements for other SCell activation scenarios | Nokia, Nokia Shanghai Bell |
| R4-2311580 | draftCR on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2311853 | Further discuss the remaining issues for FR2 SCell activation enhancement | Xiaomi |
| R4-2311890 | Discussion on general aspects for FR2 SCell activation | CMCC |
| R4-2311891 | Discussion on L3 part enhancement for FR2 SCell activation | CMCC |
| R4-2311892 | Discussion on enhancement for FR1 SCell activation | CMCC |
| R4-2311913 | Draft CR on FR2 unknown SCell activation enhancement with single deactivated SCell R18 | Apple |
| R4-2312128 | Discussion on RRM core requirements for FR1-FR1 NR-DC | vivo |
| R4-2312412 | Discussion on general aspects for SCell activation delay reduction | Huawei, HiSilicon |
| R4-2312413 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2312540 | Discussion on L3 part enhancement for FR2 SCell activation | China Telecom |
| R4-2312655 | On L3 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2312656 | On L1 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2312657 | On remaining issues of RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2312658 | Big CR for RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2313072 | discussion on SCG activation for FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2313073 | draftCR on SCG activation for FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2313106 | Discussion on the L3 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2313296 | Discussion on remaining issues on L3 aspects of FR2 SCell activation delay reduction | vivo |
| R4-2313297 | Discussion on aspects related to multiple SCell activation for FR2 SCell activation delay reduction | vivo |
| R4-2313507 | Discussion for FR1-FR1 NR DC RRM requirements | Ericsson |
| R4-2313508 | [Draft] LS on clarification on UE behavior during RACH-less SCG activation | Ericsson |
| R4-2313570 | Discussion for FR1-FR1 NR DC RRM requirements | Ericsson |
| R4-2313689 | On FR2 SCell activation delay reduction: L3 aspects | Ericsson |
| R4-2313690 | On FR2 SCell activation delay reduction: L1 RSRP | Ericsson |
| R4-2313691 | Discussion on FR2 SCell activation enhancements to multiple SCells | Ericsson |
| R4-2313720 | Discussion on the general aspects of FR2 SCell activation enhancement | MediaTek inc. |
| R4-2313721 | Discussion on L3 part enhancement for FR2 SCell activation | MediaTek inc. |

RAN2#123 meeting

| Tdoc | Title | Source |
| --- | --- | --- |
| R2-2307039 | LS on FR2 SCell activation enhancements (R4-2310083; contact: Apple) | RAN4 |
| R2-2307278 | Discussion on FR2 unknown SCell activation enhancement | CATT |
| R2-2307770 | Scell activation and L3 reporting | Nokia, Nokia Shanghai Bell |
| R2-2307850 | FR2 SCell Enhancement | Apple |
| R2-2308072 | Discussion on FR2 unknown SCell activation | ZTE Corporation, Sanechips |
| R2-2308208 | RAN2 signalling design for FR2 unknown SCell activation enhancement | Huawei, HiSilicon |
| R2-2308443 | Introduction of FR2 SCell enhancements | Ericsson |
| R2-2308835 | Discussion on FR2 unknown SCell activation enhancement | vivo |
| R2-2309253 | Report of [At123][025] FR2 SCell Enhancement | Apple |
| R2-2309254 | Introduction of FR2 SCell enhancements | Apple |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template