3GPP TSG-RAN Meeting # 101	RP-232109

# Bangalore, India, Sep 11-14, 2023

Title:	LS on signaling support for type 2 UE

Response to:

Release:	Rel-18

Work Item:	NonCol_intraB_ENDC_NR_CA

Source:	RAN

To:	RAN2

Cc:	RAN4

Contact person:	Yang.Tang@Apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

Regarding R18 WI Support of intra-band non-collocated EN-DC NR-CA deployment, RAN4 agreed to introduce a Type 2 UE for intra-band non-collocated CA which is characterized by:

- Capable of handling 25dB power imbalance and 33us MRTD between CCs (type 2 capabiilty)

- Support maximum 2MIMO layer per CC in non-collocated scenario (type 2 capability)

- Support maximum 4MIMO layer per CC in collocated scenario (type 1 capability).

Note 1: Type 1 UE is the legacy UE capable of handling 6dB power imbalance, 3us MRTD between CCs and 4MIMO layer per CC in intra-band collocated CA scenario.

A separate new capability signalling is needed to support such UE reporting. This new UE capability is expected to be per BC and specific to TDD-TDD intra-band non-collocated CA. A detailed capability descripted is provided as below:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandNonColocatedCA-r18 Indicates the UE supports TDD-TDD intra-band non-collocated CA operation with NR carrier aggregation MRTD according to Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated CA in 7.10A in 38.101-1 in additional to the RF requirements for intra-band collocated CA. If the capability is not reported, the UE only supports the RF requirements for TDD-TDD intra-band collocated CA, e.g. NR carrier aggregation MRTD according to Table 7.6.4-1 in 38.133 [5] and the RF requirements for intra-band CA RF requirements except for 7.10A in 38.101-1. | BC | No | TDD only | FR1 only |

From Type 2 UE hardware capability perspective, it can be configured as Type 1 UE in collocated scenario or Type 2 UE in non-collocated. It is essential to enable such configurable function for Type 2 UE by providing UE the collocation information by network signalling. When a Scell CC is configured and network signalling indicates non-collocated deployment, Type 2 UE shall configure its Rx chain to type 2 capability.

# 2	Actions

To RAN2:

ACTION: 	RAN2 is respectfully requested to take the above information into consideration and develop a solution supporting Type 2 UE reconfiguration between type 1 capability and type 2 capability before November meeting.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #108bis                  09th – 13th Oct 2023			Xiamen, China

TSG-RAN WG4 Meeting #109                      13th – 17th Nov 2023	          	      	Chicago, US