3GPP TSG-RAN Meeting #101             	   				    RP-232111

Bangalore, India, September 11-15, 2023

Agenda Item:		8A.2.11

Source:			Beijing University of Posts and Telecommunications, China Telecom

Title:	Feasibility study on enabling Integrated Communication and Sensing (ISAC) for BS

Document for:	 	Discussion

- Introduction

In last RAN release 19 workshop, ISAC has been captured in the endorsed RAN chair’s summary [1] as one of the topics that companies show very strong interest in. The detailed summary for ISAC is listed as follows.

- Strong interest in exploring study in new areas (RAN plenary invovlement?)

  - Very strong interest in channel modeling for integrated sensing and communication (ISAC), and also interest in other possible areas (e.g., use cases, scenarios, key performance metrics (KPIs), potential spec impact, etc.)

It could be seen that besides channel modeling, some other possible areas are also worth to be studied. Of which, to study how to enable BS supporting ISAC (i.e. gNB-based ISAC) is one of options as proposed in [2] and [3]. The objectives may include use cases, scenarios, key performance metrics (KPIs), potential spec impact, etc. as mentioned in the summary.

According to discussion during and after the last RAN workshop, we see some concerns on enabling BS supporting ISAC in release 19. The comments mainly focus on

- Which popular use case/scenario that BS supporting ISAC could apply to.

- How much performance benefit that BS supporting ISAC could bring.

In this contribution, we provide our views and hardware test to try to address above comments. With hardware testbed results, we verify the performance benefit and feasibility to enable ISAC for BS.

- Discussion

  - Use case/scenario

In 2023, the Shenzhen Government has claimed to build a low-altitude economic center, and CAAC (Civil Aviation Administration of China) has issued the “Approval Letter for the Trial Operation of UAVs” and the “General Aviation Enterprise Business License” for Meituan (美团，a Chinese tech-driven retail company that provides food delivery, transportation, travel, shopping and entertainment services), to allow UAVs for logistics distribution in the airspace below 120 meters.

By the May 2023, Meituan’s drone delivery has landed in 6 business districts in Shenzhen and Shanghai of China. The routes cover communities, business districts, scenic spots, and office buildings. The cumulative delivery orders have exceeded 167,000 orders, and the types of goods that can be delivered More than 20,000 kinds. Below is the photo that one of Meituan’s drone is delivering the logistic in Shenzhen.

Observation 1: The low-altitude economic development is rising with representative use case of UAV delivering the logistic.

ISAC could bring much benefit for many use cases according to description in 3GPP study item [4].  As mentioned in the Introduction, we are more interested in the ISAC implemented by gNB-based sensing mode. Therefore, we retrieve and propose top 3 use cases that gNB-based ISAC could better apply to.  They are listed as

- Unmanned Aerial Vehicle (UAV) trajectory tracing: UAV could be used in low-altitude economic business such as logistic delivering as mentioned above. In these use cases, compared with visual sensors which are sensitive to ambient light conditions, wireless sensing is more stable and reliable. Although there exist dedicated UAV surveillance equipment and radar, their large-scale deployment has great challenges due to lack of available sites and high installation and maintenance cost. In comparison, gNB-based ISAC can provide a cost-effective way to trace these UAVs, e.g., 5G network infrastructures with ubiquitous coverage can better trace the flight trajectory of each UAV.

- Environment monitoring: ISAC network can use the characteristics of radio signals to obtain environmental information such as temperature, humidity, and air particle concentration. By analyzing attenuation properties of the millimeter wave, it is possible to monitor rainfall or other changes in the atmospheric environment, such as water vapor, air pollutants and insects.

- Intruder detection/Road dynamic information (RDI) perception: RDI perception includes traffic road jam detection and traffic violation detection, which mainly rely on ISAC to perceive the behavior of vehicles on the road. By comparing to the traditional traffic congestion detection schemes, cellular enabled ISAC could provide larger deployment coverage without additional cost, possibly solves the most issues on the traffic congestion and traffic violation detection with the high accuracy of detection.

Observation 2: gNB-based ISAC has the natural advantage of coverage and networking for UAV tracing and even management.

  - Hardware testbed

In order to verify the performance benefit for BS supporting ISAC by leveraging the natural cooperation and computation ability, we built an ISAC hardware testbed. In general, the system is based on the 5G NR frame structure and towards autonomous driving scenario to measure the position related information for moving vehicle. The sensing and communication functions share the same baseband and radio frequency hardware, and allocate the corresponding resources in a time-division manner to realize the integration of sensing and communication.

    - Hardware Testbed Introduction

The ISAC hardware system consists of two 5G mmWave communication platforms, two 64-element phased array antennas (PAA), one horn antenna and one sensing cooperative center as shown in Fig. 1. Each 5G mmWave platform includes two PAAs which are connected to radio heads and configured as integrated signal (sensing and communication) transmitter and sensing signal receiver respectively. While the communication signal receiver is taken by the horn antenna which is connected to an independent radio frequency chain. With these two receivers and corresponding processing board cards, the sensing and communication signals could be simultaneously received in the platform. The sensing cooperative center is in charge of sensing data fusion and result generation.

Figure 1. Architecture of Hardware Testbed including the sensing cooperative center to fuse the data from two ISAC systems in multi-dimension

The signal transmitting and receiving flowchart is described as following steps. In the transmitter, the integrated baseband signal is first passed through the digital-to-analog conversion module and intermediate frequency module, then up-converted to 28 GHz through the radio frequency head, and finally transmitted through the PAA to realize target positioning and communication signal transmission within the same frame structure. In the receiver, the PAA (Phased Array Antennas) beam direction of each ISAC system is first measured by the received echo signal to determine the angle of the target. Then based on the measured coarse angle, the transmitted beam direction is adjusted to make it staring at the target precisely. After that, the ISAC system 1 and system 2 send ISAC signals to detect the relative distance of the target. When two ISAC systems are placed in different positions, two sets of target sensing data can be obtained respectively, including the relative angle and distance of the same target. The raw sensing data of the target could be shared between ISAC system 1 and ISAC system 2 to realize cooperative sensing data sharing in the sensing cooperative center. Finally, the sensing cooperative center will fuse the raw sensing data from two ISAC systems by utilizing the certain date fusion algorithm such as weighted mean accuracy-based sensing data fusion, and use RMSE (Root Mean Square Error) to estimate the accuracy of distance measurement and angle measurement. At the same time, a receiver mounted on the platform continuously receives communication data from the integrated signal.

Figure 2. Layout of hardware testbed including ISAC system 1 and ISAC system 2 and other trial components

    - Trial Layout and Results

Fig. 2 shows the layout of the field trial, in which the target of corner reflector to be measured is placed on the Baidu Apollo automated driving vehicle (ADV), and the ADV is configured to move on a circular trajectory at a speed of 1 m/s. The two ISAC systems are placed at a distance of 2.7m to confirm the beams staring at target are from different directions and to improve the data fusion performance. After collecting enough observation samples (e.g. 50 samples per cluster, 50 clusters per handling packet) from the two ISAC systems, the sensing cooperative center performs the aggregation by using the date fusion algorithm to improve the accuracy of target distance and angle detection.

The output results from sensing cooperative center include ranging and velocity information for the ISAC system 1, ISAC system 2 and their cooperation as shown in Fig. 3. The results are measured in 16 different positions during the ADV moving on a circle. It can be observed with the sensing cooperative center and corresponding fusion algorithm involved, the two cooperative ISAC systems have obvious gains by comparing to single ISAC system no matter in terms of ranging or velocity estimation. Especially in ranging, the RMSE improvement could reach more than 50% for two cooperative ISAC systems in a certain position.  At the same time, we can also see the communication data throughput is still kept at a high level of 2.8Gbps as shown in the Communication Receiver window of Fig. 2.

Figure 3. Hardware test results for ranging and velocity by comparing single gNB and double gNB

Observation 3: BS supporting ISAC has little impact to the data throughput.

Observation 4: The networking gNB-based ISAC shows obvious benefits comparing to single station sensing.

Observation 5: To specify the network interface and signalling is meaningful and necessary to enable gNB supporting ISAC.

- Conclusions

In this contribution, we first briefly introduce that in China the low-altitude economic development is rising with representative use case of UAV delivering the logistic. Then, according to the study item in SA1, we find the typical ISAC use case is just to trace the UAV, which implies that the ISAC standardization pace needs to be accelerated. The hardware test results show to specify the network interface and signalling is meaningful and necessary to enable gNB supporting ISAC. The specific observations are copied as follows.

Observation 1: The low-altitude economic development is rising with representative use case of UAV delivering the logistic.

Observation 2: gNB-based ISAC has the natural advantage of coverage and networking for UAV tracing and even management.

Observation 3: BS supporting ISAC has little impact to the data throughput.

Observation 4: The networking gNB-based ISAC shows obvious benefits comparing to single station sensing.

Observation 5: To specify the network interface and signalling is meaningful and necessary to enable gNB supporting ISAC.

References

[1]	RWS-230488, RAN Chair's summary of Rel-19 workshop, RAN Chair (Qualcomm)

[2]	RWS-230242, Integrating Sensing and Communication for Rel-19, China Telecom

[3]	RWS-230186, Integrated Sensing and Communications in Rel-19, Huawei, HiSilicon

[4]	TR 22.837