3GPP TSG-RAN Meeting #101	RP-232119

# Bangalore, India, September 11-15, 2023

Source: 	KDDI Corporation

Title: 	Views on the Type 2 EN-DC/NR-CA UE for Non-collocated deployment

Agenda Item:	9.3.4.10

Document for:	Discussion

- Introduction

In the RAN4#108 meeting, RAN4 discussed the issue on how UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2, but no consensus. In this paper, we make some proposals to conclude the issue.

- Discussion

RAN4 agreed to introduce a new requirement for UE with more power imbalance (Power of wanted carrier +25 dBm) for intra-band non-contiguous CA (7.10A, R4-2303557) addition to the existing requirement for UE with less power imbalance (Power of wanted carrier +6 dBm). The new requirement is only applicable to two Rx ports and not applicable to four Rx ports per component carrier in Rel-18, assuming separate RF chains where each RF chain can handle each carrier separately (so-called Type 2), on the other hand the existing requirement assuming common RF chain where only one RF chain for two carrier (so-called Type 1).

Type 1 capability requirement is Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1. Type 2 capability requirement is Table 7.5.4.1/Table 7.6.4-2 in 38.133 and UE RF requirements for intra-band non-collocated NR-CA in 7.10A in 38.101-1. There are following two options on the table.

< Issue 1-1-1: How UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2 >

•	Proposals

o	Option 1: Introduce new BS signaling during RRC Reconf.

o	Option 2: Reuse the existing RRC Reconf.

With this option 1, we understand expected UE behaviors are as follows.

| UE category | Case# | New BS Signaling | Existing RRC Reconf. (maxMIMOLayer) | UE RF requirement (Power imbalance) | UE capability (UE architecture) |
| --- | --- | --- | --- | --- | --- |
| Only Type 1 capable | 1-1-1 | absent | 4 | 6 dB | Type 1 (shared) |
|  | 1-1-2 | absent | 2 | 6 dB | Type 1 (shared) |
|  | 1-1-3 | indicated | 4 | 6 dB | Type 1 (shared) |
|  | 1-1-4 | indicated | 2 | 6 dB | Type 1 (shared) |
| Both Type 1 & Type 2 capable* | 1-2-1 | absent | 4 | 6 dB | Type 1 (shared) |
|  | 1-2-2 | absent | 2 | 6 dB | Type 1 (shared) |
|  | 1-2-3 | indicated | 4 | 6 dB | Type 2 (separated) |
|  | 1-2-4 | indicated | 2 | 25 dB (see 7.10) | Type 2 (separated) |

* UE indicating new UE capability [intraBandNonColocatedCA-r18]

With this option 2, we understand expected UE behaviors are as follows.

| UE category | Case# | Existing RRC Reconf. (maxMIMOLayer) | UE RF requirement (Power imbalance) | UE capability (UE architecture) |
| --- | --- | --- | --- | --- |
| Only Type 1 capable | 2-1-1 | 4 | 6 dB | Type 1 (shared) |
|  | 2-1-1 | 2 | 6 dB | Type 1 (shared) |
| Both Type 1 & Type 2 capable* | 2-2-1 | 4 | 6 dB | Type 1 (shared) |
|  | 2-2-2 | 2 | 25 dB (see 7.10) | Type 2 (separated) |

* UE indicating new UE capability [intraBandNonColocatedCA-r18]

With option 1, BS can indicate UE whether to apply the existing requirement or new requirement using new BS signaling and UE apply Type 1 or Type 2 according to the new BS singling. With option 2, when BS signal ServingCellConfig.maxMIMO-Layers=2 via RRC Reconfiguration, UE capable for both Type 1 and Type 2 always comply with new requirement (25 dB power imbalance) using Type 2 (separated Rx chains). Considering n77/n78 UE is required to be equipped with a minimum of four Rx antenna ports, using Type 2 (separated Rx chains) for 2-layer case is possible. In our view, there seems no actual cases where BS needs to indicate Case_1-2-2 or Case_1-2-3 to UE (yellow highlighted cases), so we prefer option 2. But we would like to check our view with experts, thus we propose the follows.

Proposal 1: RAN discuss whether there are any actual cases where BS needs to indicate Case_1-2-2 or Case_1-2-3 to UE (yellow highlighted cases).

Proposal 2: If RAN conclude there are no actual cases, then agree option 2, which means UE always comply with new requirement (25 dB power imbalance) using Type 2 capability (separated Rx chains).

Proposal 3: If RAN can identify the actual cases where BS needs to indicate Case_1-2-2 or Case_1-2-3 to UE (yellow highlighted cases), then agree option 1, introduce a new BS signaling with the clear statement of the expected UE behavior when BS indicates Case_1-2-1/1-2-2/1-2-3/1-2-4 accordingly.

Proposal 4: If RAN reach the consensus, RAN agree to task RAN2 with work of relevant signaling based on the RAN decision.

- 3. Conclusion

Proposal 1: RAN discuss whether there are any actual cases where BS needs to indicate Case_1-2-2 or Case_1-2-3 to UE (yellow highlighted cases).

Proposal 2: If RAN conclude there are no actual cases, then agree option 2, which means UE always comply with new requirement (25 dB power imbalance) using Type 2 capability (separated Rx chains).

Proposal 3: If RAN can identify the actual cases where BS needs to indicate Case_1-2-2 or Case_1-2-3 to UE (yellow highlighted cases), then agree option 1, introduce a new BS signaling with the clear statement of the expected UE behavior when BS indicates Case_1-2-1/1-2-2/1-2-3/1-2-4 accordingly.

Proposal 4: If RAN reach the consensus, RAN agree to task RAN2 with work of relevant signaling based on the RAN decision.

- 4. Reference