- 3GPP TSG RAN Meeting #101		          			                       	    RP-232153

- Bangalore, India, September 11-15, 2023

Source:	ZTE

Title:	Views on channel modeling in Rel-19

Agenda Item:	8A.2.11

Document for: Discussion

- Introduction

Typically, to ensure accurate evaluation and study on new techniques, research on relevant channel model should always be done first. For example, at the beginning of 5G, extensive study on the channel model had been conducted with introduction of new features in [1], e.g., Generic channel model from 0.5-100 GHz, Blockage, spatial consistency, CDL to TDL mapping, Industrial IoT channel model, etc, which are used as the basis in the evaluation of 5G.

Then, in 5G-A, considering the emerging use cases and technical solutions, additional study on new component of channel model should be considered. In this contribution, based on the summary in [2], our views on channel modeling for ISAC, RIS and new spectrum considering new extreme massive MIMO network topologies are provided together with some initial details and results from trials and simulation in the Appendix.

- Discussion on scope for channel modeling in Rel-19

In Section 2.1, our consideration on channel modeling for integrated sensing and communication is provided. In Section 2.2, channel modeling for RIS is discussed.  In Section 2.3, our initial views on potential channel model updates considering new extreme massive MIMO deployment in new spectrum are elaborated.

  - Channel modeling for Integrated sensing and communication

2.1.1 Use cases and scenario for ISAC

As an important potential key technology of 5G-Advanced, integrated sensing and communication (ISAC) has drawn extensive attention. The integrated design of communication and sensing can reduce costs, reduce power consumption, and optimize resource utilization compared to two independent systems. The research topic "Integrated Sensing and Communication Research" in SA1 already contains 32 use cases and their corresponding requirements. These use cases can be simply divided into the following categories, as shown in Table 2-1.

Table 2-1 The categories of use cases in TR22.837 [3]

| Categories | Use case classification | Use case |
| --- | --- | --- |
| Object detection and tracking | UAV | 1. UAV flight trajectory tracing (5.10) 2. Network assisted sensing to avoid UAV collision (5.12) 3. UAV intrusion detection (5.13) |
|  | Smart transportation | - Pedestrian/animal intrusion detection on a highway (5.2) - Railway intrusion detection (5.7) - Sensing at crossroads with/without obstacle (5.11) - Accurate sensing for automotive maneuvering and navigation service (5.26) - Use case on Vehicles Sensing for ADAS (5.28) - Use case on sensing for automotive maneuvering and navigation service when not served by RAN (5.30) - Use case on blind spot detection (5.31) |
|  |  | 1. Sensing Assisted Automotive Maneuvering and Navigation (5.8) |
|  | Smart city | - Tourist spot traffic management (5.14) - Sensing for Parking Space Determination (5.20) - UAVs/vehicles/pedestrians detection near Smart Grid equipment (5.22) - Use case public safety search and rescue or apprehend (5.27) |
|  | Smart home | 1. Intruder detection in smart home (5.1) 2. Intruder detection in surroundings of smart home (5.6) 3. Immersive experience based on sensing (5.25) |
|  | Smart factory | - AMR collision avoidance in smart factories (5.23) - AGV detection and tracking in factories (5.9) - Use case of integrated sensing and positioning in factory hall (5.32) |
| Motion monitoring | Smart home | - Contactless sleep monitoring (5.15) - Health monitoring (5.17) - Service continuity of unobtrusive health monitoring (5.18) - Roaming for sensing service of sports monitoring (5.24) - Use case on Gesture Recognition for Application Navigation and Immersive Interaction (5.29) |
| Environment monitoring | Smart city | 1. Sensing for flooding in smart cities (5.5) 2. Rainfall monitoring (5.3) |
| Others | N/A | 1. Transparent Sensing Use Case (5.4) 2. Protection of Sensing Information (5.16) 3. Sensor Groups (5.19) 4. Seamless XR streaming (5.21) |

Among the above categories of use cases for ISAC, from the perspective of channel model, it will require different methodologies and solutions. For example,

- For the object detection and tracking type of use cases: The legacy radar-based approach, e.g., modelling the object via RCS model, can be considered as a baseline, which can be easily merged with existing channel model framework in [1]. Meanwhile, this feature is mainly enabled in the outdoor scenario, e.g., clear sky or UMa, etc.

- For the motion monitoring: Since this kind of application mainly relies on the processing of signal to/from the human body, the study on the in-body channel is needed. However, the relevant features, e.g., response of human over different frequency, impacts on the clothes, have not been well investigated in current stage and will require different methodologies. For this feature, in addition to the typical indoor and outdoor scenarios, the in-body case should also be defined as a new type of scenario.

- For others, e.g., environment monitoring, theoretical analysis and dedicated measurement to investigate the impact of different conditions (e.g., density of cloud, various dust particles) should be considered. And, the corresponding scenario also highly depends on the purpose, e.g., whether monitoring for outdoor.

So, given the different impacts on channel model study, identification of target use cases and scenarios should be considered firstly.

Proposal 1: A RAN-led study should be considered to identify the use cases and scenarios for ISAC in 5G-A.

Considering the strong interests and many trial/measurements as listed in Appendix, the detection, positioning and tracking type of use cases is promising to be deployed in NR. Then, the prioritization of this type of use cases can be considered. Meanwhile, for this type, different assumptions of the scenario setup for sensing should also be studied based on different implementation of sensing method. In general, sensing modes can be divided into mono-static sensing and bi-static sensing as illustrated in Figure 1. As mentioned in [4], six sensing modes can be considered:

- gNB mono-static sensing

- gNB bi-static sensing

- gNB as transmitter and UE as receiver

- UE as transmitter and gNB as receiver

- UE bi-static sensing

- UE mono-static sensing

Figure 1 Illustration for gNB mono-static sensing and gNB bi-static sensing

In our view, modes 1, 2, 3 and 4 should be prioritized since gNBs can provide wide-area coverage for these use cases listed in [3] and ensure perceived service. Especially for mode 6, it causes higher UE complexity as UE needs support of full duplex to send and receive signals simultaneously. Hence, this mode may be more appropriate for study in later stage e.g. 6G.

Proposal 1-1: The object-detection related use cases (e.g., Detection Positioning and tracking including smart transportation, UAV and smart factory scenarios) can be prioritized in Rel-19.

Proposal 1-2: gNB mono-static sensing and gNB bi-static sensing should be prioritized because of large coverage, high capability at gNB side.

2.1.2 Details of Channel model for ISAC

As mentioned above, the channel model for the object-detection related use cases can be developed based on the legacy framework in [1].

Proposal 2: The channel model for sensing can be developed based on the mechanism in TR 38.901.

More specifically, on top of the procedures of the existing channel model specified in TR 38.901, the following enhancements of stochastic geometry channel modelling can be considered:

- Modelling of scattering characteristics of object via RCS:

Based on the application in radar-area, the response of object on the impinging wave is modeled as RCS to emulate the scattering characteristics. For the performance evaluation, it will affect small-scale fading channel generation. Typically, the relevant information is obtained based on extensive measurement since due to the irregular shape of object and non-homogeneous characteristic.

For example, as shown in Figure 2 and Figure 3 based on our real test for UAV, the different RCS value is obtained per angle at center frequency 23.5GHz with measurement bandwidth equal to 5GHz.

Moreover, some typical values of RCS following a uniform distribution, Gaussian distribution, or other distribution for different targets are listed in Table 1, which can be used in the relevant simulation as approximation, e.g., for large scale.

Figure 2 The radar cross section varies when the angle between Tx and Rx is 10° (real test)

Figure 3 The radar cross section varies when the angle between Tx and Rx is 45° (real test)

Table 1 Typical value of RCS

| Type of target | Typical value of RCS（） |
| --- | --- |
| Insect or Bird |  |
| Human | 0.5-2 |
| Small drone | 0.01-0.5 |
| Car or truck | 100-300 |

Proposal 2-1: RCS model is introduced to emulate the scattering pattern of object in the channel model for sensing.

- LOS and NLOS probability of sensing link

In [1], the LOS probability for each link is defined as the equation related to the distance between UE and gNB. Similar method can be used to determine the LOS probability of the sensing link for mono-static sensing and bi-static sensing. As shown in Figure 4, for mono-static sensing, the LOS probability only needs to be determined once using channel reciprocity. But for bi-static sensing, the LOS probability should be determined twice as illustrated in Figure 5, i.e., one is between the transmitting station and the target object, and the other is between the target object and the receiving station.

More specifically, if one of the transceivers is NLOS with the target object, the large-scale loss will be significantly increased, which will lead to the weak signal strength of the sensing link, and could be probably ignored for modelling. Hence, whether this NLOS link (or called as two hops reflecting) should be modeled needs to be further studied.

Figure 4 LOS and NLOS for mono-static sensing

Figure 5 LOS and NLOS for bi-static sensing

Proposal 2-2: The legacy method in TR 38.901 can be optimized to determine the LOS probability for the sensing link.  The pure LOS scenario can be prioritized.

- Pathloss of sensing link

For the sensing link, the additional impacts of scattering via object should be considered, which is not considered in the path loss defined in [1]. As mentioned above, similar to the legacy radar system, when calculating the path loss, the equation can be depicted as:

Where, the  and  refer to the traditional PL for the two segments in one sensing link, and the rest refers to the impact of RCS. More specifically, for some cases, e.g. UAV scenarios, based on the formula 4.2-1 with RMa parameters described in TR 38.901, and based on the formula 4.2-1 with TR36.777 RMa-AV, it can be found that there is no much difference among formulas in the LOS only links for UAV scenarios as shown in Figure 6.

Figure 6  Pathloss comparisons for UAV

Proposal 2-3: The large scale impact of RCS can be considered as add-on component to determine the pathloss for sensing.

Meanwhile, the impact of clutter, e.g., clutter loss can also be considered in the sensing model, which also has impact on the performance due to the potential degradation of signal strength.

- Small scale channel model parameters for sensing link

For the small-scale feature, in the existing model, the multiple clusters and rays, as shown in Figure 7 are modeled with different parameters, e.g. power, angel, delay, etc. For the sensing scenario, modelling of additional link in small scale level is also necessary.

Figure 7 Illustration for channel model based on clusters and rays

Typically, the following two methods can be used to reflect the effect of target object:  in the wireless sensing channel modelling:

- Cascaded channel modelling. We consider the channel model between sensing transmitter and sensing receiver as a whole.

Figure 8 Illustration for the whole channel modelling

- Segmented channel modelling. First, we consider the channel model between sensing transmitter and the target object, and the channel between the target object and sensing receiver respectively. Second, we combine the two channel models into one channel model.

Figure 9 Illustration for the segmented channel modelling

Additionally, for small scale parameters of each sensing link, the calculation of the time delay, angle and power need to be considered. Meanwhile, to emulate the impact of environment, the multi-paths channel for each typical scenario, e.g., UMa, UMi, should also be generated. Then, the association of small-scale channel between the sensing link and other clusters in the environment (i.e. clutter) are also needed.

Proposal 2-4: The small-scale feature of two segments in one sensing link can be modelled in cascaded way or as whole. Time delay, angle, path power, and the clutter in the environment for sensing should be re-modelled compared to the existing channel model.

- Procedure for channel realization:

Based on the above description, the overall procedure to generalize the channel realization with considering the additional sensing link is shown in Figure 10.  More specifically, after obtaining the channel for each link, the final response can be described as H= Hobj+Henvironment+Noise .

Figure 10 Illustration of Channel modelling for target and environment

Proposal 2-5: The update on the procedure of channel generation is needed with considering the sensing objects, clutters in the environment and noise.

  - Channel model for RIS

2.2.1 Use cases and scenario for RIS

To boost the performance of existing network and explore the potential new network topologies, as shown in Figure 11, RIS (reconfigurable intelligent surface) has been extensively studied with the following identified usages including [5][6][7][8]:

- Coverage extension for Outdoor, Indoor, O2I and O2X scenario;

- Throughput improvement with one or multiple RIS.

Figure 11 Typical usage of RIS

Meanwhile, from the perspective of channel, the same modeling can be used to support these two usages without any difference. Additionally, regarding scenarios, since RIS is considered as an additional entity in the network, it can be deployed in the all relevant scenarios, e.g., UMi, Indoor, identified in TR 38.901[1].

Proposal 3: For channel modeling of RIS, the same modeling methodology can be used to support different use cases in all legacy scenarios defined in TR 38.901

Regarding RIS types, although multiple types of RIS are proposed, as shown in Figure 12, reflective RIS and transmissive RIS are preferred considering the maturity and commercial needs.

Figure 12 Typical RIS types

To achieve the benefits of lower cost and power consumption, RIS in passive mode (i.e., without integrated amplifier) is more promising in Rel-19. Meanwhile, as a generic technology, RIS can be used for different frequency ranges, e.g., FR1/FR2/FR3.

Proposal 4: For channel modeling of RIS, reflective and transmissive RIS without integrated amplifier should be prioritized in Rel-19.

2.2.2 Details of channel modeling for RIS

For the modeling of RIS, as mentioned above, RIS is considered as an additional entity in the system. Its channel model can be based on the existing TR 38.901 with the corresponding changes for introduction of RIS.

Proposal 5: The channel model for RIS can be developed based on the mechanism in TR 38.901.

- Modelling of scattering characteristics of RIS:

Typically, RIS is deployed as 2-D rectangular planar array consisting of M*N RIS elements as shown in Figure 12. In this case, the manipulation of impinging radio wave is considered as the impact of the whole array with different configured “codebook” to control the response per RIS element. In this case, there are following two ways to model the overall scattering characteristics:

  - Option-1: Define the overall scattering pattern of RIS:

In this way, the overall pattern of RIS is defined as whole to emulate the response of RIS panel with different combinations of dimension of RIS panel, incident angle and departure angle of the radio wave.

  - Option-2: Define the scattering pattern per element:

In this way, similar to the legacy definition of pattern for each antenna element, the scattering pattern is introduced per RIS element. For example, as shown in Figure 13, different solutions can be considered to emulate the response per RIS element.

Figure 13 Different scattering patterns of RIS element

(Left: Lambertain pattern, Right: Mathematically modeled pattern)

Then, once the response is determined, the whole pattern of RIS will be generated in the similar way to legacy beamforming mechanism.

Proposal 5-1: For channel modeling of RIS, the following two options can be considered to model the scattering characteristics of RIS:

  - Option-1: Define the overall scattering pattern of RIS

  - Option-2: Define the scattering pattern per element

- LOS and NLOS probability of RIS-involved link:

As shown in Figure 12, the LOS and NLOS condition for two segments of the RIS-involved link is decoupled and determined by the surrounding environment. Then, in this case, the legacy methodology defined in 38.901 can be reused to generate the LOS/NLOS condition in the similar way as mentioned in section 2.1.1. Meanwhile, as the extension of BS, the LOS condition can be assumed for the BS-RIS segment of this link as typical scenario.

Proposal 5-2: The legacy method in TR38.901 can be used to determine the LOS probability for the RIS-involved link.  The pure LOS scenario can be prioritized for BS-RIS link.

- Large scale channel model parameters of RIS-involved link:

As shown in Figure 12, the overall path loss of the RIS-involved link is equivalent to the summation of PL per segment, i.e., BS-RIS and RIS-UE. These two components can be directly obtained by reusing the legacy pathloss model defined in 3GPP. For example, the path loss defined in TR 38.901[1] can be used to generate the PL for segment of RIS-UE. But in some cases, given the different location of RIS (e.g., height of RIS), the height-dependent feature (e.g., defined in TR 38.777[9]) can be considered to generate the PL for the segment of BS-RIS.

Proposal 5-3: The legacy PL channel model with height-dependent feature can be considered to generate the PL for RIS-involved link.

- Small scale channel model parameters for RIS-involved link:

As shown in Figure 14, for the RIS-involved scenario, the Cascaded channel modelling method can be considered. More specifically, in this way, the small-scale parameters for BS-RIS and RIS-UE link are generated separately according to the existing channel model. The adjustment of power and overall delay of the RIS-involved link should be further considered, e.g., impacts on the scattering pattern of RIS element.  Then, the association between these two segments should be considered as illustrated in the right side. The selection of effective ray can be based on the power threshold as legacy approach.

Figure 14 Small-scale channel model feature for RIS-involved scenario

(Left: Cluster distribution; Right: Ray association between segments)

Proposal 5-4: The small-scale channel model feature of two segments in one RIS-involved link can be modeled in a cascaded manner with cluster association. The impacts of RIS scattering pattern should be considered for the power of cluster.

- Procedure for channel realization:

Based on the above description, the procedure to generate the channel realization with considering the additional link can be conducted by following the legacy procedure with the corresponding changes, i.e., association of cluster for cluster for two segments within one RIS-involved link. The final channel realization is a summation between RIS-involved link and other legacy clusters as H= HBS-RIS-UE+HBS-UE+Noise, which is applicable for both stochastic model and hybrid model.

Proposal 5-5: The updates on the procedure for channel realization is needed with considering the RIS-involved link additionally.

  - Channel model for new spectrum

As one of the bases to boost the performance of wireless system, the introduction of new spectrum can dramatically increase user throughput and network performance. Recently, potential new spectrum for 5G-A and 6G, e.g., 6.425GHz-7.125GHz, 7-24 GHz, has been extensively discussed. As mentioned before, the investigation on the propagation characteristics is critical for the study on relevant techniques. In general, we may start with generic channel model, e.g., PL, fast fading parameters, for new frequency range. However, as shown in the TR38.901[1], according to the introduction of frequency-dependent model, the corresponding parameter sets can already be derived by following the corresponding equations in the current channel model. Since the channel model covering 0.5-100GHz is based on extensive study and results obtained before, the update of relevant model is not expected to require major work unless new scenarios/techniques are considered in the potential new spectrum.

Except for the generic channel, we need to further consider new essential techniques/scenarios to support the usage of new spectrum. Extreme massive MIMO (i.e. larger scale) is expected to be used in the new spectrum in the near future. Given the development of RF techniques, the eLAA (extreme Larger Antenna Array) will be introduced in the deployment.  Meanwhile, as mentioned before, RIS will be also deployed in the network to enhance the spatial domain transmission with even larger size as eLAA.  In addition, ubiquitous/distributed MIMO requires extra changes to the existing channel model. Some common characteristics can be commonly considered for massive MIMO network with or without RIS.  In this case, further study on the following spatial domain features for channel is needed to support the relevant usage as the common part:

- Near field effect:

Traditionally, the far-field assumption is used to model the transmission in the channel realization since the separate distance between Tx and UE is larger than the Rayleigh distance (i.e.,D2λ, where D refers to the largest dimension of array and λ is the wavelength). However, in case of eLAA and RIS, the dimension of panel will be increased with extremely large number of elements and the future deployment of massive MIMO nodes, e.g. RIS in above 6GHz frequency is expected to be closer to terminals (e.g., O2I). Also, the modelling of distributed MIMO with dense APs may also involve near field effect.  Then, the study of near field effect should be considered with new spectrum.

- Spatial non-stationary feature:

In the existing channel model, the same set of clusters is assumed to be observed by all elements in the panel. However, with increased size, the effective cluster for different region of antennas will be varied as illustrated in Figure 15.

Figure 15 Spatial non-stationary feature due to the varied effective clusters

Then, the study of this phenomenon should be considered if the gain of full array is expected in the transmission, which is important for both middle band and higher bands.

- The existing model of other MIMO features like blockage, spatial consistency, frequency selectivity, polarization should be revisited to check whether any updates are needed for new extreme massive MIMO network topologies. e.g. Distributed MIMO as illustrated in Figure 16.

Figure 16 Channel model features for distributed MIMO

(Left: Distance and delay correlation; Right: Cluster correlation)

- Ubiquitous/Distributed MIMO is expected to be an essential network topology deployed under the potential new spectrum in the near future.  It requires extra changes to the existing channel model. In this case, multiple APs (access point, or MIMO TRP) are deployed in a distributed and denser manner in a specific spatial range. In the current TR38.901, the channel coefficient generation procedure corresponding to each AP is independent for the modules, e.g., channel delays generation, clusters generation, and angles generation for NLOS paths. However, in the scenario where APs are closer, several correlations between channel generation modules corresponding to different APs need to be considered, e.g., the correlation of distances and delays due to specific geometrical relationship, and a common cluster of the AP-UE links Then, different from the legacy scheme, the study of this phenomenon should be considered.

- Some simulation comparison with and without modeling accurate frequency selectivity in extreme massive MIMO is done in Appendix D.

Proposal 6: New MIMO features including near-field effect and spatial non-stationary feature should be studied in channel model for Rel-19 together with new spectrum. The existing model of MIMO features should be revisited under new extreme massive MIMO network topologies.

Proposal 7: A RAN1-led study on new spectrum for extreme massive MIMO deployment with/without RIS should be considered in Rel-19.

- Conclusions

In this contribution, the views on channel modeling in Rel-19 are elaborated with the following proposals:

For ISAC:

Proposal 1: A RAN-led study should be considered to identify the use cases and scenarios for ISAC in 5G-A.

- Proposal 1-1: The object-detection related use cases (e.g., Detection Positioning and tracking including smart transportation, UAV and smart factory scenarios) can be prioritized in Rel-19.

- Proposal 1-2: gNB mono-static sensing and gNB bi-static sensing should be prioritized because of large coverage, high capability at gNB side.

Proposal 2: Following the RAN-level study, the channel model for sensing can be studied and developed based on the mechanism in TR 38.901.

- Proposal 2-1: RCS model is introduced to emulate the scattering pattern of object in the channel model for sensing.

- Proposal 2-2: The legacy method in TR38.901 can be optimized to determine the LOS probability for the sensing link.  The pure LOS scenario can be prioritized.

- Proposal 2-3: The large scale impact of RCS can be considered as add-on component to determine the pathloss for sensing.

- Proposal 2-4: The small-scale feature of two segments in one sensing link can be modelled in cascaded way or as whole. Time delay, angle, path power, and the clutter in the environment for sensing should be re-modelled compared to the existing channel model.

- Proposal 2-5: The update on the procedure of channel generation is needed with considering the sensing objects, clutters in the environment and noise.

For RIS:

Proposal 3: For the channel model of RIS, the same modelling methodology can be used to support the different use case in all legacy scenarios defined in TR 38.901

Proposal 4: For the channel model of RIS, the reflective and transmissive RIS without integrated amplifier should be prioritized in Rel-19.

Proposal 5: The channel model for RIS can be developed based on the mechanism in TR 38.901.

- Proposal 5-1: For the channel model of RIS, the following two options can be considered to model the scattering characteristics of RIS:

- Option-1: Define the overall scattering pattern of RIS:

- Option-2: Define the scattering pattern per element.

- Proposal 5-2: The legacy method in TR38.901 can be used to determine the LOS probability for the RIS-involved link.  The pure LOS scenario can be prioritized for BS-RIS link.

- Proposal 5-3: The legacy PL channel model with height-dependent feature can be considered to generate the PL for RIS-involved link.

- Proposal 5-4: The small-scale feature of two segments in one RIS-involved link can be modeled in cascaded way with cluster association. The impacts of RIS scattering pattern should be considered for the power of cluster.

- Proposal 5-5: The updates on the procedure for channel realization is needed with considering the RIS-involved link additionally.

For extreme massive MIMO deployment under new spectrum:

Proposal 6: New MIMO features including near-field effect and spatial non-stationary feature should be studied in channel model for Rel-19 together with new spectrum. The existing model of MIMO features should be revisited under new extreme massive MIMO network topologies.

Proposal 7: A RAN1-led study on new spectrum for extreme massive MIMO deployment with/without RIS should be considered in Rel-19.

In Appendix, the simulation results and trials for ISAC, RIS and extreme MIMO features are presented to show the feasibility of these techniques in 5G Advanced system.

Reference

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

- RWS-230488, RAN Chairs summary of Rel-19 workshop

- 3GPP TR 22.837, Feasibility Study on Integrated Sensing and Communication, V0.4.0, 2023-03.

- RWS-230283,  Discussion on sensing for NR, ZTE, Sanechips

- Technology report of IMT 2030(6G) promotion group on RIS, https://www.imt2030.org.cn/html/default/en/Publications/Report/index.html?index=2

- White paper for RIS, http://www.risalliance.com/cn/riswp2023.html

- ETSI GR RIS 001, RIS, Use case, deployment scenario and requirement

- 3GPP Highlights (Issue 6), Evolution of Network Nodes: From IAB to IRS, https://www.3gpp.org/news-events/partner-news/5ga-paper-ntn

- TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles

Appendix-A Trials for ISAC

In this section, we provide a part of real field test results based on ZTE’s prototypes of ISAC in both millimeter wave and sub-6GHz.

- Trials in 2022

The sub-6GHz prototype, based on the 4.9 GHz 5G commercial base stations with 100MHz sensing bandwidth, the outdoor UAV, vehicle and pedestrian sensing test, was completed in 2022 around ZTE working area. The test results show that the maximum sensing distance of outdoor low-altitude UAV is more than 1400m in certain environment. Sensing of moving tracks of multiple targets including UAV, vehicle and pedestrian is realized at the same time. This preliminarily verifies the feasibility of low-frequency sensing in low-altitude environment for security, traffic safety management and other applications.

Figure A-1 4.9 GHz test environment

The millimeter wave prototype is using a 24GHz millimeter wave base station with 100MHz sensing bandwidth. The base station uses a single AAU to complete the sensing process.

Figure A-2 UAV test environment and single AAU prototype

At 24GHz, the ranging accuracy can reach 0.5m around 100m distance between UAV and gNB and the ranging accuracy can be less than 0.5m within 100m distance between vehicles and gNB, as shown in Figure A-3.

Figure A-3 Ranging accuracy of UAVs and vehicles

- Trials in 2023

In this year, China Telecom and ZTE Corporation conducted ISAC test results in venues of the 19th Hangzhou Asian Games.  During the Asian Games, UAV flying over the venues will bring potential danger to the normal progress of the games and the audience's viewing, as shown in Figure A-4.

Figure A-4 UAV intrusion risks at the venue

5G radio signals can be used to provide wireless access for communication as well as to generate sensing data for object detection e.g. sense presence or proximity of UAVs illegal flying in a specific area. 5G System could provide sensing service by processing sensing data and output sensing information (e.g. relative position, altitude, distance, velocity, direction, warning).

In this trial with millimeter wave, AAU base stations are deployed at the entrance of the Asian Games Village, as shown in Figure A-5 where gNB mono-static sensing mode is used.

Figure A-5 the deployment location of the ISAC base station

Electronic fences have been set up around the Asian Games Village, as shown in the blue area in figure A-6. The 5G system periodically senses the restricted area and will provide warning alarm if there is any UAV flying into the restricted area border of the Asian Games Village.

Figure A-6 Illustration of the restricted area

As shown in Figure A-7, when a UAV enters the sensing area of the base station, the base station will sense the presence of the UAV, its location, and even track the UAV. In this test, the sensing range of a single AAU can reach 1000m. The base station will send the 3GPP sensing data to 5G network and 5G network will derive the sensing result from the 3GPP sensing data and expose the sensing result to the third-party application. Further, the third party applications will display sensing results in real-time in the application. As shown in figures, the yellow dots are the sensed UAV trajectory. It can be observed that the location, velocity, and altitude of the UAV can be tracked, sensing range accuracy reaches the sub-meter level.

When the UAV flies into the restricted area border for the Asian Games Village, the third-party application will trigger an electronic fence alarm and pop up an alarm prompt box.

Figure A-7 Demonstration of UAV electronic fence intrusion

Appendix-B Trials for RIS

In this section, we provide a part of real field test results based on ZTE’s prototypes of RIS in both millimeter wave and sub-6GHz.

- Trials in 2022

In 2022, in collaboration with China Mobile Research Institute, has jointly completed the prototype verification of the industry's first 5G base station and dynamic Reconfigurable Intelligent metaSurface (RIS) collaborative beamforming technology.

Figure B-1: Test environment for dynamic RIS

The verification result shows that, compared with static RIS that can only improve fixed-point coverage, base station and dynamic RIS collaborative beamforming technology not only greatly increased the coverage capability of base stations, but also supported users’ seamless connectivity even during movement.

The research and development of both parties in the field of RIS has entered into the second phase of dynamic collaboration between base stations and RIS.

In 2021, both parties accomplished prototype verification of the first phase of the RIS technology, and initially explored its feasibility to improve fixed-point coverage in 5G blind spots and weak areas. In 2022, China Mobile Research Institute and ZTE continued to explore the RIS technology, and jointly accomplished the research and development of the dynamic RIS prototype with the collaborative beamforming capability, and accomplished the industry's first technical verification in the laboratory and field environment respectively.

In field test, the fixed-point coverage before and after dynamic RIS applications and coverage and rate tests under mobile scenarios are performed. The test results show that the dynamic RIS brings great gains in the test area, the maximum RSRP is increased by 23 dB, and the user rate is increased by five times.

- Trials-A in 2023

In this year, ZTE released the ZTE's dynamic RIS 2.0 product with significant reduction on the specification and power consumption, and the deployment process has been made more convenient. The base station and RIS dynamic cooperative beamforming technology not only significantly enhance the coverage of base stations, but also support seamless connection of users in mobile scenarios.

Figure B-2: Illustration of dynamic RIS 2.0

Through the incorporation of new materials and an evolved architecture, dynamic RIS 2.0 products achieve an impressive 80% reduction in power consumption. The integrated design also contributes to a lighter and aesthetically pleasing appearance. Additionally, dynamic RIS 2.0 is designed for easy installation and adaptability to diverse environments, making deployment, management, and maintenance more streamlined.

- Trials-B in 2023

In this year, the trial of RIS together China Telecom has been made in close proximity to the venues of the Hangzhou Asian Games, where it stands numerous high-rise buildings. By utilizing rapidly deployable RIS, the expansion of base station coverage can be achieved within shops and parking garages.

Figure B-3: Illustration of Mobile RIS Cabinet

The following figures show that outdoor RIS enhances indoor coverage. The green color shows the beam shape reflected by RIS.

Figure B-4: Illustration of Outdoor RIS enhances indoor coverage

The results have shown that the average uplink data rate increased from 99.96 Mbps to 535.94 Mbps i.e. 436% improvement, while the average downlink data rate increased from 1468 Mbps to 3134.73 Mbps i.e. 114% improvement. The synchronization signal block(SSB)  reference signal received power(RSRP) showed an increase of approximately 8.7 dBm, and the SSB signal to interference and noise ratio(SINR) increased by around 12.44 dBm.

In another case, UE was deployed in the weak signal areas and blind spots of the parking garage to assess the impact of RIS on communication performance. The average uplink data rate increased from 99.84 Mbps to 524.46 Mbps i.e. 425% improvement. Additionally, the average downlink data rate increased from 841.08 Mbps to 3406.49 Mbps i.e. 305% improvement. The SSB RSRP showed an increase of approximately 22.2 dBm, and the SSB SINR showed an increase of 24 dBm.

The experimental results indicate that with the utilization of RIS and corresponding dynamic coordinated beamforming techniques, the effective coverage area of 5G-Advanced signals increases by approximately 30%. In typical scenarios, the downlink data rate improves by over 3-6 times, while the uplink data rate can increase by up to 20 times.

Appendix-C Simulation for ISAC

- C.1 Simulation for channel model

In this subsection, highway scenario is taken for simulation. Figure C.1-1 shows the distribution of highway scenario. In Figure C.1-1, assume there are two sensing targets, which are represented by green blocks. For both of two sensing targets, the transmitters are in the 6th cell, and the receivers are in the 43th cell, i.e. bi-static sensing mode.

Figure C.1-1 Illustration on highway scenario with two sensing targets

Figure C.1-2 shows the single target channel pulse of these two sensing targets and clutter channel pulse in time domain. Figure C.1-3 shows the multi-target channel pulse merging these two sensing targets in time domain where the smaller figure (right figure) is the zoomed version in the particular range of the bigger figure (left one).

(a) Channel pulse of target 1              (b) Channel pulse of target 2                (c) Channel pulse of clutter

Figure C.1-2 Single target channel pulses in time domain

Figure C.1-3 Merging channel pulses of two targets

For multi-target sensing case, due to the effect of distance between multiple sensing targets and the effect of reflecting power of the different sensing target, the generated channel response with multiple sensing targets becomes complicated. Figure C.1-4 provides three cases of multi-target channel pulse merging two sensing targets in time domain, where the distance between two sensing targets is different in three cases. In Figure C.1-4(a), the received power of two sensing targets are comparable, and these two sensing targets can be detected distinctly. In Figure C.1-4(b), the received power of two sensing targets are dramatically different, which leads to intractable detection of these two sensing targets . In Figure C.1-4(c), due to the interference of multiple NLOS clusters, there are many small spurs in time-domain channel, which cause complicated channel response.

Sensing target

spurs

- (b)                                                         (c)

Figure C.1-4 Multi-target channel pulse merging two targets

- C.2 Simulation for sensing

For performance evaluation of multi-target sensing, we define the simulation metrics as follows.

- Number of successful detections: target number of successful detections for each receiver, which is per receiver.

- Probability of successful detections: target number of successful detections divided by total target number for each receiver, which is per receiver.

- Number of missed detections: target number of missed detections for each receive, which is per receiver.

- Probability of missed detections: target number of missed detections divided by total target number for each receiver, which is per receiver.

- Number of false alarms: number of errors of determining non-target as sensing target, which is per receiver.

- Probability of false alarm: total number of false alarms of all receivers divided by total target number of all receivers.

- Location accuracy: location accuracy of successful detected target, involving distance accuracy, horizontal angle of arrival (AOA) accuracy and zenith angle of arrival (ZOA) accuracy.

- C.2.1 Performance for UAV use case

In UAV evaluation scenarios, BSs with three cells are distributed in cellular form. UAVs are dropped in each cell in uniform distribution. The channel model between transmitting cell and receiving cell is the RMa channel model, which is specified in TR 38.901. An illustration on UAV scenario is exhibited in Figure C.2-1, where red circle represents BS’s location, red dot is UAV’s location, solid blue line is boundary of three cells of one BS, and solid red line refers to antenna boresight direction of a cell. Table C.2-1 provides evaluation parameters in UAV scenario.

Figure C.2-1 An illustration on UAV scenario

Table C.2-1 Evaluation parameters in UAV scenario

| Parameter | Value |
| --- | --- |
| Carrier frequency | 6GHz |
| Bandwidth | 100MHz |
| Subcarrier spacing | 30kHz |
| Transmitting power of BS | 35dBm |
| Antenna of BS | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5λ |
| Antenna gain of transmitting BS | 26dBi |
| Antenna gain of receiving BS | 26dBi |
| Antenna height of BS | 35m |
| Antenna height of UAV | 100m |
| Minimum horizontal distance between BS and UAV | 35m |
| Noise factor of BS | 9dBi |
| thermal noise | -174dBm/Hz |
| Cluster number | 1 |
| K-factor | 15 |
| Vehicle RCS () | 0.2 |

- Performance evaluation in mono-static sensing mode for UAV use case

Figure C.2-2 shows the number of detections and detection probability per receiver in mono-static sensing mode for UAV use case. Figure C.2-3 shows the distance accuracy in mono-static sensing mode for UAV use case. C.2-4 shows the angle accuracy in mono-static sensing mode for UAV use case.

Figure C.2-2 Number of detection and detection probability per receiver in mono-static sensing mode for UAV use case

Figure C.2-3 Distance accuracy in mono-static sensing mode for UAV use case

Figure C.2-4 Angle accuracy in mono-static sensing mode for UAV use case

- Performance evaluation in bi-static sensing mode for UAV use case

Figure C.2-5 shows the number of detections and detection probability per receiver in bi-static sensing mode for UAV use case. Figure C.2-6 shows the distance accuracy in bi-static sensing mode for UAV use case. C.2-7 shows the angle accuracy in bi-static sensing mode for UAV use case.

Figure C.2-5 Number of detection and detection probability per receiver in bi-static sensing mode for UAV use case

Figure C.2-6 Distance accuracy in bi-static sensing mode for UAV use case

Figure C.2-7 Angle accuracy in bi-static sensing mode for UAV use case

- C.2.2 Performance for V2X use case

In V2X use case, highway scenario, as a typical scenario, is employed to evaluate sensing performance. In highway scenario, vehicles are dropped as TR 37.885. BSs with three cells are distributed in cellular form. The channel model between transmitting cell and receiving cell is the RMa channel model, which is specified in TR 38.901. An illustration on V2X scenario is exhibited in Figure C.2-8, where red circle represents BS’s location, red dot is V2X’s location, solid blue line is boundary of three cells of one BS, and solid red line refers to antenna boresight direction of a cell. Table C.2-8 provides evaluation parameters in highway scenario.

Figure C.2-8 An illustration on highway scenario

Table C.2-8 Evaluation parameters in highway scenario

| Parameter |  |
| --- | --- |
| Carrier frequency | 6GHz |
| Bandwidth | 100MHz |
| Subcarrier spacing | 30kHz |
| Transmitting power of BS | 49dBm |
| Antenna of BS | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5λ |
| Antenna gain of transmitting BS | 30dBi |
| Antenna gain of receiving BS | 30dBi |
| Antenna height of BS | 35m |
| Minimum horizontal distance between BS and vehicle | 35m |
| Noise factor of BS | 9dBi |
| thermal noise | -174dBm/Hz |
| Vehicle RCS () | 200 |
| K-factor | 15 |

- Performance evaluation in mono-static sensing mode for V2X use case

Figure C.2-9 shows the number of detections and detection probability per receiver in mono-static sensing mode for highway scenario. Figure C.2-10 shows the distance accuracy in mono-static sensing mode for highway scenario. C.2-11 shows the angle accuracy in mono-static sensing mode for highway scenario.

Figure C.2-9 Number of detections per receiver in mono-static sensing mode for highway scenario

Figure C.2-10 Distance accuracy in mono-static sensing mode for highway scenario

Figure C.2-11 Angle accuracy in mono-static sensing mode for highway scenario

- Performance evaluation in bi-static sensing mode for V2X use case

Figure C.2-12 shows the number of detections and detection probability per receiver in bi-static sensing mode for highway scenario. Figure C.2-13 shows the distance accuracy in bi-static sensing mode for highway scenario. C.2-14 shows the angle accuracy in bi-static sensing mode for highway scenario.

Figure C.2-12 Number of detections and detection probability per receiver in bi-static sensing mode for highway scenario

Figure C.2-13 Distance accuracy in bi-static sensing mode for highway scenario

Figure C.2-14 Angle accuracy in bi-static sensing mode for highway scenario

Appendix-D Simulation for extreme MIMO feature

- D.1 Simulation for modeling on frequency selectivity.

Figure D.1-1. The spatial domain effect due to delay spread among micro rays each with one antenna port

In current channel model of TR38.901, the spatial domain direction for each ray is fixed for different sub bands, such as in because the product of a wavelength and a distance of antenna port is assumed to be a fixed value, such as 0.5. This assumption is not suitable for the case of the new spectrum with large band and massive MIMO.  Frequency selectivity feature should be accurately modeled in this case.

Fig D.1-2 to Fig D.1-2 show comparison of precoding with and without considering enhanced model of frequency selectivity. For each of Fig D.1-2 and Fig D.1-3, the left figure shows the mean loss for different subband unit and the right figure shows the CDF of correlation between the precoding and the channel for the last subband. The correlation between the precoding and the channel is a normalized correlation. The evaluation parameter is shown in Table D.1-1.

Table D.1-1 Evaluation parameters in highway scenario

| Parameter |  |
| --- | --- |
| Carrier frequency | 6GHz |
| Bandwidth | 200MHz |
| Subcarrier spacing | 30kHz |
| Antenna of BS | (M, N, P, Mg, Ng, Mp, Np) =(M,1,1,1,Mp,1,1) dH = dV = 0.5λ |
| Antenna of UE | (M, N, P, Mg, Ng, Mp, Np) =(1,1,1,1,1,1,1) dH = dV = 0.5λ |
| Subband size | 30kHz*12*8 |
| NumberofRay | 1, 4 |

Figure D.1-2 Comparison of precoding with and without considering enhanced model of frequency selectivity with M=64, Mp=16

Figure D.1-3 Comparison of precoding with and without considering enhanced model of frequency selectivity with M=64, Mp=64