- 3GPP TSG RAN Meeting #101					                       	      RP-232155

- Bangalore, India, September 11-15, 2023

Source:	ZTE, Sanechips, China Telecom, ETRI

Title:	Views on NCR in Rel-19

Agenda Item:	8A.2.13.1

Document for: Discussion

- Introduction

In Rel-18, the basic functionality of NCR has been specified with following assumptions [1]:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867

- For only single hop stationary network-controlled repeaters

- The NCR is transparent to the UE.

- Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

As one of candidate topic for Rel-19 [2], additional aspects (e.g., leftover of Rel-18 and new features) have been proposed by companies in last RAN-Release 19 workshop to further explore the use case and promote the capability of this feature. In this contribution, the detailed views on the scope in Rel-19 are provided.

- Discussion on scope for NCR in Rel-19

  - Power control

As the leftover of Rel-18, the power control of NCR has been discussed for both DL and UL power control with following agreements:

Agreement (SI phase in RAN1#109) Recommend to capture the following observation in TR 38.867: - The benefits of power information used to control the behavior of NCR-Fwd for the DL of access link and/or UL of backhaul link are observed by the following inputs: - [Source-1, Huawei] shows that for the uplink transmission via NCR, a fixed NCR amplifying gain may lead to interference to the gNB or NCR UL coverage loss. For the downlink transmission via NCR, a fixed NCR amplifying gain may lead to NCR RU saturation or NCR DL coverage loss. - [Source-2, vivo] shows that the optimal system performance can be achieved when repeater’s gain is set to a proper value. - [Source-3, ETRI] shows that dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration. - [Source-4, Ericsson] mentions that the gain control is needed for self-interference management due to repeater oscillation. - This agreement does not change the prioritization of PC Agreement (SI phase in RAN1#109) The controlling of the amplifying gain of NCR-Fwd is considered to enable the power control of NCR-Fwd if PC is recommended as side control information for NCR in Rel-18 - FFS: Controlling of the transmission power of NCR-Fwd

However, due to limited TU, this aspect has not been captured in the scope of normative phase in Rel-18, the continuation is expected in Rel-19. More specifically,

- For DL power control:

As justified in Rel-18, for the Case-1 shown in Figure 1, the proper power allocation for the DL of access link is expected to mitigate the cross-interference. Meanwhile, compared to the fixed power allocation via OMA-based or network-planning based approach, the power control based on the SCI can provide more flexibility on the deployment with limited restriction.

In addition, for the Case-2, support of the DL power control is also beneficial to enable the detection and mitigation of self-interference/excitation. For example, once the detected signal strength at either UE or MT-site is unchanged regardless of the allocated DL power of access link, the occurs of self-interference can be identified.

- For UL power control:

Except for the interference mitigation, as shown in Figure 2, the discussion on the power sharing between the UL of  C-link and backhaul links is also needed, especially in case that simultaneous operation of these two links are supported as UE capability (e.g., Note-2: Simultaneous transmission of the UL of C-link and UL of backhaul link is subject to NCR’s capability).

Figure 1 DL PC with two typical usages

Figure 2 UL PC: Power sharing among C-link and Backhaul link

Observation 1: The support of power control (i.e., Both DL and UL power control) in Rel-19 NCR is beneficial and necessary.

  - Out-of-band control

As mentioned above, in Rel-18, the NCR is assumed in in-band operation mode with following agreement:

Agreement (SI phase in RAN1#109) Capture the following assumption of network-controlled repeater in TR 38.867. - At least one of the NCR-MT’s carrier(s) should be within the set of carriers forwarded by the NCR-Fwd in same frequency range. - The NCR-MT and NCR-Fwd operating in the same carrier is prioritized for the study.

Then, in Rel-19, to enable the flexible scheduling, the Out-of-band control should be considered at least for the typical scenario shown in Figure 3. More specifically, to support the either simultaneously or TDM-ed forwarding over different CCs, the corresponding SCI information (e.g., beam, TDD configuration or timing) associated with target frequency information can be considered, i.e., frequency-selective scheduling.

Figure 3 Illustration of Out-of-band control with multi-carrier in same frequency range

Meanwhile, this case can be easily extended to support the simultaneously multiple beams operations if the antenna of NCR-Fwd is shared over different CCs. In this situation, with support of multi-beam operation, the overhead for the forwarding of common channel can be further optimized.

Additionally, for the Het-Net deployment with FR1-and FR-2, the controlling of forwarding behavior for FR2 can be conducted over the FR1 to ensure the robustness of C-link. To support this application, either the NCR-MT with DC (FR-1 and FR-2) capability or FR-1 only can be considered as baseline for study.

Observation 2: The support of Out-of-band control (i.e., multi-carrier operation at NCR-Fwd in same or different frequency range) in Rel-19 NCR is beneficial to extend the deployment of NCR.

Observation 3: The support of frequency-selective scheduling and simultaneously multi-beams operation can be achieved via the discussion for Out-of-band control.

  - Others

Except for the above aspects, some new directions have also been identified to boost the usage of NCR, which can also be considered in Rel-19:

- Mobility of the NCR:

Although the NCR is assumed to be stationary in Rel-18, considering the variation of channel (e.g., NCR with lower height for O2I), the re-establishment of C-link with camping on the new carrier/cell can be considered.  In this case, C-link can be switched to another CC with better signal quality.

Meanwhile, once the movement of NCR can be considered in some cases, e.g., vehicle or high-speed train with future compatibility of RIS deployment, the normal mobility procedure, e.g., inter-CU mobility, should be considered along with the potential impacts on F1 interface for CU-DU split architecture.

- Measurement-assisted NCR operation:

As mentioned in [3], the additional information, e.g., existence of UEs within target areas, signal quality (e.g., SINR or RSRP) of backhaul link, can be reported to gNB based on the measurement at NCR-MT side. With support of this mechanism, the proper selection of active NCR for each area or UE can be supported with high flexibility on scheduling and lower interference.

- Terminal-based NCR-operation:

As the extension of conceptual of controllable RF to boost the signal for coverage extension, the terminal-based NCR-operation can also be considered as the cases shown in Figure 4. In these two cases, it can be found that the powerful device (e.g., mobile phone or laptop) can be taken as the controllable repeater to forward the signal from another device with lower RF capability (e.g., wearable device). Moreover, the conceptual mode of existing NCR can be easily applied to Case-1, and more studies on Case-2 is needed to enable the exchange the SCI information between device via existing sidelink.

Meanwhile, to avoid the complexity of support full duplex at UE-A side (i.e., act as “NCR”), the links between two UEs can be assumed in different frequency, e.g., carrier, compared to the link between device and gNB.

Figure 4 Exemplary of terminal-based NCR-operation

Observation 4: The support of additional aspects, e.g., Mobility of the NCR, Measurement-assisted NCR operation and Terminal-based NCR-operation, can be considered in Rel-19 NCR.

Then, based on the above discussion, similar to Rel-18, the NCR should be considered as the RAN1-led topic in Rel-19. Regarding the detailed workplan, the aforementioned objectives can be organized with different level of priorities.

Proposal 1: As the RAN1-led item, the following objectives are considered for Rel-19 NCR:

- Specify the Power control (i.e., both DL and UL power control)

- Specify the out of band control

  - Note: Both frequency-selective controlling and multiple-beams can be supported within the scope for out of band control.

- Additional aspects with study phase

  - Mobility of NCR

  - Measurement-assisted NCR operation

  - Terminal-based NCR-operation

- Conclusions

In this contribution, the scope of NCR in Rel-19 are elaborated with following observations and proposals:

Observation 1: The support of power control (i.e., Both DL and UL power control) in Rel-19 NCR is beneficial and necessary.

Observation 2: The support of Out-of-band control (i.e., multi-carrier operation at NCR-Fwd in same or different frequency range) in Rel-19 NCR is beneficial to extend the deployment of NCR.

Observation 3: The support of frequency-selective scheduling and simultaneously multi-beams operation can be achieved via the discussion for Out-of-band control.

Observation 4: The support of additional aspects, e.g., Mobility of the NCR, Measurement-assisted NCR operation and Terminal-based NCR-operation can be considered in Rel-19 NCR.

Proposal 1: As the RAN1-led item, the following objectives are considered for Rel-19 NCR:

- Specify the Power control (i.e., both DL and UL power control)

- Specify the out of band control

  - Note: Both frequency-selective controlling and multiple-beams can be supported within the scope for out of band control.

- Additional aspects with study phase

  - Mobility of NCR

  - Measurement-assisted NCR operation

  - Terminal-based NCR-operation

Reference

- RP-230175, WID on NR network-controlled repeaters

- RWS-230488, RAN Chairs summary of Rel-19 workshop

- RWS-230292, Views on NCR and RIS in Rel-19