# 3GPP TSG RAN Meeting #101			 					                       RP-232168

# Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.15

Source:	ZTE

Title:	Consideration on Continuous Data Collection for AI/ML RAN

Document for:		Discussion and Approval

- Introduction

At present, the AI/ML RAN work item (WI) is approaching its final stages of completion, with just two meetings remaining on the agenda. Up to this point, substantial progress has been achieved in addressing three key use cases that were given top priority: energy conservation, load balancing, and mobility optimization. Additionally, one critical topic that has been an ongoing point of discussion within the AI/ML RAN context is continuous data collection. Several companies have expressed their concerns and offered insights into enhancing the existing mechanism to realize continuous data collection.

In this contribution, we present our analysis of continuous data collection in the context of AI/ML RAN. Furthermore, we elaborate on the R19 objectives aimed at addressing this outstanding issue, which has carried over from R18.

- Discussion

Upon checking the objectives of R18 WI [1], potential MDT enhancement is included.

Normative work is based on the conclusions captured in TR37.817. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3) Note: On security impacts, coordination with SA3 when needed. On OAM aspects, coordination with SA5 when needed. Note: Specify new UE measurements when needed if any. Note: Specify MDT procedure enhancements when needed if any.

During the RAN3 meetings, various companies have acknowledged that the existing MDT framework serves as the baseline for data collection from UE. Given the unique requirements of AI/ML training and inference, the continuous nature of data collection from a UE becomes crucial. However, companies have identified a potential issue when it comes to transitions from the RRC_INACTIVE/IDLE state to the RRC_CONNECTED state. Specifically, the TCE is unable to distinguish between different measurement reports originating from the same UE during such transitions.

Within the RAN3 group, there is a common understanding regarding signaling-based MDT. In this context, TCE can effectively identify the UE using a UE identifier. However, for management-based MDT, it becomes challenging for TCE to identify the UE with a UE identifier that could establish a unique UE identity. This is because management-based MDT intentionally maintains anonymity for the UE.

Observation 1: The different measurement reports generated during transitions from the RRC_INACTIVE/IDLE state to the RRC_CONNECTED state cannot be identified as originating from the same UE.

To resolve this issue above, there seems several kinds of solutions on the table:

- Solution #1: Reuse Trace ID to identify the UE across various state transitions

- Solution #2: Allocate the persistent UEID from CN/AMF

- Solution #3: Reuse the method proposed in SON/MDT (UE pointer solution). Transmitting a 'continuous flag' to the UE. The UE will then store this indication and include it in the report sent back to the RAN.

- Solution #4: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information.

As for Solution #1, it is not a feasible approach due to the fact that the trace ID differs when transitioning from the RRC_INACTIVE/IDLE state to the RRC_CONNECTED state. The OAM still cannot understand that the measurement reports during RRC_INACTIVE/IDLE state and during the RRC_CONNECTED state is from the same UE.

Observation 2: Solution #1 is not a feasible way to resolve the issue on continuous data collection.

Regarding Solution #2, this approach is associated with the RAN-based solution. In this method, the AMF can assign a UEID (or a random ID) to the UE, which is then communicated to the RAN. Consequently, the RAN can effectively identify that the measurement report originates from the same UE, even when transitioning between RRC states. However, there are several drawbacks that still need to be addressed:

- The CN is required to consistently allocate a persistent UE ID for each UE. This allocation process can potentially place an additional burden on the CN or OAM.

- The use of a persistent UE ID may not be suitable for scenarios involving inter-AMF handovers, which still can not keep the continuous measurement report from the same UE.

Observation 3: The Solution #2, RAN-based solution, is not an ideal approach for resolving the issue of continuous data collection due to its significant burden on the CN and its infeasibility in scenarios involving inter-AMF handovers.

Regarding Solution #3, this approach involves the introduction of a 'continuous MDT' flag within the logged MDT configuration. When the UE transitions to idle mode, it collects the logged MDT measurements. Then, upon moving from the IDLE/INACTIVE to the CONNECTED state, the UE signals to the RAN, as part of the RRC Setup procedure, that it is configured for Continuous MDT. This enables the RAN to promptly configure the UE for Immediate MDT measurements.

However, it's worth noting that the same value of continuous MDT flag conflict arise in scenarios involving mobility, particularly when different vendor OAM are utilized. There may be concerns regarding the consistency of the configuration by the new RAN side compared to the previous configuration.

Observation 4: The Solution #3, a kind of UE-based solution, is also not an ideal approach for resolving the issue of continuous data collection.

For the solution #4, the approach is to introduce a "continuous MDT" indication, and the logged MDT configuration can also include a measurement duration parameter optionally. When the UE moves to idle mode, it can start collecting the logged MDT measurements based on this configuration. And then when the UE moves from IDLE/INACTIVE to CONNECTED, UE continues to collect the measurements by itself based on the configuration of measurement duration. Once the measurement duration has elapsed, the UE can stop collecting the measurements and report them to the RAN. In addition, supporting company brings another solution to support the continuous data collection.

Observation 4: The Solution #4 to introduce an indication in the logged MDT measurement configuration to indicate UE to let UE collect measurement across various RRC state is the most straightforward and simple solution with minor standard impact.

Several companies raised concerns that the approach of collecting continuous historical information for AI/ML model training and inference would burden the UE since it may store a large amount of measurement data. However, these concerns are not justified.

Firstly, the measurement duration is used to determine the time range for a measurement. This means that UE will not store a large quantity of measurements as the duration would specify the amount of data collected for each measurement. Therefore, this means that the UE will only store measurements for the measurement duration specified in the MDT configuration.

Secondly, this approach is based on logged measurements. When the UE enters the connected state, the measurements stored in the UE can be reported to the RAN directly via available indication to RAN, thus there is no buffer issue,, and if the continuous collection indication is available , the UE will continue to collect the measurements.

The only change is that the current logged MDT measurement should be allowed to be performed by UE in RRC Connected mode which is a stage2 enhancement, there is no additonal burben introduced in UE side compared with reusing current immediate MDT mechanism to perform the corresponding AI/ML related measurements.

Observation 5: The Solution #4 does not bring additional burdens to the UE compared with other options.

In the context of AI/ML-based use cases, it's crucial to emphasize that measurements from a single UE hold significant value. A data-set comprising complete and continuous measurements plays a pivotal role in training effective AI/ML models and generating informed decisions. Looking ahead, there will likely be additional UE measurements required to support various AI/ML-based use cases. While this may impose some limitations on the UE, the trade-off is generally acceptable. In Rel-18, the solution is trying to reusing the current mechanism such as MDT function. In order to improve the performance of AI/ML, it is reasonable to enhance MDT function in Rel-19.

Proposal 1: For the sake of AI/ML RAN function, it is allowed to enhance MDT functions to resolve the issue of continuous data collection in R19.

It is proposed to elaborate the objectives of AI RAN in R19, with the addition of any enhancement for continuous AI/ML related data collection from the same UE.

Specify the enhancements for the continuous AI/ML related data collection from the same UE. [RAN3, RAN2]

Proposal 2: The enhancements for the continuous data collection from the same UE can be further studied in RAN3 AI RAN in R19 SI+WI.

- Conclusion

Observation 1: The different measurement reports generated during transitions from the RRC_INACTIVE/IDLE state to the RRC_CONNECTED state cannot be identified as originating from the same UE.

Observation 2: Solution #1 is not a feasible way to resolve the issue on continuous data collection.

Observation 3: The Solution #2, RAN-based solution, is not an ideal approach for resolving the issue of continuous data collection due to its significant burden on the CN and its infeasibility in scenarios involving inter-AMF handovers.

Observation 4: The Solution #4 to introduce an indication in the logged MDT measurement configuration to indicate UE to let UE collect measurement across various RRC state is the most straightforward and simple solution with minor standard impact.

Observation 5: The Solution #4 does not bring additional burdens to the UE compared with other options.

Proposal 1: For the sake of AI/ML RAN function, it is allowed to enhance MDT functions to resolve the issue of continuous data collection in R19.

Proposal 2: The enhancements for the continuous data collection from the same UE can be further studied in RAN3 AI RAN in R19 SI+WI.

4. Reference

[1] RP-231150, Artificial Intelligence (AI)/ Machine Learning (ML) for NG-RAN, CMCC, Ericsson