3GPP TSG RAN Meeting #101                                              RP-232171

Bangalore, India, September 11-15, 2023

Source:	ZTE, Sanechips

Title:	ECP for NR MBS evolution in Rel-19

Agenda item:	8A.2.13.5

Document for:	Discussion

- Introduction

This paper provides our views on NR MBS evolution in Rel-19. More specifically, a) the motivations and potential issues on supporting ECP for the 15kHz and 30kHz subcarrier spacing and b) whether explicit MBSFN shall be supported are discussed.

- ECP support for NR MBS

- Motivations

According to previous releases of NR MBS, SFN-based transmission mechanism for MBS broadcast and multicast had already been supported by implementation for improving the reception performance especially for UEs in cell edge. However, the applicable area range of the SFN transmission mechanism is limited by the CP length.

Therefore, support of ECP is a way for improving the applicable area range as well as performance of the SFN-based MBS transmission mechanism.

Observation 1: Support of ECP for 15 kHz and 30 kHz subcarrier spacing is a way for improving the applicable area range as well as performance of the SFN-based MBS transmission mechanism.

- Potential issues

As described in WID of Rel-17 NR MBS [1], simultaneous operation of MBS and unicast reception is an objective of the WID. If ECP for 15 kHz and 30 kHz is supported for SFN-based MBS transmission, the coexistence mechanism among unicast and MBS will be an issue to be further considered. Some alternatives are listed as follows,

- Alt.1: ECP is supported for both of MBS and unicast transmission.

- Alt.2: ECP is only supported for MBS transmission, and TDM is applied for ECP MBS and NCP unicast simultaneous reception.

- Alt.3: ECP is only supported for MBS transmission, and FDM is applied for ECP MBS and NCP unicast simultaneous reception.

Regarding Alt.1, the gNB can configure a common frequency resource (CFR) under a DL BWP with ECP to support both of unicast and MBS transmission with ECP. It may be a simplest and most direct way, which only requires minor standardization efforts. However, some cell specific information such as SSB, SIB, etc., in initial DL BWP has to be transmitted in NCP considering backward compatibility issue. Therefore, at least initial DL BWP and broadcast CFR associated with the initial DL BWP cannot be configured as ECP. And Alt.1 may only be applicable and beneficial to multicast transmission.

For Alt.2, this is similar to MBSFN subframe in LTE eMBMS, that is, specific time domain resources may be configured as ECP for SFN-based MBS transmission, while other time domain resources with NCP. This alternative may only require minor RAN1 standardization efforts on defining the time domain pattern and minor RAN4 standardization efforts on evaluation of switching delay between transmissions with ECP and NCP.

Regarding Alt.3, it requires a revisit of legacy concept. For example, CFR definition, i.e., do we apply different CP types for CFR and its associated DL BWP? Moreover, the CFR in such circumstances is more of an independent DL BWP, and it requires UE to support 'dual active' DL BWPs if a UE is to simultaneously receive MBS and unicast with different CP lengths. Other subsequent impacts need further investigation.

Observation 2: Different co-existence schemes brings different impacts and needs further study. And TDM manner, which is similar to that in LTE eMBMS, has a relatively less impact on standardization and better compatibility.

Proposal 1: If NR MBS is to be further enhanced in Rel-19, support ECP for 15 kHz and 30 kHz SCS for MBS transmission.

- Specify the coexistence mechanism among unicast and MBS transmission by at least supporting TDM among ECP MBS and NCP unicast transmission. [RAN1, RAN4]

- On SFN

In Rel-17, SFN is only supported implicitly, i.e., the SFN operation including any related synchronization is only achieved by network implementation and transparent to UE. Meanwhile, any design decisions taken for Rel-17 shall not prevent introducing the following features in future Releases, e.g., standardized support of SFN over multiple cells above gNB-DU level. However, if ECP is to be introduced, does it mean we will have complete support of SFN transmissions?

A complete support of SFN transmission above gNB-DU level requires a systematic support, including at least the following:

- SYNC protocol is needed. Depending on the level of synchronization, inter gNB-DUs or inter gNBs, the user plane needs to be updated to support the SYNC protocol. This will need RAN2/SA2 support, depending on the ambition level.

- A central scheduling entity, similar to legacy MCE (Multicell/Multicast Coordination Entity), is needed. Again, depending on the level of synchronization, this entity could be residing inside gNB or an entity above gNB level. This will need an architectural update and huge impacts to RAN3 on the interface, either introducing new interface or enhancing existing interface, e.g., NGAP or F1AP.

- Last but not least, MBSFN transmission in LTE eMBMS relies on the definition of new physical channel (i.e., PMCH) to really work. Different from PDSCH, PMCH is a semi-static scheduling that works with SYNC protocol and cross cell centralized coordination. Although much can be borrowed from LTE, it might bring much RAN1 impact.

Observation 3: MBSFN transmission above gNB-DU or gNB level brings systematic impacts to multiple WGs.

On the requirement, large-scale SFN networks is not clear based on our observation. And there are feasibility issues, e.g., on how to tackle the interference and coverage issues. Based on above, we suggest no MBSFN transmission related work in Rel-19.

Proposal 2: No MBSFN transmission related work in Rel-19.

- Conclusion

In this discussion paper, we focused on the ECP and MBSFN transmission aspects based on the feedback we have observed from contributions in the Workshop for Rel-19. We further suggest that, if ECP is to be supported, RAN should further investigate on the co-existence issues if new ECP is to be introduced and how UE can support simultaneous ECP based MBS and unicast reception. We have the following observations and proposals:

Observation 1: Support of ECP for 15 kHz and 30 kHz subcarrier spacing is a way for improving the applicable area range as well as performance of the SFN-based MBS transmission mechanism.

Observation 2: Different co-existence schemes brings different impacts and needs further study. And TDM manner, which is similar to that in LTE eMBMS, has a relatively less impact on standardization and better compatibility.

Proposal 1: If NR MBS is to be further enhanced in Rel-19, support ECP for 15 kHz and 30 kHz SCS for MBS transmission.

- Specify the coexistence mechanism among unicast and MBS transmission by at least supporting TDM among ECP MBS and NCP unicast transmission. [RAN1, RAN4]

Observation 3: MBSFN transmission above gNB-DU or gNB level brings systematic impacts to multiple WGs.

Proposal 2: No MBSFN transmission related work in Rel-19.

- Reference

- RP-201038, ‘WID revision NR Multicast and Broadcast Services’, Huawei, HiSilicon.