3GPP TSG RAN Meeting #101						              RP-232177

Bangalore, India, 11 – 15 September 2023

Source: 	ZTE Corporation, Sanechips, Intel Corporation, LG Electronics, CATT

Title: 	SDT enhancements in Rel-19

Agenda item:	8A.2.15

Document for: 	Discussion and Decision

- Introduction

Inactive state in NR has been mandatory since Rel-15. However, network deployments using NR SA do not implement or use the INACTIVE state in field until recently because the UEs typically generate small and infrequent data packets most of the time which keeps the UEs in connected state. As result, most NR networks (if not all) have basically skipped usage of INACTIVE state in the field.

In Rel-17 SDT feature has been added to the INACTIVE state and as a result INACTIVE state is now seen as more practical in real deployments. In this contribution we propose SDT enhancements to enable the usage of INACTIVE state in a more practical way so that more and more networks can make use of the INACTIVE state in real deployments.

- Justification SDT enhancements

Small and infrequent data transmissions are very typical in mobile networks. It can be seen from the traffic traces of the light background traffic in Figure 1 below that small and infrequent data traffic is prevalent in networks.

| Figure 1: Light background traffic packet distribution in mobile networks including results of multiple traces provided by different companies (source 3GPP TR 36.822) |  |
| --- | --- |

Inactive state which is mandatory since Rel-15 is seldom used in real networks as data transfer within INACTIVE state is not feasible until Rel-17. As a result, operators deploying 5GSA are only thinking about using the INACTIVE state in real deployments only after sufficient penetration of the UEs supporting SDT feature.

However, as SDT feature becomes more prevalent, the network capacity to support INACTIVE state soon starts to become the bottleneck. Further, with capacity problems, we also need to focus on the optimal usage of the network resources in INACTIVE state. In Rel-17 (and in Rel-18), the SDT feature has the following limitations:

Limitations of SDT (Rel-17/18)

- SDT is only applicable to initial BWP

  - SDT transmissions are limited to initial BWP and this is done to keep the SDT procedure as close as possible to initial access mechanisms (upon which SDT feature is built on). As more and more UEs support SDT (and hence the INACTIVE state), congestion on the initial BWP becomes a bottleneck. Furthermore, initial BWP is also used for IDLE mode UEs and hence camping SDT UEs always on initial BWP may not be optimal for the IDLE mode UEs either as congestion on this BWP will impact the quality of service for these UEs. Hence this restriction will severely restrict the practicality of supporting a large number of UEs in INACTIVE state in the network.

- No error recovery during SDT

  - During SDT if any error occurs, UE moves to IDLE mode and performs NAS recovery. This procedure is expensive in terms of overhead and incurs an additional latency if such error happens. As such, any mechanism to prevent such errors or any mechanism to recover efficiently from such errors would be beneficial from system perspective.

Companies seem to be interested in general in pursuing such enhancements as can be seen from contributions to the Rel-19 work shop [1][2][3]. However, companies also proposed such enhancements in RAN2 (either as part of TEI or as part of SDT Rel-18/Rel-17 WIs) but were not pursued due to lack of time [4]. It is hence expected that pursuing such enhancements would be beneficial in general given the need to support INACTIVE state in real deployments.

- Objectives for SDT enhancements in Rel-19

Based on the above discussion, we propose the following objectives for SDT in Rel-19

- Objective 1: Specify mechanisms to enable SDT operation (for both MO-SDT and MT-SDT) on non-initial BWP [RAN2]

- Objective 2: Specify mechanisms for reducing the SDT failure and mechanisms for efficient recovery from SDT failure [RAN2]

- Conclusion and proposals

In this contribution we highlight some limitations of SDT until Rel-18 and propose WI with the following objectives to overcome these limitations:

- Objective 1: Specify mechanisms to enable SDT operation (for both MO-SDT and MT-SDT) on non-initial BWP [RAN2]

- Objective 2: Specify mechanisms for reducing the SDT failure and mechanisms for efficient recovery from SDT failure [RAN2]

It is expected that this work would need around 0.5 TUs spread over 6 RAN2 meetings. i.e. a total of 3 TUs in Rel-19.

- References

- RWS-230201	Proposal on SDT enhancement in Rel-19	LG Electronics

- RWS-230297	Views on SDT enhancement in Rel-19	ZTE, Sanechips

- RWS-230486	SDT enhancements in Rel-19	Intel

- R2-2307934	Handling BWP restrictions in MT-SDT	Ericsson, T-Mobile USA, Deutsche Telekom	discussion