3GPP TSG RAN Meeting #101			     		                  	RP-232209

Bangalore, India, September 11-15, 2023

Agenda Item:	8A.2.6

Source:	CMCC

Title:	Considerations on mobility enhancements for Rel-19

Document for:	Discussion and Decision

- Introduction

Mobility is one of the key technologies to provide continues service. Multiple mechanisms are introduced to satisfy different requirements and applicable to different scenarios. In Rel-16, conditional handover (CHO) is introduced to improve the robustness. In Rel-18, LTM have been supported to reduce the latency brought by L1/L2 mechanism with baseline functions. In addition, only selective activation of SCG (i.e., subsequent CPAC) is supported and the selective activation of MCG is excluded due to limited TU. In this contribution, we tend to discuss the potential objects for further enhancement of mobility management.

- Discussion

During the initial version of 5G, the basic mobility mechanism is introduced to support basic L3 handover. In Rel-16, conditional handover (CHO) is introduced to improve the robustness without reducing HO interruption. To reduce HO interruption, dual active protocol stacks (DAPS) HO is another mobility enhancement but requires higher UE capabilities and applied in limited scenario (i.e., only PCell, not FR2 to FR2). In Rel-17, inter-cell beam management (ICBM) is studied and can only be applied intra-DU scenario. Based on the ICBM mechanism, UE utilizes the beam from the neighbour cell without performing serving cell change. For Rel-18, LTM is specified as the NW-controlled HO mechanism based on L1 measurement and triggered by MAC CE for lower HO interruption time. But Rel-18 LTM is only applicable to intra-CU scenario and the issues about robustness need to be further considered.

2.1 LTM relate enhancement

In Rel-18, the applicable scenario for LTM has been discussed and is limited to intra-CU scenario. This limitation means that UE still needs to switch to L3 HO mechanisms when handover to another gNB. However, inter-CU LTM is extremely important to support high throughput services like XR/cloud game in the outdoor. Continuous and low-interruption service are needed in the inter-gNB handover as well. Thus, the applicable scenario of LTM needs to be extended to inter-CU scenario to have a unified HO solution with low latency, overhead and interruption time. Additional Xn interaction to enable inter-CU LTM should be specified.

Proposal 1: Inter-CU LTM is supported in the Rel-19 mobility enhancement to reduce the latency, signalling overhead and interruption time.

When the UE moves across cells, it can experience very fast signal degradation especially for FR2. To improve the robustness for cell switch, CHO is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The main object for the Rel-18 LTM is to reduce the handover latency. L1 measurement report includes the measurement results of neighbour cells without enhancement on the control of L1 measurement/report. It is a fact that UE is always the 1st entity aware of the change of channel conditions or the change of the radio qualities between UE and serving cell or candidate cells. The L1 measurement result may change rapidly, especially for FR2. Since robustness is one of the key requirements for cell switch, UE should have the opportunity to trigger the LTM similar to CHO. That is, conditional LTM could reduce the risk that the UE couldn’t receive the NW command due to bad channel conditions.

Proposal 2: Conditional LTM is supported in Rel-19 to improve the cell switch robustness.

2.2 TRS tracking and CSI acquisition

Although RAN1 has finished the WI of LTM [1] in RAN1#114 meeting, some useful functions which can reduce the interruption delay of cell switching are not supported in Rel-18.

RAN1#113 [2] Conclusion There is no consensus to support the following procedures prior to the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM - CSI acquisition for candidate before reception of cell switch command Note: At least for the candidate cells which are current serving cells, the CSI acquisition prior to cell switch command will be supported. Conclusion There is no consensus to introduce additional mechanism to support the following procedures prior to and joint with the reception of L1/L2 cell switch command aiming at the reduction of handover delay/interruption in Rel-18 LTM - TRS tracking for candidate cells FFS: Whether/How the QCL reference information of TCI states of the candidate cell should be mapped to the source SSB. Note: At least for the candidate cells which are current serving cells, TRS tracking prior to cell switch command is supported.

Based on the supported features of Rel-18 LTM, UE can acquire the DL synchronization with SSB and TA before the cell switching. But if only the SSB based synchronization is supported as a coarse synchronization, it is still very limited for UE to communicate with the target cell after cell switching. Without finer synchronization with the TRS from target cell, UE cannot be scheduled with higher modulations as in connected mode. On the other side, without CSI acquisition, gNB has no knowledge of the CSI of the UE and also cannot make a valid scheduling decision for the UE. Only a lower or the lowest MCS can be scheduled to the UE. In summary, without the TRS tracking and CSI acquisition before the cell switch, UE cannot have high date rate transmission with the target cell immediately. UE has to track the TRS of the target cell over multiple samples to acquire the finer synchronization and still needs some measurements for the CSI reporting. Additional delay would be introduced before gNB can actually schedule the UE after cell switching. TRS tracking (if not supported in Rel-18) and CSI acquisition before cell switching should be supported in the Rel-19 mobility enhancement to reduce the interruption time during the cell switching.

Observation 1: Without finer synchronization of TRS from target cell, UE cannot be scheduled with higher modulations.

Proposal 3: TRS tracking (if not supported in Rel-18) is supported in the Rel-19 mobility enhancement to reduce the interruption time during the cell switching.

Observation 2: Without CSI feedback, gNB can only schedule the UE in a conservative way with lower or the lowest MCS.

Proposal 4: CSI acquisition before cell switching is supported in the Rel-19 mobility enhancement to reduce the interruption time during the cell switching.

2.3 Enhancement on UE triggered mechanisms

In Rel-18 LTM, only gNB configured L1 reporting is supported. Periodic, semi-persistent and aperiodic reporting are supported as legacy. Index and the L1-RSRP of the reference signal (RS) are reported by UE and the number of the reported RS is N*M (N for configured cell number and M for configured beam number per cell). That is, no matter if there are enough RS(s) worth to report, the number of reported RS is fixed. The current NW-configured mechanism brings a large signaling overhead since unnecessary beam reports are triggered. For the L1 measurement report mechanism, it is a trade-off between the reporting frequency and signalling overheads. If gNB wants to get the knowledge of link quality timely, more frequent reporting needs to be configured, which induces more signaling overheads and power consumption of UEs. On the other side, if the periodicity of the reporting is large, gNB may not acquire the change of link quality timely and the cell switch command could not be transmitted to the UE when the link goes down.

Observation 4: Signalling overhead and UE power consumption resulted from L1 measurement report need to be reduced.

In R18, some companies proposed to introduce L1 measurement event to improve the robustness but haven’t discussed the detail due to the lack of time [2]. From our understanding, it can be further considered in Rel-19 mobility enhancements.

Conclusion No consensus to introduce UE/event triggered report for L1 measurement results for LTM in Rel-18.

Thus, a new beam report mechanism should be considered to avoid the unnecessary L1 measurement report to reduce the signalling burden and UE power consumption.

Proposal 5: A event-triggered L1 measurement report is supported in Rel-19 to reduce the signalling overhead and UE power consumption.

2.3 Failure handling enhancement for LTM

In Rel-18, it is agreed to support CHO-like fast recovery for LTM without enhancement due to limited time.

RAN2#123 [3] - Upon an LTM cell switch failure (i.e., supervision timer expiry) or RLF, fast recovery similar to CHO: - UE performs cell selection. - If selected cell is an LTM candidate cell, UE performs RACH-based LTM cell switch on the selected cell (network-controlled). - If selected cell is not an LTM candidate cell, UE transmits RRC re-establishment request.

Upon initiation of the recovery procedure, the UE shall select the recovery cell in accordance with the cell selection process as specified in TS 38.304. The criterion for the selection is mainly about the channel condition. After cell selection while T311 is running, UE attempts LTM if the selected cell is one of the candidate cells. However, unlike CHO, LTM aiming to minimum latency requires lots of pre-operations (e.g., UL synchronization, DL synchronization, CFRA resources, etc.). To enable fast LTM based recovery, the S-measurement criterion should not be the only factor to consider when selecting the recovery cell.

One potential approach is that UE-based configuration application with enhancement on selection criterion. UE could select the suitable LTM candidate based on the information from measurement results as well as whether it has performed UL synchronization, DL synchronization, CFRA resources for the candidate cells. Besides, since NW generates the LTM configurations and triggers cell switch by the command, NW has the best knowledge of both the UE condition (e.g., L1 measurement report, DL measurement configuration, etc.) and NW condition (e.g., load balancing, candidate configurations become invalid). Another approach is the NW controlled LTM recovery. Based on the known information mentioned above, NW could provide the cell list within the switch command, e.g., in the order recommended by the NW. UE performs the LTM execution to the first cell in the list. Upon failure, UE could attempt LTM to the following cells in order.

Proposal 6: Failure handling enhancement for LTM is supported in the Rel-19 mobility enhancement.

- Conclusions

In this contribution, we discuss the potential enhancement for mobility in Rel-19. Following observations and proposals are made in this contribution:

Proposal 1: Inter-CU LTM is supported in the Rel-19 mobility enhancement to reduce the latency, signalling overhead and interruption time.

Proposal 2: Conditional LTM is supported in Rel-19 to improve the cell switch robustness.

Observation 1: Without finer synchronization of TRS from target cell, UE cannot be scheduled with higher modulations.

Proposal 3: TRS tracking (if not supported in Rel-18) is supported in the Rel-19 mobility enhancement to reduce the interruption time during the cell switching.

Observation 2: Without CSI feedback, gNB can only schedule the UE in a conservative way with lower or the lowest MCS.

Proposal 4: CSI acquisition before cell switching is supported in the Rel-19 mobility enhancement to reduce the interruption time during the cell switching.

Observation 3: Signalling overhead and UE power consumption resulted from L1 measurement report need to be reduced.

Proposal 5: A event-triggered L1 measurement report is supported in Rel-19 to reduce the signalling overhead and UE power consumption.

Proposal 6: Failure handling enhancement for LTM is supported in the Rel-19 mobility enhancement.

- References

- RP-231475, Revised WID on Further NR mobility enhancements, 3GPP TSG RAN Meeting #100, Taipei, June 12-14, 2023

- Chair’s notes, 3GPP TSG RAN WG1 #113, Incheon, Korea, May 22nd – May 26th, 2023

- Chair’s notes, 3GPP TSG RAN WG2 #123, Toulouse, France, August 21-25, 2023