3GPP TSG-RAN Meeting #101             	   				    RP-232299

Bangalore, India, September 11-15, 2023

Agenda Item:		8A.2.7

Source:			Reliance Jio

Title:	Evolution of NTN for Release19

Document for:	 	Discussion

# Introduction

During the latest RAN Release 19 workshop, NTN evolution garnered significant attention and was noted in the officially endorsed summary by the RAN chair [1] as one of the topics that drew substantial interest from participating companies. Below, the summary of the discussion towards NTN evolution as recored in chairman’s report:

- Strong desire to further evolve Non-Terrestrial networks (NTN) in both NR and Internet of Things (IoT) areas

  - The proposed possible areas for enhancements are quite broad and heavy

- Need to focus on the critical needs for commercial deployments and maintain a reasonable workload

  - Especially considering it’s a continuation from previous releases

It could be seen that there is special interest among Telecom industry to focus the future NTN evolution work towards focus on topics which have potential to pave way for NTN commercial deployment in near future.

In this contribution, we provide our views to address concern and suggest the topics which can be focussed upon in Release 19 to ensure that the focus is commercial deployment centric.

# Discussion

  - Uplink Capacity Enhancement for NR-NTN Satellite Networks

In the context of 3GPP Release 18, efforts have been made to improve uplink (UL) coverage in specific scenarios, primarily catering to 3 kbps UL low data rate and AMR 4.75 kbps for VoIP services. However, these enhancements fall short of meeting the requirements for NR UL. Additionally, the available UL resources for physical channels supporting NR NTN up to Rel-18 are limited, restricting access for only a subset of user equipment (UEs) within a satellite's coverage area. This discussion paper addresses the pressing need for Uplink Capacity/Throughput Enhancement, driven by the following key factors:

- Lack of UL Link Budget: Despite some improvements introduced in Rel-18, the UL link budget remains constrained due to limitations in UE's transmit power and antenna gain. These enhancements in Rel-18 offer support for only minimal data rates.

- Wide Coverage of NTN Satellites: NTN satellites offer extensive coverage, and when considering device density and 5G key performance indicators (KPIs), it is anticipated that a substantial number of UEs will fall within the satellite's coverage area. Ensuring successful data transmission by a large number of NR-NTN/IoT devices over NTN UEs while a low-orbit satellite passes necessitates swift access to satellite resources.

2.1.1 Proposed Enhancements

The following enhancements can be considered as part of Release19 NTN evolution:

- Enhanced UL Data Rate:

Aim to achieve UL data rates greater than 4.5 to 7 kbps for Physical Uplink Shared Channel (PUSCH) through handheld devices. Explore options such as increased repetition schemes or equivalent techniques, depending on the physical channel.

- Augmented Uplink Access Capability:

Allocate additional uplink access resources to accommodate a growing number of UEs. Investigate potential new techniques to enhance the capability of initial access, such as Non-Orthogonal Multiple Access (NOMA) for initial access technology.

The overarching goals of Uplink Capacity/Throughput Enhancement are as follows:

- Support Enhanced Uplink Data Rates:

Facilitate improved uplink data rates for handheld UEs, meeting the evolving demands of applications and services.

- Efficient Uplink Access Scheme:

Develop an uplink access scheme capable of accommodating numerous UEs across the wide coverage area of NTN satellites with minimal delay, including Control Random Access Channel (RACH) procedures.

From deployment perspective the proposed enhancements outlined in this section are expected to benefit all NTN satellite operators operating within Frequency Range 1 (FR1). By addressing the challenges of uplink capacity and throughput, these improvements will contribute to the overall performance and efficiency of NR-NTN satellite networks in Rel-18.

  - Support of Store & Forward Satellite operation

Enlisted below are the benefits associated with enabling Store and Forward (S&F) Satellite operation with regenerative payload:

- Facilitating Delay-Tolerant IoT NTN Services: The incorporation of S&F operation enables the provision of delay-tolerant, non-real-time IoT NTN services in regions covered by the satellites, without the necessity of establishing NTN gateway infrastructure. These areas may include remote regions and mid-sea locations.

- Enhancing Cost-Effective Deployment: In addition to supporting intermittent coverage, the inclusion of S&F operation is crucial for cost-effective deployment of IoT NTN services. It facilitates the rapid operationalization of services in scenarios involving (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- Alignment with Non-3GPP Solutions: Embracing S&F capabilities brings the 3GPP IoT NTN solution in line with other non-3GPP solutions designed for massive satellite IoT, as these non-3GPP solutions inherently support S&F services.

Deployment Considerations: It's important to note that the absence of support for Store & Forward Satellite operation represents a current limitation in Rel-17 & 18 for IoT NTN via NGSO. All NGSO IoT NTN satellite operators are anticipated to incorporate Store & Forward operation to address this limitation effectively.

  - Regenerative communication payload architecture

Incorporating a Regenerative Communication payload on a satellite entails a comprehensive suite of functions. This includes RF filtering, frequency conversion, amplification, as well as demodulation, decoding, switching, routing, and coding/modulation operations. Essentially, this equates to the integration of some or all of the gNB (5G New Radio gNodeB) functions directly within the satellite infrastructure.

This architecture aligns with the concept of regenerative satellite-based communication, where the NTN (Non-Terrestrial Network) platform possesses onboard processing capabilities for generating and receiving NR (New Radio) signals to and from the NTN terminal. The NR-Uu interface operates along the service link connecting the NTN terminal and the NTN platform. The radio interface between the NTN platform and the 5G Core Network (5GC) occurs via NG (Next-Generation) technology, traversing the SRI (Satellite Return Link Interface) in the air path between the NTN.

During discussions across various workgroups within 3GPP for Release 17, a decision was reached to standardize the transparent payload option, while the regenerative payload option was not selected for standardization in Release-17. With the advent of 3GPP Release-18, three regenerative payload options are being considered:

- Complete gNB Integration Onboard Satellite: This approach entails the full integration of gNB functionalities within the satellite.

- gNB-CU on Ground, gNB-DU Onboard Satellite: This option involves placing the gNB-Central Unit (CU) on the ground and the gNB-Distributed Unit (DU) onboard the satellite. However, challenges arise in managing the F1 interface between CU and DU in this configuration.

- Complete gNB on Ground: In this scenario, the entire gNB functionality is situated on the ground rather than onboard the satellite.

The adoption of this new architecture is expected to have a particularly positive impact on several deployment scenarios:

- Support for Delay-Sensitive and Delay-Tolerant Services: Especially in regions lacking NTN gateways, whether they are not yet deployed or temporarily non-operational.

- Flexibility in Ground Segment/NTN Gateway Deployment: The architecture offers flexibility in aligning ground segment and NTN gateway deployment with the space segment, as well as adapting to feeder link service availability issues.

- Traffic Routing Flexibility: It provides the flexibility to route traffic generated by groups of user equipment to specific feeder links, in accordance with national regulations and sovereignty requirements.

- Direct Satellite Communication: The architecture enables communications (unicast, multicast, and broadcast) between two or more UEs via satellite without necessitating routing through the ground network (loopback).

- Edge Computing Support: The system is capable of supporting edge computing onboard the satellite.

From a deployment perspective, it's worth noting that nearly all NGSO (Non-Geostationary Satellite Orbit) constellations planned for deployment over the next decade will incorporate inter-satellite links and therefore implement regenerative payloads. This approach is favored as it enables coverage in areas where feeder links are absent. To optimize cost-effectiveness, most satellites will be designed to incorporate only a subset of NG-RAN (Next-Generation Radio Access Network) functions.

# Conclusions

We have highlighted the potential NTN enhancement topics that may bring value in terms of practical deployment gains for NTN networks.

Following proposals can be considered for NTN WI in Release 19 -

Proposal 1: Consider the following topics as part of a NR-NTN-evolution work item:

- Uplink Capacity/Throughput Enhancement

- Support of Regenerative payloads

Proposal 2: Consider the following topics as part of a IoT-NTN-evolution work item:

- Support of S&F Satellite operation

# References

[1]	RWS-230488, RAN Chair's summary of Rel-19 workshop, RAN Chair (Qualcomm)