## Characteristics template for 3GPP 5G E-UTRA satellite access SRIT (Release 17 and beyond)

The description templates provided by 3GPP are for the description of the characteristics of 5G satellite access  developed by 3GPP. It includes one characteristics template for SRIT (encompassing NR satellite access and E-UTRA satellite access), and one characteristics template for NR satellite access RIT.

This document provides the characteristics template for the description of the characteristics of the E-UTRA satellite access RIT based on 3GPP Rel-17 work.

For this characteristics template, 3GPP has addressed all of the characteristics, and it is expected that these descriptions are helpful to assist in evaluation activities for independent evaluation groups, as well as to facilitate the understanding of the state-of-art of 3GPP development on the RIT.

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.1 | Test environment(s) |
| 8.2.5.1.1.1 | What test environments does this technology description template address? This proposal addresses all the three test environments across the three usage scenarios (eMBB-s, mMTC-s, and HRC-s) as described in Report ITU-R M.2514-0. |
| 8.2.5.1.2 | Radio interface functional aspects |
| 8.2.5.1.2.1 | Multiple access schemes Which access scheme(s) does the proposal use: TDMA, FDMA, CDMA, OFDMA, IDMA, SDMA, hybrid, or another? Describe in detail the multiple access schemes employed with their main parameters. For E-UTRA satellite access component RIT: For eMTC downlink and Uplink: The multiple access is a combination of - OFDMA: Synchronous/scheduling-based is supported for both DL and UL; the transmission to/from different UEs uses mutually orthogonal frequency assignments. Granularity in frequency assignment: For the UL: 3, 6 or 12 sub-carriers with a sub-carrier spacing of 15 kHz. For the DL: One resource block consisting of 12 subcarriers. Sub-carrier spacings of 15kHz is supported (see Item 5.2.3.2.7 and reference therein). - CP-OFDM is applied for downlink. DFT-spread OFDM is applied for uplink. - TDMA: Transmission to/from different UEs with separation in time. Granularity: One subframe of length 1 ms, (for more details on the frame structure, see Item 8.2.5.1.7 and the references therein). Repetition of a transmission is supported. - CDMA: Inter-cell interference suppressed by processing gain of channel coding allowing for a frequency reuse of one (for more details on channel-coding, see Item 8.2.5.1.2.3 and the reference therein). For NB-IoT, the multiple access is a combination of OFDMA, TDMA and CDMA, where OFDMA and TDMA are as follows - OFDMA: - UL: DFT-spread OFDM. Granularity in frequency domain: A single sub-carrier with either 3.75 kHz or 15 kHz sub-carrier spacing, or 3, 6, or 12 sub-carriers with a sub-carrier spacing of 15 kHz. A resource block consists of 12 sub-carriers with 15 kHz sub-carrier spacing, or 48 sub-carriers with 3.75 kHz sub-carrier spacing → 180 kHz. - DL: Granularity in frequency domain: one resource block consisting of 6 or 12 subcarriers with 15 kHz sub-carrier spacing→90 or 180 kHz - TDMA: Transmission to/from different UEs with separation in time - UL: Granularity: One resource unit of 1 ms, 2 ms, 4 ms, 8 ms, with 15 kHz sub-carrier spacing, depending on allocated number of sub-carrier(s); or 32 ms with 3.75 kHz sub-carrier spacing (for more details on the frame structure, see Item 8.2.5.1.7 and the references therein) - DL: Granularity: One resource unit (subframe) of length 1 ms. Repetition of a transmission is supported. |
| 8.2.5.1.2.2 | Modulation scheme |
| 8.2.5.1.2.2.1 | What is the baseband modulation scheme? If both data modulation and spreading modulation are required, describe in detail. Describe the modulation scheme employed for data and control information. What is the symbol rate after modulation? For E-UTRA satellite access component RIT: For eMTC downlink: - For both data and higher-layer control information: QPSK, 16QAM, 64QAM (see [36.211] sub-clause 6.3.2). - L1/L2 control: QPSK (see [36.211] sub-clauses 6.7.2, 6.8.3, and 6.8A.3) - Symbol rate: 168ksymbols/s per 180kHz resource block For eMTC uplink: - For both data and higher-layer control information: π/2-BPSK, QPSK and 16QAM are supported (see [36.211] sub-clause 5.3.2). - L1/L2 control: BPSK, QPSK (see [36.211] sub-clause 5.4) - Symbol rate: 168ksymbols/s per 180kHz resource block. For UL, less than one resource block may be allocated. For NB-IoT, the modulation scheme is as follows. - Data and higher-layer control: π/2-BPSK (uplink only), π/4-QPSK (uplink only), QPSK - L1/L2 control: π/2-BPSK (uplink), QPSK (downlink) - Symbol rate: 168 ksymbols/s per 180 kHz resource block. For UL, less than one resource block may be allocated. |
| 8.2.5.1.2.2.2 | PAPR What is the RF peak to average power ratio after baseband filtering (dB)? Describe the PAPR (peak-to-average power ratio) reduction algorithms if they are used in the proposed RIT. The PAPR depends on the waveform and the number of component carriers. The single component carrier transmission is assumed herein when providing the PAPR. For DFT-spread OFDM, PAPR would depend on modulation scheme as well. For uplink using DFT-spread OFDM, the cubic metric (CM) can also be used as one of the methods of predicting the power de-rating from signal modulation characteristics, if needed. For E-UTRA satellite access component RIT: For eMTC downlink: The PAPR is 8.4dB (99.9%). For eMTC uplink: - For DFT-spread OFDM: The PAPR is provided in the table below. Modulation; π/2 BPSK; QPSK; 16QAM. PAPR (99.9%); 0.3 dB; 5.8 dB; 6.5 dB. CM (99.9%); 1.2 dB; 2.1 dB. For NB-IoT, - Downlink: The PAPR is 8.0dB (99.9%) on 180kHz resource. - Uplink: The PAPR is 0.23 – 5.6 dB (99.9 %) depending on sub-carriers allocated for available NB-IoT UL modulation. PAPR-reduction algorithm for E-UTRA satellite access: Any PAPR-reduction algorithm is transmitter-implementation specific for uplink and downlink. |
| 8.2.5.1.2.3 | Error control coding scheme and interleaving |
| 8.2.5.1.2.3.1 | Provide details of error control coding scheme for both downlink and uplink? For example: – FEC or other schemes? – Unequal error protection? Explain the decoding mechanism employed. For E-UTRA satellite access component RIT: For eMTC downlink and uplink: - For data: Rate 1/3 Turbo coding, combined with rate matching based on puncturing/repetition to achieve a desired overall code rate. One transport block can be mapped to one or multiple resource units. (For more details, see [36.212] sub-clauses 5.1.3.2) - For L1/L2 control: Rate-1/3 tail-biting convolutional coding. Special block codes for some L1/L2 control signaling (For more details, see [36.212] sub-clauses 5.1.3.1) For NB-IoT, the coding scheme is as follows: - For data: Rate 1/3 Turbo coding in UL, and rate-1/3 tail-biting convolutional coding in DL, each combined with rate matching based on puncturing/repetition to achieve a desired overall code rate; one transport block can be mapped to one or multiple resource units (for more details, see [36.212] sub-clause 6.2) - For L1/L2 control: The same as above. Decoding schemes for E-UTRA satellite access: Decoding mechanism is receiver-implementation specific. Example of information on the decoding mechanism will be provided together with self evaluation. |
| 8.2.5.1.2.3.2 | Describe the bit interleaving scheme for both uplink and downlink. For E-UTRA satellite access component RIT: - Downlink and Uplink: Bit interleaving is performed as part of the encoding/rate-matching process, see [36.212] sub-clauses 5.1.3 and 5.1.4 for more details. Additional interleaving is performed in uplink, see [36.212] sub-clause 5.2.2.8 for more details. |
| 8.2.5.1.3 | Describe channel tracking capabilities (e.g. channel tracking algorithm, pilot symbol configuration, etc.) to accommodate rapidly changing delay spread profile For E-UTRA satellite access component RIT: - Downlink: - Cell-specific RS (CRS) are transmitted in every non-MBSFN subframe and over the entire frequency band. Up to four different CRS can be transmitted within a cell, with each CRS corresponding to one of up to four cell-specific antenna ports, referred to antenna port 0 to 3 respectively. The CRS can be used for downlink channel estimation for coherent demodulation of physical channels transmitted from antenna ports 0 to 3. The CRS can also be used to derive channel-state information (CSI) for the corresponding antenna ports. The CSI can e.g. be used to assist scheduling (including link adaptation, precoder-matrix/vector selection, etc.). For the detailed structure of CRS, see [36.211] sub-clause 6.10.1. - UE-specific RS and Demodulation Reference Signals can be used for downlink channel estimation for coherent demodulation of PDSCH (Physical Downlink Shared Channel) and MPDCCH (MTC Physical Downlink Shared Channel). In a given subframe, the UE-specific reference signals are only transmitted within the resource blocks that are used for PDSCH transmission to the specific UE within this subframe. For the detailed structure of UE-specific RS, see [36.211] sub-clause 6.10.3. The structure for the case of transmission from multiple antenna ports is an extension of the structure for the case of a single antenna port. - CSI-RS can be used for estimation of channel-state information (CSI) to further prepare feedback reporting to eNB (CQI for link adaptation, precoder- vector selection, etc.) to assist beamforming and scheduling. CSI-RS are transmitted periodically and aperiodically by the eNB. Periodic CSI-RS are transmitted in every Nth subframe, where N is configurable. - Narrowband Reference signals (NRS) are used in NB-IoT. NRS are transmitted in a certain minimum set of subframes, and additionally in a configured set of subframes. NRS associated with paging, random access response, and multicast transmissions on non-anchor NB-IoT carriers do not have to be transmitted on subframes far away from the associated transmissions, even if they are in the configured set of subframes. Up to two different NRS can be transmitted within a cell, with each NRS corresponding to one of up to two cell-specific antenna ports, referred to as antenna port 2000 to 2001 respectively. The NRS can be used for downlink channel estimation for coherent demodulation of physical channels transmitted from antenna ports 2000 to 2001. For the detailed structure of NRS, see [36.211] sub-clause 10.2.6. - Uplink: - Demodulation RS (DMRS) can be used for channel estimation for coherent demodulation of the Physical Uplink Shared Channel (PUSCH), and the Physical Uplink Control Channel (PUCCH). Uplink DMRS for demodulation of PUSCH are transmitted once every slot (twice every subframe) in the subframes in which PUSCH is being transmitted. Up to four uplink DMRS can be transmitted from a UE. The instantaneous bandwidth of the uplink DMRS equals the instantaneous bandwidth of the corresponding PUSCH transmission. - For the detailed structure of uplink DMRS for PUSCH transmission for the case of single antenna transmission, see [36.211] sub-clause 5.5.2.1. - For NB-IoT, uplink DMRS for demodulation of NPUSCH are transmitted once every slot (twice every subframe) in the subframes in which NPUSCH is being transmitted. The instantaneous bandwidth of the uplink DMRS equals the instantaneous bandwidth of the corresponding NPUSCH transmission. One DMRS for NPUSCH transmission can be transmitted from a UE. For the detailed structure of uplink DMRS for NPUSCH transmission, see [36.211] sub-clause 10.1.4. - Sounding RS (SRS) can be used for estimation of uplink channel-state information to assist uplink scheduling, uplink power control, and also assist downlink transmissions (e.g. downlink beamforming in scenarios with UL/DL reciprocity). Uplink SRS are transmitted either periodically every Nth subframe, where N is configurable, or aperiodically when triggered by the network. For the detailed structure of uplink SRS see [36.211] sub-clause 5.5.3. Uplink frequency and time domain pre-compensation enables the UE to adapt its uplink transmission to counter-act the effects of doppler and delay inherent in the uplink radio channel (which varies depending on the relative location and velocity between satellite and UE, and additional radio access delays inherent to the feeder link between satellite and ground station component of the Satellite Access node. The UE is provided with satellite ephemeris information and generates its own GNSS-acquired location to allow it to determine the level of uplink precompensation needed in terms of doppler and delay. The detailed behaviour for uplink precompensation is defined in [36.300] sub-clause 23.21.2.2. The support of uplink precompensation as well as other aspects inherent to the radio access design allows the system to cope with large ranges of doppler and delay channel characteristics between satellite and UE. Related information is provided in section 8.2.5.1.24 Details of channel-tracking/estimation algorithms are receiver-implementation specific, e.g. MMSE-based channel estimation with appropriate interpolation in time and frequency domain could be used. |
| 8.2.5.1.4 | Physical channel structure and multiplexing |
| 8.2.5.1.4.1 | What is the physical channel bit rate (Mbit/s) for supported bandwidths? i.e. the product of the modulation symbol rate (in symbols per second), bits per modulation symbol, and the number of streams supported by the antenna system. For E-UTRA satellite access component RIT: The physical channel bit rate depends on the modulation scheme and number of resource blocks in the channel bandwidth. and the subcarrier spacing used. When the subcarrier spacing is 15 kHz, the physical channel bit rate per layer can be expressed as Rlayer = Nmod x NRB x 168 kbps where - Nmod is the number of bits per modulation symbol for the applied modulation scheme (QPSK: 2, 16QAM: 4, 64QAM: 6) - NRB is the number of resource blocks in the aggregated frequency domain. NRB = 6 for the eMTC channel bandwidth6. NB-IoT only supports transmission of a single layer and the physical channel bit rate is as above, but with Nmod limited to 1(BPSK) or 2 (QPSK) and NRB= 1. |
| 8.2.5.1.4.2 | Layer 1 and Layer 2 overhead estimation Describe how the RIT accounts for all Layer 1 (PHY) and Layer 2 (MAC) overhead and provide an accurate estimate that includes static and dynamic overheads. For E-UTRA satellite access component RIT: - Downlink The downlink L1/L2 overhead includes: - Different types of reference signals - Cell-specific RS (CRS) transmitted within each resource block - UE-specific demodulation RS - Reference signals specifically targeting estimation of channel-state information (CSI-RS) - L1/L2 control signalling transmitted on the up to four first OFDM symbols of each subframe. - Synchronization signal and physical broadcast control channel - PDU headers in L2 sub-layers (MAC/RLC/PDCP) The combined overhead due to CRS (1a) and L1/L2 control signaling (2) depends on the number of cell-specific antenna ports and the number of OFDM symbols used for the L1/L2 control signaling. Some examples, for the case of no CSI-RS and no UE-specific RS, see 4.2.3.2.3 above, are shown below: - 1 antenna port for CRS and 1 symbol for L1/L2 control: 10.7% - 1 antenna port for CRS and 3 symbols for L1/L2 control: 25% - 4 antenna ports for CRS and 1 symbol for L1/L2 control: 19% - 4 antenna ports for CRS and 3 symbols for L1/L2 control: 31% The amount of overhead caused by synchronization signals and broadcast channel is approximately x%. The amount of overhead caused by L2 highly depends on the data packet size, and is approximately 2.7% for L1 data rates of 1Mbit/s, respectively. In a typical case, the relative overhead for CSI-RS (if present) is estimated to 0.06% per antenna port. The relative overhead due to UE-specific RS (if present) is estimated to be approximately 7%. For NB-IoT, the overhead from Narrowband RS (NRS) is dependent on the number of cell-specific antenna ports N (1 or 2) and equals 8 x N / 168 %. The overhead from NB-IoT downlink control signaling is dependent on the amount of data to be transmitted. For small infrequent data transmissions, the downlink transmissions are dominated by the L2 signaling during the connection setup. The overhead from L1 signaling is dependent on the configured scheduling cycle. The overhead due to Narrowband synchronization signal and Narrowband system information broadcast messages is only applicable to the NB-IoT anchor carrier. The actual overhead depends on the broadcasted system information messages and their periodicity. The overhead can be estimated to be around 26.25%. - Uplink L1/L2 overhead includes: - Demodulation reference symbols used e.g. for uplink channel estimation for uplink coherent demodulation, transmitted once every 0.5 ms slot. - Sounding reference signal (SRS) used for uplink channel-state estimation at the network side - L1/L2 control signalling transmitted on configurable amount of resource blocks (see also Item 5.2.3.2.4.5) - L2 control overhead due to e.g., random access, uplink time-alignment control, power headroom reports and buffer-status reports - PDU headers in L2 layers (MAC/RLC/PDCP) The amount of overhead caused by 1 is approximately 14%, corresponding to one DFTS-OFDM symbol in each slot. The relative overhead is estimated to be independent of the rank of the transmission. The amount of SRS overhead depends on the SRS transmission interval, SRS bandwidth and the usage of UpPTS for SRS. With a 10 msec SRS transmission interval and full band SRS, the relative overhead is approximately 0.7% Amount of uplink resources reserved for random access depends on the configuration. A typical case with PRACH format 0 is six resource blocks per radio frame, implying a relative overhead of 0.6%, 1.2%, and 2.4% for a channel bandwidth of 20 MHz, 10 MHz, and 5 MHz respectively. The relative overhead due to uplink time-alignment control depends on the configuration and the number of active UEs within a cell. The absolute overhead is typically less than 32 bps per UE. The amount of overhead for buffer status reports depends on the configuration. With continuous data and a 10 - 20 ms reporting interval the absolute overhead is 0.8-3.2 kbps. The amount of overhead caused by 5 highly depends on the data packet size, and is approximately 2.7%, for L1 data rates of 1Mbit/s, respectively. Above overhead calculations are based on normal CP length. For NB-IoT UL, data and control share the same resources and the overhead from L1/L2 control signaling depends on the scheduled traffic in the DL. The UL control signaling is dominated by RLC and HARQ positive or negative acknowledgments. A typical NB-IoT NPRACH overhead is in the order of 5 %. |
| 8.2.5.1.4.3 | Variable bit rate capabilities Describe how the proposal supports different applications and services with various bit rate requirements. For NR and E-UTRA satellite access component RIT: For a given combination of modulation scheme and code rate, the data rate available to a user can be controlled by the scheduler by assigning different number of resource blocks for the transmission. In case of multiple services, the available/assigned resource, and thus the available data rate, is shared between the services. |
| 8.2.5.1.4.4 | Variable payload capabilities Describe how the RIT supports IP-based application layer protocols/services (e.g. VoIP, video-streaming, interactive gaming, etc.) with variable-size payloads. For E-UTRA satellite access component RIT: The transport-block size can vary between 16 bits and ”maximum transport block size” bits. The number of bits per transport block can be assigned with a fine granularity. For eMTC, the maximum transport block size is 1000 bits in both UL and DL (optionally 2984 bits in uplink and 1736 bits in downlink) for the lowest UE category dedicated to eMTC and 4008 bits and 6968 bits for DL and UL respectively for the highest UE category dedicated to eMTC. See [36.213] sub-clause 7.1.7.2.1 for details. For NB-IoT, the maximum transport block size is 680 bits in the DL and 1000 bits in UL for the lowest UE category and 2536 bits (optionally 4968 bits) for DL and 2536 for UL for the highest UE category. See [36.213] sub-clause 16.4.1.5.1 for details. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.4.5 | Signalling transmission scheme Describe how transmission schemes are different for signalling/control from that of user data. For E-UTRA satellite access component RIT: - Downlink L1/L2 control signalling is time-multiplexed with data from a single UE perspective. Control signalling is limited to QPSK modulation. Control signalling relies on tail-biting convolutional coding. Turbo-codes are used for data with the exception for NB-IoT where also data transmission uses tail-biting convolutional coding. In case of eMTC and NB-IoT the L1/L2 control signaling is confined to a configured set of resource blocks and can be time multiplexed with data and are transmitted in scheduled subframes. - Uplink L1/L2 control signalling transmitted in one or multiple resource blocks typically at the edge of the system bandwidth and frequency multiplexed with data. For NB-IoT the L1 control signaling is time and frequency multiplexed with data. For both downlink and uplink, higher-layer signalling (e.g. MAC, RLC, PDCP headers and RRC signalling) is carried within transport blocks and thus transmitted using the same physical-layer transmitter processing as user data. |
| 8.2.5.1.5 | Mobility management (Handover) |
| 8.2.5.1.5.1 | Describe the handover mechanisms and procedures which are associated with – Inter-System handover; – Intra-System handover; – Intra-frequency and Inter-frequency; – Within the RIT or between RITs within one SRIT (if applicable). Characterize the type of handover strategy or strategies. For E-UTRA satellite access component RIT: For eMTC, E-UTRAN RRC_CONNECTED state, network-controlled UE-assisted handovers are performed and various DRX cycles are supported. Handover procedures, like processes that precede the final HO decision on the source network side (control and evaluation of UE and eNB measurements taking into account certain UE specific roaming and access restrictions), preparation of resources on the target network side, commanding the UE to the new radio resources and finally releasing resources on the (old) source network side. It contains mechanisms to transfer context data between evolved nodes, and to update node relations on C-plane and U-plane. Measurement Measurements to be performed by a UE for intra/inter-frequency mobility can be controlled by E-UTRAN, using broadcast or dedicated control. In RRC_IDLE state, a UE shall follow the measurement parameters defined for cell reselection specified by the E-UTRAN broadcast. The use of dedicated measurement control for RRC_IDLE state is possible through the provision of UE specific priorities. In RRC_CONNECTED state, a UE shall follow the measurement configurations specified by RRC directed from the E-UTRAN (e.g. as in UTRAN MEASUREMENT_CONTROL). In RRC_IDLE and RRC_CONNECTED the UE may be configured to monitor some UTRA or E-UTRA carriers according to reduced performance requirements as specified in [36.133]. Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - Intra-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are intra-frequency measurements when the current and target cell operates on the same carrier frequency. - Inter-frequency neighbour (cell) measurements: Neighbour cell measurements performed by the UE are inter-frequency measurements when the neighbour cell operates on a different carrier frequency, compared to the current cell. For more details, refer to [36.300] sub-clauses 10.1 & 10.2. For NB-IoT, network-controlled handover is not supported. Instead, intra-system (intra-frequency, inter-frequency and TN-NTN/NTN-TN) mobility is supported via cell selection and cell re-selection mechanisms. The cell re-selection shall be controlled by the network indirectly based on measurements and associated value thresholds signalled to the UE in system information broadcasted in the cell, for which cell re-selection shall be controlled. Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements take the same definition as for eMTC. Inter-system mobility support is not specified by 3GPP. |
| 8.2.5.1.5.2 | What are the handover interruption times for: – within the RIT (intra- and inter-frequency); – between various RITs within a SRIT; – between the RIT and another IMT system. <<Answer pending>> |
| 8.2.5.1.6 | Radio resource management |
| 8.2.5.1.6.1 | Describe the radio resource management, support of: – centralized and/or distributed RRM; – dynamic and flexible radio resource management; – efficient load balancing. RRM mechanism in the following is supported in both E-UTRA satellite access component RIT commonly. General E-UTRA satellite access RIT performs radio resource management to ensure the efficient use of the available radio resource. RRM functions include: - Radio bearer control (RBC): the establishment, maintenance and release of radio bearer involves the configuration of radio resource. This is located in eNB. - Radio Admission Control (RAC): RAC is to admit or reject the establishment of new radio bearer. It considers QoS requirement, the priority level, overall resource situation. This is located in eNB. - Connection Mobility Control (CMC): it controls the number of UEs in idle mode and connected mode. In idle mode for eMTC, and in idle mode and connected mode for NB-IoT, cell reselection algorithm is controlled by parameter setting and in the connected mode for eMTC, eNB controls UE mobility via handover and RRC connection release with redirection. Dynamic/flexible radio resource management E-UTRA satellite access supports dynamic and flexible radio resource management by packet scheduling that allocates and de-allocates resources to user and control plane packets. Load balancing (LB) Load balancing has the task to handle uneven distribution of the traffic load over multiple cells. The purpose of LB is thus to influence the load distribution for the higher resource utilization and QoS. LB is achieved with hand-over, redirection or cell reselection. |
| 8.2.5.1.6.2 | Inter-RIT interworking Describe the functional blocks and mechanisms for interworking (such as a network architecture model) between heterogeneous RITs within a SRIT, if supported. For NB-IoT-NR interworking, deployment of an NB-IoT carrier within an NR carrier is possible for co-located eNB and gNB, with separate interfaces for connecting to the respective core networks. Also NB-IoT deployment within an eMTC carrier is feasible with co-located eNBs. E-UTRA satellite access supports mobility between SANs operating with NTN payloads in different orbits (e.g., GSO, NGSO at different altitudes). Mobility between NTN and TN is supported: NTN TN mobility is supported in both directions, i.e. from NTN to TN and from TN to NTN. During mobility between NTN and Terrestrial Network (TN), a UE is not required to connect to both NTN and TN at the same time. This form of mobility is via handover with eMTC and with cell selection/re-selection with NB-IoT. |
| 8.2.5.1.6.3 | Connection/session management The mechanisms for connection/session management over the air-interface should be described. For example: – the support of multiple protocol states with fast and dynamic transitions; – the signalling schemes for allocating and releasing resources. For E-UTRA satellite access component RIT: RRC_IDLE and RRC_CONNECTED are supported. RRC_IDLE: - PLMN selection; - Broadcast of system information; - Cell re-selection mobility; - Paging for mobile terminated data is initiated by EPC; - Paging for mobile terminated data area is managed by EPC; - DRX for CN paging configured by NAS. - RRC_CONNECTED: - The UE AS context is stored in E-UTRAN and the UE; - E-UTRAN knows the cell which the UE belongs to; - Transfer of unicast data to/from the UE; - For eMTC, network controlled mobility including measurements; - For NB-IoT, UE controlled mobility including measurements; Transition between RRC states: - From RRC_IDLE to RRC_CONNECTED: RRC connection setup - From RRC_CONNECTED to RRC_IDLE: RRC connection release For more details, refer to [36.300] |
| 8.2.5.1.7 | Frame structure |
| 8.2.5.1.7.1 | Describe the frame structure for downlink and uplink by providing sufficient information such as: – frame length; – the number of time slots per frame; – the number and position of switch points per frame for TDD; – guard time or the number of guard bits; – user payload information per time slot; – control channel structure and multiplexing; – power control bit rate. For E-UTRA satellite access component RIT: - Frame length, sub-carrier spacing, and time slots: FDD frame structure is supported. One radio frame of length 10 ms consisting of 10 subframes, each of length 1 ms. Each subframe consists of two slots, each of length 0.5 ms. Each slot consists of seven OFDM symbols (six OFDM symbols in case of extended cyclic prefix). A minimum time unit for transmission is a TTI, which has a duration of one subframe. Sub-carrier spacings of 15kHz is supported. For NB-IoT, the minimum time unit for transmission is a subframe in the downlink and a resource unit in the uplink. The length of a resource unit is dependent on the subcarrier spacing and number of subcarriers. Up to ten subframes or resource units can be assigned to the UE for one transmission. Sub-carrier spacings of 15kHz is supported for DL, and sub-carrier spacings of 3.75kHz and 15kHz is supported for UL (see item 8.2.5.1.2.1for more details). For eMTC half-duplex FDD operation and NB-IoT, a guard period of 1ms is used for the switching time. - Downlink control channel structure: Downlink control signalling and data transmission to the same UE are time multiplexed on different subframes. - Uplink control channel structure: Uplink control signalling is frequency-multiplexed with data for other UEs (no time separation) when the UE has no data to be transmitted. Uplink control signalling is typically transmitted at the edges of the overall system bandwidth. For eMTC, uplink control signalling is piggy-backed with data i.e. transmitted with data on the PUSCH when the UE has data to be transmitted. - Power control bit rate: There is no specific power-control rate. At most one power-control command per subframe, implying 1 kHz maximum power-control rate. For more details on the (uplink) power control, see [36.213] sub-clause 5.1. For NB-IoT, only open-loop power control is supported. |
| 8.2.5.1.8 | Spectrum capabilities and duplex technologies NOTE 1 – Parameters for both downlink and uplink should be described separately, if necessary. |
| 8.2.5.1.8.1 | Spectrum sharing and flexible spectrum use Does the RIT/SRIT support flexible spectrum use and/or spectrum sharing for the bands for IMT? Provide details. For E-UTRA satellite access component RIT: For eMTC, - eMTC supports flexible spectrum use by using one or multiple carriers from the network point of view, which can be either contiguous or non-contiguous in the frequency domain. For NB-IoT, - Flexible spectrum use is supported by using one or multiple NB-IoT carriers. - A single, anchor, NB-IoT carrier of 180 kHz each for UL and DL in FDD is the minimum required spectrum. - Additional non-anchor NB-IoT carrier(s), each of 180 kHz can be associated to the same NB-IoT cell, and a UE uses either the anchor or one non-anchor NB-IoT carrier. - The anchor carrier and non-anchor carrier(s) can be either contiguous or non-contiguous in the frequency domain. NR can be configured to co-exist with NB-IoT/eMTC using frequency division multiplexing (FDM) manner. - For NB-IoT, - The downlink co-existence can be made by NR by configuring reserved resource blocks (RBs) which are declared as not available for PDSCH for NR users. These reserved resource blocks can be used by NB-IoT anchor and non-anchor carriers. For NR users that are scheduled on the resource block group (RBG) which includes the reserved RB, NR will configure the rate match pattern for those users using dynamic or semi-static indication. - For uplink, NR can use appropriate uplink resource allocation to “reserve” RBs for NB-IoT users. For example, if some of the RBs are reserved for NB-IoT, NR will allocate other RBs to its users, by either frequency domain resource allocation type 0 or type 1. By the above means, NR and NB-IoT can co-exist without any impact to each other. - For eMTC, - The downlink co-existence can be made by NR by configuring reserved resources for eMTC. It can be achieved by resource element (RE) level and RB level resource reservation indication. Similar mechanism to NB-IoT can be employed for uplink to enable NR co-existence with eMTC for uplink. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.8.2 | Channel bandwidth scalability Describe how the proposal supports channel bandwidth scalability, including the supported bandwidths. Describe whether the proposed RIT supports extensions for scalable bandwidths wider than 30 MHz. Consider, for example: – the scalability of operating bandwidths; – the scalability using single and/or multiple RF carriers. Describe multiple contiguous (or non-contiguous) band aggregation capabilities, if any. Consider for example the aggregation of multiple channels to support higher user bit rates. For E-UTRA satellite access component RIT For NB-IoT, the channel bandwidth is 200kHz. The channel bandwidth is not scalable at the UE because it is tailored for mMTC where supporting high peak user data rates are less important than maintaining low device complexity. There is no aggregation support by the UE of multiple NB-IoT carriers – see item 8.2.5.1.8.1 for more details. However, the system capacity can be scaled up by network deployment of multiple NB-IoT RF carriers (200 kHz each), which enables to also scale up the user connection density and capacity. For eMTC, the channel bandwidth is 1.4MHz. The channel bandwidth is not scalable at the UE because it is tailored for mMTC where supporting high peak user data rates are less important than maintaining low device complexity. There is no aggregation support by the UE of multiple carriers – see item 8.2.5.1.8.1 for more details. The system capacity can be scaled up by network deployment of multiple E-UTRA RF carriers (1.4 MHz each), which enables to also scale up the user connection density and capacity. The SRIT supports high peak user data rates via NR satellite access component RIT. |
| 8.2.5.1.8.3 | What are the frequency bands supported by the RIT? Please list. For E-UTRA satellite access component RIT: The following frequency bands are currently specified for eMTC and NB-IoT, in accordance with spectrum requirements defined by Report ITU-R M.2514-0. Introduction of other ITU-R identified bands are not precluded in the future. 3GPP technologies are also defined as appropriate to operate in other frequency arrangements and bands. Detailed information on the following bands can be found in [36.102] sub-clause 5.2A for eMTC and sub-clause 5.2B for NB-IoT. 450 – 6000 MHz: E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 256; 1980 MHz; –; 2010 MHz; 2170 MHz; –; 2200 MHz; FDD. 255; 1626.5 MHz; –; 1660.5 MHz; 1525 MHz; –; 1559 MHz; FDD. NOTE: Satellite bands are numbered in descending order from 256. |
| 8.2.5.1.8.4 | What is the minimum amount of spectrum required to deploy a contiguous network, including guard bands (MHz)? For E-UTRA satellite access component RIT: For eMTC, the minimum amount of paired spectrum is 2 x 1.4 MHz,. For NB-IoT, the minimum amount of paired spectrum is 2 x 0.2 MHz. |
| 8.2.5.1.8.5 | What are the minimum and maximum transmission bandwidth (MHz) measured at the 3 dB down points? For E-UTRA satellite access component RIT: For eMTC, the 3 dB bandwidth is not part of the specifications, however: - The 99% channel bandwidth (occupied bandwidth of single carrier) is 1.4 MHz. For NB-IoT, the 99% channel bandwidth is 0.2 MHz. |
| 8.2.5.1.8.6 | What duplexing scheme(s) is (are) described in this template? (e.g. TDD, FDD or half-duplex FDD). Describe details such as: – What is the minimum (up/down) frequency separation in case of full- and half-duplex FDD? – What is the requirement of transmit/receive isolation in case of full- a half-duplex FDD? Does the RIT require a duplexer in either the mobile station (MS) or space segment? – What is the minimum (up/down) time separation in case of TDD? – Whether the DL/UL Ratio variable for TDD? What is the DL/UL ratio supported? If the DL/UL ratio for TDD is variable, what would be the coexistence criteria for adjacent cells? For E-UTRA satellite access component RIT: E-UTRA satellite access supports paired spectrum and allows FDD operation on a paired spectrum. - For FDD operation, it supports both half-duplex and full-duplex FDD. - For the base station (from here onwards referred to as the “Satellite Access Node” (SAN)), a duplexer is needed for half-duplex/full-duplex FDD. For the terminal, a duplexer is needed for full-duplex FDD only. - For full-duplex FDD, the required transmit/receive isolation is a UE function of; the Tx emission mask (emission level on the Rx frequency) , the TX-Rx frequency spacing , the Tx- Rx duplex filter isolation, the TX and RX configuration (RB location, RB power and RB allocation) and the required Rx desense criteria. For the supported RIT operating bands, the following parameters are defined in [36.102]: - the minimum (up/down) Tx to Rx frequency separation is 10 MHz - the minimum Tx-Rx band gap is 10 MHz - For half-duplex FDD, for the UE there is no specified transmit / receive isolation due half duplex mode. For NB-IoT, Half-duplex FDD are supported. The terminal does not need a duplexer, and there is no specified transmit / receive isolation due to half-duplex mode. |
| 8.2.5.1.9 | Support of advanced antenna capabilities |
| 8.2.5.1.9.1 | Fully describe the multi-antenna systems supported in the User segment, Space segment, or both that can be used and/or must be used; characterize their impacts on systems performance; e.g. does the RIT have the capability for the use of: – spatial multiplexing techniques; – space-time coding (STC) techniques; – beam-forming techniques (e.g. adaptive or switched). For E-UTRA satellite access component RIT The multi-antenna systems in eMTC and NB-IoT supports the following MIMO transmission scheme at both the UE and the SAN: - Spatial transmit diversity is supported based on space frequency block coding for eMTC and NB-IoT, and frequency switched transmit diversity for eMTC. Specification transparent diversity schemes are also supported by eNB implementations. Hybrid beamforming including both digital and analog beamforming is supported at the SAN |
| 8.2.5.1.9.2 | How many antennas are supported by the Space segment and MS for transmission and reception? Specify if correlated or uncorrelated antennas in co-polar or cross-polar configurations are used. What is the antenna spacing (in wavelengths)? For E-UTRA satellite access component RIT E-UTRA satellite access supports {1, 2, 4} antenna ports in the DL and {1} antenna port in the UL. SAN supports e.g. rectangular antenna arrays described by the tuple (M,N,P). M, N are the number of antenna elements in the vertical and horizontal dimensions. P is the number of polarizations per antenna element. Antenna elements are uniformly spaced in the horizontal direction with a spacing of dH and in the vertical direction with a spacing of dV. E-UTRA satellite access supports various antenna spacing, antenna port layouts and antenna virtualization approaches. |
| 8.2.5.1.9.3 | Provide details on the antenna configuration that is used in the self-evaluation. The information will be provided with self evaluation results. |
| 8.2.5.1.9.4 | If spatial multiplexing (MIMO) is supported, does the proposal support (provide details if supported): – Single codeword (SCW) and/or multi-codeword (MCW). – Open and/or closed loop MIMO. – Cooperative MIMO. – Single-user MIMO and/or multi-user MIMO. For E-UTRA satellite access component RIT: Spatial multiplexing is not supported which means: E-UTRA satellite access supports single user MIMO with 1 layer per UE in DL and 1 layer per UE in UL. Multi-user MIMO is not supported. E-UTRA satellite access does not support coordinated multipoint transmission/reception, . |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.9.5 | Other antenna technologies Does the RIT/SRIT support other antenna technologies, for example Active Antenna System (AAS) If so, please describe. E-UTRA satellite access supports the use of AAS (with RF requirements defined in [36.108]). |
| 8.2.5.1.10 | Link adaptation and power control |
| 8.2.5.1.10.1 | Describe link adaptation techniques employed by RIT/SRIT, including: – the supported modulation and coding schemes; – the supporting channel quality measurements, the reporting of these measurements, their frequency and granularity; Provide details of any adaptive modulation and coding schemes, including: – hybrid ARQ or other retransmission mechanisms? – algorithms for adaptive modulation and coding, which are used in the self-evaluation; – other schemes? For E-UTRA satellite access component RIT: For data, eMTC supports the modulation schemes as described in section8.2.5.1.2.2.1, and code rates between approximately 0.1 and 0.9. Some xx different modulation-scheme/code-rate combinations exist. This is valid for both downlink and uplink. In both downlink and uplink, link adaptation (selection of modulation scheme, code rate and repetition number) is controlled by the SAN. In the downlink the network selection of modulation-scheme/code-rate combination can e.g. be based on Channel Quality Indicators (CQIs) reported by the terminals. Several different CQI modes exist, including frequency-selective and wideband modes, periodic and aperiodic modes. The CQI mode is controlled by the SAN. In the uplink the SAN may measure either the traffic channel or sounding reference signals and use this as input to link adaptation. More details are found in [36.213]. UE reports its headroom to its maximum power to assist the SAN with uplink link adaptation. On the MAC layer, hybrid ARQ with soft-combining between transmissions is supported. Different redundancy versions can be used for different transmissions. The modulation scheme may be changed for retransmissions. In order to minimize delay and feedback, a set of parallel stop-and-wait protocols are used. To correct possible residual errors, the MAC HARQ is complemented by a robust selective-repeat ARQ protocol on the RLC layer. More details are found in [36.321] and [36.322]. For NB-IoT π/2BPSK and QPSK modulation schemes are supported. Transmissions of a transport block can be mapped to between 1 and 10 subframes to adapt the code rate of the transmission. In its most basic form the link adaptation supports 116 alternative modulation-scheme/code-rate combinations for the UL and 104 alternatives for the DL. To further enhance the link robustness NB-IoT supports repetition-based transmission scheme using up to 2048 repetitions of each modulation-scheme/code-rate combination. During the connection setup procedure NB-IoT supports a basic UE feedback mechanism which allows the SAN to access the coupling loss experienced by a UE. In connected mode HARQ and ARQ RLC/MAC feedback is supported in similarity to E-UTRA satellite access. |
| 8.2.5.1.10.2 | Provide details of any power control scheme included in the proposal, for example: – power control step size (dB); – power control cycles per second; – power control dynamic range (dB); – minimum transmit power level with power control; – associated signalling and control messages. For E-UTRA satellite access component RIT: The RIT uplink power control is based on both signal-strength measurements done by the terminal itself (open-loop power control), as well as measurements by the SAN. The later measurements are used to generate power-control commands that are subsequently fed back to the terminals as part of the downlink control signalling (closed-loop power control). Both absolute and relative power-control commands are supported. The available relative power adjustments (“step size”) in case of relative power control are [-1 dB, 0 dB, +1 dB, +3 dB]. The time between power-control commands can be down to 1ms. The minimum transmit power requirement, – 40dBm, yields a dynamic range of -40 to 23=63dB for a terminal with maximum power 23dBm. For eMTC and NB-IoT maximum UE power supported are 20 dBm and 23 dBm. NB-IoT only supports the open-loop power control, with a constraint that full power is used when the UE is commanded to use 2 or more repetitions of a physical channel. More details about uplink power control are found in [36.213]. Downlink power control is, with the exception for NB-IoT, network-implementation specific and thus outside the scope of the specification. A simple and efficient power control strategy is to transmit with a constant output power. Variations in channel conditions and interference levels are adapted to by means of scheduling and link adaptation rather than with power control. |
| 8.2.5.1.11 | Power classes |
| 8.2.5.1.11.1 | Mobile station emitted power |
| 8.2.5.1.11.1.1 | What is the radiated antenna power measured at the antenna (dBm)? For E-UTRA satellite access component RIT: The maximum output power less than or equal to 23 dBm is measured at antenna connector assuming a 0 dBi antenna gain with omni-direction for horizontal plain. - For eMTC UE, UE power class with the maximum output power of 20dBm is defined in addition to UE power class with 23dBm maximum output power - For NB-IoT UE, UE power classes with the maximum output power of 20dBm is defined in addition to UE power class with 23dBm maximum output power. |
| 8.2.5.1.11.1.2 | What is the maximum peak power transmitted while in active or busy state? See item 8.5.1.11.1.1. |
| 8.2.5.1.11.1.3 | What is the time averaged power transmitted while in active or busy state? Provide a detailed explanation used to calculate this time average power. For E-UTRA satellite access component RIT: In TR36.942, the UE transmit power with different power control parameters was shown in RAN4 feasibility study. One example of estimate averaged transmit power is to take median of the results which is around -10~-5dBm. |
| 8.2.5.1.11.2 | Space segment emitted power |
| 8.2.5.1.11.2.1 | What is the base station transmit power per RF carrier? For E-UTRA satellite access component RIT: The SAN transmit power of one carrier is the mean power delivered to a load with resistance equal to the nominal load impedance of the transmitter. The SAN maximum transmit power of one carrier is the mean power level measured at the SAN antenna connector in a specified reference condition. SANs intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. |
| 8.2.5.1.11.2.2 | What is the maximum peak transmitted power per RF carrier radiated from antenna? SANs intended for general-purpose applications do not have limits on the maximum transmit power. However, there may exist regional regulatory requirements which limit the maximum transmit power. |
| 8.2.5.1.11.2.3 | What is the average transmitted power per RF carrier radiated from antenna? The averaged transmitted carrier power is subject to the type of signal/channel to be transmitted, bandwidth, and deployment scenario, etc. |
| 8.2.5.1.12 | Scheduler, QoS support and management, data services |
| 8.2.5.1.12.1 | QoS support – What QoS classes are supported? – How QoS classes associated with each service flow can be negotiated. – QoS attributes, for example: – data rate (ranging from the lowest supported data rate to maximum data rate supported by the MAC/PHY); – control plane and user plane latency (delivery delay); – packet error rate (after all corrections provided by the MAC/PHY layers), and delay variation (jitter). – Is QoS supported when handing off between radio access networks? Please describe. – How users may utilize several applications with differing QoS requirements at the same time. For E-UTRA satellite access component RIT: In E-UTRA satellite access, a bearer is the level of granularity for QoS control. Up to 15 data bearers can be established in parallel for a UE. Each bearer is associated with a QoS class index (QCI), and an Allocation/ Retention Priority (ARP), and optionally with guaranteed bitrate (GBR) and maximum bit rate (MBR). The QCI is an index representing the priority, allowable delay, and packet error rate of a bearer, and up to 256 QCIs could be defined by the operator (21 of which is standardised). The QCI, MBR, GBR and ARP are signalled from the CN to the RAN when the bearer is established or modified, so that the scheduler in the RAN could ensure the QoS for each bearer. The ARP as well as other QoS parameters could be used to determine which bearers to prioritise at handover. By using multiple bearers having different QoS profiles, multiple application flows with different QoS requirements could be accommodated. |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.12.2 | Scheduling mechanisms – Exemplify scheduling algorithm(s) that may be used for full buffer in the technology proposal for evaluation purposes. Describe any measurements and/or reporting required for scheduling. For E-UTRA satellite access component RIT: Dynamic scheduling on a 1 ms (subframe) basis is applied to both uplink and downlink. Typically, E-UTRA satellite access scheduling is based on the instantaneous radio-link quality as seen by the different users, and the traffic demand and quality-of-service requirements of individual users and in the cell as a whole. The former is based on CQI reports from the terminals (downlink) or measurements of sounding signals from the terminals (uplink). Based on this the SAN may e.g. apply a proportional fair scheduling algorithm. The QoS assessment is supported by means of receiving QoS information from the “higher layers”. For VoIP traffic (or any traffic having similar characteristics) semi-persistent scheduling can be applied, by which a user can be allocated time-frequency resources in a semi-persistent manner, i.e., fixed resources are allocated at certain intervals without L1/L2 control signaling each time. This is especially useful to reduce the L1/L2 control signaling overhead and to increase VoIP capacity. Moreover, the RIT supports repetitions / TTI bundling, by which time-frequency resources can be allocated consecutively to a user for a longer period than 1 ms by a single L1/L2 control signaling. A larger transport block size or a lower coding rate can be supported by this technique. This is especially useful when the coverage needs to be extended. Intercell-interference coordination mechanisms may also be realized by the scheduler. To aid inter-cell coordination, E-UTRA satellite access defines two indicators exchanged between SANs: The High-interference Indicator (HI) provides information to neighboring cells about the part of the cell bandwidth upon which the cell intends to schedule its cell-edge users. The Overload Indicator (OI) provides information on the uplink interference level experienced in each part of the cell bandwidth. For the downlink, intercell-interference coordination can be realized using a Relative Narrowband TX Power (RNTP) indicator. For NB-IoT the scheduler controls the transmission duration of control channels in number of subframes in a semi-static fashion while the transmission duration of shared channels can be varied dynamically. This is beneficial for extending coverage. See [36.423] for details. |
| 8.2.5.1.13 | Radio interface architecture and protocol stack |
| 8.2.5.1.13.1 | Describe details of the radio interface architecture and protocol stack such as: Logical channels – Control channels. – Traffic channels. Transport channels and/or physical channels. E-UTRA satellite access component RIT: NOTE: eMTC/NB-IoT uses optimized physical layer and radio procedures (e.g. for very low power consumption), thus a number of E-UTRA protocol mechanisms and functions are either not used, or are specific to eMTC/NB-IOT only. Some examples of functionalities not specified for NB-IoT are: inter-RAT mobility, handover and measurement reports. Only few exceptions/variations are highlighted below; for details please refer to stage-3 specifications. Radio Protocol stack The protocol stack for the user plane includes PDCP, RLC, MAC, and PHY sublayers (terminated in UE and eNB). For NB-IoT, the user plane is not used when transferring user data over NAS. On the Control plane, the following protocols are defined: - RRC, PDCP, RLC, MAC and PHY sublayers (terminated in UE and eNB); - NAS protocol (terminated in UE and Core Network) For NB-IoT, if certain optimizations are supported, PDCP can be bypassed (at all, or until AS security is activated) For details on protocol services and functions, please refer to 3GPP specifications (e.g. TS 36.300). Radio Channels (Physical, Transport and Logical Channels) eMTC specific physical channels are: - MTC physical downlink control channel (MPDCCH) - Resynchronization Signal (RSS) - Physical broadcast channel (PBCH) - Carries the BCH transport channel - Physical downlink shared channel (PDSCH) - Carries the DL-SCH and PCH transport channels - Physical uplink control channel (PUCCH) - Physical uplink shared channel (PUSCH) - Carries the UL-SCH transport channel - Physical random access channel (PRACH) - Carries the RACH transport channel NB-IOT specific physical channels are: - Narrowband Physical broadcast channel (NPBCH) - Narrowband Physical downlink shared channel (NPDSCH) - Narrowband Physical downlink control channel (NPDCCH) - Narrowband Physical uplink shared channel Format 1 (NPUSCH F1) - Narrowband Physical uplink shared channel Format 2 (NPUSCH F2) - Narrowband Physical random access channel (NPRACH) In addition to the above channels, two types of physical signals are defined: reference or synchronization signals. E-UTRA Logical channels (at MAC/RLC sublayer) are: - Control Channels (for the transfer of control plane information), e.g.: - Broadcast Control Channel (BCCH) - For eMTC, Bandwidth Reduced Broadcast Control Channel (BR-BCCH) - Paging Control Channel (PCCH) - Common Control Channel (CCCH) - Dedicated Control Channel (DCCH) - Traffic Channels (for the transfer of user plane information), e.g.. - Dedicated Traffic Channel (DTCH) The following mapping between logical channels and transport channels is defined: In Uplink, CCCH, DCCH and DTCH can be mapped to UL-SCH; In Downlink, - BCCH can be mapped to BCH, or DL-SCH; - BR-BCCH can be mapped to DL-SCH; - PCCH can be mapped to PCH; - CCCH, DCCH and DTCH can be mapped to DL-SCH Other Aspects - NB-IOT For NB-IoT only a specific multi-carrier operation is defined (e.g. a RRC_CONNECTED UE can be configured to a non-anchor carrier, for all unicast transmissions). For more details on E-UTRA satellite access Radio Protocol architecture and channels, refer to: [36.300], [36.401], [36.201], [37.340] |
| 8.2.5.1.13.2 | What is the bit rate required for transmitting feedback information? For NR and E-UTRA satellite access component RIT: As described in other sections (e.g. 8.2.5.1.3, 8.2.5.1.10, 8.2.5.1.13.1), from a Layer1 point of view (PHY/MAC), few control (feedback/HARQ) channels are defined (in UL and DL), with specific characteristics and transmission schemes/rates. At Layer2 level (i.e. RLC ARQ), assuming an RLC AM Status report is sent every 50 ms (configurable), with a size of few octets, e.g. 32 bits (including RLC/MAC header overhead), this results in a rate of 32/0.05= 640 bit/s. |
| 8.2.5.1.13.3 | Channel access Describe in detail how RIT/SRIT accomplishes initial channel access, (e.g. contention or non-contention based). For NR and E-UTRA satellite access component RIT (Common principles) Initial channel access is typically accomplished via the “random access procedure” (assuming no dedicated/scheduled resources are allocated). The random access procedure can be contention based (e.g. at initial connection from idle mode) or non-contention based (e.g. for eMTC, during Handover to a new cell). Random access resources and parameters are configured by the network and signalled to the UE (via broadcast or dedicated signaling). Contention based random access procedure encompasses the transmission of a random access preamble by the UE (subject to possible contention with other UEs), followed by a random access response (RAR) in DL (including allocating specific radio resources for the uplink transmission). Afterwards, the UE transmits the initial UL message (e.g. RRC connection Request) using the allocated resources, and wait for a contention resolution message in DL (to confirming access to that UE). The UE could perform multiple attempts until it is successful in accessing the channel or until a timer (supervising the procedure) elapses. Non-contention based random access procedure foresees the assignment of a dedicated random access resource/preamble to a UE (e.g. for eMTC, part of a HO command). This avoids the contention resolution phase, i.e. only the random access preamble and random access response messages are needed to get channel access. From a L1 perspective, a random access preamble is transmitted (UL) in a PRACH, random access response (DL) in a PDSCH, UL transmission in a PUSCH, and contention resolution message (DL) in a PDSCH. Other/specific RIT aspects Detailed aspects and mechanisms of the random access procedure are specific/different for each RIT (NR and E-UTRA satellite access), e.g. physical resources/channels, timing, messages/parameters, etc. Pre-compensation as described in 8.2.5.1.3 of this document shall be performed by the UE prior to initial access. For eMTC/NB-IOT, there are also specific differences, e.g. - For eMTC: PRACH configuration/repetition, decoding/interpretation of Random Access Response (RAR), etc. - For NB-IOT: dedicated NPRACH channel, configuration, RAR decoding, etc. For more details, refer to: - NR: [38.300], [38.321] and [38.213] - E-UTRA satellite access: [36.300], [36.321] and [36.213] |
| 8.2.5.1.14 | Beam selection |
| 8.2.5.1.14.1 | Describe in detail how the RIT/SRIT accomplishes cell selection to determine the serving cell for the users. For NR and E-UTRA satellite access component RIT: Cell selection is based on the following principles, common for NR and E-UTRA satellite access: - The UE NAS layer identifies a selected PLMN (and equivalent PLMNs, if any); - The UE searches the supported frequency bands (RIT specific) and for each carrier frequency it searches and identifies the strongest cell. It reads cell broadcast information to identify its PLMN(s) and other relevant parameters (e.g. related to cell restrictions); - The UE seeks to identify a suitable cell; if it is not able to identify a “suitable” cell it seeks to identify an “acceptable” cell. - A cell is “suitable” if: the measured cell attributes satisfy the cell selection criteria (based on DL radio signal strength/quality); the cell belongs to the selected/equivalent PLMN; cell is not restricted (e.g. cell is not barred/reserved or part of "forbidden" roaming areas); - An “acceptable” cell is one for which the measured cell attributes satisfy the cell selection criteria and the cell is not barred. Among the identified suitable (or acceptable) cells, the UE selects the strongest cell, (technically it “camps” on that cell). As signalled/configured by the radio network, certain frequencies or RITs could be prioritized for camping. Other/specific RIT aspects Detailed aspects and mechanisms of cell selection are specific/different for each RIT (NR and E-UTRA satellite access), e.g.: - frequency/cell search (using different DL sync signals and search procedures) - broadcast system information acquisition/signalling - RF measurement and metrics/thresholds for selection criteria Few specific mechanisms/restrictions apply to eMTC/NB-IOT, e.g.: - NB-IOT and eMTC uses specific DL signals and (optimized/limited) cell search and measurement procedures - Specific cell selection criteria may apply to eMTC For more details, refer to: - NR: [38.300] sub-clause 9.2.1.1 and [38.304] sub-clause 5.2 - E-UTRA satellite access: [36.300] sub-clause 10.1.1.1 and [36.304] sub-clause 5.2 |
| 8.2.5.1.15 | Location determination mechanisms |
| 8.2.5.1.15.1 | Describe any location determination mechanisms that may be used, e.g. to support location-based services. For E-UTRA satellite access component RIT: GNSS is supported by the UE for self-determination of location. |
| 8.2.5.1.16 | Priority access mechanisms |
| 8.2.5.1.16.1 | Describe techniques employed to support prioritization of access to radio or network resources for specific services or specific users (e.g. to allow access by emergency services). For E-UTRA satellite access component RIT: [E-UTRA satellite access RIT defines a range of access control mechanisms described in TS 22.011 section 4 that can be used to restrict UEs’ access attempts to the system. These mechanisms are detailed in the bulleted list below. In addition, the E-UTRA satellite access specification defines mechanisms for the network to prioritize access attempts after the overall access control mechanism defined for the UEs has passed and the UE has initiated RRC connection establishment, at which point the network can allocate relative priority to the request relative to other requests based on the provided RRC Establishment cause. The specified mechanisms that can be used to restrict UEs’ access attempts to the system defined in different 3GPP releases: - Access Class Barring (ACB) If the UE is a member of at least one Access Class which corresponds to the permitted classes broadcast in the system information, and the Access Class is applicable in the serving network, access attempts are allowed. Additionally, in the case of the access network being UTRAN the serving network can indicate that UEs are allowed to respond to paging and perform location registration even if their access class is not permitted. Otherwise access attempts are not allowed. Any number of these classes may be barred at any one time, and in case of multiple core networks sharing the same access network, the access network is able to apply Access Class Barring for the different core networks individually. The network operator can take the network load into account when allowing UEs access to the network. Access Classes are applicable as follows: Classes 0 - 9 - Home and Visited PLMNs; Class 10 - This bit’s presence in the access class barring information broadcast to the cell indicates whether Emergency Calls are allowed for UEs with access classes 0 to 9 and UEs without an IMSI. For UEs with access classes 11 to 15, Emergency Calls are not allowed if both "Access class 10" and the relevant Access Class (11 to 15) are barred. Classes 11 and 15 - Home PLMN only if the EHPLMN list is not present or any EHPLMN; Classes 12, 13, 14 - Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. - Coverage Enhancement level-based access class barring Supports barring of eMTC and NB-IoT devices in specific coverage enhancement levels The following mechanisms are only applicable to eMTC: - Service Specific Access Control (SSAC) SSAC provides a mechanism to minimize service availability degradation (i.e. radio resource shortage) due to the mass simultaneous mobile originating session requests and maximize the availability of the wireless access resources for non-barred services by applying independent access control for telephony services (MMTEL) for mobile originating session requests from idle-mode and connected-mode. EPS provides a capability to assign a service probability factor and mean duration of access control for each of MMTEL voice and MMTEL video by broadcasting system information parameters for: - assigning a barring rate (percentage) commonly applicable for Access Classes 0-9, - assigning a flag barring status (barred /unbarred) for each Access Class in the range 11-15, - SSAC does not apply to Access Class 10. - Extended Access Class Barring (EAB) EAB is a mechanism for controlling Mobile Originating access attempts from UEs that are configured for EAB in order to prevent overload of the access network and/or the core network. In congestion situations, the operator can restrict access from UEs configured for EAB while permitting access from other UEs. UEs configured for EAB are considered more tolerant to access restrictions than other UEs. When an operator determines that it is appropriate to apply EAB, the network broadcasts system information parameters for EAB UEs. - Overriding extended access barring Overriding Extended Access Barring is a mechanism for the operator to allow UEs that are configured for EAB to access the network under EAB conditions. The UE configured to override extended access class barring overrides EAB restrictions as long as it has an active PDN connection for which EAB has been configured to not apply. - Application Specific Congestion Control for Data Communication (ACDC) ACDC is an access control mechanism for the operator to allow/prevent new access attempts from particular, operator-identified applications in the UE in idle mode. ACDC does not apply to UEs in connected mode. For Access Control based on ACDC categories, at subscription at least four ACDC categories are allocated to the subscriber and stored in the ACDC MO or USIM. The network can prevent/mitigate overload of the access network and/or the core network. ACDC related parameterization is provided with system information broadcast.] |
| 8.2.5.1.17 | Unicast, multicast and broadcast |
| 8.2.5.1.17.1 | Describe how the RIT enables: – broadcast capabilities; – multicast capabilities; – unicast capabilities; using both dedicated carriers and/or shared carriers. Please describe how all three capabilities can exist simultaneously. For E-UTRA satellite access component RIT: The RIT is envisioned to support broadcast, multicast and unicast services. For NB-IoT/eMTC, recent broadcast/multicast enhancements are: Multicast downlink transmission based on Single-Cell Point-to-Multipoint (SC-PTM). |
| 8.2.5.1.17.2 | Describe whether the proposal is capable of providing multiple user services simultaneously to any user with appropriate channel capacity assignments? For E-UTRA satellite access component RIT (common principles) Multiple services per user can be supported by setting up multiple data radio bearers (DRBs) per user/device. Each radio bearer is characterized by an individual QoS profile/flow. Multiple services per user/device can also be supported by mapping multiple services to a single bearer, if the QoS is the same for these services. See more details on QoS in 8.2.5.1.12 and 8.2.5.1.13. |
| 8.2.5.1.17.3 | Does the RIT support multiple voice and/or video codecs? Provide details. For E-UTRA satellite access component RIT (common principles) The RIT could support various voice and video codecs, as desired. In fact, the radio interface technology (fully IP-based) is mostly agnostic to such codecs, and capable of accommodating diverse range of codec types, rates and operation (fixed/dynamic/adaptive). This enables support for all main codecs used/defined today (e.g. AMR-NB/WB, EVS), as well as the capability to support more enhanced codecs that may be defined in future. |
| 8.2.5.1.18 | Privacy, authorization, encryption, authentication and legal intercept schemes |
| 8.2.5.1.18.1 | Any privacy, authorization, encryption, authentication and legal intercept schemes that are enabled in the radio interface technology should be described. Describe whether any synchronization is needed for privacy and encryptions mechanisms used in the RIT. Describe how the RIT may be protected against attacks, for example: – man in the middle; – replay; – denial of service. Security and privacy in the RIT will be provided at a level at least as high as that of “LTE-advanced”. This will be assured by reusing mechanisms of LTE-advanced (see [TS33.401]). A thorough analysis of the security requirements and different solution alternatives for the E-UTRA satellite access architecture has been performed, assuring that attacks based on interception, modification, replay, impersonation/man-in-the-middle, denial-of-service, etc, are properly countered. The security solution will thus provide (among other things): strong mutual subscriber/network authentication, user identity protection, strong encryption of user traffic, and strong encryption and integrity protection of signaling. Protection is applied over the air interface as well as to the internal backhaul/RAN-CN interfaces. Particular care has been taken to mitigate threats related to radio SAN security, e.g. protecting the network against hostile “capture” of SANs. Lawful intercept is provided via CN nodes. Furthermore, Control Plane optimisations for Internet of Things traffic have been defined where data is allowed to be transferred via the UE-CN control plane through Non Access Stratum (NAS) messages. Data transferred in this manner is protected in the same manner as control signalling i.e. with encryption and integrity protection applied. |
| 8.2.5.1.19 | Frequency planning |

TABLE 2 (continued)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.19.1 | How does the RIT support adding new cells or new RF carriers? Provide details. For E-UTRA satellite access component RIT: 504 physical cell identities are supported. Thus, theoretically 504-cell reuse is realized. Actual cell deployment is operation specific. Self configuration is also supported. |
| 8.2.5.1.20 | Interference mitigation within radio interface |
| 8.2.5.1.20.1 | Does the proposal support Interference mitigation? If so, describe the corresponding mechanism. For E-UTRA satellite access component RIT: Static inter-cell interference mitigation is supported by means of e.g. frequency reuse, soft frequency reuse, and reuse partitioning. Inter-cell interference mitigation is supported by means of exchanging interference measurements and scheduling decisions between SANs, see also 8.2.5.1.20.2 below. For eMTC and NB-IoT a repetition-based transmission scheme is supported where coherent reception of repeated transmission supports suppression of interference. Cell and user-based scrambling is also implemented to support this mechanism. eMTC supports frequency hopping to reduce the impact from interference. |
| 8.2.5.1.20.2 | What is the signalling, if any, which can be used for inter-beam interference mitigation? For E-UTRA satellite access component RIT: To aid inter-cell interference mitigation, the RIT defines three indicators exchanged between SANs: The High-interference Indicator (HI) which provides information to neighbouring cells about the part of the cell bandwidth upon which the cell intends to schedule its cell-edge user, the Overload Indicator (OI) which provides information on the uplink interference level experienced in each part of the cell bandwidth and Relative Narrowband TX Power (RNTP) indicator, which provides information on the downlink transmission power. |
| 8.2.5.1.20.3 | Link level interference mitigation Describe the feature or features used to mitigate inter-symbol interference. For E-UTRA satellite access component RIT: Time and frequency synchronization to the DL and UL frame structures in combination with the use of a cyclic prefix OFDM transmission in both UL (with or without transform precoding) and DL, provides robustness against intersymbol interference. See also answer to 8.2.5.1.20.4 |
| 8.2.5.1.20.4 | Describe the approach taken to cope with multipath propagation effects (e.g. via equalizer, rake receiver, cyclic prefix, etc.). For E-UTRA satellite access component RIT: On the downlink, the use of OFDM transmission, in combination with a cyclic prefix, provides inherent robustness to time-dispersion/frequency-selectivity on the radio channel. On the uplink, time-dispersion/frequency-selectivity on the radio channel can be handled by receiver-side equalization. The detailed equalization approach is implementation dependent. Examples of equalization approaches include frequency-domain linear equalization and Turbo equalization. The use of cyclic prefix also for the uplink may simplify the equalizer implementation. |
| 8.2.5.1.20.5 | Diversity techniques Describe the diversity techniques supported in the MS and at the BS, including micro diversity and macro diversity, characterizing the type of diversity used, for example: – Time diversity: repetition, Rake-receiver, etc. – Space diversity: multiple sectors, etc. – Frequency diversity: frequency hopping (FH), wideband transmission, etc. – Code diversity: multiple PN codes, multiple FH code, etc. – Multi-user diversity: proportional fairness (PF), etc. – Other schemes. Characterize the diversity combining algorithm, for example, switched diversity, maximal ratio combining, equal gain combining. Provide information on the receiver/transmitter RF configurations, for example: – number of RF receivers; – number of RF transmitters. For E-UTRA satellite access component RIT: The RIT provides the following means for diversity: - Space diversity by means of multiple transmit and receiver antennas - Number of TX antenna ports: Up to 4 (DL), up to 1 (UL) logical antenna ports are defined where the mapping of logical antenna ports to physical antennas is an implementation specific configuration. - Number of RX antenna ports: 1 - Frequency diversity by means of wide overall transmission bandwidth. Possibility for uplink frequency hopping on a slot basis and downlink frequency-distributed transmission - Time diversity by means of fast retransmissions |
| 8.2.5.1.21 | Synchronization requirements |
| 8.2.5.1.21.1 | Describe RIT’s timing requirements, e.g.: – is synchronization between inter-beam signals or inter-satellite signals required? Provide precise information, the type of synchronization, i.e. synchronization of carrier frequency, bit clock, spreading code or frame, and their accuracy; – is space segment-to-network synchronization required? State frequency and timing accuracy of space segment and user equipment transmit signal. For E-UTRA satellite access component RIT: Common general aspects Tight SAN-to-SAN synchronization is not required. Likewise, tight SAN-to-network synchronization is not required. The SAN shall support a logical synchronization port for phase-, time- and/or frequency synchronization, e.g. to provide. - FDD time domain inter-cell interference coordination. A certain RAN-CN Hyper SFN synchronization is required in case of extended Idle mode DRX. Some accuracy requirements SAN transmit signals accuracy: - E-UTRA satellite access: - Frequency accuracy (wide area SAN): within ±0.05 ppm, observed over 1ms For more information please refer to - E-UTRA satellite access: [36.401], [36.133], [36.108]. |
| 8.2.5.1.21.2 | Describe the synchronization mechanisms used in the proposal, including synchronization between a user terminal and a satellite access node (base station). For NR and E-UTRA satellite access component RIT (common principles) Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the physical layer Cell ID of that cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS). PSS is used (at least) for initial symbol boundary, cyclic prefix, sub frame boundary, initial frequency synchronization to the cell. SSS is used for radio frame boundary identification. PSS and SSS together used for cell ID detection. Other synchronization mechanisms are defined e.g. for Radio link monitoring, Transmission timing adjustments, Timing for cell activation / deactivation. - For NB-IOT NB-IoT cell search/synchronization is based on dedicated narrowband signals transmitted in the downlink: the primary and secondary narrowband synchronization signals. See more information in: - NR: [38.213] sub-clause 4 and [38.211] sub-clause 7.4.2. - E-UTRA satellite access: [36.213] and [36.211]. - For eMTC In addition to support for the PSS and SSS, eMTC supports the Resynchronization Signal (RSS) which facilitates an efficient resynchronization to the serving cell. |
| 8.2.5.1.22 | Link budget template Proponents should provide the link budget results to this description template for the environments supported in the RIT. The information is provided with link budget template. |
| 8.2.5.1.23 | Other items |
| 8.2.5.1.23.1 | Coverage extension schemes Describe the capability to support/coverage extension schemes, such as relays or repeaters. For E-UTRA satellite access component RIT: E-UTRA satellite access supports the use of the following mechanisms to improve the coverage - Semi-persistent scheduling can be used for repetition uplink transmission; and HARQ-less repetition can be used for downlink transmission, both are beneficial for coverage. - E-UTRA satellite access also supports the use of different types of repeater (amplify-and-forward) functionality. However, the details of such functionality is outside the scope of the specification as the use of repeaters is transparent to both the UE and the network. For NB-IoT / eMTC, - DFT-spreading and Pi/2 BPSK and Pi/4 QPSK modulation in NB-IoT for reduced PAPR for better coverage. - Support for single sub-carrier (NB-IoT) and sub-PRB (eMTC) uplink transmission to increase connection density in extended coverage - Repetition of a transmission for both control and data can be used for better coverage. High-aggregation level downlink control is possible for better coverage. |
| 8.2.5.1.23.2 | Self-organization Describe any self-organizing aspects that are enabled by the RIT/SRIT. For E-UTRA satellite access component RIT: Support for Self Organizing Networks is an integrated part of E-UTRA satellite access. Several use cases that could benefit from SON have been introduced and the work is continuing. E-UTRA satellite accesscurrently supports the following Self-Organizing Network (SON) functions: (Details are provided in [36.300], [36.413], [36.423], [36.314]) Self-configuration: - Dynamic configuration of S1 and X2; the mechanism allows an eNB to establish an S1 interface towards the MME and/or an X2 interface towards another eNB.([36.423, Sec8.3.3, 8.3.5] [36.413, Sec8.7.3, 8.7.4, 8.7.5]) – PCI selection; the mechanism allows an eNB to select its Physical Cell Identity (PCI). The selection can be based either on a centralized or distributed PCI assignment algorithm [36.300, Sec 22.3.5] – Automatic neighbor discovery: the mechanism allows an eNB to learn information on its neighbors. The discovery mechanism can utilize the assistance of the UE (aka ANR funtion [36.300, Sec. 22.3.3, 22.3.3]) as well as the exchange of information over the network interfaces ([36.423; Sec 8.3.3, 8.3.5] as well as the radio resource control information[36.300; Sec 5.5.2, 5.5.3, 5.5.4, 5.5.5, 6.3.5]). – TNL address discovery: the mechanism allows an eNB to determine the TNL address of the neighbor candidate eNB. The discovery mechanism can utilize of the ANR function(aka ANR funtion [36.300, Sec. 22.3.6]) as well as the exchange of information over the network interfaces ([38.413; Sec8.15, 8.16, 9.1.16, 9.1.17 ) Self-optimisation: – Mobility load balancing: the mechanism allows an eNB to distribute cell load evenly among cells or to transfer part of the traffic from congested cells. ([36.300, Sec 22.4.1] [36.423, 8.3.6, 8.3.7,8.3.8]) – Mobility Robustness optimization for eMTC: the mechanism is to detect radio link connection failure that occurs due to Too Early or Too Late Handovers, or Handover to Wrong cell. ([36.300, Sec 22.4.2] [36.423, Sec 8.3.9, 8.3.10] [36.331, Sec 5.3.11]) – RACH optimization: the mechanism is supported by UE reported information and by PRACH parameters exchange between eNBs to optimize some RACH configurations. ([36.300, Sec 22.4.3] [36.423, Sec 8.3.3, 8.3.5] [36.331, Sec 5.6.5.3]). – Energy Saving: the mechanism is to reduce operational expenses through energy savings ([36.300, Sec 22.4.4]). |

TABLE 2 (end)

| Item | Item to be described |
| --- | --- |
| 8.2.5.1.23.3 | Describe the frequency reuse schemes (including reuse factor and pattern) for the assessment of cell spectrum efficiency, cell edge user spectral efficiency. Uncoordinated frequency reuse factor of 1 and frequency reuse factor of 3 are used in the performance evaluations. |
| 8.2.5.1.23.4 | Is the RIT an evolution of an existing IMT-Advanced technology? Provide details. The SRIT is an evolution of the 3GPP SRIT For E-UTRA satellite access component RIT: This RIT is part of a 3GPP release of E-UTRA satellite access. It is an evolution and enhancement of previous E-UTRA satellite access releases that are already part of the IMT-Advanced technologies, see ITU-R Recommendation M.2514-0. |
| 8.2.5.1.23.5 | Does the proposal satisfy a specific spectrum mask? Provide details. (This information is not intended to be used for sharing studies.) For E-UTRA satellite access component RIT: Yes. UE: For carrier transmission the spectrum mask is specified in terms of a normative (general) spectrum emission mask [36.102, section 6.5A.3.2 and 6.5B.3.2 for eMTC and NB-IoT respectively] and an additional spectrum mask [36.102, section 6.5A.3.3 and 6.5B.3.3 for eMTC and NB-IoT respectively]. This additional spectrum emission mask which is signaled by the network to the UE as a normative requirement can be used to address a specific regional regulatory requirement, a frequency band specific requirement, a roaming requirement and a specific deployment scenario. This additional spectrum emission mask can be used to support the many different sharing requirements in terms of co-existence for a global roaming terminal. BS: For carrier transmission, spectrum mask requirements are defined in [36.108], section 6.6.3. in form of operating band unwanted emission limits. These Operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band. The unwanted emission limits in the part of the downlink operating band that falls in the spurious domain are consistent with ITU-R Recommendation SM.329. |
| 8.2.5.1.23.6 | Describe any user terminal power saving mechanisms used in the RIT. For E-UTRA satellite access component RIT: Multiple features facilitating device energy efficiency have been specified for E-UTRA satellite access Rel-15, including Discontinuous reception (DRX) in RRC connected mode and RRC idle mode, Extended Discontinuous reception (DRX) in RRC idle mode, Paging with Wake-Up Signal in idle mode, and Power Saving Mode (PSM) operation in idle mode. |
| 8.2.5.1.23.7 | Simulation process issues Describe the methodology used in the analytical approach. Proponent should provide information on the width of confidence intervals of user and satellite system performance metrics of corresponding mean values, and evaluation groups are encouraged to provide this information. As described in Section 7.1 of M.2514-0, system simulations are iterated over M independent ‘drops’ of user locations. Statistics, mean and percentiles, are calculated over all drops, and confidence intervals are estimated by comparing the results of the different drops. The number of drops is up to each evaluator. |
| 8.2.5.1.24 | Other information Please provide any additional information that the proponent believes may be useful to the evaluation process. [E-UTRA for satellite access has the capability to support different satellite deployment scenarios. The following reference scenarios/characteristics were considered as part of the study in [36.763]. The outcome of the study led to the specification development. More information can be found in [ 36.763]. Scenarios; GEO based non-terrestrial access network - scenario A; LEO based non-terrestrial access network -Scenario B & C; MEO based non-terrestrial access network -Scenario D. Orbit type; station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point; circular orbiting at low altitude around the earth; circular orbiting at medium altitude around the earth. Altitude; 35,786 km; 600 km, 1,200 km; 10,000 km. Frequency Range; 450 - 6000 MHz). Payload; Transparent type; Transparent Type; Transparent type. Earth-fixed beams; Yes; Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite); Scenario D: The beams move with the satellite. Max beam footprint size (edge to edge) regardless of the elevation angle; 3500 km (NOTE 3); 1000 km (NOTE 2); 4018 km. Max Round Trip Delay (propagation delay only); 541.46ms (service and feeder links); 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links); 186.9 ms (service and feeder links). Max Doppler shift (earth fixed user equipment) (NOTE 4); 0.93 ppm; 24 ppm (600km) 21ppm(1200km); 7.5 ppm. NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: The Maximum beam footprint size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam. NOTE 4: Max Doppler shift in the absence of any device pre-compensation of satellite Doppler shift on the service link.. |

#### 8.2.5.2	Description template – link budget template

Link budget analysis may depend on the specific system configuration of the satellite systems intended to use the proposed RIT/SRIT. Therefore, proponents should provide appropriate link budget analysis and results, considering their targeted satellite system configurations, as part of the description template. The link budgets should refer to specific test environments to be evaluated for the RIT/SRIT, in order to ease the conduct of the evaluation process by the evaluation groups.

### 8.2.6	Compliance templates

This section provides templates for the responses that are needed to assess the compliance of a candidate satellite radio interface with the requirements of IMT-2020.

#### 8.2.6.1	Compliance template – service template

| Service-related minimum capabilities within the satellite radio interface(s) | Evaluator’s comments |
| --- | --- |
| Support of a wide range of services Does the proposal support a wide range of services? Yes/No The SRIT can support eMBB-s, HRC-s and mMTC-s usage scenarios. | The assessment of service requirement follows the evaluation method as defined in Section 8 in Report ITU-R M.2514-0. |

#### 8.2.6.2	Compliance template – spectrum template

| Spectrum capability requirements | Evaluator’s comments |
| --- | --- |
| Spectrum bands Is the proposal able to utilize at least one band identified for IMT? Yes/No Specify in which band(s) the candidate satellite radio interface(s) can be deployed | The supported frequency bands identified for IMT by NR satellite access and E-UTRA satellite access component RITs are provided in item 8.2.5.1.8.3 in the characteristics template for the SRIT. See the table for frequency range 1 (FR1) for NR satellite access component RIT, and the table for E-UTRA satellite access component RIT |

#### 8.2.6.3	Compliance template – technical performance template

| Minimum technical requirements items | Category |  |  | Required value | Value (listed for each component RIT) | Requirement met? | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Usage scenario | Test environment | Downlink or uplink |  |  |  |  |
| Peak data rate | eMBB-s | N/A | Uplink | 2 Mbit/s |  |  |  |
|  |  |  | Downlink | 70 Mbit/s |  |  |  |
| Peak spectral efficiency | eMBB-s | N/A | Uplink | 1.5 bit/s/Hz |  |  |  |
|  |  |  | Downlink | 3 bit/s/Hz |  |  |  |
| User experienced data rate | eMBB-s | Rural | Uplink | 100 kbit/s |  |  |  |
|  |  |  | Downlink | 1 Mbit/s |  |  |  |
| 5th percentile user spectral efficiency | eMBB-s | Rural | Uplink | 0.003 bit/s/Hz |  |  |  |
|  |  |  | Downlink | 0.03 bit/s/Hz |  |  |  |
| Average spectral efficiency | eMBB-s | Rural | Uplink | 0.1 bit/s/Hz |  |  |  |
|  |  |  | Downlink | 0.5 bit/s/Hz |  |  |  |
| Area traffic capacity | eMBB-s | Rural | Uplink | 1.5 kbit/s/km² |  |  |  |
|  |  |  | Downlink | 8 kbit/s/km² |  |  |  |
| User Plane latency | eMBB-s | N/A | N/A | 10 ms |  |  |  |
| Control Plane latency | eMBB-s | N/A | N/A | 40 ms |  |  |  |
| Connection density | mMTC-s | Rural | N/A | 500 devices/km² |  |  |  |
| Energy efficiency | eMBB-s | N/A | N/A | High sleep ratio and long sleep duration |  |  |  |
| Reliability | HRC-s | Rural | N/A | 1-10−3 |  |  |  |
| Mobility – UE speed | eMBB-s | Rural | N/A | 250 km/h |  |  |  |
| Mobility - Traffic channel link data rate | eMBB-s | Rural | N/A | 0.005 bit/s/Hz |  |  |  |
| Mobility interruption time | eMBB-s | N/A | N/A | 50 ms |  |  |  |
| Bandwidth | N/A | N/A | N/A | At least up to and including 30 MHz | NR satellite access: 30MHz E-UTRA satellite access: 200kHz, 1.4MHz | Yes, SRIT meets the requirement |  |

Developed by 3GPP as 5G satellite access, Release 17 and beyond.