3GPP TSG-RAN Meeting #101	RP-232386

# Bangalore, India, September 11-15, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0511 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SCell PRACH power scaling for UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Nokia, Nokia Shanghai Bell, Samsung, Intel, ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2023-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When there is overlap between periodic SRS transmission on PCell and a PDCCH order triggered PRACH transmission on SCell, current text in sub-clause 7.5 of 38.213 can be interpreted to imply that UE should prioritize periodic SRS over PDCCH order triggered PRACH during power allocation for power limited case. However, the opposite, i.e., PDCCH order triggered PRACH having higher priority over periodic SRS is the technically correct behavior. The PRACH transmission on SCell is more critical (it is used to get initial TA estimate for the SCell) than a particular instance of periodic SRS. It is also ‘aperiodic’ and general principle for power scaling behavior is that aperiodic transmissions are prioritized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in sub-clause 7.5 that UE supporting capability <FG xyz> should prioritize PDCCH order triggered PRACH on SCell over periodic/semi-persistent SRS for power allocation in power limited case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs may scale down/drop SCell PRACH overlapping with PCell periodic SRS. Frequent dropping of PRACH on SCell would have significant negative impact on SCell availability for UL CA operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.306 (RP-232387) , 38.331 (RP-232388) |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis The CR has isolated impact and would only affect colliding PDCCH ordered PRACH on SCell and/or periodic/semi-persistent SRS on another cell for a UE supporting capability prioSCellPRACH-OverSemiPersistentAndPeriodicSRS and configured with UL CA. If UE implements this CR and gNB does not, - SCell PRACH reception is not negatively impacted and may be improved. - P-SRS reception is not impacted for gNBs avoiding collision between P-SRS on PCell and PRACH on SCell. P-SRS reception could be at lower-than-expected power for the P-SRS instance colliding with SCell PRACH for gNBs that do no avoid collision between P-SRS on PCell and PRACH on SCell but this is not expected to have any significant effect on performance. If gNB implements this CR and UE does not, - SCell PRACH and P-SRS reception would be same as legacy. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR |  |  |  |  |  |  |  |  |

## 7.5	Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion  would exceed , where  is the linear value of  in transmission occasion  as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to  for that frequency range in every symbol of transmission occasion . When determining a total transmit power for serving cells in a frequency range in a symbol of transmission occasion , the UE does not include power for transmissions starting after the symbol of transmission occasion . The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot.

-	PRACH transmission on the PCell

-	PUCCH or PUSCH transmissions with higher priority index according to clause 9

-	For PUCCH or PUSCH transmissions with same priority index

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	If the UE indicates capability prioSCellPRACH-OverSemiPersistentAndPeriodicSRS

-	Aperiodic SRS transmission or PRACH transmission on a serving cell other than the PCell

-	Semi-persistent and/or periodic SRS transmission

-	otherwise,

-	SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell

In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell. In case of same priority order and for operation with two UL carriers, the UE prioritizes power allocation for transmissions on the carrier where the UE is configured to transmit PUCCH. If PUCCH is not configured for any of the two UL carriers, the UE prioritizes power allocation for transmissions on the non-supplementary UL carrier.