3GPP TSG RAN Meeting #101			RP-232400

Bangalore, India 11th – 15th Sep, 2023

Agenda item:		8A.2.12.4

Source:	China Unicom

Title:	Positioning enhancements for FR2 in Rel-19

Document for:		Discussion

Introduction

This contribution aims to discuss how to enhance positioning performance (e.g. accuracy) and reduce signalling overhead for FR2 band in Rel-19. Whether to support positioning in FR2-2 band is also discussed in this paper.

Discussion

Millimeter wave is one of the key technologies of 5G-Advanced, users can be served with good millimeter positioning service according to the advantages of millimeter, including big bandwidth, and narrow beam. Rel-16/17 supports mmWave positioning with single-TRP by RTT and AOA/AoD or multiple-TRP using TDOA methods. It’s hard to achieve 0.1m level positioning accuracy. The issues are how to reduce beam sweeping overhead, and it’s complex when coordinating multi-UE beam alignment across multiple BSs.

Millimeter wave is easy to be blocked, but in indoor scenarios with rich reflection environment, as well as outdoor scenarios with NCR, reflected signals of millimeter wave can also have better signal quality, which allows the terminal to receive direct path signals and reflected path signals with good RSRP at the same time. The scenarios are listed as below:

- Scenario 1: Indoor scenario with ground reflections

- Scenario 2: Outdoor/Indoor scenario with NCR

Figure 1 Indoor scenario with ground reflections

Figure 2 Outdoor/Indoor scenario with NCR

In comparison to multi-TRP positioning for FR2, single-TRP positioning offers the advantage of releasing resources without the need for a Tx beam sweeping procedure. This eliminates restrictions in the deployment and updating of base stations, making it convenient to integrate single-TRP positioning with existing 5G networks for widespread implementation. Various technologies, such as RTT, are commonly employed in single-TRP positioning. However, these technologies can introduce positioning errors due to clock drift and group delay in principle. For instance, a clock drift of 10 ppm in the communication link can result in a positioning error of 0.3 m every 10 ms. Similarly, a group delay jitter of 1 ns in the UE can also lead to a 0.3 m positioning error, significantly reducing positioning accuracy. Therefore, it is necessary to explore enhanced measures to mitigate these error effects. One such measure is multi-path assisted positioning, which can effectively eliminate clock drift from the origin. The group delay can also be offset by calculating multi-path link delays simultaneously. Obviously, the utilization of ground reflections or NCR can be directly applied in multi-path positioning.

In addition, owing to the high frequency feature of FR2-2, it is possible to achieve precise positioning with minimal interference. As the propagation range of FR2-2 is very limited, single-TRP positioning is more suitable for indoor environments, or applied in some outdoor scenarios with coverage enhanced by NCR. It is worthwhile to support the implementation of the new SCS/BW in FR2-2 positioning.

Based on the scenarios and gains analysis, it’s proposed to study and support FR2-related positioning features in Rel-19. So it’s proposed as below:

Proposal: It is proposed that RAN#101 confirm the requirement of the positioning enhancements targeting FR2, including multi-path assisted positioning, and new SCS/BW supported in FR2-2 could be studied and specified (if needed) in Rel-19.

Conclusions

Based on the discussion above, the following proposal is made:

Proposal: It is proposed that RAN#101 confirm the requirement of the positioning enhancements targeting FR2, including multi-path assisted positioning, and new SCS/BW supported in FR2-2 could be studied and specified (if needed) in Rel-19.

References

- RWS-230325 NR positioning enhancements in Rel-19