3GPP TSG RAN Meeting #101	RP-232409

Bengaluru, India, September 11-15, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848 on Deployment scenarios and their characteristics

Agenda item:	9.2.2

Document for:	Agreement

- Introduction

This pCR proposes text to add Deployment scenarios and their characteristics on Ambient IoT into TR 38.848 v0.2.0.

Background: Rapporteur proposes to add the ‘Recommendation for first WG study’ column as a first prioritization within the ‘Description’ based on companies’ views in their papers, where they are provided.

Note that (as mentioned in our company paper) this recommendation does not preclude later studies on more uses of a deployment scenario, and nor does it indicate any particular Release for the “first WG study”, nor for the deployment scenario as a whole.

- Proposed changes

# <Unchanged text is omitted>

### 4.2.1	Connectivity topologies

The following connectivity topologies for Ambient IoT networks and devices are defined for the purposes of the study. In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology. The links in each topology may be bidirectional or unidirectional.

BS, UE, assisting node, or intermediate node could be multiple BSs or UEs, respectively. The mixture of indoor and outdoor placement of such nodes is regarded as a network implementation choice. Account would need to be taken of potential impact on device or node complexity. In the connectivity topologies, this does not imply the existence of multi-hop assisting or intermediate nodes.

# <Unchanged text is omitted>

## 4.2	Deployment scenarios and connectivity topologies

Deployment scenarios for Ambient IoT have been studied on the basis of a list of characteristics, and the representative use case(s) applicable to a scenario. The possible descriptions of the characteristics are as follows:

Table 4.2-1: Characteristics of deployment scenarios

| Characteristic | Possible description entries |
| --- | --- |
| Environment (of the device) | Indoor Outdoor Indoor or outdoor |
| Basestation characteristic (if any) | Macro-cell-based deployment Micro-cell-based deployment Pico-cell-based deployment None |
| Connectivity topology | See section 4.2.1 |
| Spectrum | Licensed FDD Licensed TDD Unlicensed Note: In each connectivity topology of the study, if a BS is present, it is assumed to use licensed spectrum |
| Coexistence with existing 3GPP technologies | Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type. Deployed on new sites without an assumption of an existing 3GPP deployment. |
| Traffic assumption | Device-terminated (DT) Device-originated (DO) DO traffic includes DO autonomous (DO-A), and DO device-terminated triggered (DO-DTT) |
| Device characteristic | See Section 4.3: Device A Device B Device C |

The study has considered Ambient IoT deployments in-band to NR, in guard-band of NR, and in standalone band from NR.

<Unchanged text is omitted>

### 4.2.2	Deployment scenarios

#### 4.2.2.1	Deployment scenario 1: Device indoors, basestation indoors

With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1.

Table 4.2.2.1-1: Characteristics of deployment scenario 1

| Applicable representative use cases | Characteristics | Description (NOTE 1) | Recommendation for first WG study |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell | Micro-cell BS |
|  | Connectivity topology | Topology (1), (2), (3) | Topology (1) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site | Co-site or new site |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device A or Device B or Device C | Device A or Device B |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.2	Deployment scenario 2: Device indoors, basestation outdoors

With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.1-2.

Table 4.2.2.2-1: Characteristics of deployment scenario 2

| Applicable representative use cases | Characteristics | Description (NOTE 1) | Recommendation for first WG study |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor | Indoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS | Macro-cell BS |
|  | Connectivity topology | Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor | Topology (1) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site | Co-site |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2), Device B may support Topology (2), (3) | Device C |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.3	Deployment scenario 3: Device indoors, UE-based reader

With Ambient IoT device indoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.1-3.

Table 4.2.2.3-1: Characteristics of deployment scenario 3

| Applicable representative use cases | Characteristics | Description (NOTE 1) | Recommendation for first WG study |
| --- | --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor | Indoor |
|  | Basestation characteristic (if any) | None | None |
|  | Connectivity topology | Topology (4) | Topology (4) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Unlicensed |
|  | Coexistence with existing 3GPP technologies | NA | NA |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device A or Device B or Device C | Device A or Device B |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.4	Deployment scenario 4: Device outdoors, basestation outdoors

With Ambient IoT device outdoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.1-4.

Table 4.2.2.4-1: Characteristics of deployment scenario 4

| Applicable representative use cases | Characteristics | Description (NOTE 1) | Recommendation for first WG study |
| --- | --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor | Outdoor |
|  | Basestation characteristic (if any) | Macro- or Micro- cell BS | Macro-cell BS |
|  | Connectivity topology | Topology (1), (2), (3) | Topology (1) |
|  | Spectrum | Licensed FDD, licensed TDD, or unlicensed. | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site | Co-site |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device C may support Topology (1), (2), (3), Device A may support Topology (2),Device B may support Topology (2), (3) | Device C |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

#### 4.2.2.5	Deployment scenario 5: Device outdoors, UE-based reader

With Ambient IoT device outdoors and UE-based reader, this deployment scenario is characterized according to Table 4.2.2.1-5.

Table 4.2.2.5-1: Characteristics of deployment scenario 5

| Applicable representative use cases | Characteristics | Description (NOTE 1) | Recommendation for first WG study |
| --- | --- | --- | --- |
| Outdoor inventory Outdoor sensor Outdoor positioning Outdoor command | Environment (of device) | Outdoor | Outdoor |
|  | Basestation characteristic (if any) | None | None |
|  | Connectivity topology | Topology (4) | Topology (4) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed | Unlicensed |
|  | Coexistence with existing 3GPP technologies | NA | NA |
|  | Traffic assumption | DT and DO | DT and DO |
|  | Device characteristic | Device A or Device B or Device C | Device C |

NOTE 1: Descriptions may not be applicable for some Devices (A, B).

# <Unchanged text is omitted>