3GPP TSG RAN Meeting #101	RP-232410

Bengaluru, India, September 11-15, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848 on Ambient IoT device characteristics and categorization

Agenda item:	9.2.2

Document for:	Agreement

- Introduction

# This pCR proposes text on Ambient IoT device characteristics and categorization into TR 38.848 v0.2.0.

- Proposed changes

# <Unchanged text is omitted>

## 4.3	Device categorization

Ambient IoT devices are characterized in the study according to their energy storage capacity, and capability of generating RF signals for their transmissions.

The study considers that a device has either:

- No energy storage at all; or

- Limited energy storage.

Relying on these storage capacities, the study considers the following set of Ambient IoT devices:

- Device A: No energy storage, no independent signal generation/amplification, i.e. backscattering transmission.

- Device B: Has energy storage, no independent signal generation, i.e. backscattering transmission. Use of stored energy can include amplification for reflected signals.

- Device C: Has energy storage, has independent signal generation, i.e., active RF components for transmission.

A limited energy storage can be different among implementations within Device B or implementations within Device C, and different between Device B and Device C. Such storage is expected to be order(s) of magnitude smaller than an NB-IoT device would typically include.

Device A, B, and C are able to demodulate control, data, etc from the relevant entity in RAN according to connectivity topology.

# <Unchanged text is omitted>