3GPP TSG RAN Meeting #101                 	RP-232411

Bengaluru, India, September 11-15, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848 on RAN design targets for Ambient IoT

Agenda Item:	9.2.2

Document for:	Agreement

# Introduction

This pCR proposes text on design targets for Ambient IoT in TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN” V0.2.0.

Background: Rapporteur has provided options for discussion and downselection in sections 5.3 (Coverage) and 5.6 (Latency), as companies have taken various routes to this analysis which RAN should discuss.

# Proposed changes

<Unchanged text is omitted>

# 5	RAN design targets

## 5.1	Device power consumption

For Device A, the power consumption target during transmitting/receiving is ≤ 1 μW or ≤ 10 μW,

For Device B, the target during transmitting/receiving is such that:

-	Device A power consumption ≪ Device B power consumption < Device C power consumption; or

-	Device A power consumption ≤ Device B power consumption < Device C power consumption.

The device power consumption for Device C is ≤ 1 mW to ≤ 10 mW.

## 5.2 	Device complexity

For Device A, the complexity target is to be comparable to UHF RFID ISO18000-6C (EPC C1G2).

For Device B, the target is such that:

-	Device A complexity < Device B complexity < Device C complexity.

For Device C, the complexity target is to be orders-of-magnitude lower than NB-IoT.

## 5.3	Coverage

The coverage target is represented by the maximum distance:

- Between Ambient IoT device and basestation in Topology (1) and (3)

- Between Ambient IoT device and intermediate or assisting node other than UE in Topology (2) and (3), respectively

- Between Ambient IoT device and UE in Topology (2), (3) and (4).

Details such as sensitivity, BLER, etc. are for WG expertise to study further.

The design target of coverage is:

[Option 1: By the type of node directly transmitting to or receiving from the Ambient IoT device]

- The maximum distance between Ambient IoT device and BS supports continuous coverage based on the typical inter-site distance (ISD) of legacy 3GPP basestation.

- The maximum distance between Ambient IoT device and UE (including intermediate/assisting node) is at least 10 m in e.g. D2D LOS channel.

- The maximum distance between Ambient IoT device and the intermediate/assisting node other than UE is to be between the above two targets.

[Option 2: by indoor / outdoor, grouping different Devices into a range that WGs can sub-select within]

- the maximum distance of 10 – 50 m for indoor

- the maximum distance of 50 – 500 m for outdoor

NOTE: Different target values within these ranges may apply to different devices A/B/C and deployment scenarios 1-5.

Note: For Device A & B, the emitter-to-tag distance should be reported as part of the assessment.

## 5.4	User experienced data rate

The user experienced data rate target is, at least for the uplink, maximum not less than 5 kbps, and minimum not less than 0.1 kbps.

## 5.5	Maximum message size

The design target of maximum message size is 1000 bits to be received by and 1000 bits to be transmitted from the Ambient IoT device.

RAN1/RAN2 can refine as needed for TB size design.

## 5.6	Latency

The end-to-end maximum latency target, as defined in TR 22.840 is:

- 	Longer latency target: 10 seconds

-	Shorter latency target: 1 second

A use case is assigned to a latency target according to TR 22.840. RAN WGs can refine a definition of latency suitable for their work within the above.

Note: The time for charging the Ambient IoT device is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implemention issue only, and is excluded from consideration in RAN WGs.

## 5.7	Positioning accuracy

The design target of positioning accuracy is:

- 3 meters @ 90%indoor location by cellular network.

- Several tens of meters @ 90%outdoor location by cellular network.

## 5.8	Connection/device density

According to the consolidate potential KPIs in TS22.840, the maximum connection density target is:

- 150 devices per 100 m2 for indoor scenarios.

- 20 devices per 100 m2 for outdoor scenarios.

## 5.9 	Moving speed of device

The design target of moving speed of Ambient IoT device is 10 km/h for both indoor and outdoor scenarios.

Note X: Absolute speed is used in Topology (1), (2) and (3). Relative speed is used in Topology (4)

<Unchanged text is omitted>