3GPP TSG RAN Meeting #101									RP-232414

Bengaluru, India, September 11-15, 2023

Agenda item:	9.2.2

Source: 	Huawei, HiSilicon, CMCC

Title: 	Miscellaneous corrections pCR for TR 38.848 on Ambient IoT

Document for:	Discussion/Decision

# Introduction

This pCR proposes editorial changes to fix miscellaneous typos in TR 38.848 v0.2.0.

# Proposed changes

<Unchanged text is omitted>

# 4	Deployment scenarios, use cases, services

### 4.1	Use cases/services

### 4.1.1	Representative use cases

<Unchanged text is omitted>

This resulted in the following mapping from SA1 use cases and traffic scenarios onto RAN rUCs:

Table 4.1.1-1: Mapping between RAN representative use cases and SA1 use cases

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| rUC2: Indoor sensor | 5.6 Smart homes 5.13 Base station machine room environmental supervision 5.15 Smart laundry 5.20 Smart agriculture 5.23 Smart pig farm 6.2 Cow stable |
| rUC3: Indoor positioning | 5.8 Finding Remote Lost Item 5.9 Location service 5.10 Ranging in a home 5.12 Personal belongings finding 5.14 Positioning in shopping centre 5.21 Museum Guide |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |

<Unchanged text is omitted>

#### 4.2.1.2	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

Figure 4.2.1.2-1: Topology 2

In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and basestation. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers Ambient IoT data and/or signalling between BS and the Ambient IoT device.

<Unchanged text is omitted>

## 5.1	Device power consumption

For Device A, the power consumption target during transmitting/receiving is [≤ 1 μW] or [≤ 10 μW],

For Device B, the target during transmitting/receiving is such that:

-	Device A power consumption ≪ Device B power consumption < Device C power consumption; or

-	Device A power consumption ≤ Device B power consumption < Device C power consumption.

The device power consumption during transmitting/receiving for Device C is ≤ 1 mW to ≤ 10 mW.

<Unchanged text is omitted>