3GPP TSG RAN Meeting #101												RP-232416

Bengaluru, India, September 11-15, 2023

Agenda Item:	8A.2.12.1

Source:	Huawei, HiSilicon

Title:	Scope of LP-WUS in Rel-19

Document for:	Discussion and decision

# Introduction

In this paper, we present our views on scope of LP-WUS in Rel-19.

# Waveform of LP-WUS

In RAN1 #114, the RAN1 study of LP-WUS was finished and concluded in TR 38.869 [1].

In  [1], the performance and characteristic for three kinds of waveforms (i.e. OOK-1/4, FSK-2 and OFDMA) are summarized. It is noted that OOK-1/4 is the most robust to frequency error, and FSK-2 has good robustness to frequency error and moderate robustness to timing error. And for both OOK-1/4 and FSK-2, it is observed in the conclusion section of the TR that knowledge of sequence(s) used in LP-WUS waveform generation may improve performance for at least a receiver with I/Q branches. And it is concluded for either OOK or FSK that Sequences to generate ON duration in OFDMA transmitter, if specified, can help receiver (with I/Q branches) performance.

Observation 1: According to TR 38.869, OOK-1/4 and FSK-2 have different benefits and drawbacks, and sequence(s) to generate ON duration can be utilized on top of both of them to improve performance.

In the TR conclusion, it is observed that sequence(s) used in LP-WUS symbol generation with different pulse shape or spectral shape may have different performance. Therefore, the pulse shape should be discussed and specified for better performance for LP-WUS in work item phase.

Observation 2: According to TR 38.869, specifying appropriate characteristics of the OOK/FSK waveform can improve the performance.

Both OOK and FSK can be demodulated based on envelope detection, the same RF/IF processing is applied to OOK and FSK, and thus the same hardware is used for OOK/FSK receiver. Due to the same hardware modules, it is possible to have a harmonized design of OOK and FSK, which leads to the attractive opportunity to obtain the benefits of each modulation scheme.

Observation 3: Due to the same envelope detection processing, a harmonized design of OOK and FSK is possible, giving the benefits of both modulations.

Based on the above observations and analyses,, we propose a harmonized design of OOK/FSK and sequences on top of OOK/FSK.

Proposal 1: The waveform of LP-WUS is OOK+FSK with sequences on top, and necessary aspects of the waveform are specified.

In RAN1#114, some companies also proposed OFDMA-standalone waveform. However, per our understanding, sequences on top of OOK/FSK already provide the feasibility for at least receiver with I/Q branches to improve the performance, including the coverage and potentially the higher spectrum efficiency, with the potential price of slightly higher power consumption of LP-WUS receiver. If harmonized design of OOK and FSK, and also sequences on top of OOK/FSK are adopted, all the devices can utilize LP-WUS by the same LP-WUS transmission. However, if OFDMA-standalone is supported, there is no way to further jointly transmit LP-WUS for OFDMA-standalone receivers. In other words, additionally transmitting OFDMA-standalone based LP-WUS will double the resource overhead and increase network energy consumption.

On the other hand, it is proposed by some companies that OFDMA-standalone waveform can utilize the hardware/software of NB-IoT WUS. However:

- A narrow bandwidth of 180kHz is not sufficient for NR LP-WUS, where most evaluations have needed to assume 5 MHz. Therefore, hardware and specification, of NB-IoT WUS may not be reused directly for LP-WUS reception.

- Even if there are NB-IoT chipsets hardware supporting higher bandwidth reception, reusing the hardware of NB-IoT WUS does not dictate an OFDMA-standalone waveform for LP-WUS. In fact, such a receiver would be common for all discussed LP-WUS waveforms (including OOK, FSK and sequences on top of OOK/FSK), since all the discussed LP-WUS waveforms can be demodulated by OFDMA-based receiver, and the only differences are in baseband processing.

- NB-IoT was supports only 15 kHz SCS, whereas other SCS, and particularly 30 kHz in TDD, are important for NR.

Proposal 2: OFDMA-standalone waveform is not supported for LP-WUS for the following reasons

  - Additional resource is needed since it cannot be transmitted jointly with the other LP-WUS waveform candidates.

  - Infeasible to reuse the hardware/software of NB-IoT WUS.

# On the applicable RRC state for LP-WUS

In TR 38.869, it is concluded that good power saving gain can be provided for both IDLE/INACTIVE mode and CONNECTED mode [1]. For example, for CONNECTED mode, the following is concluded:

Compared with existing UE power saving techniques, significant UE power saving gain (up to more than 60%) and moderate UPT improvement (up to more than 10%) when the UE MR enters deep sleep state during LR LP-WUS monitoring, larger UPT improvement (up to more than 180%) is observed for traffic with small packets, e.g. IM.

Therefore, LP-WUS should be applicable for both IDLE/INACTIVE mode and CONNECTED mode.

Proposal 3: Both IDLE/INACTIVE mode and CONNECTED mode are supported for LP-WUS in Rel-19.

# On the functionalities and procedures for LP-WUS

In TR 38.869, it is concluded that “at least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial” [1]. Therefore, periodic LP-SS should be specified.

In TR 38.869, it is observed that “UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbor cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbor cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial” [1]. Therefore, further RRM measurement enhancements, including serving cell RRM measurement offloading to LP-WUR and relaxation of RRM measurement by MR, should be specified.

In IDLE/INACTIVE mode, LP-WUS is expected to support at least paging related functionality. And in CONNECTED mode, LP-WUS is expected to at least trigger/resume the UE MR PDCCH monitoring. The corresponding content and procedures will need to be specified.

In TR 38.869, it is concluded that “It is observed that for LP-WUS can achieve comparable MIL performance with that of NR PUSCH MSG3, with the assumed resource for each LP-WUS transmission is as the following…” Specifically, according to our observation, coverage achieving schemes (e.g. time domain repetition, frequency hopping, coding and etc.), are used in some evaluation results. Therefore, appropriate coverage enhancement schemes should be specified.

Though enhancements can be used to improve the coverage performance, it may still happen that the coverage of LP-WUS is insufficient, e.g. when there is blocking. In RAN1 #112-bis-e, it was agreed that ‘At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.’ [3]. Other conditions for the activation/deactivation of LP-WUS monitoring may also be identified.

Proposal 4: At least the following are specified in a Rel-19 LP-WUS/WUR WI:

  - LP-WUS waveform of OOK+FSK with sequences on top, including specification of appropriate characteristics of the waveform

  - Periodical LP-SS, at least for LP-WUR architectures that cannot receive existing PSS/SSS

  - RRM measurement enhancements, including serving cell RRM measurement offloading to LP-WUR, and relaxation of RRM measurement by main radio

  - The content of LP-WUS and the corresponding procedures to at least support wake-up indication for IDLE/INACTIVE mode, and triggering/resuming MR PDCCH monitoring for CONNECTED mode

  - Coverage achieving schemes, according to a MIL target at least matching Msg3 PUSCH

  - Activation/deactivation of LP-WUS monitoring

# Conclusion

In this contribution, we discuss the scope of LP-WUS in Rel-19. Based on the analysis and discussion, we have the following observations and proposals:

Observation 1: According to TR 38.869, OOK-1/4 and FSK-2 have different benefits and drawbacks, and sequence(s) to generate ON duration can be utilized on top of both of them to improve performance.

Observation 2: According to TR 38.869, specifying appropriate characteristics of the OOK/FSK waveform can improve the performance.

Observation 3: Due to the same envelope detection processing, a harmonized design of OOK and FSK is possible, giving the benefits of both modulations.

Proposal 1: The waveform of LP-WUS is OOK+FSK with sequences on top, and necessary aspects of the waveform are specified.

Proposal 2: OFDMA-standalone waveform is not supported for LP-WUS for the following reasons:

  - Additional resource is needed since it cannot be transmitted jointly with the other LP-WUS waveform candidates.

  - Infeasible to reuse the hardware/software of NB-IoT WUS.

Proposal 3: Both IDLE/INACTIVE mode and CONNECTED mode are supported for LP-WUS in Rel-19.

Proposal 4: At least the following are specified in a Rel-19 LP-WUS/WUR WI:

  - LP-WUS waveform of OOK+FSK with sequences on top, including specification of appropriate characteristics of the waveform

  - Periodical LP-SS, at least for LP-WUR architectures that cannot receive existing PSS/SSS

  - RRM measurement enhancements, including serving cell RRM measurement offloading to LP-WUR, and relaxation of RRM measurement by main radio

  - The content of LP-WUS and the corresponding procedures to at least support wake-up indication for IDLE/INACTIVE mode, and triggering/resuming MR PDCCH monitoring for CONNECTED mode

  - Coverage achieving schemes, according to a MIL target at least matching Msg3 PUSCH

  - Activation/deactivation of LP-WUS monitoring

# References

- TR 38.869, Study on low-power Wake-up Signal and Receiver for NR.

- R1-2306550, Further details on signal design and procedure for LP-WUS, Huawei, HiSilicon.

- Chairman’s notes of RAN1#112bis-e.