3GPP TSG RAN Meeting #101	RP-232417

Bangalore, India, September 11-15, 2023

Source:	Huawei, HiSilicon

Title:	Sidelink enhancements in Rel-19

Agenda item:	8A.2.12.5

Document for:	Discussion and Decision

# Introduction

NR sidelink has been developed since Rel-16, starting from 5G V2X with NR sidelink WI [1], which focused on V2X scenario and use cases. With following releases including Rel-17 (NR sidelink enhancement WI [2]) and Rel-18 (NR sidelink evolution WI [3]), the design of sidelink also focused on commercial scenario and use cases (e.g. MBB application, experience and interactive applications, i.e. XR). In Rel-18, sidelink operation extends its applicable spectrum from FR1 licensed and ITS spectrum and unlicensed spectrum, and includes a detailed study of how to further extend to FR2, both of which can benefit from its large bandwidth availability to support high data rate services. In addition, carrier aggregation was also included in the Rel-18 WID as a mean of improving data rate and reliability.

However, due to scope and TU restrictions in Rel-18, sidelink operation over FR2, both licensed and unlicensed operations, will not be specified in Rel-18. There was only a study undertaken primarily targeted for sidelink beam management. The scope of SL CA as per WID is very limited in terms of design (strictly reuse from LTE SL CA design without enhancement considering NR) and applicable scenario (only intra-band CA in FR1 ITS band).

Details of our view on Rel-19 sidelink enhancements are presented in the discussion section.

# Discussion

### Use cases and motivation

The sidelink can be used for use cases such as interactive services for AR/VR, HMD connected to smart phones, etc. As the development of new devices (e.g. VR/AR devices, robot, etc.), TR 22.261 defines audio-visual interaction, e.g. AR/VR application, including online XR gaming and interactive data sharing between variable devices, which requires Gbps user-experienced data rate with low latency < 10ms at 99.99% reliability, as shown in Table 1.

Table 1 Performance requirements for AR/VR (from TS 22.261)

| Use Cases | Characteristic parameter (KPI) |  |  |
| --- | --- | --- | --- |
|  | Max Allowed End-to-end latency | Service bit rate: user-experienced data rate | Reliability |
| Gaming or Interactive Data Exchanging NOTE: Communication includes direct wireless links (UE to UE). | 10ms | 0.1-[1] Gbps supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120fps content. | 99.99% |
| Consume VR content via tethered VR headset NOTE: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. | [5 -10] ms | 0.1-[10] Gbit/s | [99.99%] |

### Technical considerations

#### Sidelink operations over FR2 including FR2-2 unlicensed

In Rel-18, the study of sidelink operation over FR2 is conducted in RAN1 and RAN2, with a number of possible alternative procedures identified for beam management, specifically, initial beam pairing (including the cases where initial beam pairing before, during and after unicast link establishment), beam maintenance, beam failure detection and recovery. Regarding SL CSI-RS, both non-standalone and standalone SL CSI-RS are considered in FR2 beam operation. In addition, sidelink TCI state mechanism is included in the study. It can be observed that the general procedures to allow beam-based sidelink transmission/receptions to be operational over FR2 are well studied. WGs can go to specification in Rel-19 based on the study in Rel-18.

However, the Rel-18 study was focused on beam management due to scope limitation in Rel-18, and other design aspects which are also critical for FR2 performance are not studied, for example, resource allocation (both mode 1 and mode 2). Sidelink is a distributed system, where resource allocation is critical for managing interference among different UEs to ensure the reliability of sidelink communication. As per TR 22.261, reliability is required as high as 99.99%, which is difficult to achieve if only random resource selection is assumed. Given that PSCCH/PSSCH is transmitted in beams, resource allocation can be beam dependent, i.e. per-beam RA, which is similar to RA per carrier as in CA case. Taking mode 2 as an example, resource selection window can be beam dependant such that resource exclusion based on sensing is per beam, and similarly sensing window is also per beam, e.g. as in Figure 1.

Figure 1．SA,i,SA,j and SA,k are candidate resource sets based on receiving beams i, j, and k, respectively

In addition to Rel-16 resource allocation schemes, Rel-17 and Rel-18 can also be applicable to SL beam operation over FR2 band, e.g. inter-UE coordination (IUC), and multi-consecutive slots transmission (MCSt). The IUC allows the intended RX to assist resource selection of its associated TX, and this is even more useful in beam-based communication, given that interference is also in beam domain and can be very different at TX and RX side. MCSt is also beneficial for efficient COT utilization.

Observation 1: Accounting for beam-domain impacts to sidelink resource allocation is critical for ensuring reliability for sidelink communication and is therefore essential to be specified in Rel-19 in order to satisfy applications requiring high reliability, such as real-time interactive services as XR.

It is noted that Rel-18 study is limited to licensed spectrum only, however, beam management procedure itself is band independent, for instance, in Uu design, same beam management procedure applies for both FR2-1 and FR2-2. From spectrum availability point of view, FR2-2, particularly the 60GHz unlicensed band, is already available for commercial deployment, and this gives a better position for sidelink operation over FR2. In FR2-2, NR operation extending to 71GHz in Rel-17[4] as well as SL-U design in Rel-18 can be used as a baseline, and this can allow containing the scope of the work for SL operation in FR2-2, for example, directional LBT for channel access, channel/procedure enhancement due to introduction of higher SCS.

Observation 2: Same sidelink beam management procedure can be applied to both FR2-1 and FR2-2. Other enhancements including channel access procedure can be based on NR operation extending to 71GHz and SL-U design.

Proposal 1: Specify sidelink operation on FR2-1 and FR2-2 spectrum in Rel-19.

- Start normative work on sidelink beam management procedures starting from Rel-18 agreements and working assumptions.

  - Note: same sidelink beam management procedures apply to both FR2-1 and FR2-2.

- Resource allocation adaptions, including adapting Rel-16/17/18 designs, to support beam-based sidelink transmissions.

- Enhancements including channel access procedure for FR2-2 shall take NR operation extending to 71GHz and SL-U work as a baseline.

#### Sidelink carrier aggregation

Operating bands for sidelink have been extended from FR1 licensed (n38, n79) and ITS (n47), to unlicensed (n46 and n96/n102) and FR2. Although intra-band SL CA in ITS band (n47) is included in the Rel-18 WID, the value of SL CA work in Rel-18 is rather limited and is un-suitable for majority of use cases, e.g. anything apart from some V2X applications only. Thus, for most applications of future interest, NR sidelink is limited to single carrier operation, which prevents sidelink operation to efficiently utilize the large amount of spectrum resources in unlicensed and FR2 spectrum.

The evolution of sidelink operation over FR2, should firstly target at standalone operation, which aligns with the study scope of Rel-18, and therefore the consideration of FR1-assisted FR2 in Rel-19 is not urgent at this stage. In addition, the performance of standalone SL FR2 is not well evaluated, and thus whether or not enhancement on top of standalone SL FR2 is needed is questionable.

Observation 3: Standalone sidelink FR2 operation should be prioritized, and CA operation between FR1 and FR2, including FR1-assisted FR2 is questionable in terms of necessity, given that performance of standalone SL FR2 is not well evaluated and whether or not further enhancement on top of standalone SL FR2 is critical.

CA design in Rel-19 should prioritize aggregating unlicensed CCs, and aggregating licensed CCs, i.e. not aggregating unlicensed CC with licensed CC, given that sidelink operation between licensed and unlicensed are different in terms of physical channel design, procedures, resource allocation, channel access, etc. It can be expected the scope would be largely extended due to design complexity, but the benefit and necessity of such mixed unlicensed and licensed carrier aggregation is not clear, since there is not a use case that cannot be addressed without such features.

However, Rel-19 should enhance cross carrier operation, which is not supported in LTE SL CA given that LTE SL does not consider to support high reliability services, e.g. XR as commercial use cases. Cross carrier operation is beneficial for improved reliability and reduced interference, particularly for the case where resource pools within different carriers have different (pre-)configurations, and varying channel conditions in time and frequency. In addition, considering limited TX/RX capability, for example, the number of CCs a UE has for reception is larger than for transmission. In this case, reception of PSCCH/PSSCH can be in multiple CCs but transmission of PSFCH in response to the PSCCH/PSSCH may be limited to the subset of the multiple CCs, and therefore cross-carrier PSFCH to PSCCH/PSSCH will be beneficial to ensure reliable SL HARQ operation.

Overall consideration for SL CA in Rel-19 can be shown in Figure 2, as illustration purpose.

Figure 2 sidelink CA operation

In Rel-18, due to TU restriction, only mode 2 resource allocation is considered. This limitation should be lifted such that CA operation shall not be applicable in mode 2 only. It is noted that LTE SL CA supports both mode 3 and mode 4.

Proposal 2: Specify sidelink carrier aggregation beyond FR1 ITS band:

- Support aggregation of intra-band CA within FR1 licensed, intra-band CA within FR1 unlicensed, intra-band CA within FR2 licensed and intra-band CA within FR2 unlicensed.

- Support different TX/RX capability.

- Support of cross-carrier operation for PSCCH/PSSCH/PSFCH.

- Support NR SL CA in both mode 1 and mode 2.

#### Other considerations

There are other considerations for SL enhancement in Rel-19 proposed by a number of companies, such as SL power saving, SL RedCap, etc. In Rel-17, as a dedicated release for power consumption reduction for SL, both RAN1 and RAN2 have specified multiple schemes on reduced power consumption, e.g. partial sensing, random resource selection, SL DRX. A few companies mention introduction of wake-up signal (WUS) on top of SL-DRX, however, R17 specification already allows a RX UE to wake up for reception at indicated reserved resources by its associated TX UE. That is, SL resource reservation (i.e. a first stage SCI) is already a means of WUS compared to Uu. With this, further consideration of WUS in Rel-19 is not needed. These power saving designs as introduced in Rel-17 are already applicable to Rel-18 features, e.g. SL operation over unlicensed band, and can also applied in Rel-19. Further power reduction design is not needed, and there are no new power saving requirements or new use cases compared to Rel-17 (VRU in V2X, wireless devices in commercial use case) being identified.

Uu operation over unlicensed spectrum does not have specific RedCap design in Rel-16/17. Similarly, for Rel-19, no additional considerations should be prioritized. Rel-18 design already allows 20 MHz-only (minimum bandwidth granularity for performing LBT) operation of a device over FR1 unlicensed band (e.g. 5 GHz band), which can be used for low-end capability reduced devices. Further enhancement, such as below-20 MHz SL operation in Rel-19 over unlicensed band (e.g. 5 GHz band), would result in large specification impact due to backward-compatibility, and is also against regional regulatory requirements on LBT, e.g. 20 MHz LBT granularity, OCB, etc.

Observation 4: Rel-17 power saving schemes, e.g. partial sensing, random resource selection, SL DRX, are applicable to the proposed Rel-19 features without further work. Hence, further power reduction design is not needed.

# Conclusion

The scope of Rel-19 sidelink should be prioritized as an extension of Rel-18 to allow the enhanced features as introduced in Rel-18 to be fully functional, and we have following observations and proposals:

Observation 1: Accounting for beam-domain impacts to sidelink resource allocation is critical for ensuring reliability for sidelink communication and is therefore essential to be specified in Rel-19 in order to satisfy applications requiring high reliability, such as real-time interactive services as XR.

Observation 2: Same sidelink beam management procedure can be applied to both FR2-1 and FR2-2. Other enhancements including channel access procedure can be based on NR operation extending to 71GHz and SL-U design.

Observation 3: Standalone sidelink FR2 operation should be prioritized, and CA operation between FR1 and FR2, including FR1-assisted FR2 is questionable in terms of necessity, given that performance of standalone SL FR2 is not well evaluated and whether or not further enhancement on top of standalone SL FR2 is critical.

Observation 4: Rel-17 power saving schemes, e.g. partial sensing, random resource selection, SL DRX, are applicable to the proposed Rel-19 features without further work. Hence, further power reduction design is not needed.

Proposal 1: Specify sidelink operation on FR2-1 and FR2-2 spectrum in Rel-19.

- Start normative work on sidelink beam management procedures starting from Rel-18 agreements and working assumptions.

  - Note: same sidelink beam management procedures apply to both FR2-1 and FR2-2.

- Resource allocation adaptions, including adapting Rel-16/17/18 designs, to support beam-based sidelink transmissions.

- Enhancements including channel access procedure for FR2-2 shall take NR operation extending to 71GHz and SL-U work as a baseline.

Proposal 2: Specify sidelink carrier aggregation beyond FR1 ITS band:

- Support aggregation of intra-band CA within FR1 licensed, intra-band CA within FR1 unlicensed, intra-band CA within FR2 licensed and intra-band CA within FR2 unlicensed.

- Support different TX/RX capability.

- Support of cross-carrier operation for PSCCH/PSSCH/PSFCH.

- Support NR SL CA in both mode 1 and mode 2.

# Reference

- RP-190984, “Revised WID on 5G V2X with NR sidelink”, LG Electronics, RAN #84, Newport Beach, USA, June, 2019

- RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, RAN#90-e, December 2020.

- RP-230077, “WID revision: NR sidelink evolution”, OPPO, RAN#99, March 2023

- RP-210862, “WID revision: Extending current NR operation to 71 GHz”, CMCC, RAN#91-e, March 2021.