| 3GPP TR 38.858 V1.0.0 (2023-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Evolution of NR Duplex Operation (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

B.4		System level simulation results for dynamic TDD

The detailed evaluation results for dynamic TDD can be found in the attached document "B.4_Dynamic TDD.zip".

B.4.1A	Inter-gNB CLI scheme 1A: UL Resource Muting-based scheme for measuring the gNB-to-gNB CLI interference covariance matrix

- Source 1 ([19]):

2-layer Scenario B (FR1)

Large Packet Size MMSE-IRC without joint reception

| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean |  |  |  |  |  | -1.00% |  |  | -3.00% |
|  | 5% |  |  |  |  |  | 49.00% |  |  | 53.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  | 77.00% |  |  | 166.00% |
|  | 5% |  |  |  |  |  | 7.00% |  |  | 174.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  | 42.01% |  |  | 64.32% |  |  |
| UL RU (%) | Type-1 |  |  |  | 41.03% |  |  | 65.12% |  |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) |  |  |  |  |  |  |  |  |  |  |

Large Packet Size UL resource muting based E-MMSE-IRC without joint reception

| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean |  |  |  |  |  | 1.00% |  |  | -1.00% |
|  | 5% |  |  |  |  |  | 53.00% |  |  | 40.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  | 174.00% |  |  | 299.00% |
|  | 5% |  |  |  |  |  | 72.00% |  |  | 368.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  | 42.01% |  |  | 64.32% |  |  |
| UL RU (%) | Type-1 |  |  |  | 41.03% |  |  | 65.12% |  |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size MMSE-IRC with joint reception

| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean |  |  |  |  |  | -1.00% |  |  | -3.00% |
|  | 5% |  |  |  |  |  | 49.00% |  |  | 53.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  | 83.00% |  |  | 131.00% |
|  | 5% |  |  |  |  |  | 14.00% |  |  | 62.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  | 42.01% |  |  | 64.32% |  |  |
| UL RU (%) | Type-1 |  |  |  | 41.03% |  |  | 65.12% |  |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size UL resource muting based E-MMSE-IRC with joint reception

| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean |  |  |  |  |  | 1.00% |  |  | -1.00% |
|  | 5% |  |  |  |  |  | 53.00% |  |  | 40.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Average-UPT (Mbps) | Mean |  |  |  |  |  | 179.00% |  |  | 200.00% |
|  | 5% |  |  |  |  |  | 22.00% |  |  | 92.00% |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 |  |  |  | 42.01% |  |  | 64.32% |  |  |
| UL RU (%) | Type-1 |  |  |  | 41.03% |  |  | 65.12% |  |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size MMSE-IRC with joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 112.85 | 112.83 | -0.01% | 88.82 | 90.14 | 1.50% | 65.72 | 71.35 | 8.57% |
|  | 5% | 35.71 | 45.71 | 28.00% | 20.51 | 23.88 | 16.42% | 7.01 | 14.34 | 104.66% |
|  | 50% | 97.56 | 97.56 | 0.00% | 83.33 | 86.96 | 4.35% | 55.94 | 65.04 | 16.26% |
| UL Average-UPT (Mbps) | Mean | 19.88 | 19.05 | -4.16% | 21.64 | 21.96 | 1.47% | 21.48 | 21.66 | 0.84% |
|  | 5% | 0.99 | 1.16 | 16.89% | 0.77 | 0.83 | 8.16% | 0.81 | 0.80 | -1.19% |
|  | 50% | 13.40 | 12.88 | -3.86% | 24.17 | 24.84 | 2.79% | 23.67 | 23.74 | 0.30% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 7.24% | 8.17% |  | 44.20% | 41.62% |  | 68.25% | 63.51% |  |
| UL RU (%) | Type-1 | 14.55% | 14.55% |  | 37.57% | 14.76% |  | 43.70% | 42.94% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.89 | 57.85 | -42.66% | 91.46 | 33.21 | -63.69% | 62.82 | 18.79 | -70.09% |
|  | 5% | 33.61 | 30.19 | -10.19% | 20.25 | 6.05 | -70.14% | 4.37 | 1.40 | -67.90% |
|  | 50% | 95.24 | 57.55 | -39.57% | 93.02 | 30.65 | -67.05% | 52.98 | 12.35 | -76.70% |
| UL Average-UPT (Mbps) | Mean | 42.09 | 109.27 | 159.64% | 34.55 | 74.79 | 116.45% | 21.28 | 41.65 | 95.72% |
|  | 5% | 39.02 | 75.47 | 93.40% | 20.25 | 43.24 | 113.51% | 4.68 | 14.49 | 209.78% |
|  | 50% | 42.55 | 114.29 | 168.57% | 36.53 | 72.07 | 97.30% | 22.16 | 37.91 | 71.09% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.93% |  | 9.67% | 24.99% |  | 22.46% | 56.78% |  |
| UL RU (%) | Type-1 | 18.10% | 9.05% |  | 46.16% | 41.53% |  | 50.00% | 58.27% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Transparent UL resource muting Scheme1-E-MMSE-IRC with joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 112.85 | 106.97 | -5.21% | 88.82 | 82.39 | -7.23% | 65.72 | 61.81 | -5.94% |
|  | 5% | 35.71 | 43.48 | 21.74% | 20.51 | 21.77 | 6.13% | 7.01 | 12.24 | 74.66% |
|  | 50% | 97.56 | 96.39 | -1.20% | 83.33 | 79.07 | -5.12% | 55.94 | 54.07 | -3.34% |
| UL Average-UPT (Mbps) | Mean | 19.88 | 21.10 | 6.13% | 21.64 | 21.50 | -0.62% | 21.48 | 21.27 | -0.95% |
|  | 5% | 0.99 | 1.17 | 18.53% | 0.77 | 0.78 | 2.48% | 0.81 | 0.83 | 2.01% |
|  | 50% | 13.40 | 13.34 | -0.46% | 24.17 | 24.19 | 0.08% | 23.67 | 22.04 | -6.88% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 7.24% | 8.63% |  | 44.20% | 44.87% |  | 68.25% | 68.61% |  |
| UL RU (%) | Type-1 | 14.55% | 13.85% |  | 37.57% | 39.63% |  | 43.70% | 46.82% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.89 | 52.05 | -48.41% | 91.46 | 32.77 | -64.17% | 62.82 | 16.46 | -73.79% |
|  | 5% | 33.61 | 29.26 | -12.96% | 20.25 | 6.16 | -69.59% | 4.37 | 1.35 | -69.13% |
|  | 50% | 95.24 | 53.69 | -43.62% | 93.02 | 30.89 | -66.80% | 52.98 | 10.23 | -80.68% |
| UL Average-UPT (Mbps) | Mean | 42.09 | 96.63 | 129.60% | 34.55 | 90.44 | 161.76% | 21.28 | 71.88 | 237.74% |
|  | 5% | 39.02 | 88.89 | 127.78% | 20.25 | 67.80 | 234.75% | 4.68 | 33.76 | 621.51% |
|  | 50% | 42.55 | 97.56 | 129.27% | 36.53 | 94.12 | 157.65% | 22.16 | 76.92 | 247.11% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.37% |  | 9.67% | 25.24% |  | 22.46% | 72.35% |  |
| UL RU (%) | Type-1 | 18.10% | 9.82% |  | 46.16% | 37.40% |  | 50.00% | 52.62% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Transparent UL resource muting Scheme2-E-MMSE-IRC with joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 112.85 | 103.13 | -8.61% | 88.82 | 81.44 | -8.30% | 65.72 | 59.47 | -9.51% |
|  | 5% | 35.71 | 34.63 | -3.03% | 20.51 | 21.43 | 4.46% | 7.01 | 10.33 | 47.50% |
|  | 50% | 97.56 | 90.91 | -6.82% | 83.33 | 77.26 | -7.29% | 55.94 | 53.64 | -4.12% |
| UL Average-UPT (Mbps) | Mean | 19.88 | 20.85 | 4.92% | 21.64 | 21.98 | 1.59% | 21.48 | 20.72 | -3.51% |
|  | 5% | 0.99 | 1.23 | 24.19% | 0.77 | 0.79 | 3.21% | 0.81 | 0.83 | 2.05% |
|  | 50% | 13.40 | 13.70 | 2.27% | 24.17 | 23.69 | -1.98% | 23.67 | 21.02 | -11.19% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 7.24% | 8.90% |  | 44.20% | 46.26% |  | 68.25% | 69.95% |  |
| UL RU (%) | Type-1 | 14.55% | 17.34% |  | 37.57% | 41.16% |  | 43.70% | 53.85% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.89 | 56.12 | -44.37% | 91.46 | 32.66 | -64.28% | 62.82 | 16.07 | -74.42% |
|  | 5% | 33.61 | 29.43 | -12.43% | 20.25 | 5.92 | -70.79% | 4.37 | 1.68 | -61.44% |
|  | 50% | 95.24 | 54.05 | -43.24% | 93.02 | 29.18 | -68.63% | 52.98 | 11.11 | -79.03% |
| UL Average-UPT (Mbps) | Mean | 42.09 | 87.68 | 108.33% | 34.55 | 77.75 | 125.01% | 21.28 | 63.17 | 196.81% |
|  | 5% | 39.02 | 81.63 | 109.18% | 20.25 | 48.48 | 139.39% | 4.68 | 26.06 | 457.00% |
|  | 50% | 42.55 | 87.91 | 106.59% | 36.53 | 82.47 | 125.77% | 22.16 | 66.12 | 198.35% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.94% |  | 9.67% | 28.79% |  | 22.46% | 69.32% |  |
| UL RU (%) | Type-1 | 18.10% | 10.89% |  | 46.16% | 40.57% |  | 50.00% | 53.85% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Non-transparent UL resource muting-E-MMSE-IRC with joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 112.85 | 104.05 | -7.80% | 88.82 | 81.97 | -7.71% | 65.72 | 64.14 | -2.40% |
|  | 5% | 35.71 | 39.80 | 11.44% | 20.51 | 22.53 | 9.84% | 7.01 | 11.89 | 69.71% |
|  | 50% | 97.56 | 90.91 | -6.82% | 83.33 | 78.33 | -6.00% | 55.94 | 57.02 | 1.92% |
| UL Average-UPT (Mbps) | Mean | 19.88 | 20.99 | 5.61% | 21.64 | 21.55 | -0.42% | 21.48 | 21.99 | 2.41% |
|  | 5% | 0.99 | 1.18 | 18.61% | 0.77 | 0.85 | 11.42% | 0.81 | 0.81 | -0.70% |
|  | 50% | 13.40 | 13.33 | -0.52% | 24.17 | 25.05 | 3.66% | 23.67 | 23.75 | 0.33% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 7.24% | 8.51% |  | 44.20% | 44.06% |  | 68.25% | 67.67% |  |
| UL RU (%) | Type-1 | 14.55% | 18.27% |  | 37.57% | 36.07% |  | 43.70% | 43.31% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.89 | 55.05 | -16.92% | 91.46 | 32.58 | -64.37% | 62.82 | 16.49 | -73.76% |
|  | 5% | 33.61 | 30.48 | -9.34% | 20.25 | 6.08 | -69.99% | 4.37 | 1.06 | -75.74% |
|  | 50% | 95.24 | 54.42 | -42.86% | 93.02 | 31.73 | -65.89% | 52.98 | 10.81 | -79.59% |
| UL Average-UPT (Mbps) | Mean | 42.09 | 113.18 | 168.92% | 34.55 | 104.65 | 202.86% | 21.28 | 92.76 | 335.89% |
|  | 5% | 39.02 | 103.90 | 166.23% | 20.25 | 71.43 | 252.68% | 4.68 | 51.28 | 996.15% |
|  | 50% | 42.55 | 114.29 | 168.57% | 36.53 | 109.59 | 200.00% | 22.16 | 98.77 | 345.68% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.48% |  | 9.67% | 35.16% |  | 22.46% | 70.07% |  |
| UL RU (%) | Type-1 | 18.10% | 8.52% |  | 46.16% | 31.27% |  | 50.00% | 49.47% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size MMSE-IRC without joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 115.50 | 114.74 | -0.66% | 91.77 | 96.35 | 4.99% | 72.87 | 72.29 | -0.79% |
|  | 5% | 34.19 | 65.04 | 90.24% | 19.18 | 27.49 | 43.30% | 9.74 | 13.56 | 39.15% |
|  | 50% | 100.00 | 112.68 | 12.68% | 87.91 | 93.02 | 5.81% | 62.99 | 65.04 | 3.25% |
| UL Average-UPT (Mbps) | Mean | 19.65 | 19.46 | -0.97% | 22.07 | 22.11 | 0.15% | 22.60 | 22.49 | -0.46% |
|  | 5% | 1.27 | 1.36 | 6.72% | 0.77 | 0.85 | 9.76% | 0.84 | 0.90 | 6.73% |
|  | 50% | 13.63 | 12.95 | -5.02% | 24.17 | 23.67 | -2.07% | 24.02 | 24.02 | 0.00% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 8.11% | 7.39% |  | 42.10% | 38.86% |  | 63.51% | 63.83% |  |
| UL RU (%) | Type-1 | 15.80% | 16.12% |  | 39.88% | 39.75% |  | 45.51% | 45.21% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 109.39 | 58.77 | -46.27% | 86.64 | 39.79 | -54.08% | 66.11 | 28.75 | -56.51% |
|  | 5% | 47.06 | 39.41 | -16.26% | 22.88 | 10.24 | -55.22% | 6.34 | 2.56 | -59.66% |
|  | 50% | 97.56 | 58.39 | -40.15% | 89.89 | 39.41 | -56.16% | 58.82 | 24.62 | -58.15% |
| UL Average-UPT (Mbps) | Mean | 32.58 | 105.92 | 225.07% | 8.02 | 32.82 | 309.39% | 7.40 | 15.50 | 109.37% |
|  | 5% | 16.60 | 65.04 | 291.87% | 1.40 | 5.04 | 260.72% | 1.02 | 1.25 | 22.06% |
|  | 50% | 34.78 | 112.68 | 223.94% | 7.17 | 28.37 | 295.74% | 6.37 | 12.14 | 90.66% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.02% |  | 9.67% | 22.10% |  | 23.47% | 47.47% |  |
| UL RU (%) | Type-1 | 4.93% | 1.56% |  | 46.75% | 19.61% |  | 49.85% | 27.51% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Transparent UL resource muting Scheme1-E-MMSE-IRC without joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 115.50 | 105.24 | -8.89% | 91.77 | 82.90 | -9.66% | 72.87 | 65.91 | -9.55% |
|  | 5% | 34.19 | 48.19 | 40.96% | 19.18 | 21.01 | 9.49% | 9.74 | 11.96 | 22.70% |
|  | 50% | 100.00 | 90.91 | -9.09% | 87.91 | 78.33 | -10.90% | 62.99 | 56.63 | -10.10% |
| UL Average-UPT (Mbps) | Mean | 19.65 | 19.41 | -1.22% | 22.07 | 23.05 | 4.41% | 22.60 | 22.35 | -1.10% |
|  | 5% | 1.27 | 1.38 | 8.92% | 0.77 | 0.89 | 15.44% | 0.84 | 0.93 | 9.54% |
|  | 50% | 13.63 | 13.56 | -0.51% | 24.17 | 23.59 | -2.38% | 24.02 | 22.66 | -5.66% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 8.11% | 7.60% |  | 42.10% | 45.58% |  | 63.51% | 69.59% |  |
| UL RU (%) | Type-1 | 15.80% | 19.65% |  | 39.88% | 43.57% |  | 45.51% | 47.67% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 109.39 | 59.60 | -45.51% | 86.64 | 40.73 | -52.99% | 66.11 | 24.48 | -62.97% |
|  | 5% | 47.06 | 39.80 | -15.42% | 22.88 | 9.52 | -58.36% | 6.34 | 3.25 | -48.84% |
|  | 50% | 97.56 | 59.87 | -38.63% | 89.89 | 40.11 | -55.38% | 58.82 | 19.09 | -67.55% |
| UL Average-UPT (Mbps) | Mean | 32.58 | 82.74 | 153.94% | 8.02 | 33.81 | 321.66% | 7.40 | 16.35 | 120.78% |
|  | 5% | 16.60 | 46.51 | 180.23% | 1.40 | 8.62 | 517.26% | 1.02 | 1.34 | 31.10% |
|  | 50% | 34.78 | 86.96 | 150.00% | 7.17 | 30.65 | 327.58% | 6.37 | 13.89 | 118.06% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.56% |  | 9.67% | 21.23% |  | 23.47% | 55.46% |  |
| UL RU (%) | Type-1 | 4.93% | 2.27% |  | 46.75% | 25.90% |  | 49.85% | 51.04% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Transparent UL resource muting Scheme2-E-MMSE-IRC without joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 115.50 | 104.62 | -9.43% | 91.77 | 83.45 | -9.07% | 72.87 | 63.61 | -12.71% |
|  | 5% | 34.19 | 46.78 | 36.84% | 19.18 | 22.35 | 16.50% | 9.74 | 10.17 | 4.39% |
|  | 50% | 100.00 | 90.91 | -9.09% | 87.91 | 79.82 | -9.20% | 62.99 | 54.07 | -14.16% |
| UL Average-UPT (Mbps) | Mean | 19.65 | 19.30 | -1.81% | 22.07 | 23.93 | 8.40% | 22.60 | 22.29 | -1.37% |
|  | 5% | 1.27 | 1.37 | 7.97% | 0.77 | 0.87 | 12.15% | 0.84 | 0.96 | 13.62% |
|  | 50% | 13.63 | 13.59 | -0.25% | 24.17 | 23.33 | -3.46% | 24.02 | 22.36 | -6.92% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 8.11% | 9.15% |  | 42.10% | 45.22% |  | 63.51% | 71.81% |  |
| UL RU (%) | Type-1 | 15.80% | 19.97% |  | 39.88% | 44.47% |  | 45.51% | 48.32% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 109.39 | 59.30 | -45.79% | 86.64 | 40.35 | -53.43% | 66.11 | 24.88 | -62.37% |
|  | 5% | 47.06 | 39.32 | -16.44% | 22.88 | 9.53 | -58.33% | 6.34 | 2.91 | -54.18% |
|  | 50% | 97.56 | 59.66 | -38.85% | 89.89 | 40.22 | -55.25% | 58.82 | 18.79 | -68.05% |
| UL Average-UPT (Mbps) | Mean | 32.58 | 72.31 | 121.93% | 8.02 | 26.62 | 231.98% | 7.40 | 13.53 | 82.77% |
|  | 5% | 16.60 | 34.04 | 105.11% | 1.40 | 4.67 | 234.20% | 1.02 | 1.50 | 47.12% |
|  | 50% | 34.78 | 79.21 | 127.72% | 7.17 | 23.60 | 229.20% | 6.37 | 11.05 | 73.48% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.23% |  | 9.67% | 26.41% |  | 23.47% | 55.40% |  |
| UL RU (%) | Type-1 | 4.93% | 2.71% |  | 46.75% | 29.90% |  | 49.85% | 51.94% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Non-transparent UL resource muting-E-MMSE-IRC without joint reception

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 115.50 | 107.52 | -6.91% | 91.77 | 83.25 | -9.28% | 72.87 | 64.33 | -11.72% |
|  | 5% | 34.19 | 48.48 | 41.82% | 19.18 | 20.69 | 7.87% | 9.74 | 10.58 | 8.60% |
|  | 50% | 100.00 | 95.24 | -4.76% | 87.91 | 79.82 | -9.20% | 62.99 | 56.25 | -10.70% |
| UL Average-UPT (Mbps) | Mean | 19.65 | 19.82 | 0.86% | 22.07 | 23.61 | 6.94% | 22.60 | 23.12 | 2.30% |
|  | 5% | 1.27 | 1.30 | 2.25% | 0.77 | 0.84 | 8.26% | 0.84 | 0.91 | 7.71% |
|  | 50% | 13.63 | 13.74 | 0.85% | 24.17 | 23.94 | -0.94% | 24.02 | 24.48 | 1.92% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 8.11% | 8.12% |  | 42.10% | 44.19% |  | 63.51% | 70.50% |  |
| UL RU (%) | Type-1 | 15.80% | 16.02% |  | 39.88% | 39.18% |  | 45.51% | 45.55% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 109.39 | 59.01 | -46.05% | 86.64 | 39.09 | -54.88% | 66.11 | 24.80 | -62.48% |
|  | 5% | 47.06 | 39.68 | -15.68% | 22.88 | 9.88 | -56.81% | 6.34 | 3.27 | -48.49% |
|  | 50% | 97.56 | 59.07 | -39.46% | 89.89 | 39.59 | -55.95% | 58.82 | 19.47 | -66.90% |
| UL Average-UPT (Mbps) | Mean | 32.58 | 99.13 | 204.25% | 8.02 | 35.55 | 343.36% | 7.40 | 21.84 | 194.98% |
|  | 5% | 16.60 | 62.99 | 279.53% | 1.40 | 6.21 | 344.74% | 1.02 | 2.25 | 120.21% |
|  | 50% | 34.78 | 103.90 | 198.70% | 7.17 | 31.62 | 341.10% | 6.37 | 19.14 | 200.48% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.40% | 2.38% |  | 9.67% | 26.05% |  | 23.47% | 55.18% |  |
| UL RU (%) | Type-1 | 4.93% | 1.71% |  | 46.75% | 24.47% |  | 49.85% | 44.29% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 2 ([24]):

2-layer Scenario B (FR1)

Large Packet Size estimation based on UL DMRS

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 339.31 | 0.00% | 272.65 | 272.67 | 0.01% | 214.67 | 214.56 | -0.05% |
|  | 5% | 234.38 | 234.38 | 0.00% | 146.63 | 146.63 | 0.00% | 45.05 | 45.75 | 1.55% |
|  | 50% | 354.46 | 354.46 | 0.00% | 284.13 | 284.10 | -0.01% | 224.15 | 224.88 | 0.33% |
| UL Average-UPT (Mbps) | Mean | 40.62 | 34.86 | -14.16% | 31.09 | 24.17 | -22.25% | 22.66 | 15.22 | -32.82% |
|  | 5% | 4.59 | 3.78 | -17.71% | 2.90 | 1.90 | -34.48% | 1.23 | 0.66 | -46.69% |
|  | 50% | 41.40 | 33.45 | -19.20% | 29.33 | 21.74 | -25.89% | 21.48 | 13.22 | -38.46% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 12.21 | 0.00% | 22.82 | 22.78 | -0.16% | 36.13 | 36.45 | 0.88% |
|  | 5% | 10.92 | 10.92 | 0.00% | 10.96 | 10.96 | 0.00% | 11.03 | 11.03 | 0.00% |
|  | 50% | 11.26 | 11.26 | 0.00% | 12.72 | 12.71 | -0.08% | 18.85 | 18.87 | 0.08% |
| UL Packet-Latency CDF (ms) | Mean | 50.47 | 63.87 | 26.56% | 69.81 | 90.74 | 29.98% | 96.64 | 135.27 | 39.98% |
|  | 5% | 13.66 | 13.78 | 0.86% | 14.32 | 16.55 | 15.58% | 17.02 | 23.64 | 38.93% |
|  | 50% | 25.15 | 30.70 | 22.07% | 36.30 | 48.35 | 33.20% | 49.87 | 78.33 | 57.07% |
| DL RU (%) | Type-1 | 5.43% | 5.43% |  | 25.23% | 25.23% |  | 40.05% | 40.08% |  |
| UL RU (%) | Type-1 | 1.66% | 1.98% |  | 6.38% | 7.71% |  | 10.66% | 12.97% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.60 | 303.64 | 0.01% | 188.12 | 187.70 | -0.22% | 95.03 | 91.02 | -4.22% |
|  | 5% | 174.19 | 174.19 | 0.00% | 6.98 | 7.18 | 2.87% | 1.80 | 1.80 | 0.00% |
|  | 50% | 324.85 | 324.85 | 0.00% | 206.95 | 206.05 | -0.43% | 68.48 | 58.55 | -14.49% |
| UL Average-UPT (Mbps) | Mean | 81.14 | 94.07 | 15.93% | 31.79 | 34.42 | 8.28% | 18.65 | 16.58 | -11.11% |
|  | 5% | 22.22 | 18.34 | -17.46% | 0.47 | 0.38 | -19.21% | 0.19 | 0.19 | -0.28% |
|  | 50% | 72.13 | 94.10 | 30.45% | 33.52 | 34.58 | 3.16% | 13.40 | 7.50 | -44.03% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 13.37 | 0.00% | 39.56 | 39.32 | -0.60% | 72.47 | 74.20 | 2.39% |
|  | 5% | 10.95 | 10.95 | 0.00% | 11.01 | 11.01 | 0.00% | 11.10 | 11.10 | 0.00% |
|  | 50% | 11.60 | 11.60 | 0.02% | 15.32 | 15.31 | -0.03% | 24.38 | 24.63 | 1.03% |
| UL Packet-Latency CDF (ms) | Mean | 36.12 | 43.52 | 20.50% | 73.80 | 87.37 | 18.38% | 119.95 | 153.08 | 27.62% |
|  | 5% | 6.93 | 6.16 | -11.11% | 14.00 | 12.89 | -7.94% | 16.12 | 17.92 | 11.20% |
|  | 50% | 18.38 | 20.32 | 10.59% | 34.42 | 39.70 | 15.34% | 55.02 | 78.45 | 42.59% |
| DL RU (%) | Type-1 | 5.78% | 5.78% |  | 21.57% | 21.51% |  | 42.47% | 42.25% |  |
| UL RU (%) | Type-1 | 2.75% | 2.12% |  | 18.65% | 17.28% |  | 23.44% | 25.12% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size Transparent UL muting

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 339.31 | 0.00% | 272.65 | 272.69 | 0.01% | 214.67 | 214.63 | -0.02% |
|  | 5% | 234.38 | 234.38 | 0.00% | 146.63 | 145.23 | -0.95% | 45.05 | 45.55 | 1.11% |
|  | 50% | 354.46 | 354.46 | 0.00% | 284.13 | 284.13 | 0.00% | 224.15 | 223.55 | -0.27% |
| UL Average-UPT (Mbps) | Mean | 40.62 | 37.98 | -6.49% | 31.09 | 28.30 | -8.95% | 22.66 | 19.92 | -12.08% |
|  | 5% | 4.59 | 4.28 | -6.82% | 2.90 | 2.60 | -10.38% | 1.23 | 0.98 | -20.87% |
|  | 50% | 41.40 | 37.40 | -9.66% | 29.33 | 26.75 | -8.80% | 21.48 | 18.61 | -13.33% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 12.21 | 0.00% | 22.82 | 22.77 | -0.20% | 36.13 | 36.41 | 0.76% |
|  | 5% | 10.92 | 10.92 | 0.00% | 10.96 | 10.96 | 0.00% | 11.03 | 11.03 | -0.01% |
|  | 50% | 11.26 | 11.26 | 0.00% | 12.72 | 12.71 | -0.05% | 18.85 | 18.87 | 0.07% |
| UL Packet-Latency CDF (ms) | Mean | 50.47 | 54.42 | 7.83% | 69.81 | 75.20 | 7.71% | 96.64 | 106.18 | 9.88% |
|  | 5% | 13.66 | 14.12 | 3.31% | 14.32 | 15.14 | 5.74% | 17.02 | 19.05 | 11.94% |
|  | 50% | 25.15 | 27.28 | 8.45% | 36.30 | 40.15 | 10.59% | 49.87 | 57.55 | 15.40% |
| DL RU (%) | Type-1 | 5.43% | 5.43% |  | 25.23% | 25.21% |  | 40.05% | 40.06% |  |
| UL RU (%) | Type-1 | 1.66% | 1.78% |  | 6.38% | 6.82% |  | 10.66% | 11.38% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.60 | 303.68 | 0.02% | 188.12 | 194.22 | 3.24% | 95.03 | 104.18 | 9.64% |
|  | 5% | 174.19 | 174.19 | 0.00% | 6.98 | 6.78 | -2.87% | 1.80 | 1.80 | -0.05% |
|  | 50% | 324.85 | 324.85 | 0.00% | 206.95 | 216.65 | 4.69% | 68.48 | 83.25 | 21.58% |
| UL Average-UPT (Mbps) | Mean | 81.14 | 125.50 | 54.66% | 31.79 | 63.53 | 99.83% | 18.65 | 30.13 | 61.55% |
|  | 5% | 22.22 | 18.42 | -17.10% | 0.47 | 0.34 | -28.17% | 0.19 | 0.18 | -9.25% |
|  | 50% | 72.13 | 153.85 | 113.29% | 33.52 | 67.35 | 100.94% | 13.40 | 23.20 | 73.13% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 13.37 | -0.01% | 39.56 | 39.09 | -1.17% | 72.47 | 69.15 | -4.58% |
|  | 5% | 10.95 | 10.95 | 0.00% | 11.01 | 11.01 | 0.00% | 11.10 | 11.10 | -0.01% |
|  | 50% | 11.60 | 11.60 | 0.02% | 15.32 | 15.11 | -1.34% | 24.38 | 23.79 | -2.44% |
| UL Packet-Latency CDF (ms) | Mean | 36.12 | 37.02 | 2.50% | 73.80 | 68.79 | -6.78% | 119.95 | 111.50 | -7.05% |
|  | 5% | 6.93 | 5.18 | -25.20% | 14.00 | 7.57 | -45.97% | 16.12 | 13.09 | -18.79% |
|  | 50% | 18.38 | 17.63 | -4.05% | 34.42 | 26.74 | -22.30% | 55.02 | 51.42 | -6.54% |
| DL RU (%) | Type-1 | 5.78% | 5.78% |  | 21.57% | 21.37% |  | 42.47% | 42.49% |  |
| UL RU (%) | Type-1 | 2.75% | 1.44% |  | 18.65% | 10.25% |  | 23.44% | 17.88% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size Non-transparent UL muting

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 339.31 | 0.00% | 272.65 | 272.60 | -0.02% | 214.67 | 214.65 | -0.01% |
|  | 5% | 234.38 | 234.38 | 0.00% | 146.63 | 145.23 | -0.95% | 45.05 | 45.35 | 0.67% |
|  | 50% | 354.46 | 354.46 | 0.00% | 284.13 | 284.10 | -0.01% | 224.15 | 225.15 | 0.45% |
| UL Average-UPT (Mbps) | Mean | 40.62 | 40.63 | 0.05% | 31.09 | 31.14 | 0.17% | 22.66 | 22.70 | 0.20% |
|  | 5% | 4.59 | 4.59 | 0.00% | 2.90 | 3.00 | 3.42% | 1.23 | 1.25 | 1.16% |
|  | 50% | 41.40 | 40.70 | -1.69% | 29.33 | 29.52 | 0.64% | 21.48 | 21.85 | 1.75% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 12.21 | 0.00% | 22.82 | 22.80 | -0.09% | 36.13 | 36.39 | 0.71% |
|  | 5% | 10.92 | 10.92 | 0.00% | 10.96 | 10.96 | 0.00% | 11.03 | 11.03 | -0.01% |
|  | 50% | 11.26 | 11.26 | 0.00% | 12.72 | 12.71 | -0.05% | 18.85 | 18.85 | -0.05% |
| UL Packet-Latency CDF (ms) | Mean | 50.47 | 50.46 | -0.02% | 69.81 | 68.20 | -2.31% | 96.64 | 95.85 | -0.82% |
|  | 5% | 13.66 | 13.66 | 0.00% | 14.32 | 14.29 | -0.16% | 17.02 | 16.84 | -1.02% |
|  | 50% | 25.15 | 25.35 | 0.80% | 36.30 | 36.37 | 0.18% | 49.87 | 50.05 | 0.36% |
| DL RU (%) | Type-1 | 5.43% | 5.43% |  | 25.23% | 25.22% |  | 40.05% | 40.06% |  |
| UL RU (%) | Type-1 | 1.66% | 1.66% |  | 6.38% | 6.38% |  | 10.66% | 10.65% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.60 | 303.68 | 0.03% | 188.12 | 194.38 | 3.33% | 95.03 | 107.01 | 12.61% |
|  | 5% | 174.19 | 174.19 | 0.00% | 6.98 | 6.78 | -2.87% | 1.80 | 1.80 | -0.05% |
|  | 50% | 324.85 | 324.85 | 0.00% | 206.95 | 217.15 | 4.93% | 68.48 | 89.75 | 31.07% |
| UL Average-UPT (Mbps) | Mean | 81.14 | 137.20 | 69.09% | 31.79 | 69.82 | 119.63% | 18.65 | 34.77 | 86.41% |
|  | 5% | 22.22 | 22.34 | 0.54% | 0.47 | 0.44 | -6.04% | 0.19 | 0.20 | 2.00% |
|  | 50% | 72.13 | 165.66 | 129.66% | 33.52 | 74.63 | 122.65% | 13.40 | 28.30 | 111.19% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 13.37 | -0.01% | 39.56 | 39.21 | -0.88% | 72.47 | 67.73 | -6.54% |
|  | 5% | 10.95 | 10.95 | 0.00% | 11.01 | 11.01 | -0.01% | 11.10 | 11.10 | 0.00% |
|  | 50% | 11.60 | 11.60 | 0.00% | 15.32 | 15.08 | -1.58% | 24.38 | 23.53 | -3.50% |
| UL Packet-Latency CDF (ms) | Mean | 36.12 | 34.00 | -5.87% | 73.80 | 64.38 | -12.76% | 119.95 | 100.83 | -15.94% |
|  | 5% | 6.93 | 4.62 | -33.26% | 14.00 | 6.98 | -50.15% | 16.12 | 11.81 | -26.73% |
|  | 50% | 18.38 | 16.61 | -9.61% | 34.42 | 23.81 | -30.83% | 55.02 | 44.25 | -19.57% |
| DL RU (%) | Type-1 | 5.78% | 5.78% |  | 21.57% | 21.40% |  | 42.47% | 42.37% |  |
| UL RU (%) | Type-1 | 2.75% | 1.33% |  | 18.65% | 9.38% |  | 23.44% | 16.30% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 3 ([35])

2-layer Scenario B (FR1)

Symmetric Large Packet Size Baseline MMSE-IRC

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 121.69 | 118.04 | -3.00% | 89.88 | 93.24 | 3.74% | 65.71 | 62.56 | -4.79% |
|  | 5% | 47.06 | 54.42 | 15.65% | 17.62 | 26.76 | 51.84% | 7.98 | 10.35 | 29.63% |
|  | 50% | 100.00 | 119.40 | 19.40% | 86.96 | 89.89 | 3.37% | 55.17 | 54.05 | -2.03% |
| UL Average-UPT (Mbps) | Mean | 20.68 | 20.25 | -2.06% | 21.44 | 21.77 | 1.50% | 20.94 | 21.11 | 0.84% |
|  | 5% | 1.29 | 1.29 | 0.22% | 0.83 | 0.80 | -2.99% | 0.77 | 0.80 | 3.81% |
|  | 50% | 16.26 | 14.76 | -9.23% | 23.39 | 24.17 | 3.32% | 22.28 | 22.60 | 1.41% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.44% | 7.49% |  | 39.24% | 37.95% |  | 71.55% | 73.76% |  |
| UL RU (%) | Type-1 | 13.50% | 13.90% |  | 34.33% | 35.15% |  | 50.00% | 59.28% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.31 | 59.72 | -40.47% | 77.20 | 40.50 | -47.54% | 54.32 | 14.61 | -73.11% |
|  | 5% | 72.07 | 34.78 | -51.74% | 15.50 | 8.23 | -46.91% | 4.26 | 1.54 | -63.78% |
|  | 50% | 93.02 | 58.39 | -37.23% | 74.07 | 40.61 | -45.18% | 46.24 | 9.82 | -78.77% |
| UL Average-UPT (Mbps) | Mean | 41.70 | 111.46 | 167.28% | 38.21 | 82.36 | 115.56% | 19.96 | 32.67 | 63.72% |
|  | 5% | 37.38 | 81.63 | 118.37% | 27.21 | 47.62 | 75.00% | 3.29 | 6.97 | 111.76% |
|  | 50% | 42.33 | 119.40 | 182.09% | 39.60 | 81.63 | 106.12% | 21.39 | 31.01 | 44.96% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.17% | 2.99% |  | 27.60% | 15.94% |  | 56.54% | 74.16% |  |
| UL RU (%) | Type-1 | 22.97% | 11.59% |  | 42.34% | 36.93% |  | 50.00% | 59.28% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Transparent UL resource muting

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 121.69 | 106.01 | -12.88% | 89.88 | 84.61 | -5.86% | 65.71 | 53.88 | -17.99% |
|  | 5% | 47.06 | 37.91 | -19.43% | 17.62 | 26.23 | 48.85% | 7.98 | 7.75 | -2.98% |
|  | 50% | 100.00 | 90.91 | -9.09% | 86.96 | 80.81 | -7.07% | 55.17 | 43.88 | -20.46% |
| UL Average-UPT (Mbps) | Mean | 20.68 | 21.10 | 5.00% | 21.44 | 22.39 | 4.43% | 20.94 | 19.84 | -5.23% |
|  | 5% | 1.29 | 1.37 | 3.29% | 0.83 | 0.85 | 2.90% | 0.77 | 0.84 | 9.56% |
|  | 50% | 16.26 | 14.48 | -5.75% | 23.39 | 24.37 | 4.20% | 22.28 | 20.93 | -6.08% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.44% | 7.84% |  | 39.24% | 39.20% |  | 71.55% | 78.27% |  |
| UL RU (%) | Type-1 | 13.50% | 17.02% |  | 34.33% | 38.29% |  | 50.00% | 47.73% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.31 | 58.21 | -41.97% | 77.20 | 38.74 | -49.82% | 54.32 | 14.51 | -73.29% |
|  | 5% | 72.07 | 33.85 | -53.04% | 15.50 | 8.09 | -47.83% | 4.26 | 1.33 | -68.63% |
|  | 50% | 93.02 | 59.07 | -36.50% | 74.07 | 37.93 | -48.79% | 46.24 | 8.98 | -80.58% |
| UL Average-UPT (Mbps) | Mean | 41.70 | 96.05 | 130.34% | 38.21 | 89.36 | 133.87% | 19.96 | 61.93 | 210.32% |
|  | 5% | 37.38 | 86.02 | 130.11% | 27.21 | 62.50 | 129.69% | 3.29 | 20.89 | 534.73% |
|  | 50% | 42.33 | 97.56 | 130.49% | 39.60 | 94.12 | 137.65% | 21.39 | 64.52 | 201.61% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.17% | 3.78% |  | 27.60% | 22.47% |  | 56.54% | 82.16% |  |
| UL RU (%) | Type-1 | 22.97% | 12.67% |  | 42.34% | 34.96% |  | 50.00% | 57.61% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Symmetric Large Packet Size Non-transparent UL resource muting based MMSE-IRC

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 121.69 | 104.47 | -14.15% | 89.88 | 80.75 | -10.16% | 65.71 | 53.98 | -17.85% |
|  | 5% | 47.06 | 49.38 | 4.94% | 17.62 | 21.62 | 22.70% | 7.98 | 7.91 | -0.89% |
|  | 50% | 100.00 | 90.91 | -9.09% | 86.96 | 77.67 | -10.68% | 55.17 | 44.35 | -19.62% |
| UL Average-UPT (Mbps) | Mean | 20.68 | 19.89 | -3.78% | 21.44 | 22.68 | 5.75% | 20.94 | 21.46 | 2.48% |
|  | 5% | 1.29 | 1.34 | 3.90% | 0.83 | 0.86 | 3.14% | 0.77 | 0.84 | 9.42% |
|  | 50% | 16.26 | 15.48 | -4.81% | 23.39 | 24.40 | 4.29% | 22.28 | 22.40 | 0.50% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 6.44% | 6.98% |  | 39.24% | 40.57% |  | 71.55% | 79.52% |  |
| UL RU (%) | Type-1 | 13.50% | 13.81% |  | 34.33% | 33.90% |  | 50.00% | 44.43% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 100.31 | 60.18 | -40.01% | 77.20 | 37.93 | -50.87% | 54.32 | 14.28 | -73.71% |
|  | 5% | 72.07 | 35.49 | -50.76% | 15.50 | 7.81 | -49.60% | 4.26 | 1.41 | -66.93% |
|  | 50% | 93.02 | 59.87 | -35.64% | 74.07 | 38.56 | -47.95% | 46.24 | 9.21 | -80.08% |
| UL Average-UPT (Mbps) | Mean | 41.70 | 112.47 | 169.70% | 38.21 | 107.20 | 180.59% | 19.96 | 82.42 | 312.98% |
|  | 5% | 37.38 | 103.90 | 177.92% | 27.21 | 82.47 | 203.09% | 3.29 | 33.06 | 904.56% |
|  | 50% | 42.33 | 112.68 | 166.20% | 39.60 | 111.11 | 180.56% | 21.39 | 87.91 | 310.99% |
| DL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| UL Packet-Latency CDF (ms) | Mean |  |  |  |  |  |  |  |  |  |
|  | 5% |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |
| DL RU (%) | Type-1 | 1.17% | 2.60% |  | 27.60% | 27.92% |  | 56.54% | 80.07% |  |
| UL RU (%) | Type-1 | 22.97% | 9.82% |  | 42.34% | 31.10% |  | 50.00% | 53.06% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

B.4.2A	Inter-gNB CLI scheme 2A: Time Domain Scheme using UL slot(s) aligned between gNBs

- Source 1 ([44])

Urban Macro (FR1)

Large Packet Size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 606.03 | 502.71 | -17.05% | 494.42 | 376.60 | -23.83% | 0.00 | 259.57 | NaN% |
|  | 5% | 277.68 | 227.06 | -18.23% | 0.00 | 165.54 | NaN% | 0.00 | 77.15 | NaN% |
|  | 50% | 652.17 | 539.08 | -17.34% | 513.38 | 383.98 | -25.21% | 0.00 | 251.58 | NaN% |
| UL Average-UPT (Mbps) | Mean | 58.16 | 64.07 | 10.18% | 1.76 | 30.38 | 1630.98% | 0.04 | 21.69 | NaN% |
|  | 5% | 0.61 | 1.21 | 97.08% | 0.00 | 0.45 | NaN% | 0.00 | 0.29 | NaN% |
|  | 50% | 53.23 | 61.37 | 15.28% | 0.48 | 28.19 | 5770.71% | 0.00 | 18.16 | NaN% |
| DL Packet-Latency CDF (ms) | Mean | 8.21 | 10.21 | 24.27% | 11.52 | 16.53 | 43.56% | 18.08 | 44.66 | NaN% |
|  | 5% | 4.67 | 5.48 | 17.29% | 4.88 | 5.90 | 20.89% | 5.22 | 6.65 | NaN% |
|  | 50% | 6.28 | 7.57 | 20.56% | 7.99 | 10.89 | 36.25% | 10.90 | 18.01 | NaN% |
| UL Packet-Latency CDF (ms) | Mean | 95.12 | 77.89 | -18.12% | 729.68 | 124.32 | -82.96% | 1160.17 | 171.05 | NaN% |
|  | 5% | 5.02 | 5.27 | 4.90% | 78.47 | 13.67 | -82.58% | 212.32 | 15.46 | NaN% |
|  | 50% | 26.55 | 18.45 | -30.52% | 603.65 | 36.62 | -93.93% | 1235.33 | 61.56 | NaN% |
| DL RU (%) | Type-1 | 5.37% | 5.49% |  | 19.61% | 21.93% |  | 37.18% | 42.03% |  |
| UL RU (%) | Type-1 | 1.68% | 1.54% |  | 4.80% | 6.73% |  | 1.31% | 12.91% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 2 ([32])

2-layer Scenario B (FR1)

Large Packet Size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 486.30 | 474.47 | -2.43% | 448.64 | 421.00 | -6.16% | 346.75 | 312.43 | -9.90% |
|  | 5% | 197.16 | 189.85 | -3.71% | 175.05 | 153.41 | -12.36% | 95.23 | 72.06 | -24.33% |
|  | 50% | 600.24 | 574.25 | -4.33% | 527.02 | 491.22 | -6.79% | 383.21 | 320.82 | -16.28% |
| UL Average-UPT (Mbps) | Mean | 139.38 | 139.85 | 0.34% | 125.81 | 126.02 | 0.17% | 101.70 | 102.32 | 0.61% |
|  | 5% | 100.13 | 100.13 | 0.00% | 84.22 | 85.71 | 1.77% | 20.63 | 23.33 | 13.09% |
|  | 50% | 142.93 | 142.76 | -0.12% | 129.79 | 130.23 | 0.34% | 110.12 | 110.78 | 0.60% |
| DL Packet-Latency CDF (ms) | Mean | 10.67 | 10.99 | 3.00% | 12.58 | 13.86 | 10.17% | 22.51 | 26.50 | 17.73% |
|  | 5% | 5.98 | 6.02 | 0.67% | 5.98 | 6.02 | 0.67% | 6.05 | 6.09 | 0.66% |
|  | 50% | 6.70 | 6.80 | 1.49% | 6.95 | 9.09 | 30.79% | 13.27 | 14.38 | 8.36% |
| UL Packet-Latency CDF (ms) | Mean | 8.03 | 7.88 | -1.87% | 10.32 | 10.09 | -2.23% | 33.24 | 32.18 | -3.19% |
|  | 5% | 5.59 | 5.59 | 0.00% | 5.63 | 5.63 | 0.00% | 5.70 | 5.70 | 0.00% |
|  | 50% | 6.98 | 6.98 | 0.00% | 7.34 | 7.34 | 0.00% | 9.41 | 9.45 | 0.43% |
| DL RU (%) | Type-1 | 3.79% | 3.89% |  | 12.35% | 13.27% |  | 36.16% | 39.55% |  |
| UL RU (%) | Type-1 | 2.64% | 2.57% |  | 6.68% | 6.53% |  | 11.98% | 11.69% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 428.93 | 96.62 | -77.47% | 384.31 | 56.56 | -85.28% | 250.49 | 31.38 | -87.47% |
|  | 5% | 200.68 | 36.00 | -82.06% | 161.66 | 8.63 | -94.66% | 62.85 | 4.38 | -93.03% |
|  | 50% | 498.57 | 98.82 | -80.18% | 443.71 | 52.69 | -88.13% | 222.36 | 18.43 | -91.71% |
| UL Average-UPT (Mbps) | Mean | 142.87 | 413.91 | 189.71% | 132.91 | 417.99 | 214.49% | 113.16 | 400.27 | 253.72% |
|  | 5% | 123.09 | 246.14 | 99.97% | 108.53 | 369.20 | 240.18% | 64.04 | 353.53 | 452.05% |
|  | 50% | 144.14 | 429.07 | 197.68% | 134.87 | 420.77 | 211.98% | 116.84 | 400.38 | 242.67% |
| DL Packet-Latency CDF (ms) | Mean | 12.12 | 61.19 | 404.87% | 14.91 | 158.30 | 961.70% | 31.36 | 318.66 | 916.14% |
|  | 5% | 6.02 | 23.63 | 292.52% | 6.02 | 24.48 | 306.64% | 6.30 | 25.98 | 312.38% |
|  | 50% | 7.20 | 51.91 | 620.97% | 12.38 | 79.41 | 541.44% | 19.30 | 206.55 | 970.21% |
| UL Packet-Latency CDF (ms) | Mean | 8.91 | 3.17 | -64.42% | 9.82 | 4.33 | -55.91% | 13.49 | 2.79 | -79.32% |
|  | 5% | 5.55 | 1.55 | -72.07% | 5.63 | 1.55 | -72.47% | 5.66 | 1.59 | -71.91% |
|  | 50% | 6.80 | 2.55 | -62.50% | 7.13 | 2.55 | -64.24% | 7.84 | 2.59 | -66.96% |
| DL RU (%) | Type-1 | 4.50% | 4.58% |  | 14.02% | 14.46% |  | 44.23% | 19.61% |  |
| UL RU (%) | Type-1 | 2.64% | 3.90% |  | 5.99% | 6.76% |  | 10.57% | 10.16% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

B.4.2B	Inter-gNB CLI scheme 2B: Frequency Domain Coordination Scheme

- Source 1 ([26]):

Indoor Office (FR1)

Small packet size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 42.61 | -1.13% | 42.11 | 41.79 | -0.76% | 41.40 | 41.25 | -0.36% |
|  | 5% | 41.76 | 41.15 | -1.47% | 41.22 | 41.02 | -0.48% | 40.78 | 40.40 | -0.94% |
|  | 50% | 43.08 | 42.55 | -1.23% | 41.91 | 41.63 | -0.65% | 41.37 | 41.31 | -0.16% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.27 | 0.52% | 4.23 | 4.26 | 0.65% | 3.95 | 4.21 | 6.59% |
|  | 5% | 4.19 | 4.20 | 0.37% | 4.20 | 4.21 | 0.35% | 2.54 | 4.01 | 58.13% |
|  | 50% | 4.25 | 4.26 | 0.40% | 4.23 | 4.26 | 0.58% | 4.21 | 4.24 | 0.72% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 2.07 | 81.25% | 1.24 | 1.40 | 12.83% | 1.23 | 1.25 | 1.98% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.52 | -3.33% | 0.55 | 0.54 | -3.23% |
|  | 50% | 0.80 | 0.82 | 2.22% | 0.84 | 0.82 | -2.13% | 0.88 | 0.88 | 0.00% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 2.27 | 3.15% | 4.17 | 2.28 | -45.47% | 45.40 | 2.41 | -94.70% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.59 | -0.89% | 1.63 | 1.59 | -2.20% |
|  | 50% | 1.88 | 1.89 | 0.95% | 1.91 | 1.89 | -0.93% | 1.96 | 1.95 | -0.91% |
| DL RU (%) | Type-1 | 3.55% | 20.42% |  | 23.29% | 33.79% |  | 49.12% | 50.15% |  |
| UL RU (%) | Type-1 | 1.71% | 1.33% |  | 14.68% | 7.05% |  | 39.61% | 12.83% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 735.54 | -23.81% | 579.11 | 463.84 | -19.90% | 443.87 | 364.00 | -17.99% |
|  | 5% | 680.10 | 505.27 | -25.71% | 363.91 | 288.96 | -20.60% | 258.74 | 203.95 | -21.17% |
|  | 50% | 951.37 | 732.87 | -22.97% | 557.41 | 452.15 | -18.88% | 411.05 | 347.50 | -15.46% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 125.51 | -56.89% | 132.35 | 91.23 | -31.07% | 92.82 | 69.45 | -25.18% |
|  | 5% | 231.38 | 106.59 | -53.93% | 67.53 | 61.67 | -8.67% | 26.95 | 35.69 | 32.45% |
|  | 50% | 292.47 | 128.16 | -56.18% | 129.13 | 89.53 | -30.66% | 86.65 | 66.09 | -23.72% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 17.01 | 263.34% | 8.24 | 12.41 | 50.67% | 11.38 | 14.87 | 30.63% |
|  | 5% | 2.77 | 3.55 | 28.39% | 4.08 | 4.93 | 20.59% | 4.83 | 5.71 | 18.41% |
|  | 50% | 3.95 | 4.88 | 23.53% | 7.08 | 8.66 | 22.32% | 9.57 | 11.54 | 20.52% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 8.15 | 111.51% | 12.60 | 12.41 | -1.58% | 28.25 | 18.99 | -32.78% |
|  | 5% | 2.64 | 7.11 | 168.92% | 3.91 | 7.24 | 85.48% | 4.97 | 7.78 | 56.51% |
|  | 50% | 3.07 | 7.42 | 141.57% | 7.48 | 10.59 | 41.56% | 10.80 | 14.11 | 30.58% |
| DL RU (%) | Type-1 | 3.58% | 78.34% |  | 25.29% | 78.96% |  | 46.52% | 79.30% |  |
| UL RU (%) | Type-1 | 1.74% | 1.04% |  | 22.11% | 5.70% |  | 46.93% | 10.48% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Urban Macro (FR1)

Small packet size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 38.52 | 40.16 | 4.27% | 37.38 | 37.38 | 0.00% | 33.55 | 30.68 | -8.56% |
|  | 5% | 35.88 | 28.33 | -21.03% | 30.45 | 12.08 | -60.33% | 6.93 | 0.08 | -98.78% |
|  | 50% | 39.08 | 41.53 | 6.26% | 38.45 | 40.51 | 5.35% | 37.38 | 38.13 | 2.01% |
| UL Average-UPT (Mbps) | Mean | 2.71 | 3.63 | 34.24% | 2.26 | 3.50 | 54.77% | 1.63 | 3.32 | 103.83% |
|  | 5% | 0.11 | 0.39 | 265.84% | 0.00 | 0.19 | 95803.92% | 0.00 | 0.01 | NAN |
|  | 50% | 3.27 | 4.34 | 32.70% | 2.83 | 4.29 | 51.75% | 1.47 | 4.22 | 185.86% |
| DL Packet-Latency CDF (ms) | Mean | 1.41 | 1.61 | 13.82% | 1.58 | 10.94 | 592.54% | 8.11 | 19.40 | 139.23% |
|  | 5% | 0.55 | 0.55 | 0.00% | 0.55 | 0.57 | 3.23% | 0.57 | 0.57 | 0.00% |
|  | 50% | 0.92 | 0.86 | -6.80% | 0.88 | 0.88 | 0.00% | 0.93 | 0.95 | 1.92% |
| UL Packet-Latency CDF (ms) | Mean | 30.44 | 11.56 | -62.03% | 24.25 | 22.02 | -9.21% | 9.14 | 16.27 | 77.87% |
|  | 5% | 1.74 | 1.61 | -7.79% | 1.74 | 1.64 | -5.60% | 1.71 | 1.63 | -5.16% |
|  | 50% | 2.96 | 2.03 | -31.63% | 2.88 | 2.02 | -29.81% | 2.86 | 2.05 | -28.38% |
| DL RU (%) | Type-1 | 6.46% | 9.05% |  | 17.96% | 28.41% |  | 40.97% | 50.72% |  |
| UL RU (%) | Type-1 | 3.00% | 2.10% |  | 8.76% | 5.58% |  | 16.73% | 9.65% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 713.01 | 613.89 | -13.90% | 541.06 | 432.57 | -20.05% | 356.07 | 242.84 | -31.80% |
|  | 5% | 95.87 | 73.81 | -23.01% | 37.49 | 9.75 | -74.00% | 0.20 | 0.00 | -100.00% |
|  | 50% | 726.42 | 616.91 | -15.08% | 546.92 | 401.04 | -26.67% | 339.60 | 176.02 | -48.17% |
| UL Average-UPT (Mbps) | Mean | 76.15 | 49.59 | -34.87% | 56.08 | 43.36 | -22.68% | 37.05 | 34.31 | -7.40% |
|  | 5% | 0.00 | 0.00 | NAN | 0.00 | 0.00 | NAN | 0.00 | 0.00 | NAN |
|  | 50% | 52.79 | 39.08 | -25.98% | 22.05 | 34.16 | 54.94% | 0.00 | 25.19 | NAN |
| DL Packet-Latency CDF (ms) | Mean | 21.84 | 52.47 | 140.25% | 95.59 | 266.49 | 178.79% | 333.29 | 485.77 | 45.75% |
|  | 5% | 2.75 | 2.94 | 6.98% | 3.12 | 3.55 | 13.94% | 3.60 | 4.10 | 13.98% |
|  | 50% | 5.50 | 6.21 | 12.99% | 8.16 | 10.28 | 25.98% | 13.84 | 22.90 | 65.47% |
| UL Packet-Latency CDF (ms) | Mean | 83.36 | 110.07 | 32.03% | 108.20 | 106.93 | -1.18% | 59.03 | 141.04 | 138.94% |
|  | 5% | 3.99 | 7.59 | 90.38% | 4.01 | 8.05 | 100.86% | 4.26 | 9.28 | 117.62% |
|  | 50% | 12.83 | 22.68 | 76.79% | 11.78 | 24.16 | 105.05% | 13.08 | 36.54 | 179.35% |
| DL RU (%) | Type-1 | 6.37% | 9.72% |  | 20.73% | 27.55% |  | 41.76% | 50.11% |  |
| UL RU (%) | Type-1 | 3.28% | 2.07% |  | 7.35% | 3.43% |  | 12.46% | 7.17% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 2 ([24]):

2-layer Scenario B (FR1)

Large Packet Size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 265.10 | -21.87% | 272.60 | 206.38 | -24.29% | 214.65 | 153.90 | -28.30% |
|  | 5% | 234.38 | 149.98 | -36.01% | 145.23 | 82.14 | -43.44% | 45.35 | 20.35 | -55.13% |
|  | 50% | 354.46 | 294.80 | -16.83% | 284.10 | 214.79 | -24.40% | 225.15 | 157.55 | -30.02% |
| UL Average-UPT (Mbps) | Mean | 40.63 | 48.46 | 19.27% | 31.14 | 31.00 | -0.45% | 22.70 | 21.14 | -6.89% |
|  | 5% | 4.59 | 10.29 | 123.91% | 3.00 | 4.00 | 33.28% | 1.25 | 1.19 | -4.93% |
|  | 50% | 40.70 | 42.15 | 3.56% | 29.52 | 23.15 | -21.57% | 21.85 | 14.35 | -34.32% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 16.51 | 35.27% | 22.80 | 31.48 | 38.10% | 36.39 | 51.81 | 42.37% |
|  | 5% | 10.92 | 12.64 | 15.73% | 10.96 | 12.85 | 17.24% | 11.03 | 12.99 | 17.80% |
|  | 50% | 11.26 | 13.37 | 18.70% | 12.71 | 18.68 | 47.00% | 18.85 | 28.55 | 51.49% |
| UL Packet-Latency CDF (ms) | Mean | 50.46 | 35.45 | -29.75% | 68.20 | 67.21 | -1.44% | 95.85 | 117.96 | 23.07% |
|  | 5% | 13.66 | 10.11 | -26.02% | 14.29 | 12.66 | -11.41% | 16.84 | 15.86 | -5.87% |
|  | 50% | 25.35 | 23.00 | -9.27% | 36.37 | 42.17 | 15.96% | 50.05 | 63.55 | 26.97% |
| DL RU (%) | Type-1 | 5.43% | 6.43% |  | 25.22% | 27.36% |  | 40.06% | 42.19% |  |
| UL RU (%) | Type-1 | 1.66% | 2.37% |  | 6.38% | 8.80% |  | 10.65% | 13.18% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.68 | 213.48 | -29.70% | 194.38 | 108.53 | -44.17% | 107.01 | 49.60 | -53.65% |
|  | 5% | 174.19 | 81.19 | -53.39% | 6.78 | 2.68 | -60.52% | 1.80 | 1.00 | -44.41% |
|  | 50% | 324.85 | 214.68 | -33.92% | 217.15 | 104.45 | -51.90% | 89.75 | 17.55 | -80.45% |
| UL Average-UPT (Mbps) | Mean | 137.20 | 72.52 | -47.14% | 69.82 | 48.17 | -31.01% | 34.77 | 36.37 | 4.62% |
|  | 5% | 22.34 | 18.25 | -18.33% | 0.44 | 0.34 | -22.29% | 0.20 | 0.17 | -12.24% |
|  | 50% | 165.66 | 87.95 | -46.91% | 74.63 | 55.53 | -25.59% | 28.30 | 37.60 | 32.86% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 19.94 | 49.14% | 39.21 | 64.97 | 65.71% | 67.73 | 110.88 | 63.69% |
|  | 5% | 10.95 | 12.74 | 16.38% | 11.01 | 12.93 | 17.49% | 11.10 | 13.08 | 17.80% |
|  | 50% | 11.60 | 15.48 | 33.42% | 15.08 | 22.53 | 49.46% | 23.53 | 37.85 | 60.87% |
| UL Packet-Latency CDF (ms) | Mean | 34.00 | 29.16 | -14.22% | 64.38 | 72.38 | 12.41% | 100.83 | 115.86 | 14.91% |
|  | 5% | 4.62 | 9.19 | 98.68% | 6.98 | 10.15 | 45.42% | 11.81 | 11.06 | -6.36% |
|  | 50% | 16.61 | 19.24 | 15.83% | 23.81 | 29.79 | 25.12% | 44.25 | 44.97 | 1.62% |
| DL RU (%) | Type-1 | 5.78% | 7.30% |  | 21.40% | 24.50% |  | 42.37% | 38.64% |  |
| UL RU (%) | Type-1 | 1.33% | 1.64% |  | 9.38% | 7.93% |  | 16.30% | 12.23% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

B.4.3A	Inter-gNB CLI scheme 3A: Spatial Domain Coordination Scheme for gNB Tx-Beam Nulling

- Source 1 ([43]):

Urban Macro (FR1)

Large Packet Size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 1031.81 | 1052.31 | 1.99% | 781.51 | 796.71 | 1.94% | 511.12 | 519.71 | 1.68% |
|  | 5% | 310.68 | 302.29 | -2.70% | 201.17 | 203.42 | 1.12% | 30.76 | 22.26 | -27.65% |
|  | 50% | 1077.35 | 1118.81 | 3.85% | 794.01 | 814.77 | 2.61% | 464.72 | 469.50 | 1.03% |
| UL Average-UPT (Mbps) | Mean | 315.39 | 316.88 | 0.47% | 224.48 | 238.77 | 6.36% | 87.45 | 105.53 | 20.67% |
|  | 5% | 62.64 | 77.89 | 24.34% | 25.33 | 33.44 | 32.04% | 1.76 | 8.12 | 360.03% |
|  | 50% | 284.26 | 290.40 | 2.16% | 204.40 | 205.08 | 0.33% | 72.06 | 83.88 | 16.40% |
| DL Packet-Latency CDF (ms) | Mean | 5.18 | 5.18 | -0.10% | 8.96 | 8.94 | -0.30% | 30.06 | 37.73 | 25.52% |
|  | 5% | 2.52 | 2.52 | 0.00% | 2.63 | 2.63 | 0.00% | 3.02 | 3.02 | 0.00% |
|  | 50% | 3.66 | 3.55 | -2.93% | 5.34 | 5.27 | -1.34% | 8.59 | 8.84 | 2.91% |
| UL Packet-Latency CDF (ms) | Mean | 5.12 | 4.64 | -9.30% | 7.56 | 7.20 | -4.78% | 31.36 | 27.03 | -13.82% |
|  | 5% | 1.48 | 1.52 | 2.42% | 1.59 | 1.59 | 0.00% | 3.09 | 2.13 | -31.21% |
|  | 50% | 3.41 | 3.38 | -1.05% | 5.20 | 5.05 | -2.75% | 13.48 | 12.73 | -5.56% |
| DL RU (%) | Type-1 | 8.32% | 8.37% |  | 33.85% | 33.54% |  | 73.40% | 73.55% |  |
| UL RU (%) | Type-1 | 2.58% | 2.76% |  | 7.87% | 7.96% |  | 12.40% | 12.79% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 2 ([26])

Indoor Office (FR1)

Small packet size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 42.94 | -0.36% | 42.11 | 41.75 | -0.85% | 41.40 | 40.77 | -1.51% |
|  | 5% | 41.76 | 41.36 | -0.96% | 41.22 | 41.09 | -0.31% | 40.78 | 38.55 | -5.48% |
|  | 50% | 43.08 | 42.97 | -0.26% | 41.91 | 41.62 | -0.68% | 41.37 | 41.17 | -0.50% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.26 | 0.28% | 4.23 | 4.24 | 0.37% | 3.95 | 4.24 | 7.50% |
|  | 5% | 4.19 | 4.21 | 0.47% | 4.20 | 4.22 | 0.41% | 2.54 | 4.21 | 65.99% |
|  | 50% | 4.25 | 4.26 | 0.25% | 4.23 | 4.24 | 0.29% | 4.21 | 4.24 | 0.75% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 1.17 | 1.83% | 1.24 | 1.24 | -0.40% | 1.23 | 1.25 | 1.70% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.52 | -2.17% | 0.55 | 0.52 | -6.45% |
|  | 50% | 0.80 | 0.80 | 0.00% | 0.84 | 0.82 | -2.13% | 0.88 | 0.86 | -2.04% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 2.19 | -0.83% | 4.17 | 2.21 | -46.96% | 45.40 | 2.16 | -95.25% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.59 | -0.89% | 1.63 | 1.59 | -2.36% |
|  | 50% | 1.88 | 1.88 | 0.00% | 1.91 | 1.89 | -0.93% | 1.96 | 1.91 | -2.73% |
| DL RU (%) | Type-1 | 3.55% | 3.62% |  | 23.29% | 25.56% |  | 49.12% | 56.15% |  |
| UL RU (%) | Type-1 | 1.71% | 1.29% |  | 14.68% | 8.14% |  | 39.61% | 17.22% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 870.38 | -9.84% | 579.11 | 505.21 | -12.76% | 443.87 | 385.07 | -13.25% |
|  | 5% | 680.10 | 520.01 | -23.54% | 363.91 | 257.95 | -29.12% | 258.74 | 180.51 | -30.24% |
|  | 50% | 951.37 | 887.50 | -6.71% | 557.41 | 495.58 | -11.09% | 411.05 | 366.81 | -10.76% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 319.06 | 9.59% | 132.35 | 229.07 | 73.07% | 92.82 | 199.44 | 114.88% |
|  | 5% | 231.38 | 270.18 | 16.77% | 67.53 | 133.67 | 97.95% | 26.95 | 110.95 | 311.71% |
|  | 50% | 292.47 | 324.76 | 11.04% | 129.13 | 224.75 | 74.05% | 86.65 | 193.72 | 123.57% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 5.41 | 15.51% | 8.24 | 10.07 | 22.26% | 11.38 | 14.96 | 31.41% |
|  | 5% | 2.77 | 2.86 | 3.23% | 4.08 | 4.36 | 6.80% | 4.83 | 5.27 | 9.25% |
|  | 50% | 3.95 | 4.43 | 12.22% | 7.08 | 8.27 | 16.77% | 9.57 | 11.14 | 16.42% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 3.31 | -13.93% | 12.60 | 5.21 | -58.68% | 28.25 | 6.44 | -77.22% |
|  | 5% | 2.64 | 2.62 | -0.91% | 3.91 | 2.82 | -27.88% | 4.97 | 3.10 | -37.56% |
|  | 50% | 3.07 | 2.95 | -4.07% | 7.48 | 4.50 | -39.84% | 10.80 | 5.16 | -52.23% |
| DL RU (%) | Type-1 | 3.58% | 4.13% |  | 25.29% | 29.19% |  | 46.52% | 52.45% |  |
| UL RU (%) | Type-1 | 1.74% | 1.40% |  | 22.11% | 10.59% |  | 46.93% | 20.59% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 3 ([42]):

Urban Macro + indoor (FR1)

The field test is conducted under 2-layer Scenario B, where the layer1 urban macro is configured with DDDSUDDSUU pattern and layer2 indoor (gymnasium or factory) is configured with DSUUUDSUUU pattern.

| Results of the field test | Layer1: Urban | - For the urban Macro (layer1) + gymnasium (layer2) scenario, compared with the case where Tx-Beam nulling is not performed, Tx-Beam nulling shows 5.7% average DL throughput loss for urban macro for high traffic load. - For the urban Macro (layer1) + factory (layer2) scenario, compared with the case where Tx-beam nulling is not performed, Tx-beam nulling shows 4.9% average DL throughput loss for urban macro for high traffic load. |
| --- | --- | --- |
|  | Layer2: Indoor | - For the urban Macro (layer1) + gymnasium (layer2) scenario, compared with the case where Tx-Beam nulling is not performed, Tx-Beam nulling shows 28.0% average UL throughput gain for gymnasium for high traffic load. - For the urban Macro (layer1) + factory (layer2) scenario, compared with the case where Tx-beam nulling is not performed, Tx-beam nulling shows 21.0% average UL throughput gain for factory for high traffic load. |

B.4.5A	Inter-gNB CLI scheme 5A: Power Control scheme based on gNB Tx Power Adjustment

- Source 1 ([24]):

2-layer Scenario B (FR1)

Large Packet Size

Power back off = 3 dB

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 339.15 | -0.05% | 272.60 | 272.43 | -0.06% | 214.65 | 214.29 | -0.17% |
|  | 5% | 234.38 | 234.38 | 0.00% | 145.23 | 145.23 | 0.00% | 45.35 | 50.42 | 11.20% |
|  | 50% | 354.46 | 354.46 | 0.00% | 284.10 | 283.90 | -0.07% | 225.15 | 224.05 | -0.49% |
| UL Average-UPT (Mbps) | Mean | 40.63 | 40.63 | 0.00% | 31.14 | 31.12 | -0.06% | 22.70 | 22.64 | -0.27% |
|  | 5% | 4.59 | 4.59 | 0.00% | 3.00 | 3.00 | -0.08% | 1.25 | 1.18 | -5.23% |
|  | 50% | 40.70 | 40.70 | 0.00% | 29.52 | 29.38 | -0.45% | 21.85 | 21.95 | 0.46% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 12.24 | 0.29% | 22.80 | 23.04 | 1.07% | 36.39 | 36.20 | -0.54% |
|  | 5% | 10.92 | 10.92 | 0.00% | 10.96 | 10.96 | 0.00% | 11.03 | 11.03 | 0.02% |
|  | 50% | 11.26 | 11.26 | 0.01% | 12.71 | 12.73 | 0.17% | 18.85 | 18.94 | 0.52% |
| UL Packet-Latency CDF (ms) | Mean | 50.46 | 50.46 | 0.00% | 68.20 | 69.51 | 1.92% | 95.85 | 96.28 | 0.45% |
|  | 5% | 13.66 | 13.66 | 0.00% | 14.29 | 14.31 | 0.14% | 16.84 | 16.97 | 0.75% |
|  | 50% | 25.35 | 25.35 | 0.00% | 36.37 | 36.25 | -0.32% | 50.05 | 50.23 | 0.35% |
| DL RU (%) | Type-1 | 5.43% | 5.44% |  | 25.22% | 25.24% |  | 40.06% | 40.11% |  |
| UL RU (%) | Type-1 | 1.66% | 1.66% |  | 6.38% | 6.38% |  | 10.65% | 10.64% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.68 | 303.66 | -0.01% | 194.38 | 194.57 | 0.10% | 107.01 | 108.10 | 1.02% |
|  | 5% | 174.19 | 174.19 | 0.00% | 6.78 | 7.24 | 6.83% | 1.80 | 1.80 | 0.05% |
|  | 50% | 324.85 | 324.85 | 0.00% | 217.15 | 218.33 | 0.54% | 89.75 | 94.05 | 4.79% |
| UL Average-UPT (Mbps) | Mean | 137.20 | 137.84 | 0.47% | 69.82 | 73.51 | 5.29% | 34.77 | 37.51 | 7.90% |
|  | 5% | 22.34 | 22.34 | 0.00% | 0.44 | 0.46 | 3.21% | 0.20 | 0.20 | -0.06% |
|  | 50% | 165.66 | 166.68 | 0.62% | 74.63 | 78.25 | 4.86% | 28.30 | 30.78 | 8.75% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 13.37 | 0.03% | 39.21 | 38.99 | -0.56% | 67.73 | 67.63 | -0.15% |
|  | 5% | 10.95 | 10.95 | 0.00% | 11.01 | 11.01 | -0.01% | 11.10 | 11.11 | 0.02% |
|  | 50% | 11.60 | 11.60 | 0.03% | 15.08 | 15.10 | 0.19% | 23.53 | 23.44 | -0.38% |
| UL Packet-Latency CDF (ms) | Mean | 34.00 | 33.99 | -0.03% | 64.38 | 63.89 | -0.78% | 100.83 | 99.01 | -1.80% |
|  | 5% | 4.62 | 4.62 | 0.00% | 6.98 | 6.59 | -5.54% | 11.81 | 11.10 | -6.03% |
|  | 50% | 16.61 | 16.61 | 0.00% | 23.81 | 23.35 | -1.90% | 44.25 | 43.33 | -2.07% |
| DL RU (%) | Type-1 | 5.78% | 5.78% |  | 21.40% | 21.37% |  | 42.37% | 42.34% |  |
| UL RU (%) | Type-1 | 1.33% | 1.33% |  | 9.38% | 8.89% |  | 16.30% | 15.48% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size

Power back off = 6 dB

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 338.86 | -0.14% | 272.60 | 271.45 | -0.42% | 214.65 | 213.80 | -0.40% |
|  | 5% | 234.38 | 234.38 | 0.00% | 145.23 | 141.43 | -2.62% | 45.35 | 43.25 | -4.63% |
|  | 50% | 354.46 | 354.45 | 0.00% | 284.10 | 283.60 | -0.18% | 225.15 | 223.65 | -0.67% |
| UL Average-UPT (Mbps) | Mean | 40.63 | 40.63 | 0.00% | 31.14 | 31.14 | -0.01% | 22.70 | 22.63 | -0.33% |
|  | 5% | 4.59 | 4.59 | 0.00% | 3.00 | 3.10 | 3.25% | 1.25 | 1.24 | -0.72% |
|  | 50% | 40.70 | 40.70 | 0.00% | 29.52 | 29.58 | 0.23% | 21.85 | 21.26 | -2.69% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 12.26 | 0.40% | 22.80 | 23.15 | 1.54% | 36.39 | 36.31 | -0.22% |
|  | 5% | 10.92 | 10.92 | 0.00% | 10.96 | 10.96 | 0.00% | 11.03 | 11.03 | 0.03% |
|  | 50% | 11.26 | 11.26 | 0.02% | 12.71 | 12.82 | 0.83% | 18.85 | 18.94 | 0.48% |
| UL Packet-Latency CDF (ms) | Mean | 50.46 | 50.46 | 0.00% | 68.20 | 68.96 | 1.12% | 95.85 | 95.93 | 0.09% |
|  | 5% | 13.66 | 13.66 | 0.00% | 14.29 | 14.31 | 0.12% | 16.84 | 16.87 | 0.15% |
|  | 50% | 25.35 | 25.35 | 0.00% | 36.37 | 36.03 | -0.94% | 50.05 | 50.65 | 1.20% |
| DL RU (%) | Type-1 | 5.43% | 5.44% |  | 25.22% | 25.32% |  | 40.06% | 40.18% |  |
| UL RU (%) | Type-1 | 1.66% | 1.66% |  | 6.38% | 6.37% |  | 10.65% | 10.65% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.68 | 303.58 | -0.03% | 194.38 | 193.66 | -0.37% | 107.01 | 108.37 | 1.27% |
|  | 5% | 174.19 | 174.19 | 0.00% | 6.78 | 6.59 | -2.77% | 1.80 | 1.80 | -0.14% |
|  | 50% | 324.85 | 324.85 | 0.00% | 217.15 | 217.35 | 0.09% | 89.75 | 95.28 | 6.16% |
| UL Average-UPT (Mbps) | Mean | 137.20 | 138.50 | 0.94% | 69.82 | 76.81 | 10.01% | 34.77 | 40.31 | 15.96% |
|  | 5% | 22.34 | 22.34 | 0.00% | 0.44 | 0.42 | -4.86% | 0.20 | 0.20 | 0.75% |
|  | 50% | 165.66 | 166.70 | 0.63% | 74.63 | 82.95 | 11.16% | 28.30 | 33.40 | 18.02% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 13.38 | 0.10% | 39.21 | 39.51 | 0.77% | 67.73 | 67.73 | 0.00% |
|  | 5% | 10.95 | 10.95 | 0.00% | 11.01 | 11.01 | 0.00% | 11.10 | 11.11 | 0.01% |
|  | 50% | 11.60 | 11.61 | 0.06% | 15.08 | 15.18 | 0.68% | 23.53 | 23.51 | -0.07% |
| UL Packet-Latency CDF (ms) | Mean | 34.00 | 33.98 | -0.06% | 64.38 | 64.27 | -0.18% | 100.83 | 97.74 | -3.06% |
|  | 5% | 4.62 | 4.61 | -0.29% | 6.98 | 6.40 | -8.36% | 11.81 | 10.03 | -15.12% |
|  | 50% | 16.61 | 16.61 | 0.00% | 23.81 | 23.17 | -2.68% | 44.25 | 42.43 | -4.12% |
| DL RU (%) | Type-1 | 5.78% | 5.78% |  | 21.40% | 21.49% |  | 42.37% | 42.43% |  |
| UL RU (%) | Type-1 | 1.33% | 1.32% |  | 9.38% | 8.43% |  | 16.30% | 14.64% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size

Power back off = 10 dB

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 339.31 | 337.70 | -0.47% | 272.60 | 269.65 | -1.08% | 214.65 | 211.15 | -1.63% |
|  | 5% | 234.38 | 232.38 | -0.85% | 145.23 | 142.33 | -2.00% | 45.35 | 41.75 | -7.94% |
|  | 50% | 354.46 | 354.42 | -0.01% | 284.10 | 282.35 | -0.62% | 225.15 | 219.65 | -2.44% |
| UL Average-UPT (Mbps) | Mean | 40.63 | 40.63 | 0.00% | 31.14 | 31.18 | 0.12% | 22.70 | 22.69 | -0.03% |
|  | 5% | 4.59 | 4.59 | 0.00% | 3.00 | 3.10 | 3.25% | 1.25 | 1.22 | -2.05% |
|  | 50% | 40.70 | 40.70 | 0.00% | 29.52 | 29.25 | -0.90% | 21.85 | 21.66 | -0.86% |
| DL Packet-Latency CDF (ms) | Mean | 12.21 | 12.32 | 0.93% | 22.80 | 23.26 | 2.02% | 36.39 | 36.70 | 0.84% |
|  | 5% | 10.92 | 10.92 | 0.01% | 10.96 | 10.96 | 0.02% | 11.03 | 11.04 | 0.10% |
|  | 50% | 11.26 | 11.27 | 0.10% | 12.71 | 13.05 | 2.69% | 18.85 | 19.28 | 2.32% |
| UL Packet-Latency CDF (ms) | Mean | 50.46 | 50.46 | 0.00% | 68.20 | 69.09 | 1.30% | 95.85 | 95.59 | -0.27% |
|  | 5% | 13.66 | 13.66 | 0.00% | 14.29 | 14.32 | 0.17% | 16.84 | 16.93 | 0.53% |
|  | 50% | 25.35 | 25.35 | 0.00% | 36.37 | 36.19 | -0.49% | 50.05 | 49.97 | -0.16% |
| DL RU (%) | Type-1 | 5.43% | 5.46% |  | 25.22% | 25.50% |  | 40.06% | 40.55% |  |
| UL RU (%) | Type-1 | 1.66% | 1.66% |  | 6.38% | 6.38% |  | 10.65% | 10.65% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 303.68 | 303.50 | -0.06% | 194.38 | 193.11 | -0.65% | 107.01 | 108.26 | 1.17% |
|  | 5% | 174.19 | 174.19 | 0.00% | 6.78 | 6.78 | 0.00% | 1.80 | 1.79 | -0.33% |
|  | 50% | 324.85 | 324.85 | 0.00% | 217.15 | 215.85 | -0.60% | 89.75 | 95.25 | 6.13% |
| UL Average-UPT (Mbps) | Mean | 137.20 | 139.27 | 1.51% | 69.82 | 81.52 | 16.75% | 34.77 | 44.04 | 26.68% |
|  | 5% | 22.34 | 22.34 | 0.00% | 0.44 | 0.45 | 1.21% | 0.20 | 0.20 | -1.47% |
|  | 50% | 165.66 | 168.70 | 1.84% | 74.63 | 89.05 | 19.33% | 28.30 | 36.93 | 30.51% |
| DL Packet-Latency CDF (ms) | Mean | 13.37 | 13.42 | 0.35% | 39.21 | 39.18 | -0.07% | 67.73 | 67.84 | 0.16% |
|  | 5% | 10.95 | 10.95 | 0.01% | 11.01 | 11.01 | 0.02% | 11.10 | 11.12 | 0.13% |
|  | 50% | 11.60 | 11.63 | 0.24% | 15.08 | 15.29 | 1.39% | 23.53 | 23.80 | 1.16% |
| UL Packet-Latency CDF (ms) | Mean | 34.00 | 33.96 | -0.11% | 64.38 | 63.45 | -1.45% | 100.83 | 95.58 | -5.20% |
|  | 5% | 4.62 | 4.60 | -0.61% | 6.98 | 6.06 | -13.21% | 11.81 | 9.02 | -23.63% |
|  | 50% | 16.61 | 16.61 | 0.00% | 23.81 | 22.76 | -4.41% | 44.25 | 41.21 | -6.87% |
| DL RU (%) | Type-1 | 5.78% | 5.78% |  | 21.40% | 21.57% |  | 42.37% | 42.65% |  |
| UL RU (%) | Type-1 | 1.33% | 1.30% |  | 9.38% | 7.91% |  | 16.30% | 13.56% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 2 ([26]):

Indoor Office (FR1)

Small packet size, DL TRP power back off =3dB

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 43.10 | 0.01% | 42.11 | 42.15 | 0.09% | 41.40 | 41.36 | -0.09% |
|  | 5% | 41.76 | 41.77 | 0.02% | 41.22 | 41.18 | -0.09% | 40.78 | 40.74 | -0.11% |
|  | 50% | 43.08 | 43.03 | -0.12% | 41.91 | 41.93 | 0.05% | 41.37 | 41.34 | -0.07% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.25 | 0.11% | 4.23 | 4.23 | 0.16% | 3.95 | 4.15 | 5.05% |
|  | 5% | 4.19 | 4.20 | 0.26% | 4.20 | 4.20 | 0.11% | 2.54 | 3.83 | 50.87% |
|  | 50% | 4.25 | 4.25 | 0.05% | 4.23 | 4.23 | 0.07% | 4.21 | 4.22 | 0.23% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 1.16 | 1.04% | 1.24 | 1.23 | -1.38% | 1.23 | 1.27 | 3.49% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.54 | 0.00% | 0.55 | 0.55 | -0.97% |
|  | 50% | 0.80 | 0.80 | 0.00% | 0.84 | 0.84 | 0.00% | 0.88 | 0.87 | -1.02% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 2.21 | 0.14% | 4.17 | 3.91 | -6.31% | 45.40 | 13.88 | -69.44% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.60 | -0.50% | 1.63 | 1.61 | -1.10% |
|  | 50% | 1.88 | 1.88 | 0.48% | 1.91 | 1.91 | 0.00% | 1.96 | 1.95 | -0.91% |
| DL RU (%) | Type-1 | 3.55% | 3.55% |  | 23.29% | 23.36% |  | 49.12% | 49.68% |  |
| UL RU (%) | Type-1 | 1.71% | 1.50% |  | 14.68% | 11.01% |  | 39.61% | 31.35% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Small packet size, DL TRP power back off =6dB

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 43.08 | -0.03% | 42.11 | 42.19 | 0.17% | 41.40 | 41.38 | -0.04% |
|  | 5% | 41.76 | 41.76 | 0.01% | 41.22 | 41.25 | 0.07% | 40.78 | 40.68 | -0.26% |
|  | 50% | 43.08 | 43.09 | 0.04% | 41.91 | 42.05 | 0.34% | 41.37 | 41.36 | -0.02% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.25 | 0.21% | 4.23 | 4.24 | 0.30% | 3.95 | 4.22 | 6.91% |
|  | 5% | 4.19 | 4.20 | 0.34% | 4.20 | 4.21 | 0.20% | 2.54 | 4.12 | 62.50% |
|  | 50% | 4.25 | 4.25 | 0.15% | 4.23 | 4.24 | 0.17% | 4.21 | 4.23 | 0.49% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 1.16 | 1.22% | 1.24 | 1.17 | -6.19% | 1.23 | 1.30 | 5.46% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.52 | -3.33% | 0.55 | 0.52 | -6.45% |
|  | 50% | 0.80 | 0.80 | 0.00% | 0.84 | 0.82 | -2.13% | 0.88 | 0.85 | -3.06% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 2.19 | -0.73% | 4.17 | 2.42 | -41.94% | 45.40 | 4.51 | -90.07% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.59 | -0.89% | 1.63 | 1.59 | -2.20% |
|  | 50% | 1.88 | 1.88 | 0.00% | 1.91 | 1.89 | -0.93% | 1.96 | 1.91 | -2.73% |
| DL RU (%) | Type-1 | 3.55% | 3.57% |  | 23.29% | 23.51% |  | 49.12% | 49.68% |  |
| UL RU (%) | Type-1 | 1.71% | 1.38% |  | 14.68% | 8.90% |  | 39.61% | 23.54% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Small packet size, DL TRP power back off =10dB

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 42.97 | -0.28% | 42.11 | 42.25 | 0.32% | 41.40 | 41.43 | 0.08% |
|  | 5% | 41.76 | 41.67 | -0.21% | 41.22 | 41.24 | 0.04% | 40.78 | 40.75 | -0.07% |
|  | 50% | 43.08 | 42.96 | -0.26% | 41.91 | 42.08 | 0.41% | 41.37 | 41.43 | 0.14% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.26 | 0.29% | 4.23 | 4.25 | 0.42% | 3.95 | 4.24 | 7.44% |
|  | 5% | 4.19 | 4.21 | 0.45% | 4.20 | 4.21 | 0.36% | 2.54 | 4.20 | 65.43% |
|  | 50% | 4.25 | 4.26 | 0.22% | 4.23 | 4.24 | 0.29% | 4.21 | 4.24 | 0.74% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 1.16 | 0.93% | 1.24 | 1.21 | -2.43% | 1.23 | 1.18 | -3.91% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.52 | -3.33% | 0.55 | 0.51 | -7.26% |
|  | 50% | 0.80 | 0.80 | 0.00% | 0.84 | 0.80 | -4.26% | 0.88 | 0.80 | -8.16% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 2.21 | 0.18% | 4.17 | 2.22 | -46.78% | 45.40 | 2.36 | -94.79% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.57 | -2.00% | 1.63 | 1.57 | -3.30% |
|  | 50% | 1.88 | 1.88 | 0.00% | 1.91 | 1.88 | -1.87% | 1.96 | 1.86 | -5.45% |
| DL RU (%) | Type-1 | 3.55% | 3.60% |  | 23.29% | 23.85% |  | 49.12% | 49.72% |  |
| UL RU (%) | Type-1 | 1.71% | 1.29% |  | 14.68% | 7.62% |  | 39.61% | 17.41% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size, power back off =3dB

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 939.80 | -2.65% | 579.11 | 557.20 | -3.78% | 443.87 | 421.75 | -4.98% |
|  | 5% | 680.10 | 636.69 | -6.38% | 363.91 | 347.68 | -4.46% | 258.74 | 240.20 | -7.17% |
|  | 50% | 951.37 | 930.05 | -2.24% | 557.41 | 533.19 | -4.35% | 411.05 | 391.85 | -4.67% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 299.16 | 2.76% | 132.35 | 153.61 | 16.06% | 92.82 | 115.20 | 24.11% |
|  | 5% | 231.38 | 238.77 | 3.19% | 67.53 | 80.61 | 19.37% | 26.95 | 40.61 | 50.71% |
|  | 50% | 292.47 | 305.19 | 4.35% | 129.13 | 151.10 | 17.02% | 86.65 | 109.18 | 26.00% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 4.85 | 3.51% | 8.24 | 8.57 | 4.03% | 11.38 | 12.00 | 5.39% |
|  | 5% | 2.77 | 2.82 | 1.94% | 4.08 | 4.14 | 1.33% | 4.83 | 4.98 | 3.22% |
|  | 50% | 3.95 | 4.11 | 4.07% | 7.08 | 7.36 | 3.91% | 9.57 | 10.00 | 4.48% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 3.63 | -5.80% | 12.60 | 10.24 | -18.72% | 28.25 | 19.60 | -30.62% |
|  | 5% | 2.64 | 2.63 | -0.68% | 3.91 | 3.57 | -8.56% | 4.97 | 4.30 | -13.51% |
|  | 50% | 3.07 | 3.02 | -1.74% | 7.48 | 6.45 | -13.74% | 10.80 | 8.88 | -17.85% |
| DL RU (%) | Type-1 | 3.58% | 3.70% |  | 25.29% | 26.22% |  | 46.52% | 48.27% |  |
| UL RU (%) | Type-1 | 1.74% | 1.62% |  | 22.11% | 18.82% |  | 46.93% | 39.79% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size, power back off =6dB

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 908.74 | -5.87% | 579.11 | 533.42 | -7.89% | 443.87 | 395.07 | -10.99% |
|  | 5% | 680.10 | 602.39 | -11.43% | 363.91 | 329.69 | -9.40% | 258.74 | 223.12 | -13.77% |
|  | 50% | 951.37 | 902.95 | -5.09% | 557.41 | 513.33 | -7.91% | 411.05 | 361.06 | -12.16% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 309.02 | 6.15% | 132.35 | 176.62 | 33.45% | 92.82 | 138.02 | 48.70% |
|  | 5% | 231.38 | 248.99 | 7.61% | 67.53 | 97.61 | 44.55% | 26.95 | 54.29 | 101.46% |
|  | 50% | 292.47 | 316.71 | 8.29% | 129.13 | 171.34 | 32.69% | 86.65 | 132.92 | 53.40% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 5.09 | 8.65% | 8.24 | 8.93 | 8.34% | 11.38 | 12.87 | 13.07% |
|  | 5% | 2.77 | 2.91 | 5.16% | 4.08 | 4.21 | 3.19% | 4.83 | 5.20 | 7.83% |
|  | 50% | 3.95 | 4.34 | 9.95% | 7.08 | 7.73 | 9.21% | 9.57 | 10.61 | 10.82% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 3.44 | -10.68% | 12.60 | 8.81 | -30.13% | 28.25 | 13.30 | -52.94% |
|  | 5% | 2.64 | 2.63 | -0.68% | 3.91 | 3.23 | -17.25% | 4.97 | 3.77 | -24.17% |
|  | 50% | 3.07 | 2.98 | -2.91% | 7.48 | 5.61 | -25.04% | 10.80 | 7.38 | -31.74% |
| DL RU (%) | Type-1 | 3.58% | 3.84% |  | 25.29% | 27.25% |  | 46.52% | 50.51% |  |
| UL RU (%) | Type-1 | 1.74% | 1.50% |  | 22.11% | 15.78% |  | 46.93% | 33.77% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size, power back off =10dB

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 859.35 | -10.99% | 579.11 | 496.32 | -14.30% | 443.87 | 354.16 | -20.21% |
|  | 5% | 680.10 | 552.10 | -18.82% | 363.91 | 303.35 | -16.64% | 258.74 | 186.57 | -27.89% |
|  | 50% | 951.37 | 844.42 | -11.24% | 557.41 | 471.25 | -15.46% | 411.05 | 315.10 | -23.34% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 319.69 | 9.81% | 132.35 | 206.79 | 56.24% | 92.82 | 169.24 | 82.33% |
|  | 5% | 231.38 | 269.38 | 16.42% | 67.53 | 116.01 | 71.80% | 26.95 | 74.69 | 177.17% |
|  | 50% | 292.47 | 327.35 | 11.93% | 129.13 | 203.10 | 57.29% | 86.65 | 167.34 | 93.13% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 5.44 | 16.25% | 8.24 | 9.63 | 16.90% | 11.38 | 14.57 | 28.02% |
|  | 5% | 2.77 | 3.06 | 10.52% | 4.08 | 4.39 | 7.56% | 4.83 | 5.55 | 14.99% |
|  | 50% | 3.95 | 4.76 | 20.59% | 7.08 | 8.30 | 17.28% | 9.57 | 11.71 | 22.39% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 3.31 | -14.14% | 12.60 | 7.38 | -41.42% | 28.25 | 9.49 | -66.41% |
|  | 5% | 2.64 | 2.61 | -1.35% | 3.91 | 2.94 | -24.72% | 4.97 | 3.32 | -33.24% |
|  | 50% | 3.07 | 2.93 | -4.65% | 7.48 | 4.88 | -34.83% | 10.80 | 5.96 | -44.79% |
| DL RU (%) | Type-1 | 3.58% | 4.09% |  | 25.29% | 29.09% |  | 46.52% | 54.29% |  |
| UL RU (%) | Type-1 | 1.74% | 1.38% |  | 22.11% | 12.73% |  | 46.93% | 26.91% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

2-layer Scenario B (FR1)

power back off=3dB, small packet size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 38.85 | 38.89 | 0.10% | 36.95 | 37.38 | 1.16% | 33.75 | 34.90 | 3.39% |
|  | 5% | 37.00 | 37.06 | 0.15% | 34.05 | 35.40 | 3.97% | 0.17 | 10.29 | 6096.27% |
|  | 50% | 38.81 | 38.83 | 0.04% | 37.90 | 37.94 | 0.12% | 37.32 | 37.41 | 0.25% |
| UL Average-UPT (Mbps) | Mean | 1.65 | 1.65 | 0.07% | 1.64 | 1.68 | 2.26% | 1.55 | 1.64 | 5.69% |
|  | 5% | 1.56 | 1.55 | -0.19% | 1.38 | 1.61 | 16.54% | 0.05 | 0.78 | 1330.07% |
|  | 50% | 1.65 | 1.65 | -0.06% | 1.69 | 1.69 | 0.11% | 1.72 | 1.73 | 0.31% |
| DL Packet-Latency CDF (ms) | Mean | 1.10 | 1.11 | 0.30% | 4.08 | 1.29 | -68.40% | 15.42 | 10.59 | -31.33% |
|  | 5% | 0.55 | 0.55 | 0.00% | 0.54 | 0.54 | 0.00% | 0.54 | 0.54 | 0.00% |
|  | 50% | 0.86 | 0.86 | 0.00% | 0.86 | 0.86 | 0.00% | 0.88 | 0.88 | -1.01% |
| UL Packet-Latency CDF (ms) | Mean | 5.21 | 5.21 | 0.13% | 8.02 | 5.30 | -33.90% | 20.17 | 11.19 | -44.53% |
|  | 5% | 3.77 | 3.75 | -0.43% | 3.62 | 3.62 | 0.10% | 3.36 | 3.37 | 0.32% |
|  | 50% | 5.07 | 5.07 | 0.00% | 4.98 | 4.98 | 0.00% | 5.04 | 4.97 | -1.24% |
| DL RU (%) | Type-1 | 5.65% | 5.21% |  | 31.17% | 28.99% |  | 54.21% | 52.03% |  |
| UL RU (%) | Type-1 | 5.88% | 5.55% |  | 30.88% | 29.07% |  | 47.00% | 45.21% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 27.88 | 27.94 | 0.22% | 26.16 | 26.70 | 2.05% | 24.94 | 24.64 | -1.19% |
|  | 5% | 26.51 | 26.47 | -0.13% | 22.81 | 23.63 | 3.58% | 17.53 | 14.67 | -16.29% |
|  | 50% | 27.87 | 27.99 | 0.40% | 26.64 | 27.03 | 1.47% | 26.02 | 26.24 | 0.84% |
| UL Average-UPT (Mbps) | Mean | 3.42 | 3.41 | -0.21% | 3.03 | 3.17 | 4.62% | 2.74 | 2.96 | 7.98% |
|  | 5% | 3.35 | 3.13 | -6.42% | 1.60 | 1.79 | 11.75% | 0.01 | 0.29 | 2991.93% |
|  | 50% | 3.45 | 3.44 | -0.34% | 3.35 | 3.43 | 2.42% | 3.12 | 3.39 | 8.80% |
| DL Packet-Latency CDF (ms) | Mean | 1.67 | 1.68 | 0.24% | 1.74 | 1.74 | 0.18% | 2.35 | 9.41 | 299.96% |
|  | 5% | 0.61 | 0.61 | 0.00% | 0.57 | 0.59 | 2.97% | 0.59 | 0.61 | 3.03% |
|  | 50% | 1.49 | 1.48 | -0.60% | 1.52 | 1.55 | 2.35% | 1.58 | 1.78 | 12.43% |
| UL Packet-Latency CDF (ms) | Mean | 4.54 | 2.64 | -41.81% | 3.29 | 3.78 | 14.68% | 6.94 | 5.31 | -23.37% |
|  | 5% | 1.68 | 1.68 | 0.00% | 1.64 | 1.64 | 0.00% | 1.64 | 1.64 | 0.00% |
|  | 50% | 2.57 | 2.58 | 0.35% | 2.63 | 2.57 | -2.04% | 2.75 | 2.59 | -5.84% |
| DL RU (%) | Type-1 | 17.76% | 15.44% |  | 32.18% | 36.58% |  | 37.27% | 53.41% |  |
| UL RU (%) | Type-1 | 2.56% | 2.39% |  | 21.94% | 19.85% |  | 32.44% | 32.27% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Power back off=6dB, small packet size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 38.85 | 38.90 | 0.14% | 36.95 | 37.53 | 1.58% | 33.75 | 35.68 | 5.71% |
|  | 5% | 37.00 | 37.12 | 0.32% | 34.05 | 36.65 | 7.63% | 0.17 | 24.64 | 14728.28% |
|  | 50% | 38.81 | 38.88 | 0.16% | 37.90 | 38.00 | 0.26% | 37.32 | 37.47 | 0.41% |
| UL Average-UPT (Mbps) | Mean | 1.65 | 1.65 | 0.13% | 1.64 | 1.70 | 3.59% | 1.55 | 1.72 | 10.51% |
|  | 5% | 1.56 | 1.56 | -0.05% | 1.38 | 1.64 | 18.67% | 0.05 | 1.63 | 2875.02% |
|  | 50% | 1.65 | 1.65 | -0.02% | 1.69 | 1.70 | 0.30% | 1.72 | 1.73 | 0.47% |
| DL Packet-Latency CDF (ms) | Mean | 1.10 | 1.11 | 0.45% | 4.08 | 1.34 | -67.08% | 15.42 | 4.74 | -69.28% |
|  | 5% | 0.55 | 0.55 | 0.00% | 0.54 | 0.54 | 0.00% | 0.54 | 0.54 | 0.00% |
|  | 50% | 0.86 | 0.86 | 0.00% | 0.86 | 0.88 | 2.08% | 0.88 | 0.88 | -1.01% |
| UL Packet-Latency CDF (ms) | Mean | 5.21 | 5.23 | 0.47% | 8.02 | 5.09 | -36.58% | 20.17 | 5.39 | -73.30% |
|  | 5% | 3.77 | 3.76 | -0.24% | 3.62 | 3.60 | -0.47% | 3.36 | 3.32 | -1.06% |
|  | 50% | 5.07 | 5.11 | 0.70% | 4.98 | 5.04 | 1.08% | 5.04 | 4.93 | -2.13% |
| DL RU (%) | Type-1 | 5.65% | 4.93% |  | 31.17% | 27.43% |  | 54.21% | 50.58% |  |
| UL RU (%) | Type-1 | 5.88% | 5.26% |  | 30.88% | 27.45% |  | 47.00% | 43.82% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 27.88 | 27.96 | 0.28% | 26.16 | 27.24 | 4.11% | 24.94 | 25.42 | 1.94% |
|  | 5% | 26.51 | 26.54 | 0.13% | 22.81 | 25.03 | 9.69% | 17.53 | 17.38 | -0.85% |
|  | 50% | 27.87 | 27.93 | 0.20% | 26.64 | 27.46 | 3.10% | 26.02 | 26.91 | 3.43% |
| UL Average-UPT (Mbps) | Mean | 3.42 | 3.40 | -0.47% | 3.03 | 3.27 | 8.05% | 2.74 | 3.10 | 13.11% |
|  | 5% | 3.35 | 3.12 | -6.69% | 1.60 | 2.47 | 53.81% | 0.01 | 1.37 | 14512.05% |
|  | 50% | 3.45 | 3.43 | -0.58% | 3.35 | 3.45 | 2.91% | 3.12 | 3.44 | 10.33% |
| DL Packet-Latency CDF (ms) | Mean | 1.67 | 1.69 | 0.99% | 1.74 | 1.72 | -0.65% | 2.35 | 6.40 | 172.13% |
|  | 5% | 0.61 | 0.61 | 0.00% | 0.57 | 0.57 | 0.00% | 0.59 | 0.59 | 0.00% |
|  | 50% | 1.49 | 1.52 | 1.80% | 1.52 | 1.55 | 2.35% | 1.58 | 1.69 | 6.78% |
| UL Packet-Latency CDF (ms) | Mean | 4.54 | 2.59 | -43.00% | 3.29 | 2.67 | -19.08% | 6.94 | 3.84 | -44.56% |
|  | 5% | 1.68 | 1.68 | 0.00% | 1.64 | 1.63 | -1.09% | 1.64 | 1.63 | -1.09% |
|  | 50% | 2.57 | 2.59 | 0.69% | 2.63 | 2.57 | -2.04% | 2.75 | 2.52 | -8.44% |
| DL RU (%) | Type-1 | 17.76% | 14.07% |  | 32.18% | 34.95% |  | 37.27% | 53.99% |  |
| UL RU (%) | Type-1 | 2.56% | 2.36% |  | 21.94% | 17.94% |  | 32.44% | 30.16% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

power back off=10dB, small packet size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 38.85 | 38.94 | 0.22% | 36.95 | 38.03 | 2.92% | 33.75 | 37.05 | 9.76% |
|  | 5% | 37.00 | 37.04 | 0.12% | 34.05 | 37.18 | 9.20% | 0.17 | 34.54 | 20686.98% |
|  | 50% | 38.81 | 38.89 | 0.19% | 37.90 | 38.05 | 0.40% | 37.32 | 37.56 | 0.65% |
| UL Average-UPT (Mbps) | Mean | 1.65 | 1.65 | 0.18% | 1.64 | 1.71 | 3.77% | 1.55 | 1.74 | 12.23% |
|  | 5% | 1.56 | 1.56 | 0.27% | 1.38 | 1.65 | 18.98% | 0.05 | 1.68 | 2964.35% |
|  | 50% | 1.65 | 1.65 | 0.01% | 1.69 | 1.70 | 0.57% | 1.72 | 1.74 | 0.73% |
| DL Packet-Latency CDF (ms) | Mean | 1.10 | 1.12 | 1.94% | 4.08 | 1.21 | -70.25% | 15.42 | 1.25 | -91.90% |
|  | 5% | 0.55 | 0.54 | -3.23% | 0.54 | 0.52 | -3.33% | 0.54 | 0.54 | 0.00% |
|  | 50% | 0.86 | 0.86 | 0.00% | 0.86 | 0.86 | 0.00% | 0.88 | 0.88 | -1.01% |
| UL Packet-Latency CDF (ms) | Mean | 5.21 | 5.23 | 0.36% | 8.02 | 5.07 | -36.76% | 20.17 | 5.05 | -74.98% |
|  | 5% | 3.77 | 3.73 | -0.90% | 3.62 | 3.58 | -1.13% | 3.36 | 3.52 | 4.79% |
|  | 50% | 5.07 | 5.05 | -0.35% | 4.98 | 4.98 | 0.00% | 5.04 | 4.98 | -1.06% |
| DL RU (%) | Type-1 | 5.65% | 4.48% |  | 31.17% | 25.09% |  | 54.21% | 48.39% |  |
| UL RU (%) | Type-1 | 5.88% | 4.90% |  | 30.88% | 25.62% |  | 47.00% | 41.49% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 27.88 | 27.99 | 0.42% | 26.16 | 27.58 | 5.39% | 24.94 | 26.34 | 5.60% |
|  | 5% | 26.51 | 26.50 | -0.04% | 22.81 | 26.07 | 14.26% | 17.53 | 21.38 | 21.94% |
|  | 50% | 27.87 | 28.02 | 0.54% | 26.64 | 27.65 | 3.79% | 26.02 | 27.16 | 4.39% |
| UL Average-UPT (Mbps) | Mean | 3.42 | 3.40 | -0.49% | 3.03 | 3.37 | 11.39% | 2.74 | 3.30 | 20.42% |
|  | 5% | 3.35 | 3.16 | -5.61% | 1.60 | 3.33 | 107.64% | 0.01 | 2.92 | 31118.67% |
|  | 50% | 3.45 | 3.43 | -0.66% | 3.35 | 3.46 | 3.36% | 3.12 | 3.46 | 11.01% |
| DL Packet-Latency CDF (ms) | Mean | 1.67 | 1.68 | 0.53% | 1.74 | 1.69 | -2.52% | 2.35 | 4.15 | 76.55% |
|  | 5% | 0.61 | 0.59 | -2.94% | 0.57 | 0.55 | -3.13% | 0.59 | 0.61 | 2.88% |
|  | 50% | 1.49 | 1.46 | -1.80% | 1.52 | 1.48 | -2.35% | 1.58 | 1.68 | 6.21% |
| UL Packet-Latency CDF (ms) | Mean | 4.54 | 2.59 | -42.81% | 3.29 | 2.75 | -16.52% | 6.94 | 2.64 | -61.88% |
|  | 5% | 1.68 | 1.68 | 0.00% | 1.64 | 1.62 | -1.68% | 1.64 | 1.63 | -1.09% |
|  | 50% | 2.57 | 2.57 | 0.00% | 2.63 | 2.52 | -4.08% | 2.75 | 2.54 | -7.79% |
| DL RU (%) | Type-1 | 17.76% | 13.09% |  | 32.18% | 33.21% |  | 37.27% | 53.64% |  |
| UL RU (%) | Type-1 | 2.56% | 2.31% |  | 21.94% | 14.78% |  | 32.44% | 23.96% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

power back off=3dB, large packet size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 739.51 | 770.64 | 4.21% | 431.78 | 458.82 | 6.26% | 292.80 | 317.88 | 8.57% |
|  | 5% | 270.01 | 288.56 | 6.87% | 61.35 | 120.65 | 96.65% | 0.00 | 0.02 | 1166.02% |
|  | 50% | 726.21 | 748.66 | 3.09% | 435.37 | 460.85 | 5.85% | 281.51 | 304.34 | 8.11% |
| UL Average-UPT (Mbps) | Mean | 86.96 | 88.78 | 2.09% | 54.24 | 57.32 | 5.68% | 36.28 | 38.98 | 7.43% |
|  | 5% | 54.11 | 57.60 | 6.45% | 7.47 | 27.73 | 271.25% | 0.04 | 0.16 | 290.00% |
|  | 50% | 92.38 | 92.97 | 0.64% | 57.33 | 58.95 | 2.82% | 39.29 | 41.00 | 4.34% |
| DL Packet-Latency CDF (ms) | Mean | 6.88 | 6.43 | -6.53% | 70.09 | 14.28 | -79.62% | 133.18 | 107.19 | -19.51% |
|  | 5% | 2.96 | 2.88 | -3.01% | 4.30 | 4.23 | -1.66% | 5.16 | 5.09 | -1.38% |
|  | 50% | 5.53 | 5.33 | -3.55% | 9.35 | 8.79 | -6.02% | 13.63 | 12.48 | -8.39% |
| UL Packet-Latency CDF (ms) | Mean | 12.03 | 11.65 | -3.12% | 70.09 | 47.15 | -32.73% | 451.49 | 285.18 | -36.84% |
|  | 5% | 8.79 | 8.77 | -0.20% | 10.16 | 10.05 | -1.05% | 12.04 | 12.11 | 0.59% |
|  | 50% | 10.61 | 10.49 | -1.09% | 17.84 | 17.11 | -4.10% | 26.57 | 25.56 | -3.80% |
| DL RU (%) | Type-1 | 7.08% | 6.37% |  | 30.97% | 28.95% |  | 52.90% | 50.61% |  |
| UL RU (%) | Type-1 | 5.13% | 4.72% |  | 31.56% | 29.70% |  | 54.08% | 52.57% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 302.73 | 325.58 | 7.55% | 123.60 | 163.65 | 32.39% | 34.01 | 54.40 | 59.96% |
|  | 5% | 178.25 | 200.68 | 12.58% | 48.70 | 71.40 | 46.60% | 0.20 | 0.67 | 238.95% |
|  | 50% | 291.30 | 309.70 | 6.32% | 115.06 | 153.39 | 33.31% | 25.68 | 40.08 | 56.05% |
| UL Average-UPT (Mbps) | Mean | 204.38 | 208.77 | 2.15% | 113.87 | 127.34 | 11.83% | 76.31 | 89.18 | 16.86% |
|  | 5% | 135.49 | 140.22 | 3.49% | 24.14 | 36.57 | 51.49% | 0.09 | 5.44 | 5635.03% |
|  | 50% | 216.87 | 221.26 | 2.03% | 117.44 | 132.85 | 13.13% | 79.17 | 91.86 | 16.02% |
| DL Packet-Latency CDF (ms) | Mean | 15.70 | 13.89 | -11.54% | 54.93 | 36.67 | -33.24% | 1277.40 | 896.70 | -29.80% |
|  | 5% | 6.58 | 6.21 | -5.58% | 14.93 | 11.59 | -22.37% | 30.52 | 24.05 | -21.20% |
|  | 50% | 13.79 | 12.38 | -10.23% | 36.99 | 27.79 | -24.86% | 198.50 | 137.79 | -30.59% |
| UL Packet-Latency CDF (ms) | Mean | 5.20 | 4.94 | -5.07% | 10.72 | 9.98 | -6.91% | 38.41 | 17.04 | -55.62% |
|  | 5% | 3.66 | 3.64 | -0.49% | 4.96 | 4.39 | -11.35% | 6.18 | 5.59 | -9.54% |
|  | 50% | 4.71 | 4.51 | -4.36% | 8.45 | 7.29 | -13.74% | 12.51 | 10.33 | -17.44% |
| DL RU (%) | Type-1 | 15.72% | 15.18% |  | 36.95% | 34.64% |  | 72.97% | 72.90% |  |
| UL RU (%) | Type-1 | 2.23% | 2.10% |  | 18.30% | 17.00% |  | 35.64% | 34.08% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Power back off=6dB, large packet size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 739.51 | 790.52 | 6.90% | 431.78 | 480.23 | 11.22% | 292.80 | 340.75 | 16.38% |
|  | 5% | 270.01 | 306.80 | 13.62% | 61.35 | 196.96 | 221.02% | 0.00 | 10.98 | 586923.23% |
|  | 50% | 726.21 | 765.17 | 5.37% | 435.37 | 477.51 | 9.68% | 281.51 | 322.90 | 14.70% |
| UL Average-UPT (Mbps) | Mean | 86.96 | 90.27 | 3.81% | 54.24 | 60.33 | 11.22% | 36.28 | 41.42 | 14.15% |
|  | 5% | 54.11 | 59.97 | 10.85% | 7.47 | 35.91 | 380.73% | 0.04 | 9.34 | 23344.47% |
|  | 50% | 92.38 | 93.45 | 1.16% | 57.33 | 60.26 | 5.11% | 39.29 | 41.94 | 6.74% |
| DL Packet-Latency CDF (ms) | Mean | 6.88 | 6.18 | -10.25% | 70.09 | 11.17 | -84.07% | 133.18 | 69.23 | -48.02% |
|  | 5% | 2.96 | 2.82 | -4.82% | 4.30 | 4.16 | -3.32% | 5.16 | 5.02 | -2.77% |
|  | 50% | 5.53 | 5.21 | -5.65% | 9.35 | 8.41 | -10.03% | 13.63 | 11.95 | -12.32% |
| UL Packet-Latency CDF (ms) | Mean | 12.03 | 11.37 | -5.46% | 70.09 | 19.25 | -72.54% | 451.49 | 111.20 | -75.37% |
|  | 5% | 8.79 | 8.77 | -0.20% | 10.16 | 9.91 | -2.46% | 12.04 | 12.02 | -0.15% |
|  | 50% | 10.61 | 10.41 | -1.85% | 17.84 | 16.47 | -7.66% | 26.57 | 25.03 | -5.80% |
| DL RU (%) | Type-1 | 7.08% | 5.95% |  | 30.97% | 27.37% |  | 52.90% | 48.82% |  |
| UL RU (%) | Type-1 | 5.13% | 4.38% |  | 31.56% | 28.18% |  | 54.08% | 51.49% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 302.73 | 343.45 | 13.45% | 123.60 | 208.11 | 68.37% | 34.01 | 109.43 | 221.75% |
|  | 5% | 178.25 | 207.20 | 16.24% | 48.70 | 110.32 | 126.51% | 0.20 | 15.28 | 7672.84% |
|  | 50% | 291.30 | 327.81 | 12.53% | 115.06 | 194.77 | 69.27% | 25.68 | 104.70 | 307.66% |
| UL Average-UPT (Mbps) | Mean | 204.38 | 213.40 | 4.42% | 113.87 | 140.55 | 23.43% | 76.31 | 103.65 | 35.82% |
|  | 5% | 135.49 | 154.26 | 13.85% | 24.14 | 46.18 | 91.28% | 0.09 | 15.49 | 16230.60% |
|  | 50% | 216.87 | 225.67 | 4.06% | 117.44 | 147.02 | 25.19% | 79.17 | 107.12 | 35.31% |
| DL Packet-Latency CDF (ms) | Mean | 15.70 | 12.86 | -18.11% | 54.93 | 24.92 | -54.63% | 1277.40 | 244.20 | -80.88% |
|  | 5% | 6.58 | 6.02 | -8.56% | 14.93 | 9.59 | -35.77% | 30.52 | 16.45 | -46.12% |
|  | 50% | 13.79 | 11.42 | -17.16% | 36.99 | 21.35 | -42.29% | 198.50 | 47.38 | -76.13% |
| UL Packet-Latency CDF (ms) | Mean | 5.20 | 4.77 | -8.27% | 10.72 | 7.77 | -27.48% | 38.41 | 13.31 | -65.34% |
|  | 5% | 3.66 | 3.64 | -0.49% | 4.96 | 4.14 | -16.40% | 6.18 | 4.86 | -21.39% |
|  | 50% | 4.71 | 4.38 | -7.01% | 8.45 | 6.39 | -24.31% | 12.51 | 8.53 | -31.83% |
| DL RU (%) | Type-1 | 15.72% | 14.90% |  | 36.95% | 30.31% |  | 72.97% | 58.56% |  |
| UL RU (%) | Type-1 | 2.23% | 2.02% |  | 18.30% | 15.48% |  | 35.64% | 31.71% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

power back off=10dB, large packet size

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 739.51 | 825.45 | 11.62% | 431.78 | 508.44 | 17.75% | 292.80 | 368.38 | 25.81% |
|  | 5% | 270.01 | 505.47 | 87.21% | 61.35 | 239.11 | 289.72% | 0.00 | 125.58 | 6712198.82% |
|  | 50% | 726.21 | 798.88 | 10.01% | 435.37 | 506.24 | 16.28% | 281.51 | 351.20 | 24.76% |
| UL Average-UPT (Mbps) | Mean | 86.96 | 92.36 | 6.22% | 54.24 | 63.12 | 16.38% | 36.28 | 45.17 | 24.51% |
|  | 5% | 54.11 | 75.38 | 39.32% | 7.47 | 41.16 | 451.08% | 0.04 | 20.94 | 52460.43% |
|  | 50% | 92.38 | 94.19 | 1.96% | 57.33 | 62.75 | 9.46% | 39.29 | 44.67 | 13.69% |
| DL Packet-Latency CDF (ms) | Mean | 6.88 | 5.74 | -16.61% | 70.09 | 9.64 | -86.24% | 133.18 | 27.44 | -79.40% |
|  | 5% | 2.96 | 2.75 | -7.23% | 4.30 | 4.07 | -5.39% | 5.16 | 4.93 | -4.50% |
|  | 50% | 5.53 | 5.04 | -8.89% | 9.35 | 7.93 | -15.19% | 13.63 | 11.20 | -17.82% |
| UL Packet-Latency CDF (ms) | Mean | 12.03 | 11.04 | -8.26% | 70.09 | 17.93 | -74.42% | 451.49 | 29.95 | -93.37% |
|  | 5% | 8.79 | 8.75 | -0.41% | 10.16 | 9.71 | -4.39% | 12.04 | 11.79 | -2.08% |
|  | 50% | 10.61 | 10.29 | -3.03% | 17.84 | 15.59 | -12.61% | 26.57 | 23.14 | -12.90% |
| DL RU (%) | Type-1 | 7.08% | 5.26% |  | 30.97% | 25.14% |  | 52.90% | 46.52% |  |
| UL RU (%) | Type-1 | 5.13% | 3.91% |  | 31.56% | 26.09% |  | 54.08% | 48.76% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 302.73 | 354.30 | 17.04% | 123.60 | 238.73 | 93.14% | 34.01 | 159.46 | 368.87% |
|  | 5% | 178.25 | 203.27 | 14.03% | 48.70 | 120.15 | 146.69% | 0.20 | 54.72 | 27726.09% |
|  | 50% | 291.30 | 334.27 | 14.75% | 115.06 | 221.36 | 92.38% | 25.68 | 145.22 | 465.45% |
| UL Average-UPT (Mbps) | Mean | 204.38 | 221.35 | 8.31% | 113.87 | 162.39 | 42.61% | 76.31 | 130.00 | 70.35% |
|  | 5% | 135.49 | 176.58 | 30.33% | 24.14 | 69.57 | 188.19% | 0.09 | 36.35 | 38236.84% |
|  | 50% | 216.87 | 229.02 | 5.60% | 117.44 | 168.93 | 43.85% | 79.17 | 131.60 | 66.23% |
| DL Packet-Latency CDF (ms) | Mean | 15.70 | 12.22 | -22.19% | 54.93 | 21.00 | -61.78% | 1277.40 | 57.69 | -95.48% |
|  | 5% | 6.58 | 5.88 | -10.73% | 14.93 | 8.48 | -43.18% | 30.52 | 12.02 | -60.62% |
|  | 50% | 13.79 | 10.72 | -22.27% | 36.99 | 18.19 | -50.84% | 198.50 | 30.49 | -84.64% |
| UL Packet-Latency CDF (ms) | Mean | 5.20 | 4.58 | -11.98% | 10.72 | 7.40 | -30.95% | 38.41 | 8.67 | -77.42% |
|  | 5% | 3.66 | 3.63 | -0.98% | 4.96 | 3.80 | -23.25% | 6.18 | 4.21 | -31.79% |
|  | 50% | 4.71 | 4.22 | -10.42% | 8.45 | 5.52 | -34.67% | 12.51 | 6.84 | -45.32% |
| DL RU (%) | Type-1 | 15.72% | 15.39% |  | 36.95% | 31.48% |  | 72.97% | 53.60% |  |
| UL RU (%) | Type-1 | 2.23% | 1.92% |  | 18.30% | 13.96% |  | 35.64% | 26.42% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

B.4.5B	Inter-gNB CLI scheme 5B: Power Control scheme based on UE Tx Power Adjustment

- Source 1 ([24]):

Large Packet Size

P0 offset = 5 dB

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 318.69 | 318.62 | -0.02% | 234.81 | 234.33 | -0.20% | 177.50 | 177.20 | -0.17% |
|  | 5% | 180.91 | 180.51 | -0.22% | 13.46 | 12.57 | -6.59% | 2.71 | 2.65 | -1.97% |
|  | 50% | 351.04 | 351.04 | 0.00% | 268.30 | 267.90 | -0.15% | 198.63 | 197.90 | -0.37% |
| UL Average-UPT (Mbps) | Mean | 40.55 | 40.57 | 0.06% | 27.27 | 27.26 | -0.04% | 18.45 | 18.50 | 0.26% |
|  | 5% | 6.09 | 6.09 | 0.00% | 1.11 | 1.02 | -7.43% | 0.22 | 0.23 | 3.82% |
|  | 50% | 41.20 | 41.20 | 0.00% | 24.75 | 24.52 | -0.94% | 16.19 | 16.36 | 1.07% |
| DL Packet-Latency CDF (ms) | Mean | 13.55 | 13.56 | 0.04% | 43.37 | 43.80 | 1.00% | 60.12 | 60.76 | 1.06% |
|  | 5% | 10.93 | 10.93 | 0.00% | 10.98 | 10.98 | 0.00% | 11.07 | 11.07 | 0.01% |
|  | 50% | 11.32 | 11.32 | 0.00% | 14.88 | 14.84 | -0.27% | 21.30 | 21.28 | -0.10% |
| UL Packet-Latency CDF (ms) | Mean | 49.04 | 48.97 | -0.14% | 86.94 | 87.24 | 0.34% | 103.60 | 104.84 | 1.20% |
|  | 5% | 13.75 | 13.75 | 0.00% | 14.68 | 14.68 | 0.03% | 19.16 | 19.51 | 1.84% |
|  | 50% | 25.22 | 25.25 | 0.13% | 38.37 | 38.13 | -0.63% | 54.18 | 54.10 | -0.15% |
| DL RU (%) | Type-1 | 7.65% | 7.65% |  | 24.65% | 24.68% |  | 41.09% | 41.13% |  |
| UL RU (%) | Type-1 | 1.74% | 1.74% |  | 6.44% | 6.44% |  | 11.86% | 11.85% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 300.92 | 300.84 | -0.03% | 225.38 | 220.63 | -2.11% | 126.81 | 123.05 | -2.97% |
|  | 5% | 197.82 | 197.82 | 0.00% | 99.13 | 97.23 | -1.92% | 12.78 | 12.28 | -3.91% |
|  | 50% | 319.05 | 319.02 | -0.01% | 234.00 | 225.70 | -3.55% | 122.45 | 119.15 | -2.69% |
| UL Average-UPT (Mbps) | Mean | 134.95 | 180.78 | 33.96% | 73.22 | 114.28 | 56.07% | 43.52 | 61.03 | 40.24% |
|  | 5% | 69.88 | 93.94 | 34.43% | 41.75 | 58.69 | 40.59% | 6.39 | 7.36 | 15.13% |
|  | 50% | 129.65 | 187.93 | 44.95% | 71.70 | 110.50 | 54.11% | 44.18 | 60.05 | 35.94% |
| DL Packet-Latency CDF (ms) | Mean | 13.90 | 13.91 | 0.08% | 37.14 | 37.88 | 1.99% | 62.98 | 64.60 | 2.57% |
|  | 5% | 10.96 | 10.96 | 0.00% | 11.02 | 11.02 | -0.01% | 11.13 | 11.13 | 0.00% |
|  | 50% | 11.77 | 11.77 | 0.04% | 16.17 | 16.28 | 0.70% | 25.49 | 25.73 | 0.94% |
| UL Packet-Latency CDF (ms) | Mean | 35.26 | 34.38 | -2.49% | 62.28 | 60.79 | -2.40% | 85.40 | 82.99 | -2.81% |
|  | 5% | 5.33 | 4.39 | -17.56% | 10.13 | 5.95 | -41.31% | 12.88 | 9.98 | -22.55% |
|  | 50% | 16.07 | 15.95 | -0.75% | 24.94 | 23.93 | -4.03% | 43.84 | 41.25 | -5.90% |
| DL RU (%) | Type-1 | 8.95% | 8.96% |  | 24.29% | 25.03% |  | 47.02% | 48.31% |  |
| UL RU (%) | Type-1 | 2.02% | 1.47% |  | 9.59% | 6.60% |  | 15.28% | 11.97% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size

P0 offset = 10 dB

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 318.69 | 318.59 | -0.03% | 234.81 | 234.13 | -0.29% | 177.50 | 177.08 | -0.23% |
|  | 5% | 180.91 | 180.51 | -0.22% | 13.46 | 12.67 | -5.85% | 2.71 | 2.58 | -4.68% |
|  | 50% | 351.04 | 351.04 | 0.00% | 268.30 | 267.90 | -0.15% | 198.63 | 197.70 | -0.47% |
| UL Average-UPT (Mbps) | Mean | 40.55 | 40.57 | 0.06% | 27.27 | 27.22 | -0.18% | 18.45 | 18.42 | -0.18% |
|  | 5% | 6.09 | 6.09 | 0.00% | 1.11 | 1.11 | -0.13% | 0.22 | 0.23 | 6.26% |
|  | 50% | 41.20 | 41.20 | 0.00% | 24.75 | 24.22 | -2.15% | 16.19 | 16.29 | 0.63% |
| DL Packet-Latency CDF (ms) | Mean | 13.55 | 13.56 | 0.05% | 43.37 | 44.45 | 2.50% | 60.12 | 60.15 | 0.05% |
|  | 5% | 10.93 | 10.93 | 0.00% | 10.98 | 10.98 | -0.01% | 11.07 | 11.07 | 0.01% |
|  | 50% | 11.32 | 11.32 | 0.00% | 14.88 | 14.86 | -0.10% | 21.30 | 21.31 | 0.06% |
| UL Packet-Latency CDF (ms) | Mean | 49.04 | 48.97 | -0.14% | 86.94 | 88.50 | 1.79% | 103.60 | 105.16 | 1.51% |
|  | 5% | 13.75 | 13.75 | 0.00% | 14.68 | 14.69 | 0.08% | 19.16 | 19.35 | 1.04% |
|  | 50% | 25.22 | 25.25 | 0.13% | 38.37 | 38.47 | 0.27% | 54.18 | 53.95 | -0.43% |
| DL RU (%) | Type-1 | 7.65% | 7.65% |  | 24.65% | 24.70% |  | 41.09% | 41.15% |  |
| UL RU (%) | Type-1 | 1.74% | 1.74% |  | 6.44% | 6.44% |  | 11.86% | 11.86% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 300.92 | 300.79 | -0.04% | 225.38 | 219.11 | -2.78% | 126.81 | 121.57 | -4.14% |
|  | 5% | 197.82 | 197.82 | 0.00% | 99.13 | 93.33 | -5.85% | 12.78 | 11.48 | -10.18% |
|  | 50% | 319.05 | 319.02 | -0.01% | 234.00 | 223.80 | -4.36% | 122.45 | 119.75 | -2.20% |
| UL Average-UPT (Mbps) | Mean | 134.95 | 181.75 | 34.68% | 73.22 | 116.41 | 58.98% | 43.52 | 60.61 | 39.28% |
|  | 5% | 69.88 | 93.99 | 34.50% | 41.75 | 58.79 | 40.83% | 6.39 | 7.53 | 17.91% |
|  | 50% | 129.65 | 187.95 | 44.97% | 71.70 | 113.60 | 58.44% | 44.18 | 60.85 | 37.75% |
| DL Packet-Latency CDF (ms) | Mean | 13.90 | 13.91 | 0.11% | 37.14 | 38.46 | 3.54% | 62.98 | 64.46 | 2.34% |
|  | 5% | 10.96 | 10.96 | 0.00% | 11.02 | 11.02 | -0.01% | 11.13 | 11.13 | 0.02% |
|  | 50% | 11.77 | 11.77 | 0.04% | 16.17 | 16.32 | 0.93% | 25.49 | 25.73 | 0.93% |
| UL Packet-Latency CDF (ms) | Mean | 35.26 | 34.36 | -2.54% | 62.28 | 61.61 | -1.07% | 85.40 | 83.32 | -2.43% |
|  | 5% | 5.33 | 4.37 | -17.94% | 10.13 | 5.76 | -43.16% | 12.88 | 10.01 | -22.29% |
|  | 50% | 16.07 | 15.95 | -0.75% | 24.94 | 23.78 | -4.63% | 43.84 | 41.31 | -5.76% |
| DL RU (%) | Type-1 | 8.95% | 8.97% |  | 24.29% | 25.21% |  | 47.02% | 48.58% |  |
| UL RU (%) | Type-1 | 2.02% | 1.46% |  | 9.59% | 6.52% |  | 15.28% | 11.99% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large Packet Size

P0 offset = 15 dB

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 318.69 | 318.59 | -0.03% | 234.81 | 234.09 | -0.31% | 177.50 | 177.10 | -0.22% |
|  | 5% | 180.91 | 180.51 | -0.22% | 13.46 | 12.87 | -4.36% | 2.71 | 2.54 | -6.16% |
|  | 50% | 351.04 | 351.04 | 0.00% | 268.30 | 267.90 | -0.15% | 198.63 | 198.00 | -0.32% |
| UL Average-UPT (Mbps) | Mean | 40.55 | 40.57 | 0.06% | 27.27 | 27.32 | 0.19% | 18.45 | 18.42 | -0.17% |
|  | 5% | 6.09 | 6.09 | 0.00% | 1.11 | 1.09 | -1.58% | 0.22 | 0.23 | 6.70% |
|  | 50% | 41.20 | 41.20 | 0.00% | 24.75 | 24.58 | -0.71% | 16.19 | 16.51 | 1.99% |
| DL Packet-Latency CDF (ms) | Mean | 13.55 | 13.56 | 0.05% | 43.37 | 44.27 | 2.09% | 60.12 | 59.72 | -0.67% |
|  | 5% | 10.93 | 10.93 | 0.00% | 10.98 | 10.98 | -0.01% | 11.07 | 11.07 | 0.00% |
|  | 50% | 11.32 | 11.32 | 0.00% | 14.88 | 14.84 | -0.23% | 21.30 | 21.18 | -0.56% |
| UL Packet-Latency CDF (ms) | Mean | 49.04 | 48.97 | -0.14% | 86.94 | 87.21 | 0.31% | 103.60 | 105.08 | 1.43% |
|  | 5% | 13.75 | 13.75 | 0.00% | 14.68 | 14.64 | -0.23% | 19.16 | 19.50 | 1.78% |
|  | 50% | 25.22 | 25.25 | 0.13% | 38.37 | 38.20 | -0.43% | 54.18 | 54.07 | -0.22% |
| DL RU (%) | Type-1 | 7.65% | 7.65% |  | 24.65% | 24.70% |  | 41.09% | 41.16% |  |
| UL RU (%) | Type-1 | 1.74% | 1.74% |  | 6.44% | 6.44% |  | 11.86% | 11.85% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 300.92 | 300.79 | -0.04% | 225.38 | 219.28 | -2.71% | 126.81 | 120.35 | -5.10% |
|  | 5% | 197.82 | 197.82 | 0.00% | 99.13 | 95.33 | -3.83% | 12.78 | 11.68 | -8.61% |
|  | 50% | 319.05 | 319.02 | -0.01% | 234.00 | 224.60 | -4.02% | 122.45 | 117.05 | -4.41% |
| UL Average-UPT (Mbps) | Mean | 134.95 | 181.75 | 34.68% | 73.22 | 115.93 | 58.33% | 43.52 | 60.38 | 38.74% |
|  | 5% | 69.88 | 93.99 | 34.50% | 41.75 | 58.69 | 40.59% | 6.39 | 7.17 | 12.16% |
|  | 50% | 129.65 | 187.95 | 44.97% | 71.70 | 111.80 | 55.93% | 44.18 | 60.05 | 35.94% |
| DL Packet-Latency CDF (ms) | Mean | 13.90 | 13.91 | 0.11% | 37.14 | 38.32 | 3.18% | 62.98 | 64.50 | 2.40% |
|  | 5% | 10.96 | 10.96 | 0.00% | 11.02 | 11.02 | -0.01% | 11.13 | 11.13 | 0.01% |
|  | 50% | 11.77 | 11.77 | 0.04% | 16.17 | 16.35 | 1.10% | 25.49 | 25.78 | 1.14% |
| UL Packet-Latency CDF (ms) | Mean | 35.26 | 34.36 | -2.54% | 62.28 | 60.76 | -2.44% | 85.40 | 83.55 | -2.16% |
|  | 5% | 5.33 | 4.37 | -17.94% | 10.13 | 5.81 | -42.68% | 12.88 | 9.98 | -22.49% |
|  | 50% | 16.07 | 15.95 | -0.75% | 24.94 | 23.83 | -4.44% | 43.84 | 41.61 | -5.08% |
| DL RU (%) | Type-1 | 8.95% | 8.97% |  | 24.29% | 25.20% |  | 47.02% | 48.76% |  |
| UL RU (%) | Type-1 | 2.02% | 1.46% |  | 9.59% | 6.53% |  | 15.28% | 12.02% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

- Source 2 ([26]):

Indoor Office (FR1)

Large packet size, P0= -83 dBm

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 1043.39 | 8.08% | 579.11 | 591.95 | 2.22% | 443.87 | 474.72 | 6.95% |
|  | 5% | 680.10 | 756.28 | 11.20% | 363.91 | 356.50 | -2.04% | 258.74 | 277.57 | 7.28% |
|  | 50% | 951.37 | 1013.50 | 6.53% | 557.41 | 582.67 | 4.53% | 411.05 | 456.04 | 10.94% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 160.64 | -44.82% | 132.35 | 5.69 | -95.70% | 92.82 | 0.27 | -99.70% |
|  | 5% | 231.38 | 119.42 | -48.39% | 67.53 | 0.00 | -100.00% | 26.95 | 0.00 | -100.00% |
|  | 50% | 292.47 | 154.73 | -47.09% | 129.13 | 0.00 | -100.00% | 86.65 | 0.00 | -100.00% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 4.22 | -9.98% | 8.24 | 8.20 | -0.49% | 11.38 | 10.74 | -5.68% |
|  | 5% | 2.77 | 2.61 | -5.81% | 4.08 | 4.03 | -1.29% | 4.83 | 4.62 | -4.30% |
|  | 50% | 3.95 | 3.73 | -5.43% | 7.08 | 6.88 | -2.90% | 9.57 | 9.04 | -5.60% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 9.17 | 138.00% | 12.60 | 4778.03 | 37807.73% | 28.25 | 9606.24 | 33898.44% |
|  | 5% | 2.64 | 3.80 | 43.61% | 3.91 | 10.83 | 177.40% | 4.97 | 18.99 | 282.26% |
|  | 50% | 3.07 | 5.88 | 91.28% | 7.48 | 192.53 | 2473.82% | 10.80 | 3058.95 | 28214.27% |
| DL RU (%) | Type-1 | 3.58% | 3.25% |  | 25.29% | 24.85% |  | 46.52% | 44.20% |  |
| UL RU (%) | Type-1 | 1.74% | 4.29% |  | 22.11% | 46.06% |  | 46.93% | 32.81% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Large packet size, P0= -33 dBm

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 965.40 | 907.50 | -6.00% | 579.11 | 434.04 | -25.05% | 443.87 | 283.33 | -36.17% |
|  | 5% | 680.10 | 651.66 | -4.18% | 363.91 | 271.29 | -25.45% | 258.74 | 141.64 | -45.26% |
|  | 50% | 951.37 | 901.75 | -5.22% | 557.41 | 411.72 | -26.14% | 411.05 | 243.54 | -40.75% |
| UL Average-UPT (Mbps) | Mean | 291.13 | 337.02 | 15.76% | 132.35 | 255.97 | 93.40% | 92.82 | 228.04 | 145.68% |
|  | 5% | 231.38 | 294.72 | 27.38% | 67.53 | 145.98 | 116.18% | 26.95 | 110.64 | 310.56% |
|  | 50% | 292.47 | 342.07 | 16.96% | 129.13 | 257.18 | 99.16% | 86.65 | 223.28 | 157.68% |
| DL Packet-Latency CDF (ms) | Mean | 4.68 | 5.09 | 8.74% | 8.24 | 11.41 | 38.54% | 11.38 | 19.19 | 68.57% |
|  | 5% | 2.77 | 2.89 | 4.52% | 4.08 | 4.86 | 18.93% | 4.83 | 6.26 | 29.69% |
|  | 50% | 3.95 | 4.30 | 9.05% | 7.08 | 9.59 | 35.44% | 9.57 | 14.79 | 54.48% |
| UL Packet-Latency CDF (ms) | Mean | 3.85 | 3.09 | -19.83% | 12.60 | 4.77 | -62.14% | 28.25 | 5.95 | -78.93% |
|  | 5% | 2.64 | 2.61 | -1.35% | 3.91 | 2.68 | -31.42% | 4.97 | 2.73 | -45.01% |
|  | 50% | 3.07 | 2.88 | -6.40% | 7.48 | 3.82 | -48.91% | 10.80 | 4.55 | -57.85% |
| DL RU (%) | Type-1 | 3.58% | 3.86% |  | 25.29% | 33.00% |  | 46.52% | 62.51% |  |
| UL RU (%) | Type-1 | 1.74% | 1.18% |  | 22.11% | 9.07% |  | 46.93% | 17.96% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Small packet size, P0= -83 dBm

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 43.32 | 0.53% | 42.11 | 42.11 | -0.02% | 41.40 | 41.61 | 0.52% |
|  | 5% | 41.76 | 41.94 | 0.44% | 41.22 | 41.21 | -0.03% | 40.78 | 40.91 | 0.31% |
|  | 50% | 43.08 | 43.29 | 0.50% | 41.91 | 41.91 | 0.02% | 41.37 | 41.51 | 0.32% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.23 | -0.29% | 4.23 | 0.13 | -96.87% | 3.95 | 0.00 | -100.00% |
|  | 5% | 4.19 | 4.16 | -0.56% | 4.20 | 0.00 | -100.00% | 2.54 | 0.00 | -100.00% |
|  | 50% | 4.25 | 4.24 | -0.07% | 4.23 | 0.00 | -100.00% | 4.21 | 0.00 | -100.00% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 1.15 | 0.78% | 1.24 | 0.99 | -20.51% | 1.23 | 1.17 | -4.33% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.54 | 0.00% | 0.55 | 0.53 | -4.68% |
|  | 50% | 0.80 | 0.82 | 2.22% | 0.84 | 0.79 | -5.32% | 0.88 | 0.82 | -6.12% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 3.51 | 59.29% | 4.17 | 1358.94 | 32456.20% | 45.40 | 3337.55 | 7251.29% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.70 | 5.79% | 1.63 | 3275.65 | 201478.40% |
|  | 50% | 1.88 | 1.91 | 1.90% | 1.91 | 846.06 | 44179.73% | 1.96 | 3340.85 | 169979.55% |
| DL RU (%) | Type-1 | 3.55% | 3.51% |  | 23.29% | 22.63% |  | 49.12% | 45.39% |  |
| UL RU (%) | Type-1 | 1.71% | 5.27% |  | 14.68% | 26.80% |  | 39.61% | 25.18% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

Small packet size, P0= -33 dBm

|  |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 43.09 | 42.97 | -0.28% | 42.11 | 42.31 | 0.47% | 41.40 | 40.97 | -1.02% |
|  | 5% | 41.76 | 41.70 | -0.14% | 41.22 | 41.30 | 0.20% | 40.78 | 38.98 | -4.41% |
|  | 50% | 43.08 | 42.94 | -0.33% | 41.91 | 42.28 | 0.89% | 41.37 | 41.17 | -0.48% |
| UL Average-UPT (Mbps) | Mean | 4.25 | 4.15 | -2.29% | 4.23 | 4.25 | 0.56% | 3.95 | 4.26 | 7.80% |
|  | 5% | 4.19 | 4.00 | -4.57% | 4.20 | 4.22 | 0.40% | 2.54 | 4.21 | 65.91% |
|  | 50% | 4.25 | 4.13 | -2.64% | 4.23 | 4.25 | 0.39% | 4.21 | 4.25 | 1.02% |
| DL Packet-Latency CDF (ms) | Mean | 1.14 | 1.17 | 1.85% | 1.24 | 1.22 | -2.03% | 1.23 | 1.21 | -1.18% |
|  | 5% | 0.54 | 0.54 | 0.00% | 0.54 | 0.52 | -3.33% | 0.55 | 0.52 | -6.45% |
|  | 50% | 0.80 | 0.83 | 3.33% | 0.84 | 0.82 | -2.13% | 0.88 | 0.86 | -2.04% |
| UL Packet-Latency CDF (ms) | Mean | 2.21 | 2.38 | 7.78% | 4.17 | 2.24 | -46.36% | 45.40 | 2.15 | -95.27% |
|  | 5% | 1.61 | 1.61 | 0.00% | 1.60 | 1.59 | -0.89% | 1.63 | 1.57 | -3.30% |
|  | 50% | 1.88 | 1.91 | 1.90% | 1.91 | 1.89 | -0.93% | 1.96 | 1.89 | -3.64% |
| DL RU (%) | Type-1 | 3.55% | 3.71% |  | 23.29% | 26.17% |  | 49.12% | 56.65% |  |
| UL RU (%) | Type-1 | 1.71% | 1.35% |  | 14.68% | 6.67% |  | 39.61% | 14.05% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

2-layer Scenario B (FR1)

small packet size, layer 2 P0=-40 dBm

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 38.85 | 38.83 | -0.06% | 36.95 | 35.58 | -3.72% | 33.75 | 31.93 | -5.41% |
|  | 5% | 37.00 | 37.00 | -0.01% | 34.05 | 5.57 | -83.63% | 0.17 | 0.00 | -100.00% |
|  | 50% | 38.81 | 38.80 | -0.03% | 37.90 | 37.82 | -0.22% | 37.32 | 37.26 | -0.14% |
| UL Average-UPT (Mbps) | Mean | 1.65 | 1.64 | -0.21% | 1.64 | 1.63 | -0.70% | 1.55 | 1.52 | -1.77% |
|  | 5% | 1.56 | 1.55 | -0.33% | 1.38 | 1.25 | -9.60% | 0.05 | 0.02 | -61.50% |
|  | 50% | 1.65 | 1.64 | -0.31% | 1.69 | 1.69 | -0.31% | 1.72 | 1.72 | -0.35% |
| DL Packet-Latency CDF (ms) | Mean | 1.10 | 1.13 | 2.57% | 4.08 | 5.91 | 44.86% | 15.42 | 9.92 | -35.64% |
|  | 5% | 0.55 | 0.54 | -2.26% | 0.54 | 0.54 | 0.00% | 0.54 | 0.54 | 0.00% |
|  | 50% | 0.86 | 0.87 | 1.04% | 0.86 | 0.86 | 0.00% | 0.88 | 0.88 | 0.00% |
| UL Packet-Latency CDF (ms) | Mean | 5.21 | 5.25 | 0.90% | 8.02 | 8.36 | 4.26% | 20.17 | 26.55 | 31.61% |
|  | 5% | 3.77 | 3.74 | -0.76% | 3.62 | 3.64 | 0.62% | 3.36 | 3.43 | 2.13% |
|  | 50% | 5.07 | 5.11 | 0.70% | 4.98 | 5.00 | 0.36% | 5.04 | 5.05 | 0.35% |
| DL RU (%) | Type-1 | 5.65% | 5.68% |  | 31.17% | 33.81% |  | 54.21% | 57.19% |  |
| UL RU (%) | Type-1 | 5.88% | 6.04% |  | 30.88% | 31.82% |  | 47.00% | 48.13% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 27.88 | 27.89 | 0.05% | 26.16 | 25.92 | -0.93% | 24.94 | 24.72 | -0.88% |
|  | 5% | 26.51 | 26.56 | 0.20% | 22.81 | 21.95 | -3.81% | 17.53 | 16.64 | -5.06% |
|  | 50% | 27.87 | 27.91 | 0.13% | 26.64 | 26.48 | -0.60% | 26.02 | 25.82 | -0.77% |
| UL Average-UPT (Mbps) | Mean | 3.42 | 3.45 | 0.83% | 3.03 | 3.31 | 9.13% | 2.74 | 3.14 | 14.62% |
|  | 5% | 3.35 | 3.36 | 0.43% | 1.60 | 2.51 | 56.67% | 0.01 | 2.08 | 22136.34% |
|  | 50% | 3.45 | 3.45 | -0.15% | 3.35 | 3.44 | 2.51% | 3.12 | 3.38 | 8.34% |
| DL Packet-Latency CDF (ms) | Mean | 1.67 | 1.71 | 2.05% | 1.74 | 1.78 | 2.64% | 2.35 | 2.40 | 1.91% |
|  | 5% | 0.61 | 0.59 | -2.94% | 0.57 | 0.59 | 3.12% | 0.59 | 0.59 | 0.00% |
|  | 50% | 1.49 | 1.52 | 1.80% | 1.52 | 1.54 | 1.18% | 1.58 | 1.59 | 0.56% |
| UL Packet-Latency CDF (ms) | Mean | 4.54 | 2.65 | -41.61% | 3.29 | 2.81 | -14.72% | 6.94 | 5.06 | -27.08% |
|  | 5% | 1.68 | 1.66 | -1.06% | 1.64 | 1.64 | 0.00% | 1.64 | 1.63 | -1.09% |
|  | 50% | 2.57 | 2.55 | -0.69% | 2.63 | 2.46 | -6.12% | 2.75 | 2.57 | -6.49% |
| DL RU (%) | Type-1 | 17.76% | 17.78% |  | 32.18% | 33.22% |  | 37.27% | 37.91% |  |
| UL RU (%) | Type-1 | 2.56% | 1.74% |  | 21.94% | 16.54% |  | 32.44% | 27.95% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

small packet size, layer 2 P0=-70 dBm

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 38.85 | 38.86 | 0.04% | 36.95 | 36.76 | -0.53% | 33.75 | 33.85 | 0.28% |
|  | 5% | 37.00 | 37.00 | 0.00% | 34.05 | 33.67 | -1.13% | 0.17 | 0.25 | 48.24% |
|  | 50% | 38.81 | 38.81 | -0.02% | 37.90 | 37.90 | 0.01% | 37.32 | 37.35 | 0.09% |
| UL Average-UPT (Mbps) | Mean | 1.65 | 1.64 | -0.14% | 1.64 | 1.64 | -0.18% | 1.55 | 1.55 | -0.11% |
|  | 5% | 1.56 | 1.55 | -0.19% | 1.38 | 1.32 | -4.69% | 0.05 | 0.05 | -1.72% |
|  | 50% | 1.65 | 1.64 | -0.28% | 1.69 | 1.69 | -0.01% | 1.72 | 1.72 | 0.05% |
| DL Packet-Latency CDF (ms) | Mean | 1.10 | 1.10 | -0.62% | 4.08 | 4.15 | 1.82% | 15.42 | 14.55 | -5.64% |
|  | 5% | 0.55 | 0.55 | 0.00% | 0.54 | 0.54 | 0.00% | 0.54 | 0.54 | 0.00% |
|  | 50% | 0.86 | 0.86 | 0.00% | 0.86 | 0.84 | -2.08% | 0.88 | 0.88 | -1.01% |
| UL Packet-Latency CDF (ms) | Mean | 5.21 | 5.20 | -0.12% | 8.02 | 7.37 | -8.12% | 20.17 | 20.67 | 2.47% |
|  | 5% | 3.77 | 3.75 | -0.43% | 3.62 | 3.61 | -0.37% | 3.36 | 3.36 | 0.00% |
|  | 50% | 5.07 | 5.07 | 0.00% | 4.98 | 4.96 | -0.36% | 5.04 | 5.04 | 0.00% |
| DL RU (%) | Type-1 | 5.65% | 5.65% |  | 31.17% | 31.12% |  | 54.21% | 54.06% |  |
| UL RU (%) | Type-1 | 5.88% | 5.92% |  | 30.88% | 30.95% |  | 47.00% | 47.00% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 27.88 | 27.87 | -0.02% | 26.16 | 26.17 | 0.01% | 24.94 | 24.96 | 0.07% |
|  | 5% | 26.51 | 26.59 | 0.30% | 22.81 | 22.17 | -2.82% | 17.53 | 18.09 | 3.22% |
|  | 50% | 27.87 | 27.90 | 0.09% | 26.64 | 26.68 | 0.16% | 26.02 | 26.00 | -0.08% |
| UL Average-UPT (Mbps) | Mean | 3.42 | 3.23 | -5.39% | 3.03 | 1.18 | -61.00% | 2.74 | 0.82 | -70.12% |
|  | 5% | 3.35 | 1.82 | -45.45% | 1.60 | 0.00 | -100.00% | 0.01 | 0.00 | -100.00% |
|  | 50% | 3.45 | 3.42 | -1.02% | 3.35 | 0.79 | -76.48% | 3.12 | 0.00 | -99.92% |
| DL Packet-Latency CDF (ms) | Mean | 1.67 | 1.67 | -0.34% | 1.74 | 1.73 | -0.50% | 2.35 | 2.88 | 22.24% |
|  | 5% | 0.61 | 0.59 | -2.35% | 0.57 | 0.57 | 0.00% | 0.59 | 0.59 | 0.00% |
|  | 50% | 1.49 | 1.48 | -0.60% | 1.52 | 1.48 | -2.35% | 1.58 | 1.58 | 0.00% |
| UL Packet-Latency CDF (ms) | Mean | 4.54 | 5.42 | 19.46% | 3.29 | 150.63 | 4472.72% | 6.94 | 173.95 | 2408.12% |
|  | 5% | 1.68 | 1.70 | 1.06% | 1.64 | 1.79 | 8.70% | 1.64 | 1.84 | 11.96% |
|  | 50% | 2.57 | 2.63 | 2.08% | 2.63 | 4.02 | 53.06% | 2.75 | 4.79 | 74.03% |
| DL RU (%) | Type-1 | 17.76% | 17.75% |  | 32.18% | 32.19% |  | 37.27% | 37.34% |  |
| UL RU (%) | Type-1 | 2.56% | 4.30% |  | 21.94% | 21.68% |  | 32.44% | 24.01% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

large packet size, layer 2 P0=-40 dBm

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 739.51 | 732.28 | -0.98% | 431.78 | 406.14 | -5.94% | 292.80 | 270.40 | -7.65% |
|  | 5% | 270.01 | 251.41 | -6.89% | 61.35 | 0.00 | -100.00% | 0.00 | 0.00 | -100.00% |
|  | 50% | 726.21 | 718.88 | -1.01% | 435.37 | 413.66 | -4.99% | 281.51 | 260.84 | -7.34% |
| UL Average-UPT (Mbps) | Mean | 86.96 | 86.04 | -1.06% | 54.24 | 52.37 | -3.45% | 36.28 | 34.68 | -4.43% |
|  | 5% | 54.11 | 53.74 | -0.68% | 7.47 | 4.09 | -45.26% | 0.04 | 0.05 | 30.79% |
|  | 50% | 92.38 | 92.03 | -0.38% | 57.33 | 56.40 | -1.62% | 39.29 | 37.39 | -4.85% |
| DL Packet-Latency CDF (ms) | Mean | 6.88 | 7.03 | 2.09% | 70.09 | 60.04 | -14.35% | 133.18 | 200.23 | 50.34% |
|  | 5% | 2.96 | 3.00 | 1.20% | 4.30 | 4.41 | 2.49% | 5.16 | 5.22 | 1.11% |
|  | 50% | 5.53 | 5.55 | 0.48% | 9.35 | 9.52 | 1.81% | 13.63 | 14.27 | 4.72% |
| UL Packet-Latency CDF (ms) | Mean | 12.03 | 12.21 | 1.50% | 70.09 | 104.91 | 49.68% | 451.49 | 484.15 | 7.23% |
|  | 5% | 8.79 | 8.80 | 0.20% | 10.16 | 10.34 | 1.76% | 12.04 | 12.45 | 3.44% |
|  | 50% | 10.61 | 10.73 | 1.18% | 17.84 | 18.44 | 3.35% | 26.57 | 28.13 | 5.88% |
| DL RU (%) | Type-1 | 7.08% | 7.23% |  | 30.97% | 35.14% |  | 52.90% | 58.26% |  |
| UL RU (%) | Type-1 | 5.13% | 5.26% |  | 31.56% | 32.40% |  | 54.08% | 56.31% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 302.73 | 300.16 | -0.85% | 123.60 | 61.47 | -50.27% | 34.01 | 19.93 | -41.40% |
|  | 5% | 178.25 | 175.04 | -1.80% | 48.70 | 0.67 | -98.62% | 0.20 | 0.05 | -72.49% |
|  | 50% | 291.30 | 290.48 | -0.28% | 115.06 | 54.76 | -52.41% | 25.68 | 5.36 | -79.13% |
| UL Average-UPT (Mbps) | Mean | 204.38 | 234.59 | 14.78% | 113.87 | 193.22 | 69.69% | 76.31 | 158.99 | 108.34% |
|  | 5% | 135.49 | 205.76 | 51.86% | 24.14 | 112.95 | 367.86% | 0.09 | 65.65 | 69134.24% |
|  | 50% | 216.87 | 239.09 | 10.25% | 117.44 | 205.85 | 75.29% | 79.17 | 163.82 | 106.92% |
| DL Packet-Latency CDF (ms) | Mean | 15.70 | 15.86 | 1.02% | 54.93 | 478.11 | 770.35% | 1277.40 | 2466.58 | 93.09% |
|  | 5% | 6.58 | 6.63 | 0.66% | 14.93 | 23.61 | 58.16% | 30.52 | 36.60 | 19.90% |
|  | 50% | 13.79 | 13.95 | 1.22% | 36.99 | 81.13 | 119.33% | 198.50 | 407.16 | 105.12% |
| UL Packet-Latency CDF (ms) | Mean | 5.20 | 4.35 | -16.39% | 10.72 | 5.98 | -44.25% | 38.41 | 11.16 | -70.94% |
|  | 5% | 3.66 | 3.61 | -1.46% | 4.96 | 3.66 | -26.13% | 6.18 | 3.78 | -38.90% |
|  | 50% | 4.71 | 4.05 | -14.02% | 8.45 | 4.90 | -41.97% | 12.51 | 5.99 | -52.11% |
| DL RU (%) | Type-1 | 15.72% | 15.87% |  | 36.95% | 55.96% |  | 72.97% | 81.30% |  |
| UL RU (%) | Type-1 | 2.23% | 1.39% |  | 18.30% | 9.09% |  | 35.64% | 19.79% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |

large packet size, layer 2 P0=-70 dBm

| Layer-1 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 739.51 | 740.32 | 0.11% | 431.78 | 434.20 | 0.56% | 292.80 | 292.02 | -0.26% |
|  | 5% | 270.01 | 273.09 | 1.14% | 61.35 | 73.80 | 20.29% | 0.00 | 0.16 | 8560.78% |
|  | 50% | 726.21 | 725.59 | -0.08% | 435.37 | 434.64 | -0.17% | 281.51 | 278.04 | -1.23% |
| UL Average-UPT (Mbps) | Mean | 86.96 | 87.02 | 0.07% | 54.24 | 54.32 | 0.15% | 36.28 | 36.40 | 0.32% |
|  | 5% | 54.11 | 54.20 | 0.18% | 7.47 | 7.92 | 6.00% | 0.04 | 0.04 | -1.60% |
|  | 50% | 92.38 | 92.51 | 0.14% | 57.33 | 57.30 | -0.04% | 39.29 | 39.41 | 0.29% |
| DL Packet-Latency CDF (ms) | Mean | 6.88 | 6.87 | -0.25% | 70.09 | 18.76 | -73.24% | 133.18 | 148.46 | 11.47% |
|  | 5% | 2.96 | 2.98 | 0.60% | 4.30 | 4.29 | -0.41% | 5.16 | 5.18 | 0.35% |
|  | 50% | 5.53 | 5.53 | 0.00% | 9.35 | 9.32 | -0.29% | 13.63 | 14.01 | 2.82% |
| UL Packet-Latency CDF (ms) | Mean | 12.03 | 12.01 | -0.16% | 70.09 | 74.73 | 6.62% | 451.49 | 435.92 | -3.45% |
|  | 5% | 8.79 | 8.79 | 0.00% | 10.16 | 10.11 | -0.53% | 12.04 | 12.00 | -0.30% |
|  | 50% | 10.61 | 10.60 | -0.08% | 17.84 | 17.81 | -0.15% | 26.57 | 26.45 | -0.45% |
| DL RU (%) | Type-1 | 7.08% | 7.06% |  | 30.97% | 30.83% |  | 52.90% | 52.78% |  |
| UL RU (%) | Type-1 | 5.13% | 5.12% |  | 31.56% | 31.54% |  | 54.08% | 54.01% |  |
| Layer-2 |  | DL: Low, UL: Low |  |  | DL: Medium, UL: Medium |  |  | DL: High, UL: High |  |  |
|  |  | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase | Baseline operation | Target operation | Gain /Increase |
| DL Average-UPT (Mbps) | Mean | 302.73 | 303.24 | 0.17% | 123.60 | 125.39 | 1.45% | 34.01 | 37.92 | 11.49% |
|  | 5% | 178.25 | 177.41 | -0.47% | 48.70 | 51.36 | 5.45% | 0.20 | 0.27 | 39.26% |
|  | 50% | 291.30 | 293.08 | 0.61% | 115.06 | 114.94 | -0.11% | 25.68 | 28.69 | 11.70% |
| UL Average-UPT (Mbps) | Mean | 204.38 | 153.74 | -24.77% | 113.87 | 42.35 | -62.81% | 76.31 | 15.45 | -79.75% |
|  | 5% | 135.49 | 80.45 | -40.62% | 24.14 | 0.00 | -100.00% | 0.09 | 0.00 | -100.00% |
|  | 50% | 216.87 | 156.96 | -27.62% | 117.44 | 40.45 | -65.55% | 79.17 | 7.99 | -89.91% |
| DL Packet-Latency CDF (ms) | Mean | 15.70 | 15.66 | -0.27% | 54.93 | 51.98 | -5.37% | 1277.40 | 1131.68 | -11.41% |
|  | 5% | 6.58 | 6.61 | 0.39% | 14.93 | 14.79 | -0.90% | 30.52 | 29.45 | -3.51% |
|  | 50% | 13.79 | 13.80 | 0.13% | 36.99 | 36.40 | -1.61% | 198.50 | 175.90 | -11.38% |
| UL Packet-Latency CDF (ms) | Mean | 5.20 | 7.47 | 43.53% | 10.72 | 111.46 | 939.82% | 38.41 | 908.92 | 2266.66% |
|  | 5% | 3.66 | 4.00 | 9.27% | 4.96 | 8.55 | 72.61% | 6.18 | 13.80 | 123.35% |
|  | 50% | 4.71 | 6.31 | 33.87% | 8.45 | 23.31 | 176.03% | 12.51 | 76.67 | 512.90% |
| DL RU (%) | Type-1 | 15.72% | 15.69% |  | 36.95% | 36.35% |  | 72.97% | 71.38% |  |
| UL RU (%) | Type-1 | 2.23% | 3.91% |  | 18.30% | 35.19% |  | 35.64% | 43.28% |  |
| Note: - For UPT, the gain can be calculated as: Gain (%) = Target UPT / Baseline UPT - 1 - For Latency, the increase can be calculated as: Increase (%) = Target latency / Baseline latency - 1 - For RU, the increase can be calculated as: Increase (%) = Target RU (%) - Baseline RU (%) - If the Gain/Increase for DL/UL Average-UPT is quoted as NaN%, it means that the throughput for the reference scheme (baseline) is zero. |  |  |  |  |  |  |  |  |  |  |