3GPP TSG-RAN WG1 Meeting #114	R1-2308658

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0495 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of TDD UL validation and SDT operation for RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), NEC, MediaTek Inc., ZTE, NTT DOCOMO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core, NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN1 discussed TDD UL validation and SDT operation for RedCap UEs and the discussion is captured under Issue #1 and Issue #3 in the feature lead summary in R1-2308222. For RedCap TDD UEs, it needs to be clarified whether the determination of the following cases in a BWP with NCD-SSB and without CD-SSB are based on CD-SSB or NCD-SSB: - PRACH occasion validation - MsgA PUSCH occasion validation - Msg3 PUSCH repetition resource counting - CG-PUSCH occasion validation For RedCap UEs in CG/RA-SDT operation, RAN2#121 agreed CG/RA-SDT can be performed if the initial DL BWP does not include the CD-SSB but an NCD-SSB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For RedCap TDD UEs, it is clarified that the determination of the following cases in a BWP with NCD-SSB and without CD-SSB are based on CD-SSB (i.e, not based on NCD-SSB): - PRACH occasion validation - MsgA PUSCH occasion validation - Msg3 PUSCH repetition resource counting - CG-PUSCH occasion validation For RedCap UEs in CG/RA-SDT operation, it is clarified that CG/RA-SDT can be performed if the initial DL BWP does not include the CD-SSB but an NCD-SSB, and what SSB to use for this purpose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, - For RedCap TDD UEs, it is unclear whether the above mentioned cases for TDD UL validation are based on CD-SSB or NCD-SSB. - For RedCap UEs in CG/RA-SDT operation, it is unclear that CG/RA-SDT can be performed if the initial DL BWP does not include the CD-SSB but an NCD-SSB, and what SSB to use for this purpose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 17.1	RedCap UE procedures

Procedures for a RedCap UE are same as described for a UE in all other clauses of this document unless stated otherwise. In this clause, the term 'UE' refers to a RedCap UE.

A UE expects the initial DL BWP and the active DL BWP after the UE (re)establishes dedicated RRC connection to be smaller than or equal to the maximum DL bandwidth that the UE supports. A UE can be provided a DL BWP by initialDownlinkBWP-RedCap in DownlinkConfigCommonSIB, and an UL BWP by initialUplinkBWP-RedCap in UplinkConfigCommonSIB. If initialUplinkBWP in UplinkConfigCommonSIB indicates an UL BWP that is larger than a maximum UL BWP that a UE supports, the UE expects to be provided an UL BWP by initialUplinkBWP-RedCap in UplinkConfigCommonSIB that is smaller than or equal to the maximum UL bandwidth that the UE supports.

For unpaired spectrum operation, a RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set, or a CSS set provided by sdt-SearchSpace for random-access based PUSCH transmission as described in clause 19.2, is different than the center frequency for an initial UL BWP in which the RedCap UE may transmit Msg1/Msg3 or MsgA.

A UE can be provided by BWP-DownlinkDedicated a DL BWP, other than the initial DL BWP. A UE can be provided by BWP-UplinkDedicated an UL BWP, other than the initial UL BWP, that is smaller than or equal to the maximum UL bandwidth that the UE supports.

If a UE is provided an UL BWP by initialUplinkBWP-RedCap in UplinkConfigCommonSIB and is provided rach-ConfigCommon or msgA-ConfigCommon in BWP-UplinkCommon for the UL BWP, the UE uses corresponding parameters to perform the procedures in clauses 8.1, 8.1A, and 8.3; otherwise, the UE uses corresponding parameters from rach-ConfigCommon or msgA-ConfigCommon in BWP-UplinkCommon for the UL BWP provided by initialUplinkBWP.

If a UE is provided initialUplinkBWP-RedCap in UplinkConfigCommonSIB and does not have dedicated PUCCH resource configuration, the UE transmits PUCCH with HARQ-ACK information as described in clause 9.2.1 using a PUCCH resource set provided by pucch-ResourceCommonRedCap, except that frequency hopping for the PUCCH transmission is disabled if intra-SlotFH is present in PUCCH-ConfigCommon. If frequency hopping of the PUCCH transmission is disabled then, for the PUCCH transmission, the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS and determines the PRB index as

-	RBBWPoffset+RBBWPoffset-add+rPUCCHNCS, if intra-SlotFH = 'fromLowerEdge'

-	NBWPsize-RBBWPoffset-RBBWPoffset-add-1-rPUCCHNCS, otherwise

where RBBWPoffset-add is provided by additionalPRBOffset, if provided; otherwise, RBBWPoffset-add=0.

If a UE is not provided initialUplinkBWP-RedCap in UplinkConfigCommonSIB and does not have dedicated PUCCH resource configuration, the UE transmits PUCCH with HARQ-ACK information as described in clause 9.2.1 using a PUCCH resource set provided by pucch-ResourceCommonRedCap if pucch-ResourceCommonRedCap is present or by pucch-ResourceCommon if pucch-ResourceCommonRedCap is absent. For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE does not expect the initial DL BWP to include SS/PBCH blocks and the CORESET with index 0.

For an active DL BWP not provided by BWP-DownlinkDedicated, if a UE does not indicate a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0.

For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0.

For a RedCap UE indicating a capability to use an initial DL BWP that includes the SS/PBCH blocks provided by NonCellDefiningSSB for PUSCH transmission in RRC_INACTIVE state, if the UE is provided NonCellDefiningSSB in ncd-SSB-RedCapInitialBWP-SDT, then during procedure of PUSCH transmission in RRC_INACTIVE state (as described in clause 19) the UE uses the SS/PBCH blocks provided by NonCellDefiningSSB for the purposes for which the UE would otherwise have used the SS/PBCH blocks that the UE used to obtain SIB1.

If the active DL BWP provided by BWP-DownlinkDedicated, or the initial DL BWP during procedure of PUSCH transmission in RRC_INACTIVE state (as described in clause 19), includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same QCL properties, if they have the same index.

For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, collision handling between downlink receptions or uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated.

For monitoring of a PDCCH candidate by a UE configured with NonCellDefiningSSB, if the UE

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by NonCellDefiningSSB,

the UE is not required to monitor the PDCCH candidate.

The SS/PBCH blocks in clause 8.1 for determining valid PRACH occasions in unpaired spectrum correspond to the SS/PBCH blocks that the UE used to obtain SIB1.

The SS/PBCH blocks in clause 8.1A for determining valid PUSCH occasions in unpaired spectrum correspond to the SS/PBCH blocks that the UE used to obtain SIB1.

The SS/PBCH blocks in clause 6.1.2.1 in [6, TS 38.214] and clause 8.3 for determining the NPUSCHrepeat slots for a PUSCH transmission in unpaired spectrum correspond to the SS/PBCH blocks that the UE used to obtain SIB1.

The SS/PBCH blocks in clause 19.1 for determining valid PUSCH occasions in unpaired spectrum correspond to the SS/PBCH blocks that the UE used to obtain SIB1.