3GPP TSG-RAN WG1 Meeting #114	R1- 2308640

Toulouse, France, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0492 | rev |  | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DCI 2_0 fields for operation with shared spectrum channel access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For operation with shared spectrum channel access, the description of the available RB set indicator in DCI 2_0 in 38.213 Section 11.1.1 does not cover the case that intraCellGuardBandsDL-List is not provided for a serving cell. This is incomplete, since guard bands and RB sets are still defined even if intraCellGuardBandsDL-List is not provided; in this case, the UE assumes the nominal (default) values specified in in 38.101-1 Section 5.3.3. - For the following 3 fields potentially present in DCI 2_0 (available RB set indicator, channel occupancy (CO) duration, and search space set group (SSSG) switching flag), the current wording in 38.213 Section 11.1.1 incorrectly states that it is the location of the field in DCI 2_0 (configured by availableRB-SetsPerCell, CO-DurationsPerCell, and SearchSpaceSwitchTrigger) that indicates RB set availability, CO duration, and SSSG switching, respectively, rather than the field itself. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct 38.213 Section 11.1.1 so that the description of the RB set indicator field in DCI 2_0 also includes the case when intraCellGuardBandsDL-List is not provided for a serving cell, i.e., when the UE assumes the nominal (default) values for the guard bands and RB sets as specified in 38.101-1 Section 5.3.3. This results in consistency between 38.213 and 38.331. - Correct the descriptions of the available RB set indicator, channel occupancy duration, and search space set group switching flag fields in DCI 2_0 such that the bit location of a field in DCI 2_0 is not confused with the field itself. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Potentially incorrect DCI 2_0 decoding for the case when intraCellGuardBandsDL-List is not provided - Potentially incorrect UE implementation due to lack of a clear distinction between the location of a field in DCI 2_0 and the field itself. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

### 11.1.1	UE procedure for determining slot format

*** Unchanged text omitted ***

For each serving cell in the set of serving cells, the UE can be provided:

-	an identity of the serving cell by servingCellId

-	a location of a SFI-index field in DCI format 2_0 by positionInDCI

-	a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes

-	one or more slot formats indicated by a respective slotFormats for the slot format combination, and

-	a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId

-	for unpaired spectrum operation, a reference SCS configuration μSFI by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration μSFI,SUL by subcarrierSpacing2 for the supplementary UL carrier

-	for paired spectrum operation, a reference SCS configuration μSFI,DL for a DL BWP by subcarrierSpacing and a reference SCS configuration μSFI,UL for an UL BWP by subcarrierSpacing2

-	a location of an available RB set indicator field in DCI format 2_0, by availableRB-SetsPerCell, where the field is

-	one bit, if intraCellGuardBandsDL-List for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, and the serving cell remains available or unavailable for reception until the end of the remaining channel occupancy duration; or,

-	a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandsDL-List for the serving cell indicates intra-cell guard-bands are configured or if intraCellGuardBandsDL-List is not provided for the serving cell, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, and a RB set remains available or unavailable for receptions until the end of the remaining channel occupancy duration

-	a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationsPerCell, where the field indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from co-DurationList. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by co-DurationList. If CO-DurationsPerCell is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a reference SCS configuration for co-DurationList, by subcarrierSpacing-r16

-	a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger, where the field indicates a group from two groups of search space sets for PDCCH monitoring for scheduling for the serving cell or the set of serving cells, provided by CellGroupsForSwitching, as described in clause 10.4.

*** Unchanged text omitted ***