3GPP TSG- RAN WG1 Meeting #114	R1-2308589

Toulouse, France, August 21st – August 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0433 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the calculation of the number of coded modulation symbols for UCI multiplexing on TBoMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (China Telecom), Xiaomi, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2023-8-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The scaling the CB size of UCI multiplexing on an available slot of TBoMS has been captured in TS 38.212 clause 6.3.2.4.1.3, but not captured in TS 38.214. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the scaling the CB size of UCI multiplexing on an available slot of TBoMS in TS 38.214 clause 5.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment for CB size of UCI multiplexing on an available slot of TBoMS between TS 38.212 clause 6.3.2.4.1.3 and TS 38.214 clause 5.2.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.3	CSI reporting using PUSCH

< Unchanged parts are omitted >

When the UE is scheduled to transmit a transport block on PUSCH not using repetition type B multiplexed with a CSI report(s) and if numberOfSlotsTBoMS is not present in the resource allocation table, or if numberOfSlotsTBoMS is present in the resource allocation table and the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is equal to 1, Part 2 CSI is omitted only when  is larger than α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1', where parameters , , , , , , , , QACK/CG-UCI' and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes the  to be less than or equal to α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'.

When the UE is scheduled to transmit a transport block on PUSCH not using repetition type B multiplexed with a CSI report(s) and if numberOfSlotsTBoMS is present in the resource allocation table and the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is larger than 1, Part 2 CSI is omitted only when OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIl1Nsr=0CUL-SCH-1Kr  is larger than α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1', where parameters , , , , , , , Ns ,, QACK/CG-UCI' and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes the  OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIl1Nsr=0CUL-SCH-1Kr to be less than or equal to α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'.

< Unchanged parts are omitted >