3GPP TSG-RAN WG1 Meeting #114	R1-2308710

# Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0145 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-18 MIMO Evolution for Downlink and Uplink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2023-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of Rel-18 MIMO Evolution for Downlink and Uplink |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-18 MIMO Evolution for Downlink and Uplink: Multi-TRP enhancement - Clause 3.3: Add abbreviation for TCI; - Clause 7.3.1.1.2: Update Table 7.3.1.1.2-36 to reflect agreements on unified TCI frame work; - Clause 7.3.1.2.2 & 7.3.1.2.3: Add TCI selection field in DCI format 1_1 and DCI format 1_2 to reflect the agreements on unified TCI framework; CSI enhancement - Clause 6.3.2.1.2: Add Table 6.3.2.1.2-1B/1C/2C/2D/3C/5E/5F/5G/8A/8B/9A/10 to reflect agreements on CSI enhancements; - Clause 6.3.2.1.2: Update Table 6.3.2.1.2-3/5B/6/7/9 to reflect agreements on CSI enhancements; Reference signal enhancement - Clause 7.3.1.1.2: Update Antenna ports field and PTRS-DMRS association field in DCI format 0_1 and add corresponding tables to reflect the agreements on DMRS; - Clause 7.3.1.1.3: Update Antenna ports field and PTRS-DMRS association field in DCI format 0_2 and add corresponding tables to reflect the agreements on DMRS; - Clause 7.3.1.2.2: Update Antenna ports field in DCI format 1_1 and add corresponding tables to reflect the agreements on DMRS; - Clause 7.3.1.2.3: Update Antenna ports field in DCI format 1_2 and add corresponding tables to reflect the agreements on DMRS; Enhanced uplink transmission - Clause 7.3.1.1.2: Update DCI format 0_1 to reflect agreements on 2 codeword for 8TX; - Clause 7.3.1.1.2 & 7.3.1.1.3: Update DCI format 0_1 and DCI format 0_2 to reflect agreements on CB and NCB PUSCH for 8Tx; - Clause 7.3.1.1.2 & 7.3.1.1.3: Update DCI format 0_1 and DCI format 0_2 to reflect agreements on STxMP PUSCH; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 MIMO Evolution for Downlink and Uplink will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.7, 6.3.2.1.2, 6.3.2.4, 7.3.1.1.2, 7.3.1.1.3, 7.3.1.2.2, 7.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CG	Configured grant

CG-DFI	CG downlink feedback information

CG-UCI	CG uplink control information

CORESET	Control resource set

COT	Channel occupancy time

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MBS	Multicast broadcast services

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSSCH	Physical sidelink shared channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SL-BCH	Sidelink broadcast channel

SL-SCH	Sidelink shared channel

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TCI                      Transmission configuration indicator

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

< Unchanged parts are omitted >

### 6.2.7	Data and control multiplexing

In case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with highest IMCS value for the initial PUSCH, where IMCS is as defined in Clause 6.1.4.1 in [6, TS 38.214]. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block. The PUSCH for UCI multiplexing in this Clause referes to the UL-SCH transport block for UCI multiplexing.

Denote the coded bits for UL-SCH as .

Denote the coded bits for HARQ-ACK or jointly coded bits for HARQ-ACK and CG-UCI when the high layer parameter cg-UCI-Multiplexing is configured, if any, as .

Denote the coded bits for CSI part 1, if any, as .

Denote the coded bits for CSI part 2, if any, as .

Denote the coded bits for CG-UCI without HARQ-ACK, if any, as g0CG-UCI,  g1CG-UCI,  g2CG-UCI, g3CG-UCI, …, gGCG-UCI-1CG-UCI.

Denote the multiplexed data and control coded bit sequence as .

Denote  as the OFDM symbol index of the scheduled PUSCH, starting from 0 to , where  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS.

Denote  as the subcarrier index of the scheduled PUSCH, starting from 0 to , where  is expressed as a number of subcarriers.

Denote  as the set of resource elements, in ascending order of indices , available for transmission of data in OFDM symbol , for .

Denote  as the number of elements in set . Denote  as the -th element in .

Denote  as the set of resource elements, in ascending order of indices , available for transmission of UCI in OFDM symbol , for . Denote  as the number of elements in set . Denote  as the -th element in . For any OFDM symbol that carriers DMRS of the PUSCH, . For any OFDM symbol that does not carry DMRS of the PUSCH, .

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK, let

-	GCG-UCI1=NL∙Qm∙GCG-UCI2∙NL∙Qm and GCG-UCI2=NL∙Qm∙GCG-UCI2∙NL∙Qm

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

-	if only CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

If frequency hopping is not configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS;

-	if HARQ-ACK is present for transmission on the PUSCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let ;

-	if CSI is present for transmission on the PUSCH, let  and ;

-	if CG-UCI is present for transmission on the PUSCH without HARQ-ACK, let GCG-UCI1=GCG-UCI;

-	let  and .

The multiplexed data and control coded bit sequence  is obtained according to the following:

Step 1:

Set  for ;

Set  for ;

Set  for ;

Set  for ;

if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits and without CG-UCI

the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Clause 6.3.2.4.2.1, by setting ;

denote  as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements;

if frequency hopping is configured for the PUSCH, let  and ;

if frequency hopping is not configured for the PUSCH, let ;

denote  as the set of reserved resource elements for potential HARQ-ACK transmission, in OFDM symbol , for ;

Set ;

Set ;

for ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

end for

end if

;

end while

end for

else

for ;

end if

Denote  as the number of elements in .

< Unchanged parts are omitted >

#### 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.2.1.2	CSI

If cqi-BitsPerSubband is configured, this Clause 6.3.2.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

The bitwidth for PMI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Clause 6.3.1.1.2.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Clause 6.3.1.1.2.

The bitwidth for PMI of codebookType=typeII is provided in Tables 6.3.2.1.2-1, where the values of , , , , , , and  are given by Clause 5.2.2.2.3 in [6, TS 38.214].

Table 6.3.2.1.2-1: PMI of codebookType= typeII

|  | Information fields for wideband PMI |  |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-r16 is provided in Tables 6.3.2.1.2-1A, where the values of (N1, N2), (O1, O2), L, KNZ, N3, and Mll=1,…, υ are given by Clause 5.2.2.2.5 in [6, TS 38.214].

Table 6.3.2.1.2-1A: PMI of codebookType= typeII-r16

|  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | i1,1 |  |  |  | i1,2 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
| Rank=1 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 | i1,5 |  | i1,6,1 |  | i1,6,2 | i1,6,3 |  | i1,6,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A | N/A |  | log2N3-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A | N/A |  | log2N3-1M2-1 |  | log2N3-1M2-1 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A | N/A |  | log2N3-1M3-1 |  | log2N3-1M3-1 | log2N3-1M3-1 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 | N/A |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | log2N3-1M4-1 |  | 3(KNZ-4) | 4(KNZ-4) |  | 8LM4 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A | log2(2M1) |  | log22M1-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | 2LM1 |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A | log2(2M2) |  | log22M2-1M2-1 |  | log22M2-1M2-1 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | 4LM2 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A | log2(2M3) |  | log22M3-1M3-1 |  | log22M3-1M3-1 | log22M3-1M3-1 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 6LM3 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 | log2(2M4) |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | log22M4-1M4-1 |  | 3(KNZ-4) | 4(KNZ-4) |  | 8LM4 |

Note:	the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ.

The bitwidth for PMI of codebookType=typeII-CJT is provided in Tables 6.3.2.1.2-1B, where the values of (N1, N2), (O1, O2), O3, KNZ, N3, N0, Ln, σ(n) and Mll=1,…, υ are given by Clause 5.2.2.2.8 in [6, TS 38.214].

Table 6.3.2.1.2-1B: PMI of codebookType= typeII-CJT

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,1 |  | i1,2 |  | i1,8,1 |  | i1,8,2 |  |  |  | i1,8,3 | i1,8,4 |
| Rank=1 N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2KNZ |  | N/A |  |  |  | N/A | N/A |
| Rank=2 N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |  | N/A | N/A |
| Rank=3 N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |  | log2(n=0N0-12Lσ(n)) | N/A |
| Rank=4 N3≤19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |
| Rank=1 N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2KNZ |  | N/A |  |  |  | N/A | N/A |
| Rank=2 N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |  | N/A | N/A |
| Rank=3 N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |  | log2(n=0N0-12Lσ(n)) | N/A |
| Rank=4 N3>19 | N0log2(O1O2) |  | n=0N0-1log2N1N2Lσ(n) |  | log2(n=0N0-12Lσ(n)) |  | log2(n=0N0-12Lσ(n)) |  |  |  | log2(n=0N0-12Lσ(n)) | log2(n=0N0-12Lσ(n)) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  |  | i1,5 | i1,6,1 |  | i1,6,2 |
| Rank=1 N3≤19 | 4 | N/A |  | N/A |  | N/A |  |  | N/A | log2N3-1M1-1 |  | N/A |
| Rank=2 N3≤19 | 4 | 4 |  | N/A |  | N/A |  |  | N/A | log2N3-1M2-1 |  | log2N3-1M2-1 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 |  | N/A |  |  | N/A | log2N3-1M3-1 |  | log2N3-1M3-1 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 |  | 4 |  |  | N/A | log2N3-1M4-1 |  | log2N3-1M4-1 |
| Rank=1 N3>19 | 4 | N/A |  | N/A |  | N/A |  |  | log2(2M1) | log22M1-1M1-1 |  | N/A |
| Rank=2 N3>19 | 4 | 4 |  | N/A |  | N/A |  |  | log2(2M2) | log22M2-1M2-1 |  | log22M2-1M2-1 |
| Rank=3 N3>19 | 4 | 4 |  | 4 |  | N/A |  |  | log2(2M3) | log22M3-1M3-1 |  | log22M3-1M3-1 |
| Rank=4 N3>19 | 4 | 4 |  | 4 |  | 4 |  |  | log2(2M4) | log22M4-1M4-1 |  | log22M4-1M4-1 |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i1,6,3 |  |  | i1,6,4 | i1,9 |  |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-1) |  | 4(KNZ-1) |  | 2∙n=0N0-1Lσ(n)M1 |
| Rank=2 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-2) |  | 4(KNZ-2) |  | 4∙n=0N0-1Lσ(n)M2 |
| Rank=3 N3≤19 | log2N3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-3) |  | 4(KNZ-3) |  | 6∙n=0N0-1Lσ(n)M3 |
| Rank=4 N3≤19 | log2N3-1M4-1 |  |  | log2N3-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-4) |  | 4(KNZ-4) |  | 8∙n=0N0-1Lσ(n)M4 |
| Rank=1 N3>19 | N/A |  |  | N/A | N0-1log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-1) |  | 4(KNZ-1) |  | 2∙n=0N0-1Lσ(n)M1 |
| Rank=2 N3>19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-2) |  | 4(KNZ-2) |  | 4∙n=0N0-1Lσ(n)M2 |
| Rank=3 N3>19 | log22M3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-3) |  | 4(KNZ-3) |  | 6∙n=0N0-1Lσ(n)M3 |
| Rank=4 N3>19 | log22M4-1M4-1 |  |  | log22M4-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  |  | 3(KNZ-4) |  | 4(KNZ-4) |  | 8∙n=0N0-1Lσ(n)M4 |
| Note: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2n=0N0-1Lσ(n)Mυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.8 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler is provided in Tables 6.3.2.1.2-1C, where the values of (N1, N2), (O1, O2), L, KNZ, N3, N4, Q and Mll=1,…, υ are given by Clause 5.2.2.2.10 in [6, TS 38.214].

Table 6.3.2.1.2-1C: PMI of codebookType=typeII-Doppler

|  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i1,1 |  |  |  | i1,2 |  |  |  | i1,8,1 |  |  | i1,8,2 |  |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
| Rank=1 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | i1,5 |  | i1,6,1 |  |  | i1,6,2 |  |  | i1,6,3 |  |  | i1,6,4 |  |  |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A |  | N/A |  | log2N3-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A |  | N/A |  | log2N3-1M2-1 |  |  | log2N3-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A |  | N/A |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 |  | N/A |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A |  | log2(2M1) |  | log22M1-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A |  | log2(2M2) |  | log22M2-1M2-1 |  |  | log22M2-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A |  | log2(2M3) |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 |  | log2(2M4) |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i1,10,1 |  |  | i1,10,2 |  |  |  | i1,10,3 |  |  | i1,10,4 |  |  | {i2,4,l}l=1,…,υ |  |  | {i2,5,l}l=1,…,υ |  |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| Rank=1 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| Note: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυQ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,τ(3), kl,i,f,τ(2), and cl,i,f,τ, respectively), where KlNZ as defined in Clause 5.2.2.2.10 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType= typeII-PortSelection is provided in Tables 6.3.2.1.2-2, where the values of , , , , , , and  are given by Clause 5.2.2.2.4 in [6, TS 38.214].

Table 6.3.2.1.2-2: PMI of codebookType= typeII-PortSelection

|  | Information fields for wideband PMI |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r16 is provided in Tables 6.3.2.1.2-2A, where the values of PCSI-RS, d, L, KNZ, N3, and  Mll=1,…, υ are given by Clause 5.2.2.2.6 in [6, TS 38.214].

Table 6.3.2.1.2-2A: PMI of codebookType= typeII-PortSelection-r16

|  |  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | i1,1 |  |  |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |  |  |
| Rank=1 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
| Rank=1 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 | i1,5 | i1,6,1 |  | i1,6,2 |  | i1,6,3 | i1,6,4 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A | N/A | log2N3-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A | N/A | log2N3-1M2-1 |  | log2N3-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A | N/A | log2N3-1M3-1 |  | log2N3-1M3-1 |  | log2N3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 | N/A | log2N3-1M4-1 |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A | log2(2M1) | log22M1-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A | log2(2M2) | log22M2-1M2-1 |  | log22M2-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A | log2(2M3) | log22M3-1M3-1 |  | log22M3-1M3-1 |  | log22M3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 | log2(2M4) | log22M4-1M4-1 |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |

Note:	the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ.

The bitwidth for PMI of codebookType=typeII-PortSelection-r17 is provided in Tables 6.3.2.1.2-2B, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.7 in [6, TS 38.214].

Table 6.3.2.1.2-2B: PMI of codebookType= typeII-PortSelection-r17

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |

Note:	the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.7 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ.

The bitwidth for PMI of codebookType=typeII-CJT-PortSelection is provided in Tables 6.3.2.1.2-2C, where the values of PCSI-RS, K1,n, KNZ, N3, N, M and σn are given by Clause 5.2.2.2.9 in [6, TS 38.214].

Table 6.3.2.1.2-2C: PMI of codebookType= typeII-CJT-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  |  | i1,6 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=0N0-1K1,σ(n)M |  |  | N/A |  | N/A |  | N/A |
| Rank=2 | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=0N0-1K1,σ(n)M |  |  | log2n=0N0-1K1,σ(n)M |  | N/A |  | N/A |
| Rank=3 | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=0N0-1K1,σ(n)M |  |  | log2n=0N0-1K1,σ(n)M |  | log2n=0N0-1K1,σ(n)M |  | N/A |
| Rank=4 | n=0N0-1log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=0N0-1K1,σ(n)M |  |  | log2n=0N0-1K1,σ(n)M |  | log2n=0N0-1K1,σ(n)M |  | log2n=0N0-1K1,σ(n)M |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | i1,9 |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=n=0N0-1K1,σ(n)M; n=0N0-1K1,σ(n)M otherwise |  |
| Rank=2 | 4 | 4 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=2n=0N0-1K1,σ(n)M; 2n=0N0-1K1,σ(n)M otherwise |  |
| Rank=3 | 4 | 4 | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3∙n=0N0-1K1,σ(n)M |  |
| Rank=4 | 4 | 4 | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 4∙n=0N0-1K1,σ(n)M |  |
| Note: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are n=0N0-1K1,σ(n)M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.9 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler-PortSelection is provided in Tables 6.3.2.1.2-2D, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.11 in [6, TS 38.214].

Table 6.3.2.1.2-2D: PMI of codebookType= typeII-Doppler-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| Note: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2D is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.11 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

For CSI on PUSCH, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence  starting with . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence  starting with .

The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8A. The mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8B. The mapping order of CSI fields of one report for TDCP reporting is provided in Table 6.3.2.1.2-3C. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, CRI/SINR, SSBRI/SINR reporting, or TDCP reporting.

Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4/6, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
|  | Indicator of the N0 selected CSI-RS resources by a bitmap with NTRP bits, this field is present only if NTRP>1 and restrictedCMR-Selection is configured to OFF |
|  | Indicator of selected Ln value combination or αn value combination with bitwidth of log2(NL), this field is present only if NL>1 |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ as in Table 6.3.2.1.2-8/8A/8B/9/9A, if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3C: Mapping order of CSI fields of one CSI report for reportQuantity=tdcp

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Amplitude value for the configured delay values as in Table 6.3.2.1.2-10, if reported; |
|  | Phase value for the configured delay values as in Table 6.3.2.1.2-10, if reported; |

< Unchanged parts are omitted >

Table 6.3.2.1.2-5B: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-2B, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*K1M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-2B: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS38.214, if reported |

< Unchanged parts are omitted >

Table 6.3.2.1.2-5D: Mapping order of CSI fields of one CSI report, CSI part 2 subband, ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.2.1.2-5E: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-CJT

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1B, if reported; |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,9, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2∙n=0N0-1Lσ(n)Mυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if present and if reported; |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if reported; |

Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType= typeII-Doppler

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.1.1.2-8B, if present and reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q defined in clause 5.2.3 of TS38.214, if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f, q defined in clause 5.2.3 of TS38.214, if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.1.1.2-8B, if present and if reported |

Table 6.3.2.1.2-5G: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-CJT-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2C, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ}, i1,9 and ν*n=0N-1K1,nM-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if reported |

Table 6.3.2.1.2-6: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI part 1 of CSI report #1 as in Table 6.3.2.1.2-3/3A/3B or Table 6.3.1.1.2-8/8A/8B or Table 6.3.2.1.2-3C |
|  | CSI part 1 of CSI report #2 as in Table 6.3.2.1.2-3/3A/3B or Table 6.3.1.1.2-8/8A/8B or Table 6.3.2.1.2-3C |
|  | … |
|  | CSI part 1 of CSI report #n as in Table 6.3.2.1.2-3/3A/3B or Table 6.3.1.1.2-8/8A/8B or Table 6.3.2.1.2-3C |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-6 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.2.1.2-7: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G, if CSI part 2 exists for CSI report #n |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-7 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

The bitwidth for RI/CQI of codebookType= typeII-r16 or codebookType=typeII-PortSelection-r16 is provided in Table 6.3.2.1.2-8.

Table 6.3.2.1.2-8: RI and CQI of codebookType=typeII-r16 or typeII-PortSelection-r16

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5 and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType= typeII-CJT is provided in Table 6.3.2.1.2-8A.

Table 6.3.2.1.2-8A: RI and CQI of codebookType= typeII-CJT

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers, and all CSI-RS resources if configured, KNZ | log22βM1Lmax if max allowed rank is 1; log222βM1Lmaxotherwise |
| where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.8 [6, TS 38.214], Lmax is the maximum of n=1NTRPLn for all NL configured Ln combinations, where Lnn=1,…, NTRP, M1, and β are given by Clause 5.2.2.2.8 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.8 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

The bitwidth for RI/CQI of codebookType= typeII-Doppler is provided in Table 6.3.2.1.2-8B.

Table 6.3.2.1.2-8B: RI and CQI of codebookType= typeII-Doppler

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| The second time-domain wide-band CQI | 4 |
| The second time-domain subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |
| where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.10 [6, TS 38.214], K0=2Lp1×N3RβQ, where p1, N3, R, β and Q are given by Clauses 5.2.2.2.10 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.10 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

The bitwidth for RI/CQI of codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection is provided in Table 6.3.2.1.2-9.

Table 6.3.2.1.2-9: RI and CQI of codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection-r18

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |

where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.7 and Clauses 5.2.2.2.11 [6, TS 38.214], K0=K1Mβ, where K1, M, and β are given by Clause 5.2.2.2.7 and Clauses 5.2.2.2.11 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.7 and Clauses 5.2.2.2.11 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1.

The bitwidth for RI/CQI of codebookType=typeII-CJT-PortSelection is provided in Table 6.3.2.1.2-9A.

Table 6.3.2.1.2-9A: RI and CQI of codebookType= typeII-CJT-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers, and all CSI-RS resources if configured, KNZ | log2βMK1,max if max allowed rank is 1; log22βMK1,max otherwise |
| where nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.9 [6, TS38.214], K1,max is the maximum of n=1NTRPK1, n for all NL configured αn combinations, where K1.nn=1,…, NTRP, M, and β are given by Clause 5.2.2.2.9 in [6, TS 38.214], and. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.9 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

The bitwidth for reportQuantity=tdcp is provided in Table 6.3.2.1.2-10.

Table 6.3.2.1.2-10: Amplitude and phase values for reportQuantity=tdcp

| Field | Bitwidth |
| --- | --- |
| Amplitude value | 4 |
| Phase value | 4 |

< Unchanged parts are omitted >

#### 6.3.2.4	Rate matching

In case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with highest IMCS value for the initial PUSCH, where IMCS is as defined in Clause 6.1.4.1 in [6, TS 38.214]. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block. The PUSCH for UCI multiplexing in this Clause referes to the UL-SCH transport block for UCI multiplexing.

##### 6.3.2.4.1	UCI encoded by Polar code

###### 6.3.2.4.1.1	HARQ-ACK

For HARQ-ACK transmission on PUSCH not using repetition type B with UL-SCH and if numberOfSlotsTBoMS is not present in the resource allocation table, or if numberOfSlotsTBoMS is present in the resource allocation table and the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is equal to 1, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows:

where

-	 is the number of HARQ-ACK bits;

-	if , ; otherwise  is the number of CRC bits for HARQ-ACK determined according to Clause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise,  is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling;

-	 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission.

< Unchanged parts are omitted >

#### 7.3.1.1	DCI formats for scheduling of PUSCH

< Unchanged parts are omitted >

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured, or if the higher layer parameter frequencyHopping is not configured and the higher layer parameter pusch-RepTypeIndicatorDCI-0-1 is not configured to pusch-RepTypeB, or if the higher layer parameter frequencyHoppingDCI-0-1 is not configured and pusch-RepTypeIndicatorDCI-0-1 is configured to pusch-RepTypeB, or if only resource allocation type 2 is configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version – – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2 (only present if maxRank > 4 or maxMIMO-Layers > 4):

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-0-1 is configured; otherwise 4 bits

-	1st downlink assignment index – 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook for unicast and multicast if pdsch-HARQ-ACK-Codebook = semiStatic is configured for both unicast and multicast and the higher layer parameter fdmed-ReceptionMulticast is not configured; otherwise for semi-static HARQ-ACK codebook for unicast;

-	2 bits for dynamic HARQ-ACK codebook for unicast, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included = true.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st  downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	2nd downlink assignment index – 0, 2 or 4 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks for unicast, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included = true;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	3rd downlink assignment index – 0, 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook for multicast if the higher layer parameter fdmed-ReceptionMulticast is configured;

-	2 bits for the dynamic HARQ-ACK codebook for multicast;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 3rd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 3rd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 3rd downlink assignment index until the bit width of the 3rd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	Second TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213] if higher layer parameter SecondTPCFieldDCI-0-1 is configured; 0 bit otherwise.

-	SRS resource set indicator – 0 or 2 bits

-	2 bits according to Table 7.3.1.1.2-36 if

-	txConfig = nonCodeBook, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with the usage of value 'nonCodeBook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, or

-	txConfig=codebook, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided;

-	0 bit otherwise.

-	SRS resource indicator – NSRSnumber of bits determined by the following:

-	 bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B/31C/31D/31E/31F if the higher layer parameter txConfig = nonCodebook, where

-	 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI,

-	otherwise NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList and associated with the higher layer parameter usage of value 'nonCodeBook',

and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSdm} if maxMIMO-LayersforSdm is configured

-	Lmax is given by max{maxMIMO-Layers, maxMIMO-LayersforSfn} if maxMIMO-LayersforSfn is configured

-	Lmax is given by maxMIMO-Layers otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook, where

-	 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI,

-	otherwise NSRS is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList and associated with the higher layer parameter usage of value 'codeBook'.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI,  and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook' or 'nonCodeBook', the first SRS resource set is associated with coresetPoolIndex value 0 and the second SRS resource set is associated with coresetPoolIndex value 1, where the first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets.

-	Second SRS resource indicator – number of bits determined by the following:log2(maxk∈{1,2,…,min⁡{Lmax,NSRS}}NSRSk)log2(NSRS)

-	log2(maxk∈{1,2,…,min⁡{Lmax,NSRS}}NSRSk) bits according to Tables 7.3.1.1.2-28/29A/30A/31A with the same number of layers indicated by SRS resource indicator field if the higher layer parameter txConfig = nonCodebook, the higher layer paramtere maxMIMO-LayersforSdm is not configured, and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set, and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by maxMIMO-LayersforSfn if maxMIMO-LayersforSfn is configured

-	Lmax is given by maxMIMO-Layers otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-28/29 if the higher layer parameter txConfig = nonCodebook, the higher layer paramtere maxMIMO-LayersforSdm is configured and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set, and Lmax is given by maxMIMO-LayersforSdm.

-	log2(NSRS) bits according to Tables 7.3.1.1.2-32, 7.3.1.1.2-32A and 7.3.1.1.2-32B if the higher layer parameter txConfig = codebook and SRS resource set indicator field is present, where NSRS is the number of configured SRS resources in the second SRS resource set.

-	0 bit otherwise.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or max{maxRank, maxRankSfn} if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} if multipanelScheme = sdmScheme, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 2 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or max{maxRank, maxRankSfn} if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 2 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 2 if multipanelScheme = sdmScheme, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn}=1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm}=1 if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or max{maxRank, maxRankSfn} = 1 if multipanelScheme = sfnScheme or max{maxRank, maxRankSdm} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	7 bits according to Table 7.3.1.1.2-5B for 8 antenna ports, if CodebookType=Codebook1, transform precoder is disabled, maxRank = 8, and according to ULcodebookFC-N1N2;

-	7 bits according to Table 7.3.1.1.2-5C for 8 antenna ports, if CodebookType=Codebook1, transform precoder is disabled, maxRank =7, and according to ULcodebookFC-N1N2;

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports, if CodebookType=Codebook1, transform precoder is disabled, maxRank =4, 5 or 6, and according to maxRank;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports, if CodebookType=Codebook1, transform precoder is enabled or maxRank =1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRank;

-	8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=5, 6, 7 or 8, and according to maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookType=Codebook2, transform precoder is disabled, maxRank=5, 6, 7 or 8, and according to maxRank;

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookType=Codebook3, transform precoder is disabled, maxRank=5, 6, 7 or 8, and according to maxRank;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRank=2 or 3, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRank =1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1;

-	5, 7 or 9 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRank =1, 2, or 3 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1.

-	Second Precoding information – number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if multipanelScheme is not configured or maxRankSfn=2 if multipanelScheme = sfnScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or maxRankSfn=1 if multipanelScheme = sfnScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if multipanelScheme is not configured or maxRankSfn=2 if multipanelScheme = sfnScheme, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if multipanelScheme is not configured or maxRankSfn if multipanelScheme = sfnScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if multipanelScheme is not configured or maxRankSfn=1 if multipanelScheme = sfnScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSdm if multipanelScheme = sdmScheme, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSdm = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankSdm if multipanelScheme = sdmScheme, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSdm = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSdm if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRankSdm = 2 if multipanelScheme = sdmScheme, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankSdm= 1 if multipanelScheme = sdmScheme, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSdm = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by Tables 7.3.1.1.2-6A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in [Clause 6.4.1.1.1.2, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in [Clause 6.4.1.1.1.2, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/10A/11 according to the value of rank, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/14A/15/15A/15B/15C/15D according to the value of rank, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, and maxLength=2;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/18A/19/19A/19B according to the value of rank, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/22A/23/23A/23B/23C/23D according to the value of rank, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, and maxLength=2.

-	4 bits as defined by Tables 7.3.1.1.2-38/39/40/40A/41/42/43/44/45, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-46/47/48/48A/49/50/51/52/53, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, and maxLength=2;

-	5 bits as defined by Tables 7.3.1.1.2-54/55/56/56A/57/58/59/60/61, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, and maxLength=1;

-	6 bits as defined by Tables 7.3.1.1.2-62/63/64/64A/65/66/67/68/69, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, and maxLength=2.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively, and the value of rank is

-	the sum of the value determined according to the SRS resource indicator field and the value determined according to the second SRS resource indicator field, if txConfig = nonCodebook, multipanelScheme = sdmScheme and SRS resource set indicator field equals "10"

-	the sum of the value determined according to the Precoding information and number of layers field and the value determined according to the Second Precoding information, if txConfig = codebook, multipanelScheme = sdmScheme and SRS resource set indicator field equals "10"

-	determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and multipanelScheme = sdmScheme is not configured, or if the higher layer parameter txConfig = nonCodebook, multipanelScheme = sdmScheme and SRS resource set indicator field equals "00" or “01”,

-	determined according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook and multipanelScheme = sdmScheme is not configured, or if the higher layer parameter txConfig = codebook, multipanelScheme = sdmScheme and SRS resource set indicator field equals "00" or "01".

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured or if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; otherwise, 2, 4, 6, or 8 bits as defined in Clause 6.1.5 of [6, TS38.214], determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock and maxRank or maxMIMO-Layers for PUSCH.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1 and multipanelScheme is not configured, or if maxRank=1 and maxRankSfn=1, or if maxRank=1 and maxRankSdm=1 when two PTRS ports are configured by maxNrofPortsforSdm;

-	2 or 4 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26A are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or “01” and maxRank<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" or “11”, maxRank=3 or 4 and multipanelScheme is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and "11", maxRank=2 and multipanelScheme is not configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when two PTRS ports are configured by maxNrofPortsforSDM in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelScheme is configured to sdmScheme, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when one PTRS port is configured by maxNrofPortsforSDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelScheme is configured to sdmScheme, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelScheme is configured to sfnScheme, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 and multipanelScheme is not configured, this field indicates the association between PTRS port and DMRS port(s) corresponding to the selected codeword according to Table 7.3.1.1.2-25B, where the selected codeword is the codeword with higher MCS for the initial PUSCH if the MCS indices of the two codewords are different for the initial PUSCH, or codeword 0 otherwise.

-	4 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 and multipanelScheme is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-26A.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2 and multipanelScheme is not configured; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList or by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the beta_offset indicator in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

-	UL-SCH indicator – 0 or 1 bit as follows

-	0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1;

-	1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s).

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

-	Open-loop power control parameter set indication – 0 or 1 or 2 bits.

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator is present in the DCI format 0_1;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator is not present in the DCI format 0_1.

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-0-1 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Invalid symbol pattern indicator – 0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-1 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214].

-	Minimum applicable scheduling offset indicator – 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK2 is not configured;

-	1 bit if higher layer parameter minimumSchedulingOffsetK2 is configured. The 1 bit indication is used to determine the minimum applicable K2 for the active UL BWP and the minimum applicable K0 value for the active DL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP shall be the same as the minimum applicable K0 value.

-	SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

-	Sidelink assignment index – 0, 1 or 2 bits:

-	1 bit if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static and, in addition,  the UE is configured with a SL configured grant type 1 or to monitor DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI;

-	2 bits if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic and, in addition, the UE is configured with a SL configured grant type 1 or to monitor DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI;

-	0 bit otherwise.

-	PDCCH monitoring adaptation indication – 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.

If the number of information bits in DCI format 0_1 scheduling a single PUSCH prior to padding is not equal to the number of information bits in DCI format 0_1 scheduling multiple PUSCHs for the same serving cell, zeros shall be appended to the DCI format 0_1 with smaller size until the payload size is the same for scheduling a single PUSCH and multiple PUSCHs.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 0_1 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 0_1 with smaller size until the payload size is the same.

-	If application of step 4C in clause 7.3.1.0 results in additional zero padding for DCI format 0_1 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 0_1 monitored on the primary cell and DCI format 0_1 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | Configured BWP with BWP-Id = 1 |
| 01 | Configured BWP with BWP-Id = 2 |
| 10 | Configured BWP with BWP-Id = 3 |
| 11 | Configured BWP with BWP-Id = 4 |

Table 7.3.1.1.2-2: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4 or max{maxRank, maxRankSfn} = 2 or 3 or 4 or max{maxRank, maxRankSdm} = 2 or 3 or 4 or maxRankSdm= 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-2A: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or max{maxRank, maxRankSfn} = 2 or max{maxRank, maxRankSdm} = 2 or maxRankSdm= 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 1 layer: TPMI=13 | 10 | 1 layer: TPMI=13 |
| 11 | 2 layer: TPMI=6 | 11 | 2 layer: TPMI=6 |
| 12 | 1 layer: TPMI=4 | 12-15 | Reserved |
| … | … |  |  |
| 20 | 1 layer: TPMI=12 |  |  |
| 21 | 1 layer: TPMI=14 |  |  |
| 22 | 1 layer: TPMI=15 |  |  |
| 23 | 2 layers: TPMI=7 |  |  |
| … | … |  |  |
| 29 | 2 layers: TPMI=13 |  |  |
| 30-31 | Reserved |  |  |

Table 7.3.1.1.2-2B: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=13 | 12 | 1 layer: TPMI=13 |
| 13 | 2 layer: TPMI=6 | 13 | 2 layer: TPMI=6 |
| 14 | 3 layer: TPMI=1 | 14 | 3 layer: TPMI=1 |
| 15 | 1 layer: TPMI=4 | 15 | Reserved |
| … | … |  |  |
| 23 | 1 layer: TPMI=12 |  |  |
| 24 | 1 layer: TPMI=14 |  |  |
| 25 | 1 layer: TPMI=15 |  |  |
| 26 | 2 layers: TPMI=7 |  |  |
| … | … |  |  |
| 32 | 2 layers: TPMI=13 |  |  |
| 33 | 3 layers: TPMI=2 |  |  |
| 34 | 4 layers: TPMI=1 |  |  |
| 35 | 4 layers: TPMI=2 |  |  |
| 36-63 | Reserved |  |  |

Table 7.3.1.1.2-2C: Second precoding information, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4 or maxRankSfn = 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 27 | 1 layer: TPMI=27 | 11 | 1 layer: TPMI=11 | 3 | 1 layer: TPMI=3 |
| 28-31 | 1 layer: reserved | 12-15 | 1 layer: reserved | 4-7 | 1 layer: reserved |
| 0 | 2 layers: TPMI=0 | 0 | 2 layers: TPMI=0 | 0 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 21 | 2 layers: TPMI=21 | 13 | 2 layers: TPMI=13 | 5 | 2 layers: TPMI=5 |
| 22-31 | 2 layers: reserved | 14-15 | 2 layers: reserved | 6-7 | 2 layers: reserved |
| 0 | 3 layers: TPMI=0 | 0 | 3 layers: TPMI=0 | 0 | 3 layers: TPMI=0 |
| … | … | … | … | 1-7 | 3 layers: reserved |
| 6 | 3 layers: TPMI=6 | 2 | 3 layers: TPMI=2 | 0 | 4 layers: TPMI=0 |
| 7-31 | 3 layers: reserved | 3-15 | 3 layers: reserved | 1-7 | 4 layers: reserved |
| 0 | 4 layers: TPMI=0 | 0 | 4 layers: TPMI=0 |  |  |
| … | … | … | … |  |  |
| 4 | 4 layers: TPMI=4 | 2 | 4 layers: TPMI=2 |  |  |
| 5-31 | 4 layers: reserved | 3-15 | 4 layers: reserved |  |  |

Table 7.3.1.1.2-2D: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or maxRankSfn = 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … |
| 14 | 1 layer: TPMI=14 | 3 | 1 layer: TPMI=3 |
| 15 | 1 layer: TPMI=15 | 4 | 1 layer: TPMI=13 |
| 0 | 2 layers: TPMI=0 | 5-7 | 1 layer: reserved |
| … | … | 0 | 2 layers: TPMI=0 |
| 13 | 2 layers: TPMI=13 | … | … |
| 14-15 | 2 layers: reserved | 6 | 2 layers: TPMI=6 |
|  |  | 7 | 2 layers: reserved |

Table 7.3.1.1.2-2E: Second precoding information for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … |
| 14 | 1 layer: TPMI=14 | 3 | 1 layer: TPMI=3 |
| 15 | 1 layer: TPMI=15 | 4 | 1 layer: TPMI=13 |
| 0 | 2 layers: TPMI=0 | 5-7 | 1 layer: reserved |
| … | … | 0 | 2 layers: TPMI=0 |
| 13 | 2 layers: TPMI=13 | … | … |
| 14-15 | 2 layers: reserved | 6 | 2 layers: TPMI=6 |
| 0 | 3 layers: TPMI=0 | 7 | 2 layers: reserved |
| … | … | 0 | 3 layers: TPMI=0 |
| 2 | 3 layers: TPMI=2 | 1 | 3 layer: TPMI=1 |
| 3-15 | 3 layers: reserved | 2-7 | 3 layers: reserved |
| 0 | 4 layers: TPMI=0 | 0 | 4 layers: TPMI=0 |
| … | … | 1-7 | 4 layers: reserved |
| 2 | 4 layers: TPMI=2 |  |  |
| 3-15 | 4 layers: reserved |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission is either not configured or configured to fullpowerMode2 or configured to fullpower, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSfn} = 1 or max{maxRank, maxRankSdm} = 1 or maxRankSdm= 1 or maxRankSfn= 1, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-3A: Precoding information and number of layers or Second Precoding information, for 4 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission = fullpowerMode1, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSfn} = 1 or max{maxRank, maxRankSdm} = 1 or maxRankSdm= 1 or maxRankSfn= 1, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=13 | 4 | 1 layer: TPMI=13 |
| 5 | 1 layer: TPMI=4 | 5-7 | Reserved |
| … | … |  |  |
| 13 | 1 layer: TPMI=12 |  |  |
| 14 | 1 layer: TPMI=14 |  |  |
| 15 | 1 layer: TPMI=15 |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers or Second Precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or max{maxRank, maxRankSfn} = 2 or max{maxRank, maxRankSdm} = 2 or maxRankSdm= 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-4A: Precoding information and number of layers or Second Precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or max{maxRank, maxRankSfn} = 2 or max{maxRank, maxRankSdm} = 2 or maxRankSdm= 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 |

Table 7.3.1.1.2-4B: Second precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or maxRankSfn = 2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | 0 | 2 layers: TPMI=0 |
| 5 | 1 layer: TPMI=5 | 1 | 2 layers: reserved |
| 6-7 | 1 layer: reserved |  |  |
| 0 | 2 layers: TPMI=0 |  |  |
| … | … |  |  |
| 2 | 2 layers: TPMI=2 |  |  |
| 3-7 | 2 layers: reserved |  |  |

Table 7.3.1.1.2-4C: Second precoding information, for 2 antenna ports, if transform precoder is disabled, maxRank = 2 or maxRankSfn = 2, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: reserved |
| 0 | 2 layers: TPMI=0 |
| 1-3 | 2 layers: reserved |

Table 7.3.1.1.2-5: Precoding information and number of layers or Second Precoding information, for 2 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSfn} = 1 or max{maxRank, maxRankSdm} = 1 or maxRankSdm= 1 or maxRankSfn= 1, and and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

Table 7.3.1.1.2-5A: Precoding information and number of layers or Second Precoding information, for 2 antenna ports or Second Precoding information, if transform precoder is enabled and ul-FullPowerTransmission = fullpowerMode1, or if transform precoder is disabled, maxRank = 1 or max{maxRank, maxRankSfn} = 1 or max{maxRank, maxRankSdm} = 1 or maxRankSdm= 1 or maxRankSfn= 1, and ul-FullPowerTransmission = fullpowerMode1

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | Reserved |

Table 7.3.1.1.2-5B: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 8, and CodebookType=Codebook1

| Bit field mapped to index | ULcodebookFC-N1N2 = (4, 1) | Bit field mapped to index | ULcodebookFC-N1N2 = (2, 2) |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layer2: TPMI=0 |
| 17 | 2 layers: TPMI=1 | 17 | 2 layer2: TPMI=1 |
| … | … | … | … |
| 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
| 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 |
| 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 |
| … | … | … | … |
| 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 |
| 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 |
| 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 |
| … | … | … | … |
| 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 |
| 96 | 5 layers: TPMI=0 | 96 | 5 layers: TPMI=0 |
| 97 | 5 layers: TPMI=1 | 97 | 5 layers: TPMI=1 |
| … | … | … | … |
| 103 | 5 layers: TPMI=7 | 103 | 5 layers: TPMI=7 |
| 104 | 6 layers: TPMI=0 | 104 | 6 layers: TPMI=0 |
| 105 | 6 layers: TPMI=1 | 105 | 6 layers: TPMI=1 |
| … | … | … | … |
| 111 | 6 layers: TPMI=7 | 111 | 6 layers: TPMI=7 |
| 112 | 7 layers: TPMI=0 | 112 | 7 layers: TPMI=0 |
| 113 | 7 layers: TPMI=1 | 113 | 7 layers: TPMI=1 |
| … | … | … | … |
| 115 | 7 layers: TPMI=3 | 119 | 7 layers: TPMI=7 |
| 116 | 8 layers: TPMI=0 | 120 | 8 layers: TPMI=0 |
| 117 | 8 layers: TPMI=1 | 121 | 8 layers: TPMI=1 |
| … | … | … | … |
| 119 | 8 layers: TPMI=3 | 127 | 8 layers: TPMI=7 |
| 120-127 | reserved |  |  |

Table 7.3.1.1.2-5C: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 7, and CodebookType=Codebook1

| Bit field mapped to index | ULcodebookFC-N1N2= (4, 1) | Bit field mapped to index | ULcodebookFC-N1N2= (2, 2) |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layer2: TPMI=0 |
| 17 | 2 layers: TPMI=1 | 17 | 2 layer2: TPMI=1 |
| … | … | … | … |
| 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
| 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 |
| 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 |
| … | … | … | … |
| 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 |
| 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 |
| 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 |
| … | … | … | … |
| 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 |
| 96 | 5 layers: TPMI=0 | 96 | 5 layers: TPMI=0 |
| 97 | 5 layers: TPMI=1 | 97 | 5 layers: TPMI=1 |
| … | … | … | … |
| 103 | 5 layers: TPMI=7 | 103 | 5 layers: TPMI=7 |
| 104 | 6 layers: TPMI=0 | 104 | 6 layers: TPMI=0 |
| 105 | 6 layers: TPMI=1 | 105 | 6 layers: TPMI=1 |
| … | … | … | … |
| 111 | 6 layers: TPMI=7 | 111 | 6 layers: TPMI=7 |
| 112 | 7 layers: TPMI=0 | 112 | 7 layers: TPMI=0 |
| 113 | 7 layers: TPMI=1 | 113 | 7 layers: TPMI=1 |
| … | … | … | … |
| 115 | 7 layers: TPMI=3 | 119 | 7 layers: TPMI=7 |
| 116-127 | reserved | 120-127 | reserved |

Table 7.3.1.1.2-5D: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 4, 5 or 6, CodebookType=Codebook1, ULcodebookFC-N1N2 = (4,1) or (2,2)

| Bit field mapped to index | maxRank=4 | Bit field mapped to index | maxRank=5 | Bit field mapped to index | maxRank=6 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layer2: TPMI=0 |
| 17 | 2 layers: TPMI=1 | 17 | 2 layers: TPMI=1 | 17 | 2 layer2: TPMI=1 |
| … | … | … | … | … | … |
| 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
| 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 | 48 | 3 layers: TPMI=0 |
| 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 | 49 | 3 layers: TPMI=1 |
| … | … | … | … | … | … |
| 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 | 71 | 3 layers: TPMI=23 |
| 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 | 72 | 4 layers: TPMI=0 |
| 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 | 73 | 4 layers: TPMI=1 |
| … | … | … | … | … | … |
| 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 | 95 | 4 layers: TPMI=23 |
| 96-127 | reserved | 96 | 5 layers: TPMI=0 | 96 | 5 layers: TPMI=0 |
|  |  | 97 | 5 layers: TPMI=1 | 97 | 5 layers: TPMI=1 |
|  |  | … | … | … | … |
|  |  | 103 | 5 layers: TPMI=7 | 103 | 5 layers: TPMI=7 |
|  |  | 104-127 | reserved | 104 | 6 layers: TPMI=0 |
|  |  |  |  | 105 | 6 layers: TPMI=1 |
|  |  |  |  | … | … |
|  |  |  |  | 111 | 6 layers: TPMI=7 |
|  |  |  |  | 112-127 | reserved |

Table 7.3.1.1.2-5E: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 or 2 or 3 if transform precoder is disabled, CodebookType=Codebook1, ULcodebookFC-N1N2 = (4,1) or (2,2)

| Bit field mapped to index | transform precoder is disabled, or maxRank=1 if transform precoder is enabled | Bit field mapped to index | transform precoder is disabled, and maxRank=2 | Bit field mapped to index | transform precoder is disabled, and maxRank=3 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
|  |  | 16 | 2 layer2: TPMI=0 | 16 | 2 layer2: TPMI=0 |
|  |  | 17 | 2 layer2: TPMI=1 | 17 | 2 layer2: TPMI=1 |
|  |  | … | … | … | … |
|  |  | 47 | 2 layers: TPMI=31 | 47 | 2 layers: TPMI=31 |
|  |  | 48-63 | reserved | 48 | 3 layers: TPMI=0 |
|  |  |  |  | 49 | 3 layers: TPMI=1 |
|  |  |  |  | … | … |
|  |  |  |  | 71 | 3 layers: TPMI=23 |
|  |  |  |  | 72-127 | reserved |

Table 7.3.1.1.2-5F: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook4

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
| … | … | … | … | … | … | … | … |
| 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
| 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 | 92 | 4 layers: TPMI=92 |
| … | … | … | … | … | … | … | … |
| 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 | 161 | 4 layers: TPMI=161 |
| 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 | 162 | 5 layers: TPMI=162 |
| … | … | … | … | … | … | … | … |
| 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 | 217 | 5 layers: TPMI=217 |
| 218-255 | reserved | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 | 218 | 6 layers: TPMI=218 |
|  |  | … | … | … | … | … | … |
|  |  | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 | 245 | 6 layers: TPMI=245 |
|  |  | 246-255 | reserved | 246 | 7 layers: TPMI=246 | 246 | 7 layers: TPMI=246 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 253 | 7 layers: TPMI=253 | 253 | 7 layers: TPMI=253 |
|  |  |  |  | 254-255 | reserved | 254 | 8 layers: TPMI=254 |
|  |  |  |  |  |  | 255 | reserved |

Table 7.3.1.1.2-5G: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | maxRank = 2 | Bit field mapped to index | maxRank = 3 | Bit field mapped to index | maxRank = 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36-63 | reserved | 36 | 3 layers: TPMI=36 | 36 | 3 layers: TPMI=36 |
|  |  | … | … | … | … |
|  |  | 91 | 3 layers: TPMI=91 | 91 | 3 layers: TPMI=91 |
|  |  | 92-127 | reserved | 92 | 4 layers: TPMI=92 |
|  |  |  |  | … | … |
|  |  |  |  | 161 | 4 layers: TPMI=161 |
|  |  |  |  | 162-255 | reserved |

Table 7.3.1.1.2-5H: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookType=Codebook4, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | Precoding information and number of layers |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 7 | 1 layer: TPMI=7 |

Table 7.3.1.1.2-5I: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook2

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
| 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
| 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
| 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 | 568 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 | 635 | 4 layers: TPMI=67 |
| 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 | 636 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 | 667 | 5 layers: TPMI=31 |
| 698-1023 | reserved | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 | 668 | 6 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 | 683 | 6 layers: TPMI=15 |
|  |  | 684-1023 | reserved | 684 | 7 layers: TPMI=0 | 684 | 7 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 691 | 7 layers: TPMI=7 | 691 | 7 layers: TPMI=7 |
|  |  |  |  | 692-1023 | reserved | 692 | 8 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 695 | 8 layers: TPMI=3 |
|  |  |  |  |  |  | 696-1023 | reserved |

Table 7.3.1.1.2-5J: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook2, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 | Bit field mapped to index | transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 | 31 | 1 layer: TPMI=31 |
|  |  | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 | 32 | 2 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 | 303 | 2 layers: TPMI=271 |
|  |  | 303-511 | reserved | 304 | 3 layers: TPMI=0 | 304 | 3 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 567 | 3 layers: TPMI=263 | 567 | 3 layers: TPMI=263 |
|  |  |  |  | 568-1023 | reserved | 568 | 4 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 635 | 4 layers: TPMI=67 |
|  |  |  |  |  |  | 636-1023 | reserved |

Table 7.3.1.1.2-5K: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7 or 8, and CodebookType=Codebook3

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
| 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
| 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 |
| 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 |
| 864-1023 | reserved | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 |
|  |  | 944-1023 | reserved | 944 | 7 layers: TPMI=0 | 944 | 7 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 975 | 7 layers: TPMI=31 | 975 | 7 layers: TPMI=31 |
|  |  |  |  | 976-1023 | reserved | 976 | 8 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 991 | 8 layers: TPMI=15 |
|  |  |  |  |  |  | 992-1023 | reserved |

Table 7.3.1.1.2-5L: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook3, and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 | Bit field mapped to index | transform precoder is disabled and maxRank = 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
|  |  | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
|  |  | … | … | … | … | … | … |
|  |  | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
|  |  | 119-127 | reserved | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
|  |  |  |  | … | … | … | … |
|  |  |  |  | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
|  |  |  |  | 424-511 | reserved | 424 | 4 layers: TPMI=0 |
|  |  |  |  |  |  | … | … |
|  |  |  |  |  |  | 703 | 4 layers: TPMI=279 |
|  |  |  |  |  |  | 704-1024 | reserved |

Table 7.3.1.1.2-5M: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 2, 3 or 4, CodebookType=Codebook4, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | maxRank = 2 | Bit field mapped to index | maxRank = 3 |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … |
| 7 | 1 layer: TPMI=7 | 7 | 1 layer: TPMI=7 |
| 8 | 2 layers: TPMI=8 | 8 | 2 layers: TPMI=8 |
| … | … | … | … |
| 35 | 2 layers: TPMI=35 | 35 | 2 layers: TPMI=35 |
| 36 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] | 36 | 3 layers: TPMI=36 |
| 37 | 2 layers: TPMI=16 in Table 6.3.1.5-30 of [4, TS 38.211] | … | … |
| 38-63 | reserved | 91 | 3 layers: TPMI=91 |
|  |  | 92 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] |
|  |  | 93 | 2 layers: TPMI=16 in Table 6.3.1.5-30 of [4, TS 38.211] |
|  |  | 94 | 3 layers: |
|  |  | 95-127 | reserved |

Table 7.3.1.1.2-5N: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled or maxRank=1 if transform is disabled, CodebookType=Codebook4, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | Precoding information and number of layers |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 7 | 1 layer: TPMI=7 |
| 8 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] |
| 9-15 | reserved |

Table 7.3.1.1.2-5O: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1 if transform precoder is disabled, CodebookType=Codebook2, and ul-FullPowerTransmission configured to fullpowerMode1

| Bit field mapped to index | Precoding information and number of layers |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| … | … |
| 31 | 1 layer: TPMI=31 |
| 32 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] |
| 33-63 | reserved |

Table 7.3.1.1.2-5P: Precoding information and number of layers, for 8 antenna ports, if transform precoder is enabled, or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, CodebookType=Codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | Transform precoder is enabled, or maxRank = 1 if transform precoder is disabled | Bit field mapped to index | transform precoder is disabled and maxRank = 2 | Bit field mapped to index | transform precoder is disabled and maxRank = 3 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| 17-31 | reserved | … | … | … | … |
|  |  | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
|  |  | 120 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] | 120 | 3 layers: TPMI=0 |
|  |  | 121 | 2 layers: TPMI=16 in Table 6.3.1.5-30 of [4, TS 38.211] | … | … |
|  |  | 122-127 | reserved | 423 | 3 layers: TPMI=303 |
|  |  |  |  | 424 | 1 layer: TPMI=0 in Table 6.3.1.5-9 of [4, TS 38.211] |
|  |  |  |  | 425 | 2 layers: TPMI=16 in Table 6.3.1.5-30 of [4, TS 38.211] |
|  |  |  |  | 426 | 3 layers: |
|  |  |  |  | 427-511 | reserved |

Table 7.3.1.1.2-6: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2-BPSK modulation is used

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |

Table 7.3.1.1.2-6A: Antenna port(s), transform precoder is enabled, dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2-BPSK modulation is used, dmrs-Type=1, maxLength=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0, nSCID= 0 |
| 1 | 2 | 0, nSCID= 1 |
| 2 | 2 | 2, nSCID= 0 |
| 3 | 2 | 2, nSCID= 1 |

Table 7.3.1.1.2-7: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2-BPSK modulation is used

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 1 |
| 1 | 2 | 1 | 1 |
| 2 | 2 | 2 | 1 |
| 3 | 2 | 3 | 1 |
| 4 | 2 | 0 | 2 |
| 5 | 2 | 1 | 2 |
| 6 | 2 | 2 | 2 |
| 7 | 2 | 3 | 2 |
| 8 | 2 | 4 | 2 |
| 9 | 2 | 5 | 2 |
| 10 | 2 | 6 | 2 |
| 11 | 2 | 7 | 2 |
| 12-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-7A: Antenna port(s), transform precoder is enabled, dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2-BPSK modulation is used, dmrs-Type=1, maxLength=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0, nSCID= 0 | 1 |
| 1 | 2 | 0, nSCID= 1 | 1 |
| 2 | 2 | 2, nSCID= 0 | 1 |
| 3 | 2 | 2, nSCID= 1 | 1 |
| 4 | 2 | 0, nSCID= 0 | 2 |
| 5 | 2 | 0, nSCID= 1 | 2 |
| 6 | 2 | 2, nSCID= 0 | 2 |
| 7 | 2 | 2, nSCID= 1 | 2 |
| 8 | 2 | 4, nSCID= 0 | 2 |
| 9 | 2 | 4, nSCID= 1 | 2 |
| 10 | 2 | 6, nSCID= 0 | 2 |
| 11 | 2 | 6, nSCID= 1 | 2 |
| 12-15 | Reserved | Reserved | Reserved |

T Table 7.3.1.1.2-8: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6-7 | Reserved | Reserved |

Table 7.3.1.1.2-9: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4-7 | Reserved | Reserved |

Table 7.3.1.1.2-10: Antenna port(s), transform precoder is disabled, multipanelScheme is not configured to sdmScheme, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1-7 | Reserved | Reserved |

Table 7.3.1.1.2-10A: Antenna port(s), transform precoder is disabled, multipanelScheme = sdmScheme, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | 0,2,3 |
| 2-7 | Reserved | Reserved |

Table 7.3.1.1.2-11: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1-7 | Reserved | Reserved |

Table 7.3.1.1.2-12: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, multipanelScheme is not to sdmScheme, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14A: Antenna port(s), transform precoder is disabled, multipanelScheme = sdmScheme, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 0,2,3 | 1 |
| 4-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15A: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15C: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15D: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-17: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7-15 | Reserved | Reserved |

Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, multipanelScheme is not configured to sdmScheme, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-18A: Antenna port(s), transform precoder is disabled, multipanelScheme = sdmScheme, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 2 | 0,2,3 |
| 4 | 3 | 0,2,3 |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-19: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank =4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2-15 | Reserved | Reserved |

Table 7.3.1.1.2-19A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-19B: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22: Antenna port(s), transform precoder is disabled, multipanelScheme is not configured to sdmScheme, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22A: Antenna port(s), transform precoder is disabled, multipanelScheme = sdmScheme, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 2 | 0,2,3 | 1 |
| 7 | 3 | 0,2,3 | 1 |
| 8-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank=4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23C: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23D: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 0_2, 1_1, 1_2, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 1 when triggered by DCI formats 0_1, 0_2, 1_1, and 1_2 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 2 when triggered by DCI formats 0_1, 0_2, 1_1, and 1_2 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured by SRS-ResourceSet with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 SRS resource set(s) configured by SRS-PosResourceSet with an entry in aperiodicSRS-ResourceTriggerList set to 3 when triggered by DCI formats 0_1, 0_2, 1_1, and 1_2 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

Table 7.3.1.1.2-25: PTRS-DMRS association or Second PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |

Table 7.3.1.1.2-25A: PTRS-DMRS association for UL PTRS port 0 or for the actual UL PT-RS port if multipanelScheme is not configured, or PTRS-DMRS association for UL PTRS port 0 and 1 if multipanelScheme is configured to sdmScheme and maxNrofPortsforSDM is set to 2

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st scheduled DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field |  | 0 | 1st scheduled DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field |
| 1 | 2nd scheduled DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field |  | 1 | 2nd scheduled DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field |

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0, maxRank>4

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port corresponding to the selected Codeword |
| 1 | 2nd scheduled DMRS port corresponding to the selected Codeword |
| 2 | 3rd scheduled DMRS port corresponding to the selected Codeword |
| 3 | 4th scheduled DMRS port corresponding to the selected Codeword |

Table 7.3.1.1.2-26: PTRS-DMRS association or Second PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-26A: PTRS-DMRS association for UL PTRS ports 0 and 1, maxRank>4

| Value of 2 MSBs | DMRS port | Value of 2 LSBs | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-27: void

Table 7.3.1.1.2-28: SRI indication or Second SRI indication, for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 2 | 2 | 2 | 2 |
|  |  | 3 | reserved | 3 | 3 |

Table 7.3.1.1.2-28A: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=1

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5-7 | reserved | 5 | 5 | 5 | 5 | 5 | 5 |
|  |  | 6-7 | reserved | 6 | 6 | 6 | 6 |
|  |  |  |  | 7 | reserved | 7 | 7 |

Table 7.3.1.1.2-29: SRI indication or Second SRI indication, for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6-7 | reserved | 6 | 0,3 |
|  |  |  |  | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10-15 | reserved |

Table 7.3.1.1.2-29A: Second SRI indication for non-codebook based PUSCH transmission, Lmax=2

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0,1 | 2 | 2 | 2 | 2 |
| 1 | 2 layers: reserved | 3 | 1 layer: reserved | 3 | 3 |
|  |  | 0 | 0,1 | 4-7 | 1 layer: reserved |
|  |  | 1 | 0,2 | 0 | 0,1 |
|  |  | 2 | 1,2 | 1 | 0,2 |
|  |  | 3 | 2 layers: reserved | 2 | 0,3 |
|  |  |  |  | 3 | 1,2 |
|  |  |  |  | 4 | 1,3 |
|  |  |  |  | 5 | 2,3 |
|  |  |  |  | 6-7 | 2 layers: reserved |

Table 7.3.1.1.2-29B: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=2

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | … | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | … | 7 | s00,s10 | 7 | 7 |
| 15 | reserved | 20 | s014,s114 | … | … | 8 | s00,s10 |
|  |  | 21-31 | reserved | 27 | s020,s120 | … | … |
|  |  |  |  | 28-31 | reserved | 35 | s027,s127 |
|  |  |  |  |  |  | 36-63 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-29B-1: SRI(s) for 2 layers, NSRS>4

| SRI(s) | SRI(s) for 2 layers, NSRS=5 | SRI(s) for 2 layers, NSRS=6 | SRI(s) for 2 layers, NSRS=7 | SRI(s) for 2 layers, NSRS=8 |
| --- | --- | --- | --- | --- |
| s00,s10 | 0,1 | 0,1 | 0,1 | 0,1 |
| s01,s11 | 0,2 | 0,2 | 0,2 | 0,2 |
| s02,s12 | 0,3 | 0,3 | 0,3 | 0,3 |
| s03,s13 | 0,4 | 0,4 | 0,4 | 0,4 |
| s04,s14 | 1,2 | 0,5 | 0,5 | 0,5 |
| s05,s15 | 1,3 | 1,2 | 0,6 | 0,6 |
| s06,s16 | 1,4 | 1,3 | 1,2 | 0,7 |
| s07,s17 | 2,3 | 1,4 | 1,3 | 1,2 |
| s08,s18 | 2,4 | 1,5 | 1,4 | 1,3 |
| s09,s19 | 3,4 | 2,3 | 1,5 | 1,4 |
| s010,s110 |  | 2,4 | 1,6 | 1,5 |
| s011,s111 |  | 2,5 | 2,3 | 1,6 |
| s012,s112 |  | 3,4 | 2,4 | 1,7 |
| s013,s113 |  | 3,5 | 2,5 | 2,3 |
| s014,s114 |  | 4,5 | 2,6 | 2,4 |
| s015,s115 |  |  | 3,4 | 2,5 |
| s016,s116 |  |  | 3,5 | 2,6 |
| s017,s117 |  |  | 3,6 | 2,7 |
| s018,s118 |  |  | 4,5 | 3,4 |
| s019,s119 |  |  | 4,6 | 3,5 |
| s020,s120 |  |  | 5,6 | 3,6 |
| s021,s121 |  |  |  | 3,7 |
| s022,s122 |  |  |  | 4,5 |
| s023,s123 |  |  |  | 4,6 |
| s024,s124 |  |  |  | 4,7 |
| s025,s125 |  |  |  | 5,6 |
| s026,s126 |  |  |  | 5,7 |
| s027,s127 |  |  |  | 6,7 |

Table 7.3.1.1.2-30: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14-15 | reserved |

Table 7.3.1.1.2-30A: Second SRI indication for non-codebook based PUSCH transmission, if multipanelScheme is not configured, Lmax=3

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0,1 | 2 | 2 | 2 | 2 |
| 1 | 2 layers: reserved | 3 | 1 layer: reserved | 3 | 3 |
|  |  | 0 | 0,1 | 4-7 | 1 layer: reserved |
|  |  | 1 | 0,2 | 0 | 0,1 |
|  |  | 2 | 1,2 | 1 | 0,2 |
|  |  | 3 | 2 layers: reserved | 2 | 0,3 |
|  |  | 0 | 0,1,2 | 3 | 1,2 |
|  |  | 1-3 | 3 layers: reserved | 4 | 1,3 |
|  |  |  |  | 5 | 2,3 |
|  |  |  |  | 6-7 | 2 layers: reserved |
|  |  |  |  | 0 | 0,1,2 |
|  |  |  |  | 1 | 0,1,3 |
|  |  |  |  | 2 | 0,2,3 |
|  |  |  |  | 3 | 1,2,3 |
|  |  |  |  | 4-7 | 3 layers: reserved |

Table 7.3.1.1.2-30B: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=3

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25-31 | reserved | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
|  |  | 41-63 | reserved | 62 | s034,s134,s234 | … | ... |
|  |  |  |  | 63-127 | reserved | 91 | s055,s155,s255 |
|  |  |  |  |  |  | 92-127 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1 and Table 7.3.1.1.2-30B-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-30B-1: SRI combinations for 3 layers, NSRS>4

|  | SRI(s) for 3 layers, NSRS=5 | SRI(s) for 3 layers, NSRS=6 | SRI(s) for 3 layers, NSRS=7 | SRI(s) for 3 layers, NSRS=8 |
| --- | --- | --- | --- | --- |
| s00,s10,s20 | 0,1,2 | 0,1,2 | 0,1,2 | 0,1,2 |
| s01,s11,s21 | 0,1,3 | 0,1,3 | 0,1,3 | 0,1,3 |
| s02,s12,s22 | 0,1,4 | 0,1,4 | 0,1,4 | 0,1,4 |
| s03,s13,s23 | 0,2,3 | 0,1,5 | 0,1,5 | 0,1,5 |
| s04,s14,s24 | 0,2,4 | 0,2,3 | 0,1,6 | 0,1,6 |
| s05,s15,s25 | 0,3,4 | 0,2,4 | 0,2,3 | 0,1,7 |
| s06,s16,s26 | 1,2,3 | 0,2,5 | 0,2,4 | 0,2,3 |
| s07,s17,s27 | 1,2,4 | 0,3,4 | 0,2,5 | 0,2,4 |
| s08,s18,s28 | 1,3,4 | 0,3,5 | 0,2,6 | 0,2,5 |
| s09,s19,s29 | 2,3,4 | 0,4,5 | 0,3,4 | 0,2,6 |
| s010,s110,s210 |  | 1,2,3 | 0,3,5 | 0,2,7 |
| s011,s111,s211 |  | 1,2,4 | 0,3,6 | 0,3,4 |
| s012,s112,s212 |  | 1,2,5 | 0,4,5 | 0,3,5 |
| s013,s113,s213 |  | 1,3,4 | 0,4,6 | 0,3,6 |
| s014,s114,s214 |  | 1,3,5 | 0,5,6 | 0,3,7 |
| s015,s115,s215 |  | 1,4,5 | 1,2,3 | 0,4,5 |
| s016,s116,s216 |  | 2,3,4 | 1,2,4 | 0,4,6 |
| s017,s117,s217 |  | 2,3,5 | 1,2,5 | 0,4,7 |
| s018,s118,s218 |  | 2,4,5 | 1,2,6 | 0,5,6 |
| s019,s119,s219 |  | 3,4,5 | 1,3,4 | 0,5,7 |
| s020,s120,s220 |  |  | 1,3,5 | 0,6,7 |
| s021,s121,s221 |  |  | 1,3,6 | 1,2,3 |
| s022,s122,s222 |  |  | 1,4,5 | 1,2,4 |
| s023,s123,s223 |  |  | 1,4,6 | 1,2,5 |
| s024,s124,s224 |  |  | 1,5,6 | 1,2,6 |
| s025,s125,s225 |  |  | 2,3,4 | 1,2,7 |
| s026,s126,s226 |  |  | 2,3,5 | 1,3,4 |
| s027,s127,s227 |  |  | 2,3,6 | 1,3,5 |
| s028,s128,s228 |  |  | 2,4,5 | 1,3,6 |
| s029,s129,s229 |  |  | 2,4,6 | 1,3,7 |
| s030,s130,s230 |  |  | 2,5,6 | 1,4,5 |
| s031,s131,s231 |  |  | 3,4,5 | 1,4,6 |
| s032,s132,s232 |  |  | 3,4,6 | 1,4,7 |
| s033,s133,s233 |  |  | 3,5,6 | 1,5,6 |
| s034,s134,s234 |  |  | 4,5,6 | 1,5,7 |
| s035,s135,s235 |  |  |  | 1,6,7 |
| s036,s136,s236 |  |  |  | 2,3,4 |
| s037,s137,s237 |  |  |  | 2,3,5 |
| s038,s138,s238 |  |  |  | 2,3,6 |
| s039,s139,s239 |  |  |  | 2,3,7 |
| s040,s140,s240 |  |  |  | 2,4,5 |
| s041,s141,s241 |  |  |  | 2,4,6 |
| s042,s142,s242 |  |  |  | 2,4,7 |
| s043,s143,s243 |  |  |  | 2,5,6 |
| s044,s144,s244 |  |  |  | 2,5,7 |
| s045,s145,s245 |  |  |  | 2,6,7 |
| s046,s146,s246 |  |  |  | 3,4,5 |
| s047,s147,s247 |  |  |  | 3,4,6 |
| s048,s148,s248 |  |  |  | 3,4,7 |
| s049,s149,s249 |  |  |  | 3,5,6 |
| s050,s150,s250 |  |  |  | 3,5,7 |
| s051,s151,s251 |  |  |  | 3,6,7 |
| s052,s152,s252 |  |  |  | 4,5,6 |
| s053,s153,s253 |  |  |  | 4,5,7 |
| s054,s154,s254 |  |  |  | 4,6,7 |
| s055,s155,s255 |  |  |  | 5,6,7 |

Table 7.3.1.1.2-31: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14 | 0,1,2,3 |
|  |  |  |  | 15 | reserved |

Table 7.3.1.1.2-31A: Second SRI indication for non-codebook based PUSCH transmission, if multipanelScheme is not configured, Lmax=4

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0,1 | 2 | 2 | 2 | 2 |
| 1 | 2 layers: reserved | 3 | 1 layer: reserved | 3 | 3 |
|  |  | 0 | 0,1 | 4-7 | 1 layer: reserved |
|  |  | 1 | 0,2 | 0 | 0,1 |
|  |  | 2 | 1,2 | 1 | 0,2 |
|  |  | 3 | 2 layers: reserved | 2 | 0,3 |
|  |  | 0 | 0,1,2 | 3 | 1,2 |
|  |  | 1-3 | 3 layers: reserved | 4 | 1,3 |
|  |  |  |  | 5 | 2,3 |
|  |  |  |  | 6-7 | 2 layers: reserved |
|  |  |  |  | 0 | 0,1,2 |
|  |  |  |  | 1 | 0,1,3 |
|  |  |  |  | 2 | 0,2,3 |
|  |  |  |  | 3 | 1,2,3 |
|  |  |  |  | 4-7 | 3 layer: reserved |
|  |  |  |  | 0 | 0,1,2,3 |
|  |  |  |  | 1-7 | 4 layers: reserved |

Table 7.3.1.1.2-31B: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=4

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30-31 | reserved | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
|  |  | 56-63 | reserved | 97 | s034,s134,s234,s334 | … | ... |
|  |  |  |  | 98-127 | reserved | 161 | s069,s169,s269,s369 |
|  |  |  |  |  |  | 162-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1 and Table 7.3.1.1.2-31B-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31B-1: SRI combinations for 4 layers, NSRS>4

|  | SRI(s) for 4 layers, NSRS=5 | SRI(s) for 4 layers, NSRS=6 | SRI(s) for 4 layers, NSRS=7 | SRI(s) for 4 layers, NSRS=8 |
| --- | --- | --- | --- | --- |
| s00,s10,s20,s30 | 0,1,2,3 | 0,1,2,3 | 0,1,2,3 | 0,1,2,3 |
| s01,s11,s21,s31 | 0,1,2,4 | 0,1,2,4 | 0,1,2,4 | 0,1,2,4 |
| s02,s12,s22,s32 | 0,1,3,4 | 0,1,2,5 | 0,1,2,5 | 0,1,2,5 |
| s03,s13,s23,s33 | 0,2,3,4 | 0,1,3,4 | 0,1,2,6 | 0,1,2,6 |
| s04,s14,s24,s34 | 1,2,3,4 | 0,1,3,5 | 0,1,3,4 | 0,1,2,7 |
| s05,s15,s25,s35 |  | 0,1,4,5 | 0,1,3,5 | 0,1,3,4 |
| s06,s16,s26,s36 |  | 0,2,3,4 | 0,1,3,6 | 0,1,3,5 |
| s07,s17,s27,s37 |  | 0,2,3,5 | 0,1,4,5 | 0,1,3,6 |
| s08,s18,s28,s38 |  | 0,2,4,5 | 0,1,4,6 | 0,1,3,7 |
| s09,s19,s29,s39 |  | 0,3,4,5 | 0,1,5,6 | 0,1,4,5 |
| s010,s110,s210,s310 |  | 1,2,3,4 | 0,2,3,4 | 0,1,4,6 |
| s011,s111,s211,s311 |  | 1,2,3,5 | 0,2,3,5 | 0,1,4,7 |
| s012,s112,s212,s312 |  | 1,2,4,5 | 0,2,3,6 | 0,1,5,6 |
| s013,s113,s213,s313 |  | 1,3,4,5 | 0,2,4,5 | 0,1,5,7 |
| s014,s114,s214,s314 |  | 2,3,4,5 | 0,2,4,6 | 0,1,6,7 |
| s015,s115,s215,s315 |  |  | 0,2,5,6 | 0,2,3,4 |
| s016,s116,s216,s316 |  |  | 0,3,4,5 | 0,2,3,5 |
| s017,s117,s217,s317 |  |  | 0,3,4,6 | 0,2,3,6 |
| s018,s118,s218,s318 |  |  | 0,3,5,6 | 0,2,3,7 |
| s019,s119,s219,s319 |  |  | 0,4,5,6 | 0,2,4,5 |
| s020,s120,s220,s320 |  |  | 1,2,3,4 | 0,2,4,6 |
| s021,s121,s221,s321 |  |  | 1,2,3,5 | 0,2,4,7 |
| s022,s122,s222,s322 |  |  | 1,2,3,6 | 0,2,5,6 |
| s023,s123,s223,s323 |  |  | 1,2,4,5 | 0,2,5,7 |
| s024,s124,s224,s324 |  |  | 1,2,4,6 | 0,2,6,7 |
| s025,s125,s225,s325 |  |  | 1,2,5,6 | 0,3,4,5 |
| s026,s126,s226,s326 |  |  | 1,3,4,5 | 0,3,4,6 |
| s027,s127,s227,s327 |  |  | 1,3,4,6 | 0,3,4,7 |
| s028,s128,s228,s328 |  |  | 1,3,5,6 | 0,3,5,6 |
| s029,s129,s229,s329 |  |  | 1,4,5,6 | 0,3,5,7 |
| s030,s130,s230,s330 |  |  | 2,3,4,5 | 0,3,6,7 |
| s031,s131,s231,s331 |  |  | 2,3,4,6 | 0,4,5,6 |
| s032,s132,s232,s332 |  |  | 2,3,5,6 | 0,4,5,7 |
| s033,s133,s233,s333 |  |  | 2,4,5,6 | 0,4,6,7 |
| s034,s134,s234,s334 |  |  | 3,4,5,6 | 0,5,6,7 |
| s035,s135,s235,s335 |  |  |  | 1,2,3,4 |
| s036,s136,s236,s336 |  |  |  | 1,2,3,5 |
| s037,s137,s237,s337 |  |  |  | 1,2,3,6 |
| s038,s138,s238,s338 |  |  |  | 1,2,3,7 |
| s039,s139,s239,s339 |  |  |  | 1,2,4,5 |
| s040,s140,s240,s340 |  |  |  | 1,2,4,6 |
| s041,s141,s241,s341 |  |  |  | 1,2,4,7 |
| s042,s142,s242,s342 |  |  |  | 1,2,5,6 |
| s043,s143,s243,s343 |  |  |  | 1,2,5,7 |
| s044,s144,s244,s344 |  |  |  | 1,2,6,7 |
| s045,s145,s245,s345 |  |  |  | 1,3,4,5 |
| s046,s146,s246,s346 |  |  |  | 1,3,4,6 |
| s047,s147,s247,s347 |  |  |  | 1,3,4,7 |
| s048,s148,s248,s348 |  |  |  | 1,3,5,6 |
| s049,s149,s249,s349 |  |  |  | 1,3,5,7 |
| s050,s150,s250,s350 |  |  |  | 1,3,6,7 |
| s051,s151,s251,s351 |  |  |  | 1,4,5,6 |
| s052,s152,s252,s352 |  |  |  | 1,4,5,7 |
| s053,s153,s253,s353 |  |  |  | 1,4,6,7 |
| s054,s154,s254,s354 |  |  |  | 1,5,6,7 |
| s055,s155,s255,s355 |  |  |  | 2,3,4,5 |
| s056,s156,s256,s356 |  |  |  | 2,3,4,6 |
| s057,s157,s257,s357 |  |  |  | 2,3,4,7 |
| s058,s158,s258,s358 |  |  |  | 2,3,5,6 |
| s059,s159,s259,s359 |  |  |  | 2,3,5,7 |
| s060,s160,s260,s360 |  |  |  | 2,3,6,7 |
| s061,s161,s261,s361 |  |  |  | 2,4,5,6 |
| s062,s162,s262,s362 |  |  |  | 2,4,5,7 |
| s063,s163,s263,s363 |  |  |  | 2,4,6,7 |
| s064,s164,s264,s364 |  |  |  | 2,5,6,7 |
| s065,s165,s265,s365 |  |  |  | 3,4,5,6 |
| s066,s166,s266,s366 |  |  |  | 3,4,5,7 |
| s067,s167,s267,s367 |  |  |  | 3,4,6,7 |
| s068,s168,s268,s368 |  |  |  | 3,5,6,7 |
| s069,s169,s269,s369 |  |  |  | 4,5,6,7 |

Table 7.3.1.1.2-31C: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=5

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62-63 | reserved | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  |  |  | 119-127 | reserved | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  |  |  | 218-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1 and Table 7.3.1.1.2-31C-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31C-1: SRI combinations for 5 layers, NSRS>4

|  | SRI(s) for 5 layers, NSRS=6 | SRI(s) for 5 layers, NSRS=7 | SRI(s) for 5 layers, NSRS=8 |
| --- | --- | --- | --- |
| s00,s10,s20,s30,s40 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 |
| s01,s11,s21,s31,s41 | 0,1,2,3,5 | 0,1,2,3,5 | 0,1,2,3,5 |
| s02,s12,s22,s32,s42 | 0,1,2,4,5 | 0,1,2,3,6 | 0,1,2,3,6 |
| s03,s13,s23,s33,s43 | 0,1,3,4,5 | 0,1,2,4,5 | 0,1,2,3,7 |
| s04,s14,s24,s34,s44 | 0,2,3,4,5 | 0,1,2,4,6 | 0,1,2,4,5 |
| s05,s15,s25,s35,s45 | 1,2,3,4,5 | 0,1,2,5,6 | 0,1,2,4,6 |
| s06,s16,s26,s36,s46 |  | 0,1,3,4,5 | 0,1,2,4,7 |
| s07,s17,s27,s37,s47 |  | 0,1,3,4,6 | 0,1,2,5,6 |
| s08,s18,s28,s38,s48 |  | 0,1,3,5,6 | 0,1,2,5,7 |
| s09,s19,s29,s39,s49 |  | 0,1,4,5,6 | 0,1,2,6,7 |
| s010,s110,s210,s310,s410 |  | 0,2,3,4,5 | 0,1,3,4,5 |
| s011,s111,s211,s311,s411 |  | 0,2,3,4,6 | 0,1,3,4,6 |
| s012,s112,s212,s312,s412 |  | 0,2,3,5,6 | 0,1,3,4,7 |
| s013,s113,s213,s313,s413 |  | 0,2,4,5,6 | 0,1,3,5,6 |
| s014,s114,s214,s314,s414 |  | 0,3,4,5,6 | 0,1,3,5,7 |
| s015,s115,s215,s315,s415 |  | 1,2,3,4,5 | 0,1,3,6,7 |
| s016,s116,s216,s316,s416 |  | 1,2,3,4,6 | 0,1,4,5,6 |
| s017,s117,s217,s317,s417 |  | 1,2,3,5,6 | 0,1,4,5,7 |
| s018,s118,s218,s318,s418 |  | 1,2,4,5,6 | 0,1,4,6,7 |
| s019,s119,s219,s319,s419 |  | 1,3,4,5,6 | 0,1,5,6,7 |
| s020,s120,s220,s320,s420 |  | 2,3,4,5,6 | 0,2,3,4,5 |
| s021,s121,s221,s321,s421 |  |  | 0,2,3,4,6 |
| s022,s122,s222,s322,s422 |  |  | 0,2,3,4,7 |
| s023,s123,s223,s323,s423 |  |  | 0,2,3,5,6 |
| s024,s124,s224,s324,s424 |  |  | 0,2,3,5,7 |
| s025,s125,s225,s325,s425 |  |  | 0,2,3,6,7 |
| s026,s126,s226,s326,s426 |  |  | 0,2,4,5,6 |
| s027,s127,s227,s327,s427 |  |  | 0,2,4,5,7 |
| s028,s128,s228,s328,s428 |  |  | 0,2,4,6,7 |
| s029,s129,s229,s329,s429 |  |  | 0,2,5,6,7 |
| s030,s130,s230,s330,s430 |  |  | 0,3,4,5,6 |
| s031,s131,s231,s331,s431 |  |  | 0,3,4,5,7 |
| s032,s132,s232,s332,s432 |  |  | 0,3,4,6,7 |
| s033,s133,s233,s333,s433 |  |  | 0,3,5,6,7 |
| s034,s134,s234,s334,s434 |  |  | 0,4,5,6,7 |
| s035,s135,s235,s335,s435 |  |  | 1,2,3,4,5 |
| s036,s136,s236,s336,s436 |  |  | 1,2,3,4,6 |
| s037,s137,s237,s337,s437 |  |  | 1,2,3,4,7 |
| s038,s138,s238,s338,s438 |  |  | 1,2,3,5,6 |
| s039,s139,s239,s339,s439 |  |  | 1,2,3,5,7 |
| s040,s140,s240,s340,s440 |  |  | 1,2,3,6,7 |
| s041,s141,s241,s341,s441 |  |  | 1,2,4,5,6 |
| s042,s142,s242,s342,s442 |  |  | 1,2,4,5,7 |
| s043,s143,s243,s343,s443 |  |  | 1,2,4,6,7 |
| s044,s144,s244,s344,s444 |  |  | 1,2,5,6,7 |
| s045,s145,s245,s345,s445 |  |  | 1,3,4,5,6 |
| s046,s146,s246,s346,s446 |  |  | 1,3,4,5,7 |
| s047,s147,s247,s347,s447 |  |  | 1,3,4,6,7 |
| s048,s148,s248,s348,s448 |  |  | 1,3,5,6,7 |
| s049,s149,s249,s349,s449 |  |  | 1,4,5,6,7 |
| s050,s150,s250,s350,s450 |  |  | 2,3,4,5,6 |
| s051,s151,s251,s351,s451 |  |  | 2,3,4,5,7 |
| s052,s152,s252,s352,s452 |  |  | 2,3,4,6,7 |
| s053,s153,s253,s353,s453 |  |  | 2,3,5,6,7 |
| s054,s154,s254,s354,s454 |  |  | 2,4,5,6,7 |
| s055,s155,s255,s355,s455 |  |  | 3,4,5,6,7 |

Table 7.3.1.1.2-31D: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=6

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62 | 0,1,2,3,4,5 | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  | 63 | reserved | 119 | s00,s10,s20,s30,s40,s50 | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  | … | ... | 218 | s00,s10,s20,s30,s40,s50 |
|  |  |  |  | 125 | s06,s16,s26,s36,s46,s56 | … | ... |
|  |  |  |  | 126-127 | reserved | 245 | s027,s127,s227,s327,s427,s527 |
|  |  |  |  |  |  | 246-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1, Table 7.3.1.1.2-31C-1 and Table 7.3.1.1.2-31D-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31D-1: SRI combinations for 6 layers, NSRS>6

|  | SRI(s) for 6 layers, NSRS=7 | SRI(s) for 6 layers, NSRS=8 |
| --- | --- | --- |
| s00,s10,s20,s30,s40,s50 | 0,1,2,3,4,5 | 0,1,2,3,4,5 |
| s01,s11,s21,s31,s41,s51 | 0,1,2,3,4,6 | 0,1,2,3,4,6 |
| s02,s12,s22,s32,s42,s52 | 0,1,2,3,5,6 | 0,1,2,3,4,7 |
| s03,s13,s23,s33,s43,s53 | 0,1,2,4,5,6 | 0,1,2,3,5,6 |
| s04,s14,s24,s34,s44,s54 | 0,1,3,4,5,6 | 0,1,2,3,5,7 |
| s05,s15,s25,s35,s45,s55 | 0,2,3,4,5,6 | 0,1,2,3,6,7 |
| s06,s16,s26,s36,s46,s56 | 1,2,3,4,5,6 | 0,1,2,4,5,6 |
| s07,s17,s27,s37,s47,s57 |  | 0,1,2,4,5,7 |
| s08,s18,s28,s38,s48,s58 |  | 0,1,2,4,6,7 |
| s09,s19,s29,s39,s49,s59 |  | 0,1,2,5,6,7 |
| s010,s110,s210,s310,s410,s510 |  | 0,1,3,4,5,6 |
| s011,s111,s211,s311,s411,s511 |  | 0,1,3,4,5,7 |
| s012,s112,s212,s312,s412,s512 |  | 0,1,3,4,6,7 |
| s013,s113,s213,s313,s413,s513 |  | 0,1,3,5,6,7 |
| s014,s114,s214,s314,s414,s514 |  | 0,1,4,5,6,7 |
| s015,s115,s215,s315,s415,s515 |  | 0,2,3,4,5,6 |
| s016,s116,s216,s316,s416,s516 |  | 0,2,3,4,5,7 |
| s017,s117,s217,s317,s417,s517 |  | 0,2,3,4,6,7 |
| s018,s118,s218,s318,s418,s518 |  | 0,2,3,5,6,7 |
| s019,s119,s219,s319,s419,s519 |  | 0,2,4,5,6,7 |
| s020,s120,s220,s320,s420,s520 |  | 0,3,4,5,6,7 |
| s021,s121,s221,s321,s421,s521 |  | 1,2,3,4,5,6 |
| s022,s122,s222,s322,s422,s522 |  | 1,2,3,4,5,7 |
| s023,s123,s223,s323,s423,s523 |  | 1,2,3,4,6,7 |
| s024,s124,s224,s324,s424,s524 |  | 1,2,3,5,6,7 |
| s025,s125,s225,s325,s425,s525 |  | 1,2,4,5,6,7 |
| s026,s126,s226,s326,s426,s526 |  | 1,3,4,5,6,7 |
| s027,s127,s227,s327,s427,s527 |  | 2,3,4,5,6,7 |

Table 7.3.1.1.2-31E: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=7

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62 | 0,1,2,3,4,5 | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  | 63 | reserved | 119 | s00,s10,s20,s30,s40,s50 | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  | … | ... | 218 | s00,s10,s20,s30,s40,s50 |
|  |  |  |  | 125 | s06,s16,s26,s36,s46,s56 | … | ... |
|  |  |  |  | 126 | 0,1,2,3,4,5,6 | 245 | s027,s127,s227,s327,s427,s527 |
|  |  |  |  | 127 | reserved | 246 | 0,1,2,3,4,5,6 |
|  |  |  |  |  |  | 247 | 0,1,2,3,4,5,7 |
|  |  |  |  |  |  | 248 | 0,1,2,3,4,6,7 |
|  |  |  |  |  |  | 249 | 0,1,2,3,5,6,7 |
|  |  |  |  |  |  | 250 | 0,1,2,4,5,6,7 |
|  |  |  |  |  |  | 251 | 0,1,3,4,5,6,7 |
|  |  |  |  |  |  | 252 | 0,2,3,4,5,6,7 |
|  |  |  |  |  |  | 253 | 1,2,3,4,5,6,7 |
|  |  |  |  |  |  | 254-255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1, Table 7.3.1.1.2-31C-1 and Table 7.3.1.1.2-31D-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-31F: SRI indication, for non-codebook based PUSCH transmission, NSRS>4, Lmax=8

| Bit field mapped to index | SRI(s), NSRS=5 | Bit field mapped to index | SRI(s), NSRS=6 | Bit field mapped to index | SRI(s), NSRS=7 | Bit field mapped to index | SRI(s), NSRS=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| 5 | s00,s10 | 5 | 5 | 5 | 5 | 5 | 5 |
| … | ... | 6 | s00,s10 | 6 | 6 | 6 | 6 |
| 14 | s09,s19 | … | ... | 7 | s00,s10 | 7 | 7 |
| 15 | s00,s10,s20 | 20 | s014,s114 | … | ... | 8 | s00,s10 |
| … | ... | 21 | s00,s10,s20 | 27 | s020,s120 | … | ... |
| 24 | s09,s19,s29 | … | ... | 28 | s00,s10,s20 | 35 | s027,s127 |
| 25 | s00,s10,s20,s30 | 40 | s019,s119,s219 | … | ... | 36 | s00,s10,s20 |
| … | ... | 41 | s00,s10,s20,s30 | 62 | s034,s134,s234 | … | ... |
| 29 | s04,s14,s24,s34 | … | ... | 63 | s00,s10,s20,s30 | 91 | s055,s155,s255 |
| 30 | 0,1,2,3,4 | 55 | s014,s114,s214,s314 | … | ... | 92 | s00,s10,s20,s30 |
| 31 | reserved | 56 | s00,s10,s20,s30,s40 | 97 | s034,s134,s234,s334 | … | ... |
|  |  | … | ... | 98 | s00,s10,s20,s30,s40 | 161 | s069,s169,s269,s369 |
|  |  | 61 | s05,s15,s25,s35,s45 | … | ... | 162 | s00,s10,s20,s30,s40 |
|  |  | 62 | 0,1,2,3,4,5 | 118 | s020,s120,s220,s320,s420 | … | ... |
|  |  | 63 | reserved | 119 | s00,s10,s20,s30,s40,s50 | 217 | s055,s155,s255,s355,s455 |
|  |  |  |  | … | ... | 218 | s00,s10,s20,s30,s40,s50 |
|  |  |  |  | 125 | s06,s16,s26,s36,s46,s56 | … | ... |
|  |  |  |  | 126 | 0,1,2,3,4,5,6 | 245 | s027,s127,s227,s327,s427,s527 |
|  |  |  |  | 127 | reserved | 246 | 0,1,2,3,4,5,6 |
|  |  |  |  |  |  | 247 | 0,1,2,3,4,5,7 |
|  |  |  |  |  |  | 248 | 0,1,2,3,4,6,7 |
|  |  |  |  |  |  | 249 | 0,1,2,3,5,6,7 |
|  |  |  |  |  |  | 250 | 0,1,2,4,5,6,7 |
|  |  |  |  |  |  | 251 | 0,1,3,4,5,6,7 |
|  |  |  |  |  |  | 252 | 0,2,3,4,5,6,7 |
|  |  |  |  |  |  | 253 | 1,2,3,4,5,6,7 |
|  |  |  |  |  |  | 254 | 0,1,2,3,4,5,6,7 |
|  |  |  |  |  |  | 255 | reserved |
| where SRIs s0,Bs1,…,Bsυ-1B, B∈0,1,…,CNSRS,υ-1 are as given in Table 7.3.1.1.2-29B-1, Table 7.3.1.1.2-30B-1, Table 7.3.1.1.2-31B-1, Table 7.3.1.1.2-31C-1 and Table 7.3.1.1.2-31D-1. |  |  |  |  |  |  |  |

Table 7.3.1.1.2-32: SRI indication or Second SRI indication, for codebook based PUSCH transmission, if ul-FullPowerTransmission is not configured, or ul-FullPowerTransmission = fullpowerMode1, or ul-FullPowerTransmission = fullpowerMode2, or ul-FullPowerTransmission = fullpower and NSRS=2

| Bit field mapped to index | SRI(s), |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

Table 7.3.1.1.2-32A: SRI indication or Second SRI indication, for codebook based PUSCH transmission, if ul-FullPowerTransmission = fullpowerMode2 and NSRS=3

| Bit field mapped to index | SRI(s), NSRS=3 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | Reserved |

Table 7.3.1.1.2-32B: SRI indication or Second SRI indication, for codebook based PUSCH transmission, if ul-FullPowerTransmission = fullpowerMode2 and NSRS=4

| Bit field mapped to index | SRI(s), NSRS=4 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

Table 7.3.1.1.2-33: Joint indication of minimum applicable scheduling offset K0/K2

| Bit field mapped to index | Minimum applicable K0 for the active DL BWP, if minimumSchedulingOffsetK0 is configured for the DL BWP | Minimum applicable K2 for the active UL BWP, if minimumSchedulingOffsetK2 is configured for the UL BWP |
| --- | --- | --- |
| 0 | The first value configured by minimumSchedulingOffsetK0 for the active DL BWP | The first value configured by minimumSchedulingOffsetK2 for the active UL BWP |
| 1 | The second value configured by minimumSchedulingOffsetK0 for the active DL BWP if the second value is configured; 0 otherwise | The second value configured by minimumSchedulingOffsetK2 for the active UL BWP if the second value is configured; 0 otherwise |

Table 7.3.1.1.2-34: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 2 |

Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1 and DCI format 0_2, configured by higher layer parameter ul-AccessConfigListDCI-0-1 and ul-AccessConfigListDCI-0-2, respectively, in frequency range 1

| Entry index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, 38.211] | CAPC |
| --- | --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 2 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 3 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 4 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 1 |
| 5 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 2 |
| 6 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 3 |
| 7 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 4 |
| 8 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 1 |
| 9 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 2 |
| 10 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 3 |
| 11 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 0 | 4 |
| 12 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 1 |
| 13 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 2 |
| 14 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 3 |
| 15 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.2 in 37.213] | 2 | 4 |
| 16 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 1 |
| 17 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 2 |
| 18 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 3 |
| 19 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 4 |
| 20 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 1 |
| 21 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 2 |
| 22 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 3 |
| 23 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 4 |
| 24 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 1 |
| 25 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 2 |
| 26 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 3 |
| 27 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 4 |
| 28 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 1 |
| 29 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 2 |
| 30 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 3 |
| 31 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 4 |
| 32 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 1 |
| 33 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 2 |
| 34 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 3 |
| 35 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 4 |
| 36 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 1 |
| 37 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 2 |
| 38 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 3 |
| 39 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 4 |
| 40 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 1 |
| 41 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 2 |
| 42 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 3 |
| 43 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 4 |

Table 7.3.1.1.2-35A: Allowed entries for DCI format 0_1 and DCI format 0_2, configured by higher layer parameter ul-AccessConfigListDCI-0-1 in frequency range 2-2

| Entry index | Channel Access Type |
| --- | --- |
| 0 | Type 1 channel access defined in clause 4.4.1 of 37.213 |
| 1 | Type 2 channel access defined in clause 4.4.2 of 37.213 |
| 2 | Type 3 channel access defined in clause 4.4.3 of 37.213 |

Table 7.3.1.1.2-36: SRS resource set indication

| Bit field mapped to index | SRS resource set indication |
| --- | --- |
| 0 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved; If there are two indicated joint/UL TCI states, the first indicated joint/UL TCI state is applied to the corresponding PUSCH transmission occasions. |
| 1 | SRS resource indicator field and Precoding information and number of layers field are associated with the second SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved; If there are two indicated joint/UL TCI states, the second indicated joint/UL TCI state is applied to the the corresponding PUSCH transmission occasions. |
| 2 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set; If there are two indicated joint/UL TCI states, the first indicated joint/UL TCI state is applied to the PUSCH transmission occasions/antenna ports associated with the first SRS resource set, and the second indicated joint/UL TCI state is applied to the PUSCH transmission occasions/antenna ports associated with the second SRS resource set. |
| 3 | SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set; If there are two indicated joint/UL TCI states, the first indicated joint/UL TCI state is applied to the PUSCH transmission occasions associated with the first SRS resource set, and the second indicated joint/UL TCI state is applied to the PUSCH transmission occasions associated with the second SRS resource set. If multipanelScheme is configured, this row is reserved. |
| NOTE 1: The first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook' if txConfig=nonCodebook or 'codebook' if txConfig=codebook. When only one SRS resource set is configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook' respectively, the first SRS resource set is the SRS resource set. The association of the first and second SRS resource sets to PUSCH repetitions for each bit field index value is as defined in Clause 6.1.2.1 of [6, TS 38.214]. NOTE 2: For DCI format 0_2, the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook', respectively, except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'. |  |

Table 7.3.1.1.2-37: SRS offset indicator

| Bit field mapped to index | Available slot offset, K=2 | Bit field mapped to index | Available slot offset, K=3 | Bit field mapped to index | Available slot offset, K=4 |
| --- | --- | --- | --- | --- | --- |
| 0 | The 1st entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 0 | The 1st entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 0 | The 1st entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |
| 1 | The 2nd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 1 | The 2nd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 1 | The 2nd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |
|  |  | 2 | The 3rd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise | 2 | The 3rd entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |
|  |  | 3 | Reserved | 3 | The 4th entry in availableSlotOffsetList, if configured for the aperiodic SRS resource set; 0, otherwise |

Table 7.3.1.1.2-38: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 1 | 8 |
| 7 | 1 | 9 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-39: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4 | 1 | 8,9 |
| 5 | 2 | 8,9 |
| 6 | 2 | 10,11 |
| 7 | 2 | 9,11 |
| 8-15 | Reserved | Reserved |

Table 7.3.1.1.2-40: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelScheme is not configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | 8-10 |
| 2 | 1 | 0,1,8 |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-40A: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelScheme is configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 2 | 8-10 |
| 2 | 1 | 0,1,8 |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5 | 2 | 0,2,3 |
| 6-15 | Reserved | Reserved |

Table 7.3.1.1.2-41: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 2 | 8-11 |
| 2 | 1 | 0,1,8,9 |
| 3 | 2 | 0,1,8,9 |
| 4 | 2 | 2,3,10,11 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-42: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-43: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,10 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-44: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-45: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10,11 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-46: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| 28 | 1 | 0 | 2 |
| 29 | 1 | 1 | 2 |
| 30 | 1 | 8 | 2 |
| 31 | 1 | 9 | 2 |

Table 7.3.1.1.2-47: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | 2 | 8,9 | 2 |
| 14 | 2 | 10,11 | 2 |
| 15 | 2 | 12,13 | 2 |
| 16 | 2 | 14,15 | 2 |
| 17 | 2 | 9,11 | 1 |
| 18 | 1 | 0,1 | 2 |
| 19 | 1 | 8,9 | 2 |
| 20 | 1 | 4,5 | 2 |
| 21 | 1 | 12,13 | 2 |
| 22-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-48: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelScheme is not configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 1 | 0,1,8 | 1 |
| 4 | 2 | 0,1,8 | 1 |
| 5 | 2 | 2,3,10 | 1 |
| 6 | 2 | 5,8,9 | 2 |
| 7 | 2 | 7,10,11 | 2 |
| 8 | 2 | 7,12,13 | 2 |
| 9-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-48A: Antenna port(s), transform precoder is disabled, dmrs-Type=1, multipanelScheme is configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 1 | 0,1,8 | 1 |
| 4 | 2 | 0,1,8 | 1 |
| 5 | 2 | 2,3,10 | 1 |
| 6 | 2 | 5,8,9 | 2 |
| 7 | 2 | 7,10,11 | 2 |
| 8 | 2 | 7,12,13 | 2 |
| 9 | 2 | 0,2,3 | 1 |
| 10-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-49: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4 | 2 | 8,9,12,13 | 2 |
| 5 | 2 | 10,11,14,15 | 2 |
| 6 | 1 | 0,1,8,9 | 1 |
| 7 | 2 | 0,1,8,9 | 1 |
| 8 | 2 | 2,3,10,11 | 1 |
| 9-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-50: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1 | 2 | 0,1,2,3,8 | 1 |
| 2 | 1 | 0,1,4,5,8 | 2 |
| 3 | 2 | 0,1,4,5,8 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-51: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1,4,5,8,12 | 2 |
| 3 | 2 | 0,1,4,5,8,12 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-52: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1 | 2 | 0,1,2,3,8,9,10 | 1 |
| 2 | 1 | 0,1,4,5,8,9,12 | 2 |
| 3 | 2 | 0,1,4,5,8,9,12 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-53: Antenna port(s), transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 2 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 3 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 4-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-54: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12 | 1 | 12 |
| 13 | 1 | 13 |
| 14 | 2 | 12 |
| 15 | 2 | 13 |
| 16 | 2 | 14 |
| 17 | 2 | 15 |
| 18 | 3 | 12 |
| 19 | 3 | 13 |
| 20 | 3 | 14 |
| 21 | 3 | 15 |
| 22 | 3 | 16 |
| 23 | 3 | 17 |
| 24-31 | Reserved | Reserved |

Table 7.3.1.1.2-55: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7 | 1 | 12,13 |
| 8 | 2 | 12,13 |
| 9 | 2 | 14,15 |
| 10 | 3 | 12,13 |
| 11 | 3 | 14,15 |
| 12 | 3 | 16,17 |
| 13 | 3 | 13,15 |
| 14 | 2 | 13,15 |
| 15-31 | Reserved | Reserved |

Table 7.3.1.1.2-56: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelScheme is not configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 1 | 0,1,12 |
| 4 | 2 | 0,1,12 |
| 5 | 2 | 2,3,14 |
| 6 | 3 | 0,1,12 |
| 7 | 3 | 2,3,14 |
| 8 | 3 | 4,5,16 |
| 9 | 3 | 13,15,17 |
| 10-31 | Reserved | Reserved |

Table 7.3.1.1.2-56A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelScheme is configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 1 | 0,1,12 |
| 4 | 2 | 0,1,12 |
| 5 | 2 | 2,3,14 |
| 6 | 3 | 0,1,12 |
| 7 | 3 | 2,3,14 |
| 8 | 3 | 4,5,16 |
| 9 | 3 | 13,15,17 |
| 10 | 2 | 0,2,3 |
| 11-31 | Reserved | Reserved |

Table 7.3.1.1.2-57: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2 | 1 | 0,1,12,13 |
| 3 | 2 | 0,1,12,13 |
| 4 | 2 | 2,3,14,15 |
| 5 | 3 | 0,1,12,13 |
| 6 | 3 | 2,3,14,15 |
| 7 | 3 | 4,5,16,17 |
| 8-31 | Reserved | Reserved |

Table 7.3.1.1.2-58: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1 | 2 | 0,1,2,3,12 |
| 2 | 3 | 0,1,2,3,12 |
| 3-31 | Reserved | Reserved |

Table 7.3.1.1.2-59: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1 | 2 | 0,1,2,3,12,14 |
| 2 | 3 | 0,1,2,3,12,14 |
| 3-31 | Reserved | Reserved |

Table 7.3.1.1.2-60: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3,12-14 |
| 1 | 3 | 0-3,12-14 |
| 2-31 | Reserved | Reserved |

Table 7.3.1.1.2-61: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3,12-15 |
| 1 | 3 | 0-3,12-15 |
| 2-31 | Reserved | Reserved |

Table 7.3.1.1.2-62: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28 | 1 | 12 | 1 |
| 29 | 1 | 13 | 1 |
| 30 | 2 | 12 | 1 |
| 31 | 2 | 13 | 1 |
| 32 | 2 | 14 | 1 |
| 33 | 2 | 15 | 1 |
| 34 | 3 | 12 | 1 |
| 35 | 3 | 13 | 1 |
| 36 | 3 | 14 | 1 |
| 37 | 3 | 15 | 1 |
| 38 | 3 | 16 | 1 |
| 39 | 3 | 17 | 1 |
| 40 | 3 | 12 | 2 |
| 41 | 3 | 13 | 2 |
| 42 | 3 | 14 | 2 |
| 43 | 3 | 15 | 2 |
| 44 | 3 | 16 | 2 |
| 45 | 3 | 17 | 2 |
| 46 | 3 | 18 | 2 |
| 47 | 3 | 19 | 2 |
| 48 | 3 | 20 | 2 |
| 49 | 3 | 21 | 2 |
| 50 | 3 | 22 | 2 |
| 51 | 3 | 23 | 2 |
| 52 | 1 | 12 | 2 |
| 53 | 1 | 13 | 2 |
| 54 | 1 | 18 | 2 |
| 55 | 1 | 19 | 2 |
| 56-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-63: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19 | 1 | 12,13 | 1 |
| 20 | 2 | 12,13 | 1 |
| 21 | 2 | 14,15 | 1 |
| 22 | 3 | 12,13 | 1 |
| 23 | 3 | 14,15 | 1 |
| 24 | 3 | 16,17 | 1 |
| 25 | 3 | 12,13 | 2 |
| 26 | 3 | 14,15 | 2 |
| 27 | 3 | 16,17 | 2 |
| 28 | 3 | 18,19 | 2 |
| 29 | 3 | 20,21 | 2 |
| 30 | 3 | 22,23 | 2 |
| 31 | 1 | 12,13 | 2 |
| 32 | 1 | 18,19 | 2 |
| 33 | 2 | 12,13 | 2 |
| 34 | 2 | 14,15 | 2 |
| 35 | 2 | 18,19 | 2 |
| 36 | 2 | 20,21 | 2 |
| 37 | 3 | 13,15 | 1 |
| 38 | 2 | 13,15 | 1 |
| 39-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-64: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelScheme is not configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 1 | 0,1,12 | 1 |
| 7 | 2 | 0,1,12 | 1 |
| 8 | 2 | 2,3,14 | 1 |
| 9 | 3 | 0,1,12 | 1 |
| 10 | 3 | 2,3,14 | 1 |
| 11 | 3 | 4,5,16 | 1 |
| 12 | 3 | 7,12,13 | 2 |
| 13 | 3 | 9,14,15 | 2 |
| 14 | 3 | 11,16,17 | 2 |
| 15 | 3 | 9,18,19 | 2 |
| 16 | 3 | 18,19,20 | 2 |
| 17 | 3 | 21,22,23 | 2 |
| 18 | 3 | 13,15,17 | 1 |
| 19-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-64A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, multipanelScheme is configured to sdmScheme, enhanced-dmrs-Type is configured, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 1 | 0,1,12 | 1 |
| 7 | 2 | 0,1,12 | 1 |
| 8 | 2 | 2,3,14 | 1 |
| 9 | 3 | 0,1,12 | 1 |
| 10 | 3 | 2,3,14 | 1 |
| 11 | 3 | 4,5,16 | 1 |
| 12 | 3 | 7,12,13 | 2 |
| 13 | 3 | 9,14,15 | 2 |
| 14 | 3 | 11,16,17 | 2 |
| 15 | 3 | 9,18,19 | 2 |
| 16 | 3 | 18,19,20 | 2 |
| 17 | 3 | 21,22,23 | 2 |
| 18 | 3 | 13,15,17 | 1 |
| 19 | 2 | 0,2,3 | 1 |
| 20-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-65: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5 | 3 | 12,13,18,19 | 2 |
| 6 | 3 | 14,15,20,21 | 2 |
| 7 | 3 | 16,17,22,23 | 2 |
| 8 | 1 | 0,1,12,13 | 1 |
| 9 | 2 | 0,1,12,13 | 1 |
| 10 | 2 | 2,3,14,15 | 1 |
| 11 | 3 | 0,1,12,13 | 1 |
| 12 | 3 | 2,3,14,15 | 1 |
| 13 | 3 | 4,5,16,17 | 1 |
| 14-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-66: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 2 | 2 | 0,1,2,3,12 | 1 |
| 3 | 3 | 0,1,2,3,12 | 1 |
| 4 | 1 | 0,1,6,7,12 | 2 |
| 5 | 2 | 0,1,6,7,12 | 2 |
| 6 | 3 | 0,1,6,7,12 | 2 |
| 7-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-67: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2 | 2 | 0-3,12,14 | 1 |
| 3 | 3 | 0-3,12,14 | 1 |
| 4 | 1 | 0,1,6,7,12,18 | 2 |
| 5 | 2 | 0,1,6,7,12,18 | 2 |
| 6 | 3 | 0,1,6,7,12,18 | 2 |
| 7-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-68: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1 | 2 | 0-3,12-14 | 1 |
| 2 | 3 | 0-3,12-14 | 1 |
| 3 | 1 | 0,1,6,7,12,13,18 | 2 |
| 4 | 2 | 0,1,6,7,12,13,18 | 2 |
| 5 | 3 | 0,1,6,7,12,13,18 | 2 |
| 6-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-69: Antenna port(s), transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1 | 2 | 0-3,12-15 | 1 |
| 2 | 3 | 0-3,12-15 | 1 |
| 3 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 4 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 5 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 6-63 | Reserved | Reserved | Reserved |

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if the higher layer parameter frequencyHoppingDCI-0-2 is not configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme –5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 0, 1 or 2 bits determined by higher layer parameter numberOfBitsForRV-DCI-0-2

-	If 0 bit is configured, rvid to be applied is 0;

-	1 bit according to Table 7.3.1.2.3-1;

-	2 bits according to Table 7.3.1.1.1-2.

-	HARQ process number – number of bits determined by the following:

-	5 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2-v1700 if configured;

-	otherwise 0, 1, 2, 3 or 4 bits determined by higher layer parameter harq-ProcessNumberSizeDCI-0-2

-	Downlink assignment index – 0, 1, 2 or 4 bits

-	0 bit if the higher layer parameter downlinkAssignmentIndexDCI-0-2 is not configured;

-	1, 2, 3, 4, 5 or 6 bits otherwise,

-	1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook for unicast and multicast if pdsch-HARQ-ACK-Codebook = semiStatic is configured for both unicast and multicast and the higher layer parameter fdmed-ReceptionMulticast is not configured; otherwise for semi-static HARQ-ACK codebook for unicast;

-	2 bits for dynamic HARQ-ACK codebook for unicast.

-	2nd downlink assignment index – 0 or 2 bits

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks for unicast;

-	0 bit otherwise.

-	3rd downlink assignment index – 0, 1 or 2 bits

-	1 bit for semi-static HARQ-ACK codebook for multicast if the higher layer parameter fdmed-ReceptionMulticast is configured;

-	2 bits for the dynamic HARQ-ACK codebook for multicast;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the 1st or 2 nd Downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the 1st or 2 nd Downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st or 2 nd Downlink assignment index until the bit width of the 1st or 2 nd Downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the 3rd downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the 3rd downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 3rd downlink assignment index until the bit width of the 3rd downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	Second TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213] if higher layer parameter SecondTPCFieldDCI-0-2 is configured; 0 bit otherwise.

-	SRS resource set indicator – 0 or 2 bits

-	2 bits according to Table 7.3.1.1.2-36 if

-	txConfig = nonCodeBook, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with the usage of value 'nonCodeBook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, or

-	txConfig=codebook, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook', and is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided;

-	0 bit otherwise.

-	SRS resource indicator –   log2k=1minLmax, NSRS,0_2NSRS,0_2k log2NSRS, 0_2 NSRS, 0_2NSRS, 0_2NSRS, 0_2number of bits determined by the following:

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/28A/29/29B/30/30B/31/31B if the higher layer parameter txConfig = nonCodebook, where

-	NSRS, 0_2 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI,

-	otherwise NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 and associated with the higher layer parameter usage of value 'nonCodeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set, or in the SRS resource set with lower srs-ResourceSetId of two SRS resources sets, configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList',

and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by max{maxMIMO-LayersDCI-0-2, maxMIMO-LayersforSdmDCI-0-2} if maxMIMO-LayersforSdmDCI-0-2 is configured

-	Lmax is given by max{maxMIMO-LayersDCI-0-2, maxMIMO-LayersforSfnDCI-0-2} if maxMIMO-LayersforSfnDCI-0-2 is configured

-	Lmax is given by maxMIMO-LayersDCI-0-2 otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2NSRS, 0_2  bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where

-	NSRS, 0_2 is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field if present,

-	NSRS is the number of configured SRS resources in the SRS resource set associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2, if the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI,

-	otherwise NSRS, 0_2 is the number of configured SRS resources in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 and associated with the higher layer parameter usage of value 'codeBook', where the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook' or 'nonCodeBook', the first SRS resource set is associated with coresetPoolIndex value 0 and the second SRS resource set is associated with coresetPoolIndex value 1, where the first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets, and the first and second SRS resource sets are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codebook' or 'nonCodeBook', respectively, except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList'.

-	Second SRS resource indicator – number of bits determined by the following:  log2(maxk∈{1,2,…,min⁡{Lmax,NSRS,0_2}}NSRS,  0_2k) log2NSRS, 0_2

-	log2(maxk∈{1,2,…,min⁡{Lmax,NSRS,0_2}}NSRS,0_2k)  bits according to Tables 7.3.1.1.2-28/29A/30A/31A with the same number of layers indicated by SRS resource indicator field if the higher layer parameter txConfig = nonCodebook, the higher layer parameter maxMIMO-LayersforSdmDCI-0-2 is not configured, and SRS resource set indicator field is present, where NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set, and

-	if UE supports operation with maxMIMO-LayersDCI-0-2 and the higher layer parameter maxMIMO-LayersDCI-0-2 of PUSCH-ServingCellConfig of the serving cell is configured,

-	Lmax is given by maxMIMO-LayersforSfnDCI-0-2 if maxMIMO-LayersforSfnDCI-0-2 is configured

-	Lmax is given by maxMIMO-LayersDCI-0-2 otherwise

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	log2k=1minLmax, NSRS,0_2NSRS,0_2k bits according to Tables 7.3.1.1.2-28/29 if the higher layer parameter txConfig = nonCodebook, the higher layer parameter maxMIMO-LayersforSdmDCI-0-2 is configured and SRS resource set indicator field is present, where NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set, where the second SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set, or in the SRS resource set with higher srs-ResourceSetId of two SRS resources sets, configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'nonCodeBook', except for the higher layer parameters 'srs-ResourceSetId' and 'srs-ResourceIdList', and Lmax is given by maxMIMO-LayersforSdmDCI-0-2.

-	log2NSRS, 0_2  bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook and SRS resource set indicator field is present, where  NSRS, 0_2 is the number of configured SRS resources in the second SRS resource set.

-	0 bit otherwise.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 2 if multipanelScheme = sfnScheme or max{maxRanDCI-0-2k, maxRankSdmDCI-0-2} = 2 if multipanelScheme = sdmScheme, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 1 if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 2 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 2 if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2}=1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2}=1 if multipanelScheme = sdmScheme, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if multipanelScheme is not configured or max{maxRankDCI-0-2, maxRankSfnDCI-0-2} = 1 if multipanelScheme = sfnScheme or max{maxRankDCI-0-2, maxRankSdmDCI-0-2} = 1 if multipanelScheme = sdmScheme, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2.

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook1, transform precoder is disabled, maxRankDCI-0-2 =4, and according to maxRankDCI-0-2;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook1, transform precoder is enabled or maxRankDCI-0-2 =1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookType=Codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRankDCI-0-2;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRank=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookType=Codebook4, transform precoder is disabled, maxRankDCI-0-2=2 or 3, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRankDCI-0-2;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookType=Codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookType=Codebook2, transform precoder is enabled or maxRankDCI-0-2 =1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1;

-	5, 7 or 9 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookType=Codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, or 3 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided enableSTx2PofmDCI, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2.

-	Second Precoding information – number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or maxRankSfnDCI-0-2 if multipanelScheme = sfnScheme, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if multipanelScheme is not configured or maxRankSfnDCI-0-2=2 if multipanelScheme = sfnScheme, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or maxRankSfnDCI-0-2 if multipanelScheme = sfnScheme and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if multipanelScheme is not configured or maxRankSfnDCI-0-2=1 if multipanelScheme = sfnScheme, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or maxRankSfnDCI-0-2 if multipanelScheme = sfnScheme, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if multipanelScheme is not configured or maxRankSfnDCI-0-2=2 if multipanelScheme = sfnScheme, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 if multipanelScheme is not configured or maxRankSfnDCI-0-2 if multipanelScheme = sfnScheme, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if multipanelScheme is not configured or maxRankSfnDCI-0-2=1 if multipanelScheme = sfnScheme, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

-	Antenna ports – number of bits determined by the following:

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-0-2 is not configured;

-	2, 3, 4, 5 or 6 bits otherwise,

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by 7.3.1.1.2-6A, if transform precoder is enabled, and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled, and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/10A/11 according to the value of rank, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/14A/15 according to the value of rank, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is not configured, and maxLength=2;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/18A/19 according to the value of rank, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/22A/23 according to the value of rank, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is not configured, and maxLength=2.

-	4 bits as defined by Tables 7.3.1.1.2-38/39/40/40A/41, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, and maxLength=1;

-	5 bits as defined by Tables 7.3.1.1.2-46/47/48/48A/49, if transform precoder is disabled, dmrs-Type=1, enhanced-dmrs-Type is configured, and maxLength=2;

-	5 bits as defined by Tables 7.3.1.1.2-54/55/56/56A/57, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, and maxLength=1;

-	6 bits as defined by Tables 7.3.1.1.2-62/63/64/64A/65, if transform precoder is disabled, dmrs-Type=2, enhanced-dmrs-Type is configured, and maxLength=2.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively, and the value of rank is

-	the sum of the value determined according to the SRS resource indicator field and the value determined according to the second SRS resource indicator field, if txConfig = nonCodebook and multipanelScheme = sdmScheme and SRS resource set indicator field equals "10"

-	the sum of the value determined according to the Precoding information and number of layers field and the value determined according to the Second Precoding information, if txConfig = codebook and multipanelScheme = sdmScheme and SRS resource set indicator field equals "10"

-	determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and multipanelScheme = sdmScheme is not configured, , or if the higher layer parameter txConfig = nonCodebook, multipanelScheme = sdmScheme and SRS resource set indicator field equals "00" or “01”

-	determined according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook and multipanelScheme = sdmScheme is not configured, or if the higher layer parameter txConfig = codebook, multipanelScheme = sdmScheme and SRS resource set indicator field equals "00" or "01".

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2  and dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 and is configured with antennaPortsFieldPresenceDCI-0-2, the bitwidth of this field equals maxxA,xB, where xA  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-0-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23.

-	SRS request – 0, 1, 2 or 3 bits

-	0 bit if the higher layer parameter srs-RequestDCI-0-2 is not configured;

-	1 bit as defined by Table 7.3.1.1.3-1 if higher layer parameter srs-RequestDCI-0-2 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	2 bits if higher layer parameter srs-RequestDCI-0-2 = 1 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second bit is defined by Table 7.3.1.1.3-1;

-	2 bits as defined by Table 7.3.1.1.2-24 if higher layer parameter srs-RequestDCI-0-2 = 2 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	3 bits if higher layer parameter srs-RequestDCI-0-2 = 2 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24;

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSizeDCI-0-2.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1 and multipanelScheme is not configured, or if maxRankDCI-0-2=1 and maxRankSfnDCI-0-2=1, or if maxRankDCI-0-2=1 and maxRankSdmDCI-0-2=1 when two PTRS ports are configured by maxNrofPortsforSdm;

-	2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or “01” and maxRankDCI-0-2<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" or “11”, maxRankDCI-0-2=3 or 4 and  multipanelScheme is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	When one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and "11" and maxRankDCI-0-2=2 and multipanelScheme is not configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	When two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelScheme is configured to sdmScheme, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	When one PTRS port is configured by maxNrofPortsforSDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelScheme is configured to sdmScheme, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25.

-	When one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelScheme is configured to sfnScheme, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRankDCI-0-2>2 and multipanelScheme is not configured; 0 bit otherwise. Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

-	beta_offset indicator – 0 bit if the higher layer parameter betaOffsetsDCI-0-2 = semiStaticDCI-0-2; otherwise 1 bit if 2 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3A in [5, TS 38.213], and 2 bits if 4 offset indexes are configured by higher layer parameter dynamicDCI-0-2 as defined by Table 9.3-3 in [5, TS 38.213].

When two HARQ-ACK codebooks are configured by pdsch-HARQ-ACK-CodebookList or by pdsch-HARQ-ACK-CodebookListMulticast for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the beta_offset indicator in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the beta_offset indicator in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller beta_offset indicator until the bit width of the beta_offset indicator in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization – 0 or 1 bit

-	0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled;

-	1 bit if transform precoder is disabled and the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is configured.

-	UL-SCH indicator – 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. If a UE does not support triggering SRS only in DCI, except for DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to receive a DCI format 0_2 with UL-SCH indicator of "0" and CSI request of all zero(s). If a UE supports triggering SRS only in DCI, except for DCI format 0_2 with CRC scrambled by SP-CSI-RNTI, the UE is not expected to recerive a DCI format 0_2 with UL-SCH indicator of "0", CSI request of all zero(s) and SRS request of all zero(s).

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-2 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or the number of entries in the high layer parameter ul-AccessConfigListDCI-0-1 for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ul-AccessConfigListDCI-0-2 in frequency range 1. One or more entries from Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1 in frequency range 2-2.

-	Open-loop power control parameter set indication – 0 or 1 or 2 bits.

-	0 bit if the higher layer parameter p0-PUSCH-SetList is not configured;

-	1 or 2 bits otherwise,

-	1 bit if SRS resource indicator is present in the DCI format 0_2;

-	1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-2 if SRS resource indicator is not present in the DCI format 0_2;

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-0-2 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Invalid symbol pattern indicator – 0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-2 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214].

-	PDCCH monitoring adaptation indication – 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

A UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_2 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_2 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 0_2 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 0_2 with smaller size until the payload size is the same.

-	If application of step 4B in clause 7.3.1.0 results in additional zero padding for DCI format 0_2 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 0_2 monitored on the primary cell and DCI format 0_2 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

Table 7.3.1.1.3-1: 1 bit SRS request in DCI format 0_2 and DCI format 1_2

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_2 and 1_2 |
| --- | --- |
| 0 | No aperiodic SRS resource set triggered |
| 1 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 |

< Unchanged parts are omitted >

#### 7.3.1.2	DCI formats for scheduling of PDSCH

< Unchanged parts are omitted >

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	Antenna port(s) – 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10. The UE can receive an entry with DMRS ports equals to 1000, 1002, 1003 when two TCI states are indicated in a codepoint of DCI field 'Transmission Configuration Indication'.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_1,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	TCI selection – 0 bit if higher layer parameter tciSelection-PresentInDCI is not configured; otherwise 2 bits according to Table 7.3.1.2.2-11.

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

< Unchanged parts are omitted >

Table 7.3.1.2.2-6A: Allowed entries for DCI format 1_1 and DCI format 1_2, configured by higher layer parameter ul-AccessConfigListDCI-1-1 in frequency range 2-2

| Entry index | Channel Access Type |
| --- | --- |
| 0 | Type 1 channel access defined in clause 4.4.1 of 37.213 |
| 1 | Type 2 channel access defined in clause 4.4.2 of 37.213 |
| 2 | Type 3 channel access defined in clause 4.4.3 of 37.213 |

Table 7.3.1.2.2-7: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 2 | 0,2 |  |  |  |
| 12 | 1 | 8 |  |  |  |
| 13 | 1 | 9 |  |  |  |
| 14 | 1 | 8,9 |  |  |  |
| 15 | 2 | 8 |  |  |  |
| 16 | 2 | 9 |  |  |  |
| 17 | 2 | 10 |  |  |  |
| 18 | 2 | 11 |  |  |  |
| 19 | 2 | 8,9 |  |  |  |
| 20 | 2 | 10,11 |  |  |  |
| 21 | 1 | 0,1,8 |  |  |  |
| 22 | 1 | 0,1,8,9 |  |  |  |
| 23 | 2 | 0,1,8 |  |  |  |
| 24 | 2 | 0,1,8,9 |  |  |  |
| 25 | 2 | 2,3,10 |  |  |  |
| 26 | 2 | 2,3,10,11 |  |  |  |
| 27-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-7A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 2 | 0,2 |  |  |  |
| 12 | 1 | 8 |  |  |  |
| 13 | 1 | 9 |  |  |  |
| 14 | 1 | 8,9 |  |  |  |
| 15 | 2 | 8 |  |  |  |
| 16 | 2 | 9 |  |  |  |
| 17 | 2 | 10 |  |  |  |
| 18 | 2 | 11 |  |  |  |
| 19 | 2 | 8,9 |  |  |  |
| 20 | 2 | 10,11 |  |  |  |
| 21 | 1 | 0,1,8 |  |  |  |
| 22 | 1 | 0,1,8,9 |  |  |  |
| 23 | 2 | 0,1,8 |  |  |  |
| 24 | 2 | 0,1,8,9 |  |  |  |
| 25 | 2 | 2,3,10 |  |  |  |
| 26 | 2 | 2,3,10,11 |  |  |  |
| 27 | 2 | 0,2,3 |  |  |  |
| 28-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-8: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 1 | 0,1,4,5,8,12 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 2 | 0 | 2 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 2 | 1 | 2 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 2 | 2 | 2 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 3 | 2 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 4 | 2 | 16~127 | Reserved | Reserved | Reserved |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | 1 | 8 | 1 |  |  |  |  |
| 32 | 1 | 9 | 1 |  |  |  |  |
| 33 | 1 | 8,9 | 1 |  |  |  |  |
| 34 | 2 | 8 | 1 |  |  |  |  |
| 35 | 2 | 9 | 1 |  |  |  |  |
| 36 | 2 | 10 | 1 |  |  |  |  |
| 37 | 2 | 11 | 1 |  |  |  |  |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| 40 | 2 | 8 | 2 |  |  |  |  |
| 41 | 2 | 9 | 2 |  |  |  |  |
| 42 | 2 | 10 | 2 |  |  |  |  |
| 43 | 2 | 11 | 2 |  |  |  |  |
| 44 | 2 | 12 | 2 |  |  |  |  |
| 45 | 2 | 13 | 2 |  |  |  |  |
| 46 | 2 | 14 | 2 |  |  |  |  |
| 47 | 2 | 15 | 2 |  |  |  |  |
| 48 | 2 | 8,9 | 2 |  |  |  |  |
| 49 | 2 | 10,11 | 2 |  |  |  |  |
| 50 | 2 | 12,13 | 2 |  |  |  |  |
| 51 | 2 | 14,15 | 2 |  |  |  |  |
| 52 | 1 | 0,1,8 | 1 |  |  |  |  |
| 53 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 54 | 2 | 0,1,8 | 1 |  |  |  |  |
| 55 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 56 | 2 | 2,3,10 | 1 |  |  |  |  |
| 57 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 58 | 2 | 0,1,8 | 2 |  |  |  |  |
| 59 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 60 | 2 | 4,5,12 | 2 |  |  |  |  |
| 61 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 62 | 2 | 2,3,10 | 2 |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 64 | 2 | 6,7,14 | 2 |  |  |  |  |
| 65 | 2 | 6,7,14,15 | 2 |  |  |  |  |
| 66-127 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-8A: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, enhanced-dmrs-Type is configured, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 1 | 0,1,4,5,8 | 2 |
| 9 | 2 | 0-2 | 1 | 9 | 1 | 0,1,4,5,8,12 | 2 |
| 10 | 2 | 0-3 | 1 | 10 | 1 | 0,1,4,5,8,9,12 | 2 |
| 11 | 2 | 0,2 | 1 | 11 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 12 | 2 | 0 | 2 | 12 | 2 | 0,1,4,5,8 | 2 |
| 13 | 2 | 1 | 2 | 13 | 2 | 0,1,4,5,8,12 | 2 |
| 14 | 2 | 2 | 2 | 14 | 2 | 0,1,4,5,8,9,12 | 2 |
| 15 | 2 | 3 | 2 | 15 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 16 | 2 | 4 | 2 | 16~127 | Reserved | Reserved | Reserved |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | 1 | 8 | 1 |  |  |  |  |
| 32 | 1 | 9 | 1 |  |  |  |  |
| 33 | 1 | 8,9 | 1 |  |  |  |  |
| 34 | 2 | 8 | 1 |  |  |  |  |
| 35 | 2 | 9 | 1 |  |  |  |  |
| 36 | 2 | 10 | 1 |  |  |  |  |
| 37 | 2 | 11 | 1 |  |  |  |  |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| 40 | 2 | 8 | 2 |  |  |  |  |
| 41 | 2 | 9 | 2 |  |  |  |  |
| 42 | 2 | 10 | 2 |  |  |  |  |
| 43 | 2 | 11 | 2 |  |  |  |  |
| 44 | 2 | 12 | 2 |  |  |  |  |
| 45 | 2 | 13 | 2 |  |  |  |  |
| 46 | 2 | 14 | 2 |  |  |  |  |
| 47 | 2 | 15 | 2 |  |  |  |  |
| 48 | 2 | 8,9 | 2 |  |  |  |  |
| 49 | 2 | 10,11 | 2 |  |  |  |  |
| 50 | 2 | 12,13 | 2 |  |  |  |  |
| 51 | 2 | 14,15 | 2 |  |  |  |  |
| 52 | 1 | 0,1,8 | 1 |  |  |  |  |
| 53 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 54 | 2 | 0,1,8 | 1 |  |  |  |  |
| 55 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 56 | 2 | 2,3,10 | 1 |  |  |  |  |
| 57 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| 58 | 2 | 0,1,8 | 2 |  |  |  |  |
| 59 | 2 | 0,1,8,9 | 2 |  |  |  |  |
| 60 | 2 | 4,5,12 | 2 |  |  |  |  |
| 61 | 2 | 4,5,12,13 | 2 |  |  |  |  |
| 62 | 2 | 2,3,10 | 2 |  |  |  |  |
| 63 | 2 | 2,3,10,11 | 2 |  |  |  |  |
| 64 | 2 | 6,7,14 | 2 |  |  |  |  |
| 65 | 2 | 6,7,14,15 | 2 |  |  |  |  |
| 66 | 2 | 0,2,3 | 1 |  |  |  |  |
| 67-127 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,12 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,12,14 |
| 4 | 2 | 1 | 4 | 2 | 0-3,12-14 |
| 5 | 2 | 2 | 5 | 2 | 0-3,12-15 |
| 6 | 2 | 3 | 6 | 3 | 0,1,2,3,12 |
| 7 | 2 | 0,1 | 7 | 3 | 0,1,2,3,12,14 |
| 8 | 2 | 2,3 | 8 | 3 | 0-3,12-14 |
| 9 | 2 | 0-2 | 9 | 3 | 0-3,12-15 |
| 10 | 2 | 0-3 | 10~63 | Reserved | Reserved |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| 33 | 3 | 12 |  |  |  |
| 34 | 3 | 13 |  |  |  |
| 35 | 3 | 14 |  |  |  |
| 36 | 3 | 15 |  |  |  |
| 37 | 3 | 16 |  |  |  |
| 38 | 3 | 17 |  |  |  |
| 39 | 3 | 12,13 |  |  |  |
| 40 | 3 | 14,15 |  |  |  |
| 41 | 3 | 16,17 |  |  |  |
| 42 | 1 | 0,1,12 |  |  |  |
| 43 | 1 | 0,1,12,13 |  |  |  |
| 44 | 2 | 0,1,12 |  |  |  |
| 45 | 2 | 0,1,12,13 |  |  |  |
| 46 | 2 | 2,3,14 |  |  |  |
| 47 | 2 | 2,3,14,15 |  |  |  |
| 48 | 3 | 0,1,12 |  |  |  |
| 49 | 3 | 0,1,12,13 |  |  |  |
| 50 | 3 | 2,3,14 |  |  |  |
| 51 | 3 | 2,3,14,15 |  |  |  |
| 52 | 3 | 4,5,16 |  |  |  |
| 53 | 3 | 4,5,16,17 |  |  |  |
| 54-63 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-9A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2 | 2 | 0,1,2,3,12 |
| 3 | 2 | 0 | 3 | 2 | 0,1,2,3,12,14 |
| 4 | 2 | 1 | 4 | 2 | 0-3,12-14 |
| 5 | 2 | 2 | 5 | 2 | 0-3,12-15 |
| 6 | 2 | 3 | 6 | 3 | 0,1,2,3,12 |
| 7 | 2 | 0,1 | 7 | 3 | 0,1,2,3,12,14 |
| 8 | 2 | 2,3 | 8 | 3 | 0-3,12-14 |
| 9 | 2 | 0-2 | 9 | 3 | 0-3,12-15 |
| 10 | 2 | 0-3 | 10~63 | Reserved | Reserved |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| 33 | 3 | 12 |  |  |  |
| 34 | 3 | 13 |  |  |  |
| 35 | 3 | 14 |  |  |  |
| 36 | 3 | 15 |  |  |  |
| 37 | 3 | 16 |  |  |  |
| 38 | 3 | 17 |  |  |  |
| 39 | 3 | 12,13 |  |  |  |
| 40 | 3 | 14,15 |  |  |  |
| 41 | 3 | 16,17 |  |  |  |
| 42 | 1 | 0,1,12 |  |  |  |
| 43 | 1 | 0,1,12,13 |  |  |  |
| 44 | 2 | 0,1,12 |  |  |  |
| 45 | 2 | 0,1,12,13 |  |  |  |
| 46 | 2 | 2,3,14 |  |  |  |
| 47 | 2 | 2,3,14,15 |  |  |  |
| 48 | 3 | 0,1,12 |  |  |  |
| 49 | 3 | 0,1,12,13 |  |  |  |
| 50 | 3 | 2,3,14 |  |  |  |
| 51 | 3 | 2,3,14,15 |  |  |  |
| 52 | 3 | 4,5,16 |  |  |  |
| 53 | 3 | 4,5,16,17 |  |  |  |
| 54 | 2 | 0,2,3 |  |  |  |
| 55-63 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| 9 | 2 | 0-2 | 1 | 9 | 2 | 0-3,12-15 | 1 |
| 10 | 2 | 0-3 | 1 | 10 | 3 | 0,1,2,3,12 | 1 |
| 11 | 3 | 0 | 1 | 11 | 3 | 0-3,12,14 | 1 |
| 12 | 3 | 1 | 1 | 12 | 3 | 0-3,12-14 | 1 |
| 13 | 3 | 2 | 1 | 13 | 3 | 0-3,12-15 | 1 |
| 14 | 3 | 3 | 1 | 14 | 1 | 0,1,6,7,12 | 2 |
| 15 | 3 | 4 | 1 | 15 | 1 | 0,1,6,7,12,18 | 2 |
| 16 | 3 | 5 | 1 | 16 | 1 | 0,1,6,7,12,13,18 | 2 |
| 17 | 3 | 0,1 | 1 | 17 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 18 | 3 | 2,3 | 1 | 18 | 2 | 0,1,6,7,12 | 2 |
| 19 | 3 | 4,5 | 1 | 19 | 2 | 0,1,6,7,12,18 | 2 |
| 20 | 3 | 0-2 | 1 | 20 | 2 | 0,1,6,7,12,13,18 | 2 |
| 21 | 3 | 3-5 | 1 | 21 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 22 | 3 | 0-3 | 1 | 22 | 3 | 0,1,6,7,12 | 2 |
| 23 | 2 | 0,2 | 1 | 23 | 3 | 0,1,6,7,12,18 | 2 |
| 24 | 3 | 0 | 2 | 24 | 3 | 0,1,6,7,12,13,18 | 2 |
| 25 | 3 | 1 | 2 | 25 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 26 | 3 | 2 | 2 | 26~255 | Reserved | Reserved | Reserved |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58 | 1 | 12 | 1 |  |  |  |  |
| 59 | 1 | 13 | 1 |  |  |  |  |
| 60 | 1 | 12,13 | 1 |  |  |  |  |
| 61 | 2 | 12 | 1 |  |  |  |  |
| 62 | 2 | 13 | 1 |  |  |  |  |
| 63 | 2 | 14 | 1 |  |  |  |  |
| 64 | 2 | 15 | 1 |  |  |  |  |
| 65 | 2 | 12,13 | 1 |  |  |  |  |
| 66 | 2 | 14,15 | 1 |  |  |  |  |
| 67 | 3 | 12 | 1 |  |  |  |  |
| 68 | 3 | 13 | 1 |  |  |  |  |
| 69 | 3 | 14 | 1 |  |  |  |  |
| 70 | 3 | 15 | 1 |  |  |  |  |
| 71 | 3 | 16 | 1 |  |  |  |  |
| 72 | 3 | 17 | 1 |  |  |  |  |
| 73 | 3 | 12,13 | 1 |  |  |  |  |
| 74 | 3 | 14,15 | 1 |  |  |  |  |
| 75 | 3 | 16,17 | 1 |  |  |  |  |
| 76 | 3 | 12 | 2 |  |  |  |  |
| 77 | 3 | 13 | 2 |  |  |  |  |
| 78 | 3 | 14 | 2 |  |  |  |  |
| 79 | 3 | 15 | 2 |  |  |  |  |
| 80 | 3 | 16 | 2 |  |  |  |  |
| 81 | 3 | 17 | 2 |  |  |  |  |
| 82 | 3 | 18 | 2 |  |  |  |  |
| 83 | 3 | 19 | 2 |  |  |  |  |
| 84 | 3 | 20 | 2 |  |  |  |  |
| 85 | 3 | 21 | 2 |  |  |  |  |
| 86 | 3 | 22 | 2 |  |  |  |  |
| 87 | 3 | 23 | 2 |  |  |  |  |
| 88 | 3 | 12,13 | 2 |  |  |  |  |
| 89 | 3 | 14,15 | 2 |  |  |  |  |
| 90 | 3 | 16,17 | 2 |  |  |  |  |
| 91 | 3 | 18,19 | 2 |  |  |  |  |
| 92 | 3 | 20,21 | 2 |  |  |  |  |
| 93 | 3 | 22,23 | 2 |  |  |  |  |
| 94 | 1 | 12 | 2 |  |  |  |  |
| 95 | 1 | 13 | 2 |  |  |  |  |
| 96 | 1 | 18 | 2 |  |  |  |  |
| 97 | 1 | 19 | 2 |  |  |  |  |
| 98 | 1 | 12,13 | 2 |  |  |  |  |
| 99 | 1 | 18,19 | 2 |  |  |  |  |
| 100 | 2 | 12,13 | 2 |  |  |  |  |
| 101 | 2 | 14,15 | 2 |  |  |  |  |
| 102 | 2 | 18,19 | 2 |  |  |  |  |
| 103 | 2 | 20,21 | 2 |  |  |  |  |
| 104 | 1 | 0,1,12 | 1 |  |  |  |  |
| 105 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 106 | 2 | 0,1,12 | 1 |  |  |  |  |
| 107 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 108 | 2 | 2,3,14 | 1 |  |  |  |  |
| 109 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 110 | 3 | 0,1,12 | 1 |  |  |  |  |
| 111 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 112 | 3 | 2,3,14 | 1 |  |  |  |  |
| 113 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 114 | 3 | 4,5,16 | 1 |  |  |  |  |
| 115 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| 116 | 2 | 0,1,12 | 2 |  |  |  |  |
| 117 | 2 | 0,1,12,13 | 2 |  |  |  |  |
| 118 | 2 | 6,7,18 | 2 |  |  |  |  |
| 119 | 2 | 6,7,18,19 | 2 |  |  |  |  |
| 120 | 2 | 2,3,14 | 2 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 2 |  |  |  |  |
| 122 | 2 | 8,9,20 | 2 |  |  |  |  |
| 123 | 2 | 8,9,20,21 | 2 |  |  |  |  |
| 124 | 3 | 0,1,12 | 2 |  |  |  |  |
| 125 | 3 | 0,1,12,13 | 2 |  |  |  |  |
| 126 | 3 | 6,7,18 | 2 |  |  |  |  |
| 127 | 3 | 6,7,18,19 | 2 |  |  |  |  |
| 128 | 3 | 2,3,14 | 2 |  |  |  |  |
| 129 | 3 | 2,3,14,15 | 2 |  |  |  |  |
| 130 | 3 | 8,9,20 | 2 |  |  |  |  |
| 131 | 3 | 8,9,20,21 | 2 |  |  |  |  |
| 132 | 3 | 4,5,16 | 2 |  |  |  |  |
| 133 | 3 | 4,5,16,17 | 2 |  |  |  |  |
| 134 | 3 | 10,11,22 | 2 |  |  |  |  |
| 135 | 3 | 10,11,22,23 | 2 |  |  |  |  |
| 136-255 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-10A: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, enhanced-dmrs-Type is configured, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| 9 | 2 | 0-2 | 1 | 9 | 2 | 0-3,12-15 | 1 |
| 10 | 2 | 0-3 | 1 | 10 | 3 | 0,1,2,3,12 | 1 |
| 11 | 3 | 0 | 1 | 11 | 3 | 0-3,12,14 | 1 |
| 12 | 3 | 1 | 1 | 12 | 3 | 0-3,12-14 | 1 |
| 13 | 3 | 2 | 1 | 13 | 3 | 0-3,12-15 | 1 |
| 14 | 3 | 3 | 1 | 14 | 1 | 0,1,6,7,12 | 2 |
| 15 | 3 | 4 | 1 | 15 | 1 | 0,1,6,7,12,18 | 2 |
| 16 | 3 | 5 | 1 | 16 | 1 | 0,1,6,7,12,13,18 | 2 |
| 17 | 3 | 0,1 | 1 | 17 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 18 | 3 | 2,3 | 1 | 18 | 2 | 0,1,6,7,12 | 2 |
| 19 | 3 | 4,5 | 1 | 19 | 2 | 0,1,6,7,12,18 | 2 |
| 20 | 3 | 0-2 | 1 | 20 | 2 | 0,1,6,7,12,13,18 | 2 |
| 21 | 3 | 3-5 | 1 | 21 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 22 | 3 | 0-3 | 1 | 22 | 3 | 0,1,6,7,12 | 2 |
| 23 | 2 | 0,2 | 1 | 23 | 3 | 0,1,6,7,12,18 | 2 |
| 24 | 3 | 0 | 2 | 24 | 3 | 0,1,6,7,12,13,18 | 2 |
| 25 | 3 | 1 | 2 | 25 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 26 | 3 | 2 | 2 | 26~255 | Reserved | Reserved | Reserved |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58 | 1 | 12 | 1 |  |  |  |  |
| 59 | 1 | 13 | 1 |  |  |  |  |
| 60 | 1 | 12,13 | 1 |  |  |  |  |
| 61 | 2 | 12 | 1 |  |  |  |  |
| 62 | 2 | 13 | 1 |  |  |  |  |
| 63 | 2 | 14 | 1 |  |  |  |  |
| 64 | 2 | 15 | 1 |  |  |  |  |
| 65 | 2 | 12,13 | 1 |  |  |  |  |
| 66 | 2 | 14,15 | 1 |  |  |  |  |
| 67 | 3 | 12 | 1 |  |  |  |  |
| 68 | 3 | 13 | 1 |  |  |  |  |
| 69 | 3 | 14 | 1 |  |  |  |  |
| 70 | 3 | 15 | 1 |  |  |  |  |
| 71 | 3 | 16 | 1 |  |  |  |  |
| 72 | 3 | 17 | 1 |  |  |  |  |
| 73 | 3 | 12,13 | 1 |  |  |  |  |
| 74 | 3 | 14,15 | 1 |  |  |  |  |
| 75 | 3 | 16,17 | 1 |  |  |  |  |
| 76 | 3 | 12 | 2 |  |  |  |  |
| 77 | 3 | 13 | 2 |  |  |  |  |
| 78 | 3 | 14 | 2 |  |  |  |  |
| 79 | 3 | 15 | 2 |  |  |  |  |
| 80 | 3 | 16 | 2 |  |  |  |  |
| 81 | 3 | 17 | 2 |  |  |  |  |
| 82 | 3 | 18 | 2 |  |  |  |  |
| 83 | 3 | 19 | 2 |  |  |  |  |
| 84 | 3 | 20 | 2 |  |  |  |  |
| 85 | 3 | 21 | 2 |  |  |  |  |
| 86 | 3 | 22 | 2 |  |  |  |  |
| 87 | 3 | 23 | 2 |  |  |  |  |
| 88 | 3 | 12,13 | 2 |  |  |  |  |
| 89 | 3 | 14,15 | 2 |  |  |  |  |
| 90 | 3 | 16,17 | 2 |  |  |  |  |
| 91 | 3 | 18,19 | 2 |  |  |  |  |
| 92 | 3 | 20,21 | 2 |  |  |  |  |
| 93 | 3 | 22,23 | 2 |  |  |  |  |
| 94 | 1 | 12 | 2 |  |  |  |  |
| 95 | 1 | 13 | 2 |  |  |  |  |
| 96 | 1 | 18 | 2 |  |  |  |  |
| 97 | 1 | 19 | 2 |  |  |  |  |
| 98 | 1 | 12,13 | 2 |  |  |  |  |
| 99 | 1 | 18,19 | 2 |  |  |  |  |
| 100 | 2 | 12,13 | 2 |  |  |  |  |
| 101 | 2 | 14,15 | 2 |  |  |  |  |
| 102 | 2 | 18,19 | 2 |  |  |  |  |
| 103 | 2 | 20,21 | 2 |  |  |  |  |
| 104 | 1 | 0,1,12 | 1 |  |  |  |  |
| 105 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 106 | 2 | 0,1,12 | 1 |  |  |  |  |
| 107 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 108 | 2 | 2,3,14 | 1 |  |  |  |  |
| 109 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 110 | 3 | 0,1,12 | 1 |  |  |  |  |
| 111 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 112 | 3 | 2,3,14 | 1 |  |  |  |  |
| 113 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 114 | 3 | 4,5,16 | 1 |  |  |  |  |
| 115 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| 116 | 2 | 0,1,12 | 2 |  |  |  |  |
| 117 | 2 | 0,1,12,13 | 2 |  |  |  |  |
| 118 | 2 | 6,7,18 | 2 |  |  |  |  |
| 119 | 2 | 6,7,18,19 | 2 |  |  |  |  |
| 120 | 2 | 2,3,14 | 2 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 2 |  |  |  |  |
| 122 | 2 | 8,9,20 | 2 |  |  |  |  |
| 123 | 2 | 8,9,20,21 | 2 |  |  |  |  |
| 124 | 3 | 0,1,12 | 2 |  |  |  |  |
| 125 | 3 | 0,1,12,13 | 2 |  |  |  |  |
| 126 | 3 | 6,7,18 | 2 |  |  |  |  |
| 127 | 3 | 6,7,18,19 | 2 |  |  |  |  |
| 128 | 3 | 2,3,14 | 2 |  |  |  |  |
| 129 | 3 | 2,3,14,15 | 2 |  |  |  |  |
| 130 | 3 | 8,9,20 | 2 |  |  |  |  |
| 131 | 3 | 8,9,20,21 | 2 |  |  |  |  |
| 132 | 3 | 4,5,16 | 2 |  |  |  |  |
| 133 | 3 | 4,5,16,17 | 2 |  |  |  |  |
| 134 | 3 | 10,11,22 | 2 |  |  |  |  |
| 135 | 3 | 10,11,22,23 | 2 |  |  |  |  |
| 136 | 2 | 0,2,3 | 1 |  |  |  |  |
| 137-255 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-11: TCI selection

| Bit field mapped to index | TCI selection |
| --- | --- |
| 0 | The first indicated joint/DL TCI state is applied to the scheduled PDSCH |
| 1 | The second indicated joint/DL TCI state is applied to the scheduled PDSCH |
| 2 | Both indicated joint/DL TCI states are applied to the scheduled PDSCH |
| 3 | Reserved |

##### 7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	Antenna port(s) – 0, 4, 5, 6, 7 or 8 bits

-	0 bit if higher layer parameter antennaPortsFieldPresenceDCI-1-2 is not configured;

-	Otherwise 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10.

-	If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 and is configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

If a UE is not configured with higher layer parameter antennaPortsFieldPresenceDCI-1-2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4/7/8/9/10.

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentDCI-1-2 is not configured; otherwise 1 or 2 or 3 bits determined by higher layer parameter tci-PresentDCI-1-2 as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentDCI-1-2 is not configured for the CORESET used for the PDCCH carrying the DCI format 1_2,

-	the UE assumes tci-PresentDCI-1-2 is not configured for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentDCI-1-2 is configured for all CORESETs in the indicated bandwidth part with the same value configured for the CORESET used for the PDCCH carrying the DCI format 1_2.

-	TCI selection – 0 bit if higher layer parameter tciSelection-PresentInDCI is not configured; otherwise 2 bits according to Table 7.3.1.2.2-11.

-	SRS request – 0, 1, 2 or 3 bits

-	0 bit if the higher layer parameter srs-RequestDCI-1-2 is not configured;

-	1 bit as defined by Table 7.3.1.1.3-1 if the higher layer parameter srs-RequestDCI-1-2 = 1 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	2 bits if the higher layer parameter srs-RequestDCI-1-2 = 1 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second bit is defined by Table 7.3.1.1.3-1;

-	2 bits as defined by Table 7.3.1.1.2-24 if the higher layer parameter srs-RequestDCI-1-2 = 2 and for UEs not configured with supplementaryUplink in ServingCellConfig in the cell;

-	3 bits if the higher layer parameter srs-RequestDCI-1-2 = 2 and for UEs configured with supplementaryUplink in ServingCellConfig in the cell, where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24;

< Unchanged parts are omitted >