3GPP TSG-RAN WG1 Meeting #114	R1-2308716

Toulouse, France, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0437 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of specification support for MIMO enhancements on CSI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  | Date: |  | 2023-09-07 |
| Category: | B |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of specification support for MIMO Evolution for Downlink and Uplink. |  |  |  |  |  |
| Summary of change: |  | This document is introducing specification support for CSI |  |  |  |  |  |

| Consequences if not approved: | Specification does not support MIMO Evolution for Downlink and Uplink. |  |  |  |
| --- | --- | --- | --- | --- |
| Clauses affected: | 3.3, 5.1.6.1.1, 5.2.1, 5.2.1.1, 5.2.1.2, 5.2.1.4, 5.2.1.4.1, 5.2.1.4.2, 5.2.1.4.5 (new), 5.2.1.6, 5.2.2.2.8 (new), 5.2.2.2.9 (new), 5.2.2.2.10 (new), 5.2.2.2.11 (new), 5.2.2.3.1, 5.2.2.5.1c (new), 5.2.3, 5.2.5, 5.4 |  |  |  |
|  | Y | N |  |  |
| Other specs | X |  | Other core specifications | TS 38.211, TS 38.212, TS 38.213 |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

<omitted text>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BWP	Bandwidth part

CBG	Code block group

CJT	Coherent Joint Transmission

CLI	Cross Link Interference

CP	Cyclic prefix

CQI	Channel quality indicator

CPU	CSI processing unit

CRB	Common resource block

CRC	Cyclic redundancy check

CRI	CSI-RS Resource Indicator

CSI	Channel state information

CSI-RS	Channel state information reference signal

CSI-RSRP	CSI reference signal received power

CSI-RSRQ	CSI reference signal received quality

CSI-SINR	CSI signal-to-noise and interference ratio

CW	Codeword

DCI	Downlink control information

DL	Downlink

DM-RS	Demodulation reference signals

DRX	Discontinuous Reception

EPRE	Energy per resource element

IAB-MT	Integrated Access and Backhaul – Mobile Terminal

L1-RSRP	Layer 1 reference signal received power

LI	Layer Indicator

MCS	Modulation and coding scheme

NCJT	Non-Coherent Joint Transmission

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PSS	Primary Synchronisation signal

PUCCH	Physical uplink control channel

QCL	Quasi co-location

PMI	Precoding Matrix Indicator

PRB	Physical resource block

PRG	Precoding resource block group

PRS	Positioning reference signal

PT-RS	Phase-tracking reference signal

RB	Resource block

RBG	Resource block group

RI	Rank Indicator

RIV	Resource indicator value

RS	Reference signal

SCI	Sidelink control information

SLIV	Start and length indicator value

SR	Scheduling Request

SRS	Sounding reference signal

SS	Synchronisation signal

SSS	Secondary Synchronisation signal

SS-RSRP	SS reference signal received power

SS-RSRQ	SS reference signal received quality

SS-SINR	SS signal-to-noise and interference ratio

TB	Transport Block

TCI	Transmission Configuration Indicator

TDM	Time division multiplexing

TDCP	Time Domain Channel Properties

UE	User equipment

UL	Uplink

<omitted text>

### 5.1.6	UE procedure for receiving reference signals

<omitted text>

##### 5.1.6.1.1	CSI-RS for tracking

A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same.

-	For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.

-	For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as:

-	Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location.

-	Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being configured with qcl-Type set to 'typeA' and 'typeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, where beamSwitchTiming is UE reported value defined in [13, TS 38.306], the reported value is one of the values of {14, 28, 48}∙2max(0, μCSIRS-3), and the beam switching timing delay d is defined in Table 5.2.1.5.1a-1 if  µPDCCH < µCSIRS , else d is zero. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set.

A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'.

A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'tdcp' or 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a CSI-ReportConfig for periodic NZP CSI-RS resource set configured with trs-Info, except for reportQuantity set to 'tdcp'.

A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition.

Each CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions:

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping.

-	if carrier Ngridsize,μ=52, NBWP,isize=52, μ=0 and the carrier is configured in paired spectrum, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is X resource blocks, where X ≥ 28 resource blocks if the UE indicates trs-AddBW-Set1 for the trs-AdditionalBandwidth capability for CSI-RS for tracking or addBW-Set1 for the aperiodicCSI-RS-AdditionalBandwidth capability for aperiodic CSI-RS for fast SCell activation and X ≥ 32 if the UE indicates trs-AddBW-Set2 for the AdditionalBandwidth capability for CSI-RS for tracking or addBW-Set2 for the aperiodicCSI-RS-AdditionalBandwidth capability for aperiodic CSI-RS for fast SCell activation; in these cases, if the UE is configured with CSI-RS comprising X<52 resource blocks, the UE does not expect that the total number of PRBs allocated for DL transmissions but not overlapped with the PRBs carrying CSI-RS for tracking is more than 4, where all CSI-RS resource configurations shall span the same set of resource blocks; otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access in FR1, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks.

-	the UE is not expected to be configured with the periodicity of  slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks.

-	the periodicity and slot offset for periodic NZP CSI-RS resources, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211].

-	same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources.

A UE in RRC_IDLE or RRC_INACTIVE can receive a higher layer configuration of TRS occasions via a trs-ResourceSetConfig.

-	For frequency range 1, the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon, then the UE may be configured with one or more TRS resource set(s), where a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.

-	For frequency range 2 the UE may be configured with one or more TRS resource set(s), where each TRS resource set configured by a TRS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot or by a TRS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter TRS-ResourceSet with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE:

-	the number of periodic NZP CSI-RS resources configured by a TRS-ResourceSet is given by nrofResources

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the TRS-ResourceSet and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211].

-	the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter nrofRBs, startingRB and frequencyDomainAllocation in a TRS-ResourceSet and applies to all resources in a TRS-ResourceSet. Bandwidth, nrofRBs, and the initial CRB index, startingRB, of the NZP CSI-RS resource configured by TRS-ResourceSet are not restricted by initial DL BWP.

-	UE is not required to receive TRS occasions outside the initial DL BWP.

-	the periodicity for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a TRS-ResourceSet. The slot offset given by the higher layer parameter periodicityAndOffset configured by a TRS-ResourceSet provides the location of the first slot containing the periodic NZP CSI-RS resources configured by by a TRS-ResourceSet.

-	the UE does not expect the TRS-ResourceSet to be configured with the periodicity of  slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks.

-	the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by TRS-ResourceSet to be same as the sub-carrier spacing of the initial DL BWP.

-	powerControlOffsetSS given by a TRS-ResourceSet applies to all resources in a TRS-ResourceSet.

-	the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a TRS-ResourceSet, is a SS/PBCH block, applies to all resources in a TRS-ResourceSet.

-	One or more scrambling IDs according to scramblingID-Info where if a single scramblingIDforCommon is configured, it applies to all NZP-CSI-RS resources in the resource set, otherwise, each NZP-CSI-RS resource is provided with a scrambling IDs according to scramblingIDperResourceListWith2 or scramblingIDperResourceListWith4.

-	the UE may assume the following quasi co-location type(s):

-	'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block.

For each TRS-ResourceSet the index of the associated bit in TRS availability indication field [5, TS 38.212], is given by the higher layer parameter indBitID.

<omitted text>

### 5.2.1	Channel state information framework

The procedures on aperiodic CSI reporting described in this clause assume that the CSI reporting is triggered by DCI format 0_1, but they equally apply to CSI reporting triggered by DCI format 0_2, by applying the higher layer parameter reportTriggerSizeDCI-0-2 instead of reportTriggerSize.

The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. CSI may consist of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), SS/PBCH Block Resource indicator (SSBRI), layer indicator (LI), rank indicator (RI), L1-RSRP, L1-SINR, CapabilityIndex or time-domain channel properties (TDCP).

For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex, TDCP a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig.

#### 5.2.1.1	Reporting settings

Each Reporting Setting CSI-ReportConfig is associated with a single downlink BWP (indicated by higher layer parameter BWP-Id) given in the associated CSI-ResourceConfig for channel measurement and contains the parameter(s) for one CSI reporting band: codebook configuration including codebook subset restriction, time-domain behavior, frequency granularity for CQI and PMI, measurement restriction configurations, and the CSI-related quantities to be reported by the UE such as the layer indicator (LI), L1-RSRP, L1-SINR, CRI, SSBRI (SSB Resource Indicator), CapabilityIndex and TDCP.

The time domain behavior of the CSI-ReportConfig is indicated by the higher layer parameter reportConfigType and can be set to 'aperiodic', 'semiPersistentOnPUCCH', 'semiPersistentOnPUSCH', or 'periodic'. For 'periodic' and 'semiPersistentOnPUCCH'/'semiPersistentOnPUSCH' CSI reporting, the configured periodicity and slot offset applies in the numerology of the UL BWP in which the CSI report is configured to be transmitted on. The higher layer parameter reportQuantity indicates the CSI-related, L1-RSRP-related, L1-SINR-related, CapabilityIndex-related or TDCP-related quantities to report. The reportFreqConfiguration indicates the reporting granularity in the frequency domain, including the CSI reporting band and if PMI/CQI reporting is wideband or sub-band. The timeRestrictionForChannelMeasurements parameter in CSI-ReportConfig can be configured to enable time domain restriction for channel measurements and timeRestrictionForInterferenceMeasurements can be configured to enable time domain restriction for interference measurements. The CSI-ReportConfig can also contain CodebookConfig, which contains configuration parameters for Type-I, Type II, Enhanced Type II CSI, Further Enhanced Type II Port Selection, Enhanced Type II for coherent joint transmission (CJT), Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI, or Further Enhanced Type II Port Selection for predicted PMI including codebook subset restriction when applicable, and configurations of group-based reporting. A UE is not expected to be configured with a CSI report setting associated with a dormant DL BWP if the reportConfigType is set to ‘aperiodic’.

#### 5.2.1.2	Resource settings

Each CSI Resource Setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), where the list is comprised of references to either or both of NZP CSI-RS resource set(s) and SS/PBCH block set(s) or the list is comprised of references to CSI-IM resource set(s). Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter BWP-id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

The time domain behavior of the CSI-RS resources within a CSI Resource Setting are indicated by the higher layer parameter resourceType and can be set to aperiodic, periodic, or semi-persistent. For periodic and semi-persistent CSI Resource Settings, when the UE is configured with groupBasedBeamReporting-r17, the number of CSI Resource Sets configured is S=2, otherwise the number of CSI-RS Resource Sets configured is limited to S=1, except for periodic CSI Resource Settings, when the UE is configured with TDCP reporting, for which the number of CSI-RS Resource Sets in the CSI Resource Setting for channel measurement is KTRS∈{1,2,3} and all the CSI-RS Resource Sets are configured with the higher layer parameter trs-Info. For periodic and semi-persistent CSI Resource Settings, the configured periodicity and slot offset is given in the numerology of its associated DL BWP, as given by BWP-id. When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs. When a UE is configured with multiple CSI-ResourceConfigs consisting the same CSI-IM resource ID, the same time-domain behavior shall be configured for the CSI-ResourceConfigs. All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior.

The following are configured via higher layer signaling for one or more CSI Resource Settings for channel and interference measurement:

-	CSI-IM resource for interference measurement as described in Clause 5.2.2.4.

-	NZP CSI-RS resource for interference measurement as described in Clause 5.2.2.3.1.

-	NZP CSI-RS resource for channel measurement as described in Clause 5.2.2.3.1.

The UE may assume that the NZP CSI-RS resource(s) for channel measurement and the CSI-IM resource(s) for interference measurement configured for one CSI reporting are resource-wise QCLed with respect to 'typeD'. When NZP CSI-RS resource(s) is used for interference measurement, the UE may assume that the NZP CSI-RS resource for channel measurement and the CSI- IM resource or NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'typeD'.

For TDCP measurement, one aperiodic or periodic CSI Resource Setting is configured, and the Resource Setting is for channel measurement on CSI-RS for tracking.

For L1-SINR measurement:

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel and interference measurement on NZP CSI-RS for L1-SINR computation. UE may assume that same 1 port NZP CSI-RS resource(s) with density 3 REs/RB is used for both channel and interference measurements.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement on SSB or NZP CSI-RS and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on 1 port NZP CSI-RS with density 3 REs/RB, where each SSB or NZP CSI-RS resource for channel measurement is associated with one CSI-IM resource or one NZP CSI-RS resource for interference measurement by the ordering of the SSB or NZP CSI-RS resource for channel measurement and CSI-IM resource or NZP CSI-RS resource for interference measurement in the corresponding resource sets. The number of SSB(s) or CSI-RS resources for channel measurement equals to the number of CSI-IM resources or the number of NZP CSI-RS resource for interference measurement.

-	UE may apply the SSB, or 'typeD' RS configured with qcl-Type set to 'typeD' to the NZP CSI-RS resource for channel measurement, as the reference RS for determining 'typeD' assumption for the corresponding CSI-IM resource or the corresponding NZP CSI-RS resource for interference measurement configured for one CSI reporting.

-	UE may expect that the NZP CSI-RS resource set for channel measurement and the NZP-CSI-RS resource set for interference measurement, if any, are configured with the higher layer parameter repetition.

#### 5.2.1.3	(void)

#### 5.2.1.4	Reporting configurations

The UE shall calculate CSI parameters (if reported) assuming the following dependencies between CSI parameters (if reported)

-	LI shall be calculated conditioned on the reported CQI, PMI, RI and CRI

-	CQI shall be calculated conditioned on the reported PMI, RI and CRI

-	PMI shall be calculated conditioned on the reported RI and CRI

-	RI shall be calculated conditioned on the reported CRI.

The Reporting configuration for CSI can be aperiodic (using PUSCH), periodic (using PUCCH) or semi-persistent (using PUCCH, and DCI activated PUSCH). The CSI-RS Resources can be periodic, semi-persistent, or aperiodic. Table 5.2.1.4-1 shows the supported combinations of CSI Reporting configurations and CSI-RS Resource configurations and how the CSI Reporting is triggered for each CSI-RS Resource configuration. Periodic CSI-RS is configured by higher layers. Semi-persistent CSI-RS is activated and deactivated as described in Clause 5.2.1.5.2. Aperiodic CSI-RS is configured and triggered/activated as described in Clause 5.2.1.5.1.

Table 5.2.1.4-1: Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations.

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

When the UE is configured with higher layer parameter NZP-CSI-RS-ResourceSet and when the higher layer parameter repetition is set to 'off', the UE shall determine a CRI from the supported set of CRI values as defined in Clause 6.3.1.1.2 of [5, TS 38.212] and report the number in each CRI report. When the higher layer parameter repetition for a CSI-RS Resource Set for channel measurement is set to 'on', CRI for the CSI-RS Resource Set for channel measurement is not reported. CRI reporting is not supported when the higher layer parameter codebookType is set to 'typeII', 'typeII-PortSelection', 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'.

For a periodic or semi-persistent CSI report on PUCCH, the periodicity TCSI (measured in slots) and the slot offset Toffset are configured by the higher layer parameter reportSlotConfig. Unless specified otherwise, the UE shall transmit the CSI report in frames with SFN nf and slot number within the frame ns,fμ satisfying

Nslotframe,μnf+ns,fμ-Toffsetmod TCSI=0

where  μ is the SCS configuration of the UL BWP the CSI report is transmitted on.

For a semi-persistent CSI report on PUSCH, the periodicity TCSI (measured in slots) is configured by the higher layer parameter reportSlotConfig. Unless specified otherwise, the UE shall transmit the CSI report in frames with SFN nf and slot number within the frame ns,fμ satisfying

Nslotframe,μ(nf-nfstart) +ns,fμ-ns,fstartmod TCSI=0

where nfstart and ns,fstart are the SFN and slot number within the frame respectively of the initial semi-persistent PUSCH transmission according to the activating DCI.

For a semi-persistent or aperiodic CSI report on PUSCH, the allowed slot offsets are configured by the following higher layer parameters:

-	if triggered/activated by DCI format 0_2 and the higher layer parameter reportSlotOffsetListDCI-0-2 or reportSlotOffsetListDCI-0-2-r17 is configured, the allowed slot offsets are configured by reportSlotOffsetListDCI-0-2 or reportSlotOffsetListDCI-0-2-r17, and

-	if triggered/activated by DCI format 0_1 and the higher layer parameter reportSlotOffsetListDCI-0-1 or reportSlotOffsetListDCI-0-2-r17 is configured, the allowed slot offsets are configured by reportSlotOffsetListDCI-0-1 or reportSlotOffsetListDCI-0-2-r17, and

-	otherwise, the allowed slot offsets are configured by the higher layer parameter reportSlotOffsetList or reportSlotOffsetList-r17.

The offset is selected in the activating/triggering DCI.

For CSI reporting, a UE can be configured via higher layer signaling with one out of two possible subband sizes, where a subband is defined as  contiguous PRBs and depends on the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2.

Table 5.2.1.4-2: Configurable subband sizes

| Bandwidth part (PRBs) | Subband size (PRBs) |
| --- | --- |
| 24 – 72 | 4, 8 |
| 73 – 144 | 8, 16 |
| 145 – 275 | 16, 32 |

The reportFreqConfiguration contained in a CSI-ReportConfig indicates the frequency granularity of the CSI Report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates:

-	the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported.

-	A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource.

-	If a CSI-IM resource is linked to the CSI Report Setting, a UE is not expected to be configured with csi-ReportingBand which contains a subband where not all PRBs in the subband have the CSI-IM REs present.

-	wideband CQI or subband CQI reporting, as configured by the higher layer parameter cqi-FormatIndicator. When wideband CQI reporting is configured, a wideband CQI is reported for each codeword for the entire CSI reporting band. When subband CQI reporting is configured, one CQI for each codeword is reported for each subband in the CSI reporting band.

-	wideband PMI or subband PMI reporting as configured by the higher layer parameter pmi-FormatIndicator. When wideband PMI reporting is configured, a wideband PMI is reported for the entire CSI reporting band. When subband PMI reporting is configured, except with 2 antenna ports, a single wideband indication (i1 in Clause 5.2.2.2) is reported for the entire CSI reporting band and one subband indication (i2 in clause 5.2.2.2) is reported for each subband in the CSI reporting band. When subband PMIs are configured with 2 antenna ports, a PMI is reported for each subband in the CSI reporting band.

-	a UE is not expected to be configured with pmi-FormatIndicator if codebookType is set to 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'.

A CSI Reporting Setting is said to have a wideband frequency-granularity if

-	reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or

-	reportQuantity is set to 'cri-RI-PMI-CQI', codebookType is set to 'typeII-PortSelection-r17', 'typeII-CJT-PortSelection-r18' or 'typeII-Doppler-PortSelection-r18' with M=1 and cqi-FormatIndicator is set to 'widebandCQI', or

-	reportQuantity is set to 'cri-RI-i1' or

-	reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or

-	reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR' or 'cri-RSRP-Index' or 'ssb-Index-RSRP-Index' or 'cri-SINR-Index', or 'ssb-Index-SINR-Index', or

-	reportQuantity is set to 'tdcp'

otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity.

A CSI Reporting Setting with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, can be configured with wideband frequency-granularity only if csi-ReportMode is set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 is set to X=0, as described in clause 5.2.1.4.2.

If the UE is configured with a CSI Reporting Setting for a bandwidth part with fewer than 24 PRBs, the CSI reporting setting is expected to have a wideband frequency-granularity, and, if applicable, the higher layer parameter codebookType is set to 'typeI-SinglePanel'.

The first subband size is given by  and the last subband size given by  if  and if

If a UE is configured with semi-persistent CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic or semi-persistent. If a UE is configured with aperiodic CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic, semi-persistent or aperiodic.

A UE configured with DCI format 0_1 or 0_2 does not expect to be triggered with multiple CSI reports with the same CSI-ReportConfigId.

##### 5.2.1.4.1	Resource Setting configuration

For aperiodic CSI, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig not configured with groupBasedBeamReporting-r17 is linked to periodic, or semi-persistent, or aperiodic resource setting(s):

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on NZP CSI-RS.

-	When three Resource Settings are configured, the first Resource Setting (higher layer parameter resourcesForChannelMeasurement) is for channel measurement, the second one (given by higher layer parameter csi-IM-ResourcesForInterference) is for CSI-IM based interference measurement and the third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for NZP CSI-RS based interference measurement.

For aperiodic CSI, and for periodic and semi-persistent CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is linked to periodic or semi-persistent, setting(s):

-	When one Resource Setting is configured, the Resource setting is given by resourcesForChannelMeasurement for L1-RSRP measurement. In such a case, the number of configured CSI Resource Sets in the Resource Setting is S=2

For aperiodic CSI, and for aperiodic CSI resource settings, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig configured with groupBasedBeamReporting-r17 is associated with resourcesForChannel and resourcesForChannel2, which correspond to first and second resource sets, respectively, for L1-RSRP measurement.

For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s):

-	When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM. For L1-SINR computation, the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourceForInterference) is used for interference measurement performed on CSI-IM or on NZP CSI-RS.

For aperiodic CSI, a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp' is expected to be configured with one CSI Resource Setting (given by higher layer parameter resourcesForChannelMeasurement). The CSI Resource Setting can be configured with trs-Info and it may be periodic, with KTRS∈{1, 2, 3} CSI-RS Resource Sets or aperiodic, with a single CSI-RS Resource Set. The support of KTRP=2 or 3 is subject to UE capability indication. For a periodic CSI-ResourceConfig, the UE can assume that all the CSI-RS resources in the KTRS CSI-RS Resource Sets share the same QCL-TypeA/C and, if applicable, TypeD. The UE expects that all the CSI-RS resources in the CSI-RS Resource Set(s) are configured with the same bandwidth and subcarrier locations.

A UE is not expected to be configured with more than one CSI-RS resource in resource set for channel measurement for a CSI-ReportConfig with the higher layer parameter codebookType set to 'typeII', 'typeII-PortSelection', 'typeII-r16', 'typeII-PortSelection-r16', or 'typeII-PortSelection-r17'. A UE is not expected to be configured with more than 64 NZP CSI-RS resources and/or SS/PBCH block resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'cri-RI-CQI', 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index' or 'ssb-Index-SINR- Index'. If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources.

A UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' is expected to be configured with 1≤K≤4 CSI-RS resources in a resource set for channel measurement. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet.

A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet.

An NZP CSI-RS Resource Set for channel measurement with 2≤Ks≤8 resources can be configured with two Resource Groups, with K1≥1 resources in Group 1 and K2≥1 resources in Group 2, such that K1+K2=Ks, and with N∈{1,2} Resource Pairs. Each Resource Pair consists of one resource from Group 1 and one resource from Group 2. The same resource can be associated with two Resource Pairs in frequency range 1 but not in frequency range 2.

Except for L1-SINR, if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for interference measurement. Except for L1-SINR, the UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set.

For CSI measurement(s) other than L1-SINR, a UE assumes:

-	each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer.

-	all interference transmission layers on NZP CSI-RS ports for interference measurement take into account the associated EPRE ratios configured in 5.2.2.3.1;

-	other interference signal on REs of NZP CSI-RS resource for channel measurement, NZP CSI-RS resource for interference measurement, or CSI-IM resource for interference measurement.

For L1-SINR measurement with dedicated interference measurement resources, a UE assumes:

-	the total received power on dedicated NZP CSI-RS resource for interference measurement or dedicated CSI-IM resource for interference measurement corresponds to interference and noise.

##### 5.2.1.4.2	Report Quantity configurations

A UE may be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'cri-RI-LI-PMI-CQI', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index' or 'tdcp'.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', then the UE shall not report any quantity for the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', the UE shall report a preferred precoder matrix for the entire reporting band, or a preferred precoder matrix per subband, according to Clause 5.2.2.2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1-CQI',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band. The CQI is calculated conditioned on the reported assuming PDSCH transmission with  precoders (corresponding to the same but different  in Clause 5.2.2.2.1), where the UE assumes that one precoder is randomly selected from the set of  precoders for each PRG on PDSCH, where the PRG size for CQI calculation is configured by the higher layer parameter pdsch-BundleSizeForCSI.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI',

-	if the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence  of port indices, where  are the CSI-RS port indices associated with rank ν and  where is the number of ports in the CSI-RS resource. The UE shall only report RI corresponding to the configured fields of PortIndexFor8Ranks.

-	if the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices  are associated with ranks  where  is the number of ports in the CSI-RS resource.

-	When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by .

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance nrofReportedGroups, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-SINR', 'ssb-Index-SINR', 'cri-SINR- Index' or 'ssb-Index-SINR- Index',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'tdcp'

-	the value of Y∈1,2,3,4 is configured by higher layer parameter Y, and Y delay values, D1,…,DY, are configured by higher layer parameter D, such that the UE is expected to report the amplitude of TDCP measurement, as defined in Clause 5.1 of [7, TS 38.215], for each of the configured delays. Values of Y>1 can be configured subject to UE capability. The configurable delay values are Di∈4symbols1,2,3,4,5,6,10slots, i=1,…,Y, where the value Di=10 slots is restricted to subcarrier spacing configuration μ≥1, the values other than Di=10 slots are applicable to subcarrier spacing configurations μ≥0, and where the values Di>Dbasic can be configured subject to UE capability, with Dbasic=1 slot.

-	For Y>1, if the higher layer parameter phase is configured, the UE is expected to report the amplitude and phase of TDCP measurement for each of the configured delays, if supported by UE capability.

Except for a CSI-ReportConfig configured with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18', if the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RI-LI-PMI-CQI', 'cri-SINR', or 'cri-SINR- Index ', and Ks>1 resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured for CSI-ReportConfig with reportQuantity set to 'cri-SINR' or 'cri-SINR- Index ') for interference measurement. If Ks=2 CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If 2<Ks≤8 CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 1≤K≤4 resources, each resource can contain, at most, 32 CSI-RS ports.

Subject to UE capability, a UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter N4. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter d, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2}δ is configured by higher layer parameter delta, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability.

-	For N4=1, the UE is expected to report a predicted PMI for slot interval [l,l+d-1] and the slot offset value δ=-nCSI_ref can be configured only for d>1. A UE can be configured with N4=1 if the higher layer parameter codebookType is set to 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18'.

-	The reported CQI is associated with slot l and the reported PMI.

-	For N4>1, the UE is expected to report a PMI which indicates predicted precoder matrices associated with slot intervals [l+j⋅d, l+(j+1)⋅d-1], for j=0,…,N4-1. A UE can be configured with N4>1 if the higher layer parameter codebookType is set to 'typeII-Doppler-r18'.

-	The UE is configured by higher layer parameter TDCQI to report X∈{1,2} CQIs for each subband in the CSI reporting band, if cqi-FormatIndicator is set to 'subbandCQI', or X∈{1,2}  CQIs for the entire CSI reporting band, if cqi-FormatIndicator is set to 'widebandCQI'. For X=2, the second CQI includes a 4-bit wideband CQI index and, if subband CQI reporting is configured, a 2-bit subband CQI index, calculated independently from the first CQI, as described in Clause 5.2.2.1, and the two CQIs are reported in the same CSI report.

-	If the higher layer parameter TDCQI is set to '1-1', X=1 and the CQI is associated with slot l and the precoder matrices for slot interval [l,l+d-1].

-	If the higher layer parameter TDCQI is set to '1-2', X=1 and the CQI is associated with slot l and the precoder matrices for slot interval [l,l+d-1] and with slot l+N4⋅d-1 and the precoder matrices for slot interval l+(N4-1⋅d,l+N4⋅d-1].

-	If the higher layer parameter TDCQI is set to '2', X=2. The first CQI is associated with slot l and the precoder matrices for slot interval [l,l+d-1]. The second CQI is associated with slot l+N42⋅d and the precoder matrices for slot interval l+N42⋅d,l+N42+1⋅d-1.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs:

-	each resource can contain, subject to UE capability, at most 32 CSI-RS ports. For two Resource Groups with Ki  resources (i=1,2), if maxK1,K2=1, the resource in NZP-CSI-RS-ResourceSet shall contain at most 32 CSI-RS ports; if  maxK1,K2=2, each resource in NZP-CSI-RS-ResourceSet shall contain at most 16 CSI-RS ports; if  2<maxK1,K2<8, each resource in NZP-CSI-RS-ResourceSet shall contain at most 8 CSI-RS ports.

-	each of the N Resource Pairs is associated to a CRI value.

-	The CSI-ReportConfig may be configured with higher layer parameter sharedCMR. M1 and M2 are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with M=M1+M2, such that the total number of CRI values configured for the CSI-ReportConfig is M+N.

-	If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0}, M1=M2=0; otherwise,

-	if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{1,2}, or if csi-ReportMode is set to 'Mode2',

-	if sharedCMR is configured: M1=K1 and M2=K2; otherwise

-	if sharedCMR is not configured, only the resources in Group 1 and Group 2 that are not referred to in any Resource Pair are associated to M CRI values other than the N CRIs defined above.

-	If interference measurement is performed on CSI-IM, M+N resources are configured in the corresponding csi-IM-ResourceSet. The M resources for channel measurement defined above are resource-wise associated with the first M CSI-IM resources by the ordering of the CSI-RS resources and CSI-IM resources in the corresponding Resource Set. The N Resource Pairs for channel measurement are associated to the last N CSI-IM resources by the ordering of the CSI-RS Resource Pairs and CSI-IM resources in the CSI-IM Resource Set. The UE may assume that the two CSI-RS resources for channel measurement in a Resource Pair and the associated CSI-IM resource for interference measurement are resource-wise QCLed with respect to 'typeD'.

-	The UE is not expected to be configured with NZP CSI-RS for interference measurement other than the NZP CSI-RS resources for channel measurement configured in the N Resource Pairs.

-	The UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel', and

-	The UE shall derive the CSI parameters other than CRI(s) conditioned on the reported CRI(s), as follows:

-	If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0,1,2}, X+1 CRI(s) are reported:

-	one CRI k1 k1≥0 corresponds to the configured (k1+1)-th entry of the associated N Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (M+k1+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report two RIs, two PMIs, two LIs (if configured), associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1+1)-th Resource Pair, and one CQI; and

-	if X=1, one CRI k2 (k2≥0) corresponds to the configured (k2+1)-th entry of the associated M resources in the corresponding CSI-RS Resource Set for channel measurement, and (k2+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k2; or

-	if X=2, one CRI k2 (k2≥0) corresponds to the configured (k2+1)-th entry of the associated M1 resources in Group 1 of the corresponding CSI-RS Resource Set for channel measurement, and (k2+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured, and one CRI k3 (k3≥0) corresponds to the configured (k3+1)-th entry of the associated M2 resources in Group 2 of the corresponding CSI-RS Resource Set for channel measurement, and (M1+k3+1)-th entry of the corresponding CSI-IM Resource Set, if configured. The UE shall report one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k2 and one RI, one PMI, one LI (if configured) and one or two CQIs conditioned on CRI k3.

-	If the higher layer parameter csi-ReportMode is set to 'Mode2', one CRI k1 (k1≥0) is reported, which corresponds to the (k1+1)-th entry of the M+N resources or Resource Pairs in the corresponding CSI-RS Resource Set for channel measurement, and (k1+1)-th entry of the associated resources in the corresponding CSI-IM Resource Set, if configured. The first M codepoints of the CRI correspond to resources associated to Group 1 and Group 2. The last N codepoints of the CRI correspond to the N configured Resource Pairs. The UE shall report one RI, one PMI, one LI, if configured, and one or two CQIs conditioned on CRI k1 if k1<M; or two RIs, two PMIs, two LIs, if configured, associated to the resource in Group 1 and the resource in Group 2, respectively, of the (k1-M+1)-th Resource Pair, and one CQI, otherwise.

-	For a reported CRI corresponding to an entry of the N Resource Pairs configured in the corresponding CSI-RS Resource Set for channel measurement:

-	the UE shall not report a total number of layers larger than four.

-	the two RIs are reported with a joint RI index corresponding to one of the four rank combinations: 1,1, 1,2, 2,1, {2,2}.

-	The CodebookConfig in CSI-ReportConfig can be configured with two RI restriction parameters. One parameter applies to a reported RI when conditioned on a CRI corresponding to an entry of the M CSI-RS resources defined above. Another parameter applies to a reported joint RI index when conditioned on a CRI corresponding to an entry of the N Resource Pairs and indicates one or more of the four rank combinations that are allowed to correspond to the reported PMIs and RIs.

-	The CodebookConfig in CSI-ReportConfig can be configured with two Codebook Subset Restrictions. The first restriction applies to a reported PMI associated to a CSI-RS resource in Group 1. The second restriction applies to a reported PMI associated to a CSI-RS resource in Group 2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP' or 'ssb-Index-RSRP- Index', the UE shall report SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-SINR' or 'ssb-Index-SINR- Index', the UE shall derive L1-SINR conditioned on the reported SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList  in the corresponding CSI-SSB-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet (if configured) for interference measurement.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', ' cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', then the UE is not expected to be configured with more than 8 CSI-RS resources in a CSI-RS resource set contained within a resource setting that is linked to the CSI-ReportConfig, except when the UE is configured with a CSI-ReportConfig with the higher layer parameter N4, reportQuantity set to 'cri-RI-PMI-CQI' and the corresponding CSI-RS resource set for channel measurement is aperiodic with K=12 resources.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-LI-PMI-CQI', UE does not expect the CSI-ReportConfig to be configured with higher layer parameter codebookType set to 'typeII-r16' or 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'.

If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index' or 'cri-SINR- Index and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting.

The LI indicates which column of the precoder matrix of the reported PMI corresponds to the strongest layer of the codeword corresponding to the largest reported wideband CQI. If two wideband CQIs are reported and have equal value, the LI corresponds to strongest layer of the first codeword. If the UE is configured with a CSI-ReportConfig with reportQuantity set to 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with two Resource Groups and N Resource Pairs, and the UE reports a CRI associated to a Resource Pair, and a rank combination {ν1,ν2}, the first LI indicates which column of the precoder matrix of the first reported PMI corresponds to the strongest of the first ν1 layers of the codeword and the second LI indicates which column of the precoder matrix of the second reported PMI corresponds to the strongest of the last ν2 layers of the codeword.

For operation with shared spectrum channel access in FR1, or in FR2-2 when the UE is provided ChannelAccessMode2-r17 = 'enabled', if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging two or more instances of any periodic or semi-persistent nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement or for interference measurement located in different DL transmissions,

-	the instances of the nzp-CSI-RS-Resources are not in the same channel occupancy duration indicated by DCI format 2_0, if the UE is provided at least one of SlotFormatIndicator or co-DurationList; or

-	the instances of the nzp-CSI-RS-Resources occur within a set of consecutive symbols which are not all occupied by PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats, if any, and the corresponding PDCCH(s), if the UE is neither provided with CO-DurationsPerCell nor SlotFormatIndicator, but is provided with csi-RS-ValidationWithDCI

-	the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

If the UE is configured with the higher layer parameter SSB-MTC-AdditionalPCI, the UE is allowed to report in a single reporting instance up to four SSBRIs for each report setting, where SSB resources are associated with PCI indices referring to the PCI of the serving cell and PCI(s) different from the PCI of the serving cell within the set of PCIs configured.

<omitted text>

##### 5.2.1.4.5	TDCP Reporting

For a CSI-ReportConfig with higher layer parameter reportQuantity set to ‘tdcp’ and higher layer parameters Y≥1 and {D1,…,DY}, the reported TDCP amplitude(s) corresponding to the Y configured delays are indicated by

kTDCP=[k1…kY]

ki∈{0,1,…,15}

and the corresponding amplitude values are obtained from: 1-ai, for i=1,…,Y, where the mapping from ki to ai is given in Table 5.2.1.4.5-1.

Table 5.2.1.4.5-1: Mapping of elements of kTDCP: ki to TDCP amplitudes: 1-ai

| ki; ai. 0; 1256. 1; 11282. 2; 1128. 3; 1642. | ki; ai. 4; 164. 5; 1322. 6; 132. 7; 1162. | ki; ai. 8; 116. 9; 182. 10; 18. 11; 142. | ki; ai. 12; 14. 13; 122. 14; 12. 15; 12. |
| --- | --- | --- | --- |

For Y>1, if the higher layer parameter phase is configured, the reported TDCP phases are indicated by

cTDCP=[c1…cY]

ci∈{0,1,…,15}

and the corresponding phase values are given by: ej2πci16.

<omitted text>

#### 5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds.

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings. Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured

-	OCPU=1  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR', 'cri-RSRP- Index', 'ssb-Index-RSRP- Index', 'cri-SINR- Index', 'ssb-Index-SINR- Index ' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured)

-	OCPU=Y+1⋅X, for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'tdcp' and with number of delays Y configured by higher layer parameter Y, where the value of X∈{1, 2} is reported by UE capability.

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if max{ µPDCCH, µCSI-RS, µUL} ≤ 3, and if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	if a CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to mTRP-CSI-numCPU-r17 and M is defined in clause 5.2.1.4.2,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 1<NTRP≤4 resources, OCPU=X⋅NTRP, where X∈{1, 1.5, 2} is reported by UE capability indication,

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18',

-	if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with K CSI-RS resources, OCPU=Y1⋅K, where Y1∈{2/3, 1, 2, 3} is reported by UE capability indication,

-	if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with a single CSI-RS resource, OCPU=4 for N4=1 and OCPU=Y2⋅N4≥4, for N4>1, where the value of N4 is configured by the higher layer parameter N4, and Y2∈{2/3, 1, 2, 3} is reported by UE capability indication,

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation.

where (Z3,Z3') are defined in the table 5.4-2.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released.

If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, if a CSI-RS resource is referred X times by one of the M CSI-RS resources, where M is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times.

For a periodic or semi-persistent CSI-RS resource in a CSI-RS resource set for channel measurement linked to a CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted KP times, where the value of KP∈{1,2,4} is indicated by UE capability.

<omitted text>

### 5.2.2	Channel state information

<omitted text>

##### 5.2.2.2.8	Enhanced Type II codebook for CJT

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, and the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-r18'

-	the values of N1, N2  and O1, O2 are the same for all NTRP CSI-RS resources and configured with the higher layer parameter. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2 for each of the NTRP CSI-RS resources.

-	A set of NL∈{1,2,4} combinations of values of {L1,…,LNTRP} is configured by the higher layer parameter paramCombination-CJT-L-r18, where the value of NL is configured by the higher layer parameter numberOfSDCombinations and the mapping is given in Table 5.2.2.2.8-1, with the value of Ln corresponding to CSI-RS resource n, for n=1,…,NTRP. A single value of pν and β is configured by the higher layer parameter paramCombination-CJT-r18, where the mapping is given in Table 5.2.2.2.8-2. The configurable combinations of {L1,…,LNTRP} and {pυ,β} are marked with 'x' in Table 5.2.2.2.8-3.

-	The UE is not expected to be configured with paramCombination-CJT-L-r18 equal to

-	2, 3, 5, 6, 7, 9, 10, 11, 12, 14, 15 or 16 when PCSI-RS=4,

-	3 when PCSI-RS<32

-	3 when higher layer parameter typeII-CJT-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	3 when R=2.

-	If NL>1, the UE is expected to select one of the NL configured combinations of {L1,…,LNTRP} and report the index of the selected combination, where the index value 0 corresponds to the first configured combination and the index value NL-1 corresponds to the NL-th configured combination. If NL=1, a single combination of {L1,…,LNTRP} is configured and the selection is not reported.

Table 5.2.2.2.8-1: Codebook parameter configurations for {L1,…, LNTRP}

| NTRP | paramCombination-CJT-L-r18 | {L1,…, LNTRP} |
| --- | --- | --- |
| 1 | 1 | {2} |
|  | 2 | {4} |
|  | 3 | {6} |
| 2 | 4 | {2,2} |
|  | 5 | {2,4} |
|  | 6 | {4,2} |
|  | 7 | {4,4} |
| 3 | 8 | {2,2,2} |
|  | 9 | {2,2,4} |
|  | 10 | {2,4,2} |
|  | 11 | {4,2,2} |
|  | 12 | {4,4,4} |
| 4 | 13 | {2,2,2,2} |
|  | 14 | {2,2,2,4} |
|  | 15 | {2,2,4,4} |
|  | 16 | {4,4,4,4} |

Table 5.2.2.2.8-2: Codebook parameter configurations for {pυ,β}

| paramCombination-CJT-r18 | pυ |  | β |
| --- | --- | --- | --- |
|  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 1/8 | 1/16 | ¼ |
| 2 | 1/8 | 1/16 | ½ |
| 3 | ¼ | 1/8 | ¼ |
| 4 | ¼ | 1/8 | ½ |
| 5 | ¼ | ¼ | ¾ |
| 6 | ½ | ¼ | ½ |
| 7 | ½ | ½ | ½ |

Table 5.2.2.2.8-3: Configurable combinations of {L1,…, LNTRP} and {pυ,β}

| paramCombination-CJT-L-r18 | paramCombination-CJT-r18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  | x | x |  |  |  |
| 2 |  |  | x | x | x | x |  |
| 3 |  |  |  | x | x |  |  |
| 4 | x |  |  |  |  |  |  |
| 5 | x |  |  |  |  |  |  |
| 6 | x |  |  |  |  |  |  |
| 7 |  | x |  | x |  |  | x |
| 8 | x | x |  |  |  |  |  |
| 9 | x | x |  |  |  |  |  |
| 10 | x | x |  |  |  |  |  |
| 11 | x | x |  |  |  |  |  |
| 12 | x | x | x | x | x |  | x |
| 13 | x |  |  |  |  |  |  |
| 14 | x |  |  |  |  |  |  |
| 15 |  |  |  | x | x |  |  |
| 16 |  | x |  | x | x |  |  |

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-CJT-r18, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-CJT-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-CJT-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N=NTRP. Otherwise, the UE is expected to select N CSI-RS resources, with 1≤N≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N selected CSI-RS resources corresponds to the nonzero bit with lowest index.

The PMI value for the N selected CSI-RS resources corresponds to the codebook indices of i1 and i2 where

i1=i1,1i1,2i1,5i1,6,1i1,7,1i1,8,1 i1,9                                                                                 υ=1i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2     i1,7,2     i1,8,2     i1,9υ=2i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2     i1,7,2     i1,8,2     i1,6,3     i1,7,3     i1,8,3     i1,9   υ=3i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2i1,7,2     i1,8,2     i1,6,3i1,7,3i1,8,3     i1,6,4i1,7,4     i1,8,4i1,9υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                                           υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                             υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The precoding matrices indicated by the PMI are determined from j=1NLσj+Mυ vectors, where {σ1,…,σN} are the indices of the N selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN≤NTRP, and {Lσ1, …, LσN} are the corresponding values from the selected combination of {L1, …, LNTRP}.

The Lσj vectors, vm1,ji,m2,j(i), i=0,1,…,Lσj-1, corresponding to the j-th selected CSI-RS resource, for j=1,…,N, are indicated by i1,1, i1,2, where

i1,1=[i1,1,1 … i1,1,N]

i1,1,j=[q1,j q2,j]

q1,j∈{0,1,…,O1-1}

q2,j∈{0,1,…,O2-1}

and

i1,2=[i1,2,1 …i1,2,N]

i1,2,j∈0,1,…, N1N2Lσj-1

Let

n1=[n1,1… n1,N]

n1,j=n1,j0…n1,jLσj-1

n1,j(i)∈{0,1,…,N1-1}

and

n2=[n2,1… n2,N]

n2,j=n2,j0…n2,jLσj-1

n2,j(i)∈{0,1,…,N2-1}

the index i1,2,j, for the j-th selected CSI-RS resource, j=1,…,N, is obtained from the Lσj indices nj(i)=N1n2,j(i)+ n1,j(i), i=0,…,Lσj-1, as described in Clause 5.2.2.2.3 for the indicator i1,2, obtained from the L indices n(i)=N1n2(i)+ n1(i), i=0,…,L-1. Vector vm1,ji,m2,j(i) is then derived from the indices m1,j(i)=O1n1,j(i)+q1,j and m2,j(i)=O2n2,j(i)+q2,j, as described in Clause 5.2.2.2.3 for vector vm1i,m2(i), derived from indices m1(i), m2(i).

The Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, for layer l=1,…,υ are common for all the N selected CSI-RS resources and are indicated by i1,5 (for N3>19) and i1,6,l for Mυ>1 and l=1,…, υ, which are obtained as described in Clause 5.2.2.2.5, where i1,6,l is derived from the selected vectors identified by the indices n3,l(f), f=0,1,…,Mυ-1. The Mυ vectors’ elements are given by

yt,l(f)=ej2πtn3,l(f)N3

for t=0,1,…N3-1, and f=0,1,…,Mυ-1.

If the higher layer parameter codebookMode is set to 'mode1', an offset dj is reported for the j-th selected CSI-RS resource, with j=2,…,N, relative to the first of the N selected CSI-RS resources. The N-1 reported offsets are common for all ν layers and are indicated by i1,9, given by

i1,9=[d2… dN]

dj∈{0, 1,…,N3O3-1}

where the value of O3∈{1,4} is configured by higher layer parameter numberOfO3. The offsets are represented by

ψ=[ψ2… ψN]

ψj=djO3

If codebookMode is set to 'mode2’, the offset indicator, i1,9, is not reported and ψj=0 for j=21,…,N.

The reference amplitude coefficient indicator i2,3,l, for layer l=1,…υ, is given by

i2,3,l=kl,01kl,11

kl,p(1)∈1,…,15

The reference amplitude coefficients for layer l=1,…,υ are represented by

pl(1)=[pl,01 pl,1(1)]

and the mapping from kl,p(1), to pl,p(1) is given in Table 5.2.2.2.5-2.

The amplitude coefficient indicator i2,4,l, for layer l=1,…,υ, is given by

i2,4,l=[i2,4,l,1…i2,4,l,N]

i2,4,l,j=kl,0,j(2)…kl,Mυ-1,j(2)

kl,f,j(2)=kl,0,f,j(2)…kl,2Lσj-1,f,j(2)

kl,i,f,j(2)∈0,…,7

The amplitude coefficients for layer l=1,…,υ are represented by

pl(2)=[pl,12 … pl,N(2)]

pl,j(2)=[pl,0,j2…pl,Mν-1,j(2)]

pl,f,j(2)=[pl,0,f,j2…pl,2Lσj-1,f,j(2)]

and the mapping from kl,i,f,j(2) to pl,i,f,j(2) is given in Table 5.2.2.2.5-3.

The phase coefficient indicator i2,5,l, for l=1,…,υ, is given by

i2,5,l=[i2,5,l,1…i2,5,l,N]

i2,5,l,j=cl,0,j…cl,Mυ-1,j

cl,f,j=cl,0,f,j…cl,2Lσj-1,f,j

cl,i,f,j∈0,…,15

The phase coefficients for layer l=1,…,υ are represented by

φl=[φl,1…φl,N]

φl,j=φl,0,j…φl,Mυ-1,j

φl,f,j=φl,0,f,j…φl,2Lσj-1,f,j

and the mapping from cl,i,f,j to φl,i,f,j is given by

φl,i,f,j=ej2πcl,i,f,j16

Let K0=2βM1j=1NLσj. The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported for layer l=1,…,υ, is indicated by i1,7,l

i1,7,l=[i1,7,l,1…i1,7,l,N]

i1,7,l,j=kl,0,j3…kl,Mυ-1,j3

kl,f,j3=kl,0,f,j3…kl,2Lσj-1,f,j3

kl,i,f,j(3)∈0,1

Let KlNZ=j=1Ni=02Lσj-1f=0Mυ-1kl,i,f,j(3)≤K0 be the number of nonzero coefficients for layer l=1,…,υ, the total number of nonzero coefficients is reported and given by KNZ=l=1υKlNZ≤2K0.

The indices of i2,4,l, i2,5,l and i1,7,l are associated to the N selected CSI-RS resources.

Let fl*∈0,1,…,Mυ-1 and il*∈0,1,…,2j=1NLσj-1 be the indices which identify the strongest coefficient of layer l, for  l=1,…,υ, i.e., the element kl,yl*,fl*,jl*(2) of i2,4,l, where the indices jl*∈{1,…,N}, yl*∈0,…,2Lσjl*-1 are such that il*=2k=1jl*-1Lσk+yl*. The codebook indices of n3,lkmi are remapped with respect to n3,l(fl*) as n3,l(f)=n3,l(f)-n3,l(fl*)mod N3kmi*=0, such that n3,l(fl*)=0, after remapping. The index f is remapped with respect to fl* as f=f-fl*mod Mυ, such that the index of the strongest coefficient is fl*=0, for each layer l=1,…,υ, after remapping. The indices of i2,4,l, i2,5,l and i1,7,l indicate amplitude coefficients, phase coefficients and bitmap after remapping.

The strongest coefficient of layer l=1,…, υ is identified by

i1,8,l∈0,1,…,2j=1NLσj-1

and is obtained as follows, where, for υ=1, κ1,I,0(3)=k1,i,0,j(3) and index I is such that I=2k=1j-1Lσk+i

i1,8,l=I=0i1*κ1,I,0(3)-1υ=1il*1<υ≤4

The amplitude and phase coefficient indicators are reported as follows:

-	kl,yl*Lσjl*(1)=15, kl,yl*,0,jl*(2)=7, kl,yl*,0,jl*(3)=1 and cl,yl*,0,jl*=0, for l=1,…,υ. The reference amplitude, kl,yl*Lσjl*(1), amplitude coefficient, kl,yl*,0,jl*(2) and phase coefficient, cl,yl*,0,jl*, are not reported for l=1,…,υ.

-	The reference amplitude, kl,yl*Lσjl*+1 mod 2(1), is reported for l=1,…,υ.

-	The KNZ-υ amplitude coefficients, kl,i,f,j(2), for which kl,i,f,j(3)=1, i≠yl*, f≠0, j≠jl* are reported.

-	The KNZ-υ phase coefficients, cl,i,f,j, for which kl,i,f,j(3)=1, i≠yl*, f≠0, j≠jl* are reported.

-	The remaining 2j=1NLσj⋅Mυ⋅ν-KNZ amplitude coefficients, kl,i,f,j(2), are not reported.

-	The remaining 2j=1NLσj⋅Mυ⋅ν-KNZ phase coefficients, cl,i,f,j, are not reported.

The codebooks for 1-4 layers are given in Table 5.2.2.2.8-4, where t=0,1,…,N3-1, is the index associated with the precoding matrix, l=1,…, υ is the layer index, and where, for coefficients with kl,i,f,j(3)=0, amplitude and phase are set to zero, i.e., pl,i,f,j(2)=0 and φl,i,f,j=0.

Table 5.2.2.2.8-4: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS of N selected CSI-RS resources

| Layers |  |
| --- | --- |
| υ=1 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t(1)=Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 |
| υ=2 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1, n3,2,p2(1),p22,φ2,ψ,t(2)=12Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,φ2,ψ,t2 |
| υ=3 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,n3,2,p2(1),p22,φ2,n3,3,p3(1),p32,φ3,ψ,t(3)=13Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,φ2,ψ,t2 Wq1,q2, n1, n2,n3,3,p3(1),p32,φ3,ψ,t3 |
| υ=4 | Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,n3,2,p2(1),p22,φ2,n3,3,p3(1),p32,φ3,n3,4,p4(1),p42,φ4,ψ,t(4)=12Wq1,q2, n1, n2,n3,1,p1(1),p12,φ1,ψ,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,φ2,ψ,t2 Wq1,q2, n1, n2,n3,3,p3(1),p32,φ3,ψ,t3 Wq1,q2, n1, n2,n3,4,p4(1),p42,φ4,ψ,t4 |
| Where Wq1,q2, n1, n2,n3,l,pl(1),pl2,φl,ψ,tl=1N1N2γt,li=0Lσ1-1vm1,1(i),m2,1(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f,1(2)φl,i,f,1i=0Lσ1-1vm1,1(i),m2,1(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+Lσ1,f,1(2)φl,i+Lσ1,f,1ej2πtψ2N3i=0Lσ2-1vm1,2(i),m2,2(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f,2(2)φl,i,f,2ej2πtψ2N3i=0Lσ2-1vm1,2(i),m2,2(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+Lσ2,f,2(2)φl,i+Lσ2,f,2⋮ej2πtψNN3i=0LσN-1vm1,N(i),m2,N(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f,N(2)φl,i,f,Nej2πtψNN3i=0LσN-1vm1,N(i),m2,N(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+LσN,f,N(2)φl,i+LσN,f,N γt,l=j=1Ni=02Lσj-1pl,iLσj12f=0Mυ-1yt,lfpl,i,f,j2φl,i,f,j2, l=1,2,3,4, and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, ψ, and from i2 to p1(1), p2(1), p3(1), p4(1), p1(2), p2(2), p3(2), p4(2), φ1, φ2, φ3, φ4 are as described above, including the ranges of the constituent indices of i1 and i2. |  |

The bitmap parameter n1-n2-codebookSubsetRestriction-CJT-r18 is configured per CSI-RS resource and for at least one of the NTRP CSI-RS resources, and it is configured as described in Clause 5.2.2.2.5, where only the bit values ‘00’ or ‘11’ of Table 5.2.2.2.5-6 are configurable. If parameter n1-n2-codebookSubsetRestriction-CJT-r18 is not configured for a CSI-RS resource, no restriction is applied to the selection of vectors vm1,ji,m2,j(i) corresponding to that resource.

##### 5.2.2.2.9	Further enhanced Type II port selection codebook for CJT

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-PortSelection-r18'

-	the number of CSI-RS ports for each CSI-RS resource, PCSI-RS, is configured as in clause 5.2.2.2.4.

-	A set of NL∈{1,2,4} combinations of values of {α1,…,αNTRP} is configured by the higher layer parameter paramCombination-CJT-PS-alpha-r18, where the value of NL is configured by the higher layer parameter numberOfSDCombinations-PS and the mapping is given in Table 5.2.2.2.9-1, with the value of αn corresponding to CSI-RS resource n, for n=1,…,NTRP. A single value of M and β is configured by the higher layer parameter paramCombination-CJT-PS-r18, where the mapping is given in Table 5.2.2.2.9-2. The configurable combinations of {α1,…,αNTRP} and {M,β} are marked with 'x' in Table 5.2.2.2.9-3.

-	The UE is not expected to be configured with paramCombination-CJT-PS-alpha-r18 equal to

-	2, 7, 10, 11 or 12 when PCSI-RS∈{4,12},

-	3, 8, 16 or 20 when paramCombination-CJT-PS-r18 is configured to 4 or 5 and PCSI-RS=32,

-	1, 4, 5, 6, 9, 10, 11, 12, 13, 14, 15, 17, 18 or 19 when PCSI-RS=4 and higher layer parameter typeII-CJT-PS-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	If NL>1, the UE is expected to select one of the NL configured combinations of {α1,…,αNTRP} and report the index of the selected combination, where the index value 0 corresponds to the first configured combination and the index value NL-1 corresponds to the NL-th configured combination. If NL=1, a single combination of {α1,…,αNTRP} is configured and the selection is not reported.

Table 5.2.2.2.9-1: Codebook parameter configurations for {α1,…, αNTRP}

| NTRP | paramCombination-CJT-PS-alpha-r18 | {α1,…, αNTRP} |
| --- | --- | --- |
| 1 | 1 | {1/2} |
|  | 2 | {3/4} |
|  | 3 | {1} |
| 2 | 4 | {1/2,1/2} |
|  | 5 | {1/2,1} |
|  | 6 | {1,1/2} |
|  | 7 | {3/4,3/4} |
|  | 8 | {1,1} |
| 3 | 9 | {1/2,1/2,1/2} |
|  | 10 | {1/2,1/2,3/4} |
|  | 11 | {1/2,3/4,1/2} |
|  | 12 | {3/4,1/2,1/2} |
|  | 13 | {1/2,1/2,1} |
|  | 14 | {1/2,1,1/2} |
|  | 15 | {1,1/2,1/2} |
|  | 16 | {1,1,1} |
| 4 | 17 | {1/2,1/2,1/2,1/2} |
|  | 18 | {1/2,1/2,1/2,1} |
|  | 19 | {1/2,1/2,1,1} |
|  | 20 | {1,1,1,1} |

Table 5.2.2.2.9-2: Codebook parameter configurations for {M,β}

| paramCombination-CJT-PS-r18 | M | β |
| --- | --- | --- |
| 1 | 1 | ½ |
| 2 | 1 | ¾ |
| 3 | 1 | 1 |
| 4 | 2 | ½ |
| 5 | 2 | ¾ |

Table 5.2.2.2.9-3: Configurable combinations of {α1,…, αNTRP} and {M,β}

| paramCombination-CJT-PS-alpha-r18 | paramCombination-CJT-PS-r18 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| 1 |  |  |  | x |  |
| 2 | x |  |  | x |  |
| 3 | x | x | x | x | x |
| 4 | x |  |  | x |  |
| 5 | x |  |  |  |  |
| 6 | x |  |  |  |  |
| 7 |  | x |  |  |  |
| 8 |  | x |  | x |  |
| 9 | x |  |  | x |  |
| 10 | x |  |  |  |  |
| 11 | x |  |  |  |  |
| 12 | x |  |  |  |  |
| 13 |  | x |  | x |  |
| 14 |  | x |  | x |  |
| 15 |  | x |  | x |  |
| 16 |  | x |  |  | x |
| 17 | x |  |  |  |  |
| 18 | x |  |  |  |  |
| 19 |  | x | x | x |  |
| 20 |  |  | x |  |  |

-	The value of NM∈{2,4} is configured with the higher-layer parameter valueOfN-CJT-r18, when M=2.

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-CJT-PS-r18, when M=2, and R=1 when M=1, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-CJT-PS-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-CJT-PS-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0, where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…, 3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N=NTRP. Otherwise, the UE is expected to select N CSI-RS resources, with 1≤N≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N selected CSI-RS resources corresponds to the nonzero bit with lowest index.

The PMI value for the N selected CSI-RS resources corresponds to the codebook indices of i1 and i2 where

i1=i1,2i1,6i1,7,1i1,8,1i1,9                                                                                                    υ=1i1,2i1,6i1,7,1i1,8,1i1,7,2i1,8,2i1,9υ=2i1,2i1,6i1,7,1i1,8,1i1,7,2i1,8,2i1,7,3i1,8,3i1,9υ=3i1,2i1,6i1,7,1i1,8,1i1,7,2i1,8,2i1,7,3i1,8,3i1,7,4i1,8,4i1,9υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                    υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                               υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The precoding matrices indicated by the PMI are determined from j=1NLσj+M vectors, where {σ1,…,σN} are the indices of the N selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN≤NTRP, and Lσj=K1,σj/2, K1,σj=ασjPCSI-RS, where {ασ1, …, ασN} are the corresponding values from the selected combination of {α1, …, αNTRP}.

K1,σj ports are selected from the PCSI-RS ports of the j-th selected CSI-RS resource, for j=1,…,N, based on Lσj vectors, vmji, i=0,1,…,Lσj-1, which are indicated by i1,2, where

i1,2=[i1,2,1 …i1,2,N]

i1,2,j∈0,1,…, PCSI-RS/2Lσj-1

Let

m=[m1… mN]

mj=[mj0…mj(Lσj-1)]

mj(i)∈0,1,…,PCSI-RS2-1

the index i1,2,j, for the j-th selected CSI-RS resource, j=1,…,N, is obtained from the Lσj elements of mj, as described in Clause 5.2.2.2.7 for the indicator i1,2, obtained from the L elements of m. Vector vmj(i) is a PCSI-RS/2 -element column vector containing a value of 1 in the element of index mj(i) and zeros elsewhere, and where the first element is the element of index 0.

-	If ασj=1 for the j-th selected CSI-RS resource, mj(i)=i, for i=0,1,…,PCSI-RS2-1, and i1,2,j is not reported.

The M vectors, y0f,y1f,…,yN3-1fT, f=0,1,…,M-1, are common for all the N selected CSI-RS resources and are identified by n3, where

n3=n30… n3M-1

n3(f)∈{0}M=10,1,…,min⁡(NM,N3)-1M=2.

with the indices f∈{0,…,M-1} assigned such that n3(f) increases with f. n3 is indicated by the index i1,6, when M=2 and NM=4, where

i1,6∈0,1,2.

-	If M=1, or M=2 and NM=2, i1,6 is not reported.

-	If M=2 and NM=4, the nonzero offset between n3(0) and n3(1) is reported with i1,6 assuming that n3(0) (reference for the offset) is 0. The nonzero offset values are mapped to the index values of i1,6 in increasing order with offset value 1 mapped to index value '0'.

The M vectors’ elements are given by

yt(f)=ej2πtn3(f)N3

for t=0,1,…N3-1, and f=0,…,M-1.

If the higher layer parameter codebookMode is set to 'mode1', an offset dj is reported for the j-th selected CSI-RS resource, with j=2,…,N, relative to the first of the N selected CSI-RS resources. The N-1 reported offsets are common for all ν layers and are indicated by i1,9, given by

i1,9=[d2… dN]

dj∈{0, 1,…,N3O3-1}

where the value of O3∈{1,4} is configured by higher layer parameter numberOfO3. The offsets are represented by

ψ=[ψ2… ψN]

ψj=djO3

If codebookMode is set to 'mode2’, the offset indicator, i1,9, is not reported and ψj=0 for j=1,…,N.

The reference amplitude coefficient indicator i2,3,l, for layer l=1,…υ, is given by

i2,3,l=kl,01kl,11

kl,p(1)∈1,…,15

The reference amplitude coefficients for layer l=1,…,υ are represented by

pl(1)=[pl,01 pl,1(1)]

and the mapping from kl,p(1) to pl,p(1) is given in Table 5.2.2.2.5-2.

The amplitude coefficient indicator i2,4,l, for layer l=1,…,υ, is given by

i2,4,l=[i2,4,l,1…i2,4,l,N]

i2,4,l,j=kl,0,j(2)…kl,M-1,j(2)

kl,f,j(2)=kl,0,f,j(2)…kl,K1,σj-1,f,j(2)

kl,i,f,j(2)∈0,…,7

The amplitude coefficients for layer l=1,…,υ are represented by

pl(2)=[pl,12 … pl,N(2)]

pl,j(2)=[pl,0,j2…pl,M-1,j(2)]

pl,f,j(2)=[pl,0,f,j2…pl,K1,σj-1,f,j(2)]

and the mapping from kl,i,f,j(2) to pl,i,f,j(2) is given in Table 5.2.2.2.5-3.

The phase coefficient indicator i2,5,l, for l=1,…,υ, is given by

i2,5,l=[i2,5,l,1…i2,5,l,N]

i2,5,l,j=cl,0,j…cl,M-1,j

cl,f,j=cl,0,f,j…cl,K1,σj-1,f,j

cl,i,f,j∈0,…,15

The phase coefficients for layer l=1,…,υ are represented by

φl=[φl,1…φl,N]

φl,j=φl,0,j…φl,M-1,j

φl,f,j=φl,0,f,j…φl,K1,σj-1,f,j

and the mapping from cl,i,f,j to φl,i,f,j is given by

φl,i,f,j=ej2πcl,i,f,j16

Let K0=βMj=1NK1,σj. The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported for layer l=1,…,υ, is indicated by i1,7,l

i1,7,l=[i1,7,l,1…i1,7,l,N]

i1,7,l,j=kl,0,j3…kl,M-1,j3

kl,f,j3=kl,0,f,j3…kl,K1,σj-1,f,j3

kl,i,f,j(3)∈0,1

Let KlNZ=j=1Ni=0K1,σj-1f=0M-1kl,i,f,j(3)≤K0 be the number of nonzero coefficients for layer l=1,…,υ, the total number of nonzero coefficients is reported and given by KNZ=l=1υKlNZ≤2K0.

-	If υ≤2 and KNZ=Mυj=1NK1,σj, i1,7,l is not reported, for l=1,…,υ.

The indices of i2,4,l, i2,5,l and i1,7,l are associated to the N selected CSI-RS resources.

Let fl*∈0,…,M-1 and il*∈0,1,…,j=1NK1,σj-1 be the indices which identify the strongest coefficient of layer l, for  l=1,…,υ, i.e., the element kl,yl*,fl*,jl*(2) of i2,4,l, where the indices jl*∈{1,…,N}, yl*∈0,…,K1,σjl*-1 are such that il*=k=1jl*-1K1,σk+yl*. The strongest coefficient of layer l=1,…,υ is identified by the index

i1,8,l∈0,1,…,Mj=1NK1,σj-1

which is found from

i1,8,l=fl*j=1NK1,σj+il*.

The amplitude and phase coefficient indicators are reported as follows:

-	kl,yl*Lσjl*(1)=15, kl,yl*,fl*,jl*(2)=7, kl,yl*,fl*,jl*(3)=1 and cl,yl*,fl*,jl*=0, for l=1,…,υ. The reference amplitude, kl,yl*Lσjl*(1), amplitude coefficient, kl,yl*,fl*,jl*(2) and phase coefficient, cl,yl*,fl*,jl*, are not reported for l=1,…,υ.

-	The reference amplitude, kl,yl*Lσjl*+1 mod 2(1), is reported for l=1,…,υ.

-	The KNZ-υ amplitude coefficients, kl,i,f,j(2), for which kl,i,f,j(3)=1, i≠yl*, f≠fl*, j≠jl* are reported.

-	The KNZ-υ phase coefficients, cl,i,f,j, for which kl,i,f,j(3)=1, i≠yl*, f≠fl*, j≠jl* are reported.

-	The remaining j=1NK1,σj⋅M⋅υ-KNZ amplitude coefficients, kl,i,f,j(2), are not reported.

-	The remaining j=1NK1,σj⋅M⋅υ-KNZ phase coefficients, cl,i,f,j, are not reported.

The codebooks for 1-4 layers are given in Table 5.2.2.2.9-4, where t=0,1,…,N3-1, is the index associated with the precoding matrix, l=1,…, υ is the layer index, and where, for coefficients with kl,i,f,j(3)=0, amplitude and phase are set to zero, i.e., pl,i,f,j(2)=0 and φl,i,f,j=0.

Table 5.2.2.2.9-4: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS of N selected CSI-RS resources

| Layers |  |
| --- | --- |
| υ=1 | Wm,n3,p1(1),p12,φ1,ψ,t(1)=Wm,n3,p1(1),p12,φ1,ψ,t1 |
| υ=2 | Wm,n3,p1(1),p12,φ1,p2(1),p22,φ2,ψ,t(2)=12Wm,n3,p1(1),p12,φ1,ψ,t1 Wm,n3,p2(1),p22,φ2,ψ,t2 |
| υ=3 | Wm,n3,p1(1),p12,φ1,p2(1),p22,φ2,p3(1),p32,φ3,ψ,t(3)=13Wm,n3,p1(1),p12,φ1,ψ,t1 Wm,n3,p2(1),p22,φ2,ψ,t2 Wm,n3,p3(1),p32,φ3,ψ,t3 |
| υ=4 | Wm,n3,p1(1),p12,φ1,p2(1),p22,φ2,p3(1),p32,φ3,p4(1),p42,φ4,ψ,t(4)=12Wm,n3,p1(1),p12,φ1,ψ,t1 Wm,n3,p2(1),p22,φ2,ψ,t2 Wm,n3,p3(1),p32,φ3,ψ,t3 Wm,n3,p4(1),p42,φ4,ψ,t4 |
| Where Wm,n3,pl(1),pl2,φl,ψ,tl=1γt,li=0Lσ1-1vm1(i)pl,0(1)f=0M-1yt(f)pl,i,f,1(2)φl,i,f,1i=0Lσ1-1vm1(i)pl,1(1)f=0M-1yt,l(f)pl,i+Lσ1,f,1(2)φl,i+Lσ1,f,1ej2πtψ2N3i=0Lσ2-1vm2(i)pl,0(1)f=0M-1yt(f)pl,i,f,2(2)φl,i,f,2ej2πtψ2N3i=0Lσ2-1vm2(i)pl,1(1)f=0M-1yt(f)pl,i+Lσ2,f,2(2)φl,i+Lσ2,f,2⋮ej2πtαψNN3i=0LσN-1vmN(i)pl,0(1)f=0M-1yt(f)pl,i,f,N(2)φl,i,f,Nej2πtψNN3i=0LσN-1vmN(i)pl,1(1)f=0M-1yt(f)pl,i+LσN,f,N(2)φl,i+LσN,f,N, γt,l=j=1Ni=02Lσj-1pl,iLσj12f=0Mυ-1yt,lfpl,i,f,j2φl,i,f,j2, l=1,2,3,4 and the mappings from i1 to m, n3, ψ and from i2 to p1(1), p2(1), p3(1), p4(1), p1(2), p2(2), p3(2), p4(2), φ1, φ2, φ3, φ4 are as described above, including the ranges of the constituent indices of i1 and i2. |  |

##### 5.2.2.2.10	Enhanced Type II codebook for predicted PMI

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement and with codebookType set to 'typeII-Doppler-r18'

-	The values of N1 and N2 are configured with the higher layer parameter n1-n2‑codebookSubsetRestriction-Doppler-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2.

-	The values of L, β and pυ are determined by the higher layer parameter paramCombination-Doppler-r18, where the mapping is given in Table 5.2.2.2.10-1.

-	The UE is not expected to be configured with paramCombination-Doppler-r18 equal to

-	4, 5, 6, 7, 8, or 9 when PCSI-RS=4,

-	8 or 9 when PCSI-RS<32

-	8 or 9 when higher layer parameter typeII-Doppler-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	8 or 9 when R=2.

Table 5.2.2.2.10-1: Codebook parameter configurations for L,  β and pυ

| paramCombination-Doppler-r18 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | 1/8 | 1/16 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | ¼ | ¼ |
| 5 | 4 | ¼ | ¼ | ½ |
| 6 | 4 | ¼ | ¼ | ¾ |
| 7 | 4 | ½ | ¼ | ½ |
| 8 | 6 | ¼ | - | ½ |
| 9 | 6 | ¼ | - | ¾ |

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-Doppler-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-Doppler-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The value of N4∈{1,2,4,8} is configured by the higher layer parameter N4, such that the PMI indicates N3 precoder matrices for each of the N4 consecutive slot intervals of duration d slots, as defined in Clause 5.2.1.4.2.

If N4=1, the PMI value corresponds to the codebook indices of i1 and i2 as described in Clause 5.2.2.2.5 and the precoder matrices for 1-4 layers are obtained from the PMI codebook as in Table 5.2.2.2.5-5.

If N4∈{2,4,8}, the PMI value corresponds to the codebook indices of i1 and i2, where

i1=i1,1i1,2i1,5i1,6,1i1,7,1i1,8,1 i1,10,1                                                                  υ=1i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,10,1i1,6,2     i1,7,2     i1,8,2     i1,10,2υ=2i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,10,1i1,6,2     i1,7,2     i1,8,2     i1,10,2i1,6,3     i1,7,3     i1,8,3     i1,10,3           υ=3i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,10,1i1,6,2i1,7,2     i1,8,2      i1,10,2    i1,6,3     i1,7,3     i1,8,3     i1,10,3                                                                                                                                           i1,6,4     i1,7,4     i1,8,4     i1,10,4υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                                                υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                                  υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The precoding matrices indicated by the PMI are determined from L+Mυ+Q vectors, where Q=2.

The L vectors, vm1i,m2(i),i=0,1,…,L-1, indicated by i1,1, i1,2 and the Mυ vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, for layer l=1,…,ν, indicated by i1,5 (for N3>19) and i1,6,l for Mυ>1 and l=1,…, υ, are obtained as in Clause 5.2.2.2.5.

The Q=2 vectors, z0,lτ,z1,lτ,…,zN4-1,lτT, τ=0,1, for layer l=1,…,ν  are identified by n4,l, where

n4,l=n4,l0 n4,l1

n4,l(τ)∈{0,1,…,N4-1}

with the indices τ∈{0,1} assigned such that n4,l(τ) increases with τ. n4,l is indicated by the index i1,10,l, for layer l=1,…,υ, where

i1,10,l∈0,1,…,N4-2

-	If N4=2, i1,10,l, for layer l=1,…,υ are not reported.

-	If N4∈{4,8}, the nonzero offset between n4,l(0) and n4,l(1) is reported with i1,10,l assuming that n4,l(0) (reference for the offset) is 0. The nonzero offset values are mapped to the index values of i1,10,l in increasing order with offset value 1 mapped to index value '0'.

The Q=2 vectors’ elements are given by

zι,l(τ)=ej2π⋅ι⋅n4,l(τ)N4

for ι=0,1,…N4-1, and τ=0,1.

The reference amplitude coefficient indicator i2,3,l, for layer l=1,…υ, is given by

i2,3,l=kl,01kl,11

kl,p(1)∈1,…,15

The reference amplitude coefficients for layer l=1,…,υ are represented by

pl(1)=[pl,01 pl,1(1)]

and the mapping from kl,p(1) to pl,p(1) is given in Table 5.2.2.2.5-2.

The amplitude coefficient indicator i2,4,l, for layer l=1,…,υ, is given by

i2,4,l=[i2,4,l,0 i2,4,l,1]

i2,4,l,τ=kl,0,τ(2)…kl,Mυ-1,τ(2)

kl,f,τ(2)=kl,0,f,τ(2)…kl,2L-1,f,τ(2)

kl,i,f,τ(2)∈0,…,7

The amplitude coefficients for layer l=1,…,υ are represented by

pl(2)=[pl,02  pl,1(2)]

pl,τ(2)=[pl,0,τ2…pl,Mν-1,τ(2)]

pl,f,τ(2)=[pl,0,f,j2…pl,2L-1,f,τ(2)]

and the mapping from kl,i,f,τ(2) to pl,i,f,τ(2) is given in Table 5.2.2.2.5-3.

The phase coefficient indicator i2,5,l, for l=1,…,υ, is given by

i2,5,l=[i2,5,l,0 i2,5,l,1]

i2,5,l,τ=cl,0,τ…cl,Mυ-1,τ

cl,f,τ=cl,0,f,τ…cl,2L-1,f,τ

cl,i,f,τ∈0,…,15

The phase coefficients for layer l=1,…,υ are represented by

φl=[φl,0 φl,1]

φl,τ=φl,0,τ…φl,Mυ-1,τ

φl,f,τ=φl,0,f,τ…φl,2L-1,f,τ

and the mapping from cl,i,f,τ to φl,i,f,τ is given by

φl,i,f,τ=ej2πcl,i,f,τ16

Let K0=2βLM1Q. The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported for layer l=1,…,υ, is indicated by i1,7,l

i1,7,l=[i1,7,l,0 i1,7,l,1]

i1,7,l,τ=kl,0,τ3…kl,Mυ-1,τ3

kl,f,τ3=kl,0,f,τ3…kl,2L-1,f,τ3

kl,i,f,τ(3)∈0,1

Let KlNZ=τ=0Q-1i=02L-1f=0Mυ-1kl,i,f,τ(3)≤K0 be the number of nonzero coefficients for layer l=1,…,υ, the total number of nonzero coefficients is reported and given by KNZ=l=1υKlNZ≤2K0.

The indices of i2,4,l, i2,5,l and i1,7,l are associated to the Q codebook indices in n4,l.

Let τl*∈{0,1}, fl*∈0,1,…,Mυ-1 and il*∈0,1,…,2L-1 be the indices which identify the strongest coefficient of layer l, for  l=1,…,υ, i.e., the element kl,il*,fl*,τl*(2) of i2,4,l. The codebook indices of n3,lkmi are remapped with respect to n3,l(fl*) as n3,l(f)=n3,l(f)-n3,l(fl*)mod N3kmi*=0, such that n3,l(fl*)=0, after remapping. The index f is remapped with respect to fl* as f=f-fl*mod Mυ, such that the index of the strongest coefficient is fl*=0 (l=1,…,υ), after remapping. The indices of i2,4,l, i2,5,l and i1,7,l indicate amplitude coefficients, phase coefficients and bitmap after remapping.

The strongest coefficient of layer l=1,…,υ is identified by i1,8,l∈0,1,…,2LQ-1, which is obtained as follows, where, for υ=1, κ1,I,0(3)=k1,i,0,τ(3) and index I is such that I=2Lτ+i

i1,8,l=I=02Lτ1*+i1*κ1,I,0(3)-1υ=12Lτl*+il*1<υ≤4

The amplitude and phase coefficient indicators are reported as follows:

-	kl,il*L(1)=15, kl,il*,0,τl*(2)=7, kl,il*,0,τl*(3)=1 and cl,il*,0,τl*=0, for l=1,…,υ. The reference amplitude, kl,il*L(1), amplitude coefficient, kl,il*,0,τl*(2) and phase coefficient, cl,il*,0,τl*, are not reported for l=1,…,υ.

-	The reference amplitude, kl,il*L+1 mod 2(1), is reported for l=1,…,υ.

-	The KNZ-υ amplitude coefficients, kl,i,f,τ(2), for which kl,i,f,τ(3)=1, τ≠τl*, i≠il*, f≠0 are reported.

-	The KNZ-υ phase coefficients, cl,i,f,τ, for which kl,i,f,τ(3)=1, τ≠τl* i≠il*, f≠0 are reported.

-	The remaining 2L⋅Mυ⋅Q⋅υ-KNZ amplitude coefficients, kl,i,f,τ(2), are not reported.

-	The remaining 2L⋅Mυ⋅Q⋅υ-KNZ phase coefficients, cl,i,f,τ, are not reported.

The codebooks for 1-4 layers are given in Table 5.2.2.2.10-2, where t=0,1,…,N3-1, ι=0,1,…,N4-1 are the indices associated with the precoding matrix, l=1,…, υ is the layer index, and where, for coefficients with kl,i,f,τ(3)=0, amplitude and phase are set to zero, i.e., pl,i,f,τ(2)=0 and φl,i,f,τ=0.

Table 5.2.2.2.10-2: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| Layers |  |
| --- | --- |
| υ=1 | Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1,t,ι(1)=Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1,t,ι1 |
| υ=2 | Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1, n3,2,n4,2,p2(1),p22,φ2,t,ι(2)=12Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1,t,ι1 Wq1,q2, n1, n2,n3,2,n4,2,p2(1),p22,φ2,t,ι2 |
| υ=3 | Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1,n3,2,n4,2,p2(1),p22,φ2,n3,3,n4,3,p3(1),p32,φ3,t,ι(3)=13Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1,t,ι1 Wq1,q2, n1, n2,n3,2,n4,2,p2(1),p22,φ2,t,ι2 Wq1,q2, n1, n2,n3,3,n4,3,p3(1),p32,φ3,t,ι3 |
| υ=4 | Wq1,q2, n1, n2,n3,1,n4,1,p1(1),p12,φ1,n3,2,n4,2,p2(1),p22,φ2,n3,3,n4,3,p3(1),p32,φ3,n3,4,n4,4,p4(1),p42,φ4,t,ι(4)=12Wq1,q2, n1, n2,n3,1,n4,1,p11,p12,φ1,t,ι1 Wq1,q2, n1, n2,n3,2,n4,2,p21,p22,φ2,t,ι2 Wq1,q2, n1, n2,n3,3,n4,3,p31,p32,φ3,t,ι3Wq1,q2, n1, n2,n3,4,n4,4,p4(1),p42,φ4,t,ι4 |
| Where Wq1,q2, n1, n2,n3,l,n4,l,pl(1),pl2,φl,t,ιl=1N1N2γt,ι,li=0L-1vm1i,m2(i)pl,0(1)f=0Mυ-1yt,l(f)τ=0Q-1zι,l(τ)pl,i,f,τ(2)φl,i,f,τi=0L-1vm1i,m2(i)pl,1(1)f=0Mυ-1yt,l(f)τ=0 Q-1zι,l(τ)pl,i+L,f,τ(2)φl,i+L,f,τ, γt,ι,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)τ=0Q-1zι,l(τ)pl,i,f,τ(2)φl,i,f,τ2, l=1,2,3,4 and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, n4,1, n4,2, n4,3, n4,4 and from i2 to p1(1), p2(1), p3(1), p4(1), p1(2), p2(2), p3(2), p4(2), φ1, φ2, φ3, φ4 are as described above, including the ranges of the constituent indices of i1 and i2. |  |

The bitmap parameter n1-n2-codebookSubsetRestriction-Doppler-r18 is configured as described in Clause 5.2.2.2.5, where only the bit values ‘00’ or ‘11’ of Table 5.2.2.2.5-6 are configurable.

##### 5.2.2.2.11	Further enhanced Type II port selection codebook for predicted PMI

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement and with codebookType set to 'typeII-Doppler-PortSelection-r18'

-	the number of CSI-RS ports, PCSI-RS, is configured as in clause 5.2.2.2.4.

-	The values α, M and β are determined by the higher layer parameter paramCombination-Doppler-PS-r18, where the mapping is given in Table 5.2.2.2.7-1 and the applicable configuration restrictions are described in Clause 5.2.2.2.7.

-	The parameter N∈{2,4} is configured with the higher-layer parameter valueOfN-Doppler-r18, when M=2.

-	The parameter R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-Doppler-PS-r18, when M=2, and R=1 when M=1, where R and the corresponding value of parameter N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-Doppler-PS-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-Doppler-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The value of N4=1 is configured by the higher layer parameter N4, such that the PMI indicates N3 precoder matrices for one slot interval of duration d slots, as defined in Clause 5.2.1.4.2.

The PMI value corresponds to the codebook indices of i1 and i2 as described in Clause 5.2.2.2.7 and the precoder matrices for 1-4 layers are obtained from the PMI codebook as in Table 5.2.2.2.7-3.

#### 5.2.2.3	Reference signal (CSI-RS)

##### 5.2.2.3.1	NZP CSI-RS

The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s).

The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration:

-	nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity.

-	periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources.

-	resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	nrofPorts in resourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	density in resourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid.

-	cdm-Type in resourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	powerControlOffset: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size. For CQI calculation based on a pair of NZP CSI-RS resources, powerControlOffset of each NZP CSI-RS resource in the pair of NZP CSI-RS resources for channel measurement is the assumed ratio of EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size.

-	powerControlOffsetSS: which is the assumed ratio of NZP CSI-RS EPRE to SS/PBCH block EPRE.

-	scramblingID defines scrambling ID of CSI-RS with length of 10 bits.

-	BWP-Id in CSI-ResourceConfig defines which bandwidth part the configured CSI-RS is located in.

-	repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether UE can assume the CSI-RS resources within the NZP CSI-RS Resource Set are transmitted with the same downlink spatial domain transmission filter or not as described in Clause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP', 'cri-SINR', 'cri-RSRP- Index', 'cri-SINR- Index' or 'none'.

-	qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS configured with qcl-Type set to 'typeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP. The reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell.

-	trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Clause 5.1.6.1.1 and can be configured when reporting setting is not configured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'tdcp' or 'none'.

All CSI-RS resources within one set are configured with same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement.

The UE expects that all the CSI-RS resources of a resource set are configured with the same starting RB and number of RBs and the same cdm-type.

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within  X∈{1,2}  slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot, and X=2 implies that the two resources are configured within two adjacent slots.

For a NZP-CSI-RS-ResourceSet for channel measurement with 1<K≤4 resources and linked to a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', the slot offsets of the K CSI-RS resources are configured within  X∈{1,2}  slots, without DL/UL switching in between the two resources, where X=1 implies that the K resources are configured in the same slot, and X=2 implies that the K resources are configured within two adjacent slots.

The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in Clause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).

<omitted text>

#### 5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1,

-	where n=n'⋅2μDL2μUL +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL and μDLand μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and  are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211]

-	where for periodic and semi-persistent CSI reporting

-	if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or

-	if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to 5⋅2μDL, such that it corresponds to a valid downlink slot.

-	where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to Z'/Nsymbslot, such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4.

-	when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

A slot in a serving cell shall be considered to be a valid downlink slot if:

-	it comprises at least one higher layer configured downlink or flexible symbol, and

-	it does not fall within a configured measurement gap for that UE

If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.

For a CSI-ReportConfig configured with codebookType set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18', the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for each of the CSI-RS resources in the corresponding CSI-RS Resource Set for channel measurement and/or one CSI-IM occasion for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise.

For a CSI-ReportConfig configured with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18', the UE reports a CSI report only if receiving at least one aperiodic or Kp periodic or semipersistent consecutive CSI-RS transmission occasions for each CSI-RS resource in the corresponding CSI-RS Resource Set for channel measurement and/or one CSI-IM occasion for interference measurement no later than the CSI reference resource and within the same DRX Active Time, when DRX is configured, and drops the report otherwise. The value of Kp∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6.

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When DRX is configured and the CSI-RS Resource Set for channel measurement corresponding to a CSI report is configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for each CSI-RS resource in a Resource Pair within the same DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', and 'ssb-Index-RSRP- Index ' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', or 'ssb-Index-RSRP- Index' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer in DRX-Config also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' or 'cri-RSRP- Index' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer in DRX-Config outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

When deriving CSI feedback, the UE is not expected that a NZP CSI -RS resource for channel measurement overlaps with CSI-IM resource for interference measurement or NZP CSI -RS resource for interference measurement.

#### 5.2.2.5.1	UE assumptions for CQI/PMI/RI calculation

If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI:

-	The first 2 OFDM symbols are occupied by control signaling.

-	The number of PDSCH and DM-RS symbols is equal to 12.

-	The same bandwidth part subcarrier spacing configured as for the PDSCH reception

-	The bandwidth as configured for the corresponding CQI report.

-	The IAB-MT shall only assume the frequency resources as indicated by the DL TX power adjustment MAC CE, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321].

-	The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception

-	No resource elements used by primary or secondary synchronization signals or PBCH.

-	Redundancy Version 0.

-	The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1.

-	In addition, the IAB-MT shall apply the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321].

-	Assume no REs allocated for NZP CSI-RS and ZP CSI-RS.

-	Assume the same number of front-loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig.

-	Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition.

-	Assume the PDSCH symbols are not containing DM-RS.

-	Assume PRB bundling size of 2 PRBs.

-	The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211],  is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2. The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1.

#### 5.2.2.5.1a	UE assumptions for CQI/PMI/RI calculation for NCJT

If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, the reported CRI corresponds to an entry of the N Resource Pairs, and the reported rank combination is {ν1,ν2}, as described in clause 5.2.1.4.2, for CQI calculation, the UE should assume that

-	PDSCH signals on antenna ports in the set [1000,…,1000+ν1-1] for ν1 layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of the Group 1 CSI-RS resource in the Resource Pair indicated by the CRI, and PDSCH signals on antenna ports in the set [1000+ν1,…,1000+ν1+ν2-1] for ν2 layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of the Group 2 CSI-RS resource in the Resource Pair indicated by the CRI, as given by

yj3000(i)⋯⋮yj3000+P-1(i)=Wjix(j-1)⋅ν1i⋯⋮xν1+(j-1)⋅ν2-1i

where Wj(i), j=1,2 are the two precoding matrices corresponding to the two reported PMIs applicable to x(i), as described in clause 5.2.1.4.2; and the indices j=1,2 are associated to the two Resource Groups configured in the corresponding CSI-RS Resource Set for channel measurement; that the signals yj, j=1,2, fully overlap in time and frequency, and that, for the calculation of RI, PMI and LI (if configured) of νj layers, j=1,2, the interference from the other νj mod 2+1 layers is derived from channel measurement and precoding matrix corresponding to the other νj mod 2+1 layers.

-	The UE shall assume that the corresponding PDSCH signals for νj layers transmitted on the P antenna ports of the CSI-RS resource in Group j would have a ratio of EPRE to CSI-RS EPRE equal to the powerControlOffset of the respective CSI-RS resource, for j=1,2.

#### 5.2.2.5.1b	UE assumptions for CQI/PMI/RI calculation for CJT

If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-PMI-CQI', the higher layer parameter codebookType is set to 'typeII-CJT-r18' or ' typeII-CJT-PortSelection-r18', and the corresponding CSI-RS Resource Set for channel measurement is configured with 1<≤NTRP≤4 CSI-RS resources, for CQI calculation

-	a UE should assume PDSCH signals on antenna ports in the set [1000,…,1000+υ-1] for υ layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1] of each of the N selected CSI-RS resources, as given by

yσ13000(i)⋮yσ13000+P-1(i)yσ23000(i)⋮yσ23000+P-1(i)⋮yσN3000(i)⋮yσN3000+P-1(i)=Wix0i⋮xυ-1i

where W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.2.2.8 and 5.2.2.2.9 for codebookType set to 'typeII-CJT-r18' and ' typeII-CJT-PortSelection-r18', respectively, and {σ1,…,σN} are the indices of the N selected CSI-RS resources in increasing order, such that 1≤σ1<…<σN≤NTRP. A UE should assume that the signals yσj, j=1,…,N, fully overlap in time and frequency.

-	a UE can assume that the PDSCH signals for υ layers transmitted on the P antenna ports of CSI-RS resource σj would have the same ratio of EPRE to CSI-RS EPRE for all j=1,…,N, equal to the powerControlOffset of the respective CSI-RS resource.

#### 5.2.2.5.1c	UE assumptions for CQI/PMI/RI calculation for predicted CSI

If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-PMI-CQI', the higher layer parameter codebookType is set to 'typeII-Doppler-r18' or ' typeII-Doppler-PortSelection-r18', and the corresponding CSI-RS Resource Set for channel measurement is aperiodic with K CSI-RS resources, for CQI calculation, a UE can assume the same ratio of EPRE to CSI-RS EPRE for all K configured CSI-RS resources.

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II, Enhanced Type II, Further Enhanced Type II Port Selection, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI, Further Enhanced Type II Port Selection for predicted PMI and TDCP reporting.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 or DCI format 0_2 which activates a semi-persistent CSI trigger state. DCI format 0_1 and DCI format 0_2 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I, Type II with wideband, and sub-band frequency granularities, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI and Further Enhanced Type II Port Selection for predicted PMI. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

Type I CSI feedback is supported for CSI Reporting on PUSCH. Type I wideband and sub-band CSI is supported for CSI Reporting on the PUSCH. Type II CSI is supported for CSI Reporting on the PUSCH.

For Type I, Type II, Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for CJT, Further Enhanced Type II Port Selection for CJT, Enhanced Type II for predicted PMI and Further Enhanced Type II Port Selection for predicted PMI feedback on PUSCH, a CSI report comprises of two parts. Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Part 1 shall be transmitted in its entirety before Part 2.

-	For Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4. For a CSI-ReportConfig configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, Part 1 contains RI(s), CRI(s), CQI(s) for the first codeword and is zero padded to a fixed payload size (if needed). Part 2 contains the CQI(s) for the second codeword (if reported) when RI is larger than 4, LIs (if reported) and PMI(s).

-	For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded.

-	For Enhanced Type II CSI feedback (see Clause 5.2.2.2.5), Further Enhanced Type II Port Selection CSI feedback (see Clause 5.2.2.2.7), Enhanced Type II for predicted PMI with N4=1 (see Clause 5.2.2.2.10) and Further Enhanced Type II Port Selection for predicted PMI (see Clause 5.2.2.2.11), Part 1 contains RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II, Further Enhanced Type II Port Selection CSI, Enhanced Type II for predicted PMI with N4=1 or Further Enhanced Type II Port Selection for predicted PMI. Part 1 and 2 are separately encoded.

-	For Enhanced Type II for CJT (see Clause 5.2.2.2.8) and Further Enhanced Type II Port Selection for CJT (see Clause 5.2.2.2.9), Part 1 contains RI (if reported), CQI, the total number of reported non-zero amplitude coefficients across layers, the bitmap selecting N CSI-RS resources (if reported) and the selected combination of {L1,…,LNTRP} (if reported). The fields of Part 1 – RI (if reported), CQI, the total number of reported non-zero amplitude coefficients across layers, the bitmap selecting N CSI-RS resources (if reported) and the selected combination of {L1,…,LNTRP} (if reported) – are separately encoded. Part 2 contains the PMI of the Enhanced Type II for CJT or Further Enhanced Type II Port Selection for CJT. Part 1 and 2 are separately encoded.

-	For Enhanced Type II for predicted PMI with N4>1 (see Clause 5.2.2.2.10), Part 1 contains RI (if reported), the CQI (if the higher layer parameter TDCQI is set to '1-1' or '1-2') or the first CQI (if the higher layer parameter TDCQI is set to '2') and the total number of reported non-zero amplitude coefficients across layers. The fields of Part 1 – RI (if reported), CQI, and the total number of reported non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the second CQI (if the higher layer parameter TDCQI is set to '2') and the PMI of the Enhanced Type II for predicted PMI. Part 1 and 2 are separately encoded.

A Type II CSI report that is carried on the PUSCH shall be computed independently from any Type II CSI report that is carried on the PUCCH formats 3 or 4 (see Clause 5.2.4 and 5.2.2).

When the higher layer parameter reportQuantity is configured with one of the values 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', or 'cri-RSRP-Index', 'ssb-Index-RSRP-Index ', 'cri-SINR-Index ', 'ssb-Index-SINR-Index', 'tdcp', the CSI feedback consists of a single part.

For both Type I and Type II reports configured for PUCCH but transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH as described in Clause 5.2.4.

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand with value '1' are numbered continuously in increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

-	For Enhanced Type II and Enhanced Type II for predicted PMI configured with higher layer parameter N4=1, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l and the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υν,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υν,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Further Enhanced Type II Port Selection and Further Enhanced Type II Port Selection for predicted PMI, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, i and f, is associated with a priority value Pril,i,f=K1⋅υ⋅f+υ⋅i+l, with l=1,2,…,υ, i=0,1,…,K1-1 and f=0,…,M-1. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

-	Group 0 includes i1,2 (if reported), i1,8,l (l=1,…,υ) and i1,6 (if reported).

-	Group 1 includes the υK1M-KNZ/2 highest priority elements of i1,7,l (if reported), i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l and the  max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l (if reported), the minKNZ-υν,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υν,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Enhanced Type II for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,ij, f and j, is associated with a priority value Pril,ij,f,j=2⋅k=1NLσk⋅υ⋅πf+υ⋅2k=1j-1Lσk+ij+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1), for l=1,…,υ, ij=0,1,…,2Lσj-1, f=0,1,…,Mυ-1 and j=1,…,N,  and where n3,l(f) and Lσj are defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυk=1NLσk-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ) and i1,9 (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Further Enhanced Type II Port Selection for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, ij, f and j, is associated with a priority value Pril,ij,f,j=k=1NK1,k⋅υ⋅f+υ⋅k=1j-1K1,k+ij+l, for l=1,…,υ, ij=0,1,…,K1,j-1, f=0,…,M-1 and j=1,…,N,  and where K1,j is defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

-	Group 0 includes i1,2 (if reported), i1,8,l (l=1,…,υ) and i1,6 (if reported).

-	Group 1 includes the υMk=1NK1,k-KNZ/2 highest priority elements of i1,7,l (if reported), i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the  max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ) and i1,9 (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l (if reported), the minKNZ-ν,KNZ2 lowest priority elements of i2,4,l and the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ).

-	For Enhanced Type II for predicted PMI configured with N4>1, for a given CSI report n, each reported element of  i2,4,l i2,5,l and i1,7,l, indexed by l,i, f and j, is associated with a priority value Pril,i,f,j=2L⋅υ⋅Mν⋅j+2L⋅υ⋅f+υ⋅i+l, for l=1,…,υ, i=0,1,…,2L-1, f=0,1,…,Mυ-1 and j=0,1. The element with the highest priority has the lowest associated value Pril,i,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported), i1,8,l (l=1,…,υ) and the second wideband CQI (if reported).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυQ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the max0,KNZ2-υ highest priority elements of i2,4,l, the max0,KNZ2-υ highest priority elements of i2,5,l (l=1,…,υ), i1,10,l (if reported) and the second subband CQI of even subbands (if reported).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,4,l, the minKNZ-υ,KNZ2 lowest priority elements of i2,5,l (l=1,…,υ) and the second subband CQI of odd subbands (if reported).

Table 5.2.3-1: Priority reporting levels for Part 2 CSI

Priority 0: For CSI reports 1 to NRep, Group 0 CSI for CSI reports configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 wideband CSI for CSI reports configured otherwise

Priority 1: Group 1 CSI for CSI report 1, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of even subbands for CSI report 1, if configured otherwise

Priority 2: Group 2 CSI for CSI report 1, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of odd subbands for CSI report 1, if configured otherwise

Priority 3: Group 1 CSI for CSI report 2, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of even subbands for CSI report 2, if configured otherwise

Priority 4: Group 2 CSI for CSI report 2, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'. Part 2 subband CSI of odd subbands for CSI report 2, if configured otherwise

⁞

Priority 2NRep-1: Group 1 CSI for CSI report NRep, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of even subbands for CSI report NRep, if configured otherwise

Priority 2NRep: Group 2 CSI for CSI report NRep, if configured as 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'; Part 2 subband CSI of odd subbands for CSI report NRep, if configured otherwise

When the UE is scheduled to transmit a transport block on PUSCH not using repetition type B multiplexed with a CSI report(s), Part 2 CSI is omitted only when  is larger than α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1', where parameters , , , , , , , , QACK/CG-UCI' and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes the  to be less than or equal to α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI'-QCSI-1'.

When the UE is scheduled to transmit a transport block on PUSCH using repetition type B multiplexed with a CSI report(s), Part 2 CSI is omitted only when

OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr

is larger than

minα∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI'-QCSI-1' ,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1',

where parameters , , , Nsymb,nominalPUSCH, Nsymb,actualPUSCH, Msc,nominalUCIl, Msc,actualUCIl, , , QACK/CG-UCI', , and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes

OCSI-2+LCSI-2∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr

to be less than or equal to

minα∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI'-QCSI-1' ,  l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'-QCSI-1'.

When part 2 CSI is transmitted on PUSCH with no transport block, lower priority bits are omitted until Part 2 CSI code rate, which is given by OCSI-2+LCSI-2/(NL⋅Q'CSI,2⋅Qm) where OCSI-2, LCSI-2, NL,Q'CSI,2,Qm are given in clause 6.3.2.4 of [5, 38.212] before HARQ-ACK puncturing part 2 CSI if any, is below a threshold code rate lower than one, where

-	is the CSI offset value from Table 9.3-2 of [6, TS 38.213]

-	R is signaled code rate in DCI

If the UE is in an active semi-persistent CSI reporting configuration on PUSCH, the CSI reporting is deactivated when either the downlink BWP or the uplink BWP is changed. Another activation command is required to enable the semi-persistent CSI reporting.

<omitted text>

### 5.2.5	Priority rules for CSI reports

For two overlapping PUSCHs, the priority rules in this clause are applied for physical channels with same priority index according to clause 9 in [6, TS 38.213].

CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where

-	 for aperiodic CSI reports to be carried on PUSCH  for semi-persistent CSI reports to be carried on PUSCH,  for semi-persistent CSI reports to be carried on PUCCH and  for periodic CSI reports to be carried on PUCCH;

-	 for CSI reports carrying L1-RSRP or L1-SINR and  for CSI reports not carrying L1-RSRP or L1-SINR;

-	c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells;

-	s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations.

A first CSI report is said to have priority over second CSI report if the associated  value is lower for the first report than for the second report.

Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports,

-	if y values are different between the two CSI reports, the following rules apply except for the case when one of the y value is 2 and the other y value is 3 (for CSI reports transmitted on PUSCH, as described in Clause 5.2.3; for CSI reports transmitted on PUCCH, as described in Clause 5.2.4):

-	The CSI report with higher  value shall not be sent by the UE.

-	otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Clause 9.2.5.2 in [6, TS 38.213].

If a semi-persistent CSI report to be carried on PUSCH overlaps in time with PUSCH data transmission in one or more symbols on the same carrier, and if the earliest symbol of these PUSCH channels starts no earlier than N2+d2,1 symbols after the last symbol of the DCI scheduling the PUSCH where d2,1 is the maximum of the d2,1 associated with the PUSCH carrying semi-persistent CSI report and the PUSCH with data transmission, the CSI report shall not be transmitted by the UE. Otherwise, if the timeline requirement is not satisfied this is an error case.

If a UE would transmit a first PUSCH that includes semi-persistent CSI reports and a second PUSCH that includes an UL-SCH on the same carrier, and the first PUSCH transmission would overlap in time with the second PUSCH transmission, the UE does not transmit the first PUSCH and transmits the second PUSCH. The UE expects that the first and second PUSCH transmissions satisfy the above timing conditions for PUSCH transmissions that overlap in time when at least one of the first or second PUSCH transmissions is in response to a DCI format detection by the UE.

<omitted text>

## 5.4	UE CSI computation time

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report,

-	if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and

-	if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n),

where Zref is defined as the next uplink symbol with its CP starting  after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied.

If the PUSCH indicated by the DCI is overlapping with another PUCCH or PUSCH, then the CSI report(s) are multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213] and clause 5.2.5 when applicable, otherwise the CSI report(s) are transmitted on the PUSCH indicated by the DCI.

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts earlier than at symbol Zref,

-	the UE may ignore the scheduling DCI if no HARQ-ACK or transport block is multiplexed on the PUSCH.

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts earlier than at symbol Z'ref(n),

-	the UE may ignore the scheduling DCI if the number of triggered reports is one and no HARQ-ACK or transport block is multiplexed on the PUSCH

-	Otherwise, the UE is not required to update the CSI for the n-th triggered CSI report.

When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the last symbol of the PDCCH triggering the CSI report(s), the PDCCH candidate that ends later in time is used.

Z, Z' and µ are defined as:

Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as

-	(Z1,Z1') of the table 5.4-1 if max{ µPDCCH, µCSI-RS, µUL} ≤ 3 and if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or

-	(Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or

-	(Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', 'cri-SINR', 'ssb-Index-SINR- Index ', or 'cri-SINR- Index ', or

-	(Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index ',  where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or

-	(Z2,Z2') or (Z2+Z2'r,2Z2'+r), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if codebookType is set to 'typeII-CJT-r18' or 'typeII-CJT-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with 1<NTRP≤4 resources, or

-	(Z2+14K-1m,Z2'), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to ‘typeII-Doppler-r18’ or ‘typeII-Doppler-PortSelection-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or

-	(Z2+w,Z2'), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to ‘typeII-Doppler-r18’ or ‘typeII-Doppler-PortSelection-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or

-	(Z2+14K-1m,Z2') or (Z2+14K-1m+Z2'r,2Z2'+r), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to ‘typeII-Doppler-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or

-	(Z2+w,Z2') or (Z2+w+Z2'r,2Z2'+r), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to ‘typeII-Doppler-r18’ and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or

-	(Z2,Z2') of table 5.4-2 otherwise.

-	µ of table 5.4-1 and table 5.4-2 corresponds to the min (µPDCCH, µCSI-RS, µUL) where the µPDCCH corresponds to the subcarrier spacing of the PDCCH with which the DCI was transmitted and µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS corresponds to the minimum subcarrier spacing of the aperiodic CSI-RS triggered by the DCI

Table 5.4-1: CSI computation delay requirement 1

|  | Z1 [symbols] |  |
| --- | --- | --- |
|  | Z1 | Z'1 |
| 0 | 10 | 8 |
| 1 | 13 | 11 |
| 2 | 25 | 21 |
| 3 | 43 | 36 |

Table 5.4-2: CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  | Z3 [symbols] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 | Z3 | Z'3 |
| 0 | 22 | 16 | 40 | 37 | 22 | X0 |
| 1 | 33 | 30 | 72 | 69 | 33 | X1 |
| 2 | 44 | 42 | 141 | 140 | min(44, X3X2+ KB1) | X2 |
| 3 | 97 | 85 | 152 | 140 | min(97, X3+ KB2) | X3 |
| 5 | 388 | 340 | 608 | 560 | min(388, X5+ KB3) | X5 |
| 6 | 776 | 680 | 1216 | 1120 | min(776, X6+ KB4) | X6 |

<omitted text>