3GPP TSG RAN Meeting #101										RP-232459

Bangalore, September 11-15, 2023

## Status Report to TSG

Agenda item:			10.3.3

| WI / SI Name | Introduction of the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) for IoT NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_extLband |  |  |  |  |
| Unique ID | 991041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-231487 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2023 03/2024 | Performance part: 12/2023 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 30% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Luca Lodigiani |
|  | Company | Inmarsat |
|  | Email | Luca.lodigiani@inmarsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | YES |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

- RAN4 experienced very slow progress in reaching agreement on basic system parameters for the band, starting from the frequency range.  This has significantly hindered progress in the early phases of the project.

- RAN4 has finally agreed on the frequency range and baseline parameters during RA4#108.

- The Work Item can now proceed as normal, however, the progress is already behind schedule, with Core Part set to complete by RAN#101.  This is currently not possible, at least 1 quarter extension is required, preferable 2 quarters, to give companies enough time to finish the work.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

## 2.2	RAN2

## 2.3	RAN3

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#108

Band Definition and System Parameters

Agreement:

- Agree Option 1

  - Remove [] from band number.

- Option 1: The Extended L-band should be defined as Table 2.

Table 2: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [253] | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

Agreement:

- Do not specify any guard band.

UE RF Requirements

Agreement:

- Agree Option 1

  - Remove [] from band number.

  - Option 1: The Extended L-band should be defined as Table 2.1-1.

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [253] | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

Agreement:

- Agree Option 1

  - Remove [] from band number.

  - Put [] on the offset range and channel raster

  - Option 1: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.2-1.

Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [253] | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Agreement:

- Specify UE receiver blocking requirements by starting with ECC Report 263. Companies are encouraged to study how to capture these requirements in the specification.

  - Study the feasibility of specifying UE receiver blocking requirements at the 1518 MHz band edge and at the 1520 MHz channel offset.

Agreement:

  - Include the following note in TS 36.102 to capture the potential interference issue:

    - NOTE: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base station.

Agreement:

- Capture a note based on Option 1 text in the TR.

  - Further discussion on band 54.

  - Option 1: For the time being, re-use TN-NTN coexistence assumptions from TR 38.863, which would yield a total separation distance of 5250 m from TN BS site and NTN UE, at least for low-gain land-based NTN UE, compatible with smartphone and IoT use cases.

SAN RF Requirements

Agreement:

- Agree Option 1

  - Remove [] from band number.

  - Option 1: The Extended L-band should be defined as Table 2.1-1.

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [253] | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

Agreement:

- Agree Option 1

  - Remove [] from band number.

  - Put [] on the offset range and channel raster

  - Option 1: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.2-1.

Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [253] | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Running Draft CRs

| TDoc | CR Name |
| --- | --- |
| R4-2314717 | Draft CR to TS36.102 Introduction of the Extended L-band |
| R4-2314718 | Draft CR to TS36.108 Introduction of the Extended L-band |
| R4-2314719 | Draft CR to TS36.133 Introduction of the Extended L-band |

RAN4#107

- Work Plan was Approved: R4-2309660 - Draft Work plan for IoT NTN Extended L-band

- For the extended L-band, the spurious emissions requirements of TS36.102 can be reused as a starting point.

- Agree on option 1: For the extended L-band, the UE co-existence emission requirements for band 255 can be reused as starting point.

  - FFS on how to capture ETSI requirements

RAN4#106bis-e

- Agree on Band [253] as the band numbering.

- Consider if Option 1 can be agreed as a starting point, pending further checking of the frequency range.

  - Option1: Agree on Default TX-RX separation of -150 MHz

- Companies to further check ECC Report 263 and evaluate, based on current blocking requirements for NB1, NB2 and M1, whether additional blocking requirements need to be considered.

- For the Extended L-band, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Table 2.5-1 and 2.5-2, respectively (ZTE)
 
Table 2.5-1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [253] |  |  | 23 | +/-2 | [20] | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 2.5-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| [253] | 23 | +/-2 | [20] | +/-2 |

- Reuse existing MPR requirements from 36.102 at least as a starting point for M1 and NB1/NB2. Different MPR tables for category M1 and NB1/NB2 separately

- For the Extended L-band, use UE channel bandwidth and transmission bandwidth configuration for eMTC NTN UL operation defined in Table 2.2-1 below.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

- Further discuss whether A-MPR requirements are needed

- Further discuss spurious emissions requirements.

- For the Extended L-band, use UE channel bandwidth and transmission bandwidth configuration for NB-IoT NTN UL operation defined in Table 2.2-2 below as a starting point.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

- Consider in-band blocking requirements from 36.102 as a starting point for further discussion, pending clarifications on system parameters.

- Further discuss out-of-band blocking requirements.

- Draft running CR for TS 36.102: Postpone discussion of CRs to later meetings when agreement is reached on system parameters and general requirements

- For the Extended L-band, use SAN channel bandwidth and transmission bandwidth configuration for eMTC NTN operation defined in Table 2.2-1 below as a starting point, and further discuss to align with UE.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

- For the Extended L-band, use SAN channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should defined in Table 2.2-2 as a starting point and further discuss to make sure it is aligned with agreements on the UE side.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

- Draft running CR for TS 36.108:  Postpone discussion of CRs to later meetings when agreement is reached on system parameters and general requirements

RAN4#107

- Work Plan was Approved: R4-2309660 - Draft Work plan for IoT NTN Extended L-band

- For the extended L-band, the spurious emissions requirements of TS36.102 can be reused as a starting point.

- Agree on option 1: For the extended L-band, the UE co-existence emission requirements for band 255 can be reused as starting point.

  - FFS on how to capture ETSI requirements

#### Remaining Open issues

After RAN4#108

- REFSENS values.

- Majority of SAN RF requirements

- Majority of UE RF requirements.

- Capturing of UE Blocking Requirements in line with ECC 263

- Alignment to ETSI EN 301 681

- Coexistence note for band 54

- Alignment to common TR with other IoT NTN band (e.g. LSband).

After RAN4#107

- Band definition: DL frequency range.

- General system parameters

- Majority of SAN RF requirements

- Majority of UE RF requirements.

- Alignment to common TR with other IoT NTN band (e.g. LSband).

The main open issue remains the DL frequency range.  The MSS Extended L-band band is recognized internationally at ITU-R, WRC and regional regulatory level, and moreover agreed in RAN#99, to have the following range:

- UL 1668-1675MHz (Earth-to-space), DL 1518-1525MHz (space-to-Earth)

Therefore, the following frequency table has been proposed:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 253 | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

However, one company does not agree with the DL frequency range and would like to introduce a 3 MHz guard band.  The issue was discussed and left open in RAN4#106-bis-e.  During discussion in RAN4#107, a proposal was discussed on whether to define one single band with the full DL frequency range, or define two bands, one with the full DL frequency range, one with a reduced range including TBD guard band.

Agreement could not be reached.

The issue remains open and is blocking progress.

As a secondary open issue, the question remains on how to handle UE blocking requirements from ECC Report 263.

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN WG4#106bis-e meeting

- R4-2304492	Discussion on NTN FDD band (extended L band) UE RF requirements	Mediatek India Technology Pvt.

- R4-2304798	Discussion on UE RF requirements for the Extended L-band	ZTE Corporation

- R4-2304799	Draft CR to TS36.102 Introduction of the Extended L-band	ZTE Corporation

- R4-2304800	Discussion on SAN RF requirements for the Extended L-band	ZTE Corporation

- R4-2304801	Draft CR to TS36.108 Introduction of the Extended L-band	ZTE Corporation

- R4-2305825	Extended L band for LTE NB-IoT	Qualcomm Incorporated

- R4-2306198	Topic summary for [106-bis-e][118] IoT_NTN_extLband	Moderator (Inmarsat)

- R4-2306281	Topic summary for [106-bis-e][118] IoT_NTN_extLband	Moderator (Inmarsat)

- R4-2306658	WF on IoT NTN Extended L-band		Moderator (Inmarsat)

## 4.2	RAN WG4#107 meeting

- R4-2308590	Discussion on UE RF requirements for the Extended L-band	ZTE Corporation

- R4-2308591	Draft CR to TS36.102 Introduction of the Extended L-band	ZTE Corporation

- R4-2308592	Discussion on SAN RF requirements for the Extended L-band	ZTE Corporation

- R4-2308593	Draft CR to TS36.108 Introduction of the Extended L-band	ZTE Corporation

- R4-2309506	Guard band discussion for the extended L band	Qualcomm Incorporated

- R4-2309507	UE blocking requirements for the Extended L band	Qualcomm Incorporated

- R4-2309660	Draft Work plan for IoT NTN Extended L-band	Inmarsat

- R4-2309661	Clarification on IoT NTN Extended L-band Parameters	Inmarsat

- R4-2309721	IoT NTN Extended L-band UE requirements	Inmarsat

- R4-2310005	Topic summary for [107][122] IoT_NTN_extLband	Moderator (Inmarsat)

- R4-2310295	WF on IoT_NTN_extLband		Moderator (Inmarsat)