3GPP TSG-WG1 Meeting # 114	R1-2308693

# Toulouse, France, August 21st - August 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1436 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of IoT (Internet of Things) NTN (non-terrestrial network) enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of IoT (Internet of Things) NTN (non-terrestrial network) enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for NB-IoT/eMTC enhancements for Non-Terrestrial Networks according to RAN1 agreements: - disabling of HARQ feedback for IoT NTN - NPDCCH monitoring restriction for UL HARQ mode B operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of IoT NTN enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.3, 10.2, 16.4.2, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.1	UE procedure for receiving the physical downlink shared channel

<Unchanged parts are omitted>

For BL/CE UEs, the set of BL/CE DL subframes is indicated as follows

-	If DL resource reservation is enabled for the UE as specified in [11],

-	for PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PDSCH transmission without a corresponding MPDCCH,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE DL subframes corresponds to all downlink subframes and special subframes during the PDSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE DL subframes corresponds to all downlink subframes and special subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the PDSCH transmission are reserved in the subframe);

-	for MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	the set of BL/CE DL subframes corresponds to all downlink subframes and available special subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the MPDCCH transmission are reserved in the subframe).

-	In all other cases, the set of BL/CE DL subframes is indicated by the higher layers according to fdd-DownlinkOrTddSubframeBitmapBR [11].

A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A, 6-1B, 6-2 intended for the UE, decode the corresponding PDSCH in one more BL/CE DL subframes as described in Clause 7.1.11, with the restriction of the number of transport blocks defined in the higher layers.

For a BL/CE UE in a NTN FDD serving cell with a PDSCH ending in subframe n, and the UE configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap or higher layer parameter downlinkHARQ-FeedbackDisabled-DCI, if the UE shall not provide HARQ-ACK for a HARQ process associated with a transport block in the PDSCH, the UE is not expected to receive a MPDCCH or a PDSCH without a corresponding MPDCCH for the same HARQ process as the PDSCH ending in subframe n in any BL/CE DL subframe starting from subframe n+1 to subframe n+3.

For the purpose of decoding PDSCH containing SystemInformationBlockType2, a BL/CE UE shall assume that subframes in which SystemInformationBlockType2 is scheduled are non-MBSFN subframes.

If a UE is configured with more than one serving cell and if the frame structure type of any two configured serving cells is different, then the UE is considered to be configured for FDD-TDD carrier aggregation.

<Unchanged parts are omitted>

## 7.3	UE procedure for reporting HARQ-ACK

If the UE is not configured with shortTTI, the term 'subframe/slot' refers to a subframe in this clause.

<Unchanged parts are omitted>

If the UE is configured with higher layer parameter blindSlotSubslotPDSCH-Repetitions for a given serving cell, UE procedure for HARQ-ACK reporting for the serving cell corresponding to a PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G with CRC scrambled by C-RNTI is given in this clause assuming the slot/subslot-PDSCH is received in the last slot/subslot of the set of received k DL slots/subslots according to the PDCCH/SPDCCH information as described in clause 7.1.

For a BL/CE UE in a NTN FDD serving cell, and the UE not configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap indicating enabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, the UE shall provide HARQ-ACK for the HARQ process associated with the transport block.

For a BL/CE UE in a NTN FDD serving cell, and the UE configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, or the UE configured with CEModeB and higher layer parameter downlinkHARQ-FeedbackDisabled-DCI, the UE shall provide HARQ-ACK for a HARQ process associated with a transport block in a detected PDSCH

- if the UE is configured with CEModeA, and configured with higher layer parameter harq-FeedbackEnablingforSPSactive = 'enabled', and the detected PDSCH is the first SPS PDSCH after SPS activation, or

- if the UE is configured with CEModeB, and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI, and the value of the HARQ-ACK resource offset field in the DCI format 6-1B of the corresponding MPDCCH is not set to ‘3’.

For a BL/CE UE in half-duplex FDD operation in a NTN serving cell, if the UE is configured with CEModeA, and configured with higher layer parameter ce-HARQ-AckBundling, and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, the UE is not expected to receive the corresponding DCI with HARQ-ACK bundling flag set to 1.

For a BL/CE UE, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI format 6-1A with CRC scrambled by C-RNTI,

-	for the UE in a NTN FDD serving cell, if the UE shall provide HARQ-ACK for at least one TB of the multiple TB, and if the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block of the multiple TB, the UE shall generate an ACK for HARQ-ACK corresponding to the transport block associated with the HARQ process with disabled HARQ-ACK information;

-	for HARQ-ACK transmission associated with the corresponding DCI, the UE shall generate M HARQ-ACK bits by performing a logical AND operation of HARQ-ACKs across all TBs in each TB bundle Ab where b = 1, …, M;

-	the set of TBs that belong to TB bundle Ab and the number of TB bundles M are given by Table 7.3-1;

-	the value of is the number of scheduled TB determined in the corresponding DCI.

<Unchanged parts are omitted>

## 10.2	Uplink HARQ-ACK timing

For TDD or for FDD-TDD and primary cell frame structure type 2 or for FDD-TDD and primary cell frame structure type 1, if a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, "UL/DL configuration" of the serving cell in Clause 10.2 refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

<Unchanged parts are omitted>

For FDD, if a BL/CE UE is configured with CEModeA, and if the UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki+Koffset with b=0,1,⋯NTB-1,  i =0,1, …, N-1, where

-	if the UE is in a NTN serving cell and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH

-	 is the number of scheduled TB associated with HARQ processes with enabled HARQ-ACK information and with TB indices in increasing order denoted by t0,t1,t2, …tNTB-1

-	otherwise

-	is the number of scheduled TB determined in the corresponding DCI, and tb=b;

-	if the UE is not configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config and the UE is not in half-duplex FDD operation

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	otherwise

-	s0=max n0+4,  nL+2, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	nb is the last subframe in which the PDSCH containing TB tb bis transmitted;

-	subframe nL is the last subframe in which the PDSCH is transmitted;

-	Nb denotes the number of consecutive subframes including non-BL/CE subframes where the PUCCH with HARQ ACK for TB tb bwith repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) sb+ki+Koffset with i=0,1,…,N-1 for TB tb bare N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+Koffset, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

<Unchanged parts are omitted>

### 16.4.2	UE procedure for reporting ACK/NACK

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	 DL subframe for FDD,

-	 NB-IoT UL subframes following the end of n+12 subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where

-	, where

-	the value of is given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise,

-	the value of  is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and

-	if the UE is configured with higher layer parameter harq-ACK-Bundling in npdsch-MultiTB-Config, or if the UE is in a NTN serving cell and multiple TB are scheduled in the NPDCCH corresponding to the NPDSCH and the UE is not configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the NPDSCH, then , otherwise , where the value of is determined by the Number of scheduled TB for Unicast field if present in the NPDCCH corresponding to the NPDSCH, otherwise ,

-	allocated subcarrier for ACK/NACK and value of k0 is determined by the ACK/NACK resource field in the DCI format of the corresponding NPDCCH according to Table 16.4.2-1, and Table 16.4.2-2,

-	for FDD, .

-	for TDD, .

-	For

-	if the UE is configured with higher layer parameter harq-AckBundling in npdsch-MultiTB-Config, and the NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI,

-	if the UE is in a NTN serving cell and if the UE is not configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the NPDSCH, the UE shall generate an ACK for HARQ-ACK corresponding to the transport block

-	the ACK/NACK response is generated by performing a logical AND operation of HARQ-ACKs corresponding to the TBr+1 ,

-	otherwise,

-	if

-	the ACK/NACK response is the HARQ-ACK corresponding to the transport block associated with the HARQ process with enabled HARQ-ACK information

-	otherwise

-	NB-IoT UL slots  with  of the NPUSCH carry ACK/NACK response for TBr+1 ,

except if the UE is in a NTN serving cell, and the UE is not configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information for all HARQ process(es) associated with transport block(s) in the NPDSCH, or the HARQ feedback disabled indicator is present in the NPDCCH corresponding to the NPDSCH.

Table 16.4.2-1: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 38 | 13 |
| 1 | 39 | 13 |
| 2 | 40 | 13 |
| 3 | 41 | 13 |
| 4 | 42 | 13 |
| 5 | 43 | 13 |
| 6 | 44 | 13 |
| 7 | 45 | 13 |
| 8 | 38 | 21 |
| 9 | 39 | 21 |
| 10 | 40 | 21 |
| 11 | 41 | 21 |
| 12 | 42 | 21 |
| 13 | 43 | 21 |
| 14 | 44 | 21 |
| 15 | 45 | 21 |

Table 16.4.2-2: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 0 | 13 |
| 1 | 1 | 13 |
| 2 | 2 | 13 |
| 3 | 3 | 13 |
| 4 | 0 | 15 |
| 5 | 1 | 15 |
| 6 | 2 | 15 |
| 7 | 3 | 15 |
| 8 | 0 | 17 |
| 9 | 1 | 17 |
| 10 | 2 | 17 |
| 11 | 3 | 17 |
| 12 | 0 | 18 |
| 13 | 1 | 18 |
| 14 | 2 | 18 |
| 15 | 3 | 18 |

<Unchanged parts are omitted>

## 16.6	Narrowband physical downlink control channel related procedures

Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0.

A UE shall monitor a set of NPDCCH candidates (described in Clause 10.2.5.1 of [3]) as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the NPDCCHs in the set according to all the monitored DCI formats.

The set of NPDCCH candidates to monitor are defined in terms of NPDCCH search spaces.

<Unchanged parts are omitted>

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3, or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 except if the UE is configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’ for the same HARQ process ID, or if the NPUSCH transmission carries ACK/NACK response, as determined in clause 16.4.2, for the same HARQ process ID, and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information for the same HARQ process ID and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3] for FDD; and

-	the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3, or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 except if the UE is configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’, or if the NPUSCH transmission carries ACK/NACK response as determined in clause 16.4.2 and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabled-Bitmap-NB indicating disabled HARQ-ACK information and configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI-NB.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5

otherwise

-	the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

<Unchanged parts are omitted>