3GPP TSG-RAN WG1 Meeting #114	R1-2308713

# Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0148 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR positioning enhancement in Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of NR positioning enhancement in Rel-18 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-18 NR positioning enhancement: - Section 3.3: Add the abbreviation of SL-PRS - Section 7.3.1 & 7.3.1.4.3: Define the new DCI format for scheduling SL-PRS in the dedicated resource pool for scheme 1 - Section 8.3.1.1 & 8.3.1.2: Define the new SCI format for scheduling SL-PRS in the dedicated resource pool - Section 8.4.1.4: Define the new 2nd-stage SCI for scheduling SL-PRS in the shared resource pool - Section 8.4.4: Update the number of symbols for rate matching for the 2nd- stage SCI considering the symbols configured for SL PRS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR positioning enhancement in Rel-18 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.3.1, 7.3.1.4.3 (new), 8.3.1.1, 8.3.1.2 (new), 8.4.1.4 (new), 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CG	Configured grant

CG-DFI	CG downlink feedback information

CG-UCI	CG uplink control information

CORESET	Control resource set

COT	Channel occupancy time

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MBS	Multicast broadcast services

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSSCH	Physical sidelink shared channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SL-BCH	Sidelink broadcast channel

SL PRS               Sidelink positioning reference signal

SL-SCH	Sidelink shared channel

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

< Unchanged parts are omitted >

## 7.3	Downlink control information

A DCI transports downlink control information for one or more cells with one RNTI.

The following coding steps can be identified:

-	Information element multiplexing

-	CRC attachment

-	Channel coding

-	Rate matching

### 7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI) |
| 0_2 | Scheduling of PUSCH in one cell |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of one or multiple PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback |
| 1_2 | Scheduling of PDSCH in one cell |
| 2_0 | Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |
| 2_4 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE |
| 2_5 | Notifying the availability of soft resources as defined in Clause 9.3.1 of [10, TS 38.473] |
| 2_6 | Notifying the power saving information outside DRX Active Time for one or more UEs |
| 2_7 | Notifying paging early indication and TRS availability indication for one or more UEs. |
| 3_0 | Scheduling of NR sidelink in one cell |
| 3_1 | Scheduling of LTE sidelink in one cell |
| 3_2 | Scheduling of NR SL PRS in one cell |
| 4_0 | Schedulng of PDSCH with CRC scrambled by MCCH-RNTI/G-RNTI for broadcast |
| 4_1 | Schedulng of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast |
| 4_2 | Schedulng of PDSCH with CRC scrambled by G-RNTI/G-CS-RNTI for multicast |

The fields defined in the DCI formats below are mapped to the information bits  to  as follows.

Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit  and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to .

If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12.

The size of each DCI format is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary.

If a UE is configured with pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookList-r16 in this clause.

If a UE is configured with pdsch-HARQ-ACK-CodebookListMulticast-r17, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookListMulticast-r17 in this clause.

< Unchanged parts are omitted >

#### 7.3.1.4	DCI formats for scheduling of sidelink

< Unchanged parts are omitted >

##### 7.3.1.4.3	Format 3_2

DCI format 3_2 is used for scheduling of NR SL PRS in one cell.

The following information is transmitted by means of the DCI format 3_2 with CRC scrambled by XX-RNTI or YY-CS-RNTI:

-	Resource pool index –log2I bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in [6, TS 38.214]

-	First SL PRS indicator - log2NSL-PRS bits indicating the SL PRS resource ID for the first SL PRS transmission, where the value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated resource pool for SL PRS transmission and provided by the higher layer parameter XYZ.

-	SCI format 1-B fields according to clause 8.3.1.2:

-	Time resource assignment

-	Resource ID indication

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_2 with CRC scrambled by YY-CS-RNTI; otherwise 3 bits as defined in clause XYZ of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_2 with CRC scrambled by YY-CS-RNTI, this field is reserved for DCI format 3_2 with CRC scrambled by XX-RNTI.

-	Padding bits, if required

< Unchanged parts are omitted >

## 8.3	Sidelink control information on PSCCH

SCI carried on PSCCH is a 1st-stage SCI, which transports sidelink scheduling information.

### 8.3.1	1st-stage SCI formats

The fields defined in each of the 1st-stage SCI formats below are mapped to the information bits a0 to aA-1 as follows:

Each field is mapped in the order in which it appears in the description, with the first field mapped to the lowest order information bit a0 and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to a0.

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

< Unchanged parts are omitted >

-	2nd-stage SCI format – 2 bits as defined in Table 8.3.1.1-1.

-	Beta_offset indicator – 2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-2.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Additional MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl-Additional-MCS-Table; 0 bit otherwise.

-	PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

-	Reserved – a number of bits as determined by the following:

-	Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter sl-IndicationUE-B is not configured, or if higher layer parameter sl-IndicationUE-B is configured to 'disabled';

-	(Nreserved-1) bits otherwise, with value set to zero.

-	Conflict information receiver flag – 0 or 1 bit

-	1 bit if higher layer parameter sl-IndicationUE-B is configured to 'enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213];

-	0 bit otherwise.

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 | SCI format 2-A |
| 01 | SCI format 2-B |
| 10 | SCI format 2-C |
| 11 | SCI format 2-D |

Table 8.3.1.1-2: Mapping of Beta_offset indicator values to indexes in Table 9.3-2 of [5, TS38.213]

| Value of Beta_offset indicator | Beta_offset index in Table 9.3-2 of [5, TS38.213] |
| --- | --- |
| 00 | 1st index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 01 | 2nd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 10 | 3rd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 11 | 4th index provided by higher layer parameter sl-BetaOffsets2ndSCI |

Table 8.3.1.1-3: Number of DMRS port(s)

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

< Unchanged parts are omitted >

#### 8.3.1.2	SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS for a dedicated resource pool.

The following information is transmitted by means of the SCI format 1-B:

-	Priority – 3 bits as specified in clause x.x of [12, TS 23.586] and clause x.x of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Source ID – 12 or 24 bits determined by higher layer parameter XYZ, as defined in clause x.x of [6, TS 38.214].

-	Destination ID – 24 bits as defined in clause x.x of [6, TS 38.214].

-	Cast type indicator – 2 bits as defined in Table 8.3.1.2-1 and in clause x.x of [6, TS 38.214].

-	Resource reservation period – log2Nrsv_period bits as defined in clause xx of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP, if higher layer parameter reservationPeriodAllowed-Dedicated-SL-PRS-RP is configured; 0 bit otherwise.

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserveSL-PRS is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserveSL-PRS is configured to 3, as defined in clause x.x.x of [6, TS 38.214].

-	Resource ID indication – log2NSL-PRS bits when the value of the higher layer parameter sl-MaxNumPerReserveSL-PRS is configured to 2; otherwise x bits when the value of the higher layer parameter sl-MaxNumPerReserveSL-PRS is configured to 3. The value NSL-PRS is the total number of SL PRS resources within a slot in a dedicated resource pool for SL PRS transmission and provided by the higher layer parameter XYZ.

-	SL PRS request – 1 bit as defined in clause x.x of [TS 38.214] when the higher layer parameter XYZ is provided; 0 bit otherwise.

-	Reserved - Nreserved bits as configured by higher layer parameter XYZ, with value set to zero.

Table 8.3.1.2-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast |
| 10 | Unicast |
| 11 | Reserved |

< Unchanged parts are omitted >

## 8.4	Sidelink control information on PSSCH

SCI carried on PSSCH is a 2nd-stage SCI, which transports sidelink scheduling information, and/or inter-UE coordination related information.

### 8.4.1	2nd-stage SCI formats

The fields defined in each of the 2nd-stage SCI formats below are mapped to the information bits  a0 to aA-1 as follows:

Each field is mapped in the order in which it appears in the description, with the first field mapped to the lowest order information bit a0 and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to a0.

< Unchanged parts are omitted >

#### 8.4.1.4	SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared resource pool.

The following information is transmitted by means of the SCI format 2-D:

-	SL PRS resource ID – log2NSL-PRS bits, where the value NSL-PRS is the total number of SL PRS resource IDs within a slot in a shared resource pool for SL PRS transmission and provided by the higher layer parameter XYZ.

-	SL PRS request – 1 bit as defined in clause x.x of [6, TS 38.214] when the higher layer parameter XYZ is provided; 0 bit otherwise.

-	Embedded SCI format – 2 bits. This field indicates the embedded SCI format as defined in Table 8.4.1.4-1.

-	Embedded SCI format payload – number of bits determined according to Table 8.4.1.4-1. This field is set to the associated payload of the embedded SCI format indicated by the ‘Embedded SCI format’ field as defined in Table 8.4.1.4-1.

Table 8.4.1.4-1: Embedded SCI format and payload

| Value of the Embedded SCI format field | Embedded SCI format | Embedded SCI format payload |
| --- | --- | --- |
| 00 | SCI format 2-A | Set to all fields included in SCI format 2-A |
| 01 | SCI format 2-B | Set to all fields included in SCI format 2-B |
| 10 | Reserved | Reserved |
| 11 | Reserved | Reserved |

< Unchanged parts are omitted >

### 8.4.4	Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the 2nd-stage SCI bits

-	LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is 24 bits.

-	βoffsetSCI2 is indicated in the corresponding 1st-stage SCI.

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers.

-	MscPSCCH(l) is the number of subcarriers in OFDM symbol l that carry PSCCH and PSCCH DMRS associated with the PSSCH transmission.

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH-1 and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH-NsymbSL PRS, in PSSCH transmission, where NsymbshNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214]. NsymbSL PRS is the number of symbols for SL PRS provided by the higher layer parameter XYZ if the 2nd-stage SCI is SCI format 2-D, and NsymbSL PRS = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 2 or 4,  NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	MscSCI2(l) = MscPSSCH(l) - MscPSCCH(l)

-	γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs.

-	R is the coding rate as indicated by "Modulation and coding scheme" field in SCI format 1-A.

-	α is configured by higher layer parameter sl-Scaling.

The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits.

Rate matching is performed according to Clause 5.4.1 by setting IBIL=1.

The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2, where GSCI2=QSCI2'∙ QmSCI2 and QmSCI2 is modulation order of the 2nd-stage SCI. A UE is not expected to have GSCI2>4096.