3GPP TSG-RAN WG1 Meeting #114 	R1-2308737

Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0155 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-18 XR enhancements for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of Rel-18 XR enhancements for NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-18 XR enhancements for NR. Capture agreements on UTO-UCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 XR enhancements for NR will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.7, 6.3.2.1.3A(new), 6.3.2.1.4, 6.3.2.1.5, 6.3.2.4.2, 6.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38. 214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CG	Configured grant

CG-DFI	CG downlink feedback information

CG-UCI	CG uplink control information

CORESET	Control resource set

COT	Channel occupancy time

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MBS	Multicast broadcast services

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSSCH	Physical sidelink shared channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SL-BCH	Sidelink broadcast channel

SL-SCH	Sidelink shared channel

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

UTO-UCI 	Unused transmission occasion uplink control information

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

< Unchanged parts are omitted >

### 6.2.7	Data and control multiplexing

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "when higher layer parameter cg-UCI-Multiplexing is configured" with "when UTO-UCI and HARQ-ACK are transmitted on a PUSCH".

Denote the coded bits for UL-SCH as .

Denote the coded bits for HARQ-ACK or jointly coded bits for HARQ-ACK and CG-UCI when the high layer parameter cg-UCI-Multiplexing is configured, if any, as .

Denote the coded bits for CSI part 1, if any, as .

Denote the coded bits for CSI part 2, if any, as .

Denote the coded bits for CG-UCI without HARQ-ACK, if any, as g0CG-UCI,  g1CG-UCI,  g2CG-UCI, g3CG-UCI, …, gGCG-UCI-1CG-UCI.

Denote the multiplexed data and control coded bit sequence as .

Denote  as the OFDM symbol index of the scheduled PUSCH, starting from 0 to , where  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS.

Denote  as the subcarrier index of the scheduled PUSCH, starting from 0 to , where  is expressed as a number of subcarriers.

Denote  as the set of resource elements, in ascending order of indices , available for transmission of data in OFDM symbol , for .

Denote  as the number of elements in set . Denote  as the -th element in .

Denote  as the set of resource elements, in ascending order of indices , available for transmission of UCI in OFDM symbol , for . Denote  as the number of elements in set . Denote  as the -th element in . For any OFDM symbol that carriers DMRS of the PUSCH, . For any OFDM symbol that does not carry DMRS of the PUSCH, .

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK, let

-	GCG-UCI1=NL∙Qm∙GCG-UCI2∙NL∙Qm and GCG-UCI2=NL∙Qm∙GCG-UCI2∙NL∙Qm

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

-	if only CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

If frequency hopping is not configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS;

-	if HARQ-ACK is present for transmission on the PUSCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let ;

-	if CSI is present for transmission on the PUSCH, let  and ;

-	if CG-UCI is present for transmission on the PUSCH without HARQ-ACK, let GCG-UCI1=GCG-UCI;

-	let  and .

The multiplexed data and control coded bit sequence  is obtained according to the following:

Step 1:

Set  for ;

Set  for ;

Set  for ;

Set  for ;

if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits and without CG-UCI

the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Clause 6.3.2.4.2.1, by setting ;

denote  as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements;

if frequency hopping is configured for the PUSCH, let  and ;

if frequency hopping is not configured for the PUSCH, let ;

denote  as the set of reserved resource elements for potential HARQ-ACK transmission, in OFDM symbol , for ;

Set ;

Set ;

for ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

end for

end if

;

end while

end for

else

for ;

end if

Denote  as the number of elements in .

Step 2:

if HARQ-ACK is present for transmission on the PUSCH and the number of HARQ-ACK information bits is more than 2 or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH,

Set ;

Set ;

Set ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

end if

Step 2A:

If CG-UCI is present for transmission on the PUSCH without HARQ-ACK,

Set mcountCG-UCI1=0;

Set mcountCG-UCI2=0;

Set mcount,allCG-UCI=0;

for i=1 to NhopPUSCH

l=l(i);

while mcountCG-UCIi<GCG-UCI(i)

if MscUCIl>0

if GCG-UCI(i) -mcountCG-UCI1≥MscUCIl.NL.Qm

d=1;

mcountRE=MscUCIl;

end if

if GCG-UCI(i) -mcountCG-UCI1<MscUCIl.NL.Qm

d=MscUCIl.NL.QmGCG-UCI(i) -mcountCG-UCIi;

mcountRE=GCG-UCI(i) -mcountCG-UCIiNL.Qm;

end if

for j=0 to  mcountRE-1

k=ΦlUCI(j.d);

for v=0 to NL.Qm-1

gl,k,v=gmcount,allCG-UCICG-UCI;

mcount,allCG-UCI=mcount,allCG-UCI+1;

mcountCG-UCIi=mcountCG-UCIi+1;

end for

end for

Φl,tmpUCI=∅;

for j=0 to mcountRE-1

Φl,tmpUCI=Φl,tmpUCI∪ΦlUCI(j.d);

end for

ΦlUCI=ΦlUCI\Φl,tmpUCI;

ΦlUL-SCH=ΦlUL-SCH\Φl,tmpUCI;

MscUCIl=ΦlUCI;

MscUL-SCHl=ΦlUL-SCH;

end if

l=l+1;

end while

end for

end if

Step 3:

if CSI is present for transmission on the PUSCH,

Set ;

Set ;

Set ;

for  to

;

while

;

end while

while

if

if

;

;

end if

if

;

;

end if

;

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

Set ;

Set ;

Set ;

for  to

;

while

;

end while

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

end if

Step 4:

if UL-SCH is present for transmission on the PUSCH,

Set ;

for  to

if

for  to

;

for  to

;

;

end for

end for

end if

end for

end if

Step 5:

if HARQ-ACK is present for transmission on the PUSCH without CG-UCI and the number of HARQ-ACK information bits is no more than 2,

Set ;

Set ;

Set ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

end if

;

end while

end for

end if

Step 6:

Set ;

for  to

for  to

;

for  to

;

;

end for

end for

end for

< Unchanged parts are omitted >

### 6.3.2	Uplink control information on PUSCH

< Unchanged parts are omitted >

#### 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

##### 6.3.2.1.3	CG-UCI

For CG-UCI bits transmitted on a CG PUSCH when the higher layer parameter cg-RetransmissionTimer is configured, the CG-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiCG-UCI  for i=0,1, …, OCG-UCI-1 and A=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.

Table 6.3.2.1.3-1: Mapping order of CG-UCI fields

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 5 if nrofHARQ-Processes-v1700 in ConfiguredGrantConfig is configured; 4 otherwise. |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ul-toDL-COT-SharingED-Threshold and higher layer parameter cg-COT-SharingList are configured, or if both higher layer parameter semiStaticChannelAccessConfigUE and higher layer parameter cg-COT-SharingList are configured, or if higher layer parameter cg-COT-SharingList is configured in frequency range 2-2, where C is the number of combinations configured in cg-COT-SharingList; 1 if higher layer parameter ul-toDL-COT-SharingED-Threshold is not configured, and if higher layer parameter semiStaticChannelAccessConfigUE is not configured, and if higher layer parameter cg-COT-SharingOffset is configured; 0 otherwise. If a UE indicates COT sharing other than "no sharing" in a CG PUSCH within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent CG PUSCHs, if any, occurring within the same UE's initiated COT such that the same DL starting point and duration are maintained. |

##### 6.3.2.1.3A	UTO-UCI

For UTO-UCI bits transmitted on a CG PUSCH when the higher layer parameter nrof_UTO_UCI is configured, the UTO-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiUTO-UCI  for i=0,1, …, OUTO-UCI-1 and A=OUTO-UCI, where OUTO-UCI is provided by nrof_UTO_UCI, and the UTO-UCI bit sequence o0UTO-UCI, o1UTO-UCI, …, oOUTO-UCI-1UTO-UCI is given by clause x.x of [5, TS 38.213].

##### 6.3.2.1.4	HARQ-ACK and CG-UCI/UTO-UCI

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.1.4 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "When higher layer parameter cg-UCI-Multiplexing is configured" with "When UTO-UCI and HARQ-ACK are transmitted on a PUSCH".

When higher layer parameter cg-UCI-Multiplexing is configured, the UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows, where A=OCG-UCI+OACK.

-	The CG-UCI bits are mapped to the UCI bit sequencea0, a1, a2, a3, …,aOCG-UCI-1 , where ai=oiCG-UCI for i=0,1, …, OCG-UCI-1. The CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part, and OCG-UCI is number of CG-UCI bits;

-	The HARQ-ACK bits are mapped to the UCI bit sequence aOCG-UCI, aOCG-UCI+1,  …,aOCG-UCI+OACK-1, where ai+OCG-UCI=oiACK for i=0,1, …, OACK-1. The HARQ-ACK bit sequence o0ACK, o1ACK, …, oOACK-1ACK is given by Clause 9.1 of [5, TS38.213], and OACK is number of HARQ-ACK bits.

##### 6.3.2.1.5	UCI with different priority indexes

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.1.5 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part" with "is given by clause x.x of [5, TS 38.213]".

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1, the following UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	If CSI part 1 is also transmitted on the PUSCH,

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1).

-	Set  ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Otherwise, set  ai(1)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0, the following UCI bit sequences are generated, a0, a1, a2, a3, …,aA-1, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 if any, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	If HARQ-ACK bits associated with priority index 1 and CSI are transmitted on the PUSCH without UL-SCH and the CSI includes CSI part 1 without CSI part 2, and there is only one HARQ-ACK bit associated with priority index 1 given by Clause 9.1 of [5, TS 38.213], set a0=o0ACK-HP, a1=0, and A=2; otherwise, set ai=oiACK-HP  for i=0,1,…,OACK-HP-1 and A=OACK-HP, where the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP associated with priority index 1 is given by Clause 9.1 of [5, TS 38.213];

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, if CSI part 1 is also transmitted on the PUSCH, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1);

-	Set ai(2) for i=0,1,…,A(2)-1 as the bit sequence of CSI part 2, if CSI part 2 is also transmitted on the PUSCH, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence a0(2), a1(2), a2(2), a3(2), …,aA(1)-12 starting with a0(2).

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH, the following UCI bit sequences are generated, a0, a1, a2, a3, …,aA-1, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	Set ai=oiACK-HP  for i=0,1,…,OACK-HP-1 and A=OACK-HP if HARQ-ACK bits associated with priority index 1 are transmitted without CG-UCI associated with priority index 1, where the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP associated with priority index 1 is given by Clause 9.1 of [5, TS 38.213];

-	Set ai=oiCG-UCI  for i=0,1,…,OCG-UCI-1 and A=OCG-UCI if CG-UCI associated with priority index 1 is transmitted without HARQ-ACK bits associated with priority index 1, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI associated with priority index 1 is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part;

-	Set a0, a1, a2, a3, …,aA-1 as follows, if both CG-UCI associated with priority index 1 and HARQ-ACK bits associated with priority index 1 are transmitted, where A=OCG-UCI+OACK-HP

-	The CG-UCI bits are mapped to the UCI bit sequence a0, a1, a2, a3, …,aOCG-UCI-1, where ai=oiCG-UCI for i=0,1, …, OCG-UCI-1. The CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part, and OCG-UCI is number of CG-UCI bits

-	The HARQ-ACK bits are mapped to the UCI bit sequence aOCG-UCI, aOCG-UCI+1,  …,aOCG-UCI+OACK-HP-1, where ai+OCG-UCI=oiACK-HP for i=0,1, …, OACK-HP-1. The HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP associated with priority index 1 is given by Clause 9.1 of [5, TS 38.213].

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Set ai(1) for i=0,1,…,A(1)-1 as the bit sequence of CSI part 1, where the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 starting with a0(1).

-	Set  ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(2)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Otherwise,

-	Set  ai(1)=oiACK-LP  for i=0,1,…,OACK-LP-1 and A(1)=OACK-LP, where the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Set  ai(2)=ai(1)  for i=0,1,…,A1-1 and A(2)=A(1), if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0, where the CSI part 1 sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-1(1) is given by Table 6.3.2.1.2-6 by replacing a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the CSI part 1 sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-1(1) starting with a0(1).

If uci-MuxWithDiffPrio is configured, and CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 0, the following UCI bit sequences are generated, a0, a1, a2, a3, …,aA-1, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12  if any, according to the following:

-	Set ai=oiACK-HP  for i=0,1,…,OACK-HP-1 and A=OACK-HP, where the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP associated with priority index 1 is given by Clause 9.1 of [5, TS 38.213];

-	Set ai(1)=oiCG-UCI  for i=0,1, …, OCG-UCI-1 and A(1)=OCG-UCI if CG-UCI associated with priority index 0 is transmitted without HARQ-ACK bits associated with priority index 0, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI associated with priority index 0 is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part;

-	Set a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 as follows if both CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 are transmitted, where A(1)=OCG-UCI+OACK-LP

-	The CG-UCI bits are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aOCG-UCI-11, where ai(1)=oiCG-UCI for i=0,1, …, OCG-UCI-1. The CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part, and OCG-UCI is number of CG-UCI bits

-	The HARQ-ACK bits are mapped to the UCI bit sequence aOCG-UCI(1), aOCG-UCI+1(1),  …,aOCG-UCI+OACK-LP-11, where ai+OCG-UCI(1)=oiACK-LP for i=0,1, …, OACK-LP-1. The HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP associated with priority index 0 is given by Clause 9.1 of [5, TS 38.213].

-	Set ai(2)=ai(1)  for i=0,1,…,A1-1 and A(2)=A(1), if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0, where the CSI part 1 sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-1(1) is given by Table 6.3.2.1.2-6 by replacing a0(1), a1(1), a2(1), a3(1), …,aA(1)-11, and the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the CSI part 1 sequence a0(1), a1(1), a2(1), a3(1), …,aA(1)-1(1) starting with a0(1).

< Unchanged parts are omitted >

##### 6.3.2.4.2	UCI encoded by channel coding of small block lengths

If the higher layer parameter nrof_UTO_UCI is configured, the procedures in this clause and the clauses it refers to apply by replacing CG-UCI with UTO-UCI in all the notations and texts.

###### 6.3.2.4.2.1	HARQ-ACK

For HARQ-ACK transmission on PUSCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined according to Clause 6.3.2.4.1.1, by setting the number of CRC bits .

The input bit sequence to rate matching is .

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length , where

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as .

###### 6.3.2.4.2.2	CSI part 1

For CSI part 1 transmission on PUSCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined according to Clause 6.3.2.4.1.2, by setting the number of CRC bits .

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length , where

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as .

###### 6.3.2.4.2.3	CSI part 2

For CSI part 2 transmission on PUSCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined according to Clause 6.3.2.4.1.3, by setting the number of CRC bits .

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length , where

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as .

###### 6.3.2.4.2.4	CG-UCI

For CG-UCI transmission on PUSCH, the number of coded modulation symbols per layer for CG-UCI transmission, denoted as QCG-UCI', is determined according to Clause 6.3.2.4.1.4, by setting the number of CRC bits LCG-UCI=0.

The input bit sequence to rate matching is d0, d1, d2, …, dN-1.

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length

E=NL∙QCG-UCI'∙Qm, where

-	NL is the number of transmission layers of the PUSCH;

-	Qm is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as f0, f1,f2, …, fE-1.

###### 6.3.2.4.2.5	HARQ-ACK and CG-UCI

For HARQ-ACK and CG-UCI transmission on PUSCH, the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmission, denoted as QACK', is determined according to Clause 6.3.2.4.1.5, by setting the number of CRC bits LACK=0.

The input bit sequence to rate matching is d0, d1, d2, …, dN-1.

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length E=NL∙QACK'∙Qm, where

-	NL is the number of transmission layers of the PUSCH;

-	Qm is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as f0, f1,f2, …, fE-1.

###### 6.3.2.4.2.6	UCI with different priority indexes

In this clause, βoffsetHARQ-ACK-LP is equal to βoffsetHARQ-ACK,0 defined in [5, TS38.213] in case of PUSCH associated with priority index 1, and equal to βoffsetHARQ-ACK defined in [5, TS38.213] in case of PUSCH associated with priority index 0. βoffsetHARQ-ACK-HP is equal to βoffsetHARQ-ACK,1 defined in [5, TS38.213] in case of PUSCH associated with priority index 0, and equal to βoffsetHARQ-ACK defined in [5, TS38.213] in case of PUSCH associated with priority index 1.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1:

-	If CSI part 1 is also transmitted on the PUSCH,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.2.2 is 0 bit.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.2.3 is 0 bit.

-	Otherwise, perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and assuming the number of HARQ-ACK information bits to be transmitted on PUSCH in clause 6.3.2.4.2.2 is 0 bit.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH.

-	Perform rate matching for CSI part 2 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 2 is also transmitted on the PUSCH.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP, if HARQ-ACK bits associated with priority index 1 are transmitted without CG-UCI associated with priority index 1.

-	Perform rate matching for CG-UCI with priority index 1 according to clause 6.3.2.4.2.4, if CG-UCI associated with priority index 1 is transmitted without HARQ-ACK bits associated with priority index 1.

-	Perform rate matching for CG-UCI with priority index 1 and HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.5, if both CG-UCI associated with priority index 1 and HARQ-ACK bits associated with priority index 1 are transmitted, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	If CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1,

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.3, by taking HARQ-ACK with priority index 0 as CSI part 2 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Otherwise,

-	Perform rate matching for HARQ-ACK with priority index 0 according to clause 6.3.2.4.2.2, by taking HARQ-ACK with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 if any as HARQ-ACK, and taking CG-UCI associated with priority index 1 if any as CG-UCI.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.3, by taking CSI part 1 as CSI part 2 and replacing βoffsetPUSCH by βoffsetCSI-part1, taking HARQ-ACK with priority index 0 as CSI-part 1 and taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

If uci-MuxWithDiffPrio is configured, and CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 0:

-	Perform rate matching for HARQ-ACK with priority index 1 according to clause 6.3.2.4.2.1, by taking HARQ-ACK with priority index 1 as HARQ-ACK and replacing βoffsetPUSCH by βoffsetHARQ-ACK-HP.

-	Perform rate matching for CG-UCI associated with priority index 0 according to clause 6.3.2.4.2.2, if CG-UCI associated with priority index 0 is transmitted without HARQ-ACK bits associated with priority index 0, by taking CG-UCI associated with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetCG-UCI, and taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Perform rate matching for CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 according to clause 6.3.2.4.2.2, if both CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 are transmitted, by taking CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 as CSI-part 1 and replacing βoffsetPUSCH by βoffsetHARQ-ACK-LP, and taking HARQ-ACK with priority index 1 as HARQ-ACK.

-	Perform rate matching for CSI part 1 according to clause 6.3.2.4.2.3, by taking CSI part 1 as CSI part 2 and replacing βoffsetPUSCH by βoffsetCSI-part1, taking CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any as CSI-part 1 and taking HARQ-ACK with priority index 1 as HARQ-ACK, if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

< Unchanged parts are omitted >

#### 6.3.2.7	Multiplexing of coded UCI bits with different priority indexes to PUSCH

If the higher layer parameter nrof_UTO_UCI is configured, the procedure in this clause 6.3.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 1,

-	If CSI part 1 is also transmitted on the PUSCH, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 0 as CSI part 2, and assuming the number of HARQ-ACK information in Clause 6.2.7 is 0 bit;

-	Otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 0 as CSI-part 1, and assuming the number of HARQ-ACK information in Clause 6.2.7 is 0 bit.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 1, and CSI if any are transmitted on a PUSCH associated with priority index 0, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK.

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and/or CG-UCI associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH,

-	if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 1, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, and taking HARQ-ACK with priority index 0 as CSI part 2;

-	otherwise, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 if any as HARQ-ACK, taking CG-UCI associated with priority index 1 if any as CG-UCI, taking HARQ-ACK with priority index 0 as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.

If uci-MuxWithDiffPrio is configured, and CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any, HARQ-ACK bits associated with priority index 1, and CSI part 1 if any are transmitted on a PUSCH associated with priority index 0, the coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7 by taking HARQ-ACK with priority index 1 as HARQ-ACK, taking CG-UCI associated with priority index 0 and HARQ-ACK bits associated with priority index 0 if any as CSI part 1, and taking CSI part 1 as CSI part 2 if CSI part 1 is also transmitted on the PUSCH and the PUSCH is associated with priority index 0.