3GPP TSG-RAN4 Meeting #108	R4-2313411

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3589 | rev | - | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR_unlic CR correcting NR-U infra frequency RSRP test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correction of intra frequency test case with CCA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of parameters related to intra-freuqency measurement procedure with CCA. New parameter tables based on configuration 3 of Table A.6.6.1.1.2-2 and Table A.6.6.1.1.2-3, where - some of the configurations were changed to CCA specific configurations - CCA model parameters were included - LCCA -s the minimum among LPSSmax, LindMax, LmasMax = 7 - WCCA is max(T SSB_measurement_period_intra_CCA, TPSS/SSS_sync_intra_CCA)= 600 - Noc, Es/Iot, Et/Iot, RSRP, and Io are kept in brackets for further verification. - D1 delay requirement R4-2307202 was endorsed in RAN4#107, missing test parameters are now included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Intra frequency test cases with CCA for conformace test of NR_unlicensed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2307202 |  |  |  |  |  |  |  |  |

# <Start of change 1>

### A.11.5.1	Intra-frequency measurements

#### A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX

##### A.11.5.1.1.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.11.5.1.1.2	Test parameters

Two cells are deployed in the test, which are PCell (Cell 1) and a neighbour cell (Cell 2) on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model. The test parameters for the two cells are given in Table A.11.5.1.1.2-1 and A.11.5.1.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

-	In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

-	In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

-	In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.11.5.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.5.1.1.2-2: General test parameters for intra-frequency event triggered reporting without gaps

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Active cell |  |  | 1 | Cell 1 |  |
| Neighbour cell |  |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  |  | 1 | 1: Cell 1 and Cell 2 |  |
| DL CCA model |  |  | 1 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1 | As specified in clause A.3.26.2.2 |  |
| SSB configuration | Semi-static channel access Note 1,3 |  | 1 | SSB.1 CCA |  |
|  | Dynamic channel access Note 2,3 |  | 1 | SSB.2 CCA |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| A3-Offset |  | dB | 1 | -4.5 |  |
| CP length |  |  | 1 | Normal |  |
| Hysteresis |  | dB | 1 | 0 |  |
| Time To Trigger |  | s | 1 | 0 |  |
| Filter coefficient |  |  | 1 | 0 | L3 filtering is not used |
| DRX |  |  | 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  |  | 1 | 3 s | Synchronous cells |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 5 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |

Table A.11.5.1.1.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gaps

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1 | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1 | PCCA_DL_1=0.75 |  | PCCA_DL_1=0.75 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1 | PCCA_DL_2=0.75 |  | PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | 1 | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  | 1 | 7 |  | 7 |  |
| WCCA_DL | ms | 1 | 600 |  | 600 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 CCA |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 CCA |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 CCA |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | [-95] |  |  |  |
| Note 2 | dBm/15 kHz | 1 | [-98] |  |  |  |
|  | dB | 1 | [4] | [-1.46] | -Infinity | [-1.46] |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | [4] | [4] | -Infinity | [4] |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | [-91] | [-91] | -Infinity | [-91] |
| Io | dBm/38.16 MHz | 1 | [-58.50] | [-56.16] | [-58.50] | [-56.16] |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.11.5.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than 840 ms from the beginning of time period T2.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX

##### A.11.5.1.2.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.11.5.1.2.2	Test parameters

Two cells are deployed in the test, which are PCell (Cell 1) and a neighbour cell (Cell 2) on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model. The test parameters for the two cells are given in Table A.11.5.1.2.2-1 and A.11.5.1.2.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

Table A.11.5.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.5.1.2.2-2: General test parameters for intra-frequency event triggered reporting without gaps

| Parameter |  | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test |  |
| Active cell |  |  | 1 | Cell 1 |  |  |
| Neighbour cell |  |  | 1 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  |  | 1 | 1: Cell 1 and Cell 2 |  |  |
| DL CCA model |  |  | 1 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  |  | 1 | As specified in clause A.3.26.2.2 |  |  |
| SSB configuration | Semi-static channel access Note 1,3 |  | 1 | SSB.1 CCA |  |  |
|  | Dynamic channel access Note 2,3 |  | 1 | SSB.2 CCA |  |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |  |
| A3-Offset |  | dB | 1 | -4.5 |  |  |
| CP length |  |  | 1 | Normal |  |  |
| Hysteresis |  | dB | 1 | 0 |  |  |
| Time To Trigger |  | s | 1 | 0 |  |  |
| Filter coefficient |  |  | 1 | 0 |  | L3 filtering is not used |
| DRX |  |  | 1 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  |  | 1 | 3 s |  | Synchronous cells |
| T1 |  | s | 1 | 5 |  |  |
| T2 |  | s | 1 | 5 | 15 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |

Table A.11.5.1.2.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gaps

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1 | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1 | PCCA_DL_1=0.75 |  | PCCA_DL_1=0.75 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1 | PCCA_DL_2=0.75 |  | PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | 1 | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  | 1 | Test 1: LCCA_DL = 5 Test 2: LCCA_DL = 3 |  | Test 1: LCCA_DL = 5 Test 2: LCCA_DL = 3 |  |
| WCCA_DL | ms | 1 | Test 1: WCCA_DL = 600 Test 2: WCCA_DL = 5120 |  | Test 1: WCCA_DL = 600 Test 2: WCCA_DL = 5120 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 CCA |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 CCA |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 CCA |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | [-95] |  |  |  |
| Note 2 | dBm/15 kHz | 1 | [-98] |  |  |  |
|  | dB | 1 | [4] | [-1.46] | -Infinity | [-1.46] |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | [4] | [4] | -Infinity | [4] |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | [-91] | [-91] | -Infinity | [-91] |
| Io | dBm/38.16 MHz | 1 | [-58.50] | [-56.16] | [-58.50] | [-56.16] |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.11.5.1.2.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report  with a measurement reporting delay less than 1200 ms from the beginning of time period T2.

In test 2, the UE shall send one Event A3 triggered measurement report  with a measurement reporting delay less than 10240 ms from the beginning of time period T2.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

lNOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# <End of change 1>