3GPP TSG-RAN WG4 Meeting #108	R4-2313067

Toulouse, FR, Aug 21 – Aug 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0642 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_ext_to_71GHz-Core] Editorial modification CR for TS 38.101-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | there is typo in the current specification that can be misleading in understanding what is described in the chapter. They should be corrected to the correct word in the sentence they are intended to describe. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 6.2.2.3-2c MPRWT,BWchannel = 400MHz -> >=800MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect words and sentences that do not match the description can lead to misunderstanding of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1 >>

6.2.2.3	UE maximum output power reduction for power class 3

For power class 3, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

For transmission bandwidth configuration less than or equal to 200MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil((2/3NRB)- LCRB) < RBstart ≤ NRB-LCRB:

-	MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz,

-	MPRnarrow = 2.0 dB, when 1.44 MHz < BWalloc,RB <= 4.32 MHz,

-	otherwise MPRnarrow = 0 dB.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined for FR2-1 in Table 6.2.2.3-1.

Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |

MPRWT is defined for FR2-2 in Table 6.2.2.3-1b.

Table 6.2.2.3-1b MPRWT for power class 3, BWchannel = 100 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 100 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 0.5 |
|  | QPSK | 0.0 | ≤ 0.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 8.5 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 1.5 | ≤ 1.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2.3-1:

-	RBStart,Low = max(1, LCRB), where max() indicates the largest value of all arguments.

-	RBStart,High = NRB – RBStart,Low – LCRB,

An RB allocation belonging to table 6.2.2.3-1 is a Region 1 inner RB allocation if:

-	RBStart,Low  ≤  RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(NRB/3), where ceil(x) is the smallest integer greater than or equal to x.

For transmission bandwidth configuration equal to 400MHz,

MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined for FR2-1 in Table 6.2.2.3-2.

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz, FR2-1

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

MPRWT is defined for FR2-2 in Table 6.2.2.3-2b and 6.2.2.3-2c.

Table 6.2.2.3-2b MPRWT for power class 3, BWchannel = 400 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 | ≤ 3.0 |
|  | QPSK | ≤ 1.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.0 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Table 6.2.2.3-2c MPRWT for power class 3, BWchannel >= 800 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel >= 800 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 | ≤ 4.0 |
|  | QPSK | ≤ 1.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.0 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 8.0 |
|  | 64 QAM | ≤ 10.5 | ≤ 10.5 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2.3-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

An RB allocation belonging to table 6.2.2.3-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

For all transmission bandwidth configurations, an RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation.

<< End of Change 1 >>