3GPP TSG-RAN WG4 Meeting # 108	R4-2313278

Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0411 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_ext_to_71GHz-Perf] CR on 38.101-4: Updates to TDD UL-DL configuration for FR2-2 480kHz SCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the TDD UL-DL configuration for 480kHz SCS, following parameters are wrong: - For TDD Slot Configuration pattern, the denotation of special slot should be divided into S1 slot and S2 slot since there are two special slot configuration for 480kHz SCS - The number of slots between PDSCH and corresponding HARQ-ACK information is TBA |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replaced the denotation “S” by “S1” and “S2” - Added the number of slots between PDSCH and corresponding HARQ-ACK information based on the agreement in R4-221066 shown as follows: Agreement on scheduling and HARQ feedback pattern in R4-2210665: “The scheduling pattern for 480 kHz SCS can be summarized as follows: - One scheduling pattern includes two TDD pattern period which is 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U. - There are 7 DCIs in every scheduling pattern and each DCI schedules 4 PDSCHs if not overlapped with SSB, otherwise, each DCI schedules 3 PDSCHs. PDSCH is scheduled in every DL slots except slot #33 and #34 and slot contain SSB - The HARQ-ACK information are transmitted in slot #19 for DL slots from slot#0 to slot#11 and transmitted in slot #39 for DL slots from slot#12 to slot#39 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The denotation of special slot will still be wrong - The parameter “number of slots between PDSCH and corresponding HARQ-ACK information ” will still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table A.1.3-3: TDD UL-DL configuration for SCS 480 kHz

| Parameter |  | Unit | UL-DL pattern |  |
| --- | --- | --- | --- | --- |
|  |  |  | FR2.480-1 |  |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDDDDDDDDDDDDS1S2UUUU |  |
| Special Slot Configuration (Note 2) |  |  | S1:12D+2G+0U S2: 0D+6G+8U |  |
| referenceSubcarrierSpacing |  | kHz | 480 |  |
| pattern1 | dl-UL-TransmissionPeriodicity | ms | 0.625 |  |
|  | nrofDownlinkSlots |  | 14 |  |
|  | nrofDownlinkSymbols |  | 12 |  |
|  | nrofUplinkSlot |  | 4 |  |
|  | nrofUplinkSymbols |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information(Note 3) |  |  | 19-mod(i,40) if mod(i,40) =0,1,…,11 39-mod(i,40) if mod(i,40) =12,13,14,20,21,…, 31 |  |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,319} |  |  |  |  |