3GPP TSG-RAN WG4 Meeting #108	R4-2313931

# Toulouse, 21st - 25th Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0413 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_HST_FR1_enh-Perf] CR on HST-SFN CA UE capability (TS38.101-4, Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-Perf |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 agreed to introduce UE capability for HST-SFN CA scenario per band combination granularity in WF R4-2207194. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the test applicability for the corresponding HST-SFN CA test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.7.4, 5.2A.2.4, 5.2A.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

##### 5.1.1.7.4	Applicability of different requirements for HST

The applicability rules for different HST requirements in section 5 are specified in Table 5.1.1.7.4-1.

Table 5.1.1.7.4-1: Applicability of requirements for HST

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3), and Clause 5.2A.2.5 (Test 1-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 1-3) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1), and Clause 5.2A.2.5 (Test 1-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3), and Clause 5.2A.3.5 (Test 1-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 1-3), |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1), and Clause 5.2A.3.5 (Test 1-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 1-1), |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-3) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-3) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3) | FR1 2Rx | PDSCH | Clause 5.2.2.1.9 (Test 1-1), Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.9 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-3) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1), Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1) | FR1 2Rx | PDSCH | Clause 5.2.2.1.9 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-1) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2) | FR1 2Rx | PDSCH | Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.9 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-2) | FR1 2Rx | PDSCH | Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3) | FR1 4Rx | PDSCH | Clause 5.2.3.1.9 (Test 1-1), Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.9 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-3) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1), Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1) | FR1 4Rx | PDSCH | Clause 5.2.3.1.9 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-1) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2) | FR1 4Rx | PDSCH | Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.9 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-2) | FR1 4Rx | PDSCH | Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-1) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-2) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-3) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 1-3) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-1) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-3) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 1-3) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 3), and Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 2-3) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 1), and Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2A.2.4 (Test 2), and Clause 5.2A.2.5 (Test 2-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 3), and Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 2-3), |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 1), and Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2A.3.4 (Test 2), and Clause 5.2A.3.5 (Test 2-1), |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-3) | FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-1) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-3) | FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-1) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-3) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1), Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1), Clause 5.2.2.1.10 (Test 1-2), Clause 5.2.2.2.10 (Test 1-2) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-1) | FR1 2Rx | PDSCH | Clause 5.2.2.1.10 (Test 1-1), Clause 5.2.2.1.10 (Test 1-2) |  |
| FR1 2Rx | PDSCH | Clause 5.2A.2.5 (Test 2-2) | FR1 2Rx | PDSCH | Clause 5.2.2.2.1 (Test 1-11), Clause 5.2.2.2.10 (Test 1-1), Clause 5.2.2.2.10 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-3) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1), Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1), Clause 5.2.3.1.10 (Test 1-2), Clause 5.2.3.2.10 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-1) | FR1 4Rx | PDSCH | Clause 5.2.3.1.10 (Test 1-1), Clause 5.2.3.1.10 (Test 1-2) |  |
| FR1 4Rx | PDSCH | Clause 5.2A.3.5 (Test 2-2) | FR1 4Rx | PDSCH | Clause 5.2.3.2.1 (Test 1-11), Clause 5.2.3.2.10 (Test 1-1), Clause 5.2.3.2.10 (Test 1-2) |  |

Table 5.1.1.7.4-2: Applicability of requirements for HST-SFN CA test

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Enhanced demodulation requirements for CA in HST SFN FR1 (demodulationEnhancementCA-r17) | FDD 15 kHz + FDD 15 kHz CA | PDSCH | Clause 5.2A.2.4 Table 5.2A.2.4-3 Clause 5.2A.3.4 Table 5.2A.3.4-3 |  |
|  | TDD 30 kHz + TDD 30 kHz CA | PDSCH | Clause 5.2A.2.4 Table 5.2A.2.4-4 Clause 5.2A.3.4 Table 5.2A.3.4-4 |  |
|  | FDD 15 kHz + TDD 30 kHz CA | PDSCH | Clause 5.2A.2.4 Table 5.2A.2.4-3 and Table 5.2A.2.4-4 per CC Clause 5.2A.3.4 Table 5.2A.3.4-3 and Table 5.2A.3.4-4 per CC |  |

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

#### 5.2A.2.4	Minimum requirements for HST-SFN CA

For HST-SFN CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.2.4-5 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.4-3 and Table 5.2A.2.4-4. Test parameters are specified in Table 5.2A.2.4-2, Table 5.2A-2, and Table 5.2A-3 with downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 5.2A.2.4-1.

Table 5.2A.2.4-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions in the HST-SFN scenario defined in B.3.2 with CA | 1,2,3 |

Table 5.2A.2.4-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | FDD: 10 for CSI-RS resource 1,2,3,4. TDD: 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| Number of PUCCH ResourceGroups |  |  | 1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more than 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A.2.4-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-13.1 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 12.9 |
| 10 | R.PDSCH.1-8.3 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.1 |
| 15 | R.PDSCH.1-13.2 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.4 |
| 20 | R.PDSCH.1-13.3 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.9 |
| 25 | R.PDSCH.1-13.4 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.0 |
| 30 | R.PDSCH.1-13.5 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.9 |
| 35 | R.PDSCH.1-14.3 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.8 |
| 40 | R.PDSCH.1-14.1 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.0 |
| 45 | R.PDSCH.1-14.4 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.9 |
| 50 | R.PDSCH.1-14.2 FDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.0 |

Table 5.2A.2.4-4 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-19.1 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.4 |
| 10 | R.PDSCH.2-19.2 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.7 |
| 15 | R.PDSCH.2-19.3 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.8 |
| 20 | R.PDSCH.2-19.4 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.8 |
| 25 | R.PDSCH.2-19.5 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.1 |
| 30 | R.PDSCH.2-20.1 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.4 |
| 40 | R.PDSCH.2-10.4 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.6 |
| 50 | R.PDSCH.2-20.2 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.7 |
| 60 | R.PDSCH.2-20.3 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.4 |
| 80 | R.PDSCH.2-20.4 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.9 |
| 90 | R.PDSCH.2-20.5 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 15.4 |
| 100 | R.PDSCH.2-21.1 TDD | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 14.8 |

Table 5.2A.2.4-5: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.2.4-3 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.4-4 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.4-3 and Table 5.2A.2.4-4 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. |  |  |

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

#### 5.2A.3.4	Minimum requirements for HST-SFN CA

For HST-SFN CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.4-5 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.4-3 ~ Table 5.2A.3.4-4, with the parameters in Table 5.2A.3.4-2, Table 5.2A-2, Table 5.2A-3, and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 5.2A.3.4-1.

Table 5.2A.3.4-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 with CA | 1, 2, 3 |

Table 5.2A.3.4-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | FDD: 10 for CSI-RS resource 1,2,3,4. TDD: 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| Number of PUCCH ResourceGroups |  |  | 1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more than 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A.3.4-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-13.1 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 10.5 |
| 10 | R.PDSCH.1-8.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 10.7 |
| 15 | R.PDSCH.1-13.2 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.1 |
| 20 | R.PDSCH.1-13.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.5 |
| 25 | R.PDSCH.1-13.4 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.6 |
| 30 | R.PDSCH.1-13.5 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 35 | R.PDSCH.1-14.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.7 |
| 40 | R.PDSCH.1-14.1 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |
| 45 | R.PDSCH.1-14.4 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.7 |
| 50 | R.PDSCH.1-14.2 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |

Table 5.2A.3.4-4: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-19.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.0 |
| 10 | R.PDSCH.2-19.2 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 15 | R.PDSCH.2-19.3 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.1 |
| 20 | R.PDSCH.2-19.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 25 | R.PDSCH.2-19.5 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |
| 30 | R.PDSCH.2-20.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.2 |
| 40 | R.PDSCH.2-10.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.4 |
| 50 | R.PDSCH.2-20.2 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.6 |
| 60 | R.PDSCH.2-20.3 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.5 |
| 80 | R.PDSCH.2-20.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |
| 90 | R.PDSCH.2-20.5 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |
| 100 | R.PDSCH.2-21.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |

Table 5.2A.3.4-5: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.4-3 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.4-4 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.4-3 and Table 5.2A.3.4-4 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in Section 5.1.1.7. |  |  |

<END OF THE CHANGE 3>