3GPP TSG-TSG RAN4 Meeting #108	R4- 2313933

# Toulouse, France, 21th Aug – 25th Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0396 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CR on Rel-17 FR2 HST UE demodulation test setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson, Anritsu, Cybercore |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Perf |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-17 FR2 HST, PDSCH was scheduled in the special slot for TDD.The agreed TDD patten is DDDSU (S= 10D: 2GP:2U) where 10 symbols are allocated for downlink, Based on Rel-17 FR2 HST simulation assumption, 3 DMRS symbols will be configured. 3 DMRS symbols should be available based on duration length of DL symbols in special slot. If 3 DMRS avaible Since 3 DMRS symbols are available for special DL slots, the FRC should be updated to align 3 DMRS configuration Maximum number of SSB configured are 4 and 8 per each RRH for unidirectional and bi-directional scenario, where 2 slots and 4 slots are needed for SSB/PBCH transmission, then, the related CSI-RS should be updated Meanwhile, the number of CSI-RS source number is 8 for bi-directional scenario in each slot, there will be 4RE CSI-RS REs overlapping with PR-RS RE. Then, the corresponding overhead should be counted into the binary Channel Bits Per Slot calculation |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the FRC to reflect the 3DMRS configuraiton, number of SSB and CSI-RS configuration - Correction the value of binary channel bits per slot for Bi-directional scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-17 FR2 HST UE demodualtion requirement can not be verfied well |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.4, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revison of R4-2312215 |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table 7.2.2.2.4-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 1 and 2 |
|  |  |  |  | 6 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 5 and 7 |
|  |  |  |  | l0 = 8 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 5 and 6 |
|  |  |  |  | 6 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 9 and 11 |
|  |  |  |  | l0 = 9 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 9 and 10 |
|  |  |  |  | 6 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #6 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 13 and 15 |
|  |  |  |  | l0 = 8 for CSI-RS resource 14 and 16 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 13 and 14 |
|  |  |  |  | 6 for CSI-RS resource 15 and 16 |
|  |  | QCL info |  | TCI state #7 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 17 and 19 |
|  |  |  |  | l0 = 9 for CSI-RS resource 18 and 20 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 17 and 18 |
|  |  |  |  | 6 for CSI-RS resource 19 and 20 |
|  |  | QCL info |  | TCI state #12 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 21 and 23 |
|  |  |  |  | l0 = 8 for CSI-RS resource 22 and 24 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 21 and 22 |
|  |  |  |  | 6 for CSI-RS resource 23 and 24 |
|  |  | QCL info |  | TCI state #13 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 25 and 27 |
|  |  |  |  | l0 = 9 for CSI-RS resource 26 and 28 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 25 and 26 |
|  |  |  |  | 6 for CSI-RS resource 27 and 28 |
|  |  | QCL info |  | TCI state #14 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 29 and 31 |
|  |  |  |  | l0 = 8 for CSI-RS resource 30 and 32 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 29 and 30 |
|  |  |  |  | 6 for CSI-RS resource 31 and 32 |
|  |  | QCL info |  | TCI state #15 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #17 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #18 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #19 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #20 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #21 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #22 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #23 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #24 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #8 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #9 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #10 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #11 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type D |
| TCI state #6 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type D |
| TCI state #7 | Type 1 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type D |
| TCI state #12 (Note2) | Type 1 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type D |
| TCI state #13 (Note2) | Type 1 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type D |
| TCI state #14 (Note2) | Type 1 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type D |
| TCI state #15 (Note2) | Type 1 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |
| Note 1: For Test 1-1, SSB # (2k mod 8) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH; SSB # ((2k mod 8)+1) , CSI-RS (for tracking) resource set # ((k mod 4) + 13), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 17) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 21) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0 (i≠0). PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=12k-2n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=2,3,4… to slot# 2k-1n+THARQ+TMAC proc,k=1,2,3…, PDCCH and PDSCH associated with TCI # ((k mod 4)+8) is transmitted by kth RRH from slot# 2k+1n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=0,1,2… to slot# [2k+1n+THARQ+TMAC proc],k=0,1,2…, where k is the RRH number, n = 28800 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing, TfirstSSB = 132 is the number of slots to first SSB transmission occasion after MAC CE command is decoded by the UE, TSSB proc= 16 is the number of slots for SSB processing, TfirstTRSafterSSB = 66 is the number of slots to first TRS transmission occasion after first SSB is processed by the UE, TTRSproc= 16 is the number of slots for TRS processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, SSB # (k mod 4) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0. PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=1k-1n+2+THARQ+TMAC proc,k=2,3,4… to slot# [k * n+THARQ+TMAC proc], k=1,2,3…, where k is the RRH number, n = 57600 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Note 2: Only configured for Test 1-1. |  |  |  |  |

<End of Change 1>

<Start of Change 2>

Table A.3.2.2.5-12: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and HST-DPS scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD | R.PDSCH.5-12.2 TDD |  |  |  |
| Channel bandwidth | MHz | 200 | 200 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 132 | 132 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 |  |  |  |
| For Slot i=1 |  | N/A (Note 1) | N/A(Note 4) |  |  |  |
| For Slot i=2 |  | 13 | N/A (Note 4) |  |  |  |
| For Slot i=3 |  | 9 | N/A (Note 4) |  |  |  |
| Allocated slots per 2 frames |  | 126 | 124 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 17 | 17 |  |  |  |
| Modulation |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.43 | 0.43 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 |  |  |  |
| For Slot i = 1 |  | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 |  | 18 | N/A (Note 4) |  |  |  |
| For Slot i=3 |  | 18 | N/A (Note 4) |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | 57376 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | 90176 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 | Bits | 90176 | N/A (Note 4) |  |  |  |
| For Slot i=3 | Bits | 57376 | N/A (Note 4) |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i = 2 | Bits | 24 | N/A (Note 4) |  |  |  |
| For Slot i = 3 | Bits | 24 | N/A (Note 4) |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 7 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | 11 |  |  |  |
| For Slot i = 1 | CBs | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 | CBs | 11 | N/A (Note 4) |  |  |  |
| For Slot i=3 | CBs | 7 | N/A (Note 4) |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 195696 | 181968 |  |  |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 195696 | 181968 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {8,…,84, 87, …,159} | Bits | 137808 | 137808 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | 210672 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i= 2 | Bits | 210672 | N/A (Note 4) |  |  |  |
| For Slot i= 3 | Bits | 137808 | N/A (Note 4) |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 515.62 | 508.25 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 and slot #1 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #0, slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |  |

<End of Change 2>