# 3GPP TSG-RAN WG4 Meeting #108	R4-2312089

# Toulouse, France, 21 – 25 August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3452 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_pos_enh-Core]: The correction of IE for positioning core requirements. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS38.133, the IE is not accuracy according to the TS37.355: TS37.355: NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE { nr-DL-PRS-AssistanceData-r16 NR-DL-PRS-AssistanceData-r16 OPTIONAL, -- Need ON OPTIONAL, -- Cond UEB NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE { nr-DL-PRS-RstdMeasurementInfoRequest-r16 ENUMERATED { true } OPTIONAL,-- Need ON supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17 ENUMERATED { supported } OPTIONAL, reducedDL-PRS-ProcessingSamples-r17 ENUMERATED { requested, ... } OPTIONAL, -- Need ON RAN4 shall follow the TS37.355 and correct the IE for positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the existing IE for positioning according to TS37.355. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IE is inaccuracy and can not find the IE in TS37.355. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.5, 5.6.3.5, 9.9.2.5, 9.9.2.7, 9.9.3.5, 9.9.3.6, 9.9.4.4, 9.9.4.5, 9.9.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.355 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE 1>

5.6.2.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L−1∗maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i= Kcarrier_PRS∗ NRx,TEG,i∗NRxBeam,i∗NPRS,islotN'∗Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-		NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation .

-		NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,i is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Nsample is the number of PRS RSTD samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i∗Tavailable_PRS,i

Where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX

-	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS witℎ muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

-	TperPRS witℎ muting=Nmuting∗TperPRS, is the PRS periodicity with muting per PRS resource,

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where

-	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	{N,T} is the UE capability combination per band for RRC_INACTIVE state where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34],  T (ms) corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], [ and T-N (>0) is the time required to process duration N of DL PRS symbols already buffered in memory], for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot [in RRC_INACTIVE state as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 specified in TS 37.355 [34].

The time TRSTD,Total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.

< END OF CHANGE 1>

< START OF CHANGE 2>

5.6.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS−RSRP,total ms.

TPRS−RSRP,i=Kcarrier_PRSi∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS−RSRP,i=Kcarrier_PRSi∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast

Where:

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRxBeam,i is the scaling factor for Rx beam sweeping:

- NRxBeam,i=1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-		NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-AoD-RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

-	Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of PRS-RSRP measurement samples and

-	Nsample= 1, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

-	Teffect,i= TiTavailable_PRS,i∗Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX is the least common multiple between TPRS,i and TDRX,

-	TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

-	TDRX is the DRX cycle length.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS witℎ muting=Nmuting∗TperPRS, the least common multiple of  TperPRS witℎ muting among the DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS−RSRP,total starts from the first DRX cycle containing the DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 5.6.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 5.6.4.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state.

The requirements in clause 5.6.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If the DRX cycle is reconfigured during the PRS-RSRP measurement period then the PRS-RSRP measurement period can be longer.

If cell reselection occurs while PRS-RSRPP measurement is being performed, then the UE shall continue and complete the on-going PRS-RSRP measurement after the cell selection is completed. The PRS-RSRP measurement period can be longer.

If the UE’s RRC state changes from the RRC_INACTIVE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

The UE shall meet the PRS-RSRP measurement accuracy requirements in clause 10.1.24.

< END OF CHANGE 2>

< START OF CHANGE 3>

9.9.2.5	Measurements Period Requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L−1∗maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=kmultiTEG,i∗CSSFPRS,i∗ceil( Kp,PRS,i)∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1;

and in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 inNR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1.5.2.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the

sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, Tlast,i = Ti +MGL. Otherwise, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i∗Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS witℎ muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS witℎ muting=Nmuting∗TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and  NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the RSTD measurement in each reporting period and activate the location report at the time when the periodic timer expires.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured, the measurement period can be longer. When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.

< END OF CHANGE 3>

< START OF CHANGE 4>

9.9.2.7	Measurements Period Requirements without Measurement Gaps

When physical layer receives last of NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD_wo_gap,i+L−1∗maxTeffect,i, if any of the positioning frequency layers are in Case 1, or

TRSTD,Total=maxTRSTD_wo_gap,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2,

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TRSTD,Total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TRSTD_wo_gap,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below.

TRSTD_wo_gap,i=kmultiTEG,i∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2,NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not supported measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i,

only the PRS resources unmuted and fully or partially overlapped with PPW are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then  Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i∗Tavailable_PRS,i

Where,

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

Tavailable_PRS,i= LCMTPRS,i,PPWRPi, the least common multiple between TPRS,i and PPWRPi.

PPWRPi is the repetition periodicity of the PRS processing window applicable for measurements in the positioning frequency layer i.

TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS witℎ muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS witℎ muting=Nmuting∗TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity,

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the conditions for PRS measurement outside measurement gaps as defined in clause 9.9.1.2 are considered.

N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2.

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

The time TRSTD_wo_gap,i starts from the first instance of the activated PPW for measurement of positioning frequency layer i aligned with a DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformationmessage are delivered from LMF to the physical layer of UE via LPP [34].

If during the measurement period of one or more positioning frequency layers, the PPW is re-configured or reactivated, the measurement period can be longer. When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD_wo_gap,i within PPW due to collisions with other signals; otherwise, the measurement period can be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.

If any positioning frequency layer is in Case 2, the requirements in this clause apply provided that the PPWL corresponding to the positioning frequency layer is larger than (T2+X) ms.

The requirements in this clause apply provided that a single positioning frequency layer is configured for measurement in each PPW.

< END OF CHANGE 4>

< START OF CHANGE 5>

9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS−RSRP,total ms.

TPRS−RSRP, total=i=1LTPRS−RSRP,i+L−1∗maxTeffect,i

where

# i is the index of positioning frequency layer,

L is total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS−RSRP,i=CSSFi∗ceil( Kp,PRS,i)∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioining frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and if positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time, if not all PRS resources to be measured are available in the same measurement gap occasion during Tavailable_PRS,i, otherwise Tlast,i = Ti + MGL,

Teffect,i= TiTavailable_PRS,i∗Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

MGRPi is the measurement gap repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS witℎ muting=Nmuting∗TperPRS, the least common multiple of  TperPRS witℎ muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

When PRS-RSRP measurements are configured for DL-AoD, except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TPRS−RSRP,total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TPRS−RSRP,total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the PRS-RSRP measurement in each reporting period and activate the location report at the time when the periodic timer expires.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9.4.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource:

- if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

- if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TPRS−RSRP, total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

The requirements in clause 9.9.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The PRS-RSRP measurement period can be longer. The UE shall meet the PRS-RSRP measurement accuracy requirements in clause 10.1.24.

< END OF CHANGE 5>

< START OF CHANGE 6>

9.9.3.6	Measurement Period Requirements without Measurement Gaps

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements as defined in TS 38.215 [4] without measurement gap, on configured positioning frequency layer i, within TPRS−RSRP,total ms.

TPRS−RSRP, total=i=1LTPRS−RSRP,i+L−1∗maxTeffect,i if any of the positioning frequency layers are in Case 1, or

TPRS−RSRP, total=maxTPRS−RSRP,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2,

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TPRS−RSRP, total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TPRS−RSRP,i is the measurement period for PRS-RSRP measurement in positioning frequency layer i as specified below.

TPRS−RSRP,i=NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast

where

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1. If positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i,

only unmuted PRS resource instances that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast  is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

Teffect,i= TiTavailable_PRS,i∗Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2

Tavailable_PRS,i= LCMTPRS,i,PPWRPi is the least common multiple between TPRS,i and PPWRPi,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

PPWRPi is the PRS processing window repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS witℎ muting=Nmuting∗TperPRS, the least common multiple of  TperPRS witℎ muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS−RSRP,i starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9.4.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource:

if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If during the measurement period of one or more positioning frequency layers, the PPW is reconfigured or reactivated, the PRS-RSRP measurement period can be longer.

If any positioning frequency layer is in Case 2, the requirements in this clause apply provided that the PPWL corresponding to the positioning frequency layer is larger than (T2+X) ms.

The requirements in this clause apply provided that a single positioning frequency layer is configured for measurement in each PPW.

< END OF CHANGE 6>

< START OF CHANGE 7>

9.9.4.4 Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.25.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25.

< END OF CHANGE 7>

< START OF CHANGE 8>

9.9.4.5	Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L−1∗maxTeffect,i.

where i is the index of positioning frequency layer,

TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in positioning frequency layer i as further defined in this clause,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

TUERxTx,i=kmultiTEG,i∗CSSFi∗ceil( Kp,PRS,i)∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i

Where

CSSFi is the carrier-specific scaling factor for NR PRS-based measurement in the positioning frequency layer i as defined in clause 9.1.5.2,

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs or RxTx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and  if positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in  NR-Multi-RTT-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3  of TS 37.355 [34],

Nsample is the number of UE Rx-Tx time difference measurement samples:

-	Nsample= 4 if the UE is not capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34].

-	Nsample= 1 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] and the following conditions are met:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=  2 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and the LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] but the following conditions are not met:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailable_PRS,i∗Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

MGRPi is the measurement gap repetition periodicity in positioning frequency layer i.

TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS witℎ muting=Nmuting∗TperPRS, the least common multiple of TperPRS witℎ muting among DL PRS resource sets is used to derive TPRS,i, where

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1

Note:	For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TUERxTx,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TUERxTx,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the PRS-RSRP measurement in each reporting period and activate the location report at the time when the periodic timer expires.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TUERxTx,Total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

< END OF CHANGE 8>

< START OF CHANGE 9>

9.9.4.6	Measurement Period Requirements without Measurement Gaps

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx_wo_gap,Total ms.

TUERxTx_wo_gap, Total=i=1LTUERxTx_wo_gap,i+L−1∗maxTeffect,i, if any of the positioning frequency layers are in Case 1, or

TUERxTx_wo_gap, Total=maxTUERxTx_wo_gap,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2.

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TUERxTx_wo_gap, Total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TUERxTx_wo_gap,i is the measurement period for UE Rx-Tx time difference measurement in positioning frequency layer i as specified below.

TUERxTx_wo_gap,i=kmultiTEG,i∗NRxBeam,i∗NPRS,islotN'Lavailable_PRS,iN∗Nsample−1∗Teffect,i+Tlast,i

Where

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not supported measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs or RxTx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1.  If positioning frequency layer i is in FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in  NR-Multi-RTT-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].  For calculation of Lavailable_PRS,i,

only the unmuted PRS resources that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

-	N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to ppw-maxNumOfDL-PRS-ResProcessedPerSlot as specified in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of UE Rx-Tx time difference measurement samples:

-	Nsample= 4 if the UE is not capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34].

-	Nsample= 1 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] and the following conditions are met:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=  2 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-CONNECTED defined in [34] and the LMF requests the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34] but the following conditions are not met:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then  Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

-	Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailablePRS,i∗TavailablePRS,i

where

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

Tavailable_PRS,i= LCMTPRS,i,PPWRPi, the least common multiple between TPRS,i and PPWRPi

PPWRPi is the PRS processing window repetition periodicity in positioning frequency layer i.

TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS witℎ muting=Nmuting∗TperPRS, the least common multiple of TperPRS witℎ muting among DL PRS resource sets is used to derive TPRS,i, where

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS∗Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

The time TUERxTx_wo_gap,i starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the PRS processing window is reconfigured or reactivated either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TUERxTx_wo_gap,i within PRS processing window due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.4.6 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

If any positioning frequency layer is in Case 2, the requirements in this clause apply provided that the PPWL corresponding to the positioning frequency layer is larger than (T2+X) ms.

The requirements in this clause apply provided that a single positioning frequency layer is configured for measurement in each PPW.

< END OF CHANGE 9>