3GPP TSG-RAN4 Meeting #108	R4-2314416

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | 0025 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for NavIC L5 A-GNSS support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Reliance Jio, CEWiT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel17 Positioning item led by RAN2 included objective - Specify support for NavIC to NR. However, the respective updates to NR Requirements for support of A-GNSS specification was missed. This CR includes the respective updates to the given specification based on respective LTE specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. New references, symbols, abbreviations items for NavIC. 2. Table 4.1: relative signal power level for two signal type for NavIC L5. 3. Test parameters on reference signal power level for NavIC in section 6 to evaluate minimum performance requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No minimum requirements support of NavIC L5 in NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 3.3, 4.8, 6.2, B.1.4, B.1.5.2, E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2312648 |  |  |  |  |  |  |  |  |

---------------------------- START OF CHANGE ----------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

·	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

·	For a specific reference, subsequent revisions do not apply.

·	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity", Stage 2.

[3]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[4]	3GPP TS 38.215: "NR; Physical layer; Measurements".

[5]	ETSI TR 102 273-1-2: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[6]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[7]	P. Axelrad, R.G. Brown, "GPS Navigation Algorithms", in Chapter 9 of "Global Positioning System: Theory and Applications", Volume 1, B.W. Parkinson, J.J. Spilker (Ed.), Am. Inst. of Aeronautics and Astronautics Inc., 1996.

[8]	S.K. Gupta, "Test and Evaluation Procedures for the GPS User Equipment", ION-GPS Red Book, Volume 1, p. 119.

[9]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[10]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[11]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[12]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[13]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[14]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[15]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[16]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", China Satellite Navigation Office, February 2019.

[17]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[19]	3GPP TS 37.571-1: " User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance".

[20]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[21]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[22]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[23]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[24]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply:

EN-DC: E-UTRA-NR Dual Connectivity as defined in TS 37.340 [2].

en-gNB: as defined in TS 37.340 [2].

gNB: as defined in in TS 38.300 [17].

Horizontal Dilution Of Precision (HDOP): measure of position determination accuracy that is a function of the geometrical layout of the satellites used for the fix, relative to the receiver antenna

NE-DC: NR-E-UTRA Dual Connectivity as defined in TS 37.340 [2].

ng-eNB: as defined in TS 38.300 [17].

NR-DC: NR-NR Dual Connectivity as defined in TS 37.340 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

B1I	BeiDou B1I navigation signal with carrier frequency of 1561.098 MHz.

B1C	BeiDou B1C navigation signal with carrier frequency of 1575.420 MHz.

B2a	BeiDou B2a navigation signal with carrier frequency of 1176.450 MHz.

B3I	BeiDou B3I navigation signal with carrier frequency of 1268.520 MHz.

E1	Galileo E1 navigation signal with carrier frequency of 1575.420 MHz.

E5	Galileo E5 navigation signal with carrier frequency of 1191.795 MHz.

E6	Galileo E6 navigation signal with carrier frequency of 1278.750 MHz.

G1	GLONASS navigation signal in the L1 sub-bands with carrier frequencies 1602 MHz ± k  562.5 kHz.

G2	GLONASS navigation signal in the L2 sub-bands with carrier frequencies 1246 MHz ± k  437.5 kHz.

k	GLONASS channel number, k = -7…13.

L1 C/A	GPS or QZSS L1 navigation signal carrying the Coarse/Acquisition code with carrier frequency of 1575.420 MHz.

L1C	GPS or QZSS L1 Civil navigation signal with carrier frequency of 1575.420 MHz.

L2C	GPS or QZSS L2 Civil navigation signal with carrier frequency of 1227.600 MHz.

L5	GPS or QZSS or NavIC L5 navigation signal with carrier frequency of 1176.450 MHz.

G	Geometry Matrix.

Measured pseudo-range of satellite i of GNSSm.

W	Weighting Matrix.

Line of sight unit vector from the user to the satellite i of GNSSm.

State vector of user position and clock bias.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

5GS	5G System

A-GNSS	Assisted Global Navigation Satellite System

A-GPS	Assisted - Global Positioning System

AWGN	Additive White Gaussian Noise

BDS	BeiDou Navigation Satellite System

C/A	Coarse/Acquisition

DC	Dual Connectivity

DUT	Device Under Test

ECEF	Earth Centred, Earth Fixed

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

GEO	Geostationary Earth Orbit

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

HDOP	Horizontal Dilution Of Precision

ICD	Interface Control Document

IGSO	Inclined Geosynchronous Satellite Orbit

IRNSS	Indian Regional Navigation Satellite System

IS	Interface Specification

LMF	Location Management Function

LOS	Line Of Sight

LPP	LTE Positioning Protocol

MEO	Medium Earth Orbit

NavIC	NAVigation with Indian Constellation

NE-DC	NR-E-UTRA Dual Connectivity

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

QZSS	Quasi-Zenith Satellite System

RRC	Radio Resource Control

SBAS	Space Based Augmentation System

SFN	System Frame Number

SS	System Simulator

SV	Space Vehicle

TDD	Time Division Duplex

TOW	Time Of Week

TTFF	Time To First Fix

UE	User Equipment

WLS	Weighted Least Square

WGS-84	World Geodetic System 1984

----------------------------  THE NEXT CHANGE  ----------------------

## 4.8	UEs supporting multiple signals

For UEs supporting multiple signals, different minimum performance requirements may be associated with different signals. The satellite simulator shall generate all signals supported by the UE. Signals not supported by the UE do not need to be simulated. The relative power levels of each signal type for each GNSS are defined in Table 4.1. The individual test scenarios in clause 6 define the reference signal power level for each satellite. The power level of each simulated satellite signal type shall be set to the reference signal power level defined in each test scenario in clause 6 plus the relative power level defined in Table 4.1.

Table 4.1: Relative signal power levels for each signal type for each GNSS

|  | BDS |  |  | Galileo |  | GLONASS |  | GPS/Modernized GPS |  | QZSS |  | SBAS |  | NavIC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal power levels relative to reference power levels | B1I | D1 | 0 dB | E1 | 0 dB | G1 | 0 dB | L1 C/A | 0 dB | L1 C/A | 0 dB | L1 | 0 dB | L5 | 0 dB |
|  |  | D2 | +5 dB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | B1C | DMEO | +4 dB | E6 | +2 dB | G2 | -6 dB | L1C | +1.5 dB | L1C | +1.5 dB |  |  |  |  |
|  |  | DIGSO | +2dB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | B2a | DMEO | +7 dB | E5 | +2 dB |  |  | L2C | -1.5 dB | L2C | -1.5 dB |  |  |  |  |
|  |  | DIGSO | +5dB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | B3I | D1 | 0 dB |  |  |  |  | L5 | +3.6 dB | L5 | +3.6 dB |  |  |  |  |
|  |  | D2 | +5 dB |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE 1:	For test cases which involve “Modernized GPS”, the satellite simulator shall also generate the GPS L1 C/A signal if the UE supports “GPS” in addition to “Modernized GPS”.

NOTE 2:	The signal power levels in the Test Parameter Tables represent the total signal power of the satellite per channel not e.g. pilot and data channels separately.

NOTE 3:	For test cases which involve "BDS", D1 represents MEO/IGSO satellites for B1I and B3I signal types and D2 represents GEO satellites for B1I and B3I signal type.

NOTE 4:	For test cases which involve "BDS", DMEO represents MEO satellites for B1C and B2a signal types and DIGSO represents IGSO satellites for B1C and B2a signal type.

NOTE 5:	For test cases involving NavIC all satellites are either GEO or IGSO as per ICD [24]

----------------------------  THE NEXT CHANGE  ----------------------

## 6.2	Nominal accuracy

Nominal accuracy requirement verifies the accuracy of A-GNSS position estimate in ideal conditions. The primarily aim of the test is to ensure good accuracy for a position estimate when satellite signal conditions allow it. This test case verifies the performance of the first position estimate.

In this requirement 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. If SBAS is to be tested one additional satellite shall be generated. AWGN channel model is used. The number of simulated satellites for each constellation is as defined in Table 6.8.

Table 6.7: Test parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.8 |
|  | Total number of generated satellites | - | 6 or 7(2) or 8(3) |
|  | HDOP Range | - | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| GPS(1) | Reference signal power level for all satellites | dBm | -128.5 |
| QZSS | Reference signal power level for all satellites | dBm | -128.5 |
| SBAS | Reference signal power level for all satellites | dBm | -131 |
| NavIC | Reference signal power level for all satellites | dBm | -129 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply for case of single or dual constellation and SBAS or NavIC is supported or case of triple constellation and SBAS or NavIC is not supported. NOTE 3: 8 satellites apply only for case of triple constellation and SBAS is supported. |  |  |  |

If QZSS is supported, one of the GPS satellites will be replaced by a QZSS satellite with respective signal support. If SBAS is supported, the SBAS satellite with the highest elevation will be added to the scenario.  If NavIC is supported, the NavIC satellite with the highest elevation will be added to the scenario.

Table 6.8: Satellite allocation

|  | Satellite allocation for each constellation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GNSS 1(1) | GNSS 2(1) | GNSS 3(1) | SBAS | NavIC |
| Single constellation | 6 | -- | -- | 1 | 1 |
| Dual constellation | 3 | 3 | -- | 1 | 1 |
| Triple constellation | 3 | 2 | 2 | 1 | 1 |
| NOTE 1: GNSS refers to global systems i.e. BDS, Galileo, GLONASS, GPS. |  |  |  |  |  |

### 6.2.1	Minimum requirements (nominal accuracy)

The position estimates shall meet the accuracy and response time requirements in Table 6.9.

Table 6.9: Minimum requirements

| System | Success rate | 2-D position error | Max response time |
| --- | --- | --- | --- |
| All | 95 % | 15 m | 20 s |

----------------------------  THE NEXT CHANGE  ----------------------

## B.1.4	Reference and UE locations

There is no limitation on the selection of the reference location, consistent with achieving the required HDOP for the Test Case. For each test instance the reference location shall change sufficiently such that the UE shall have to use the new assistance data. The uncertainty of the semi-major axis is 3 km. The uncertainty of the semi-minor axis is 3 km. The orientation of major axis is 0 degrees. The uncertainty of the altitude information is 500 m. The confidence factor is 68 %.

For every Test Instance in each TTFF test case, the UE location shall be randomly selected to be within 3 km of the Reference Location. The Altitude of the UE shall be randomly selected between 0 m to 500 m above WGS-84 reference ellipsoid. These values shall have uniform random distributions.

For test cases which include satellites from regional systems, such as QZSS, NavIC and SBAS, the reference location shall be selected within the defined coverage area of the systems.

## B.1.5	Satellite constellation and assistance data

### B.1.5.1	UE supports A-GPS L1 C/A only

In the case of test cases in clause 5 (UE supports A-GPS L1 C/A only), the GPS satellite constellation shall consist of 24 satellites. Almanac assistance data shall be available for all these 24 satellites. At least 9 of the satellites shall be visible to the UE (that is above 5 degrees elevation with respect to the UE). Other assistance data shall be available for 9 of these visible satellites. In each test, signals are generated for only a sub-set of these satellites for which other assistance data is available. The number of satellites in this sub-set is specified in the test. The satellites in this sub-set shall all be above 15 degrees elevation with respect to the UE. The HDOP for the test shall be calculated using this sub-set of satellites. The selection of satellites for this sub-set shall be selected consistent with achieving the required HDOP for the test.

### B.1.5.2	UE supports other A-GNSSs

In the case of test cases in clause 6 (UE supports other GNSSs), the satellite constellation shall consist of 35 satellites for BDS (5 GEO, 27 MEO, 3 IGSO); 27 satellites for Galileo; 24 satellites for GLONASS; 27 satellites for GPS/Modernized GPS; 3 satellites for QZSS; 2 satellites for SBAS and 7 satellites for NavIC (3GEO, 4 IGSO). Almanac assistance data shall be available for all these satellites. At least 7 of the satellites per BDS, Galileo, GLONASS, GPS/Modernized GPS constellation shall be visible to the UE (that is, above 15 degrees elevation with respect to the UE). At least 1 of the satellites for QZSS shall be within 15 degrees of zenith; at least 1 of the satellites of NavIC shall be visible to the UE above 15 degrees elevation angle over NavIC service area. (Service area as defined in ICD [24]) and at least 1 of the satellites for SBAS shall be visible to the UE. For BDS with reference location in Asia, at least 1 of the visible satellites shall be a GEO (above 15 degrees elevation with respect to the UE). All other satellite specific assistance data shall be available for all visible satellites. In each test, signals are generated for only 6 satellites for single constellation and dual constellation and 7 satellites for triple constellation (or one additional satellite if SBAS is included). The HDOP for the test shall be calculated using these satellites. The simulated satellites for BDS, Galileo, GLONASS GPS/Modernized GPS shall be selected from the visible satellites for each constellation consistent with achieving the required HDOP for the test. For BDS with reference location in Asia, 1 of the simulated satellites shall be a GEO. For NavIC with reference location in the NavIC service area as defined in [24], the simulated satellite shall be an IGSO.

----------------------------  THE NEXT CHANGE  ----------------------

# E.2	GNSS assistance data

a)	GNSS-ReferenceTime IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.2: GNSS-ReferenceTime IE

| Information Element | All tests except Sensitivity Fine Time Assistance | Sensitivity Fine Time Assistance test |
| --- | --- | --- |
| GNSS-ReferenceTime |  |  |
| > gnss-SystemTime |  |  |
| >> gnss-TimeID | Yes | Yes |
| >> gnss-DayNumber | Yes | Yes |
| >> gnss-TimeOfDay | Yes | Yes |
| >> gnss-TimeOfDayFrac-msec | Yes | Yes |
| >> notificationOfLeapSecond | Yes if gnss-TimeID = ‘glonass’ | Yes if gnss-TimeID = ‘glonass’ |
| >> gps-TOW-Assist | Yes if gnss-TimeID = ‘gps’ | Yes if gnss-TimeID = ‘gps’ |
| > referenceTimeUnc | Yes | No |
| > gnss-ReferenceTimeForOneCell | No | Yes |
| >> networkTime |  | Yes |
| >>> secondsFromFrameStructureStart |  | Yes |
| >>> fractionalSecondsFromFrameStructureStart |  | Yes |
| >>> frameDrift |  | Yes |
| >>> cellID |  | Yes |
| >>> CHOICE eUTRA |  |  |
| >>>> physCellId |  | Yes |
| >>>> cellGlobalIdEUTRA |  | Yes |
| >>>> earfcn/earfcn-v9a0 |  | Yes |
| >>> CHOICE nr-r15 |  |  |
| >>>> nrPhysCellId-r15 |  | Yes |
| >>>> nrCellGlobalID-r15 |  | Yes |
| >>>> nrARFCN-r15 |  | Yes |
| >> referenceTimeUnc |  | Yes |

b)	GNSS-ReferenceLocation IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.3: GNSS-ReferenceLocation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-ReferenceLocation | threeDlocation |

c)	GNSS-IonosphericModel IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.4: GNSS-IonosphericModel IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-IonosphericModel | KlobucharModelParameter |
|  | NeQuickModelParameter(1) |
|  | KlobucharModel2Parameter(2) |
| NOTE 1: Only required if GNSSs supported include Galileo. NOTE 2: Only required if GNSSs supported include BDS B1C/B2a. |  |

d)	GNSS-TimeModelList IE. This information element is only required for multi system tests, and is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.5: GNSS-TimeModelList IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-TimeModelList |  |
|  | gnssTOID For each GNSS included in the test. |
|  | deltaT |

e)	GNSS-NavigationModel IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.6: GNSS-NavigationModel IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-NavigationModel |  |

Table E.7: GNSS Clock and Orbit Model choices

| GNSS | Clock and Orbit Model choice |
| --- | --- |
| BDS B1I/B3I | Model-6 |
| BDS B1C/B2a | Model-7 |
| Galileo | Model-1 |
| GLONASS | Model-4 |
| GPS L1 C/A | Model-2 |
| Modernized GPS | Model-3 |
| QZSS QZS-L1 C/A | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3 |
| SBAS | Model-5 |
| NavIC | Model-8 |

f)	GNSS-AcquisitionAssistance IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.8: GNSS-AcquisitionAssistance IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AcquisitionAssistance |  |

g)	GNSS-Almanac IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.9: GNSS-Almanac IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-Almanac |  |

Table E.10: GNSS Almanac choices

| GNSS | Almanac Model choice |
| --- | --- |
| BDS B1I/B3I | Model-7 |
| BDS B1C/B2a | Model-3, 4 |
| Galileo | Model-1 |
| GLONASS | Model-5 |
| GPS L1 C/A | Model-2 |
| Modernized GPS | Model-3, 4 |
| QZSS QZS-L1 C/A | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3, 4 |
| SBAS | Model-6 |
| NavIC | Model-8 |

h)	GNSS-UTC-Model IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.11: GNSS-UTC-Model IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-UTC-Model |  |

Table E.12: GNSS UTC Model choices

| GNSS | UTC Model choice |
| --- | --- |
| BDS B1I/B3I | Model-5 |
| BDS B1C/B2a | Model-2 |
| Galileo | Model-1 |
| GLONASS | Model-3 |
| GPS L1 C/A | Model-1 |
| Modernized GPS | Model-2 |
| QZSS QZS-L1 C/A | Model-1 |
| QZSS QZS-L1C/L2C/L5 | Model-2 |
| SBAS | Model-4 |
| NavIC | Model-2 |

i)	GNSS-AuxiliaryInformation IE. This information element is defined in clause 6.5.2.2 of TS 36.355 [3].

Table E.13: GNSS-AuxiliaryInformation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AuxiliaryInformation |  |

--------------------------- END OF CHANGE ----------------------