3GPP TSG-RAN WG4 Meeting # 108		R4-2314468

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7249 | rev | 2 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_unlic-Core] CR on inter-RAT measurement on NR-U with eDRX in IDLE mode R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-07-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4 #107 meeting, following proposals have been agreed in chairman notes: Topic #11: Rel-17 TEI Sub-topic 11-1: inter-RAT NR-U measurement with LTE eDRX Agreements For a LTE UE to perform inter-RAT NR cell measurement with CCA and with eDRX≥10.24s, the following criteria shall be considered when design the PTW window in the requirement: - The extension shall not exceed the upper limit threshold (Mm,max, Md,max, or Me,max), and - The extension of measurement/evaluation period due to CCA shall be limited in the single PTW window Agreements For a LTE UE to perform inter-RAT NR cell measurement with CCA and with eDRX≥10.24s, the lower bound of PTW window shall be derived based on (Tevaluate,NR+Me,max)*DRX_cycle1.28*1.28. Note: Tevaluate,NR is the legacy inter-RAT NR cell evaluation period without any extension due to CCA, as same as in Table 4.2.2.5.6-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To reflect the above agreements into the TS36.133 requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-RAT measurement requirement on NR-U with eDRX in IDLE mode is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

##### 4.2.2.5.7	Measurements of NR cells subject to CCA

The UE shall be able to identify new inter-RAT cells subject to CCA and perform SS-RSRP or SS-RSRQ measurements of identified inter-RAT cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT layers of higher priority at least every Thigher_priority_search.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this subclause.

The UE shall be able to evaluate whether a newly detectable inter-RAT cell meets the reselection criteria defined in TS38.304 within Kcarrier * Tdetect,NR_Inter + Kcarrier_CCA * Tdetect,NR_Inter_CCA  if at least carrier frequency information is provided for inter-RAT neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB in FR1 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The parameter Kcarrier is the number of NR inter-RAT carriers on licensed band and Kcarrier_CCA is the number of NR inter-RAT carriers on unlicensed band indicated by the serving cell. An inter-RAT cell is considered to be detectable according to the conditions defined in Annex B.x.y for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_CCA. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter + Kcarrier_CCA * Tmeasure,NR_Inter_CCA (see table 4.2.2.5.7-1) for identified lower priority inter-RAT cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

For a cell that is already identified, after 2 unsuccessful measurement attempts due to exceeding the maximum number of SMTC occasions not available at the UE, the UE shall detect cells on any of the configured serving- and/or non-serving carriers.

In the requirements of clause 4.2.2.5.7, the term SMTC occasion not available at the UE refers to when the SMTC contains SSBs configured by gNB in a cell on a carrier frequency subject to CCA, but the N candidate SSB positions for the same SSB index within the discovery burst transmission window are not available at the UE due to DL CCA failures at gNB during the corresponding detection, measurement, or evaluation period, where:

-	For the cell detection procedure: N is at least one candidate SSB position (NOTE: the one candidate SSB position for the cell detection shall not be impacted by the set of candidate SSB positions which are already being measured by the UE within the current measurement period of the on-going measurements), and

-	For other procedures in clause 4.2.2.5.7: N are the first two successive candidate SSB positions when two or more candidate SSB positions are configured for this SSB index in one discovery burst transmission window, otherwise N is one candidate SSB position;

otherwise the SMTC occasion is considered as available at the UE.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher and lower priority inter-RAT cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter_CCA/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-RAT cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-RAT cell has met reselection criterion defined TS 38.304 within Kcarrier * Tevaluate,NR_Inter + Kcarrier_CCA * Tevaluate,NR_Inter_CCA when Treselection = 0 as specified in table 4.2.2.5.7-1 if eDRX_IDLE cycle is not configured and in table 4.2.2.5.7-2 if eDRX_IDLE cycle is configured, provided that the reselection criteria is met by

-	6dB in FR1 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 for SS-RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT cell is satisfied with the reselection criteria, the UE shall evaluate this inter-RAT cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2.2.5.7-1: Tdetect,NR_Inter_CCA, Tmeasure,NR_Inter_CCA and Tevaluate,NR_Inter_CCA

| DRX cycle length [s] | Tdetect,NR_Inter_CCA [s] (number of DRX cycles) | Tmeasure,NR_Inter_CCA [s] (number of DRX cycles) | Tevaluate,NR_Inter_CCA [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 0.32x([36]+Md) {([36]+ Md) } | 0.32x([4]+Mm) {([4]+ Mm) } | 0.32x([16]+Me) {([16]+ Me) } |
| 0.64 | 0.64x([28]+ Md) {[28]+ Md } | 0.64x([2]+ Mm) {[2]+ Mm } | 0.64x([8]+ Me) {[8]+ Me } |
| 1.28 | 1.28x([25]+ Md) {[25]+ Md } | 1.28x([1]+ Mm) {[1]+ Mm } | 1.28x([5]+ Me) {[5]+ Me } |
| 2.56 | 2.56x([23]+ Md) {[23]+ Md } | 2.56x([1]+ Mm) {[1]+ Mm } | 2.56x([3]+ Me) {[3]+ Me } |
| Note 1: Md, Mm, Me are the number of DRX cycles each with at least one SMTC occasion not available at the UE during the Tdetect,NR_Inter_CCA,, Tmeasure,NR_Inter_CCA and Tevaluate,NR_Inter_CCA, respectively. Mm,max, Md,max and Me,max are the maximum values of Mm, Md and Me, respectively. Note 2: Mm ≤ Mm,max, where: Mm,max = [16] for DRX cycle = 0.32 seconds, Mm,max = [8] for DRX cycle = 0.64 seconds, Mm,max = [4] for DRX cycle = 1.28 seconds, Mm,max = [4] for DRX cycle = 2.56 seconds, Note 3: Md ≤ Md,max, where: Md,max = [4] * Mm,max, Note 4: Me ≤ Me,max, where: Me,max = [2] * Mm,max, |  |  |  |

Table 4.2.2.5.7-2: Tdetect,NR_Inter_CCA, Tmeasure,NR_Inter_CCA and Tevaluate,NR_Inter_CCA for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods Note 5) | Tdetect,NR_Inter_CCA [s] (number of DRX cycles Note 4) | Tmeasure,NR_Inter_CCA [s] (number of DRX cycles Note 4) | Tevaluate,NR_Inter_CCA [s] (number of DRX cycles Note 4) |
| --- | --- | --- | --- | --- | --- |
| 5.12 | - | - | 5.12x([23]+ Md) {[23]+ Md} | 5.12 *(1+Mm) {1+ Mm } | 5.12*(2+ Me) {2+ Me } |
| 10.24 ≤ eDRX_IDLE cycle length ≤2621.44 | 0.32 | ≥11.52 (9) | eDRX_cycle_length×(23+Md)PTW/DRX_cycle_length (23+Md) | 0.32 x (1.5+ Mm)) {1.5+ Mm } | 0.32 x (2*1.5+ Me) {3+ Me} |
|  | 0.64 | ≥11.52(9) |  | 0.64 x (1+ Mm) {1+ Mm } | 0.64 x (2+ Me) {2+ Me } |
|  | 1.28 | ≥12.8 (10) |  | 1.28 x (1+ Mm) {1+ Mm } | 1.28 x (2+ Me) {2+ Me } |
|  | 2.56 | ≥25.6 (20) |  | 2.56 x (1+ Mm) {1+ Mm } | 2.56 x (2+ Me) {2+ Me } |
| Note 1: Md, Mm, Me are the number of DRX cycles each with at least one SMTC occasion not available at the UE during the Tdetect,NR_Inter_CCA,, Tmeasure,NR_Inter_CCA and Tevaluate,NR_Inter_CCA, respectively. Mm,max, Md,max and Me,max are the maximum values of Mm, Md and Me, respectively. Note 2: Mm ≤ Mm,max, where: Mm,max = 16 for DRX cycle = 0.32 seconds, Mm,max = 8 for DRX cycle = 0.64 seconds, Mm,max = 4 for DRX cycle = 1.28 seconds, Mm,max = 4 for DRX cycle = 2.56 seconds, Note 3: Md ≤ Md,max, where: Md,max = 4 * Mm,max, Note 4: Me ≤ Me,max, where: Me,max = 2 * Mm,max, Note 5: The lower bound of PTW window shall be derived based on (Tevaluate,NR+Me,max)*DRX_cycle1.28*1.28. Tevaluate,NR is the inter-RAT NR cell evaluation period without CCA, as defined in Table 4.2.2.5.6-3. NOTE 6: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 7: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. |  |  |  |  |  |

The UE shall restart the measurements upon exceeding Mm,max, Md.max, or Me,max.

End of Change 1