3GPP TSG-RAN WG4 Meeting #108	R4-2311683

Toulouse, France, 21st - 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7227 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for clarification on Ksatellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Values of Ksatellite are missing or not specified for corresponding GSO scenario |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add values for parameter Ksatellite - Additionally for 6.5A.2.1, 6.5A.2.2, revise Ksatellite as carrier specific. And removing brackets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing values of Ksatellite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6A.2.2, 4.6A.2.4, 4.6A.2.5, 4.6A.2.6, 4.7A.2.1.3, 4.7A.2.2.3, 6.5A.2.1, 6.5A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change>

#### 4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_NC when Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_NC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite*Tmeasure,NB_Intra-NC/2

The UE shall not consider an NB-IoT neighbour cell in cell reselection if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Ksatellite*Tevaluate,NB_intra-NC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

### <Next Change>

#### 4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage

The UE shall be able to identify new intra-frequency cells and perform NRSRP measurements of identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite*Tdetect,NB_Intra_EC when that Treselection= 0 .  An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.4 for a corresponding Band.

The UE shall measure NRSRP at least every Ksatellite*Tmeasure,NB_Intra_EC for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter NRSRP measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite*Tmeasure,NB_Intra_ EC/2

The UE shall not consider a NB-IoT neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving NB-IoT cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Ksatellite*Tevaluate,NB_intra_EC when Treselection = 0, provided that the cell is at least XdB better ranked, where ‘X’ is specified in Table 4.6A.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

### <Next Change>

#### 4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev ≤ SnonIntraSearchP then the UE shall search for and measure inter-frequency layers in preparation for possible reselection.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite *Pcarrier * Tdetect,NB_Inter_NC, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving NB-IoT cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least Y dB, where Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell and ‘Y’ is specified by Table 4.6A.2.6-3 (when Q1 -6 dB). An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least [2] measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite *Tmeasure, Inter_NB-IoT_NC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Ksatellite * Pcarrier * Tevaluate,NB_Inter_NC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

### <Next Change>

#### 4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage

The UE shall be able to identify new inter-frequency cells and perform NRSRP measurements of identified inter-frequency cells if carrier frequency information is provided by the serving NB-IoT cell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite *Pcarrier * Tdetect,NB_Inter_EC. An inter-frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.1.5 for a corresponding Band.

The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

The UE shall filter NRSRP measurements of each measured inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite *Tmeasure,NB_Inter _EC/2.

If an inter-frequency cell has been already detected but that has not been reselected to the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Ksatellite * Pcarrier * Tevaluate,NB_Inter_EC. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and inter-frequency NB-IoT cells.

The parameter Ksatellite is the scaling factor for measurements correspond to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

### <Next Change>

##### 4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage

The requirements in this subclause apply if UE is in the normal coverage area of the serving cell served by satellite access node. The UE is considered to be in normal coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall be able to identify new inter-frequency cells and perform RSRP or RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided. The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite * Kcarrier*Tdetect,EUTRAN_Inter_NC, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 8 dB for reselections based on ranking or 8 dB for RSRP reselections based on absolute priorities or 5.5 dB for RSRQ reselections based on absolute priorities. Kcarrier is the number of inter-frequency carriers in the neighbour cell list. An inter frequency cell is considered to be detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.8 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,E-UTRAN_Inter_NC . If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure RSRP or RSRQ at least every Ksatellite * Kcarrier*Tmeasure,EUTRAN_Inter_NC for identified lower or equal priority inter-frequency cells. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter RSRP or RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite * Tmeasure,EUTRAN_Inter_NC/2.

The UE shall not consider a E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Ksatellite * Kcarrier*Tevaluate,E-UTRAN_Inter_NC, when Treselection = 0 provided that the reselection criteria is met by a margin of at least 7 dB for reselections based on ranking or 7dB for RSRP reselections based on absolute priorities or 5dB for RSRQ reselections based on absolute priorities.

The parameter Ksatellite is the scaling factor for measurements corresponding to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

### <Next Change>

##### 4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage

The requirements in this subclause apply if UE is in the enhanced coverage area of the serving cell. The UE is considered to be in enhanced coverage area of serving cell according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot of the serving cell defined in Annex B.1.3 for a corresponding Band.

The UE shall be able to identify new inter-frequency cells and perform RSRP or RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided. The UE shall not cause any interruption to the paging reception and acquisition of SI while performing measurement on serving or any neighbor cells.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS36.304 within Ksatellite * Kcarrier*Tdetect,EUTRAN_Inter_EC, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 8 dB for reselections based on ranking. Kcarrier is the number of inter-frequency carriers in the neighbour cell list. An inter frequency cell is considered to be detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.8 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,E-UTRAN_Inter_EC . If, after detecting a ce	ll in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure RSRP or RSRQ at least every Ksatellite * Kcarrier*Tmeasure,EUTRAN_Inter_EC for identified lower or equal priority inter-frequency cells. If the UE detects on a E-UTRA carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter RSRP or RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 4 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Ksatellite * Tmeasure,EUTRAN_Inter_EC/2.

The UE shall not consider a E-UTRA neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 36.304 within Ksatellite * Kcarrier*Tevaluate,E-UTRAN_Inter_EC, when Treselection = 0 provided that the reselection criteria is met by a margin of at least 8 dB for reselections based on ranking.

The parameter Ksatellite is the scaling factor for measurements corresponding to multiple NGSO satellites. Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite  equals to the number NGSO satellites to be measured if NGSO satellites are monitored.

### <Next Change>

#### 6.5A.2.1	UE Re-establishment delay requirement in normal coverage

The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [2] is detected by the UE to the time when the UE sends PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) requirement shall be less than:

TUE-re-establish_delay_NB-IoT=100 ms+i=1NNB-IoT-freqTsearch_NB1-NC,i+TSI_NB1-NC+TPRACH_NB-IoT

-	Tsearch_NB1-NC,i: It is the time required by the UE to search the target cell:

-	If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-NC,i = Ksatellite,i *80 ms. Otherwise, Tsearch_NB1-NC,i = Ksatellite,i *1400 ms. Where Ksatellite,i is defined as the number NGSO satellites to be measured on i-th frequency for RRC re-establishment. Ksatellite,i = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite,i equals to the number NGSO satellites to be measured on the carrier if NGSO satellites are monitored.

### <Next Change>

#### 6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage

The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 36.331 [2] is detected by the UE to the time when the UE sends PRACH preamble to the target cell. The UE re-establishment delay (TUE_re-establish_delay_NB-IoT) requirement shall be less than:

TUE-re-establish_delay_NB-IoT=100 ms+i=1NNB-IoT-freqTsearch_NB1-EC,i+TSI_NB1-EC+TPRACH_NB-IoT

-	Tsearch_NB1-EC: It is the time required by the UE to search the target cell:

-	If the target cell is known, then Tsearch_NB1-EC,i = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-EC,i = Ksatellite *80 ms. Otherwise, Tsearch_NB1-EC,i = Ksatellite,i *14800 ms. Where Ksatellite,i is defined as the number NGSO satellites to be measured on i-th frequency for RRC re-establishment. Ksatellite,i = 1, if GSO satellite(s) is/are measured on the carrier. Ksatellite,i equals to the number NGSO satellites to be measured on the carrier if NGSO satellites are monitored.

### <End of Change>