3GPP TSG RAN WG4 Meeting #108		R4-2314660

# Auguest 21 – Auguest 25, 2023 in Toulouse, France

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR | 0002 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR 38.872 to introduce some potential RF impacts for for CA_n26-n28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., [Skyworks Solutions Inc., Apple, Qualcomm, Huawei] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce some potential RF impacts according TPs in R4-2303576 R4-2311116, R4-2311753 and R4-2313371 for CA_n26-n28 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce some potential RF impacts for for CA_n26-n28 when uplink is n26 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The outcomes for CA_n26-n28 for this SI can’t be captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.6.3.5, 5.6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change for TR 38.872>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-221464, “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, RAN#96

[3]	TS 38.101-1

[4]	TS 38.101-3

[5]	RP-230789, “ Study on enhancement for sub-1Ghz NR band combinations”. RAN 99

[6]	3GPP TR 38 892 , APT 600 MHz NR band.

[7]	R4-2308581, Discussion on RF architecture and potential MSD issues for CA_n5-n105, Huawei, HiSilicon, Spark NZ

[8]	R4-2309299, Input on CA_n5-n105 cross-band MSD and DeltaT/R, Skyworks Solutions Inc.

[9]	R4-2309356, Considerations on CA_n5-n105, Qualcomm France

[10]	R4-2309358, Considerations on CA_n26-n28, Qualcomm France

[11]	R4-2306468, WF on CA_n5-n28 and CA_n8-n20-n28, Huawei, HiSilicon

[12]	R4-2308583, Discussion on RF architecture and potential MSD issues for CA_n26-n28, Huawei, HiSilicon

[13]	R4-2307477, Input on CA_n26-n28 requirements, Skyworks Solutions Inc.

[14] 	R4-2306470, WF on RF architecture and critical issues for CA_n28-n105

[17]	R4-2303576 TP for CA_n26-n28, MediaTek Inc.

[18]	R4-2311116 CA_n26-n28 cross band MSD based on measurements, Skyworks Solutions Inc.

[19]	R4-2311753 MSD analysis for CA_n5-n105, CA_n28-n105, and CA_n26-n28, Apple Inc.

[20]	R4-2313371 Considerations on CA_n26-n28, Qualcomm France

## <<Unchanged Parts are Omitted>>

#### 5.6.3.5	REFSENS exception evaluation for one UL band

1) Referring to the contribution [12], the following band n28A MSD evaluations, assuming 35dB and 45dB Tx filter rejection of band n26.

Table 5.6.3.5-1: band n28A MSD evaluations due to cross band isolation for CA_n26-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28A | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 25.6 | ACLR2 |
| n26 | n28A | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 15.9 | ACLR2 |

Referring to the contribution [12], we have the following band n28B MSD evaluations, assuming filter performance with set 1 and set 2 in Table 5.6.3.5-3.

Table 5.6.3.5-2: band n28B MSD evaluations due to cross band isolation for CA_n26-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28B | 824 | 20 | 15 | 25 (RBstart=0) | 801.5 | 5 | 45.9 (Set 1) | ACLR1 |
| n26 | n28B | 824 | 20 | 15 | 25 (RBstart=0) | 801.5 | 5 | 28.5 (Set 2) | ACLR1 |

Table 5.6.3.5-3: Filter performance assumption

|  | Set 1 | Set 2 |
| --- | --- | --- |
| n28B Rx filter rejection at 814~849MHz | 12 dB | 33 dB |
| n26 Tx filter rejection at 773~803MHz | 35 dB | 45 dB |

2) Referring to the contribution [13], the following analysis were provided.

To evaluate the potential 1UL and 2UL for CA_n26-n28 with band n28 full (n28F) or the lower 30MHz (n28A), it is useful to compare existing cases for CA_n5-n28F and CA_n18-n28F and CA_n18-n28A. This can be done by observing the IMD landscape provided in Figure 5.6.3.5-1, and the related wanted UL RB allocation and its image IMDs overlaps for 5MHz and 30MHz n28DL for n28F or n28A scenarios that are collected in Table 5.6.3.5-4.

In addition, 1UL cross band MSD for n28F and n28A were proposed in 1UL cross band MSD for n28F and n28A.

Figure 5.6.3.5-1: IMD landscape of n5, n18, n26 and n28 for n28F and n28A cases.

Table 5.6.3.5-4: Distance to n28 victim DL and IMD overlap for different UL and DL scenarios

| Band | Max UL BW | #RB | Distance to (MHz) |  | IMD overlap |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n28A | n28F | n28A 5MHz | n28F 5MHz | n28A 30MHz | n28F 30MHz |
| n26 | 20MHz | 25 | 26 | 11 | IMD5/7 | IMD3 | IMD5/7 | IMD3/5/7 |
| n5 | 20MHz | 20 | 21 | 36 | IMD5/7 | IMD5 | IMD5/7 | IMD5/7 |
| n18 | 15MHz | 25 | 17 | 12 | IMD5/7 | IMD3/5 | IMD5/7 | IMD3/5/7 |
| n28A | 30MHz | 25 | 25 | na | na | na | IMD3/5 | na |
| n28F | 30MHz | 25 | na | 10 | na | na | na | IMD3/5 |

Table 5.6.3.5-5: 1UL cross band MSD for n28F and n28A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 822.5 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | [>30] | ACLR1 |
| n26 | n28 | 822.5 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | [18] | ACLR2 |

3) Referring to the contribution [10], the technical analysis for two cases (case 1: n28 Cross-band MSD for lower 30MHz of n28 / case 2: n28 Cross-band MSD for entire n28) are shown below.

Table 5.6.3.5-6: Cross-band MSD for lower 30MHz of n28

Table 5.6.3.5-7: n28 Cross-band MSD for entire n28

It’s observed that MSD is about 15-20dB higher if the entire n28 must be supported, compared to case when only lower 30MHz of n28 is supported.

Table 5.6.3.5-8: n28 Cross-band MSD test point for lower 30MHz of n28 and entire n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 17.0 | ACLR2 |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | 33.7 | ACLR |

4) Referring to the contribution [17], the following analysis were provided.

The combo is a L-L band combination with small gap between uplink range of n26 and downlink range of n28. The maximum uplink bandwidth of n26 is up to 20MHz that its ACP1 would fall into DL range of n28 directly if uplink is allocated full RB. Below table lists our assumption for MSD characterization. Here we consider three-antenna implementation since the frequency gap between n26 uplink and n28 downlink is only 11MHz that we don’t think a low band diplexer is feasible with existing technology. Based on previous RAN4 agreement, there shall not be frequency range constraint on n28 thus MSD shall be characterized with full frequency range.

Table 5.6.3.5-9 Receiver performance parameters for MSD analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection at n26 UL range | 15 | dB |
| n26 ACP1 | -30 | dBc |
| n26 CIM3 | -60 | dBc |
| Front-end loss | 4 | dB |
| PCB isolation | 67 | dB |
| PA RXBN noise | -115 | dBm/Hz |
| Thermal noise at n28 RX ANT port | -165.5 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

With above parameters, we calculate MSD due to cross band isolation as below:

Table 5.6.3.5-10 Link analysis for Band 28 5MHz REFSENS calculation at fc = 800.5 MHz

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n26 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n26PA output noise power at n28 LNA input port | -71.3 | -71.3 | dBm/5MHz |
| n28 LNA IM2 due to n26 uplink | -53 | -53 | dBm/5MHz |
| Thermal noise at RX ant port | -162.5 |  | dBm/Hz |
| CIM3 noise at n28 LNA input port | -55.8 | -55.8 | dBm |
| Total noise level refer to B28 LNA input port | -51.1 | -51.1 |  |
| Combined |  |  |  |
| MSD (5MHz BW) | 47.4 |  | dB |

Table 5.6.3.5-11: band n28B MSD evaluations due to cross band isolation for CA_n26-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28B | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | 47.4 | ACLR1 |

5) Referring to the contribution [18], new evaluation based on measurements were provided compared to [13] which was based on extrapolated from similar combination:

- 20MHz band n26 output noise in n28A (first 30MHz of band n28) and n28B (last 30MHz of band n28) for:

  - 5, 20, 25 and 30MHz band n28 CBW

  - At 23dBm for 1UL and 20dBm for 2UL

- Band n28 self-desense for 5, 20, 25 and 30MHz band n28 CBW at 23dBm for 1UL and 20dBm for 2UL.

Based on the chosen architecture, the following RF front-end performance is assumed:

- Post PA and pre LNA losses = 4dB

- Antenna isolation 10dB

- Band n26 UL filter from n26UL/n26DL/n28DL triplexer performance:

  - n26 UL to n28A DL T/R isolation = 38dB

  - n26UL to n28A DL antenna rejection = 35dB

  - n26 UL to n28A DL T/R isolation = 35dB (11MHz distance)

  - n26UL to n28A DL antenna rejection = 32dB (11MHz distance)

  - These performances are justified by the fact that band n26 is one of the most critical low band duplexers (35MHz BW with 10MHz duplex gap)

- Band n28 UL filter from n28AorBUL/n26DL/n28AorBDL triplexer performance

  - n28 UL to n26 DL T/R isolation = 40dB

  - n28 UL to n26 DL antenna rejection = 38dB

These performances are justified for a full band n28 design and the proximity of band n26.

For CA_n26-n28, the 1UL scenario is using the largest n28 and n26 UL CBW (respectively 30MHz and 20MHz) closest to the victim band with a 5MHz Rx CBW. Still, in previous meetings, there was discussion regarding whether the 30MHz lower band n28 or the full band n28 should be considered, for this we evaluated n28A and n28B (representative of full band) cases:

- n28B (upper 30MHz) is victim of IMD3 of band n26 UL wanted RBs and their image including its peak

- n28A (lower 30MHz) is victim of IMD5 of band n26 UL wanted RBs and their image including its peak

- n26 is victim of n28B IMD9 and n28A IMD11, at these high IMD offsets, the noise is dominated by the wideband transmitter white noise floor.

We performed measurements for both sub-band n28A and n28B cases and the MSD after MRC results are summarized in Table 3.

Table 5.6.3.5-12: Post MRC MSD calculations based on measured transmitter noise.

| UL/BW/IM-DL |  | n26/20/5-n28A |  | n26/20/3-n28B |  | n28AB/30/9-n26 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW/NRB | [MHz]/# | 5 | 25 | 5 | 25 | 5 | 25 |  |
| Meas Tx noise / transmitter noise floor | [dBm/CBW] | -43.1 | -63.5 | -23.3 | -63.5 | -85.0 | -63.5 |  |
| REFSENS | [dBm/CBW] | -98.5 |  | -98.5 |  | -97.5 |  |  |
| RX noise floor wo Tx noise : Main/Div | [dBm/CBW] | -94.5 | -94.5 | -94.5 | -94.5 | -93.5 | -93.5 |  |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 38 | 35 | 35 | 32 | 37 | 34 |  |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  | 4 |  |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 | 0 | 10 |  |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -77.1 | -84.1 | -54.3 | -61.3 | -96.4 | -103.4 |  |
| Noise degradation due to Tx noise : Main/Div | [dB] | 17.5 | 10.8 | 40.2 | 33.2 | 1.8 | 0.4 |  |
| REFSENS degradation after MRC | cor [dB] | 15.3 |  | 37.8 |  | 1.7 |  |  |

Observations:

- With only the transmitter noise floor contributing, the band n26 MSD due to band n28 is small

- The MSD of band n28B victim of band n26 UL IMD3 is confirmed to be significantly above 30dB

- The MSD of band n28A victim of band n26 UL IMD5 is slightly lower than estimated in [2].

These observations lead to the following 1UL cross band isolation MSD proposals, which provides MSD for full band and lower 30MHz of band n28 as spectrum availability varies in different regions.

Table 5.6.3.5-13: 1UL cross band MSD for CA_n26-n28 accounting for both full band and lower 30MHz only of band n28.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | 37.8 | ACLR1 |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 15.3 | ACLR2 |
| n28 | n26 | 733 | 30 | 15 | 25 (RBstart=135) | 861.5 | 5 | 1.7 | >ACLR2 |

6) Referring to the contribution [19], new evaluation for one uplink is provided:

During the study phase for the CA_n26-n28 band combination, there was no agreement on whether to consider only the lower 30MHz section of n28 or the entire band, as captured in the following WF:

Case 1: lower 30MHz of n28 (UL: 703~733MHz, DL: 758~788MHz) is supported.

Case 2: entire n28 frequency range (UL: 703~748MHz, DL: 758~803MHz) is supported.

As a consequence of this, we will do the MSD analysis and access the ∆TIB,c and ∆RIB,c for both cases. The corresponding aggregated spectrums are shown in Figures 2.3-1 and 2.3-2 below:

Figure 5.6.3.5-2: Aggregated spectrum allocation for the CA_n26-n28 combination (full n28 band)

Figure 5.6.3.5-3: Aggregated spectrum allocation for the CA_n26-n28 combination (lower 30MHz of n28 band)

The two-antenna architecture is captured in the following figure.

Figure 5.6.3.5-4: CA_n26-n28 RF front-end architecture based on 2 antennas and 2 triplexers

    - CA_n26-n28 MSD Analysis

As can be seen from the MSD contributor table below, for n28 UL n26 DL, there is more than 110MHz of spectrum gap. Therefore, the cross-band interference for that case will be minimum and so the MSD for n26 DL will be small. However, for the n26 UL  n28 DL, there is only 11MHz of separation when the n28 full 45MHz band is considered and 26MMHz when the lower 30MHz of n28 (n28A) is assumed. Due to that small frequency separation, there is a strong cross-band interference that will affect the n28 DL. Next, we will examine the n28 DL MSD for both cases.

| Interference Source | 1 Uplink (n26UL or n28UL) | 2 Uplinks (n26UL+n28UL) |
| --- | --- | --- |
| Harmonic/harmonic mixing | No issue | No issue |
| Intermodulation | No issue | No significant issue |
| Cross-band isolation | Option # 1: Entire n28 band - Significant ACLR1 and cross band noise issue for n26ULn28DL case - Potential TX noise leakage issue Option # 2: Lower 30MHz of 28 band - Significant ACLR2 and cross band noise issue for n26ULn28DL case - Potential TX noise leakage issue |  |

Table 5.6.3.5-14: CA_n26-n28 MSD cross-band interference sources

For the n28 DL MSD analysis, we will use the following worst-case assumptions for the RF front-end performance parameters:

- n26 Tx filter rejection at n28 RX (758MHz-803MHz): 30dB

- Cross-band TX to RX isolation: 40dB

- ACLR2 sideband level: 30dBc

- ACLR3 sideband level: 30dBc

- Front-end insertion loss: 4.5dB

- Antenna coupling: 10dB

- RxBW: 4.5MHz

- Output Power at antenna: 23dBm

Given the assumptions above, the MSD analysis is summarized in the table below

| MSD Source | Parameter | Unit | n26 Tx n28 RX Case 1 (Full 45MHz BW) |  | n26 Tx n28 RX Case 2 (Lower 30MHz BW only) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n28 main path | n28 div. path | n28 main path | n28 div. path |
| PA noise + ACLR | TX noise leakage at PA output due to ACLR2 (PC3) | dBm/RxBW | -14 |  | -39.5 |  |
|  | Total PA noise @ ANT | dBm/RXBW | -48.5 | -58.5 | -79 | -89 |
| IIP2 effects | Typical Rx IIP2 | dBm | 54 | 54 | 54 | 54 |
|  | TX IM2 noise level @ ANT | dBm | -98 | -108 | -98 | -108 |
| Thermal Noise | 3GPP REFSENS | dBm | -98.5 | -98.5 | -98.5 | -98.5 |
|  | SNR | dB | -1 | -1 | -1 | -1 |
|  | Thermal noise | dBm | -97.5 | -97.5 | -97.5 | -97.5 |
| Composite | Composite noise | dBm | -48.5 | -58.5 | -78.9 | -88.3 |
|  | Composite noise after MRC | dBm | -65.7 |  | -88.7 |  |
| MSD |  | dB | >31.0 |  | 8.7 |  |

Table 5.6.3.5-15: CA_n26-n28 MSD analysis

    - CA_n26-n28 ∆TIB,c and ∆RIB,c Values

For CA_n26-n28, we can reuse the values already derived for the CA_n5-n28 as proposed in [10].

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.7 |
|  | n28 | 0.7 |

Table 5.6.3.5-16: ∆TIB,c  for CA_n26-n28

| Inter-band CA Configuration | NR Band | RIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.2 |
|  | n28 | 0.2 |

Table 5.6.3.5-17: ∆RIB,c  for CA_n26-n28

7) Referring to the contribution [20], new evaluation for one uplink is provided:

We did further MSD analyses, resulting in slightly different MSD numbers.

Conventional 3GPP assumptions for the TX for the PA noise measurements were used, as well as in factoring the CIM3/CIM5 contributions:

- 20 MHz 100RB DFT-s-OFDM

- 30dBc ACLR

- 4dB Post-PA loss

- Image -28dB

- LO -28dBc

- CIM3 -60dBc

- CIM5 -70dBc

Figure 5.6.3.5-5 n28 Cross-band MSD for lower 30MHz of n28

Figure 5.6.3.5-5 n28 Cross-band MSD for entire n28

Table 5.6.3.5-17 MSD test points when deciding the specifications for CA_n26A-n28A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 17.1 | ACLR2 |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | 31.4 | ACLR |

## <<Unchanged Parts are Omitted>>

#### 5.6.4.3	REFSENS exception evaluation for two UL band

1) MSD due to IMD

If the following test frequency range was assumed, we can calculate the frequency range which is hit by IMD3 from UL CA_n26-n28 as 880~895 MHz. It can be observed the IMD3 missed the DL channel of band n26 by 5MHz.

n26: UL 814~819	DL 859~864

n28: UL 743~748	DL 798~803

IMD3 range is 880~895 MHz

That means there is no direct test frequency point which can be found for IMD3 MSD definition.

2) MSD due to two UL cross band isolation

Even if the performance of duplexer/filter’s isolation can be guaranteed by one UL cross band isolation MSD, two PA linearity performance should be guaranteed simultaneously by two UL cross band isolation MSD requirements especially for the case that two 1st / 2nd adjacent channel interferences from two different UL aggressor bands hit the DL part of victim band. The following test points were proposed in contribution R4-2307477 for further discussion.

Table 5.6.4.3-1: 2UL cross band MSD for n28F and n28A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 788 | 30 | [>6] | ACLR1 from n26 UL band and ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 773 | 30 | [3] | ACLR2 from n26 UL band and ACLR1+ACLR2 from n28 UL band |
| n28 |  | 718 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

New evaluation based on measurement in [16] is also provided as below:

Since CA_n26-n28 UL configuration is also considered and, similar to the work performed for CA_n5-n28, the MSD due to the addition of the band n28 UL and band n26 UL noises must be evaluated. For this, we measured the noise levels at 20dBm for the band n26UL for 20/25/30MHz CBW of band n28 and again looking at the lower or upper 30MHz.

This time we had to measure the transmitter noise, evaluate the corresponding noise leaking to the receiver for each UL accounting for the front-end performance and assuming 3dB lower noise floor at 20dBm than 23dBm.

This resulted in the interference levels at the receiver in:

- Table 5.6.4.3-2 for 20dBm band n26 UL in band n28A which correspond to an IMD5 case (ACLR2)

- Table 5.6.4.3-3 for 20dBm band n26 UL in band n28B which correspond to an IMD3 case (ACLR1)

- Table 5.6.4.3-4 for the 20dBm band n28 UL self desense. It corresponds to an IMD5 case for 20 and 25MHz CBW and IMD3 case for 30MHz CBW.

Table 5.6.4.3-2: 20dBm band n26 UL noise in band n28A for 20/25/20MHz CBW.

| UL/BW/IM-DL |  | n26/20/5-n28A |  | n26/20/5-n28A |  | n26/20/5-n28A |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW/NRB | [MHz]/# | 20 | 106 | 25 | 133 | 30 | 160 |
| Meas Tx noise / transmitter noise floor | [dBm/CBW] | -48.3 | -60.2 | -46.5 | -59.2 | -45.7 | -58.4 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 38 | 35 | 38 | 35 | 38 | 35 |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  | 4 |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -81.1 | -88.1 | -80.3 | -87.3 | -79.5 | -86.5 |

Table 5.6.4.3-3: 20dBm band n26 UL noise in band n28B for 20/25/20MHz CBW.

| UL/BW/IM-DL |  | n26/20/3-n28B |  | n26/20/3-n28B |  | n26/20/3-n28B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CBW/NRB | [MHz]/# | 20 | 106 | 25 | 133 | 30 | 160 |
| Meas Tx noise / transmitter noise floor | [dBm/CBW] | -33.4 | -60.2 | -28.8 | -59.2 | -28.2 | -58.4 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 35 | 32 | 35 | 32 | 35 | 32 |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  | 4 |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -64.4 | -71.4 | -59.8 | -66.8 | -59.2 | -66.2 |

Table 5.6.4.3-4: 20dBm band n28 UL noise for 20/25/20MHz CBW.

| CBW/NRB | [MHz]/# | 20 | 106 | 25 | 133 | 30 | 160 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Meas Tx noise / transmitter noise floor | [dBm/CBW] | -55.0 | -60.2 | -43.0 | -59.2 | -36.6 | -58.4 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 50 | 46 | 50 | 46 | 50 | 46 |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  | 4 |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -99.9 | -105.9 | -88.9 | -94.9 | -82.6 | -88.6 |

From the above, we can derive the MSD post MRC accounting for the addition of n26 and n28 UL noises, but also considering that the main and diversity antennas are swapped between the two bands in the proposed architecture. The calculations are provided for the three band n28 victim bandwidths in Table 5.6.4.3-5 for n28A and Table 5.6.4.3-6 for n28B.

Table 5.6.4.3-5: REFSENS degradation in n28A.

| n28CBW | 20MHz n28A |  | 25MHz n28A |  | 30MHz n28A |  |
| --- | --- | --- | --- | --- | --- | --- |
| antenna | main | div | main | div | main | div |
| n28 20dBm UL noise [dBm/CBW] | -99.9 | -105.9 | -88.9 | -94.9 | -82.6 | -88.6 |
| n26 20dBm UL noise [dBm/CBW] | -89.0 | -82.0 | -87.3 | -80.3 | -86.5 | -79.5 |
| n28 REFSENS [dBm/CBW] | -90.8 |  | -84.2 |  | -78.5 |  |
| 2UL interference levels [dBm/CBW] | -88.7 | -82.0 | -85.0 | -80.1 | -81.1 | -79.0 |
| Noise degradation due to Tx noise [dB] | 2.7 | 6.1 | 3.3 | 3.5 | 3.6 | 2.1 |
| REFSENS degradation after MRC [dB] | 5.4 |  | 3.3 |  | 1.9 |  |

Table 5.6.4.3-6: REFSENS degradation in n28B.

| n28CBW | 20MHz n28B |  | 25MHz n28B |  | 30MHz n28B |  |
| --- | --- | --- | --- | --- | --- | --- |
| antenna | main | div | main | div | main | div |
| n28 20dBm UL noise [dBm/CBW] | -99.9 | -105.9 | -88.9 | -94.9 | -82.6 | -88.6 |
| n26 20dBm UL noise [dBm/CBW] | -71.4 | -64.4 | -66.8 | -59.8 | -66.2 | -59.2 |
| n28 REFSENS [dBm/CBW] | -90.8 |  | -84.2 |  | -78.5 |  |
| 2UL interference levels [dBm/CBW] | -71.4 | -64.4 | -66.8 | -59.8 | -66.1 | -59.2 |
| Noise degradation due to Tx noise [dB] | 15.5 | 22.4 | 13.6 | 20.4 | 9.0 | 15.4 |
| REFSENS degradation after MRC [dB] | 20.1 |  | 18.2 |  | 13.5 |  |

Observations:

- Even in the n28A case, the 2UL noise is above the band n28 REFSENS levels that already suffers from IMD3 or IMD5 self-desense.

- In many n28A cases the band n26 UL noise dominates

- In the n28B case the band n26 noise dominates compared to band n28 self desense and although significantly lower than the n26 1UL case, it still results in significant MSD

- Given that it shows the contribution from both UL the 30MHz CBW for band n28 is preferred.

With the above, the following 2UL MSD is proposed considering both full band n28 and upper 30MHz cases as spectrum availability varies in different regions.

Table 5.6.4.3-7: 2UL cross band MSD for CA_n26-n28 accounting for both full band and lower 30MHz only of band n28.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 788 | 30 | 13.5 | ACLR1 from n26 and n28 |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 773 | 30 | 1.9 | ACLR2 from n26 and ACLR1 from n28 |
| n28 |  | 718 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

## <<End of Change>>