3GPP TSG-RAN WG4 Meeting#108	R4-2314674

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1702 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of applicability of time mask for Tx swtiching with dual TAG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correct the applicability for - the existing time masks (the RRC configured switching gap locations apply) - and for the case when an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in 38.214 and the UE is not scheduled or configured with uplink transmissions for a duration of at least the uplink switching gap advertised in the UE capabiltity for a band pair (RRC configured switching gap locations apply) for up to two TAGs for both UL CA and SUL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.3A.3.3.1: to cases introduced -- when the time masks apply with uplinkTxSwitchingPeriodLocation as configured by RRC, the TA difference should be considered in the time masks and the UE is allowed to drop symbols overlapping with the the switching period for any TA difference -- when the uplinkTxSwitchingPeriodLocation can be ignored (transmission gaps including any TA difference) modify applicability of requirements to apply for up to two TAGs: the UE scheduled or configured such that - simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band - and transmission of any one of the carriers during the uplink switching period indicated in UE capability. do not occur for a time delay up to MTTD = 34.6 µs in the case of dual TAG. Clauses 6.3A.3.3.4 and 6.3A.3.3.5: the above conditions apply with the carrier within a band belong to the same TAG. Clauses 6.3A.3.3.2-6.3A.3.3.5 and 6.3C.3.1-6.3C.3.4: clarify the location of the transient period after the switch. New clause 6.3C.3.0: correction to dual TAG for 3-4 band combinations with SUL carriers (put in square brackets for this version of the specification) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location of transient periods unclear when dual TAG is configured with Tx switching for UL CA and SUL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3.2, 6.3A.3.3.3, 6.3A.3.3.4, 6.3A.3.3.5, 6.3C.3.0 (new), 6.3C.3.1, 6.3C.3.2, 6.3C.3.3, 6.3C.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

##### 6.3A.3.3.1	General

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3.3 apply.

For inter-band carrier aggregation with two contiguous carriers assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3A.3.1 apply for those carriers.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3A.3.2 apply for those carriers.

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

Time masks for Tx switching due to switching period are defined in clauses 6.3A.3.3.2-6.3A.3.3.5 for both single TAG and dual-TAG scenarios. When a UE is configured with dual-TAG, [for up to two band pairs] with at least two cells corresponding to two TAGs involved in one switching event, the timing advance difference should be considered in the time masks in sub-clauses 6.3A.3.3.2-6.3A.3.3.5 for two uplink carriers [and 6.3A.3.3.6 for 3-4 uplink bands]. The UE may omit uplink transmission on OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference.

When the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE, the length and location of allowed transient periods for dual TAG are as specified in 6.3A.3.3.2 – [6.3A.3.3.6] with the UE scheduled or configured with uplink transmissions that do not result in

-	simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band

-	transmission of any of the carriers for a duration of at least the uplink switching gap indicated by UE capability

for any timing difference between uplink carriers in different bands up to the MTTD specified for UL CA in clause 7.5.4 of [38.133] in case of dual TAG.

Carriers within the same band belong to the same TAG in all cases.

##### 6.3A.3.3.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power for CA power class 3 when the capability uplinkTxSwitching-PowerBoosting is present and the IE uplinkTxSwitchingPowerBoosting is enabled, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 as specified in [38.306].

The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The following applies for the uplink switching cases specified in clause 6.1.6.2 of [10] with uplinkTxSwitchingOption set to either switchedUL or dualUL when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of both non-co-located and co-located and synchronized network deployment for the two uplink carriers.

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.

##### 6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod2T2T is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1 and carrier 2.

The switching periods described in Figure 6.3A.3.3.3-1a and Figure 6.3A.3.3.3-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod2T2T.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.3.3-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.3-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The following applies for the uplink switching cases specified in clause 6.1.6.2 of [10] with uplinkTxSwitchingOption set to either switchedUL or dualUL when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod2T2T on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of both non-co-located and co-located and synchronized network deployment for the two uplink carriers.

.

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.

##### 6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod  is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 in band A is capable of one transmit antenna connector, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 and carrier 3 in band B.

The switching periods described in Figure 6.3A.3.3.4-1a and Figure 6.3A.3.3.4-1b are located in either NR band A or band B as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod .

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.3.4-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3A.3.3.4-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The following applies for the uplink switching cases specified in clause 6.1.6.2 of [10] with uplinkTxSwitchingOption set to either switchedUL or dualUL when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of both non-co-located and co-located and synchronized network deployment for the three uplink carriers.

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.

##### 6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod2T2T is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR UL carrier 1 in band A is capable of two transmit antenna connectors, NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1, carrier 2 and carrier 3 in the two bands.

The switching periods described in Figure 6.3A.3.3.5-1a and Figure 6.3A.3.3.5-1b are located in either NR band A or band B as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod2T2T.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.3.5-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3A.3.3.5-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The following applies for the uplink switching cases specified in clause 6.1.6.2 of [10] with uplinkTxSwitchingOption set to either switchedUL or dualUL when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod2T2T on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of both non-co-located and co-located and synchronized network deployment for the three uplink carriers.

The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.

#### 6.3A.3.4	Void

< text omitted >

### 6.3C.3	Transmit ON/OFF time mask for SUL

### 6.3C.3.0	General

[Time masks for Tx switching due to switching period are defined in clause 6.3C.3.5 for both single TAG and dual-TAG scenarios. When a UE is configured with dual-TAG, for up to two band pairs with at least two cells corresponding to two TAGs involved in one switching event, the timing advance difference is considered in the time masks in sub-clause 6.3C.3.5. The UE may omit uplink transmissionon OFDM symbols that partially or fully overlap with the configured switching period for any timing advance difference.]

#### 6.3C.3.1	Time mask for switching between two uplink carriers

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod is present, is only applicable for uplink switching mechanisms specified in clause 6.16 of TS 38.214 [10], where NR SUL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 as specified in [38.306].

The switching periods described in Figure 6.3C.3.1-1a and Figure 6.3C.3.1-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3C.3.1-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.1-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The following applies for the uplink switching case specified in clause 6.1.6.3 of [10] when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3C.3.2	Time mask for switching between two uplink carriers with two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod2T2T is present, is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR SUL carrier 1 is capable of two transmit antenna connectors and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1 and carrier 2.

The switching periods described in Figure 6.3C.3.2-1a and Figure 6.3C.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod2T2T.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3C.3.2-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.2-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The following applies for the uplink switching case specified in clause 6.1.6.3 of [10] when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod2T2T on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3C.3.3	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod  is present, is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR SUL carrier 1 in band A is capable of one transmit antenna connector and NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 and carrier 3 in band B.

The switching periods described in Figure 6.3C.3.3-1a and Figure 6.3C.3.3-1b are located in either NR band A or band B as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod .

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3C.3.3-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3C.3.3-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The following applies for the uplink switching case specified in clause 6.1.6.3 of [10] when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of co-located and synchronized network deployment for the three uplink carriers.

The requirements apply for the case of single TAG for the three uplink carriers, i.e., the same uplink timing for the three carriers as described in clause 4.2 of TS 38.213 [8].

#### 6.3C.3.4	Time mask for switching between two uplink bands with two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod2T2T is present, is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where NR SUL carrier 1 in band A is capable of two transmit antenna connectors and NR UL carrier 2 and carrier 3 in band B are capable of two transmit antenna connectors. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers, and band A and band B are different bands with different carrier frequencies. The UE shall support the switch between two-layer transmission with two antenna ports and two-layer transmission with two antenna ports on the two uplink bands following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 1, carrier 2 and carrier 3 in the two bands.

The switching periods described in Figure 6.3C.3.4-1a and Figure 6.3C.3.4-1b are located in either NR band A or band B as indicated in RRC signalling uplinkTxSwitchingPeriodLocation  [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod2T2T.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3C.3.4-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A

Figure 6.3C.3.4-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

The following applies for the uplink switching case specified in clause 6.1.6.3 of [10] when the configuration of the location of the switching period by uplinkTxSwitchingPeriodLocation is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by uplinkTxSwitchingPeriod2T2T on any of the carriers before T0, transient periods of 10 ms are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of co-located and synchronized network deployment for the three uplink carriers.

The requirements apply for the case of single TAG for the three uplink carriers, i.e., the same uplink timing for the three carriers as described in clause 4.2 of TS 38.213 [8].

< end of changes >