3GPP TSG-RAN WG4 Meeting # 108	R4-2314624

# Toulouse, France, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 6003 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on addition of PC1 operation for band 12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, U.S. Cellular |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_HPUE_FWVM_R18-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of PC1 operation for band 12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MOP and A-MPR tables are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1 operation for band 12 is not allowed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Corresponding draft CR in R4-2306533 was endorsed at RAN4#106bis-e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

### 6.2.2	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration unless otherwise stated. The period of measurement shall be at least as defined in Table 6.2.2-0.

Table 6.2.2-0: Measurement period for UE maximum output power

| TTI pattern | Minimum measurement period |
| --- | --- |
| Subframe | 1ms |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

Table 6.2.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±25 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 236 | ±26 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 | 31 | +2/-3 |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 41 |  |  | 26 | ±22 | 23 | ±22 |  |  |
| 42 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| 50 |  |  |  |  | 23 | ±2 |  |  |
| 51 |  |  |  |  | 23 | ±2 |  |  |
| 52 |  |  |  |  | 23 | +2/-3 |  |  |
| 53 |  |  |  |  | 23 | ±2 |  |  |
| 54 |  |  |  |  | 23 | ±2 |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2 |  |  |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 72 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 73 |  |  |  |  | 23 | ±2 |  |  |
| 74 |  |  |  |  | 23 | ±2 |  |  |
| 85 |  |  |  |  | 23 | ±22 |  |  |
| 87 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 88 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 106 |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. NOTE 7: Void. NOTE 8: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 9: Void. |  |  |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5;

***************************** Next change ************************************

### 6.2.4	UE maximum output power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 1, 2 and 3 the specific requirements and identified subclauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 to 6.2.4-15 are in addition to the allowed MPR requirements specified in subclause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5, 10, 15, 20 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36, 66, 70 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2, 6.6.3.3.19 | 41 | 5, 10, 15, 20 | Table 6.2.4-4, Table 6.2.4-4a |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 (NOTE1) | ≤ 1 (NOTE 1) |
|  |  |  | 15, 20 | Table 6.2.4-18 (NOTE2) |  |
|  |  | 65 (NOTE 3) | 10,15,20 | ≥ 50 | ≤ 1 (NOTE 1) |
|  |  |  | 15,20 | Table 6.2.4-18 (NOTE 2) |  |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | Clause 6.2.4-34b (NOTE 9) |
|  |  | 85 | 5, 10 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 |  |
| NS_08 | 6.6.3.3.3 | 19 | 10, 15 | > 44 | ≤ 3 |
| NS_09 | 6.6.3.3.4 | 21 | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
|  |  | 74 (NOTE 6) | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4-3 |  |
| NS_11 | 6.6.2.2.1 6.6.3.3.13 | 23 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-5 |  |
| NS_12 | 6.6.3.3.5 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4-6 |  |
| NS_13 | 6.6.3.3.6 | 26 | 5 | Table 6.2.4-7 |  |
| NS_14 | 6.6.3.3.7 | 26 | 10, 15 | Table 6.2.4-8 |  |
| NS_15 | 6.6.3.3.8 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4-9 Table 6.2.4-10 |  |
| NS_16 | 6.6.3.3.9 | 27 | 3, 5, 10 | Table 6.2.4-11, Table 6.2.4-12, Table 6.2.4-13 |  |
| NS_17 | 6.6.3.3.10 | 28 | 5, 10 | Table 5.6-1 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | 5 | ≥ 2 | ≤ 1 |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |
| NS_19 | 6.6.3.3.12 | 44 | 10, 15, 20 | Table 6.2.4-14 |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.14 | 23 | 5, 10, 15, 20 | Table 6.2.4-15 |  |
| NS_21 | 6.6.2.2.1 6.6.3.3.15 | 30 | 5, 10 | Table 6.2.4-16 |  |
| NS_22 | 6.6.3.3.16 | 42(NOTE 8), 43 | 5, 10, 15, 20 | Table 6.2.4-17 |  |
| NS_23 | 6.6.3.3.17 | 42(NOTE 8), 43 | 5, 10, 15, 20 | N/A |  |
| NS_24 | 6.6.3.3.20 | 65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.4-19 |  |
| NS_25 | 6.6.3.3.21 | 65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.4-20 |  |
| NS_26 | 6.6.3.3.22 | 68 | 10, 15 | Table 6.2.4-21 |  |
| NS_27 | 6.6.2.2.5, 6.6.3.3.23 | 48 | 5, 10, 15, 20 | Table 6.2.4-22 |  |
| NS_28 | 6.2.2A, 6.6.2.2.6 6.6.3.3.24 | 46 (NOTE 5) | 20 | Table 6.2.4-23 |  |
| NS_29 | 6.2.2A, 6.6.2.3.1a, 6.6.3.3.25 | 46 (NOTE 5) | 20 | Table 6.2.4-24 |  |
| NS_30 | 6.2.2A, 6.6.3.3.26 | 46 (NOTE 5) | 20 | Table 6.2.4-25 |  |
| NS_31 | 6.2.2A, 6.6.3.3.27 | 46 (NOTE 5) | 20 | Table 6.2.4-26 |  |
| NS_32 | - | - | - | - | - |
| … |  |  |  |  |  |
| NS_35 | 6.6.2.2.7 | 71 | 5, 10, 15, 20 | N/A |  |
| NS_36 | 6.6.3.3.28 | 68 | 5, 10, 15 | Table 6.2.4-27 |  |
| NS_38 | 6.6.3.3.29 | 74 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-28 |  |
| NS_39 | 6.6.3.3.30 | 74 | 10, 15, 20 | Table 6.2.4-29 |  |
| NS_40 | 6.6.3.3.31 | 51 | 3, 5 | Table 6.2.4-30a, Table 6.2.4-30b |  |
| NS_41 | 6.6.3.3.31 | 50 | 3, 5, 10, 15, 20 | Table 6.2.4-31 |  |
| NS_42 | 6.6.3.3.32 | 50 | 3, 5, 10, 15, 20 | Table 6.2.4-32 |  |
| NS_43 | 6.6.2.2.5 6.6.3.3.23 | 49 | 20 | Table 6.2.4-33 |  |
| NS_44 | 6.6.3.3.33 | 38 (Note 7) | 5, 10, 15, 20 | Table 6.2.4-34 |  |
| NS_45 | 6.6.3.3.34 | 53 | 1.4, 3, 5, 10 | N/A |  |
| .. |  |  |  |  |  |
| NS_56 | 6.6.3.3.35 | 24 | 5, 10 | Table 6.2.4-34a |  |
| .. |  |  |  |  |  |
| NS_62 | 6.6.3.3.36 | 54 | 1.4, 3, 5 | N/A |  |
| NOTE 1: Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in NOTE2 as the emissions requirement in 6.6.3.3.1 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. NOTE 2: Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. NOTE 3: Applicable when the E-UTRA carrier is within 1920-1980 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 5: Applicable only for an LAA Scell configured in Band 46. NOTE 6: Applicable when the E-UTRA carrier is within 1447.9 – 1462.9 MHz NOTE 7: Applicable for power class 2 UE in E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz NOTE 8: Not applicable for power class 2 UE in Band 42 NOTE 9: Applicable only for power class 1 UE in Band 12. Not applicable for all other cases. |  |  |  |  |  |

Table 6.2.4-2: A-MPR for "NS_07"

| Parameters | Region A |  | Region B |  | Region C |
| --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 12 |  | 13 – 18 | 19 – 42 | 43 – 49 |
| LCRB [RBs] | 6-8 | 1 to 5 and 9-50 | ≥8 | ≥18 | ≤2 |
| A-MPR [dB] | ≤ 8 | ≤ 12 | ≤ 12 | ≤ 6 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4-3: A-MPR for "NS_10"

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 2 |
| 20 | RBstart | 0 – 15 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 5 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. |  |  |

Table 6.2.4-4: A-MPR requirements for "NS_04" for Power Class 3 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  |  | 9 – 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 2 |  |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  |  | > 2504 |
|  | RBstart | 0 - 8 |  |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 |  | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A |  | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 | ≤ 1 |  | ≤ 2 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  |  | > 2510.8 |
|  | RBstart | 0 - 13 |  |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 or ≥ 36 |  | > 18 and < 36 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 or ≥ 40 |  | > 18 and < 40 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |  |  |  |

Table 6.2.4-4a: A-MPR requirements for "NS_04" for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  | 9 - 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 3 |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  | > 2504 |
|  | RBstart | 0 – 8 |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 3 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  | > 2510.8 |
|  | RBstart | 0 – 13 |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 36 | ≥ 36 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 40 | ≥ 40 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |  |  |

For a power class 2 capable UE operating in Band 41, A-MPR according to Table 6.2.4-4 for power class 3 is allowed when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6.

Table 6.2.4-5: A-MPR for "NS_11"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Fc [MHz] | <2004 |  |  | ≥2004 |  |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-15 |  |  | >5 |  |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤5 |  |  | ≤ 1 |  |  |  |  |  |  |  |  |  |  |
| 5 | Fc [MHz] | <2004 |  |  | 2004 ≤ Fc <2007 |  |  |  |  |  |  | ≥2007 |  |  |  |
|  | LCRB [RBs] | 1-25 |  |  | 1-6 & 15-25 |  |  |  | 8-12 |  |  | >6 |  |  |  |
|  | A-MPR [dB] | ≤7 |  |  | ≤ 4 |  |  |  | 0 |  |  | ≤ 1 |  |  |  |
| 10 | Fc [MHz] | 2005 ≤ Fc <2015 |  |  |  |  |  |  | 2015 |  |  |  |  |  |  |
|  | RBstart | 0-49 |  |  |  |  |  |  | 0-49 |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-50 |  |  |  |  |  |  | 1-50 |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 12 |  |  |  |  |  |  | 0 |  |  |  |  |  |  |
| 15 | Fc [MHz] | <2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-4 | 5-21 |  |  |  |  |  |  |  | 22-56 |  |  |  | 57-74 |
|  | LCRB [RBs] | ≥1 | 7-50 |  |  | 0-6 & ≥50 |  |  |  |  | ≤25 |  | >25 |  | >0 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  | ≤10 |  |  |  |  | 0 |  | ≤6 |  | ≤15 |
|  | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 |  | 13-39 |  |  |  |  |  | 40-65 |  |  |  | 66-74 |  |
|  | LCRB [RBs] | ≥1 |  | ≥30 |  |  |  | <30 |  | ≥ (69 – RBstart) |  |  |  | ≥1 |  |
|  | A-MPR [dB] | ≤10 |  | ≤6 |  |  |  | 0 |  | ≤2 |  |  |  | ≤6.5 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 | 13-29 |  |  |  |  |  |  | 30-68 |  |  |  |  | 69-99 |
|  | LCRB [RBs] | ≥1 | 10-60 |  |  |  | 1-9 & >60 |  |  | 1-24 |  |  | ≥25 |  | ≥1 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  |  | ≤10 |  |  | 0 |  |  | ≤7 |  | ≤15 |

Table 6.2.4-6: A-MPR for "NS_12"

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 1.4 | RBstart | 0 |  | 1-2 |
|  | LCRB [RBs] | ≤3 | ≥4 | ≥4 |
|  | A-MPR [dB] | ≤3 | ≤6 | ≤3 |
| 3 | RBstart | 0-3 |  | 4-5 |
|  | LCRB [RBs] | 1-15 |  | ≥9 |
|  | A-MPR [dB] | ≤4 |  | ≤3 |
| 5 | RBstart | 0-6 |  | 0-9 |
|  | LCRB [RBs] | ≤8 |  | ≥9 |
|  | A-MPR [dB] | ≤5 |  | ≤3 |
| 10 | RBstart | 0-15 |  | 0-22 |
|  | LCRB [RBs] | ≤18 |  | ≥20 |
|  | A-MPR [dB] | ≤4 |  | ≤2 |
| 15 | RBstart | 0-30 |  | 0-30 |
|  | LCRB [RBs] | ≤30 |  | ≥32 |
|  | A-MPR [dB] | ≤4 |  | ≤3 |

Table 6.2.4-7: A-MPR for "NS_13"

| Channel bandwidth [MHz] | Parameters | Region A |  |
| --- | --- | --- | --- |
| 5 | RBstart | 0-2 |  |
|  | LCRB [RBs] | ≤5 | ≥18 |
|  | A-MPR [dB] | ≤3 | ≤2 |

Table 6.2.4-8: A-MPR for "NS_14"

| Channel bandwidth [MHz] | Parameters | Region A |  |
| --- | --- | --- | --- |
| 10 | RBstart | 0 |  |
|  | LCRB [RBs] | ≤5 | =50 |
|  | A-MPR [dB] | ≤3 | ≤1 |
| 15 | RBstart | ≤8 |  |
|  | LCRB [RBs] | ≤16 | ≥50 |
|  | A-MPR [dB] | ≤3 | ≤1 |

Table 6.2.4-9: A-MPR for "NS_15" for E-UTRA highest channel edge > 845 MHz and ≤ 849 MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 1.4 | RBend [RB] |  |  | 4-5 |
|  | A-MPR [dB] |  |  | ≤3 |
| 3 | RBend [RB] | 0-1 | 8-12 | 13-14 |
|  | LCRB [RB] | ≤2 | ≥8 | >0 |
|  | A-MPR [dB] | ≤4 | ≤4 | ≤9 |
| 5 | RBend [RB] | 0-4 | 12-19 | 20-24 |
|  | LCRB [RB] | ≤2 | ≥8 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |
| 10 | RBend [RB] | 0-12 | 23-36 | 37-49 |
|  | LCRB [RB] | ≤2 | ≥15 | >0 |
|  | A-MPR [dB] | ≤4 | ≤6 | ≤9 |
| 15 | RBend [RB] | 0-20 | 26-53 | 54-74 |
|  | LCRB [RB] | ≤2 | ≥20 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |

Table 6.2.4-10: A-MPR for "NS_15" for E-UTRA highest channel edge ≤ 845 MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 5 | RBend [RB] |  |  | 19-24 |
|  | LCRB [RB] |  |  | ≥18 |
|  | A-MPR [dB] |  |  | ≤2 |
| 10 | RBend [RB] | 0-4 | 29-44 | 45-49 |
|  | LCRB [RB] | ≤2 | ≥24 | >0 |
|  | A-MPR [dB] | ≤4 | ≤4 | ≤9 |
| 15 | RBend [RB] | 0-12 | 44-61 | 62-74 |
|  | LCRB [RB] | ≤2 | ≥20 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |

Table 6.2.4-11: A-MPR for "NS_16" with channel lower edge at ≥807 MHz and <808.5 MHz

| Channel bandwidth [MHz] | Parameter | Region A | Region B | Region C | Region D | Region E |
| --- | --- | --- | --- | --- | --- | --- |
| 3 MHz | RBstart | 0 | 1-2 |  |  |  |
|  | LCRB [RBs] | ≥12 | 12 |  |  |  |
|  | A-MPR [dB] | ≤2 | ≤1 |  |  |  |
| 5 MHz | RBstart | 0-1 | 2 | 2-9 | 2-5 |  |
|  | LCRB [RBs] | 1 - 25 | 12 | 15-18 | 20 |  |
|  | A-MPR [dB] | ≤5 | ≤1 | ≤2 | ≤3 |  |
| 10 MHz | RBstart | 0 - 8 | 0-14 |  | 15-20 | 15-24 |
|  | LCRB [RBs] | 1 - 12 | 15-20 | ≥24 | ≥30 | 24-27 |
|  | A-MPR [dB] | ≤5 | ≤3 | ≤7 | ≤3 | ≤1 |

Table 6.2.4-12: A-MPR for "NS_16" with channel lower edge at ≥808.5 MHz and <812 MHz

| Channel bandwidth [MHz] | Parameter | Region A | Region B | Region C | Region D | Region E |
| --- | --- | --- | --- | --- | --- | --- |
| 5 MHz | RBstart | 0 | 0-1 | 1-5 |  |  |
|  | LCRB [RBs] | 16-20 | ≥24 | 16-20 |  |  |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤1 |  |  |
| 10 MHz | RBstart | 0-6 |  | 0-10 | 0-14 | 11-20 |
|  | LCRB [RBs] | 1-12 | 15-20 | 24-32 | ≥36 | 24-32 |
|  | A-MPR [dB] | ≤5 | ≤2 | ≤4 | ≤5 | ≤1 |

Table 6.2.4-13: A-MPR for "NS_16" with channel lower edge at ≥812 MHz

| Channel bandwidth [MHz] | Parameter | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 10 MHz | RBstart | 0 - 9 | 0 | 1-14 | 0-5 |
|  | LCRB [RBs] | 27-32 | 36-40 | 36-40 | ≥45 |
|  | A-MPR [dB] | ≤1 | ≤2 | ≤1 | ≤3 |

Table 6.2.4-14: A-MPR for "NS_19"

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 10 | RBstart |  |  | 0-6 |
|  | LCRB [RBs] |  |  | ≥40 |
|  | A-MPR [dB] |  |  | ≤1 |
| 15 | RBstart | 0-6 |  | 7-20 |
|  | LCRB [RBs] | ≤18 | ≥36 | ≥42 |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤2 |
| 20 | RBstart | 0-14 |  | 15-30 |
|  | LCRB [RBs] | ≤40 | ≥45 | ≥50 |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤2 |

Table 6.2.4-15: A-MPR for "NS_20"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | < 2007.5 |  |  | 2007.5 ≤ Fc < 2012.5 |  |  |  |  |  |  |  |  |  |  |  | 2012.5 ≤ Fc ≤ 2017.5 |  |  |  |
|  | RBstart | ≤24 |  |  | 0-3 |  |  |  |  |  |  |  | 4-6 |  |  |  | ≤24 |  |  |  |
|  | LCRB [RBs] | >0 |  |  | 15-19 |  |  |  | ≥20 |  |  |  | ≥18 |  |  |  | 1-25 |  |  |  |
|  | A-MPR [dB] | ≤17 |  |  | ≤1 |  |  |  | ≤4 |  |  |  | ≤2 |  |  |  | ≤ 0 |  |  |  |
| 10 | Fc [MHz] | 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-25 |  |  |  |  | 26-34 |  |  |  |  |  |  |  |  | 35-49 |  |  |  |  |
|  | LCRB [RBs] | >0 |  |  |  |  | 8-15 |  |  |  |  | >15 |  |  |  | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤16 |  |  |  |  | ≤2 |  |  |  |  | ≤5 |  |  |  | ≤ 6 |  |  |  |  |
|  | Fc [MHz] | 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-5 |  |  |  |  |  |  |  |  | 6-10 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | ≥32 |  |  |  |  |  |  |  |  | ≥40 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |  |  |  |  |  | ≤2 |  |  |  |  |  |  |  |  |  |
| 15 | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-14 |  |  |  |  |  | 15-24 |  |  |  |  |  |  |  |  | 25-39 |  | 61-74 |  |
|  | LCRB [RBs] | 1-9 & 40-75 |  | 10-39 |  |  |  | 24-29 |  |  |  |  |  | ≥30 |  |  | ≥36 |  | ≤6 |  |
|  | A-MPR [dB] | ≤11 |  | ≤6 |  |  |  | ≤1 |  |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤6 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-21 | 22-31 |  |  |  |  |  |  | 32-38 |  |  |  |  | 39-49 |  |  | 50-68 |  | 69-99 |
|  | LCRB [RBs] | >0 | 1-9 & 31-75 |  |  | 10-30 |  |  |  | ≥15 |  |  |  |  | ≥24 |  |  | ≥25 |  | >0 |
|  | A-MPR [dB] | ≤17 | ≤12 |  |  | ≤6 |  |  |  | ≤9 |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤16 |
| NOTE 1: When NS_20 is signaled the minimum requirements for the 10 MHz bandwidth are specified for E-UTRA UL carrier center frequencies of 2005 MHz or 2015 MHz. NOTE 2: When NS_20 is signaled the minimum requirements for the 15 MHz channel bandwidth are specified for E-UTRA UL carrier center frequency of 2012.5 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4-16: A-MPR for "NS_21"

| Channel Bandwidth [MHz] | Parameters | Region A |  | Region B |  |
| --- | --- | --- | --- | --- | --- |
| 10 | RBstart | 0 – 6 | 0 – 6 | N/A | N/A |
|  | RBend | N/A | N/A | 43 – 49 | 43 – 49 |
|  | LCRB [RBs] | 1 – 2 | 3 – 12, 32 - 50 | 1 – 2 | 3 – 12, 32 - 50 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 |

Table 6.2.4-17: A-MPR for "NS_22"

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 5 | No A-MPR is needed for 5 MHz channel bandwidth |  |  |  |  |
| 10 | RBstart | 0-13 | 0-17 | ≤ 6 | ≥12 |
|  | LCRB [RBs] | > 36 | 33-36 | ≤ 32 | ≤ 32 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥44 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 3 |
| 15 | RBstart | 0-24 | 0-38 | ≤ 14 | ≥ 23 |
|  | LCRB [RBs] | > 50 | 37-50 | ≤ 36 | ≤ 36 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥59 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| 20 | RBstart | 0-35 | 0-51 | ≤ 21 | ≥ 31 |
|  | LCRB [RBs] | > 64 | 49-64 | ≤ 48 | ≤ 48 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥79 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4-18: A-MPR for "NS_05"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Fc [MHz] | 1932.5 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-7 | 8 – 66 |  |  |  |  |  | 67-74 |  |  |  |
|  | LCRB [RBs] | ≥1 | ≤30 |  | 31 – 54 |  | >54 |  | ≤6 |  | >6 |  |
|  | A-MPR [dB] | ≤11 | 0 |  | ≤3 |  | ≤5 |  | ≤5 |  | ≤1 |  |
| 20 | Fc [MHz] | 1930 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-23 | 24-75 |  |  |  |  |  |  | 76-99 |  |  |
|  | LCRB [RBs] | ≥1 | ≤24 | 25 – 40 |  | 41 – 50 |  | > 50 |  | ≤6 |  | >6 |
|  | A-MPR [dB] | ≤11 | 0 | ≤3 |  | ≤5 |  | ≤10 |  | ≤5 |  | ≤1 |

Table 6.2.4-18E: A-MPR requirements for "NS_05" for Cat-M2 power class 3 UE

| BW [MHz] | 20 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fc [MHz] | 1930 |  |  |  |  |
| (NBindex,RBstart) | (0,<6) | (0,<6) | (1,<6) | (2,<6) | (2,<6) |
| LCRB | 9, 12 | ≥15 | ≥15 | 18 | 24 |
| A-MPR [dB] | ≤1 | ≤4 | ≤3 | ≤1 | ≤2 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. |  |  |  |  |  |

Table 6.2.4-19: A-MPR for "NS_24"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | Fc > [1987.5] |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0 - 24 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 0 - 24 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 10 |  |  |  |  |  |  |  |  |  |
| 10 | Fc [MHz] | 1975 < Fc ≤ 1985 |  |  |  |  |  |  | 1985<Fc≤1995 |  | Fc>1995 |
|  | RBstart | 0 - 1 | 2 - 14 |  | 15 - 26 | 36 - 49 |  |  | 0 - 49 |  | 0 - 49 |
|  | LCRB [RBs] | > 10 | ≥ 35 |  | N/A | ≤ 2 |  | > 11 | 0 - 49 |  | 0 - 49 |
|  | RBend | N/A | N/A |  | > 48 | N/A |  | N/A | N/A |  | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 2 |  | 1 | ≤ 3 |  | ≤ 1 | ≤ 9 |  | ≤ 17 |
| 15 | Fc [MHz] | 1972.5 < Fc ≤ 1987.5 |  |  |  |  |  |  |  | Fc > 1987.5 |  |
|  | RBstart | 0 - 11 |  |  |  |  | 12 - 74 |  |  | 0 - 74 |  |
|  | LCRB [RBs] | ≤ 45 |  | > 45 |  |  | > 3 |  |  | 0 - 74 |  |
|  | RBend | N/A |  | N/A |  |  | ≥ 45 |  |  | N/A |  |
|  | A-MPR [dB] | ≤ 2 |  | ≤ 8 |  |  | ≤ 7 |  |  | ≤ 17 |  |
| 20 | Fc [MHz] | Fc > 1970 |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0 - 99 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 0 - 99 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 17 |  |  |  |  |  |  |  |  |  |

Table 6.2.4-20: A-MPR for "NS_25"

| Channel Bandwidth [MHz] |  | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] |  | Fc > [1997.5] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart |  | 0 - 9 |  |  |  |  |  |  |  | 10 - 24 |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  | > 12 |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
|  | RBend |  | N/A |  |  |  |  |  |  |  | ≥ 22 |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  | ≤ 5 |  |  |  |  |  |  |  | ≤ 2 |  |  |  |  |  |  |  |
| 10 | Fc [MHz] |  | 1975 < Fc ≤ 1985 |  |  | 1985 < Fc ≤ 1995 |  |  |  |  |  |  |  | Fc > 1995 |  |  |  |  |
|  | RBstart |  | 0-1 | 2-49 |  | 0 |  |  |  | 1 - 18 |  | 19-49 |  | 0-6 |  | 7-15 |  | 16-49 |
|  | LCRB [RBs] |  | > 10 | N/A |  | ≤ 25 |  | > 25 |  | > 25 |  | N/A |  | N/A |  | > 20 |  | N/A |
|  | RBend |  | N/A | > 48 |  | N/A |  | N/A |  | N/A |  | > 42 |  | N/A |  | N/A |  | > 35 |
|  | A-MPR [dB] |  | ≤ 1 | ≤ 1 |  | ≤ 1 |  | ≤ 5 |  | ≤ 5 |  | ≤ 1 |  | ≤ 10 |  | ≤ 7 |  | ≤ 11 |
| 15 | Fc [MHz] |  | 1972.5 < Fc ≤ 1987.5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Fc > 1987.5 |  |
|  | RBstart |  | 0 - 4 |  | 5 - 30 |  |  |  | 31 - 62 |  |  |  | 63 - 74 |  |  |  | 0 - 74 |  |
|  | LCRB [RBs] |  | ≥ 15 |  | ≥ 45 |  |  |  | N/A |  |  |  | N/A |  |  |  | 0 - 74 |  |
|  | RBend |  | N/A |  | N/A |  |  |  | > 71 |  |  |  | N/A |  |  |  | N/A |  |
|  | A-MPR [dB] |  | ≤ 4 |  | ≤ 3 |  |  |  | ≤ 1 |  |  |  | ≤ 1 |  |  |  | ≤ 13 |  |
| 20 | Fc [MHz] |  | 1970 < Fc ≤ 1990 |  |  |  |  |  |  |  |  |  |  |  | Fc > 1990 |  |  |  |
|  | RBstart |  | 0 - 13 |  |  |  | 14 - 40 |  |  |  | 41 - 99 |  |  |  | 0 - 99 |  |  |  |
|  | LCRB [RBs] |  | N/A |  |  |  | ≥ 32 |  |  |  | N/A |  |  |  | 0 - 99 |  |  |  |
|  | RBend |  | N/A |  |  |  | N/A |  |  |  | > 72 |  |  |  | N/A |  |  |  |
|  | A-MPR [dB] |  | ≤ 11 |  |  |  | ≤ 11 |  |  |  | ≤13 |  |  |  | ≤ 13 |  |  |  |

Table 6.2.4-21: A-MPR for "NS_26"

| Bandwidth (MHz) | RBstart | L_crb | A-MPR |
| --- | --- | --- | --- |
| 10 | 0 - 10 | ≥ 1 | ≤ 1 |
| 15 | 0 - 17 | ≥ 1 | ≤ 1 |

Table 6.2.4-22: A-MPR for "NS_27"

| Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | RBstart | RBend | LCRB | A-MPR |
| 15 | 0 – 6 |  | ≤ 15 | ≤ 4 dB |
|  |  | 68 – 74 |  |  |
|  | ≥ 0 |  | ≥ 60 | ≤ 2 dB |
| 20 | 0 – 12 |  | ≤ 20 | ≤ 4 dB |
|  |  | 87 – 99 |  |  |
|  | 13 – 15 |  | ≤ 3 | ≤ 1 dB |
|  |  | 84 – 86 |  |  |
|  | ≥ 0 |  | ≥ 60 | ≤ 3 dB |

Table 6.2.4-23: A-MPR for "NS_28"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 5160 ≤ FC < 5179.8 5320.2 < FC ≤ 5340 5480 ≤ FC < 5499.8 | L = 10 (RIV = 19) | 9 |
|  |  | Any other RIV | 8 |
|  | 5680.2 < FC ≤ 5715 | L = 10 (RIV = 19) | 9 |
|  |  | Any other RIV | 8 |
|  | 5179.8 ≤ FC ≤ 5320.2 5499.8 ≤ FC ≤ 5680.2 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 2 |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-24: A-MPR for "NS_29"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 5179.8 ≤ FC ≤ 5320.2 5499.8 ≤ FC ≤ 5700.2 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 1 |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-25: A-MPR for "NS_30"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 5160 ≤ FC < 5179.8 5320.2 < FC ≤ 5340 | L = 10 (RIV = 19) | 15 |
|  |  | Any other RIV | 16 |
|  | 5179.8 ≤ FC < 5199.8 5300.2 < FC ≤ 5320.2 | L = 10 (RIV = 19) | 3 |
|  |  | Any other RIV | 3 |
|  | 5480 ≤ FC < 5499.8 | L = 10 (RIV = 19) | 11 |
|  |  | Any other RIV | 11 |
|  | 5680.2 < FC ≤ 5715 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5199.8 ≤ FC ≤ 5300.2 5499.8 ≤ FC ≤ 5680.2 | L = 10 (RIV = 19) | 1 |
|  |  | Any other RIV | N/A |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-26: A-MPR for "NS_31"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| --- | --- | --- | --- |
| 20 | 5239.8 ≤ FC ≤ 5240.2 | L = 10 (RIV = 19) | 7 |
|  |  | Any other RIV | 7 |
|  | 5160 ≤ FC < 5179.8 5260 ≤ FC < 5279.8 5320.2 < FC ≤ 5340 5480 ≤ FC < 5499.8 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5680.2 < FC ≤ 5715 5805.2 < FC ≤ 5840 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5735 ≤ FC < 5764.8 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5179.8 ≤ FC ≤ 5220.2 5279.8 ≤ FC ≤ 5320.2 5499.8 ≤ FC ≤ 5680.2 5764.8 ≤ FC ≤ 5805.2 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 1 |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-27: A-MPR for “NS_36”

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 MHz | Fc [MHz] | 700.5 ≤ Fc < 705.5 |  |  |  |
|  | RBstart | 0 |  | 1-4 |  |
|  | LCRB [RBs] | 1 | ≥ 15 | 15 - 23 | 24 |
|  | A-MPR [dB] | ≤ 2 | ≤ 4 | ≤ 1 | ≤ 3 |
| 10 MHz | Fc [MHz] | 703 ≤ Fc < 708 |  |  |  |
|  | RBstart | 0 - 8 | 9 - 12 | 13 - 39 | 40 - 43 |
|  | LCRB [RBs] | > 0 | > 12 | > 16 | > 0 |
|  | A-MPR [dB] | ≤ 10 | ≤ 8 | ≤ 6 | ≤ 6 |
| 15 MHz | Fc [MHz] | Fc = 705.5 |  |  |  |
|  | RBstart | 0 - 16 | 17 - 23 | 24 - 56 | 57 - 60 |
|  | LCRB [RBs] | > 0 | > 12 | > 20 | < 6 |
|  | A-MPR [dB] | ≤ 10 | ≤ 9 | ≤ 7 | ≤ 6 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4-28: A-MPR for "NS_38"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  | Region E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 1427.7 ≤ FC < 1429.1 | A-MPR [dB] | ≤ [16] |  |  |  |  |  |  |  |  |  |
| 3 | 1428.5 ≤ FC < 1431.5 | RBstart | 0 |  | 1– 2 |  | 3 – 6 |  |  |  |  |  |
|  |  | LCRB [RBs] | 1 | ≥2 | 2 | ≥ 3 | 5-7 | ≥8 |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 11 | ≤ 16 | ≤ 9 | ≤ 15 | ≤ 12 | ≤ 14 |  |  |  |  |
| 5 | 1429.5 ≤ FC < 1434.5 | RBstart | 0 - 1 |  | 2 – 4 |  | 5 – 7 |  | 8 – 10 |  |  |  |
|  |  | LCRB [RBs] | 2 | ≥3 | 4-6 | ≥ 7 | 8-10 | ≥11 | 12-14 | ≥15 |  |  |
|  |  | A-MPR [dB] | ≤ 12 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 15 | ≤ 10 | ≤ 12 |  |  |
| 10 | 1432 ≤ FC < 1442 | RBstart | 0 |  | 1 – 4 |  | 5 – 9 |  | 10 – 14 |  | 15 – 21 |  |
|  |  | LCRB [RBs] | 3-4 | ≥5 | 5-8 | ≥ 9 | 9-16 | ≥17 | 15-23 | ≥24 | 24-29 | ≥30 |
|  |  | A-MPR [dB] | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 11 | ≤ 14 | ≤ 11 | ≤ 13 |
| 15 | 1434.5 ≤ FC < 1447.5 | RBstart | 0 -1 |  | 2 – 7 |  | 8 – 17 |  | 18 – 23 |  | 24 – 30 |  |
|  |  | LCRB [RBs] | 5-10 | ≥11 | 8-12 | ≥ 13 | 15-25 | ≥26 | 26-40 | ≥41 | ≥ 36 |  |
|  |  | A-MPR [dB] | ≤ 14 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 14 | ≤ 12 |  |
| 20 | 1437 ≤ FC < 1450 | RBstart | 0 -4 |  | 5 – 9 |  | 10 – 19 |  | 20 – 29 |  | 30 – 40 |  |
|  |  | LCRB [RBs] | 6-10 | ≥11 | 11-20 | ≥ 21 | 17-32 | ≥33 | 28-45 | ≥46 | 41-54 | ≥55 |
|  |  | A-MPR [dB] | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 14 | ≤ 11 | ≤ 13 |

Table 6.2.4-29: A-MPR for "NS_39"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 1462 < FC ≤1465 | RBstart | 43-50 |  |  |  |  |  |  |
|  |  | LCRB [RBs] | ≤ 3 | > 3 and ≤ 25 | ≥ 32 |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 |  |  |  |  |
| 15 | 1456.3 < FC ≤ 1462.5 | RBstart | 56-75 |  |  | 64-75 |  | 60-75 |  |
|  |  | LCRB [RBs] | ≥ 36 |  |  | ≥ 13 and ≤ 35 |  | ≥ 5 and ≤ 12 | < 5 |
|  |  | A-MPR [dB] | ≤ 3 |  |  | ≤ 2 |  | ≤ 3 | ≤ 4 |
| 20 | 1450.8 < FC ≤ 1460 | RBstart | 71-100 |  |  | >76 |  |  |  |
|  |  | LCRB [RBs] | ≥ 36 |  |  | ≥ 7 and ≤ 35 | < 7 |  |  |
|  |  | A-MPR [dB] | ≤ 3 |  |  | ≤ 3 | ≤ 4 |  |  |

Table 6.2.4-30a: A-MPR for "NS_40"

| Channel bandwidth confined to 1427-1432MHz (B51) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  |
| 3MHz | 1428.5 MHz ≤ FC ≤ 1430.5 MHZ | RBstart | 0 |  | 1– 2 |  | 3 – 6 |  |  |  |
|  |  | LCRB [RBs] | 1 | ≥2 | 2 | ≥ 3 | 5-6 | ≥8 |  |  |
|  |  | A-MPR [dB] | ≤ 11 | ≤ 16 | ≤ 9 | ≤ 15 | ≤ 12 | ≤ 14 |  |  |
| 5MHz | 1429.5 MHz ≤ FC ≤ 1432 MHz | RBstart | 0 - 1 |  | 2 – 4 |  | 5 – 7 |  | 8 – 10 |  |
|  |  | LCRB [RBs] | 2 | ≥3 | 4-6 | ≥ 7 | 8-10 | ≥11 | 12 | ≥15 |
|  |  | A-MPR [dB] | ≤ 12 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 15 | ≤ 10 | ≤ 12 |

Table 6.2.4-30b: A-MPR for "NS_40"

| Channel bandwidth confined to 1429-1432MHz (B51) |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |
| 3MHz | 1430.5 MHz < FC ≤ 1432 MHZ | RBstart | 0 |
|  |  | LCRB [RBs] | 12 |
|  |  | A-MPR [dB] | ≤ 9 |

Table 6.2.4-31: A-MPR for "NS_41"

| Channel bandwidth confined to 1432-1452MHz (B50) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | Parameters | Region A | Region B |  | Region C | Region D | Region E | Region F | Region G | Region H |
| 3MHz | RBstart |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  |  |  |  |  |  |  |  |  |
| 5MHz | RBstart |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  |  |  |  |  |  |  |  |  |
| 10MHz | RBstart | 0 – 4 | 5 – 6 |  |  |  |  |  |  |  |
|  | LCRB [RBs] | ≥ 36 | ≥ 40 |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 11 | ≤ 10 |  |  |  |  |  |  |  |
| 15MHz | RBstart | 0-1 | 02-06 |  | 07-11 | 12-13 | 14-15 |  |  |  |
|  | LCRB [RBs] | ≥ 36 | ≥ 40 |  | ≥ 48 | 26-40 | ≥ 54 |  |  |  |
|  | A-MPR [dB] | ≤ 12 | ≤ 12 |  | ≤ 11 | ≤ 10 | ≤ 10 |  |  |  |
| 20 MHz | RBstart | 0 | 1 – 4 |  | 5-6 | 7-9 | 10 – 11 | 12 – 16 | 17 – 20 | 21 – 26 |
|  | LCRB [RBs] | ≥ 36 | 45-53 | ≥ 54 | ≥ 33 | ≥ 48 | ≥ 50 | ≥ 60 | ≥ 64 | ≥ 64 |
|  | A-MPR [dB] | ≤ 12 | ≤ 11 | ≤ 12 | ≤ 12 | ≤ 12 | ≤ 12 | ≤ 12 | ≤ 11 | ≤ 10 |

Table 6.2.4-32: A-MPR for “NS_42”

| Channel bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  | Region E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3MHz | 1512.5 MHz ≤ FC ≤ 1515.5 MHZ | RBstart | 0 – 3 |  | 4 – 7 |  | 8-10 |  | 11-14 |  |  |  |
|  |  | LCRB [RBs] | 8-11 | ≥ 12 | ≥ 8 |  | ≥ 3 |  | ≥ 1 |  |  |  |
|  |  | A-MPR [dB] | ≤ 3 | ≤ 5 | ≤ 6 |  | ≤ 3 |  | ≤ 3 |  |  |  |
| 3MHz | 1511 MHz ≤ FC < 1512.5 MHz | RBstart | 0 |  |  |  |  |  |  |  |  |  |
|  |  | LCRB [RBs] | 12 |  |  |  |  |  |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 1 |  |  |  |  |  |  |  |  |  |
| 5MHz | 1507 MHz ≤ FC ≤ 1514.5 MHz | RBstart | 0 – 5 |  | 6 – 11 |  | 12-16 |  | 17-24 |  |  |  |
|  |  | LCRB [RBs] | 12-16 | ≥ 17 | 9-12 | ≥ 13 | 5-8 | ≥ 9 | 1-6 | ≥ 7 |  |  |
|  |  | A-MPR [dB] | ≤ 5 | ≤ 9 | ≤ 6 | ≤ 10 | ≤ 4 | ≤ 9 | ≤ 4 | ≤ 6 |  |  |
| 10MHz | 1502 MHz ≤ FC ≤ 1512 MHz | RBstart | 0 – 22 |  | 23 – 27 |  | 28-35 |  | 36-49 |  |  |  |
|  |  | LCRB [RBs] | ≥ 20 |  | 15-19 | ≥ 20 | 3-14 | ≥ 15 | 1-9 | ≥ 10 |  |  |
|  |  | A-MPR [dB] | ≤ 4 |  | ≤ 4 | ≤ 9 | ≤ 4 | ≤ 10 | ≤ 3 | ≤ 8 |  |  |
| 10MHz | 1497 MHz ≤ FC < 1502 MHz | RBstart | 0 – 5 |  |  |  |  |  |  |  |  |  |
|  |  | LCRB [RBs] | 40 |  |  |  |  |  |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 1 |  |  |  |  |  |  |  |  |  |
| 15MHz | 1497 MHz ≤ FC ≤ 1509.5 MHz | RBstart | 0 – 22 |  | 23 – 48 |  | 49-60 |  | 61-74 |  |  |  |
|  |  | LCRB [RBs] | ≥ 36 |  | 9-23 | ≥ 24 | 1-14 | ≥ 15 | ≥ 1 |  |  |  |
|  |  | A-MPR [dB] | ≤ 4 |  | ≤ 3 | ≤ 5 | ≤ 3 | ≤ 9 | ≤ 4 |  |  |  |
| 20 MHz | 1492 MHz ≤ FC ≤ 1507 MHz | RBstart | 0 – 20 |  | 21 – 40 |  | 41 – 60 |  | 61 – 80 |  | 81 – 99 |  |
|  |  | LCRB [RBs] | 24-47 | ≥ 48 | ≥ 40 |  | 15-29 | ≥ 30 | 1-17 | ≥18 | 1-14 | ≥ 15 |
|  |  | A-MPR [dB] | ≤ 2 | ≤ 4 | ≤ 3 |  | ≤2 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 | ≤ 6 |

Table 6.2.4-32a: Void

Table 6.2.4-32b: Void

Table 6.2.4-33: A-MPR for “NS_43”

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth (MHz) | Carrier centre frequency (FC) (MHz) | Uplink resource allocation | A-MPR (dB) |
| 20 | 3580 ≤ FC ≤ 3670 | L = 10 (RIV = 19) | [1] |
|  |  | Any other RIV | [2] |
|  | 3560 ≤ FC < 3580 3670 < FC ≤ 3690 | L = 10 (RIV = 19) | [4] |
|  |  | Any other RIV | [4] |
| NOTE 1: The uplink resource allocation is defined in Clause 8.1.4 of TS 36.213 [6]. |  |  |  |

Table 6.2.4-34: A-MPR requirements for “NS_44” for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Fc [MHz] | ≥2605 |  |  |  | <2605 |
|  | RBstart | 0 – 5 |  | 38-49 |  | 0 - 49 |
|  | LCRB [RBs] | ≥ 45 |  | < 13 |  | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | NA |  | N/A |
|  | A-MPR [dB] | ≤ 1 |  | ≤ 1 |  | 0 |
| 15 | Fc [MHz] | ≥2597.5 |  |  |  | <2597.5 |
|  | RBstart | 0 – 18 | 19-30 |  | 54-74 | 0 - 74 |
|  | LCRB [RBs] | N/A | ≥ 45& <56 |  | < 17 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 63 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 1 | ≤ 1 |  | ≤ 1 | 0 |
| 20 | Fc [MHz] | ≥2590 |  |  |  | <2590 |
|  | RBstart | 0 – 31 | 32-55 |  | 70-99 | 0 - 99 |
|  | LCRB [RBs] | N/A | ≥ 45& <68 |  | < 19 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 76 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 1 |  | ≤ 1 | 0 |

Table 6.2.4-34a: A-MPR for "NS_56"

| Channel bandwidth confined to 1627.5- 1637.5MHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region A | Region B |  | Region C | Region D | Region E | Region F | Region G |
| 5 MHz | 1630.0, 1630.3 | RBstart | ≤ 8 | ≤ 8 |  | N/A | N/A | N/A | N/A | N/A |
|  |  | LCRB [RBs] | ≤ 8 | > 8 |  | N/A | N/A | N/A | N/A | N/A |
|  |  | A-MPR [dB] | 8 | 2 |  | N/A | N/A | N/A | N/A | N/A |
|  | 1635.0 | No A-MPR needed |  |  |  |  |  |  |  |  |
|  | 1649.0 |  |  |  |  |  |  |  |  |  |
|  | 1654.0 |  |  |  |  |  |  |  |  |  |
| 10 MHz | 1632.5 | RBstart | ≤ 5 | ≤ 5 | 6 to18 | ≤ 18 | 35 to 39 | 35 to 39 | ≥ 40 | ≥ 40 |
|  |  | LCRB [RBs] | ≤ 5 | 6 to 12 | ≤ 12 | > 12 | ≤ 7 | > 7 | ≤ 7 | > 7 |
|  |  | A-MPR [dB] | 7 | 5 |  | 7 | 4 | 2 | 5 | 3 |
|  | 1651.5 | No A-MPR needed |  |  |  |  |  |  |  |  |

Table 6.2.4-34b: A-MPR for "NS_06" for Power Class 1 UE in Band 12

| Channel bandwidth [MHz] | LCRB | Allocation positions | Modulation / A-MPR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 1.4 | all | all | 0 | 0 | 0 | 0 |
| 3 | = 15 | RBstart = 0 | 1 | 0.5 | 0 | 0 |
| 5 | 15 | RBstart 2 or RBstart + LCRB 23 | 1 | 0.5 | 0 | 0 |
| 10 | 18 | RBstart 2 or RBstart + LCRB 48 | 1 | 0.5 | 0 | 0 |

For PRACH, PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each TTI pattern, the A-MPR shall be evaluated per Teval period as specified in table 6.2.4-35 and given by the maximum value taken over the transmission(s) within that period; the maximum A-MPR over the TREF is then applied for TREF.

Table 6.2.4-35: A-MPR evaluation period

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2.5 apply.

***************************** End of changes ************************************